Turning Off DTCs

- Open a file browser and navigate to G:\Rewrite\Configuration\Components\Emissions\

- Back up EmissionsFaults.xml before making modifications

- Right click on EmissionsFaults.xml and open with notepad

- Locate the fault you would like to turn off (edit->find and search for the fault code)

Line will be similar to the following:

<ModuleFault_0572 Code="P0572” Description="Cruise Control Brake Switch Circuit 1 Low Voltage" FailureStatusMask="0x12"/>

- Change the value of FailureStatusMask to 0x00

<ModuleFault_0572 Code="P0572” Description="Cruise Control Brake Switch Circuit 1 Low Voltage" FailureStatusMask="0x12"/>

Becomes

<ModuleFault_0572 Code="P0572” Description="Cruise Control Brake Switch Circuit 1 Low Voltage" FailureStatusMask="0x00"/>

- Save and closed the file when finished with all modifications

Adding DTCs

- Open a file browser and navigate to G:\Rewrite\Configuration\Components\Emissions\

- Back up EmissionsFaults.xml before making modifications

- Right click on EmissionsFaults.xml and open with notepad

- Copy and paste a current fault line and replace ModuleFault_XXXX , Code=”XXXXX” , Description=”X” , and FailureStatusMask=”0xXX” with desired values (see below for failure status mask description)

Ie

P0719 TCC Brake Switch Input Failed LOW

<ModuleFault_0719 Code="P0719" Description="TCC Brake Switch Input Failed Low" FailureStatusMask="0x16"/>

- Save and closed the file when finished with all modifications

Failure Status Mask Values

0x00 = Ignore DTC

0x16 = Fail for: History DTCs, Test Not Passed Since DTC Cleared, and Current DTCs

0x12 = Fail for History DTCs and Current DTCs

0x10 = Fail for: History DTCs

0x04 = Fail for: Test Not Passed Since DTC Cleared

0x02 = Fail for: Current DTCs

0x14 = Fail for History and Test Not Passed Since DTC Cleared

0x06 = Fail for Test Not Passed Since DTC Cleared and Current DTCs

Bolded are the masks currently in use

From GMW 3110

historyDTC

A history DTC indicates that a current DTC status has met

sufficient criteria for storing a code into long term memory.

Sufficient criteria may consist of a current status detected over a

span of time or over multiple test cycles.

If DTC criteria to activate a warning indicator are satisfied, then

the DTC history bit shall be set to a value of 1 and the DTC status

must be stored to long term memory.

1. Bit set to 1 = DTC is history.

2. Bit set to 0 = DTC is not history.

3. Bit will be set to a value of 0 on a successful code clear.

testNotPassedSinceDTCCleared

This bit shall indicate a value of 0 once the test indicates a

passed result. A value of 1 indicates that the test has not run, or

that the test has run and failed.

1. Bit set to 1 = Test not passed since DTC cleared.

2. Bit set to 0 = Test passed since DTC cleared.

3. Bit will be set to a value of 1 on a successful code clear.

currentDTC

A current DTC shall indicate that test conditions have been met

and the test results show that a fault is currently present. This bit

shall indicate the results of the last diagnostic test performed. A

current code status may span multiple power cycles since it

reflects the result of the last test performed.

1. Bit set to 1 = DTC is current.

2. Bit set to 0 = DTC is not current.

3. Bit will be set to a value of 0 on a successful code clear.