Vehicle Build File Modification Procedure

Modifying Build File

DVT

- Open a file browser and navigate to G:\Rewrite\Configuration\VehicleTest\BuildRecords\

- Locate the build file you would like to modify.  Files are named: PsModelCode_bookCode_ModelYear_LotNumber.xml

Note: If the lot number is “00P” DVT/Flash Station will load file with lot number 000

- Create a backup of this file (copy to desktop or another location)

- Right click on the file in G:\Rewrite\Configuration\VehicleTest\BuildRecords\ and open with notepad

- Locate and modify the part numbers required ie if the old Part Number is 12640754 and the new part number is  12640755 <PCMSoftwareModule03PartNo>12640754Z0</PCMSoftwareModule03PartNo>

Becomes

<PCMSoftwareModule03PartNo>12640755Z0</PCMSoftwareModule03PartNo>

- Once finished, save and close the file

- Copy the file to your USB memory stick to be loaded on the flash station.

Flash Station

- Open a file browser and navigate to C:\FlashStation\BuildFiles

- Copy file from your USB memory stick to C:\FlashStation\BuildFiles

Restore Backup Build File

DVT

- Copy the backup file created in DVT step 3 to G:\Rewrite\Configuration\VehicleTest\BuildRecords\ and your USB memory stick.

Flash Station

- Copy the backup file from your USB memory stick to C:\FlashStation\BuildFiles