General Motors Corporation Third Generation

Protocol Specification

January 24, 2011

General Motors Corporation, Common DVT Systems

INTRODUCTION

This document describes a new method of communication between the Test Stand and external test equipment such as an ALDL test head.  The objectives of the new protocol are; 1.  Allow for Traction Control Testing (test results and values tested), 2.  Improve ABS testing by increasing communication speeds and adding test results and values tested to the data, 3.  Improve data integrity, and 4.  Allow for future expansion.  All objectives are met without affecting current communications so that current tests and debug tools will still operate.

1.	Specification Change Log	4

2.	Overview of New Communication Method	6

3.	Detailed Specification	8

3.1	Message Format	8

3.1.1	Message Identification Byte	8

3.1.2	Message Length Byte	8

3.1.3	Data Words	8

3.1.4	Check Sum	8

3.2	Communication Protocol	8

3.3	Message Definition	10

3.3.1	Mode 1 Definitions	10

3.3.1.1	Mode 1 Message 0:  Short Data List	10

3.3.1.1.1	Message Sent From Test Equipment to Test Stand	10

3.3.1.1.2	Message Sent From Test Stand to Test Equipment	10

3.3.1.2	Mode 1 Message 1: Reserved	12

3.3.1.2.1	Message Sent From Test Equipment to Test Stand	12

3.3.1.2.2	Message Sent From Test Stand to Test Equipment	12

3.3.1.3	 Mode 1 Message 2: Reserved	13

3.3.1.3.1	Message Sent From Test Equipment to Test Stand	13

3.3.1.3.2	Message Sent From Test Stand to Test Equipment	13

3.3.1.4	Mode 1 Message 3: Positraction Test Results List	13

3.3.1.4.1	Message Sent From Test Equipment to Test Stand	13

3.3.1.4.2	Message Sent From Test Stand to Test Equipment	13

3.3.1.5	Mode 1 Message 4: Axle Ratio Test Results List	14

3.3.1.5.1	Message Sent From Test Equipment to Test Stand	14

3.3.1.5.2	Message Sent From Test Stand to Test Equipment	14

3.3.1.6	Mode 1 Message 5: Read wheel distance of travel	15

3.3.1.6.1	Message Sent From Test Equipment to Test Stand	15

3.3.1.6.2	Message Sent From Test Stand to Test Equipment	15

3.3.1.7	Mode 1 Message 6: Vibration Analysis Test Results List	16

3.3.1.7.1	Message Sent From Test Equipment to Test Stand	16

3.3.1.7.2	Message Sent From Test Stand to Test Equipment	16

3.3.1.8	Mode 1 Message 7: Test Marker Data	16

3.3.1.8.1	Message Sent From Test Equipment to Test Stand	16

3.3.1.8.2	Message Sent From Test Stand to Test Equipment	17

3.3.1.9	Mode 1 Message 8: ABS Controller Data List	17

3.3.1.9.1	Message Sent From Test Equipment to Test Stand	17

3.3.1.9.2	Message Sent From Test Stand to Test Equipment	17

3.3.1.10	Mode 1 Message 9: Keyboard Parameter List	18

3.3.1.10.1	Message Sent From Test Equipment to Test Stand	18

3.3.1.10.2	Message Sent From Test Stand to Test Equipment	18

3.3.1.11	Mode 1 Message 10: Reserved	20

3.3.1.11.1	Message Sent From Test Equipment to Test Stand	20

3.3.1.11.2	Message Sent From Test Stand to Test Equipment	20

3.3.1.12	Mode 1 Message 11: Memory Pointers to Wheel Speed Data	20

3.3.1.12.1	Message Sent From Test Equipment to Test Stand	20

3.3.1.12.2	Message Sent From Test Stand to Test Equipment	21

3.3.1.13	Mode 1 Message 12: Software Revision Level	21

3.3.1.13.1	Message Sent From Test Equipment to Test Stand	21

3.3.1.13.2	Message Sent From Test Stand to Test Equipment	22

3.3.1.14	Mode 1 Message 13: Machine Configuration Parameters	23

3.3.1.14.1	Message Sent From Test Equipment to Test Stand	23

3.3.1.14.2	Message Sent From Test Stand to Test Equipment	23

3.3.1.15	Mode 1 Message 14: Machine Control Parameters	25

3.3.1.15.1	Message Sent From Test Equipment to Test Stand	25

3.3.1.15.2	Message Sent From Test Stand to Test Equipment	25

3.3.1.16	Mode 1 Message 15: 4WAL Vehicle DTC List	26

3.3.1.16.1	Message Sent From Test Equipment to Test Stand	26

3.3.1.16.2	Message Sent From Test Stand to Test Equipment	26

3.3.1.17	Mode 1 Message 16: 4WAL Machine DTC List	27

3.3.1.17.1	Message Sent From Test Equipment to Test Stand	27

3.3.1.17.2	Message Sent From Test Stand to Test Equipment	27

3.3.1.18	Mode 1 Message 17: Machine Error Codes List	28

3.3.1.18.1	Message Sent From Test Equipment to Test Stand	28

3.3.1.18.2	Message Sent From Test Stand to Test Equipment	28

3.3.1.19	Mode 1 Message 20: Quad Steering (QS4) data	29

3.3.1.19.1	Message Sent From Test Equipment to Test Stand	29

3.3.1.19.2	Message Sent From Test Stand to Test Equipment	29

3.3.1.20	Mode 1 Message 21: Short Speeds/Forces Message	29

3.3.1.20.1	Message Sent From Test Equipment to Test Stand	29

3.3.1.20.2	Message Sent From Test Stand to Test Equipment	30

3.3.1.21	Mode 1 Message 22: Brake/Speed Data Large message	32

3.3.1.21.1	Message Sent From Test Equipment to Test Stand	32

3.3.1.21.2	Message Sent From Test Stand to Test Equipment	32

3.3.2	Mode 2 Definition	33

3.3.2.1	Data Sent from the Test Equipment to the Test Stand.	33

3.3.2.2	Data Sent from the Test Stand to the Test Equipment	33

3.3.3	Mode 4 Definition	34

3.3.3.1	Data Sent from the Test Equipment to the Test Stand.	34

3.3.3.2	Data Sent from the Test Stand to the Test Equipment	36

3.3.3.3	Mode 4 Command Failsafes	37

3.3.4	Mode 6 Definition	37

3.3.4.1	Data Sent from the Test Equipment to the Test Stand.	37

3.3.4.2	Data Sent from the Test Stand to the Test Equipment.	37

3.3.5	Mode 7 Definition	38

3.3.5.1	Data Sent from the Test Equipment to the Test Stand.	38

3.3.5.2	Data Sent from the Test Stand to the Test Equipment.	38

3.3.6	Mode 20 Definition	38

3.2.6.1	Data Sent from the Test Equipment to the Test Stand.	38

3.3.6.2	Data Sent from the Test Stand to the Test Equipment.	39

3.3.7	Mode 42 (0x2A) Definition	41

3.3.7.1	Data Sent from the Test Equipment to the Test Stand.	41

3.3.7.2	Data Sent from the Test Stand to the Test Equipment.	41

4.	Appendices	42

4.1	Appendix A	42

4.1.1	Marker Types Table.	42

4.2	Appendix B	43

4.2.1	Marker Names Table	43

4.3	Appendix C	45

4.3.1	Machine Error Codes	45

4.3.1.1 	Burke E. Porter Machine Error Codes	45

4.3.1.2	 Durr Machine Error Codes	50

4.4	Appendix D	57

4.4.1	Roll Test Machine Software Revisions	57

4.4.1.1	Burke E Porter Machine Software Revisions	57

4.4.1.1.1 VME Based Machines	57

4.4.1.1.1 CCRT Based Machines	57

4.4.1.2	DURR Machine Software Revisions	58

4.5	 Figure 1	59

4.6	Sample Tests	60

4.6.1	Mode 1 message 21 graphed as test executes.	60

4.6.2	Burke E. Porter Machinery real-time graph data	61

4.6.3	Actual Test head command sequence	62

# Specification Change Log

- .  The additions and modifications are highlighted in gray in the log and in the specification.

- 3/18/99	G. McCraw added parameter #28 Boost  to Arm and Mode 4 Boost to Arm.  The parameter and Mode 4 are used for retesting and used for governored vehicles (trucks).

- 4/20/99	G. McCraw added Mode 07 and modified Message 07 and Mode 02

- 6/10/99	G. McCraw added Mode 20 to replace Mode 01 Message 01, 02 and 05.  Added marker tables for Mode 02 and 07.

- 10/12/99	G. McCraw added Mode 04 Reverse Test.

- 01/11/01	G. McCraw removed parameter #35 Axle Sample Speed.  Start Axle Ratio test command will be sent by the test head at the sample condition speed rather than using a speed parameter.  The axle test results will be available 1 second after sending the command.

- 01/11/01	G. McCraw removed mode 4 1F Hex/ 31 Dec Execute Reverse Gear test.  The reverse speeds will be given in reverse starting with FE, IF a wheel speed direction board is installed.

- 01/11/01	G. McCraw added parameter #86(100) Reverse Direction Board Installed.  On boot the roll test software will detect the reverse direction board and update parameter #86(100).  Parameter #86(100)  can be used to diagnose reverse direction board functionality and if reverse speeds are available.

- 01/11/01	G. McCraw, FYI, parameter file is now string based to allow editing file with any text editor.

- 01/11/01	G. McCraw added graph logging parameter #73(78).  This enables the engineer to store up to 100 pass and 100 fail brake results.  The Windows version of the BP utility must be used to view the files.  Use BEPGD.BAT file to quickly zip and copy files from DATA directory to A:\ ,floppy must be in “A” drive.

- 01/11/01	G. McCraw, FYI, a calibration file was added to log date, time, machine losses and roll factors. The file is string based and can be viewed with any text editor.

- 01/11/01	G. McCraw, FYI, a fault log file was added.  The file will log any faults displayed, date and time on the roll test monitor and when the fault went away.  The max.  size of the file is 1Mb.  The file is string based and can be viewed with any text editor.

- 01/11/01	G. McCraw, FYI, when a 172 board is present, NEVER use the reset button when updating software.  ALWAYS use the ABORT button.  A 172FLASH.EXE utility is available to flash 172 cards.

- 01/11/01	G. McCraw, FYI, PLC support was added for the following PLCs; LVT, 6008, V30B, RS LOGIC and GE.

- 01/11/01	G. McCraw, FYI, displayed S/W part numbers will be changed.  The new format will be Base Part # and Revision.  For Example; Base Part # = CD00278  Revision = L

- 01/11/01	G. McCraw, FYI, V5B boards will replace obsolete 1486 boards.  V5B will now handle the Baldor download/uploads and support 18H and 25M drives. 1486 boards are no longer available.

- 01/11/01	G. McCraw added two parameters.  Parameter #6 Front Retaining Rolls Relaxation Pressure and Parameter #7 Rear Retaining Rolls Relaxation Pressure.  Both are displayed in tenths.  For Example; 200 = 20Lbs.

- 01/11/01	G. McCraw removed parameters #71.2 (73) through #71.5 (76), #74(79), and #89.0 (103) though #90.7 (120) as not needed.

- 2/20/01	G. McCraw changed the Mode 1 Message 9 length formula to (N + 85) to make it common with the other messages.

- 5/25/02         D. Jones Added Traction control test and Park Brake test.

- 5/25/02 	D. Jones Enabled Clutch control commands.

- 5/25/02         D. Jones Allowed Axle Ration Test to be called at any time Armed or Not.

- 5/26/02	D. Jones Added Mode 1 Message 21 for higher message rate and signed speeds in tenths and signed forces for accelerations.

- 5/27/02 	D. Jones Added Mode 42 (0x2A) to allow for scheduling mode 1 messages at a 50 ms. rate.

- 5/30/02 	D. Jones Changed Mode 1 Message 21 to include all necessary information to run test curve. This should reduce the number of messages required to run a DVT test. Message should be able to sustain a 50 ms. rate.

- 10/08/02	B. Van Enk Added Mode 1 Message 20 for Quad Steering (QS4) rear thrust values and alignment test progress.

- 10/08/02	B. Van Enk Added Mode 1 Message 100 for Quad Steering (QS4) test completion status.

- 11/28/02	D. Jones Added Mode 1 Message 22 Read Measured data back in big blocks. 10 points at a time.

- 12/06/02	D. Jones Added Mode 1 Message 5 for reading the roll travel distances. Each roll is independent measured.

- 12/06/02       D. Jones Added Mode 4 command 0x21/ 33 = Zero wheel/roll distance counters

- 8/22/03	G. McCraw corrected the paragraph under 3.2 Communications Protocol.  Changed the text “terminates sending the previous protocol message every 500 ms” to “terminates sending the previous protocol message every 80 ms.” Changed the text  “to return to the old protocol or after no messages is received for 5 minutes” to “to return to the old protocol or after no messages is received for 10 seconds.”  This corrects the specification to reflect the way the software actually functions.

- 10/28/03	G. McCraw added dual height retainer rolls functionality.  Added Mode 1 Message 0 Byte 19, bit 5 & Message 21 Byte 12, bit 5.  Added Mode 4 command byte - 22Hex/34Dec Retaining Rolls Position, High/Low.

- 10/17/07	B. Conlin, R. Houston, R. Brown II updated document to reflect current functionality of Roll Test Machines. Also corrected the numbering scheme and have added an appendix D with 3rd Roll Test Software Revisions for Burke E Porter and DURR machines.

- 10/18/08	Bob Houston corrected an error in the document found by Tim Proctor.  The response for the Mode 1 Message 22, the check sum is listed as 169 when it should be 167.

- 1/24/11	B. Conlin added definition of Mode 1 Message 22 Test Stand Stuts Btye #5 bit 5 and Mode 1 Message 0 Test Stand Stuts Btye #5 bit 5.  Added Machine Error Fault definition in Appendix C.  Updated the Software Revision Tables.

# 2.	Overview of New Communication Method

Until commanded otherwise, the General Motors Corp. Test Stand will communicate exactly as it does today.  Once commanded to switch to the new communications method, the General Motors Corp. Test Stand will act as a slave device to the external test equipment.  This communication method will mimic the current GM Corporate Protocol for ALDL communications and the message format will switch from ASCII to binary.

Three of the 10 modes will be supported, Mode 1, Mode 4 and Mode 6.  Mode 1 is used to retrieve data from the test stand, Mode 4 is used to command the test stand to do something, and Mode 6 is used to download new parameters to the test stand.

Mode 1 will have multiple messages and is expandable in the future.  The messages that are currently defined are as follows:

Message 0.Short Data List.  This list includes four wheel speeds, prompt character, relative brake force, test completion status, and a test stand status byte.

Message 1.Reserved.

Message 2.Reserved.

Message 3.Positraction Test Result List.  This list includes pass/fail flags and values monitored during the test.

Message 4.Axle Ratio Test Result List.  This list includes pass/fail flags and values monitored during the test.

Message 5. Read roll distances travel since last Arm command or commanded distance reset.

Message 6.Vibration Analysis Test Results List.  This list includes the pass/fail flags and values monitored during the test.

Message 7.Test Marker Data. Retrival of test data markers.

Message 8.ABS Controller Data List.  This list contains data from the ABS controllers that communicate directly to the Test stand.

Message 9.Keyboard Parameter List.  This list contains all the keyboard parameters.

Message 10. Reserved.

Message 11.Memory Pointers to Wheel Speed Data. Retrieval of test data markers.

Message 12.Software Revision Level.  This list contains the software revision levels and machine type.

Message 13.Machine Configuration Parameters.  This list contains the roll factors and losses, card identifiers, calibration setup, English/metric, MPH/KPH, I/O option and machine serial number.

Message 14.Machine Control parameters.  This list contains the previous and last test results and printer controls.

Message 15.4WAL Vehicle DTC List.  This list contains up to ten DTCs that can be stored on 4WAL vehicles.

Message 16.4WAL Machine DTC List.  This list contains up to ten DTCs that can be stored by the BP machine for 4WAL vehicles.

Message 17.Machine Error Codes List.  This list contains up to ten machine error codes that could cause false test results.

Message 20. Quad Steering (QS4) message.  This message passes rear thrust values and several status bits that communicate the status of the alignment test.

Message 21. Short Speeds/Forces message. This message was keep short as possible to allow fast access. It has speeds to tenths of a MPH for better comparison to verious ABS’s. The speeds will reflect a negitive number if the vehicle is rolling reverse. The forces are signed as well to allow for acceration testing or brake testing.

Message 22 Read sampled data using big blocks 10 sample points at a time.

Mode 2.Message is used to retrieve markers set by the test equipment and related data.

Mode 4.Messages communicates commands to the test stand.  Some examples of these are: 1.  Reset,  2.  Declutch,   3.  Arm,  4.  ABS Vehicle, etc.

Mode 6.Message is used to download and use new Test Stand Parameters.  These parameters are specified one at a time.

Mode 7.Message is used to set markers from the test equipment in the test stand during the rolling resistance, base brake, and traction and ABS tests.

Mode 20.Message is used to retrieve brake data for the rolling resistance, base brake, traction, and ABS to determine a pass/fail status.

Mode 42 is used to setup a mode 1 message schedule with a 50ms. rate.  There are 20 slots each being a 50ms. time slot.  This helps to lighten the load of the DVT test requesting mode 1 messages.

# 3.	Detailed Specification

## 3.1	Message Format

Any and all data transmitted on the serial bus must be part of a message.

All messages must be of the following format:

Message Identification Byte (ID)

Message Length (85 + N)

N Bytes of Data

Check Sum

### 3.1.1	Message Identification Byte

The first word of each message shall be a Message Identification (ID) Byte.  Each Message ID must be unique; therefore, all message Ids will be assigned and tracked by GM.  Message Ids are used so that multiple remote units can receive and transmit data without interfering with each other’s communications.  Each remote unit, which is designed to transmit on the bus, may only do so after it has received a complete valid message that begins with its message ID. The Burke-Porter message ID

is 07.

### 3.1.2	Message Length Byte

The message length byte shall indicate the total number of data words in the remainder of the message plus 85 (decimal).  The maximum number of data bytes within one message that can be transmitted by any transmitter is 170 bytes.  Thus, a valid message length byte must lie in the range of 85 to 255.  Many messages with no data words are possible; for such messages, the Message Length Byte would contain the binary word 0101 0101 (MSB-LSB).  This pattern has been selected because, under an abnormally severe noise environment), there is a higher probability that an erroneously received message will be detected as such.

### 3.1.3	Data Words

Data words must be in a predetermined order, predetermined scaling, predetermined parameters or commands.  Detailed requirements will vary from application to application.

### 3.1.4	Check Sum

The last byte to be transmitted in a message is the two’s complement of the sum of all the other bytes in the message, including the message ID and message length bytes.  Any carryout of this eight bit byte while both the transmitter and receivers are forming it shall be neglected.  The two’s complement is used so that if the receivers sum all the bytes in the message, then the result should be zero for a valid message.

For a complete list of all duties to be performed, please read the excerpts from XDE-5024.

## 3.2	Communication Protocol

The new communication protocol is entered when the first valid new protocol message with the test stand’s ID is receive by the test stand.  At this point, the test stand processes the received message, terminates sending the previous protocol message every 80 ms, and waits for the next message before transmitting again.  For definition, the test stand is a “slave” device and the test equipment is the “master” device.  While a slave device, the test stand must start responding within 10 ms of receiving a valid message from the master.  The test stand remains a slave device until it receives a mode 4 message telling it to return to the old protocol or after no messages is received for 10 seconds.

The following is an example of how the new protocol can be entered and exited assuming the Test Stand ID to be hex 07.

| Message in hex | Time Stamp ms | Data Sent From |
| --- | --- | --- |
| 02 30 35 34 20 20 20 20 46 20 20 20 20 20 20 30 33 35 30 34 33 30 32 32 30 34 31 30 39 37 32 36 35 30 37 32 30 36 39 30 32 35 30 35 35 30 37 37 38 32 46 20 20 36 33 30 30 03 0D 0A | 0 | BP |
| 02 30 33 34 20 20 20 20 46 20 20 20 20 20 20 30 33 35 30 34 33 30 32 32 30 34 31 30 39 37 32 36 35 30 37 32 30 36 39 30 32 35 30 35 35 30 37 37 38 32 46 20 20 36 33 30 30 03 0D 0A | 500 | BP |
| 02 30 32 34 20 20 20 20 46 20 20 20 20 20 20 30 33 35 30 34 33 30 32 32 30 34 31 30 39 37 32 36 35 30 37 32 30 36 39 30 32 35 30 35 35 30 37 37 38 32 46 20 20 36 33 30 30 03 0D 0A | 1000 | BP |
| 07 57 01 00 CS | 1030 | ATH |
| 07 67 01 00 04 02 01 0B 0C 00 A1 00 A7 00 42 00 37 52 C0 40 CS | 1033 | BP |
| 07 57 01 01 CS | 1187 | ATH |
| 07 6C 01 01 08 80 00 23 00 2B 00 16 00 29 00 61 01 09 00 48 00 45 19 37 4D CS | 1190 | BP |
| 07 58 04 14 00 CS | 3010 | ATH |
| 07 56 04 CS | 3013 | BP |
| 07 58 04 FF 00 CS | 4126 | ATH |
| 02 30 30 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 4E 4E 4E 30 30 30 30 03 0D 0A | 4500 | BP |

## 3.3	Message Definition

The following is a list of defined messages to be communicated from the test stand to the test equipment when requested.  For a complete definition of all modes of communication, please see the CORPORATE SPECIFICATION FOR ALDL COMMUNICATIONS document.

### 3.3.1	Mode 1 Definitions

#### 3.3.1.1	Mode 1 Message 0:  Short Data List

##### 3.3.1.1.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID 07 |
| 2 |  | Message Length (N+85) 57 |
| 3 |  | Mode Number (01) 01 |
| 4 |  | Message Number (00) 00 |
| 5 |  | Check Sum A1 |

##### 3.3.1.1.2	Message Sent From Test Stand to Test Equipment

| Byte No. |  | Bit No. |  | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| _______ |  | ______ |  | ______________________________________________________________ |  |  |
| 1 |  |  |  | Message ID |  |  |
| 2 |  |  |  | Message Length (N+85) |  |  |
| 3 |  |  |  | Mode Number (01) |  |  |
| 4 |  |  |  | Message Number (00) |  |  |
| 5 |  |  |  | Left Front Wheel Speed in MPH (MSB) (MPH = X / 100) |  |
| 6 |  |  |  | Left Front Wheel Speed in MPH (LSB) 0000 to 7FFF = Forward |  |
| 7 |  |  |  | Right Front Wheel Speed in MPH (MSB) FFFF to 8000 = Reverse |  |
| 8 |  |  |  | Right Front Wheel Speed in MPH (LSB) |  |
| 9 |  |  |  | Left Rear Wheel Speed in MPH (MSB) |  |
| 10 |  |  |  | Left Rear Wheel Speed in MPH (LSB) |  |
| 11 |  |  |  | Right Rear Wheel Speed in MPH (MSB) |  |
| 12 |  |  |  | Right Rear Wheel Speed in MPH (LSB) |  |
| 13 |  |  |  | Absolute Brake Force lbs (MSB) |  |
| 14 |  |  |  | Absolute Brake Force lbs (LSB) |  |
| 15 |  |  |  | Test Completion Status Byte #1 |  |
|  |  | 0 |  | 1 = Reset Complete/VME-PLC Test Ready (0 = Reset in Progress/VME not Ready) |  |
|  |  | 1 |  | 1 = Test - Arm Test Complete/Shift to Neutral |  |
|  |  | 2 |  | 1 = Rolling Resistance (Drag) Test Complete /Apply Brakes |  |
|  |  | 3 |  | 1 = Traction Control Test Complete /Apply Brakes |  |
|  |  | 4 |  | 1 = Base Brake Test Complete/ Ready to Start ABS |  |
|  |  | 5 |  | 1 = ABS Test Complete |  |
|  |  | 6 |  | Undefined |  |
|  |  | 7 |  | 1 = Test Complete - (Sent at 0 wheel speed or Mode 4, 09, byte 5 End ABS test or Fault) |  |
| 16 |  |  |  | Test Completion Status Byte #2 |  |
|  |  | 0 |  |  | 1 = Max. Brake Force Test Complete |
|  |  | 1 |  | 1 = Max. Parking Brake Force Test Complete |  |
|  |  | 2 |  | 1 = Tire Calibration Programming Complete |  |

| Byte No. | Bit No. | Definition |  |  |
| --- | --- | --- | --- | --- |
| _______ | _______ | _______________________________________________________________ |  |  |
|  | 3 | 1 = Axle Ratio Test Complete |  |  |
|  | 4 | 1 = Positraction Test Complete |  |  |
|  | 5 | 1 = Vibration Analysis Test Complete |  |  |
|  | 6 | 1 = RWAL/4WAL stand alone ABS Test Complete |  |  |
|  | 7 | Undefined |  |  |
| 17 |  | Test Stand Status Byte #1 |  |  |
|  | 0 | 1 = Machine Error Faults 0=NO DTCs 1= DTCs Set |  |  |
|  | 1 | 1 = ALDL Ready to Arm /Neutral Speed Achieved | All clear at |  |
|  | 2 | 1 = 4WAL Vehicle DTC Set Flag, 0=No DTCs 1=DTCs Set | time of reset |  |
|  | 3 | 1 = 4WAL WSS Test Status, 0=Fail, 1=Pass |  |  |
|  | 4 | 1 = 4WAL Machine DTC Set Flag, 0=No DTCs 1=DTCs Set |  |  |
|  | 5 | 1 = Maximum Machine Speed Exceeded |  |  |
|  | 6 | Undefined |  |  |
|  | 7 | Undefined |  |  |
| 18 |  | Test Stand Status Byte #2 |  |  |
|  | 0 | 1 = Front Retaining Rolls Up |  |  |
|  | 1 | 1 = Rear Retaining Rolls Up |  |  |
|  | 2 | 1 = Declutched Front to Rear (Hydraulics) |  |  |
|  | 3 | 1 = Declutched Front Side to Side |  |  |
|  | 4 | 1 = Declutched Rear Side to Side |  |  |
|  | 5 | 1 = Adjusting Wheel Base |  |  |
|  | 6 | 1 = Emergency Stop was Depressed |  |  |
|  | 7 | 1 = Roll Test Ready/Retaining Rolls Up Locked |  |  |
| 19 |  | Test Stand Status Byte #3 |  |  |
|  | 0 | 1 = Front Right Roll Brake Applied |  |  |
|  | 1 | 1 = Front Left Roll Brake Applied |  |  |
|  | 2 | 1 = Rear Right Roll Brake Applied |  |  |
|  | 3 | 1 = Rear Left Roll Brake Applied |  |  |
|  | 4 | 1 = Pump Motor Running |  |  |
|  | 5 | 1 = Retaining Rolls High Position (Dual Height Option Only) |  |  |
|  | 6 | 1 = Front ESB Motor Driving |  |  |
|  | 7 | 1 = Rear ESB Motor Driving |  |  |
| 20 |  | Test Stand Status Byte #4 |  |  |
|  | 0 | 1 = Buzzer On |  |  |
|  | 1 | 1 = PLC Auxiliary 1 Feedback On |  |  |
|  | 2 | 1 = PLC Auxiliary 2 Feedback On |  |  |
|  | 3 | Undefined |  |  |
|  | 4 | Undefined |  |  |
|  | 5 | Undefined |  |  |
|  | 6 | Undefined |  |  |
|  | 7 | Undefined |  |  |
| 21 |  | Test Stand Status Byte #5 |  |  |
|  | 0 | 1 = Driver Force Front Minimum Fault |  |  |
|  | 1 | 1 = Driver Force Front Maximum Fault | All clear at |  |
|  | 2 | 1 = Driver Force Rear Minimum Fault | time of reset |  |
|  | 3 | 1 = Driver Force Rear Maximum Fault |  |  |
|  | 4 | Undefined |  |  |
|  | 5 | 1 = Driver Interface Button Pressed |  |  |
|  | 6 | Undefined |  |  |
|  | 7 | Undefined |  |  |

| Byte No. | Bit No. | Definition |  |
| --- | --- | --- | --- |
| ________ | ________ | ___________________________________________________ |  |
| 22 |  | Test Stand Status Byte #6 |  |
|  | 0 | Wheel Base Position #1 |  |
|  | 1 | Wheel Base Position #2 |  |
|  | 2 | Wheel Base Position #3 |  |
|  | 3 | Wheel Base Position #4 |  |
|  | 4 | Wheel Base Position #5 |  |
|  | 5 | Wheel Base Position #6 |  |
|  | 6 | Wheel Base Position #7 |  |
|  | 7 | Wheel Base Position #8 |  |
| 23 |  | Current Index (MSB) |  |
| 24 |  | Current Index (LSB) |  |
| 25 |  | Check Sum |  |

#### 3.3.1.2	Mode 1 Message 1: Reserved

##### 3.3.1.2.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (01) |

##### 3.3.1.2.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (01) |
| 5 |  | Check Sum |

#### 3.3.1.3		Mode 1 Message 2: Reserved

##### 3.3.1.3.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (02) |
| 5 |  | Check Sum |

##### 3.3.1.3.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (02) |
| 5 |  | Check Sum |

#### 3.3.1.4	Mode 1 Message 3: Positraction Test Results List

##### 3.3.1.4.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (03) |
| 5 |  | Check Sum |

##### 3.3.1.4.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (03) |
| 5 |  | 1 = Positraction Test Complete, 0 = Not Complete |
| 6 |  | Positraction Test Result in % |
| 7 |  | Check Sum |

#### 3.3.1.5	Mode 1 Message 4: Axle Ratio Test Results List

Axle Ratio Test Result (%) = ((LF+RF)/2)/((LR+RR)/2)*100

##### 3.3.1.5.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (04) |
| 5 |  | Check Sum |

##### 3.3.1.5.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (04) |
| 5 |  | 1 = Axle Ratio Test Complete, 0 = Not Complete |
| 6 |  | Axle Ratio Test Result in % *see note for calculation |
| 7 |  | Check Sum |

#### Mode 1 Message 5: Read wheel distance of travel

Applicable to Burke E Porter Roll Test Machines only:

Information: The roll test equipment uses 1250 pulse per revolution encoders.

1 roll revolution = 1250 encoder pulses

1 roll revolution = 111.919 inches of travel

1 encoder pulse = .0895352 inches of travel

1 mile = 566.1237 roll revolutions = 707654.64 encoder pulses

DURR Machine applications of this mode unknown at this date.

##### 3.3.1.6.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (05) |
| 5 |  | Check Sum |

##### 3.3.1.6.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (05) |
| 5 |  | LF Distance in encoder pulses 32bit MSB Bits 31-24 |
| 6 |  | Bits 23–16 |
| 7 |  | Bits 15-8 |
| 8 |  | Bits 7-0 LSB |
| 9 |  | LF Distance in encoder pulses 32bit MSB Bits 31-24 |
| 10 |  | Bits 23–16 |
| 11 |  | Bits 15-8 |
| 12 |  | Bits 7-0 LSB |
| 13 |  | LF Distance in encoder pulses 32bit MSB Bits 31-24 |
| 14 |  | Bits 23–16 |
| 15 |  | Bits 15-8 |
| 16 |  | Bits 7-0 LSB |
| 17 |  | LF Distance in encoder pulses 32bit MSB Bits 31-24 |
| 18 |  | Bits 23–16 |
| 19 |  | Bits 15-8 |
| 20 |  | Bits 7-0 LSB |
| 21 |  | Check Sum |

#### 3.3.1.7	Mode 1 Message 6: Vibration Analysis Test Results List

##### 3.3.1.7.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message I |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (06) |
| 5 |  | Check Sum |

##### 3.3.1.7.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (06) |
| 5 |  | 1 = Vibration Analysis Test Complete, 0 = Not Complete |
| 6 |  | Check Sum |

#### 3.3.1.8	Mode 1 Message 7: Test Marker Data

##### 3.3.1.8.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (07) |
| 5 |  | Marker Pointer (MSB) |
| 6 |  | Marker Pointer (LSB) |
| 7 |  | Type of Markers (MSB) |
| 8 |  | Type of Markers (LSB) |
| 9 |  | Check Sum |

##### 3.3.1.8.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (07) |
| 5 |  | Number of Markers (MSB) }Total number in data |
| 6 |  | Number of Markers (LSB) } |
| 7 |  | Marker Array Index (MSB) }Marker pointer in time |
| 8 |  | Marker Array Index (LSB) } |
| 9 |  | Marker Index (MSB) }Marker number |
| 10 |  | Marker Index (LSB) } |
| 11 |  | Marker Type (MSB) |
| 12 |  | Marker Type (LSB) |
| 13 |  | Marker (MSB) }Marker name See Table 1 below |
| 14 |  | Marker (LSB) } |
| 15...Nth |  | Marker Index 2nd....10th (MSB) |
| 16...Nth |  | Marker Index 2nd....10th (LSB) |
| 17...Nth |  | Marker Type 2nd....10th (MSB) |
| 18...Nth |  | Marker Type 2nd....10th (LSB) |
| 19...Nth |  | Marker 2nd....10th (MSB) |
| 20...Nth |  | Marker 2nd....10th (LSB) |
| 69 |  | Check Sum |

#### 3.3.1.9	Mode 1 Message 8: ABS Controller Data List

##### 3.3.1.9.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (08) |
| 5 |  | Check Sum |

##### 3.3.1.9.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (08) |
| 5 |  | Left Front Wheel Speed |
| 6 |  | Right Front Wheel Speed |
| 7 |  | Left Rear Wheel Speed |
| 8 |  | Right Rear Wheel Speed |
| 9 |  | Byte which contains Brake Switch Status |
| 10 |  | Check Sum |

#### 3.3.1.10	Mode 1 Message 9: Keyboard Parameter List

##### 3.3.1.10.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (09) |
| 5 |  | Check Sum |

##### 3.3.1.10.2	Message Sent From Test Stand to Test Equipment

DURR Machines do not have a human machine interface for keyboard parameter they are only downloaded from the test program.

BEP Parameter numbers are N-1.

| Byte No. | Parm # Keypad/Electronic | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (09) |
| 5 | 1 | Not Defined (MSB) |
| 6 |  | “ (LSB) |
| 7 | 2 | Neutral Speed (MSB) |
| 8 |  | Neutral speed (LSB) |
| 9 | 3 | Not Defined (MSB) |
| 10 |  | “ (LSB) |
| 11 | 4 | Not Defined (MSB) |
| 12 |  | “ (LSB) |
| 13 | 5 | Not Defined (MSB) |
| 14 |  | “ (LSB) |
| 15 | 6 | Start ABS Test Speed (MSB) |
| 16 |  | “ (LSB) |
| 17 | 7 | Front Retaining Rolls Relaxation Pressure (MSB) (in tenths – for example; 200 = 20Lbs) |
| 18 |  | “ (LSB) |
| 19 | 8 | Rear Retaining Rolls Relaxation Pressure (MSB) (in tenths – for example; 200 = 20Lbs) |
| 20 |  | “ (LSB) |
| 21 | 9 | Not Defined (MSB) |
| 22 |  | “ (LSB) |
| 23 | 10 | Not Defined (MSB) |
| 24 |  | “ (LSB) |
| 25 | 11 | Not Defined (MSB) |
| 26 |  | “ (LSB) |
| 27 | 12 | Not Defined (MSB) |
| 28 |  | “ (LSB) |
| 29 | 13 | Time Military #### (MSB) |
| 30 |  | “ (LSB) |
| 31 | 14 | Date MM/DD (MSB) |
| 32 |  | “ (LSB) |

| Byte No. | Parm # Keypad/Electronic | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 33 | 15 | Year #### (MSB) |
| 34 |  | “ (LSB) |
| 35 | 16 | Base Brake Test Time (MSB) (If Not Used = 0) |
| 36 |  | “ (LSB) |
| 37 | 17 | Stand Alone WSS Arm Speed (MPH) (MSB) |
| 38 |  | “ (LSB) |
| 39 | 18 | Total Stand Alone WSS Minimum Variation (%) (MSB) |
| 40 |  | “ (LSB) (Lowest Total Allowed, example = 95% or 00 95) |
| 41 | 19 | Total Stand Alone WSS Maximum Variation (%) (MSB) |
| 42 |  | “ (LSB) (Highest Total Allowed, example = 105% or 01 05) |
| 43 | 20 | Maximum Machine Speed (MSB) |
| 44 |  | “ (LSB) |
| 45 | 21 | Minimum Driver Fault Force Front (MSB) |
| 46 |  | “ (LSB) |
| 47 | 22 | Maximum Driver Fault Force Front (MSB) |
| 48 |  | “ (LSB) |
| 49 | 23 | Minimum Driver Fault Force Rear (MSB) |
| 50 |  | “ (LSB) |
| 51 | 24 | Maximum Driver Fault Force Rear (MSB) |
| 52 |  | “ (LSB) |
| 53 | 25 | General Motors Corp. Gen III (MSB) (Gen III Default = FF) |
| 54 |  | “ (LSB) (Gen III Default = FF) |
| 55 | 26 | Not Defined (MSB) |
| 56 |  | “ (LSB) |
| 57 | 27 | Boost Pressure (MSB) |
| 58 |  | “ (LSB) |
| 59 | 28 | Boost to Arm (MSB) (Used for Retest and Governed Vehicles on 3600 ONLY) |
| 60 |  | “ (LSB) (Can be changed anytime) |
| 61 | 29 | 2-Wheel TCS Sequence (MSB) - 0=L then R 1=R then L |
| 62 |  | “ (LSB) |
| 63 | 30 | 4-Wheel TCS Sequence (MSB) - 0=RF,LF,RR,LR 1=LF,RF,LR,RR |
| 64 |  | “ (LSB) |
| 65 | 31 | Motor Torque Loading Constant (3600 Only) (MSB) |
| 66 |  | “ (LSB) Note: The constant loading parameter is entered as percent motor torque. This term remains constant throughout the test. The intention is to simulate grade. |
| 67 | 32 | Motor Speed Loading Constant (3600 Only) (MSB) |
| 68 |  | “ (LSB) Note: The speed loading parameter is entered as percent motor torque at 100 mph. This term increases with the square of the speed. The intention is to simulate windage. The maximum we should put in this parameter is 60. |
| 69 | 33 | Motor Acceleration Loading (3600 Only) (MSB) |
| 70 |  | “ (LSB) Note: The acceleration loading parameter is entered as percent motor torque at 10 MPH/sec. This term responds only to changes in velocity. The intention is to simulate increased inertia. |
| 71 | 34 | Parking Brake Arm Speed (MSB) (360 & 3600) Results Listed in Mode 20 |
| 72 |  | “ (LSB) |
| 73 | 35 | Not Defined (MSB) |
| 74 |  | “ (LSB) |
| 75 | 36 | RWAL Valve Firing Time ms (MSB) |
| 76 |  | “ (LSB) |
| 77 | 37 | 4WAL Tire Calibration Index (MSB) |
| 78 |  | “ (LSB) |
| 79 | 38 | Max. Brake Force Start Speed (MSB) |
| 80 |  | “ (LSB) |
| 81 |  | Check Sum |

#### 3.3.1.11	Mode 1 Message 10: Reserved

##### 3.3.1.11.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (10) |
| 5 |  | Check Sum |

##### 3.3.1.11.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (10) |
| 5 |  | Check Sum |

#### 3.3.1.12	Mode 1 Message 11: Memory Pointers to Wheel Speed Data

##### 3.3.1.12.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (11) |
| 5 |  | Check Sum |

##### 3.3.1.12.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (11) |
| 5 |  | Base Drag Wheel Speed Memory Index (MSB) |
| 6 |  | Base Drag Wheel Speed Memory Index (LSB) |
| 7 |  | Traction Wheel Speed Memory Index (MSB) |
| 8 |  | Traction Wheel Speed Memory Index (LSB) |
| 9 |  | Base Brake Wheel Speed Memory Index (MSB) |
| 10 |  | Base Brake Wheel Speed Memory Index (LSB) |
| 11 |  | ABS Left Front Wheel Speed Memory Index (MSB) |
| 12 |  | ABS Left Front Wheel Speed Memory Index (LSB) |
| 13 |  | ABS Right Front Wheel Speed Memory Index (MSB) |
| 14 |  | ABS Right Front Wheel Speed Memory Index (LSB) |
| 15 |  | ABS Left Rear Wheel Speed Memory Index (MSB) |
| 16 |  | ABS Left Rear Wheel Speed Memory Index (LSB) |
| 17 |  | ABS Right Rear Wheel Speed Memory Index (MSB) |
| 18 |  | ABS Right Rear Wheel Speed Memory Index (LSB) |
| 19 |  | Check Sum |

#### 3.3.1.13	Mode 1 Message 12: Software Revision Level

##### 3.3.1.13.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (12) |
| 5 |  | Check Sum |

##### 3.3.1.13.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Parm # Keypad/Electronic | Definition |
| --- | --- | --- |
| _______ | ________________ | _____________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (12) |
| 5 | 71.0 / 71 | System Controller Base Software Number (MSB) |
| 6 |  | “ (LSB) |
| 7 | 71.1 / 72 | System Controller Revision Number (MSB) *Appendix D for Software Revision Log |
| 8 |  | “ (LSB) |
| 9 | 71.2 / 73 | Not Defined (MSB) |
| 10 |  | “ (LSB) |
| 11 | 71.3 / 74 | Not Defined (MSB) |
| 12 |  | “ (LSB) |
| 13 | 71.4 / 75 | Not Defined (MSB) |
| 14 |  | “ (LSB) |
| 15 | 71.5 / 76 | Not Defined (MSB) |
| 16 |  | “ (LSB) |
| 17 | 72 / 77 | Machine Type (MSB) 0=360, 1=3600, 2=5700, 3=1072, 4=4217, 50 = DURR |
| 18 |  | “ (LSB) 5-255=Undefined |
| 19 | 73 / 78 | Graph Logging (MSB) |
| 20 |  | “ (LSB) |
| 21 | 74 / 79 | Not Defined (MSB) |
| 22 |  | “ (LSB) |
| 23 |  | Check Sum |

#### 3.3.1.14	Mode 1 Message 13: Machine Configuration Parameters

##### 3.3.1.14.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (13) |
| 5 |  | Check Sum |

##### 3.3.1.14.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Parm # Keypad/Electronic | Definition |
| --- | --- | --- |
| _______ | ________________ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (13) |
| 5 | 75 / 80 | Left Front Machine Loss (MSB) ** Not Recommended to be written to |
| 6 |  | “ (LSB) Machine Maintained Calibration Values** |
| 7 | 76 / 81 | Right Front Machine Loss (MSB) ** Not Recommended to be written to |
| 8 |  | “ (LSB) Machine Maintained Calibration Values** |
| 9 | 77 / 82 | Left Rear Machine Loss (MSB) ** Not Recommended to be written to |
| 10 |  | “ (LSB) Machine Maintained Calibration Values** |
| 11 | 78 / 83 | Right Rear Machine Loss (MSB) ** Not Recommended to be written to |
| 12 |  | “ (LSB) Machine Maintained Calibration Values** |
| 13 | 79 / 84 | Left Front Roller Factor (MSB) ** Not Recommended to be written to |
| 14 |  | “ (LSB) Machine Maintained Calibration Values** |
| 15 | 80 / 85 | Right Front Roller Factor (MSB) ** Not Recommended to be written to |
| 16 |  | “ (LSB) Machine Maintained Calibration Values** |
| 17 | 81 / 86 | Left Rear Roller Factor (MSB) ** Not Recommended to be written to |
| 18 |  | “ (LSB) Machine Maintained Calibration Values** |
| 19 | 82 / 87 | Right Rear Roller Factor (MSB) ** Not Recommended to be written to |
| 20 |  | “ (LSB) Machine Maintained Calibration Values** |
| 21 | 83.0 / 88 | Maximum Left Machine Loss (MSB) |
| 22 |  | “ (LSB) |
| 23 | 83.1 / 89 | Minimum Left Machine Loss (MSB) |
| 24 |  | “ (LSB) |
| 25 | 83.2 / 90 | Maximum Right Machine Loss (MSB) |
| 26 |  | “ (LSB) |
| 27 | 83.3 / 91 | Minimum Right Machine Loss (MSB) |
| 28 |  | “ (LSB) |
| 29 | 84.0 / 92 | Calibration Arm Speed (MSB) |
| 30 |  | “ (LSB) |
| 31 | 84.1 / 93 | Calibration Sample Speed (MSB) |
| 32 |  | “ (LSB) |
| 33 | 84.2 / 94 | Calibration Minimum Speed (MSB) |
| 34 |  | “ (LSB) |
| 35 | 84.3 / 95 | Calibrate Warm-up Time (MSB) |

| Byte No. | Parm # Keypad/Electronic | Definition |
| --- | --- | --- |
| _______ | ________________ | ______________________________________________________________ |
| 36 |  | “ (LSB) |
| 37 | 84.4 / 96 | Calibrate Boost Pressure (MSB) |
| 38 |  | “ (LSB) |
| 39 | 84.5 / 97 | Julian Date of Last Calibration Performed YY/MM (MSB) |
| 40 |  | “ (LSB) |
| 41 | 84.6 / 98 | Julian Date of Last Calibration Performed DD/HH (Military) (MSB) |
| 42 |  | “ (LSB) |
| 43 | 85 / 99 | English/Metric Units (MSB) 0=English, 1=Metric |
| 44 |  | “ (LSB) |
| 45 | 86 / 100 | Reverse Direction Board Installed (MSB) 0=No, 1=Yes |
| 46 |  | “ (LSB) |
| 47 | 87 / 101 | I/O Option (MSB) 000=LVI, 001=OPTO22, 003=PLC5LTV, |
| 48 |  | “ (LSB) 004=PLC5V30B, 005=Serial AB, 006=Serial GE |
| 49 | 88 / 102 | Machine Serial Number (MSB) |
| 50 |  | “ (LSB) |
| 51 | 89.0 / 103 | Not Defined (MSB) |
| 52 |  | “ (LSB) |
| 53 | 89.1 / 104 | Not Defined (MSB) |
| 54 |  | “ (LSB) |
| 55 | 89.2 / 105 | Not Defined (MSB) |
| 56 |  | “ (LSB) |
| 57 | 89.3 / 106 | Not Defined (MSB) |
| 58 |  | “ (LSB) |
| 59 | 89.4 / 107 | Not Defined (MSB) |
| 60 |  | “ (LSB) |
| 61 | 89.5 / 108 | Not Defined (MSB) |
| 62 |  | “ (LSB) |
| 63 | 89.6 / 109 | Not Defined (MSB) |
| 64 |  | “ (LSB) |
| 65 | 89.7 / 110 | Not Defined (MSB) |
| 66 |  | “ (LSB) |
| 67 | 89.8 / 111 | Not Defined (MSB) |
| 68 |  | “ (LSB) |
| 69 | 89.9 / 112 | Not Defined (MSB) |
| 70 |  | “ (LSB) |
| 71 | 90.0 / 113 | Not Defined (MSB) |
| 72 |  | “ (LSB) |
| 73 | 90.1 / 114 | Not Defined (MSB) |
| 74 |  | “ (LSB) |
| 75 | 90.2 / 115 | Not Defined (MSB) |
| 76 |  | “ (LSB) |
| 77 | 90.3 / 116 | Not Defined (MSB) |
| 78 |  | “ (LSB) |
| 79 | 90.4 / 117 | Not Defined (MSB) |
| 80 |  | “ (LSB) |
| 81 | 90.5 / 118 | Not Defined (MSB) |
| 82 |  | “ (LSB) |
| 83 | 90.6 / 119 | Not Defined (MSB) |
| 84 |  | “ (LSB) |
| 85 | 90.7 / 120 | Not Defined (MSB) |
| 86 |  | “ (LSB) |
| 87 |  | Check Sum |

#### 3.3.1.15	Mode 1 Message 14: Machine Control Parameters

##### 3.3.1.15.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (14) |
| 5 |  | Check Sum |

##### 3.3.1.15.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Parm # Keypad/Electronic | Definition |
| --- | --- | --- |
| _______ | ________________ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (14) |
| 5 | 97 / 120 | Last Test Results (MSB) |
| 6 |  | “ (LSB) |
| 7 | 98 / 121 | Previous Test Results (MSB) |
| 8 |  | “ (LSB) |
| 9 | 99 / 122 | Printer Control (MSB) |
| 10 |  | “ (LSB) |
|  |  | 0 = Brake Test Printer - Off 1 = Brake Test Printer - Print All Tests |
|  |  | 2 = Brake Test Printer - Print Fail Only |
|  |  | 3 = Print Keyboard Parameters |
|  |  | 4 = Raw Data Dump |
| 11 |  | Check Sum |

#### 3.3.1.16	Mode 1 Message 15: 4WAL Vehicle DTC List

##### 3.3.1.16.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (15) |
| 5 |  | Check Sum |

##### 3.3.1.16.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (15) |
| 5 |  | 4WAL DTC Code #1 (MSB) |
| 6 |  | “ (LSB) |
| 7 |  | 4WAL DTC Code #2 (MSB) |
| 8 |  | “ (LSB) |
| 9 |  | 4WAL DTC Code #3 (MSB) |
| 10 |  | “ (LSB) |
| 11 |  | 4WAL DTC Code #4 (MSB) |
| 12 |  | “ (LSB) |
| 13 |  | 4WAL DTC Code #5 (MSB) |
| 14 |  | “ (LSB) |
| 15 |  | 4WAL DTC Code #6 (MSB) |
| 16 |  | “ (LSB) |
| 17 |  | 4WAL DTC Code #7 (MSB) |
| 18 |  | “ (LSB) |
| 19 |  | 4WAL DTC Code #8 (MSB) |
| 20 |  | “ (LSB) |
| 21 |  | 4WAL DTC Code #9 (MSB) |
| 22 |  | “ (LSB) |
| 23 |  | 4WAL DTC Code #10 (MSB) |
| 24 |  | “ (LSB) |
| 25 |  | Check Sum |

#### 3.3.1.17	Mode 1 Message 16: 4WAL Machine DTC List

##### 3.3.1.17.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (16) |
| 5 |  | Check Sum |

##### 3.3.1.17.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (16) |
| 5 |  | 4WAL Machine DTC Code #1 (MSB) |
| 6 |  | “ (LSB) |
| 7 |  | 4WAL Machine DTC Code #2 (MSB) |
| 8 |  | “ (LSB) |
| 9 |  | 4WAL Machine DTC Code #3 (MSB) |
| 10 |  | “ (LSB) |
| 11 |  | 4WAL Machine DTC Code #4 (MSB) |
| 12 |  | “ (LSB) |
| 13 |  | 4WAL Machine DTC Code #5 (MSB) |
| 14 |  | “ (LSB) |
| 15 |  | 4WAL Machine DTC Code #6 (MSB) |
| 16 |  | “ (LSB) |
| 17 |  | 4WAL Machine DTC Code #7 (MSB) |
| 18 |  | “ (LSB) |
| 19 |  | 4WAL Machine DTC Code #8 (MSB) |
| 20 |  | “ (LSB) |
| 21 |  | 4WAL Machine DTC Code #9 (MSB) |
| 22 |  | “ (LSB) |
| 23 |  | 4WAL Machine DTC Code #10 (MSB) |
| 24 |  | “ (LSB) |
| 25 |  | Check Sum |

#### 3.3.1.18	Mode 1 Message 17: Machine Error Codes List

##### 3.3.1.18.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (17) |
| 5 |  | Check Sum |

##### 3.3.1.18.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (17) |
| 5 |  | Machine Error Code #1 (MSB) |
| 6 |  | “ (LSB) |
| 7 |  | Machine Error Code #2 (MSB) |
| 8 |  | “ (LSB) |
| 9 |  | Machine Error Code #3 (MSB) |
| 10 |  | “ (LSB) |
| 11 |  | Machine Error Code #4 (MSB) |
| 12 |  | “ (LSB) |
| 13 |  | Machine Error Code #5 (MSB) |
| 14 |  | “ (LSB) |
| 15 |  | Machine Error Code #6 (MSB) |
| 16 |  | “ (LSB) |
| 17 |  | Machine Error Code #7 (MSB) |
| 18 |  | “ (LSB) |
| 19 |  | Machine Error Code #8 (MSB) |
| 20 |  | “ (LSB) |
| 21 |  | Machine Error Code #9 (MSB) |
| 22 |  | “ (LSB) |
| 23 |  | Machine Error Code #10 (MSB) |
| 24 |  | “ (LSB) |
| 25 |  | Check Sum |

#### 3.3.1.19	Mode 1 Message 20: Quad Steering (QS4) data

##### 3.3.1.19.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (20) |
| 5 |  | Check Sum |

##### 3.3.1.19.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (20) |
| 5 |  | Alignment Process Started Status (S1): 0=No, 1=Yes |
| 6 |  | Rear Thrust Angle Valid Status (S2): 0=No, 1=Yes |
| 7 |  | Steering Sweep Complete and Centered (S3): 0=No, 1=Yes |
| 8 |  | Alignment Process Failed (S4): 0=No, 1=Yes |
| 9 |  | Rear Thrust Angle * 100 (MSB) |
| 10 |  | “ (LSB) |
| 11 |  | Check Sum |

#### 3.3.1.20	Mode 1 Message 21: Short Speeds/Forces Message

##### 3.3.1.20.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (21) |
| 5 |  | Check Sum |

##### 3.3.1.20.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (21) |
| 5 |  | Prompt Number |
| 6 |  | Current Index (MSB) Current Brake graph logging index being recorded |
| 7 |  | Current Index (LSB) |
| 8 |  | Test Completion Status Byte #1 |
|  | 0 | 1 = Reset Complete/VME-PLC Test Ready (0 = Reset in Progress/VME not Ready) |
|  | 1 | 1 = Test – Arm Test Complete/Shift to Neutral |
|  | 2 | 1 = Rolling Resistance (Drag) Test Complete /Apply Brakes |
|  | 3 | 1 = Traction Control Test Complete /Apply Brakes |
|  | 4 | 1 = Base Brake Test Complete/ Ready to Start ABS |
|  | 5 | 1 = ABS Test Complete |
|  | 6 | Undefined |
|  | 7 | 1 = Test Complete – (Sent at 0 wheel speed or Mode 4, 09, byte 5 End ABS test or Fault) |
| 9 |  | Test Completion Status Byte #2 |
|  | 0 | 1 = Max. Brake Force Test Complete |
|  | 1 | 1 = Max. Parking Brake Force Test Complete |
|  | 2 | 1 = Tire Calibration Programming Complete |
|  | 3 | 1 = Axle Ratio Test Complete |
|  | 4 | 1 = Positraction Test Complete |
|  | 5 | 1 = Vibration Analysis Test Complete |
|  | 6 | 1 = RWAL/4WAL stand alone ABS Test Complete |
|  | 7 | Undefined |
| 10 |  | Test Stand Status Byte #1 |
|  | 0 | 1 = Machine Error Faults 0=NO DTCs 1= DTCs Set |
|  | 1 | 1 = ALDL Ready to Arm /Neutral Speed Achieved |
|  | 2 | 1 = 4WAL Vehicle DTC Set Flag, 0=No DTCs 1=DTCs Set |
|  | 3 | 1 = 4WAL WSS Test Status, 0=Fail, 1=Pass |
|  | 4 | 1 = 4WAL Machine DTC Set Flag, 0=No DTCs 1=DTCs Set |
|  | 5 | 1 = Maximum Machine Speed Exceeded |
|  | 6 | 1 = Zero Speed |
|  | 7 | Undefined |
| 11 |  | Test Stand Status Byte #2 |
|  | 0 | 1 = Front Retaining Rolls Up |
|  | 1 | 1 = Rear Retaining Rolls Up |
|  | 2 | 1 = Declutched Front to Rear (Hydraulics) |
|  | 3 | 1 = Declutched Front Side to Side |
|  | 4 | 1 = Declutched Rear Side to Side |
|  | 5 | 1 = Adjusting Wheel Base |
|  | 6 | 1 = Emergency Stop was Depressed |
|  | 7 | 1 = Roll Test Ready/Retaining Rolls Up Locked |
| 12 |  | Test Stand Status Byte #3 |
|  | 0 | 1 = Front Right Roll Brake Applied |
|  | 1 | 1 = Front Left Roll Brake Applied |
|  | 2 | 1 = Rear Right Roll Brake Applied |
|  | 3 | 1 = Rear Left Roll Brake Applied |
|  | 4 | 1 = Pump Motor Running |
|  | 5 | 1 = Retaining Rolls High Position (Dual Height Option Only) |
|  | 6 | 1 = Front ESB Motor Driving |
|  | 7 | 1 = Rear ESB Motor Driving |

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 13 |  | Test Stand Status Byte #4 |
|  | 0 | 1 = Buzzer On |
|  | 1 | 1 = PLC Auxiliary 1 Feedback On |
|  | 2 | 1 = PLC Auxiliary 2 Feedback On |
|  | 3 | Undefined |
|  | 4 | 1 = RWD drive *machine drives rear wheels |
|  | 5 | 1 = FWD drive *machine drives front wheels |
|  | 6 | 1 = 4WD drive *machine speed matches side to side |
|  | 7 | 1 = AWD drive * machine speed matches side to side |
| 14 |  | Test Stand Status Byte #5 |
|  | 0 | 1 = Driver Force Front Minimum Fault |
|  | 1 | 1 = Driver Force Front Maximum Fault |
|  | 2 | 1 = Driver Force Rear Minimum Fault |
|  | 3 | 1 = Driver Force Rear Maximum Fault |
|  | 4 | Undefined |
|  | 5 | 1 = Driver Interface Button Pressed |
|  | 6 | Undefined |
|  | 7 | Undefined |
| 15 |  | Test Stand Status Byte #6 |
|  | 0 | Wheel Base Position #1 |
|  | 1 | Wheel Base Position #2 |
|  | 2 | Wheel Base Position #3 |
|  | 3 | Wheel Base Position #4 |
|  | 4 | Wheel Base Position #5 |
|  | 5 | Wheel Base Position #6 |
|  | 6 | Wheel Base Position #7 |
|  | 7 | Wheel Base Position #8 |
|  | 0&3 | Wheel Base Position #9 |
|  | 1&3 | Wheel Base Position #10 |
|  | 0&1&3 | Wheel Base Position #11 |
|  | 2&3 | Wheel Base Position #12 |
|  | 0&2&3 | Wheel Base Position #13 |
|  | 1&2&3 | Wheel Base Position #14 |
| 16 |  | LF wheel speed (Signed for roll direction) MPH in tenths XXX.X |
| 17 |  | “ (LSB) Reverse goes negative. |
| 18 |  | RF wheel speed (Signed for roll direction) MPH in tenths XXX.X |
| 19 |  | “ (LSB) |
| 20 |  | LR wheel speed (Signed for roll direction) MPH in tenths XXX.X |
| 21 |  | “ (LSB) |
| 22 |  | RR wheel speed (Signed for roll direction) MPH in tenths XXX.X |
| 23 |  | “ (LSB) |
| 24 |  | LF Force in lbs. (Signed for Brake force / Acceleration) Acceleration goes negative. |
| 25 |  | “ (LSB) |
| 26 |  | RF Force in lbs. (Signed for Brake force / Acceleration) |
| 27 |  | “ (LSB) |
| 28 |  | LR Force in lbs. (Signed for Brake force / Acceleration) |
| 29 |  | “ (LSB) |
| 30 |  | RR Force in lbs. (Signed for Brake force / Acceleration) |
| 31 |  | “ (LSB) |
| 32 |  | Check Sum |

#### 3.3.1.21	Mode 1 Message 22: Brake/Speed Data Large message

Used to retrieve 160 bytes of data from the Test Stand Memory.

##### 3.3.1.21.1	Message Sent From Test Equipment to Test Stand

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (22) |
| 5 |  | Index Pointer (MSB) |
| 6 |  | Index Pointer (LSB) |
| 7 |  | N/A (MSB) |
| 8 |  | N/A (LSB) |
| 9 |  | Check Sum |

##### 3.3.1.21.2	Message Sent From Test Stand to Test Equipment

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (01) |
| 4 |  | Message Number (22) |
| 5 |  | Memory Array Index (MSB) }Starting point of data block |
| 6 |  | Memory Array Index (LSB) } |
| 7 |  | Memory Byte 3(MSB) }LF roll speed in MPH }(MPH = X / 100) |
| 8 |  | Memory Byte 3(LSB) } } |
| 9 |  | Memory Byte 4(MSB) }RF roll speed in MPH } |
| 10 |  | Memory Byte 4(LSB) } } |
| 11 |  | Memory Byte 5(MSB) }LR roll speed in MPH } |
| 12 |  | Memory Byte 5(LSB) } } |
| 13 |  | Memory Byte 6(MSB) }RR roll speed in MPH } |
| 14 |  | Memory Byte 6(LSB) } } |
| 15 |  | Memory Byte 7(MSB) }LF brake force in lbs |
| 16 |  | Memory Byte 7(LSB) } |
| 17 |  | Memory Byte 8(MSB) }RF brake force in lbs |
| 18 |  | Memory Byte 8(LSB) } |
| 19 |  | Memory Byte 9(MSB) }LR brake force in lbs |
| 20 |  | Memory Byte 9(LSB) } |
| 21 |  | Memory Byte 10(MSB) }RR brake force in lbs |
| 22 |  | Memory Byte 10(LSB) } |
| N…N |  | Memory Byte N…N+10 (MSB) |
| N…N |  | Memory Byte N N+10 (LSB) |
| 167 |  | Check Sum |

### 3.3.2	Mode 2 Definition

Mode 2 is used to retrieve 64 bytes of data from the Test Stand Memory.

#### 3.3.2.1	Data Sent from the Test Equipment to the Test Stand.

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (02) |
| 4 |  | Marker Type (MSB) See Appendix A |
| 5 |  | Marker Type (LSB) |
| 6 |  | Marker Name (MSB) See Appendix B |
| 7 |  | Marker Name (LSB) |
| 8 |  | Marker Index (MSB) |
| 9 |  | Marker Index (LSB) |
| 10 |  | Check Sum |

#### 3.3.2.2	Data Sent from the Test Stand to the Test Equipment

Returns Data.

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N+85) |
| 3 |  | Mode Number (02) |
| 4 |  | Memory Byte 1(MSB) }Total number of sample points |
| 5 |  | Memory Byte 1(LSB) } |
| 6 |  | Memory Byte 2(MSB) } Marker index (ms = X * 20) |
| 7 |  | Memory Byte 2(LSB) } |
| 8 |  | Memory Byte 3(MSB) }LF roll speed in MPH }(MPH = X / 100) |
| 9 |  | Memory Byte 3(LSB) } } |
| 10 |  | Memory Byte 4(MSB) }RF roll speed in MPH } |
| 11 |  | Memory Byte 4(LSB) } } |
| 12 |  | Memory Byte 5(MSB) }LR roll speed in MPH } |
| 13 |  | Memory Byte 5(LSB) } } |
| 14 |  | Memory Byte 6(MSB) }RR roll speed in MPH } |
| 15 |  | Memory Byte 6(LSB) } } |
| 16 |  | Memory Byte 7(MSB) }LF brake force in lbs |
| 17 |  | Memory Byte 7(LSB) } |
| 18 |  | Memory Byte 8(MSB) }RF brake force in lbs |
| 19 |  | Memory Byte 8(LSB) } |
| 20 |  | Memory Byte 9(MSB) }LR brake force in lbs |
| 21 |  | Memory Byte 9(LSB) } |
| 22 |  | Memory Byte 10(MSB) }RR brake force in lbs |
| 23 |  | Memory Byte 10(LSB) } |
| N…N |  | Memory Byte N…N(MSB) |
| N…N |  | Memory Byte N N(LSB) |
| 64 |  | Check Sum |

### 3.3.3	Mode 4 Definition

Mode 4 is used to command the General Motors Corp. Test Stand to perform certain tests or functions.

#### 3.3.3.1	Data Sent from the Test Equipment to the Test Stand.

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (04) |
| 4 |  | Test Stand Command Byte |
|  |  | HEX / DEC |
|  |  | 00 / 00 = Reset |
|  |  | 01 / 01 = Traction Control Vehicle |
|  |  | 02 / 02 = ABS Vehicle (Non KH RWAL/4WAL Vehicles) |
|  |  | 03 / 03 = Arm Brake Test |
|  |  | 04 / 04 = Execute Rolling Resistance (Drag) Test |
|  |  | 05/ 05 = Traction Test Control |
|  |  | 06 / 06 = Execute Base Brake Test |
|  |  | 07 / 07 = ABS Test Control |
|  |  | 08 / 08 = Execute Quick Stop (3600) |
|  |  | 09 / 09 = Execute Axle Ratio Test (Resending command clears previous test results) |
|  |  | 0A / 10 = Execute Positraction Test |
|  |  | 0B / 11 = Execute Vibration Analysis Test |
|  |  | 0C / 12 = 4 Wheel Drive Vehicle |
|  |  | 0D / 13 = All Wheel Drive Vehicle |
|  |  | 0E / 14 = Execute Max. Brake Force Test (3600) |
|  |  | 0F / 15 = Execute Max. Parking Brake Force Test (3600) |
|  |  | 10 / 16 = Brake Retest-Retain Options, Retain Communications type, Reclutch and Enable Brake Tests, Retain Parameter Values |
|  |  | 11 / 17 = 4WAL Reset – Brake Retest |
|  |  | 12 / 18 = Soft Reset to PLC |
|  |  | 13 / 19 = Declutch Test Stand |
|  |  | 14 / 20 = Reclutch |
|  |  | 15 / 21 = Lower Rolls |
|  |  | 16 / 22 = Machine Drive |
|  |  | 17 / 23= Drive Motor Control |
|  |  | 18 / 24 = Apply Roll Brakes |
|  |  | 19 / 25 = Select Wheel Base |
|  |  | 1A / 26 = Sound Buzzer |
|  |  | 1B / 27 = Status Light Control (90% Complete Light) |
|  |  | 1C / 28 = Auxiliary 1 Output Command To PLC |
|  |  | 1D / 29 = Auxiliary 2 Output Command To PLC |
|  |  | 1E / 30 = Boost to Arm (For Retest and Governed Vehicles for 3600 ONLY) Vehicle must have all wheels greater than 30 mph when commanded.!! |
|  |  | 1F / 31 = Reverse Test |
|  |  | 20 / 32 = Drive Type control |
|  |  | 21 / 33 = Zero wheel distance counters |
|  |  | 22 / 34 = Retaining Rolls Position |
|  |  | 23 / 35 = Re-relax Retaining Rolls for Auto Tire Sizing based on Keyboard Parameters 7 and 8. |
|  |  | 64 / 100 = QS4 Controls |
|  |  | FF / 255 = Return to Old Style Communications |

| Byte No. | Bit No. | Definition |  |  |
| --- | --- | --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |  |  |
| 5 | If Byte 4 Equals: |  |  |
| 05/05: |  |  |  |  |
|  |  | 0 = End Traction Test |  |  |
|  |  | 1 = Start 2 Channel Traction Test |  |  |
|  |  | 2 = Start 4 Channel Traction Test |  |  |
|  |  | 3 = Start 3 Channel Traction Test |  |  |
| 06/06: |  |  |  |  |
|  |  | 0 = End Base Brake Test |  |  |
|  |  | 1 = Start Base Brake Test |  |  |
| 07/07: |  |  |  |  |
|  |  | 0 = End ABS Test |  |  |
|  |  | 1 = Start Class II RWAL Test |  |  |
|  |  | 2 = Start Class II AWD ABS Test |  |  |
|  |  | 3 = Start Class II 3 and 4 Channel ABS Test |  |  |
|  |  | 4 = Start Class II 3 Channel ABS Test with Reduction/Recovery Windows |  |  |
|  |  | 5 = Start Class II 4 Channel ABS Test with Reduction/Recovery Windows |  |  |
|  |  | 6 = Start KH RWAL Test (Stand Alone Test) |  |  |
|  |  | 7 = Start KH 4WAL Test (Stand Alone Test) |  |  |
| 13/19: |  |  |  |  |
|  |  | 0 = Declutch All Clutches |  |  |
|  | Bit 0 | 1 = Declutch Front to Rear (Hydraulics) |  |  |
|  | Bit 1 | 1 = Declutch Front Side to Side |  |  |
|  | Bit 2 | 1 = Declutch Rear Side to Side |  |  |
| 14/20: |  |  |  |  |
|  |  | 0 = Reclutch All Clutches |  |  |
|  | Bit 0 | 1 = Reclutch Front to Rear |  |  |
|  | Bit 1 | 1 = Reclutch Front Side to Side |  |  |
|  | Bit 2 | 1 = Reclutch Rear Side to Side |  |  |
| 16/22: |  |  |  |  |
|  | Bit 0 | 1 = Fluid Drive Front to Rear (Front Drive Vehicle) |  |  |
|  | Bit 1 | 1 = Fluid Drive Rear to Front (Rear Drive Vehicle) NOTE: If Bit 0 and Bit 1 are zeros, then vehicle is 4 wheel drive. |  |  |
| 17/23: |  |  |  |  |
|  | Bit 0 | 1 = Front Drive Motor On |  |  |
|  | Bit 1 | 1 = Rear Drive Motor On |  |  |
| 18/24: |  |  |  |  |
|  |  | 0 = Apply All Roll Brakes |  |  |
|  | Bit 0 | 1 = Apply Front Right Roll Brake |  |  |
|  | Bit 1 | 1 = Apply Front Left Roll Brake |  |  |
|  | Bit 2 | 1 = Apply Rear Right Roll Brake |  |  |
|  | Bit 3 | 1 = Apply Rear Left Roll Brake |  |  |
| 19/25: |  |  |  |  |
|  | Bit 0 | 1 = Wheel Base Position #1 |  |  |
|  | Bit 1 | 1 = Wheel Base Position #2 |  |  |
|  | Bit 2 | 1 = Wheel Base Position #3 |  |  |
|  | Bit 3 | 1 = Wheel Base Position #4 |  |  |
|  | Bit 4 | 1 = Wheel Base Position #5 |  |  |
|  | Bit 5 | 1 = Wheel Base Position #6 |  |  |
|  | Bit 6 | 1 = Wheel Base Position #7 |  |  |
|  | Bit 7 | 1 = Wheel Base Position #8 |  |  |
|  | Bit0 & Bit 3 | 1 = Wheel Base Position #9 |  |  |
|  | Bit1 & Bit 3 | 1 = Wheel Base Position #10 |  |  |
|  | Bit0 & Bit1 & Bit 3 | 1 = Wheel Base Position #11 |  |  |
|  | Bit2 & Bit 3 | 1 = Wheel Base Position #12 |  |  |

| Byte No. | Bit No. | Definition |  |  |
| --- | --- | --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |  |  |
| 5 | If Byte 4 Equals: |  |  |
| 19/25: |  |  |  |  |
|  | Bit0 & Bit 2 & Bit 3 | 1 = Wheel Base Position #13 |  |  |
|  | Bit1 & Bit 2 & Bit 3 | 1 = Wheel Base Position #14 |  |  |
| 1B/27: |  |  |  |  |
|  |  | 1 = Light On |  |  |
|  |  | 0 = Light Off |  |  |
| 1C/28: |  |  |  |  |
|  |  | 1 = Auxiliary 1 On |  |  |
|  |  | 0 = Auxiliary 1 Off |  |  |
| 1D/29: |  |  |  |  |
|  |  | 1 = Auxiliary 2 On |  |  |
|  |  | 0 = Auxiliary 2 Off |  |  |
| 1F/31: |  |  |  |  |
|  |  | 0 = End Reverse Test |  |  |
|  |  | 1 = Start Reverse Test |  |  |
| 20/32 |  |  |  |  |
|  | Bit 0 | 1= Speed match side to side or Clutch Left to Right. |  |  |
|  | Bit 1 | 1= Speed match front and rear or Clutch front to rear (Pump On). |  |  |
|  | Bit 2- Bit 7 | Unused |  |  |
| 21 / 33 |  |  |  |  |
|  |  | 0= Reset wheel distances to zero. |  |  |
| 22 / 34 |  |  |  |  |
|  |  | 0 = Retaining Rolls Position Low |  |  |
|  |  | 1 = Retaining Rolls Position High |  |  |
| 64 / 100 |  |  |  |  |
|  | Bit 0 | 1 = QS4 Abort |  |  |
|  | Bit 1 | 1 = QS4 Rear Centered Complete |  |  |
|  | Bit 2 | 1 = QS4 Hand Wheel Centered Complete |  |  |
|  | Bit 3 | 1 = QS4 Complete |  |  |
| 6 |  | Check Sum |  |  |

#### 3.3.3.2	Data Sent from the Test Stand to the Test Equipment

Used For Communication Verification.

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N+85) |
| 3 |  | Mode Number (04) |
| 4 |  | Command Byte (Mode 04 command) |
| 5 |  | 06H/06D = ACK (Positive Response) 0FH/15D = NACK (Negative Response) |
| 6 |  | Check Sum |

#### 3.3.3.3	Mode 4 Command Failsafes

Please Add the Following Failsafes:

1.	Make sure all wheels are at 0 MPH before acting on the following Mode 4 commands.

a.	Lower Retaining Rolls

b.	Reset Machine

c.	Reclutch

d.	Select Wheel Base

e.	Brake Retest

2.	Make sure wheels are less than 10 MPH before applying roll brake when using the Mode 4 command.

### 3.3.4	Mode 6 Definition

Mode 6 is used to download new Parameters to the General Motors Corp. Test Stand

#### 3.3.4.1	Data Sent from the Test Equipment to the Test Stand.

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (85 +4) |
| 3 |  | Mode Number (06) |
| 4 |  | Parameter Number |
| 5 |  | Parameter Value (MSB) |
| 6 |  | Parameter Value (LSB) |
| 7 |  | Check Sum |

#### 3.3.4.2	Data Sent from the Test Stand to the Test Equipment.

Used for Communication Verification

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (06) |
| 4 |  | Check Sum |

### 3.3.5	Mode 7 Definition

Mode 7 is used to set markers in the brake data in the General Motors Corp. Test Stand

#### 3.3.5.1	Data Sent from the Test Equipment to the Test Stand.

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (85 +4) |
| 3 |  | Mode Number (07) |
| 4 |  | Marker Type (MSB) See Appendix A |
| 5 |  | Marker Type (LSB) |
| 6 |  | Marker name (MSB) See Appendix B |
| 7 |  | Marker (LSB) } |
| 8 |  | Check Sum |

#### 3.3.5.2	Data Sent from the Test Stand to the Test Equipment.

Used for Communication Verification

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (07) |
| 4 |  | Check Sum |

### 3.3.6	Mode 20 Definition

Mode 20 is used to retrieve brake data for rolling resistance, base brake, traction, ABS, max brake and park brake from the General Motors Corp. Test Stand

#### 3.2.6.1	Data Sent from the Test Equipment to the Test Stand.

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (85 +4) |
| 3 |  | Mode Number (20) |
| 4 |  | Test Result ID Number (MSB) {01 = Rolling Resistance, 02 = Base brake |
| 5 |  | Test Result ID Number (LSB) {03 = Traction, 04 = ABS, 05= Max brake, 06= Park brake |
| 6 |  | Check Sum |

#### 3.3.6.2	Data Sent from the Test Stand to the Test Equipment.

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (20) |
| 4 |  | Test Result ID Number (MSB) {01 = Rolling Resistance, 02 = Base brake |
| 5 |  | Test Result ID Number (LSB) {03 = Traction, 04 = ABS, 05= Max brake, 06= Park brake |
| 6 |  | LF Minimum/Reduction in lbs (MSB) |
| 7 |  | LF Minimum/Reduction in lbs (LSB) |
| 8 |  | LF Maximum/Recovery in lbs (MSB) |
| 9 |  | LF Maximum/Recovery in lbs (LSB) |
| 10 |  | LF Average in lbs (MSB) |
| 11 |  | LF Average in lbs (LSB) |
| 12 |  | LF Minimum/Reduction index number (MSB) |
| 13 |  | LF Minimum/Reduction index number (LSB) |
| 14 |  | LF Maximum/Recovery index number (MSB) |
| 15 |  | LF Maximum/Recovery index number (LSB) |
| 16 |  | RF Minimum/Reduction in lbs (MSB) |
| 17 |  | RF Minimum/Reduction in lbs (LSB) |
| 18 |  | RF Maximum/Recovery in lbs (MSB) |
| 19 |  | RF Maximum/Recovery in lbs (LSB) |
| 20 |  | RF Average in lbs (MSB) |
| 21 |  | RF Average in lbs (LSB) |
| 22 |  | RF Minimum/Reduction index number (MSB) |
| 23 |  | RF Minimum/Reduction index number (LSB) |
| 24 |  | RF Maximum/Recovery index number (MSB) |
| 25 |  | RF Maximum/Recovery index number (LSB) |
| 26 |  | LR Minimum/Reduction in lbs (MSB) |
| 27 |  | LR Minimum/Reduction in lbs (LSB) |
| 28 |  | LR Maximum/Recovery in lbs (MSB) |
| 29 |  | LR Maximum/Recovery in lbs (LSB) |
| 30 |  | LR Average in lbs (MSB) |
| 31 |  | LR Average in lbs (LSB) |
| 32 |  | LR Minimum/Reduction index number (MSB) |
| 33 |  | LR Minimum/Reduction index number (LSB) |
| 34 |  | LR Maximum/Recovery index number (MSB) |
| 35 |  | LR Maximum/Recovery index number (LSB) |
| 36 |  | RR Minimum/Reduction in lbs (MSB) |
| 37 |  | RR Minimum/Reduction in lbs (LSB) |
| 38 |  | RR Maximum/Recovery in lbs (MSB) |
| 39 |  | RR Maximum/Recovery in lbs (LSB) |
| 40 |  | RR Average in lbs (MSB) |
| 41 |  | RR Average in lbs (LSB) |
| 42 |  | RR Minimum/Reduction index number (MSB) |
| 43 |  | RR Minimum/Reduction index number (LSB) |
| 44 |  | RR Maximum/Recovery index number (MSB) |
| 45 |  | RR Maximum/Recovery index number (LSB) |
| 46 |  | Secondary LF Minimum/Reduction in lbs (MSB) |
| 47 |  | Secondary LF Minimum/Reduction in lbs (LSB) |
| 48 |  | Secondary LF Minimum/Reduction index number (MSB) |
| 49 |  | Secondary LF Minimum/Reduction index number (LSB) |
| 50 |  | Secondary RF Minimum/Reduction in lbs (MSB) |
| 51 |  | Secondary RF Minimum/Reduction in lbs (LSB) |

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 52 |  | Secondary RF Minimum/Reduction index number (MSB) |
| 53 |  | Secondary RF Minimum/Reduction index number (LSB) |
| 54 |  | Secondary LR Minimum/Reduction in lbs (MSB) |
| 55 |  | Secondary LR Minimum/Reduction in lbs (LSB) |
| 56 |  | Secondary LR Minimum/Reduction index number (MSB) |
| 57 |  | Secondary LR Minimum/Reduction index number (LSB) |
| 58 |  | Secondary RR Minimum/Reduction in lbs (MSB) |
| 59 |  | Secondary RR Minimum/Reduction in lbs (LSB) |
| 60 |  | Secondary RR Minimum/Reduction index number (MSB) |
| 61 |  | Secondary RR Minimum/Reduction index number (LSB) |
| 62 |  | Check Sum |

NOTES:

(These are guidelines and production values may be different.)

Rear Forces should be 1/3 of Front Forces

Front Balance (%) = (RF/LF+RF)*100	{>50% right side is higher

Rear Balance (%) = (RR/LR+RR)* 100	{<50% left side is higher

50/50 is ideal side to side balance

Front to Rear Balance (%) = (LR+RR/LF+RF+LR+RR)* 100	{>50% rear is higher

{<50%front is higher

### 3.3.7	Mode 42 (0x2A) Definition

Mode 42 is used to set scheduled mode 1 messages.

The Test Stand has 20 message slots. Unused slots should be set to 255 (0xFF)

The Test Stand will sequence through the 20 slots stepping every 50 ms.

A one-shot requested mode 1 messages will result in a scheduled message being suspended for a maximum of the request message transmit time plus 10 ms.

You may change scheduled messages at anytime or turn off by setting to 255’s.

Note: Mode 4 Command 0xFF return to Old mode will also turn off scheduled messages.

#### 3.3.7.1	Data Sent from the Test Equipment to the Test Stand.

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N+85) |
| 3 |  | Mode Number (42) |
| 4 |  | Slot 0 Valid mode 1 message # or 255 (0xFF) no message |
| 5 |  | Slot 1 .. |
| 6 |  | Slot 2 |
| 7 |  | Slot 3 |
| 8 |  | Slot 4 |
| 9 |  | Slot 5 |
| 10 |  | Slot 6 |
| 11 |  | Slot 7 |
| 12 |  | Slot 8 |
| 13 |  | Slot 9 |
| 14 |  | Slot 10 |
| 15 |  | Slot 11 |
| 16 |  | Slot 12 |
| 17 |  | Slot 13 |
| 18 |  | Slot 14 |
| 19 |  | Slot 15 |
| 20 |  | Slot 16 |
| 21 |  | Slot 17 |
| 23 |  | Slot 18 |
| 24 |  | Slot 19 |
| 25 |  | Check Sum |

#### 3.3.7.2	Data Sent from the Test Stand to the Test Equipment.

Used for Communication Verification

| Byte No. | Bit No. | Definition |
| --- | --- | --- |
| _______ | ______ | ______________________________________________________________ |
| 1 |  | Message ID |
| 2 |  | Message Length (N + 85) |
| 3 |  | Mode Number (42 |
| 4 |  | Check Sum |

# 4.	Appendices

## 4.1	Appendix A

Available marker types

### 4.1.1	Marker Types Table.

| ###### Type | HEX/DEC |
| --- | --- |
| LF | 0/0 |
| RF | 1/1 |
| LR | 2/2 |
| RR | 3/3 |
| LT | 4/4}T= TANDEM (USED FOR 3 AXLE MACHINE) |
| RT | 5/5} T= TANDEM (USED FOR 3 AXLE MACHINE) |
| All Rolls | 1E/30 (Marks all channels) |
| TEST SEQ | 3C/60 (Marks all channels) |
| ALDL COMM ALL | 5A/90 (Used only after arm and until min speed is reached) |
| ALDL COMM LF | 5A/91 (Used only after arm and until min speed is reached) |
| ALDL COMM RF | 5A/92 (Used only after arm and until min speed is reached) |
| ALDL COMM LR | 5A/93 (Used only after arm and until min speed is reached) |
| ALDL COMM RR | 5A/94 (Used only after arm and until min speed is reached) |

## 4.2	Appendix B

Available Marker names

### 4.2.1	Marker Names Table

| ###### Name | HEX/DEC |
| --- | --- |
| * | 0/0 |
| Arm Speed | 1/1 (Marks all channels) |
| Rolling Resistance Start Test | 2/2 (Marks all channels) |
| Rolling Resistance End Test | 3/3 (Marks all channels) |
| Base Brake Start Test | 4/4 (Marks all channels) |
| Base Brake End Test | 5/5 (Marks all channels) |
| Traction Control Start Test | 6/6 (Marks all channels) |
| Traction Control End Test | 7/7 (Marks all channels) |
| ABS Start Test | 8/8 (Marks all channels) |
| ABS End Test | 9/9 (Marks all channels) |
| ABS Fired | A/10 (Marks all channels) |
| ABS Wheel Speed | B/11 (Marks all channels) |
| Max Brake Start Test | C/12 (Marks all channels) |
| Max Brake End Test | D/13 (Marks all channels) |
| Max Sample Start Test | E/14 (Marks all channels) |
| Max Sample End Test | F/15 (Marks all channels) |
| Max Point | 10/16 (Marks all channels) |
| Cal Sample Start Test | 11/17 (Marks all channels) |
| Cal Sample End Test | 12/18 (Marks all channels) |
| Min Achieved | 13/19 (Marks all channels) |
| Reference Sample Start | 14/20 (Marks all channels) |
| Reference Sample End | 15/21 (Marks all channels) |
| Reference | 16/22 (Marks all channels) |
| Reduction | 17/23 (Marks all channels) |
| Recovery | 18/24 (Marks all channels) |
| 2nd Reduction | 19/25 (Marks all channels) |
| 2nd Reduction | 1A/26 (Marks all channels) |
| Min Speed | 1B/27 (Marks all channels) |
| Last Sample | 1C/28 (Marks all channels) |
| Axle ratio Start Test | 1E/30 (Marks all channels) |
| Axle ratio End of Test | 1F/31 (Marks all channels) |
| Generic Brake Start Test | 20/32 (Marks all channels) |
| Generic Brake End Test | 21/33 (Marks all channels) |
| Traction Build | 22/34 Type (Wheel) |
| Traction Release | 23/35 Type (Wheel) |
| Zero Wheel Travel | 24/36 (Marks all channels) |
| Read Wheel Travel | 25/37 (Marks all channels) |
| Drive Trip | 50/80 (Marks all channels) |
| LF Minimum/Reduction | 64/100 |
| LF Maximum/Recovery | 65/101 |
| LF 2nd Minimum/2nd Reduction | 66/102 |
| LF 2nd Maximum/2nd Recovery | 67/103 |
| RF Minimum/Reduction | 68/104 |
| RF Maximum/Recovery | 69/105 |
| RF 2nd Minimum/2nd Reduction | 6A/106 |
| RF 2nd Maximum/2nd Recovery | 6B/107 |
| LR Minimum/Reduction | 6C/108 |
| LR Maximum/Recovery | 6D/109 |
| LR 2nd Minimum/2nd Reduction | 6E/110 |
| LR 2nd Maximum/2nd Recovery | 6F/111 |
| RR Minimum/Reduction | 70/112 |
| RR Maximum/Recovery | 71/113 |
| RR 2nd Minimum/2nd Reduction | 72/114 |
| LF 2nd Maximum/2nd Recovery | 73/115 |
| LT Minimum/Reduction | 74/116}T= TANDEM |
| LT Maximum/Recovery | 75/117}T= TANDEM |
| LT 2nd Minimum/2nd Reduction | 76/118}T= TANDEM |
| LT 2nd Maximum/2nd Recovery | 77/119}T= TANDEM |
| RT Minimum/Reduction | 78/120} (USED FOR 3 AXLE MACHINE) |
| RT Maximum/Recovery | 79/121} (USED FOR 3 AXLE MACHINE) |
| RT 2nd Minimum/2nd Reduction | 7A/122}T= TANDEM |
| RT 2nd Maximum/2nd Recovery | 7B/123}T= TANDEM |

## 4.3	Appendix C

Machine Error Codes

### 4.3.1	Machine Error Codes

#### 4.3.1.1 	Burke E. Porter Machine Error Codes

| Message Number | Description |
| --- | --- |
| 0x0000 | CLUTCH |
| 0x0001 | DECLUTCH |
| 0x0002 | ZEROSPEED |
| 0x0003 | OVERSPEED |
| 0x0004 | BOOST |
| 0x0005 | VERIFACATION |
| 0x0006 | OPERATOR STAMP |
| 0x0007 | ABS TEST SELECTED LAMP |
| 0x0008 | TEST (ARM) LAMP |
| 0x0009 | NEUTRAL LAMP |
| 0x000A | BRAKE LAMP |
| 0x000B | ACCEPT LAMP |
| 0x000C | FAULT LAMP |
| 0x000D | DRIVERS FAULT SIGNAL |
| 0x000E | START 4WD TEST SIGNAL |
| 0x000F | RETEST SIGNAL |
| 0x0010 | LF DRAG REJECT LAMP |
| 0x0011 | RF DRAG REJECT LAMP |
| 0x0012 | LR DRAG REJECT LAMP |
| 0x0013 | RR DRAG REJECT LAMP |
| 0x0014 | LF BRAKE REJECT LAMP |
| 0x0015 | RF BRAKE REJECT LAMP |
| 0x0016 | LR BRAKE REJECT LAMP |
| 0x0017 | RR BRAKE REJECT LAMP |
| 0x0018 | FRONT BALANCE REJECT LAMP |
| 0x0019 | REAR BALANCE REJECT LAMP |
| 0x001A | FRONT TO REAR BAL REJ LAMP |
| 0x001B | ANTI-LOCK REJECT LAMP |
| 0x001C | POSITRACTION REJECT LAMP |
| 0x001D | AXLE RATIO REJECT LAMP |
| 0x001E | PARKING BRAKE REJECT LAMP |
| 0x001F | --- OPEN --- |
| 0x0020 | WHEELBASE POSITION 1 |
| 0x0021 | WHEELBASE POSITION 2 |
| 0x0022 | WHEELBASE POSITION 3 |
| 0x0023 | WHEELBASE POSITION 4 |
| 0x0024 | WHEELBASE POSITION 5 |
| 0x0025 | RWD SELECTED |
| 0x0026 | 4WD SELECTED |
| 0x0027 | PARKING BRAKE TEST |
| 0x0028 | WHEELBASE POSITION 7 |
| 0x0029 | WHEELBASE POSITION 8 |
| 0x002A | WHEELBASE POSITION 9 |
| 0x002B | WHEELBASE POSITION 10 |
| 0x002C | WHEELBASE POSITION 11 |
| 0x002D | WHEELBASE POSITION 12 |
| 0x002E | FWD SELECTED |
| 0x002F | WHEELBASE POSITION 6 |
| 0x0030 | START LAMP |
| 0x0031 | ROLL TEST LAMP |
| 0x0032 | BRAKE TEST LAMP |
| 0x0033 | COMPLETE LAMP |
| 0x0034 | ACCEL LAMP |
| 0x0035 | 40 KPH LAMP |
| 0x0036 | TRACTION CONTROL LAMP |
| 0x0037 | ECU LAMP |
| 0x0038 | HORN SIGNAL |
| 0x0039 | START EBS SENSOR TEST |
| 0x003A | CALIBRATE SWITCH ON |
| 0x003B | SIMULATE SWITCH ON |
| 0x003C | START REVERSE TEST |
| 0x003D | ABS CABLE CONNECTED |
| 0x003E | START JOUNCE (1 SEC PULSE) |
| 0x003F | F.I.S. DATA VALID |
| 0x0040 | FRONT WHEEL DRIVE SELECTED |
| 0x0041 | ALL WHEEL DRIVE SELECTED |
| 0x0042 | ABS TEST SELECTED |
| 0x0043 | MANUAL TRANSMISSION |
| 0x0044 | AUTOMATIC TRANSMISSION |
| 0x0045 | EATX |
| 0x0046 | CRUISE CONTROL SELECTED |
| 0x0047 | TILT TEST SELECTED |
| 0x0048 | TEST 90% COMPLETE |
| 0x0049 | DAY TIME RUNNING LIGHTS SELECTED |
| 0x004A | EBS DRIVE SPEED (ON=LOW,OFF=HIGH) |
| 0x004B | APPLY ROLL BRAKES |
| 0x004C | DECLUTCH FRT AND RE HYD CLUTCHES |
| 0x004D | DECLUTCH FRONT ROLL CLUTCH |
| 0x004E | DECLUTCH REAR ROLL CLUTCH |
| 0x004F | VME SYSTEM READY |
| 0x0050 | UPPER -12V POWER SUPPLY TOLERANCE |
| 0x0051 | UPPER +12V POWER SUPPLY TOLERANCE |
| 0x0052 | UPPER +5V POWER SUPPLY TOLERANCE |
| 0x0053 | LOWER +5V POWER SUPPLY TOLERANCE |
| 0x0054 | LOWER +12V POWER SUPPLY TOLERANCE |
| 0x0055 | LOWER -12V POWER SUPPLY TOLERANCE |
| 0x0056 | +24V POWER SUPPLY TOLERANCE |
| 0x0057 | POWER SENSE LOST TO I/O RACK |
| 0x0058 | BOOST PRESSURE FAILURE |
| 0x0059 | UNIT CONTROLLER COMMUN. FAULT) |
| 0x005A | SSS TESTER COMMUNICATION FAULT |
| 0x005B | SCANNER COMMUNICATION FAULT |
| 0x005C | KEYPAD COMMUNICATION FAULT |
| 0x005D | PLC DID NOT INITIALIZE |
| 0x005E | WATT TRANSDUCER FAULT |
| 0x005F | TASK MONITOR FAULT (VME FAULT) |
| 0x0060 | FRONT ROLLS NOT EQL WHEN CLUTCHED |
| 0x0061 | REAR ROLLS NOT EQL WHEN CLUTCHED |
| 0x0062 | LF MACH LOSS CHANGE OVER 5 LBS |
| 0x0063 | RF MACH LOSS CHANGE OVER 5 LBS |
| 0x0064 | LR MACH LOSS CHANGE OVER 5 LBS |
| 0x0065 | RR MACH LOSS CHANGE OVER 5 LBS |
| 0x0066 | LF MACH LOSS OUT OF LIMITS |
| 0x0067 | RF MACH LOSS OUT OF LIMITS |
| 0x0068 | LR MACH LOSS OUT OF LIMITS |
| 0x0069 | RR MACH LOSS OUT OF LIMITS |
| 0x006A | LF ENCODER RUNOUT ERROR |
| 0x006B | RF ENCODER RUNOUT ERROR |
| 0x006C | LR ENCODER RUNOUT ERROR |
| 0x006D | RR ENCODER RUNOUT ERROR |
| 0x006E | CYCLE LIMIT EXCEEDED - CALIBRATE |
| 0x006F | CYCLE LIMIT WARNING - CALIBRATE |
| 0x0070 | START REAR EBS DRIVES |
| 0x0071 | KEYBOARD TRIP SWITCH CONTROL |
| 0x0072 | 4WD FRONT ROLLS HYD ASSIST |
| 0x0073 | START PARKING PAWL TEST |
| 0x0074 | FATAL FAULT (FLASHER) |
| 0x0075 | SEN CROSS & TRAC-LOCK |
| 0x0076 | RELEASE LR BRAKE |
| 0x0077 | RELEASE LF BRAKE |
| 0x0078 | ENGAGE ROLL CLUTCHES WHEN SEN CROSS |
| 0x0079 | INTER-ROLL PRES. OFF (FOR CRUISE) |
| 0x007A | FLYWHEEL CLUTCH (NON-BRAKE TESTING) |
| 0x007B | FLYWHEEL DECLUTCH (BRAKE TESTING) |
| 0x007C | TRACTION CONTROL TEST ON |
| 0x007D | START DCL TEST |
| 0x007E | RAISE ROLLS |
| 0x007F | VALID BARCODE SCAN |
| 0x0080 | CCD TRANSLATOR NOT RESPONDING |
| 0x0081 | CCD TRANS. NOT ACCEPTING COMMANDS |
| 0x0082 | BAD MESSAGES FROM CCD TRANSLATOR |
| 0x0083 | PLC SERIAL READ FAULT |
| 0x0084 | PLC SERIAL WRITE FAULT |
| 0x0085 | Open PLC 8.5 |
| 0x0086 | Open PLC 8.6 |
| 0x0087 | Open PLC 8.7 |
| 0x0088 | LF ROLL MOTOR GENERAL FAULT |
| 0x0089 | RF ROLL MOTOR GENERAL FAULT |
| 0x008A | LR ROLL MOTOR GENERAL FAULT |
| 0x008B | RR ROLL MOTOR GENERAL FAULT |
| 0x008C | LF ROLL MOTOR DRIVE COMM FAULT |
| 0x008D | RF ROLL MOTOR DRIVE COMM FAULT |
| 0x008E | LR ROLL MOTOR DRIVE COMM FAULT |
| 0x008F | RR ROLL MOTOR DRIVE COMM FAULT |
| 0x0090 | PROGRAM WHEELBASE POSITION |
| 0x0091 | ENABLE ALL ROLL DRIVES |
| 0x0092 | DCL CONTROL |
| 0x0093 | PLC-VME COMM TASK HEARTBEAT |
| 0x0094 | WATT TRANSDUCER CONTROL |
| 0x0095 | AIR BAG TEST FAILED |
| 0x0096 | AIR BAG TEST IN PROGRESS |
| 0x0097 | AIR BAG TEST PASSED |
| 0x0098 | SPEED PRO TEST (RAISE F EL) |
| 0x0099 | CHECK WB POSITION AGAIN |
| 0x009A | DEPLOY VIBRATION SENSOR |
| 0x009B | Open PLC 9.11 |
| 0x009C | Open PLC 9.12 |
| 0x009D | Open PLC 9.13 |
| 0x009E | Open PLC 9.14 |
| 0x009F | Open PLC 9.15 |
| 0x00A0 - 0x00AF | LF WHEEL SPEED |
| 0x00B0 - 0x00BF | LR WHEEL SPEED |
| 0x00C0 - 0x00CF | VERIFICATION PRESSURE |
| 0x00D0 - 0x00DF | BOOST PRESSURE |
| 0x00E0 - 0x00EF | MINIMUM BOOST PRESSURE |
| 0x00F0 - 0x00FF | JOUNCE CYCLES |
| 0x0100 - 0x010F | JOUNCE FREQUENCY (Hz * 100) |
| 0x0110 - 0x011F | RF WHEEL SPEED |
| 0x0120 - 0x012F | RR WHEEL SPEED |
| 0x0130 | UNABLE TO COMMUNICATE TO WCC |
| 0x0131 | WCC INTERNAL FAULT |
| 0x0132 | WCC TO BROADCAST COMMUN. FAULT |
| 0x0133 | WCC FATAL FAULT |
| 0x0134 | Open PLC 19.4 |
| 0x0135 | Open PLC 19.5 |
| 0x0136 | Open PLC 19.6 |
| 0x0137 | Open PLC 19.7 |
| 0x0138 | Open PLC 19.8 |
| 0x0139 | Open PLC 19.9 |
| 0x013A | Open PLC 19.10 |
| 0x013B | Open PLC 19.11 |
| 0x013C | Open PLC 19.12 |
| 0x013D | Open PLC 19.13 |
| 0x013E | Open PLC 19.14 |
| 0x013F | Open PLC 19.15 |
| 0x0140 | INTER-ROLL PRESSURE HIGH WARNING |
| 0x0141 | INTER-ROLL PRESSURE HIGH SHUTDOWN |
| 0x0142 | Open PLC 20.2 |
| 0x0143 | Open PLC 20.3 |
| 0x0144 | Open PLC 20.4 |
| 0x0145 | Open PLC 20.5 |
| 0x0146 | Open PLC 20.6 |
| 0x0147 | Open PLC 20.7 |
| 0x0148 | UNABLE TO LOAD PARAMETER LIST |
| 0x0149 | Open PLC 20.9 |
| 0x014A | Open PLC 20.10 |
| 0x014B | Open PLC 20.11 |
| 0x014C | Open PLC 20.12 |
| 0x014D | Open PLC 20.13 |
| 0x014E | Open PLC 20.14 |
| 0x014F | Open PLC 20.15 |
| 0x0150 - 0x015F | WB TRANSDUCER VALUE |
| 0x8000 | REAR WHEEL DRIVE SIG (OFF=FWD) |
| 0x8001 | POWER SENSE |
| 0x8002 | START TEST |
| 0x8003 | ABS TEST SELECTED |
| 0x8004 | ROLLS DOWN |
| 0x8005 | ALL WHEEL DRIVE SIGNAL |
| 0x8006 | TRACTION CONTROL SELECTED |
| 0x8007 | CRUISE CONTROL SELECTED |
| 0x8008 | 4 WHEEL DRIVE SIGNAL |
| 0x8009 | POSITRACTION TEST SIGNAL |
| 0x800A | MANUAL TRANSMISSION |
| 0x800B | AUTO TRANSMISSION |
| 0x800C | EATX TRANSMISSION |
| 0x800D | TILT STEERING WHEEL TEST |
| 0x800E | DRIVER PASS SIGNAL |
| 0x800F | DRIVER FAIL SIGNAL |
| 0x8010 | PARKING BRAKE TEST SELECT |
| 0x8011 | PARKING BRAKE TEST PASSED |
| 0x8012 | PARKING BRAKE TEST FAILED |
| 0x8013 | CLEAR PUSHBUTTON |
| 0x8014 | START SEQUENCE PUSHBUTTON |
| 0x8015 | NEXT V.I.N. REQUEST |
| 0x8016 | SPECIFIC V.I.N. REQUEST |
| 0x8017 | COMPUTER MODE SELECTED |
| 0x8018 | JOUNCE TEST COMPLETE |
| 0x8019 | OP. PENDANT ON DRIVERS CONSOLE |
| 0x801A | VEHICLE PRESENT |
| 0x801B | LUCAS ABS SELECTED |
| 0x801C | DIFFERENTIAL LOCK TEST PB |
| 0x801D | BOSCH ABS SELECTED |
| 0x801E | WHEELBASE IN POSITION |
| 0x801F | CHECK +24 VOLT POWER SUPPLY |
| 0x8020 | FRONT RET. ROLLS RAISED FAULT |
| 0x8021 | REAR RET. ROLLS RAISED FAULT |
| 0x8022 | LEFT STOP POST RAISED FAULT |
| 0x8023 | RIGHT STOP POST RAISED FAULT |
| 0x8024 | FRONT RET. ROLLS LOWERED FAULT |
| 0x8025 | REAR RET. ROLLS LOWERED FAULT |
| 0x8026 | LEFT STOP POST LOWERED FAULT |
| 0x8027 | RIGHT STOP POST LOWERED FAULT |
| 0x8028 | LUBE SYSTEM CYCLE SWITCH FAULT |
| 0x8029 | HYDRAULIC OIL LEVEL FAULT |
| 0x802A | HYDRAULIC OIL TEMPERATURE FAULT |
| 0x802B | 60GPM HYD. OUTPUT FILTER BLOCKED |
| 0x802C | 12GPM HYD. OUTPUT FILTER BLOCKED |
| 0x802D | HYD. INTAKE FILTER BLOCKED |
| 0x802E | WHEELBASE MOTOR DISCONNECT OPEN |
| 0x802F | HYDRAULIC MOTOR DISCONNECT OPEN |
| 0x8030 | WHEELBASE MOTOR DRIVE FAULT |
| 0x8031 | HYDRAULIC MOTOR OVERLOAD OPEN |
| 0x8032 | WHEELBASE MOTOR OVERLOAD OPEN |
| 0x8033 | SOLENOID CIRCUIT BREAKER OPEN-PNL |
| 0x8034 | LUBE SYSTEM LEVEL LOW FAULT |
| 0x8035 | LUBE SYSTEM PRESSURE HIGH FAULT |
| 0x8036 | SYSTEM AIR PRESSURE LOW FAULT |
| 0x8037 | SYSTEM FAIL ASSERTED BY PLC-5/VME |
| 0x8038 | PLC-5 BATTERY LOW FAULT |
| 0x8039 | PLC-5 RACK FAULT |
| 0x803A | INTER. RET. ROLLS RAISED FAULT |
| 0x803B | INTER. RET. ROLLS LOWERED FAULT |
| 0x803C | WHEELBASE FILE FAULT |
| 0x803D | 15 VDC POWER SUPPLY FAULT |
| 0x803E | WHEELBASE POSITIONING FAULT |
| 0x803F | SOLENOID FUSE OPEN - PANEL |
| 0x8040 | WHEELBASE POSITION 1 SELECTED |
| 0x8041 | WHEELBASE POSITION 2 SELECTED |
| 0x8042 | WHEELBASE POSITION 3 SELECTED |
| 0x8043 | WHEELBASE POSITION 4 SELECTED |
| 0x8044 | WHEELBASE POSITION 5 SELECTED |
| 0x8045 | WHEELBASE POSITION 6 SELECTED |
| 0x8046 | WHEELBASE POSITION 7 SELECTED |
| 0x8047 | --- OPEN --- (WB 8 SEL) |
| 0x8048 | --- OPEN --- (WB 9 SEL) |
| 0x8049 | --- OPEN --- (WB 10 SEL) |
| 0x804A | --- OPEN --- (WB 11 SEL) |
| 0x804B | --- OPEN --- (WB 12 SEL) |
| 0x804C | --- OPEN --- (WB 13 SEL) |
| 0x804D | --- OPEN --- (WB 14 SEL) |
| 0x804E | --- OPEN --- (WB 15 SEL) |
| 0x804F | --- OPEN --- (WB 16 SEL) |
| 0x8050 | FRONT EBS DRIVE OVERLOAD OPEN |
| 0x8051 | REAR EBS DRIVE OVERLOAD OPEN |
| 0x8052 | FRONT EBS DRIVE NOT RAISED FAULT |
| 0x8053 | REAR EBS DRIVE NOT RAISED FAULT |
| 0x8054 | FRONT EBS DRIVE NOT LOWERED FAULT |
| 0x8055 | REAR EBS DRIVE NOT LOWERED FAULT |
| 0x8056 | FRONT EBS DRIVE DISCONNECT OPEN |
| 0x8057 | REAR EBS DRIVE DISCONNECT OPEN |
| 0x8058 | LR PARK BRAKE CYL. NOT RETRACTED |
| 0x8059 | RR PARK BRAKE CYL. NOT RETRACTED |
| 0x805A | AIR DOOR NOT RAISED FAULT |
| 0x805B | AIR DOOR NOT LOWERED FAULT |
| 0x805C | PANEL TEMPERATURE HIGH WARNING |
| 0x805D | PRIMARY PHASE OR OPEN BREAKER |
| 0x805E | EXHAUST DOOR RAISED FAULT |
| 0x805F | EXHAUST DOOR LOWERED FAULT |
| 0x8060 | FRONT ELEVATOR LOWERED FAULT |
| 0x8061 | FRONT ELEVATOR RAISED FAULT |
| 0x8062 | REAR ELEVATOR LOWERED FAULT |
| 0x8063 | REAR ELEVATOR RAISED FAULT |
| 0x8064 | REAR STOP POST RAISED FAULT |
| 0x8065 | REAR STOP POST LOWERED FAULT |
| 0x8066 | LFT JOUNCE STAB. ENGAGED FAULT |
| 0x8067 | LFT JOUNCE STAB. DISENGAGED FAULT |
| 0x8068 | RHT JOUNCE STAB. ENGAGED FAULT |
| 0x8069 | RHT JOUNCE STAB. DISENGAGED FAULT |
| 0x806A | JOUNCE MOTOR DISCONNECT OPEN |
| 0x806B | JOUNCE MOTOR DRIVE FAULT |
| 0x806C | JOUNCE SYSTEM COMPLETION FAULT |
| 0x806D | PARKING PAWL CYL. FAIL TO EXTEND |
| 0x806E | PARKING PAWL CYL. FAIL TO RETRACT |
| 0x806F | MONITOR BOOM NOT RETRACTED FAULT |
| 0x8070 | PARKING PAWL TEST PASSED |
| 0x8071 | PARKING PAWL TEST FAILED |
| 0x8072 | ABORT SIGNAL |
| 0x8073 | YES PUSH BUTTON |
| 0x8074 | NO PUSH BUTTON |
| 0x8075 | SIDE GUIDE CONTACT |
| 0x8076 | ROLLS UP |
| 0x8077 | FRONT EBS DRIVE OVERCURRENT |
| 0x8078 | REAR EBS DRIVE OVERCURRENT |
| 0x8079 | HYDRAULIC SYSTEM IDLE TIMEOUT |
| 0x807A | KEYLIST SELECT LSB |
| 0x807B | KEYLIST SELECT |
| 0x807C | KEYLIST SELECT MSB |
| 0x807D | SPD DISP SCAL (1=200 0=160 KPH) |
| 0x807E | DTRL SELECTED |
| 0x807F | STANDARD ENGINE SEL (OFF=CAL) |
| 0x8080 | ROLL DRIVE MOTOR DISCONNECT OPEN |
| 0x8081 | ROLL DRIVE MOTOR OVERLOAD OPEN |
| 0x8082 | HYD. OIL CIRC. PUMP DISC. OPEN |
| 0x8083 | HYD. OIL CIRC. PUMP OVERLOAD OPEN |
| 0x8084 | HYD. OIL HEAT EX. FAN DISC. OPEN |
| 0x8085 | HYD OIL HEAT EX FAN OVERLOAD OPEN |
| 0x8086 | CONTROL POWER CIRCUIT BREAKER OPEN |
| 0x8087 | PRIMARY VAC CIRCUIT BREAKER OPEN |
| 0x8088 | F. RET. ROLL LOCK ENGAGED FAULT |
| 0x8089 | F. RET. ROLL LOCK DISENGAGED FLT |
| 0x808A | LEFT STONE GUARD EXTENDED FAULT |
| 0x808B | LEFT STONE GUARD RETRACTED FAULT |
| 0x808C | RIGHT STONE GUARD EXTENDED FAULT |
| 0x808D | RIGHT STONE GUARD RETRACTED FAULT |
| 0x808E | FRONT HYD. CLUTCH NOT ENGAGED FLT |
| 0x808F | REAR HYD. CLUTCH NOT ENGAGED FLT |
| 0x8090 | SOL. CIRCUIT BREAKER OPENJ-BOX |
| 0x8091 | SOL. FUSE OPENJ-BOX |
| 0x8092 | FRONT EBS MOTOR DRIVE FAULT |
| 0x8093 | REAR EBS MOTOR DRIVE FAULT |
| 0x8094 | HOLDOWN PLATFORM RAISED FAULT |
| 0x8095 | HOLDOWN PLATFORM LOWERED FAULT |
| 0x8096 | HLDWN PLAT. LOCK ENGAGED FAULT |
| 0x8097 | HLDWN PLAT. LOCK DISENGAGED FAULT |
| 0x8098 | PIT VENTILATION INTERLOCK FAULT |
| 0x8099 | HYD. MOTOR OVERCURRENT |
| 0x809A | HYD. CIRC. MOTOR OVERCURRENT |
| 0x809B | HYD. OIL COOLING FAN OVERCURRENT |
| 0x809C | WHEELBASE MOTOR OVERCURRENT |
| 0x809D | STOP POST SAFETY PINS NOT REMOVED |
| 0x809E | FRONT HYD. CLUTCH ENGAGED FAULT |
| 0x809F | REAR HYD. CLUTCH ENGAGED FAULT |
| 0x80A0 | ALL FLYWHEELS AT ZEROSPEED |
| 0x80A1 | RIGHT HAND DRIVE SEL (OFF=LEFT) |
| 0x80A2 | FRONT FLYWHEELS ENGAGED |
| 0x80A3 | REAR FLYWHEELS ENGAGED |
| 0x80A4 | KELSEY HAYES RWAL TEST SELECTED |
| 0x80A5 | KELSEY HAYES 4WAL TEST SELECTED |
| 0x80A6 | VEHICLE AT BARCODE SCANNER |
| 0x80A7 | NO ABS SELECTED |
| 0x80A8 | TEVES ABS SELECTED |
| 0x80A9 | WHEELBASE JOG MODE |
| 0x80AA | START DYNO TEST |
| 0x80AB | PRINT TEST RESULTS |
| 0x80AC | RUN FLAT TEST SELECTED |
| 0x80AD | START 40 KPH TEST |
| 0x80AE | START 50 MPH TEST |
| 0x80AF | Open PLC 11.15 |
| 0x80B0 - 0x80BF | VEHICLE TYPE |
| 0x80C0 | FRONT EBS MOTOR OVERCURRENT |
| 0x80C1 | REAR EBS MOTOR OVERCURRENT |
| 0x80C2 | LUBE LEVEL LOW SHUTDOWN FAULT |
| 0x80C3 | LUBE PRESSURE HIGH SHUTDOWN FAULT |
| 0x80C4 | MOTOR BLOWER FUSE OPEN J-BOX |
| 0x80C5 | 24 VDC POSER SUPPLY FAULT |
| 0x80C6 | LR PARK BRAKE CYL FAIL TO EXTEND |
| 0x80C7 | RR PARK BRAKE CYL FAIL TO EXTEND |
| 0x80C8 | DC HOLDING BRAKE DISABLED |
| 0x80C9 | MONITOR BOOM NOT EXTENDED FAULT |
| 0x80CA | F INTER-ROLL CLUTCH ENGAGE FAULT |
| 0x80CB | R INTER-ROLL CLUTCH ENGAGE FAULT |
| 0x80CC | F INTER-ROLL CL. DISENGAGE FAULT |
| 0x80CD | R INTER-ROLL CL. DISENGAGE FAULT |
| 0x80CE | BRAKE TEST PUSHBUTTON |
| 0x80CF | PLC HEARTBEAT FAULT |
| 0x80D0 | LF ROLL MOTOR DISCONNECT OPEN |
| 0x80D1 | RF ROLL MOTOR DISCONNECT OPEN |
| 0x80D2 | LR ROLL MOTOR DISCONNECT OPEN |
| 0x80D3 | RR ROLL MOTOR DISCONNECT OPEN |
| 0x80D4 | LF ROLL MOTOR OVERLOAD OPEN |
| 0x80D5 | RF ROLL MOTOR OVERLOAD OPEN |
| 0x80D6 | LR ROLL MOTOR OVERLOAD OPEN |
| 0x80D7 | RR ROLL MOTOR OVERLOAD OPEN |
| 0x80D8 | LF ROLL MOTOR DRIVE FAULT |
| 0x80D9 | RF ROLL MOTOR DRIVE FAULT |
| 0x80DA | LR ROLL MOTOR DRIVE FAULT |
| 0x80DB | RR ROLL MOTOR DRIVE FAULT |
| 0x80DC | WHEELBASE ENCODER FAULT |
| 0x80DD | MOTOR TASK HEARTBEAT FAULT |
| 0x80DE | OPERATOR CRT DRIVE FAULT |
| 0x80DF | OPERATOR CRT POSITIONING FAULT |
| 0x80E0 | PLC-5 RACK 0 FAULT |
| 0x80E1 | PLC-5 RACK 1 FAULT |
| 0x80E2 | PLC-5 RACK 2 FAULT |
| 0x80E3 | PLC-5 RACK 3 FAULT |
| 0x80E4 | PLC-5 RACK 4 FAULT |
| 0x80E5 | PLC-5 RACK 5 FAULT |
| 0x80E6 | PLC-5 RACK 6 FAULT |
| 0x80E7 | PLC-5 RACK 7 FAULT |
| 0x80E8 | FIRE ALARM DETECTED FAULT |
| 0x80E9 | EXHAUST MOTOR OVER CURRENT |
| 0x80EA | EXIT DOOR FAULT |
| 0x80EB | ENTRANCE DOOR FAULT |
| 0x80EC | LF ROLL MOTOR OVER CURRENT |
| 0x80ED | RF ROLL MOTOR OVER CURRENT |
| 0x80EE | LR ROLL MOTOR OVER CURRENT |
| 0x80EF | RR ROLL MOTOR OVER CURRENT |
| 0x8112 | PLC Ready Fault |
| 0x8140 | PANEL E-STOP PRESSED |
| 0x8141 | RF MACHINE E-STOP PRESSED |
| 0x8142 | LR MACHINE E-STOP PRESSED |
| 0x8143 | LH PENDANT E-STOP PRESSED |
| 0x8144 | RH PENDANT E-STOP PRESSED |
| 0x8145 | AIR DOOR STOP PIN P1 |
| 0x8146 | AIR DOOR STOP PIN P2 |
| 0x8147 | LF STOP POST PIN |
| 0x8148 | RF STOP POST PIN |
| 0x8149 | EXHAUST STOP PIN P1 |
| 0x814a | EXHAUST STOP PIN P2 |
| 0x814B | ROLL MOTOR POWER SUPPLY FAULT |
| 0x814C | ROLL MOTOR POWER SUPPLY ALARM |
| 0x8150 | LF ROLL MOTOR CONTACTOR FAULT |
| 0x8151 | RF ROLL MOTOR CONTACTOR FAULT |
| 0x8152 | LR ROLL MOTOR CONTACTOR FAULT |
| 0x8153 | RR ROLL MOTOR CONTACTOR FAULT |
| 0x8154 | VENT SUPPLY FLOW FAULT |
| 0x8155 | VENT EXHAUST FLOW FAULT |
| 0x8156 | VENT AIR SUPPLY MOTOR DISC OPEN |
| 0x8157 | VENT AIR EXHAUST MOTOR DISC OPEN |
| 0x8158 | Open PLC 21.8 |
| 0x8159 | Open PLC 21.9 |
| 0x815A | Open PLC 21.10 |
| 0x815B | Open PLC 21.11 |
| 0x815C | Open PLC 21.12 |
| 0x815D | Open PLC 21.13 |
| 0x815E | Open PLC 21.14 |
| 0x815F | Open PLC 21.15 |

#### 4.3.1.2		Durr Machine Error Codes

| Fault Number | Error Code Description |
| --- | --- |
| 0x01A4 | MCP LOCAL I/O SLOT 3 FAULTED INPUT CARD |
| 0x01A6 | MCP DEVICENET SCANNER POWER E210CB NOT ON DEVICENET SCANNER 5 HAS A POWER ERROR |
| 0x01A7 | MCP PLC SCAN TIME OVER 250MS CHECK SAFETY TASK TIME AND OVERHEAD TIMESLICE |
| 0x01A8 | MCP POWER DISTRIBUTION PANEL DISC OFF |
| 0x01A9 | MCP COMMUNICATION PWR 1 E203PWS NOT ON MCP I10300 ALSO CHECK E203CB |
| 0x01AA | MCP ETHERNET POWER 1 E214CB NOT ON MCP I10301 |
| 0x01AB | MCP CONTROL POWER 1 E302CB NOT ON MCP I10302 ALSO CHECK E300CB |
| 0x01AC | MCP CONTROL POWER 1 E307CB NOT ON MCP I10303 |
| 0x01AD | MCP CONTROL POWER 1 E322CB NOT ON MCP I10304 |
| 0x01AE | MCP CONTROL POWER 2 E402CB NOT ON MCP I10310 ALSO CHECK E400CB |
| 0x01AF | MCP CONTROL POWER 2 E407CB NOT ON MCP I10311 |
| 0x01B5 | MCP DEVICENET POWER 1 E355CB NOT ON MCP I10305 ALSO CHECK E353CB |
| 0x01B6 | MCP DEVICENET 5 POWER E360CB NOT ON MCP I10307 |
| 0x01B7 | MCP DEVICENET 6 POWER E364CB NOT ON MCP I10308 |
| 0x01B8 | MCP DEVICENET 7 POWER E368CB NOT ON MCP I10309 |
| 0x01B9 | MCP DEVICENET REFRESH 1 E372CB NOT ON MCP I10306 |
| 0x01CC | MCP DNET SCANNER SLOT 05 NOT AT NODE 0 |
| 0x01CD | MCP DNET SCANNER SLOT 05 FAULTED |
| 0x01CE | MCP DNET SCANNER SLOT 05 IDLE |
| 0x01CF | MCP DNET SCANNER SLOT 06 NOT AT NODE 0 |
| 0x01D0 | MCP DNET SCANNER SLOT 06 FAULTED |
| 0x01D1 | MCP DNET SCANNER SLOT 06 IDLE |
| 0x01D2 | MCP DNET SCANNER SLOT 07 NOT AT NODE 0 |
| 0x01D3 | MCP DNET SCANNER SLOT 07 FAULTED |
| 0x01D4 | MCP DNET SCANNER SLOT 07 IDLE |
| 0x03F8 | BA010CELL HMI1 MODULE 1 DNET COMM FAULT DNET SLOT 5 NODE 1 - SAFETY AND STANDARD COMM FAULT |
| 0x03F9 | BA010CELL HMI1 MODULE 1 DNET COMM FAULT DNET SLOT 5 NODE 1 - SAFETYCOMMUNICATION FAULT |
| 0x03FA | BA010CELL HMI1 MODULE 1 DNET COMM FAULT DNET SLOT 5 NODE 1 - STANDARD COMMUNICATION FAULT |
| 0x0594 | BA010CELL CELL IS NOT RESET |
| 0x0595 | BA010CELL AN E STOP PB PRESSED IN CELL CHECK OTHER HMIS FOR DETAILS |
| 0x0780 | BA010CELL HMI1 INPUT/OUTPUT POWER ERROR CHECK E104CB AND POWER SUPPLY |
| 0x0781 | BA010CELL HMI1 SAFETY I/O INPUT FAULTED AT LEAST ONE INPUT POINT FAILED - SEE HELP SCREEN |
| 0x0782 | BA010CELL HMI1 SAFETY I/O MODULE TEST OUT FLT ONE TEST OUTPUT SHORTED - SEE HELP SCREEN |
| 0x0783 | BA010CELL HMI1 ESTOP INPUT B OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0784 | BA010CELL HMI1 ESTOP INPUT A OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0785 | BA010CELL HMI1 ESTOP INPUTS FAULTED I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0786 | BA010CELL HMI1 ESTOP PB PRESSED |
| 0x0787 | BA010CELL HMI1 RESET PB TIED DOWN I10002 NOT OFF |
| 0x0788 | BA010CELL HMI1 AUTO INIT PB TIED DOWN I10006 NOT OFF |
| 0x0789 | BA010CELL HMI1 ENABLE PB TIED DOWN I10003 NOT OFF |
| 0x078A | BA010CELL HMI1 MODE SELECTOR FAULTED I10004 & I10005 BOTH ON - CHECK FOR FORCES |
| 0x078B | BA010CELL HMI1 MODE SELECTOR FAULTED I10004 & I10005 BOTH OFF - CHECK SELECTOR SWITCH |
| 0x078C | BA010CELL HMI1 NOT IN AUTO WITH AUTO SELECTED |
| 0x078D | BA010CELL HMI1 NEXT CYCLE HOLD PB PRESSED |
| 0x078E | BA010CELL HMI1 MANUAL MODE SELECTED TURN SWITCH TO AUTO - ANOTHER HMI ALREADY IN MANUAL |
| 0x0793 | BA010CELL HMI1 DO - UNDO BUTTONS TIED DOWN RELEASE DO AND UNDO BUTTONS I10007 & I10008 |
| 0x07BC | BA010CELL CELL NO E-STOPS SAFETY CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - AN E-STOP IS MISSING |
| 0x07BD | BA010CELL CELL GATES RESET SAFETY CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - A GUARD IS MISSING |
| 0x07BE | BA010CELL TOOL MINORMTN SAFETY CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - SOMETHING IS MISSING |
| 0x07BF | BA010CELL TOOL MAJORMTN SAFETY CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - SOMETHING IS MISSING |
| 0x07C0 | BA010CELL NOBOT MOTION SAFETY CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - SOMETHING IS MISSING |
| 0x07C2 | BA010CELL ROBOT SERVOS SAFETY CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - SOMETHING IS MISSING |
| 0x07C3 | BA010CELL ONE RBTENABLE SAFETY CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - SOMETHINIG IS MISSING |
| 0x07C4 | BA010CELL TOOL MPS OK SAFETY CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - MAJOR MOTION IS MISSING |
| 0x07C5 | BA010CELL ROBOT MPS OK SAFETY CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - A ROBOT IS MISSING |
| 0x07C6 | BA010CELL CELL GUARDS OK SAFETY CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - A GUARD IS MISSING |
| 0x07C7 | BA010CELL SAFETY TASK NOT LOCKED SAFETY PROGRAM IS NOT PROTECTED |
| 0x07C8 | BA010CELL SAFETY TASK MISSING SIGNATURE GENERATE SAFETY SIGNATURE BEFORE LOCKING TASK |
| 0x07C9 | BA010CELL SAFETY TASK SIGNATURE MISMATCH SAFETY PROGRAM HAS BEEN MODIFIED |
| 0x07CA | BA010CELL CELL RESET CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - SOMETHING IS MISSING |
| 0x07CB | BA010CELL CELL NO SAFETY FAULTS CONFIG ERROR CHECK CELL SAFETY CONFIG ROUTINE - SOMETHING IS MISSING |
| 0x07D1 | BA010 I/O BLOCK BK01 INPUT GROUP SHORT CHECK TOOLING CORDS FOR SHORTS |
| 0x07D2 | BA010 I/O BLOCK BK01 DNET COMM FAULT DNET SLOT 05 NODE 30 |
| 0x07D3 | BA010 I/O BLOCK BK02 INPUT GROUP SHORT CHECK TOOLING CORDS FOR SHORTS |
| 0x07D4 | BA010 I/O BLOCK BK02 DNET COMM FAULT DNET SLOT 06 NODE 31 |
| 0x07D5 | BA010 I/O BLOCK BK03 INPUT GROUP SHORT CHECK TOOLING CORDS FOR SHORTS |
| 0x07D6 | BA010 I/O BLOCK BK03 OUTPUT FAULTED DNET SLOT 05 NODE 32 |
| 0x07D7 | BA010 I/O BLOCK BK03 DNET COMM FAULT DNET SLOT 05 NODE 32 |
| 0x07D8 | BA010 I/O BLOCK BK04 INPUT GROUP SHORT CHECK TOOLING CORDS FOR SHORTS |
| 0x07D9 | BA010 I/O BLOCK BK04 DNET COMM FAULT DNET SLOT 05 NODE 33 |
| 0x07DA | BA010 I/O BLOCK BK05 INPUT GROUP SHORT CHECK TOOLING CORDS FOR SHORTS |
| 0x07DB | BA010 I/O BLOCK BK05 DNET COMM FAULT DNET SLOT 05 NODE 34 |
| 0x07DC | BA010 I/O BLOCK BK06 INPUT GROUP SHORT CHECK TOOLING CORDS FOR SHORTS |
| 0x07DD | BA010 I/O BLOCK BK06 OUTPUT FAULTED DNET SLOT 06 NODE 30 |
| 0x07DE | BA010 I/O BLOCK BK06 DNET COMM FAULT DNET SLOT 06 NODE 30 |
| 0x07DF | BA010 I/O BLOCK BK07 INPUT GROUP SHORT CHECK TOOLING CORDS FOR SHORTS |
| 0x07E0 | BA010 I/O BLOCK BK07 DNET COMM FAULT DNET SLOT 05 NODE 35 |
| 0x07E5 | BA010 VALVE MANIFOLD PM1 AUX PWR NOT ON CHECK AUX PWR CONNECTION |
| 0x07E6 | BA010 VALVE MANIFOLD PM1 AUX PWR NOT OFF CHECK AUX PWR CONNECTION |
| 0x07E7 | BA010 VALVE MANIFOLD PM1 DNET COMM FAULT DNET SLOT 05 NODE 40 FAULTED |
| 0x07E8 | BA010 VALVE MANIFOLD PM2 AUX PWR NOT ON CHECK AUX PWR CONNECTION |
| 0x07E9 | BA010 VALVE MANIFOLD PM2 AUX PWR NOT OFF CHECK AUX PWR CONNECTION |
| 0x07EA | BA010 VALVE MANIFOLD PM2 DNET COMM FAULT DNET SLOT 05 NODE 41 FAULTED |
| 0x07EF | BA010 STACK LIGHT L1 DNET COMM FAULT DNET SLOT 06 NODE 21 |
| 0x07F0 | BA010 STACK LIGHT L2 DNET COMM FAULT DNET SLOT 05 NODE 22 |
| 0x07F9 | BA010 LVP F1 DNET COMM FAULT DNET SLOT 6 NODE 2 - SAFETY AND STANDARD COMM FAULT |
| 0x07FA | BA010 LVP F1 DNET COMM FAULT DNET SLOT 6 NODE 2 - SAFETY COMMUNICATION FAULT |
| 0x07FB | BA010 LVP F1 DNET COMM FAULT DNET SLOT 6 NODE 2 - STANDARD COMMUNICATION FAULT |
| 0x07FC | BA010 LVP F1 DNET COMM FAULT DNET SLOT 6 NODE 3 - SAFETY AND STANDARD COMM FAULT |
| 0x07FD | BA010 LVP F1 DNET COMM FAULT DNET SLOT 6 NODE 3 - SAFETY COMMUNICATION FAULT |
| 0x07FE | BA010 LVP F1 DNET COMM FAULT DNET SLOT 6 NODE 3 - STANDARD COMMUNICATION FAULT |
| 0x07FF | BA010 LVP F1 DNET COMM FAULTED DNET 6 NODES 2,3,32 - CHECK DNET TRUNK/DROP LINES AND PWR |
| 0x0800 | BA010 LVP F1 SIEMENS I/O DNET COMM FAULT DNET SLOT 6 NODE 32 - STANDARD COMMUNICATION FAULT |
| 0x0801 | BA010 LVP F1 SIEMENS I/O MODULE FAULT CHECK ET200S BUSCOUPLER MODULE |
| 0x0803 | BA010 DRIVE CONTACTOR PANEL A2 DNET COMM FAULT DNET SLOT 6 NODE 53 - SAFETY AND STANDARD COMM FAULT |
| 0x0804 | BA010 DRIVE CONTACTOR PANEL A2 DNET COMM FAULT DNET SLOT 6 NODE 53 - SAFETY COMMUNICATION FAULT |
| 0x0805 | BA010 DRIVE CONTACTOR PANEL A2 DNET COMM FAULT DNET SLOT 6 NODE 53 - STANDARD COMMUNICATION FAULT |
| 0x080D | BA010 LEFT OIC SAFETY MODULE DNET COMM FAULT DNET SLOT 5 NODE 50 - SAFETY AND STANDARD COMM FAULT |
| 0x080E | BA010 LEFT OIC SAFETY MODULE DNET COMM FAULT DNET SLOT 5 NODE 50 - SAFETY COMMUNICATION FAULT |
| 0x080F | BA010 LEFT OIC SAFETY MODULE DNET COMM FAULT DNET SLOT 5 NODE 50 - STANDARD COMMUNICATION FAULT |
| 0x0810 | BA010 LEFT OIC SIEMENS I/O MODULE FAULT CHECK ET200S BUSCOUPLER MODULE |
| 0x0811 | BA010 RIGHT OIC SAFETY MODULE DNET COMM FAULT DNET SLOT 6 NODE 56 - SAFETY AND STANDARD COMM FAULT |
| 0x0812 | BA010 RIGHT OIC SAFETY MODULE DNET COMM FAULT DNET SLOT 6 NODE 56 - SAFETY COMMUNICATION FAULT |
| 0x0813 | BA010 RIGHT OIC SAFETY MODULE DNET COMM FAULT DNET SLOT 6 NODE 56 - STANDARD COMMUNICATION FAULT |
| 0x0814 | BA010 RIGHT OIC SIEMENS I/O MODULE FAULT CHECK ET200S BUSCOUPLER MODULE |
| 0x0817 | BA010 WHEELBASE DRIVE WBD FAULT BRAKE THERMAL OL |
| 0x0818 | BA010 WHEELBASE ENCODER ENC1 WARNING |
| 0x0821 | BA010 MAOKPS AIR IS NOT ON BK04.I.D07 CHECK CELL SHUTOFF |
| 0x0822 | BA010 HAS A COMMUNICATION FAULT |
| 0x0823 | BA010 LLOKFS LUBE LEVEL LOW BK04.I.D13 NOT ON |
| 0x0824 | BA010 LCPX LUBE CYCLE OVER TIME LVP O30400 ON, BK04.I.D12 NOT CHANGING |
| 0x0834 | BA010 FLRM FRONT LEFT MOTOR ROLLER MOTOR IS NOT RUNNING CHECK CT DRIVE PANEL E502M MOTOR AND ENCODER |
| 0x0835 | BA010 FRRM FRONT RIGHT MOTOR ROLLER MOTOR IS NOT RUNNING CHECK CT DRIVE PANEL E602M MOTOR AND ENCODER |
| 0x0836 | BA010 RLRM REAR LEFT MOTOR ROLLER MOTOR IS NOT RUNNING CHECK CT DRIVE PANEL E702M MOTOR AND ENCODER |
| 0x0837 | BA010 RRRM REAR RIGHT MOTOR ROLLER MOTOR IS NOT RUNNING CHECK CT DRIVE PANEL E802M MOTOR AND ENCODER |
| 0x0838 | BA010 CT REGEN E302PM ERROR |
| 0x0839 | BA010 CT DRIVE ERROR CHECK CT DRIVES FOR ERROR CODE |
| 0x083A | BA010 CT DRIVE COMMUNICATION LOST ERROR CODE 120 |
| 0x083B | BA010 CT DRIVE WRONG MODE ERROR CODE 121 |
| 0x083C | BA010 CT DRIVE MOTOR ENCODER FAULT ERROR CODE 122 |
| 0x083D | BA010 CT DRIVE MOTOR OVER TEMPERATURE ERROR CODE 123 |
| 0x083E | BA010 FLRM CT DRIVE FAULT CHECK CT DRIVE PANEL E502PM |
| 0x083F | BA010 FRRM CT DRIVE FAULT CHECK CT DRIVE PANEL E602PM |
| 0x0840 | BA010 RLRM CT DRIVE FAULT CHECK CT DRIVE PANEL E702PM |
| 0x0841 | BA010 RRRM CT DRIVE FAULT CHECK CT DRIVE PANEL E802PM |
| 0x0842 | BA010 CT DRIVE HEARTBEAT FAULT NO COMMUNICATION FROM PLC |
| 0x0844 | BA010 FRONT LEFT ROLLER MOTOR DISCONNECT SWITCH IS OFF BK04.I.D08 CHECK MAINS AND BLOWER SWITCHES |
| 0x0845 | BA010 FRONT RIGHT ROLLER MOTOR DISCONNECT SWITCH IS OFF BK04.I.D09 CHECK MAINS AND BLOWER SWITCHES |
| 0x0846 | BA010 REAR LEFT ROLLER MOTOR DISCONNECT SWITCH IS OFF BK03.I.D00 CHECK MAINS AND BLOWER SWITCHES |
| 0x0847 | BA010 REAR RIGHT ROLLER MOTOR DISCONNECT SWITCH IS OFF BK03.I.D09 CHECK MAINS AND BLOWER SWITCHES |
| 0x0848 | BA010 E114CB2 TRIPPED CHECK CT REGEN E302PM I/O MODULE INPUT 1 |
| 0x0849 | BA010 CT PANEL TEMPERATURE TOO HIGH CHECK CT REGEN E302PM I/O MODULE INPUT 2 |
| 0x084A | BA010 SAFETY RELAY E1216SRM OFF CHECK CT REGEN E302PM I/O MODULE INPUT 3 |
| 0x084B | BA010 CT DRIVE HEALTHY BIT(S) OFF CHECK CT REGEN E302PM I/O MODULE INPUT 4 |
| 0x084C | BA010 ROLLER MOTOR FAN OVERLOAD(S) TRIPPED CHECK CT REGEN E302PM I/O MODULE INPUT 5 |
| 0x084D | BA010 CT PANEL COOLING UNIT(S) FAULTED CHECK CT REGEN E302PM I/O MODULE INPUT 6 |
| 0x084F | BA010 CT DRIVE FRONT LEFT ZERO SPEED MONITORING FAULT CHECK ZERO SPEED SWITCHES AND DRIVE BELTS |
| 0x0850 | BA010 CT DRIVE FRONT RIGHT ZERO SPEED MONITORING FAULT CHECK ZERO SPEED SWITCHES AND DRIVE BELTS |
| 0x0851 | BA010 CT DRIVE REAR LEFT ZERO SPEED MONITORING FAULT CHECK ZERO SPEED SWITCHES AND DRIVE BELTS |
| 0x0852 | BA010 CT DRIVE REAR RIGHT ZERO SPEED MONITORING FAULT CHECK ZERO SPEED SWITCHES AND DRIVE BELTS |
| 0x0867 | BA010 NO DATA FROM GATS |
| 0x0898 | BA010 FRONT RESTRAIN ROLLS HIGH PRESSURE NOT ON CHECK LVP F472R1 |
| 0x0899 | BA010 FRONT RESTRAIN ROLLS LOW PRESSURE NOT ON CHECK LVP F472R1 |
| 0x089A | BA010 REAR RESTRAIN ROLLS HIGH PRESSURE NOT ON CHECK LVP F476R3 |
| 0x089B | BA010 REAR RESTRAIN ROLLS LOW PRESSURE NOT ON CHECK LVP F476R3 |
| 0x089C | BA010 FRONT LEFT FRONT RESTRAIN ROLL CLAMPED NOT OFF C03PX1 BK01.I.D02 |
| 0x089D | BA010 FRONT LEFT FRONT RESTRAIN ROLL UNCLAMPED NOT ON C03PX2 BK01.I.D03 |
| 0x089E | BA010 FRONT RIGHT FRONT RESTRAIN ROLL CLAMPED NOT OFF C04PX3 BK02.I.D02 |
| 0x089F | BA010 FRONT RIGHT FRONT RESTRAIN ROLL UNCLAMPED NOT ON C04PX4 BK02.I.D03 |
| 0x08A0 | BA010 REAR LEFT FRONT RESTRAIN ROLL CLAMPED NOT OFF C22PX1 BK07.I.D04 |
| 0x08A1 | BA010 REAR LEFT FRONT RESTRAIN ROLL UNCLAMPED NOT ON C22PX2 BK07.I.D05 |
| 0x08A2 | BA010 REAR LEFT FRONT RESTRAIN ROLL CLAMPED NOT OFF C23PX3 BK07.I.D02 |
| 0x08A3 | BA010 REAR LEFT FRONT RESTRAIN ROLL UNCLAMPED NOT ON C23PX4 BK07.I.D03 |
| 0x08A4 | BA010 FRONT AXLE FRONT RESTRAIN ROLL LOWERED NOT OFF C04PX1 BK02.I.D00 PM1.O.V1S14 |
| 0x08A5 | BA010 FRONT AXLE FRONT RESTRAIN ROLL RAISED NOT ON C04PX2 BK02.I.D01 PM1.O.V1S14 |
| 0x08A8 | BA010 REAR AXLE FRONT RESTRAIN ROLL LOWERED NOT OFF C23PX1 BK07.I.D00 PM2.O.V3S14 |
| 0x08A9 | BA010 REAR AXLE FRONT RESTRAIN ROLL RAISED NOT ON C23PX2 BK07.I.D01 PM2.O.V3S14 |
| 0x08AC | BA010 FRONT LEFT REAR RESTRAIN ROLL CLAMPED NOT OFF C07PX3 BK01.I.D06 |
| 0x08AD | BA010 FRONT LEFT REAR RESTRAIN ROLL UNCLAMPED NOT ON C07PX4 BK01.I.D07 |
| 0x08AE | BA010 FRONT RIGHT REAR RESTRAIN ROLL CLAMPED NOT OFF C08PX1 BK02.I.D06 |
| 0x08AF | BA010 FRONT RIGHT REAR RESTRAIN ROLL UNCLAMPED NOT ON C08PX2 BK02.I.D07 |
| 0x08B0 | BA010 FRONT AXLE REAR RESTRAIN ROLL LOWERED NOT OFF C07PX1 BK01.I.D04 PM1.O.V4S14 |
| 0x08B1 | BA010 FRONT AXLE REAR RESTRAIN ROLL RAISED NOT ON C07PX2 BK01.I.D05 PM1.O.V4S14 |
| 0x08B4 | BA010 FRONT LEFT REAR RESTRAIN ROLL UNCLAMPED NOT OFF C07PX4 BK01.I.D07 PM1.O.V5S14 |
| 0x08B5 | BA010 FRONT LEFT REAR RESTRAIN ROLL CLAMPED NOT ON C07PX3 BK01.I.D06 PM1.O.V5S14 |
| 0x08B6 | BA010 FRONT RIGHT REAR RESTRAIN ROLL UNCLAMPED NOT OFF C08PX2 BK02.I.D07 PM1.O.V5S14 |
| 0x08B7 | BA010 FRONT RIGHT REAR RESTRAIN ROLL CLAMPED NOT ON C08PX1 BK02.I.D06 PM1.O.V5S14 |
| 0x08B8 | BA010 LEFT SAFETY POST LOWERED C10PX1 NOT OFF BK04.I.D00 PM1.O.V7S14 |
| 0x08B9 | BA010 LEFT SAFETY POST RAISED C10PX2 NOT ON BK04.I.D01 PM1.O.V7S14 |
| 0x08BA | BA010 RIGHT SAFETY POST LOWERED C11PX1 NOT OFF BK04.I.D02 PM1.O.V7S14 |
| 0x08BB | BA010 RIGHT SAFETY POST RAISED C11PX2 NOT ON BK04.I.D03 PM1.O.V7S14 |
| 0x08BC | BA010 FRONT AIR DOOR LOWERED C09PX1 NOT OFF BK04.I.D04 PM1.O.V6S14 |
| 0x08BD | BA010 FRONT AIR DOOR RAISED C09PX2 NOT ON BK04.I.D05 PM1.O.V6S14 |
| 0x08C0 | BA010 MONITOR LIFT RAISED C18PX1 NOT OFF BK06.I.D00 PM4.O.V1S14 |
| 0x08C1 | BA010 MONITOR LIFT LOWERED C18PX2 NOT ON BK06.I.D01 PM4.O.V1S14 |
| 0x08C4 | BA010 FRONT LEFT FRONT RESTRAIN ROLL UNCLAMPED NOT OFF C03PX2 BK01.I.D03 PM1.O.V3S14 |
| 0x08C5 | BA010 FRONT LEFT FRONT RESTRAIN ROLL CLAMPED NOT ON C03PX1 BK01.I.D02 PM1.O.V3S14 |
| 0x08C6 | BA010 FRONT RIGHT FRONT RESTRAIN ROLL UNCLAMPED NOT OFF C04PX4 BK02.I.D03 PM1.O.V3S14 |
| 0x08C7 | BA010 FRONT RIGHT FRONT RESTRAIN ROLL CLAMPED NOT ON C04PX3 BK02.I.D02 PM1.O.V3S14 |
| 0x08C8 | BA010 REAR LEFT FRONT RESTRAIN ROLL UNCLAMPED NOT OFF C22PX2 BK07.I.D05 PM2.O.V4S14 |
| 0x08C9 | BA010 REAR LEFT FRONT RESTRAIN ROLL CLAMPED NOT ON C22PX1 BK07.I.D04 PM2.O.V4S14 |
| 0x08CA | BA010 REAR RIGHT FRONT RESTRAIN ROLL UNCLAMPED NOT OFF C23PX4 BK07.I.D03 PM2.O.V4S14 |
| 0x08CB | BA010 REAR RIGHT FRONT RESTRAIN ROLL CLAMPED NOT ON C23PX3 BK07.I.D02 PM2.O.V4S14 |
| 0x08CC | BA010 FRONT LEFT ROLLER BRAKE ENGAGED C12PX2 NOT OFF BK01.I.D11 PM3.O.V1S12 |
| 0x08CD | BA010 FRONT LEFT ROLLER BRAKE DISENGAGED C12PX1 NOT ON BK01.I.D10 PM3.O.V1S12 |
| 0x08CE | BA010 FRONT RIGHT ROLLER BRAKE ENGAGED C13PX2 NOT OFF BK01.I.D13 PM3.O.V1S12 |
| 0x08CF | BA010 FRONT RIGHT ROLLER BRAKE DISENGAGED C13PX1 NOT ON BK01.I.D12 PM3.O.V1S12 |
| 0x08D0 | BA010 REAR LEFT ROLLER BRAKE ENGAGED C14PX2 NOT OFF BK05.I.D11 PM3.O.V2S12 |
| 0x08D1 | BA010 REAR LEFT ROLLER BRAKE DISENGAGED C14PX1 NOT ON BK05.I.D10 PM3.O.V2S12 |
| 0x08D2 | BA010 REAR RIGHT ROLLER BRAKE ENGAGED C15PX2 NOT OFF BK05.I.D13 PM3.O.V2S12 |
| 0x08D3 | BA010 REAR RIGHT ROLLER BRAKE DISENGAGED C15PX1 NOT ON BK05.I.D12 PM3.O.V2S12 |
| 0x08D4 | BA010 WHEELBASE MOTION HAS NOT REACHED ITS SETPOINT |
| 0x08D8 | BA010 REAR EXHAUST DOOR LOWERED C16PX1 NOT OFF BK05.I.D00 PM2.O.V1S14 |
| 0x08D9 | BA010 REAR EXHAUST DOOR RAISED C16PX2 NOT ON BK05.I.D01 PM2.O.V1S14 |
| 0x08DC | BA010 FRONT LEFT ROLLER BRAKE DISENGAGED C12PX1 NOT OFF BK01.I.D10 PM3.O.V1S14 |
| 0x08DD | BA010 FRONT LEFT ROLLER BRAKE ENGAGED C12PX2 NOT ON BK01.I.D11 PM3.O.V1S14 |
| 0x08DE | BA010 FRONT RIGHT ROLLER BRAKE DISENGAGED C13PX1 NOT OFF BK01.I.D12 PM3.O.V1S14 |
| 0x08DF | BA010 FRONT RIGHT ROLLER BRAKE ENGAGED C13PX2 NOT ON BK01.I.D13 PM3.O.V1S14 |
| 0x08E0 | BA010 REAR LEFT ROLLER BRAKE DISENGAGED C14PX1 NOT OFF BK05.I.D10 PM3.O.V2S14 |
| 0x08E1 | BA010 REAR LEFT ROLLER BRAKE ENGAGED C14PX2 NOT ON BK05.I.D11 PM3.O.V2S14 |
| 0x08E2 | BA010 REAR RIGHT ROLLER BRAKE DISENGAGED C15PX1 NOT OFF BK05.I.D12 PM3.O.V2S14 |
| 0x08E3 | BA010 REAR RIGHT ROLLER BRAKE ENGAGED C15PX2 NOT ON BK05.I.D13 PM3.O.V2S14 |
| 0x08E4 | BA010 FRONT AXLE REAR RESTRAIN ROLL RAISED NOT OFF C07PX2 BK01.I.D05 PM1.O.V4S12 |
| 0x08E5 | BA010 FRONT AXLE REAR RESTRAIN ROLL LOWERED NOT ON C07PX1 BK01.I.D04 PM1.O.V4S12 |
| 0x08E8 | BA010 LEFT SAFETY POST RAISED C10PX2 NOT OFF BK04.I.D01 PM1.O.V7S12 |
| 0x08E9 | BA010 LEFT SAFETY POST LOWERED C10PX1 NOT ON BK04.I.D00 PM1.O.V7S12 |
| 0x08EA | BA010 RIGHT SAFETY POST RAISED C11PX2 NOT OFF BK04.I.D03 PM1.O.V7S12 |
| 0x08EB | BA010 RIGHT SAFETY POS LOWERED C11PX1 NOT ON BK04.I.D02 PM1.O.V7S12 |
| 0x08EC | BA010 FRONT AIR DOOR RAISED C09PX2 NOT OFF BK04.I.D05 PM1.O.V6S12 |
| 0x08ED | BA010 FRONT AIR DOOR LOWERED C09PX1 NOT ON BK04.I.D04 PM1.O.V6S12 |
| 0x08F0 | BA010 MONITOR LIFT LOWERED C18PX2 NOT OFF BK06.I.D01 PM4.O.V1S12 |
| 0x08F1 | BA010 MONITOR LIFT RAISED C18PX1 NOT ON BK06.I.D00 PM4.O.V1S12 |
| 0x08F4 | BA010 FRONT AXLE FRONT RESTRAIN ROLL RAISED NOT OFF C04PX2 BK02.I.D01 PM1.O.V1S12 |
| 0x08F5 | BA010 FRONT AXLE FRONT RESTRAIN ROLL LOWERED NOT ON C04PX1 BK02.I.D00 PM1.O.V1S12 |
| 0x08F8 | BA010 REAR AXLE FRONT RESTRAIN ROLL RAISED NOT OFF C23PX2 BK07.I.D01 PM2.O.V3S12 |
| 0x08F9 | BA010 REAR AXLE FRONT RESTRAIN ROLL LOWERED NOT ON C23PX1 BK07.I.D00 PM2.O.V3S12 |
| 0x08FC | BA010 REAR EXHAUST DOOR RAISED C16PX2 NOT OFF BK05.I.D01 PM2.O.V1S12 |
| 0x08FD | BA010 REAR EXHAUST DOOR LOWERED C16PX1 NOT ON BK05.I.D00 PM2.O.V1S12 |
| 0x0900 | BA010 FRONT AXLE FRONT RESTRAIN ROLL WORK POSITION NOT ON C04PX5 BK02.I.D10 PM1.O.V1S14 |
| 0x0901 | BA010 REAR AXLE FRONT RESTRAIN ROLL WORK POSITION NOT ON C23PX5 BK07.I.D10 PM2.O.V3S14 |
| 0x0957 | BA010 PART IS NOT PROCESS COMPLETE SET PROCESS COMPLETE TO SHIP |
| 0x0958 | BA010 STATION NOT IN VALID POSITION RETURN TO LOAD POSITION |
| 0x0959 | BA010 STATION NOT IN VALID POSITION MOVE TO UNLOAD POSITION |
| 0x095A | BA010 STATION IN AUTO AND NOT RUNNING STATION MAY BE OUT OF SEQUENCE |
| 0x095B | BA010 STATION NOT RESETTING PROCESS COMP CHECK PART LOAD AND PROCESS ACTIONS |
| 0x0960 | BA010 WHEELBASE BRAKE RELEASE PRESSURE SWITCH FAULT CHECK BK05.I.D12 |
| 0x0961 | BA010 WHEELBASE DRIVE WBD DNET COMM FAULT DNET SLOT 6 NODE 50 |
| 0x0962 | BA010 WHEELBASE ENCODER ENC1 ALARM |
| 0x0963 | BA010 WHEELBASE DRIVE WBD BRAKE NOT RELEASED CHECK VFD |
| 0x0964 | BA010 WHEELBASE DRIVE WBD FAULT OUTPUT STAGE THERMAL OL |
| 0x0965 | BA010 WHEELBASE DRIVE WBD FAULT MOTOR LOCKED OR THERMAL OL |
| 0x0966 | BA010 WHEELBASE DRIVE WBD FAULT EEPROM ERROR |
| 0x0967 | BA010 WHEELBASE DRIVE WBD HARDWARE ENABLE FAULT CYCLE POWER TO RESET |
| 0x0968 | BA010 WHEELBASE DRIVE WBD FAULT VFD CPU FAULTED |
| 0x0969 | BA010 WHEELBASE DRIVE WBD FAULT SPEED MONITORING TRIPPED |
| 0x096A | BA010 WHEELBASE DRIVE WBD FAULT MOTOR/INVERTER ASSIGNMENT INCORRECT |
| 0x096B | BA010 WHEELBASE DRIVE WBD FAULT DC LINK VOLTAGE TOO HIGH |
| 0x096C | BA010 WHEELBASE DRIVE WBD FAULT OUTPUT SHORT CIRCUIT |
| 0x096D | BA010 WHEELBASE DRIVE WBD FAULT MAIN PHASE FAULT |
| 0x096E | BA010 WHEELBASE DRIVE WBD FAULT FIELDBUS COMMUNICATION LOST |
| 0x096F | BA010 WHEELBASE DRIVE WBD VFD NOT RUNNING |
| 0x0974 | BA010 WHEELBASE FORWARD OVERTRAVEL FAULT |
| 0x0975 | BA010 WHEELBASE REVERSE OVERTRAVEL FAULT |
| 0x0976 | BA010 WHEELBASE FORWARD SOFTWARE LIMIT FAULT |
| 0x0977 | BA010 WHEELBASE REVERSE SOFTWARE LIMIT FAULT |
| 0x0978 | BA010 WHEELBASE ENCODER ENC1 DNET COMM FAULT DNET SLOT 6 NODE 54 |
| 0x0979 | BA010 WHEELBASE ENCODER ENC1 NO MOTION DETECTED |
| 0x09A7 | BA010 OPER FIRST SAFETY DEVICE BLOCKED DURING CYCLE - PRESS RUN BUTTON TO RESTART |
| 0x09A8 | BA010 OPER SECOND SAFETY DEVICE BLOCKED DURING CYCLE - PRESS RUN BUTTON TO RESTART |
| 0x09A9 | BA010 OPER THIRD SAFETY DEVICE BLOCKED DURING CYCLE - PRESS RUN BUTTON TO RESTART |
| 0x09AA | BA010 ROBOT DLD1 NOT OK FOR OPER ENTRY OPER SAFETY DEVICE BLOCKED WHEN ROBOT WAS NOT CLEAR |
| 0x09AB | BA010 ROBOT DLD2 NOT OK FOR OPER ENTRY OPER SAFETY DEVICE BLOCKED WHEN ROBOT WAS NOT CLEAR |
| 0x09AC | BA010 ROBOT DLD3 NOT OK FOR OPER ENTRY OPER SAFETY DEVICE BLOCKED WHEN ROBOT WAS NOT CLEAR |
| 0x09AD | BA010 STATION DLD CHECKS NOT WORKING CHECK B009_SAFETY CONFIGURATION |
| 0x09AE | BA010 STATION DLD1 IS NOT OFF CHECK ROBOT BASE LIMITS OR LIGHT CURTAIN |
| 0x09AF | BA010 STATION DLD2 IS NOT OFF CHECK ROBOT BASE LIMITS OR LIGHT CURTAIN |
| 0x09B0 | BA010 STATION DLD3 IS NOT OFF CHECK ROBOT BASE LIMITS OR LIGHT CURTAIN |
| 0x09B1 | BA010 STATION ROBOT DLD1 IS NOT ON CHECK ROBOT BASE LIMITS OR LIGHT CURTAIN |
| 0x09B2 | BA010 STATION ROBOT DLD2 IS NOT ON CHECK ROBOT BASE LIMITS OR LIGHT CURTAIN |
| 0x09B3 | BA010 STATION ROBOT DLD3 IS NOT ON CHECK ROBOT BASE LIMITS OR LIGHT CURTAIN |
| 0x09B4 | BA010 STATION SAFETY OPERATOR CONFIG FAULT CHECK STATION SAFETY CONFIG IN S_B010_SAFETYCONFIGINPUTS |
| 0x09C4 | BA010 FPTRXPE FRONT TIRE PRESENT NOT OFF BK02.I.D09 |
| 0x09C5 | BA010 RTPFRXPE REAR TIRE PRESENT FRONT NOT OFF BK05.I.D08 |
| 0x09C6 | BA010 RTPRRXPE REAR TIRE PRESENT REAR NOT OFF BK05.I.D09 |
| 0x09C7 | BA010 FPTRXPE FRONT TIRE PRESENT NOT ON BK02.I.D09 |
| 0x09C8 | BA010 RTPFRXPE REAR TIRE PRESENT FRONT NOT ON BK05.I.D08 |
| 0x09C9 | BA010 RTPRRXPE REAR TIRE PRESENT REAR NOT OFF BK05.I.D09 |
| 0x09CA | BA010 FRONT RESTRAIN ROLLER PRESSURE LOWER LIMIT FAULT CHECK GATS FOR PROPER PRESSURE VALUE |
| 0x09CB | BA010 FRONT RESTRAIN ROLLER PRESSURE UPPER LIMIT FAULT CHECK GATS FOR PROPER PRESSURE VALUE |
| 0x09CC | BA010 RF RESTRAIN ROLLER PRESSURE LOWER LIMIT FAULT CHECK GATS FOR PROPER PRESSURE VALUE |
| 0x09CD | BA010 RF RESTRAIN ROLLER PRESSURE UPPER LIMIT FAULT CHECK GATS FOR PROPER PRESSURE VALUE |
| 0x0A3C | BA010 DRIVE CONTACTOR PANEL INPUT/OUTPUT POWER ERROR CHECK BREAKERS, POWER SUPPLY & INPUT POWER TO I/O MODULES |
| 0x0A3D | BA010 DRIVE CONTACTOR PANEL I/O MODULE INPUT FLTD AT LEAST ONE INPUT POINT FAILED - SEE HELP SCREEN |
| 0x0A3E | BA010 DRIVE CONTACTOR PANEL I/O MODULE OUTPUT FLTED AT LEAST ONE OUTPUT POINT FAILED - SEE HELP SCREEN |
| 0x0A3F | BA010 DRIVE CONTACTOR PANEL I/O TEST OUTPUT FAULT ONE TEST OUTPUT SHORTED - SEE HELP SCREEN |
| 0x0A40 | BA010 DRIVE CONTACTOR PANEL SD1 INPUT B OFF FLT I10006 & 7 NOT CYCLING - BLOCK SAFETY DEVICE TO CLEAR |
| 0x0A41 | BA010 DRIVE CONTACTOR PANEL SD1 INPUT A OFF FLT I10006 & 7 NOT CYCLING - BLOCK SAFETY DEVICE TO CLEAR |
| 0x0A42 | BA010 DRIVE CONTACTOR PANEL SD1 INPUTS FLTD I10006 & 7 NOT CYCLING - BLOCK SAFETY DEVICE TO CLEAR |
| 0x0A43 | BA010 DRIVE CONTACTOR PANEL CBL MON E302SRCM NOT ON CHECK FOR UNPLUGGED SKID CABLES I10005 |
| 0x0A44 | BA010 DRIVE CONTACTOR PANEL OK TO RUN F158C NOT OFF CONTACTOR PANEL I10000 |
| 0x0A45 | BA010 DRIVE CONTACTOR PANEL OK TO RUN F158C NOT ON CONTACTOR PANEL I10000 |
| 0x0A46 | BA010 DRIVE CONTACTOR PANEL OK TO RUN F162C NOT OFF CONTACTOR PANEL I10001 |
| 0x0A47 | BA010 DRIVE CONTACTOR PANEL OK TO RUN F162C NOT ON CONTACTOR PANEL I10001 |
| 0x0A48 | BA010 DRIVE CONTACTOR PANEL ENABLE P1 F170CR NOT OFF CONTACTOR PANEL I10003 |
| 0x0A49 | BA010 DRIVE CONTACTOR PANEL ENABLE P1 F170CR NOT ON CONTACTOR PANEL I10003 |
| 0x0A4A | BA010 DRIVE CONTACTOR PANEL CONFIGURATION FAULT CHECK ECS-4180 SAFETY OUTPUT CONFIGURATION |
| 0x0B54 | BA010 SL ERROR: SL ROUTINE VERSIONS NUMBERS DO NOT MATCH |
| 0x0B55 | BA010 SL ERROR: SL.CALLDOUNDO NOT ON WHEN ZZ103_DOUNDO CALLED |
| 0x0B56 | BA010 SL ERROR: SL.INGROUP ON WHEN ZZ111_STARTGROUP CALLED |
| 0x0B57 | BA010 SL ERROR: ZZ112_ENDGROUP TEXT ID DOESN'T MATCH PREV ZZ111_STARTGROUP TEXT ID |
| 0x0B58 | BA010 SL ERROR: SL.INGROUP NOT ON WHEN ZZ112_ENDGROUP CALLED |
| 0x0B59 | BA010 SL ERROR: SL.CALLDOUNDO BIT ON WHEN ZZ103_DOUNDO CALLED |
| 0x0B5A | BA010 SL ERROR: SL.CALLDOUNDO NOT ON WHEN ZZ104_TEXTLINE CALLED |
| 0x0B68 | BA010 LVP SM-1 I/O POWER ERROR CHECK F102CB & INPUT POWER TO I/O MODULES |
| 0x0B69 | BA010 LVP SM-1 I/O MODULE INPUT FAULT AT LEAST ONE INPUT POINT FAILED - SEE HELP SCREEN |
| 0x0B6A | BA010 LVP SM-1 I/O MODULE OUTPUT FAULT AT LEAST ONE OUTPUT POINT FAILED - SEE HELP SCREEN |
| 0x0B6B | BA010 LVP SM-1 LEFT ESTOP INPUT B OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B6C | BA010 LVP SM-1 LEFT ESTOP INPUT A OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B6D | BA010 LVP SM-1 LEFT ESTOP INPUTS FAULTED I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B6E | BA010 LVP SM-1 LEFT ESTOP PB PRESSED |
| 0x0B6F | BA010 LVP SM-1 RIGHT ESTOP INPUT B OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B70 | BA010 LVP SM-1 RIGHT ESTOP INPUT A OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B71 | BA010 LVP SM-1 RIGHT ESTOP INPUTS FAULTED I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B72 | BA010 LVP SM-1 RIGHT ESTOP PB PRESSED |
| 0x0B75 | BA010 LVP SM-1 I/O TEST OUTPUT FAULT ONE TEST OUTPUT SHORTED - SEE HELP SCREEN |
| 0x0B7C | BA010 LVP SM-2 INPUT/OUTPUT POWER ERR CHECK F102CB & INPUT POWER TO I/O MODULES |
| 0x0B7D | BA010 LVP SM-2 I/O MODULE INPUT FLTED AT LEAST ONE INPUT POINT FAILED - SEE HELP SCREEN |
| 0x0B7E | BA010 LVP SM-2 I/O MODULE OUTPUT FLTED AT LEAST ONE OUTPUT POINT FAILED - SEE HELP SCREEN |
| 0x0B7F | BA010 LVP SM-2 FRONT LEFT SAFETY PIN INPUT B OFF FAULT I10000 & 1 NOT CYCLING PROPERLY -PUT SAFETY PIN IN |
| 0x0B80 | BA010 LVP SM-1FRONT LEFT SAFETY PIN INPUT A OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PUT SAFETY PIN IN |
| 0x0B81 | BA010 LVP SM-1FRONT LEFT SAFETY PIN INPUT FAULTED I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B82 | BA010 LVP SM-1 FRONT LEFT SAFETY PIN REMOVED |
| 0x0B83 | BA010 LVP SM-2 FRONT RIGHT SAFETY PIN INPUT B OFF FAULT I10000 & 1 NOT CYCLING PROPEFRY -PUT SAFETY PIN IN |
| 0x0B84 | BA010 LVP SM-1FRONT RIGHT SAFETY PIN INPUT A OFF FAULT I10000 & 1 NOT CYCLING PROPEFRY - PUT SAFETY PIN IN |
| 0x0B85 | BA010 LVP SM-1FRONT RIGHT SAFETY PIN INPUT FAULTED I10000 & 1 NOT CYCLING PROPEFRY -PUT SAFETY PIN IN |
| 0x0B86 | BA010 LVP SM-1 FRONT RIGHT SAFETY PIN REMOVED |
| 0x0B87 | BA010 LVP SM-2 REAR LEFT SAFETY PIN INPUT B OFF FAULT I10000 & 1 NOT CYCLING PROPERLY -PUT SAFETY PIN IN |
| 0x0B88 | BA010 LVP SM-1REAR LEFT SAFETY PIN INPUT A OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PUT SAFETY PIN IN |
| 0x0B89 | BA010 LVP SM-1REAR LEFT SAFETY PIN INPUT FAULTED I10000 & 1 NOT CYCLING PROPERLY -PUT SAFETY PIN IN |
| 0x0B8A | BA010 LVP SM-1 REAR LEFT SAFETY PIN REMOVED |
| 0x0B8B | BA010 LVP SM-2 REAR RIGHT SAFETY PIN INPUT B OFF FAULT I10000 & 1 NOT CYCLING PROPERRY -PUT SAFETY PIN IN |
| 0x0B8C | BA010 LVP SM-1REAR RIGHT SAFETY PIN INPUT A OFF FAULT I10000 & 1 NOT CYCLING PROPERRY - PUT SAFETY PIN IN |
| 0x0B8D | BA010 LVP SM-1REAR RIGHT SAFETY PIN INPUT FAULTED I10000 & 1 NOT CYCLING PROPERRY -PUT SAFETY PIN IN |
| 0x0B8E | BA010 LVP SM-1 REAR RIGHT SAFETY PIN REMOVED |
| 0x0B8F | BA010 LVP SM-2 I/O TEST OUTPUT FAULT ONE TEST OUTPUT SHORTED - SEE HELP SCREEN |
| 0x0B90 | BA010 LEFT OIC PANEL I/O POWER ERROR CHECK F102CB & INPUT POWER TO I/O MODULES |
| 0x0B91 | BA010 LEFT OIC PANEL I/O MODULE INPUT FAULT AT LEAST ONE INPUT POINT FAILED - SEE HELP SCREEN |
| 0x0B92 | BA010 LEFT OIC PANEL I/O MODULE OUTPUT FAULT AT LEAST ONE OUTPUT POINT FAILED - SEE HELP SCREEN |
| 0x0B93 | BA010 LEFT OIC PANEL ESTOP INPUT B OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B94 | BA010 LEFT OIC PANEL ESTOP INPUT A OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B95 | BA010 LEFT OIC PANEL ESTOP INPUTS FAULTED I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B96 | BA010 LEFT OIC PANEL ESTOP PB PRESSED |
| 0x0B99 | BA010 LEFT OIC PANEL I/O TEST OUTPUT FAULT ONE TEST OUTPUT SHORTED - SEE HELP SCREEN |
| 0x0B9A | BA010 RIGHT OIC PANEL I/O POWER ERROR CHECK F102CB & INPUT POWER TO I/O MODULES |
| 0x0B9B | BA010 RIGHT OIC PANEL I/O MODULE INPUT FAULT AT LEAST ONE INPUT POINT FAILED - SEE HELP SCREEN |
| 0x0B9C | BA010 RIGHT OIC PANEL I/O MODULE OUTPUT FAULT AT LEAST ONE OUTPUT POINT FAILED - SEE HELP SCREEN |
| 0x0B9D | BA010 RIGHT OIC PANEL ESTOP INPUT B OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B9E | BA010 RIGHT OIC PANEL ESTOP INPUT A OFF FAULT I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0B9F | BA010 RIGHT OIC PANEL ESTOP INPUTS FAULTED I10000 & 1 NOT CYCLING PROPERLY - PRESS ESTOP TO CLEAR |
| 0x0BA0 | BA010 RIGHT OIC PANEL ESTOP PB PRESSED |
| 0x0BA3 | BA010 RIGHT OIC PANEL I/O TEST OUTPUT FAULT ONE TEST OUTPUT SHORTED - SEE HELP SCREEN |

## 4.4	Appendix D

Roll Test Machine Software Revisions

### 4.4.1	Roll Test Machine Software Revisions

#### 4.4.1.1	Burke E Porter Machine Software Revisions

##### 4.4.1.1.1 VME Based Machines

| Revision Number | Description of Change |
| --- | --- |
| _________ | ____________________________________________________________________________ |
| A | Initial release for development, as per the spec was understood. |
| B | Corrected many, many things due to misunderstandings and mistakes. |
| C | Corrected test arming index marker and driver fault low status. |
| D | Corrected LVI examine light on at power up and screen saver not working. |
| E | First software released to a plant. |
| F | Modified motor drives to better speed match during 4WD testing. |
| G | Added boost in 4WD mode and “drive torque to zero” command. |
| H | Added re-relax command for Shreveport H3. |
| I | Cleared wheelbase command on retainers down transition. |
| J | Error is thrown to PLC as drive fault, then waits for PLC to send clear fault bit before code will try and clear the fault. |
| K | Added motor drive fault tests and checks. |
| L | Added machine error code Mode 1 Messages 17 and 21. |
| M | Correctly added L and deleted 1st Gen. old mode. |
| N | Added parameter 31 for motor loading testing. |
| P | Encoder runout added to Mode 1 Message 17. Scheduled Messages not stopped by rolls down or reset anymore. |
| Q | Added support for AB700 drives. |

##### 4.4.1.1.1 CCRT Based Machines

| Revision Number | Description of Change |
| --- | --- |
| _________ | ____________________________________________________________________________ |
| 0909 | Wheelbase handling updates to ensure wheelbase can be moved on subsequent commands. Brake test complete logic was updated to poll for the complete flag instead of waiting for 1 second. Prompt handling was added for E-Stop. |
| 1001 | Added auto raise/lower retainers option. |
| 1004 | Moved wheelbase table out of configuration file and into a separate file. Drag force issues were corrected. Also updated so ICM data is only logged while GEN3 messages are scheduled. |
| 1007 | Motor controller software updated to place motors in torque 0 mode if the vehicle is AWD. |
| 1008 | Added default parameters so software had something other than 0 at machine startup. This fixed over speed indicators while the rollers were being raised |
| 1011 | Software updated to set default parameter values into Mode 1 Message 9 |
| 1101 | Ret roll hi/low position handling update. Also corrected automatic drive axle detection logic. |

#### 4.4.1.2	DURR Machine Software Revisions

| Revision Number | Description of Change |
| --- | --- |
| _________ | ____________________________________________________________________________ |
| 1 | First software released to a plant. |
| 2 | PLC Software and HMI Updates - Added machine state display screen to HMI (Route B004_HMIStatusData was added to logic in Tasks BA010 and BA010CTControl) and updated restraint roller raise and lower sequences to improve the mechanical lock releasing motion. (Affected Routes S085_FFRestrainRolPresToHigh, S086_RFRestrainRolPresToHigh, S090_UnclampFFRestrainRolls, S091_UnclampRFRestrainRolls, S100_RaiseFFRestrainRol, S101_RaiseRFRestrainRol, S105_FFRestrainRolPresToLow, S106_RFRestrainRolPresToLow, S110_UnclampFRRestrainRolls, S115_RaiseFRRestrainRol, S120_ClampFRRestrainRol, S140_ClampFFRestrainRolls, S141_ClampRFRestrainRolls, S175_LowerFRRestrainRolls, S195_LowerFFRestrainRolls, S196_LowerRFRestrainRolls, S300_FFRestrainRollsInPos and S301_RFRestrainRollsInPos in Task BA010). |
| 3 | PLC Software Update - Axle ratio test calculation correction (GatsExecMessage Rung 22 and GatCalcResultData Rungs 9-13 in Task BA010GATS) and auto tire sizing sequence change to include a larger range of tire sizes (S300_FFRestrainRollsInPos Rung 0 in Task BA010). |
| 4 | PLC Software Correction - Machine state display corrections for vehicle drive type and machine ready bit (GatsExecMessage Rungs 44 and 45 in Task BA010GATS and B004_HMIStatusData Rungs 43-46 in Task BA010). |
| 5 | CT Drive Parameter Change - Parameter 18.11 change from 50 to 100. PLC Software Change - Machine step process change to allow wheelbase movement when given the command from GATS at anytime if certain safety and collision conditions are met (S155_AdjWheelBase Rung 2 in Task BA010). |
| 6 | PLC Software Corrections - Machine state display corrections for Clutch, Drive Enable and Vehicle Present bits (GatsExecMessage Rungs 26, 43-44 in Task BA010GATS, B004_HMIStatusData Rungs 43-46, 48, 49, 51, 52 in Task BA010 and B004_HMIStatusData Rungs 20-23 in Task BA010CTControl ). Corrected race condition between zero speed relay and PLC by increasing the encoder feedback value from 600 to 1000 (S011_CT4MotInterlockFromCT Rungs 12-15 in Task BA010CTControl). Corrected "Machine Fault" and "Test Ready" bit reporting to GATS (S004_OutputsToGats Rungs 1 and 16 in Task BA010). Corrected machine error code reporting to GATS (zZ450_MapDiags Rungs 46-55 in Task HMI1 and GatsSendMessagePrepare Rungs 87-96 in Task BA010GATS). |

## 4.5	 Figure 1

## 4.6	Sample Tests

### 4.6.1	Mode 1 message 21 graphed as test executes.

### 4.6.2	Burke E. Porter Machinery real-time graph data

### 4.6.3	Actual Test head command sequence

Time/ms Commands

000871  DVT CMD      ->> 00  - RESET

001852  BEP PARAMS -> READ

001872  DVT PARAMS -> 2 - WRITE

001912  DVT PARAMS -> 6 - WRITE

001942  DVT PARAMS -> 7 - WRITE

001973  DVT PARAMS -> 8 - WRITE

002043  DVT PARAMS -> 17 - WRITE

002073  DVT PARAMS -> 18 - WRITE

002103  DVT PARAMS -> 19 - WRITE

002123  DVT PARAMS -> 20 - WRITE

002173  DVT PARAMS -> 22 - WRITE

002213  DVT PARAMS -> 24 - WRITE

002243  DVT PARAMS -> 27 - WRITE

002263  DVT PARAMS -> 28 - WRITE

002313  DVT PARAMS -> 36 - WRITE

002473  BEP PARAMS -> READ

002483  DVT CMD      ->> 02  - ABS VEHICLE

002523  DVT CMD      ->> 0C  - 4 WHEEL DRIVE

113763  DVT CMD      ->> 03  - TEST ARMED

115346  DVT CMD      ->> 04  - DRAG

119441  DVT CMD      ->> 05  - TRACTION - 3 CHAN

123187  DVT CMD      ->> 05  - TRACTION - END

126972  DVT CMD      ->> 06 01 - BRAKE - START

127763  DVT CMD      ->> 06 00 - BRAKE - END

132630  DVT CMD      ->> 0C  - 4 WHEEL DRIVE

140422  DVT CMD      ->> 09  - AXLERATIO

153741  DVT CMD      ->> 09  - AXLERATIO

160551  DVT CMD      ->> 10  - RESET KEEP STAT

164266  DVT CMD      ->> 1B  - 90% COMPLETE