| GM Powertrain Electronics Integration and Software GMPT GMLAN Diagnostic Test Mode Configuration Specification Document Number: DiagTestModeConfigSpec.docx-08.10.31.00 April 15, 2009 Note: If you have a printed copy of this document, you must be sure that it is the latest version before reading it. Refer to the GMPT Controls Engineering web site to verify the latest version. |  |
| --- | --- |
| Owner: Matt Kaufer Phone: (248) 535-1107 FAX: (248) 685-5021 VME: (248) 535-1107 E-Mail: matthew.l.kaufer@gm.com | GM Powertrain General Motors Corporation Mail Code 483-316-300 3300 General Motors Drive Milford, Michigan 48380 |

Table of Contents

1.	Introduction	21

1.1	Applicability	21

1.2	Assumptions	21

2.	Diagnostic Services	23

2.1	OVIR / BFPR - NegativeResponse ($7F) Service	24

2.2	ODAR - RequestCurrentPowertrainDiagnosticData ($01) Service	24

2.3	FFRR - RequestPowertrainFreezeFrameData ($02) Service	24

2.4	DFIR - RequestEmission-RelatedDiagnosticTroubleCodes ($03) Service	25

2.5	DRER - ClearDiagnosticInformation ($04) Service	26

2.6	ODAR - RequestOnBoardMonitoringTestResults ($06) Service	27

2.7	DFIR - RequestPendingEmission-RelatedDiagnosticTroubleCodes ($07) Service	27

2.8	ODAR - RequestVehicleInformation ($09) Service	27

2.9	DFIR – Request Emission-Related Diagnostic Trouble Codes With Permanent Status ($0A) Service	28

2.10	DRER - InitiateDiagnosticOperation ($10) Service	28

2.11	FFRR - ReadFailureRecordData ($12) Service	29

2.12	ODAR / BFPR - ReadDataByIdentifier ($1A) Service	30

2.13	OVIR / BFPR - ReturnToNormalMode ($20) Service	30

2.14	ODAR - ReadDataByParameterIdentifier ($22) Service	31

2.15	ODAR - ReadMemoryByAddress ($23) Service	31

2.16	OSAR / BFPR - SecurityAccess ($27) Service	32

2.17	OVIR / BFPR - DisableNormalComm ($28) Service	33

2.18	ODAR - DynamicallyDefineMessage ($2C) Service	33

2.19	ODAR - DefinePIDByAddress ($2D) Service	34

2.20	OFPR / BFPR - RequestDownload ($34) Service	35

2.21	BFPR / OFPR - TransferData ($36) Service	36

2.22	ODAR - WriteDataByIdentifier ($3B) Service	37

2.23	OVIR / BFPR - TesterPresent ($3E) Service	37

2.24	BFPR / OFPR - ReportProgrammedState ($A2) Service	38

2.25	OFPR / BFPR - ProgrammingMode ($A5) Service	38

2.26	DFIR - ReadDiagnosticInformation ($A9) Service	39

2.27	ODAR - ReadDataByPacketIdentifier ($AA) Service	40

2.28	ODCR - DeviceControl ($AE) Service	41

3.	Parameter Identifiers (PIDs)	43

3.1	Legislated PIDs	44

3.1.1	ODAR – PID $0000 – Parameter Identifiers Supported - $01 to $20	44

3.1.2	DFIR – PID $0001 – Emission Related Monitor Status	45

3.1.3	FFRR – PID $0002 – Freeze Frame Diagnostic Trouble Code	46

3.1.4	FULR – PID $0003 – Fuel System Status	47

3.1.5	MAFR – PID $0004 – Calculated Engine Load	47

3.1.6	ECTR – PID $0005 – Engine Coolant Temperature	47

3.1.7	FADR – PID $0006 – Short Term Fuel Trim Bank 1	48

3.1.8	FADR – PID $0007 – Long Term Fuel Trim Bank 1	48

3.1.9	FADR – PID $0008 – Short Term Fuel Trim Bank 2	48

3.1.10	FADR – PID $0009 – Long Term Fuel Trim Bank 2	48

3.1.11	FRPR – PID $000A – Fuel Rail Pressure (Gauge)	48

3.1.12	MAPR – PID $000B – Intake Manifold Absolute Pressure	49

3.1.13	EPSR – PID $000C – Engine Speed	49

3.1.14	VSPR – PID $000D – Vehicle Speed	49

3.1.15	SPKR – PID $000E – Spark Advance	49

3.1.16	EITR – PID $000F – Intake Air Temperature	49

3.1.17	MAFR – PID $0010 – MAF Sensor Mass Airflow	50

3.1.18	TPSR – PID $0011 – Absolute Throttle Position	50

3.1.19	AIRR – PID $0012 – Commanded Secondary Air Injection Status	50

3.1.20	OXYR – PID $0013 – Location of Oxygen Sensors	51

3.1.21	OXYR – PID $0014 – Oxygen Sensor Voltage Bank 1 Sensor 1 / Short Term Fuel Trim Bank 1	51

3.1.22	OXYR – PID $0015 – Oxygen Sensor Voltage Bank 1 Sensor 2	51

3.1.23	OXYR – PID $0018 – Oxygen Sensor Voltage Bank 2 Sensor 1 / Short Term Fuel Trim Bank 2	52

3.1.24	OXYR – PID $0019 – Oxygen Sensor Voltage Bank 2 Sensor 2	52

3.1.25	DFIR – PID $001C – On Board Diagnostic System Type	53

3.1.26	PTOR – PID $001E – Power Take Off Status	54

3.1.27	EPSR – PID $001F – Engine Run Time	54

3.1.28	ODAR – PID $0020 – Parameter Identifiers Supported - $21 to $40	55

3.1.29	VSPR – PID $0021 – Distance With Malfunction Indicator Lamp On	55

3.1.30	EGRR – PID $002C – Exhaust Gas Recirculation Desired Pintle Lift	56

3.1.31	EGRR – PID $002D – Exhaust Gas Recirculation Error	56

3.1.32	EVPR – PID $002E – Canister Purge Duty Cycle	56

3.1.33	FLVR – PID $002F – Total Fuel Level Percentage	56

3.1.34	DRER – PID $0030 – Warm-Up Cycles Since Code Clear	56

3.1.35	VSPR – PID $0031 – Distance Since Code Clear	57

3.1.36	EVPR – PID $0032 – Fuel Tank Vapor Pressure	57

3.1.37	AAPR – PID $0033 – Barometric Pressure	57

3.1.38	EGTR – PID $003C – Catalyst Temperature Bank 1 Sensor 1	57

3.1.39	EGTR – PID $003D – Catalyst Temperature Bank 2 Sensor 1	57

3.1.40	ODAR – PID $0040 – Parameter Identifiers Supported - $41 to $60	58

3.1.41	DFIR – PID $0041 – Emission Related Monitor Status This Driving Cycle	59

3.1.42	PMDR – PID $0042 – Control Module Voltage	60

3.1.43	MAFR – PID $0043 – Absolute Engine Load	60

3.1.44	FULR – PID $0044 – Equivalence Ratio	60

3.1.45	TPSR – PID $0045 – Relative Throttle Position	60

3.1.46	EVPR – PID $0046 – Ambient Air Temperature	61

3.1.47	TPSR – PID $0047 – Absolute Throttle Position B	61

3.1.48	APSR – PID $0049 – Accelerator Pedal Position D	61

3.1.49	APSR – PID $004A – Accelerator Pedal Position E	61

3.1.50	TPSR – PID $004C – Commanded Throttle Position	62

3.1.51	FULR – PID $0051 – Type of fuel currently being utilized by the vehicle	62

3.1.52	E85R – PID $0052 – Alcohol Fuel Percentage	63

3.1.53	ODAR – PID $0060 – Parameter Identifiers Supported - $61 to $80	63

3.1.54	EITR – PID $0068 – Intake Air Temperature Sensor	63

3.1.55	EPSR – PID $007F – Engine Run Time	64

3.1.56	ODAR – PID $0080 – Parameter Identifiers Supported - $81 to $A0	66

3.1.57	ODAR – PID $00A0 – Parameter Identifiers Supported - $A1 to $C0	66

3.1.58	ODAR – PID $00C0 – Parameter Identifiers Supported - $C1 to $E0	67

3.1.59	ODAR – PID $00E0 – Parameter Identifiers Supported - $E1 to $FF	68

3.2	Corporate Standard PIDs	69

3.2.1	TPSR – PID $1121 – Electronic Throttle Control Status	69

3.2.2	FANR – PID $1122 – Cooling Fan Control Status	70

3.2.3	ACCR – PID $1123 – Air Conditioning Status	71

3.2.4	BRKR – PID $1124 – Brake Pedal Status	72

3.2.5	TRNR – PID $1125 – Manual Transmission Status	74

3.2.6	OXYR – PID $1126 – Oxygen Sensor Heater Status	75

3.2.7	EODR – PID $1127 – Engine Oil Status	76

3.2.8	EVPR – PID $1128 – Evaporative Emissions Status	77

3.2.9	FWDR – PID $1129 – Four Wheel Drive Status	78

3.2.10	ENGR – PID $1130 – Power Mode / Starter Status	79

3.2.11	ENGR – PID $1131 – Fuel System Status	80

3.2.12	CDAR – PID $1133 – Displacement on Demand Status	81

3.2.13	IFRR – PID $1134 – Intake Flow Rationality Status	82

3.2.14	PTOR – PID $1135 – Power Take Off / Fast Idle Status	83

3.2.15	PMDR – PID $1141 – Run/Crank Voltage	84

3.2.16	ACCR – PID $1144 – Air Conditioning Refrigerant High Side Fluid Pressure A/D	84

3.2.17	OXYR – PID $1145 – Oxygen Sensor Voltage Bank 1 Sensor 1	84

3.2.18	OXYR – PID $1146 – Oxygen Sensor Voltage Bank 1 Sensor 2	85

3.2.19	OXYR – PID $1148 – Oxygen Sensor Voltage Bank 2 Sensor 1	85

3.2.20	OXYR – PID $1149 – Oxygen Sensor Voltage Bank 2 Sensor 2	85

3.2.21	EGRR – PID $114B – Exhaust Gas Recirculation Pintle Position A/D	85

3.2.22	GENR – PID $114D – Generator F-Terminal Duty Cycle	85

3.2.23	EVPR – PID $114E – Fuel Tank Vapor Pressure A/D	86

3.2.24	EGRR – PID $1152 – Exhaust Gas Recirculation Actual Pintle Lift	86

3.2.25	EOTR – PID $1154 – Estimated Engine Oil Temperature	86

3.2.26	FLVR – PID $1155 – Fuel Level Sensor A/D	86

3.2.27	BBVR – PID $1157 – Brake Booster Vacuum Pressure A/D	86

3.2.28	TCSR – PID $1158 – Requested Torque Input	87

3.2.29	EOPR – PID $115C – Engine Oil Pressure A/D	87

3.2.30	EPSR – PID $115D – Crank Position Sensor Pulse Counter	87

3.2.31	EPSR – PID $115E – Primary Cam Position Sensor Pulse Counter	87

3.2.32	EPSR – PID $115F – Crankshaft Resynchronization Counter	87

3.2.33	ECTR – PID $116F – Power-Up Engine Coolant Temperature	88

3.2.34	EGRR – PID $1171 – Exhaust Gas Recirculation Duty Cycle	88

3.2.35	TCSR – PID $1175 – Delivered Torque Output	88

3.2.36	FPMR – PID $1178 – Fuel Pump Duty Cycle	88

3.2.37	MSFR – PID $1186 – Misfire Catalyst Pass History Counter	88

3.2.38	MSFR – PID $1187 – Misfire Catalyst Fail History Counter	89

3.2.39	MAFR – PID $1188 – Engine Load	89

3.2.40	MSFR – PID $1189 – Misfire Failure Status	89

3.2.41	MSFR – PID $118A – Misfire Catalyst Exceedences Counter	89

3.2.42	MSFR – PID $118B – Misfire Emission Exceedences Counter	90

3.2.43	MSFR – PID $118C – Maximum Misfire Catalyst Exceedences Count	90

3.2.44	MSFR – PID $118D – Maximum Misfire Emission Exceedences Count	90

3.2.45	FADR – PID $1190 – Long Term Fuel Correction Index	90

3.2.46	TRQR – PID $119C – Torque Management Spark Retard	90

3.2.47	EPSR – PID $11A1 – Engine Run Time	91

3.2.48	KNKR – PID $11A6 – Maximum Knock Fast Spark Retard	91

3.2.49	MAFR – PID $11AC – Speed Density Mass Airflow	91

3.2.50	EGRR – PID $11BB – Exhaust Gas Recirculation Closed Valve Pintle Position	91

3.2.51	EGRR – PID $11BD – Exhaust Gas Recirculation Flow Test Trip Sample Count	91

3.2.52	EGRR – PID $11C1 – Exhaust Gas Recirculation Pintle Lift Error	92

3.2.53	IDLR – PID $11C9 – Desired Idle Engine Speed	92

3.2.54	MSFR – PID $11E4 – Cylinder Mode Highest Peak	92

3.2.55	MSFR – PID $11E5 – Cylinder Mode Misfire Threshold	92

3.2.56	MSFR – PID $11E6 – Revolution Mode Balanced Time	92

3.2.57	MSFR – PID $11E8 – Misfire Delay Counter	93

3.2.58	MSFR – PID $11EA – Cylinder 5 Misfire Counter	93

3.2.59	MSFR – PID $11EB – Cylinder 6 Misfire Counter	93

3.2.60	MSFR – PID $11EC – Cylinder 7 Misfire Counter	93

3.2.61	MSFR – PID $11ED – Cylinder 8 Misfire Counter	94

3.2.62	MSFR – PID $11F2 – Misfire Abnormal Speed Ratio	94

3.2.63	MSFR – PID $11F4 – Misfire Catalyst Failure Counter	94

3.2.64	MSFR – PID $11F5 – Maximum Misfire Catalyst Failure Count	94

3.2.65	MSFR – PID $11F6 – Misfire Emission Failure Counter	94

3.2.66	MSFR – PID $11F7 – Maximum Misfire Emission Failure Count	95

3.2.67	MSFR – PID $11F8 – Cylinder 5 Misfire History Counter	95

3.2.68	MSFR – PID $11F9 – Cylinder 6 Misfire History Counter	95

3.2.69	MSFR – PID $11FA – Cylinder 7 Misfire History Counter	95

3.2.70	MSFR – PID $11FB – Cylinder 8 Misfire History Counter	96

3.2.71	MSFR – PID $1200 – Total Misfire Counter	96

3.2.72	MSFR – PID $1201 – Cylinder 1 Misfire History Counter	96

3.2.73	MSFR – PID $1202 – Cylinder 2 Misfire History Counter	96

3.2.74	MSFR – PID $1203 – Cylinder 3 Misfire History Counter	97

3.2.75	MSFR – PID $1204 – Cylinder 4 Misfire History Counter	97

3.2.76	MSFR – PID $1205 – Cylinder 2 Misfire Counter	97

3.2.77	MSFR – PID $1206 – Cylinder 1 Misfire Counter	97

3.2.78	MSFR – PID $1207 – Cylinder 3 Misfire Counter	98

3.2.79	MSFR – PID $1208 – Cylinder 4 Misfire Counter	98

3.2.80	MSFR – PID $1227 – Misfire Emission Fail History Counter	98

3.2.81	MSFR – PID $1228 – Misfire Emission Pass History Counter	98

3.2.82	MSFR – PID $1229 – Revolution Mode Misfire Threshold	99

3.2.83	MSFR – PID $122A – Misfire Engine Cycle Counter	99

3.2.84	DRER – PID $1232 – Emissions Warm-Up Counter	99

3.2.85	DRER – PID $1233 – Non Emissions Warm-Up Counter	99

3.2.86	VSPR – PID $1234 – Distance Since Code Clear	99

3.2.87	FFRR – PID $1235 – Distance at First Failure	100

3.2.88	FFRR – PID $1236 – Distance at Last Failure	100

3.2.89	FFRR – PID $1237 – Failure Record Fail Counter	100

3.2.90	FFRR – PID $1238 – Failure Record Pass Counter	100

3.2.91	FFRR – PID $1239 – Failure Record Not Run Counter	100

3.2.92	EVPR – PID $12AB – Evaporative Emissions Test Result	101

3.2.93	APSR – PID $12B0 – Arbitrated Accelerator Pedal Position	101

3.2.94	TPSR – PID $12B1 – Arbitrated Throttle Position	101

3.2.95	APSR – PID $12B4 – Accelerator Pedal Position Sensor 1 Displacement	101

3.2.96	APSR – PID $12B5 – Accelerator Pedal Position Sensor 2 Displacement	101

3.2.97	TPSR – PID $12B7 – Throttle Position Sensor 1 Displacement	102

3.2.98	TPSR – PID $12B8 – Throttle Position Sensor 2 Displacement	102

3.2.99	TPSR – PID $12B9 – Throttle Position Sensor 2 A/D	102

3.2.100	TPSR – PID $12BA – Throttle Position Sensor 1 A/D	102

3.2.101	APSR – PID $12BC – Accelerator Pedal Position Sensor 2 A/D	102

3.2.102	APSR – PID $12BD – Accelerator Pedal Position Sensor 1 A/D	103

3.2.103	FULR – PID $12C3 – Fuel Injector Pulse Width Bank 1	103

3.2.104	FULR – PID $12C4 – Fuel Injector Pulse Width Bank 2	103

3.2.105	KNKR – PID $12D9 – Total Knock Spark Retard	103

3.2.106	MAFR – PID $12DA – Mass Airflow Sensor Frequency	104

3.2.107	RBSR – PID $12DE – Main Processor Reset Source	104

3.2.108	MAFR – PID $12E0 – Corrected Mass Airflow	104

3.2.109	RBSR – PID $12E7 – Main Processor Running Reset Source Address	104

3.2.110	VTDR – PID $12EE – Vehicle Theft Deterrent Status	105

3.2.111	MSFR – PID $12EF – Crank Angle Sensing Error Learn Attempts	106

3.2.112	MSFR – PID $12F0 – Crank Angle Sensing Error Learn Status	106

3.2.113	EPSR – PID $12F5 – Crank Position Sensor Low Resolution Period	107

3.2.114	MSFR – PID $12FF – Misfire Diagnostic Engine Speed	107

3.2.115	MSFR – PID $1300 – Misfire Diagnostic Engine Load	107

3.2.116	EPSR – PID $130E – Previous Crank Position Sensor Low Resolution Period	107

3.2.117	EPSR – PID $130F – Cylinder Identifier	108

3.2.118	EVPR – PID $1310 – Evaporative Emissions Service Bay Test Result	108

3.2.119	EVPR – PID $1311 – Evaporative Emissions Service Bay Test Abort Reason	108

3.2.120	EVPR – PID $1312 – Evaporative Emissions Service Bay Test Status	109

3.2.121	CRZR – PID $1315 – Cruise Disengage History 1	109

3.2.122	CRZR – PID $1316 – Cruise Disengage History 2	110

3.2.123	CRZR – PID $1317 – Cruise Disengage History 3	111

3.2.124	CRZR – PID $1318 – Cruise Disengage History 4	113

3.2.125	CRZR – PID $1319 – Cruise Disengage History 5	114

3.2.126	CRZR – PID $131A – Cruise Disengage History 6	115

3.2.127	CRZR – PID $131B – Cruise Disengage History 7	116

3.2.128	CRZR – PID $131C – Cruise Disengage History 8	117

3.2.129	FLVR – PID $131D – Fuel Tank Rated Capacity	119

3.2.130	TPSR – PID $131E – Desired Throttle Position	119

3.2.131	APSR – PID $131F – Accelerator Pedal Position	119

3.2.132	FLVR – PID $132A – Total Fuel Volume	119

3.2.133	FLVR – PID $1338 – Fuel Level Sensor A/D Secondary Tank	120

3.2.134	CATR – PID $1354 – Catalyst Monitor Diagnostic Test Counter Bank 1	121

3.2.135	CATR – PID $1355 – Catalyst Monitor Diagnostic Test Counter Bank 2	121

3.2.136	EGRR – PID $135D – Exhaust Gas Recirculation Control Circuit	121

3.2.137	DFIR – PID $135F – Malfunction Indicator Lamp Control Circuit	122

3.2.138	EPSR – PID $1360 – Tachometer Control Circuit	122

3.2.139	AIRR – PID $1365 – Secondary Air Injection Valve Control Circuit	123

3.2.140	AIRR – PID $1366 – Secondary Air Injection Pump Control Circuit	123

3.2.141	VSPR – PID $1367 – Vehicle Speed Output Control Circuit	124

3.2.142	TCSR – PID $1369 – Delivered Torque Output Control Circuit	124

3.2.143	RTDR – PID $136A – Vehicle Lift / Dive Output Control Circuit	125

3.2.144	STRR – PID $136B – Starter Relay Control Circuit	125

3.2.145	FPMR – PID $136C – Fuel Pump Control Circuit	126

3.2.146	ACCR – PID $136D – Air Conditioning Compressor Control Circuit	126

3.2.147	FANR – PID $13AF – Cooling Fan PWM Duty Cycle	127

3.2.148	TPSR – PID $13B1 – Throttle Position Sensor 1 Learned Minimum	127

3.2.149	TPSR – PID $13B2 – Throttle Position Sensor 2 Learned Minimum	127

3.2.150	EPSR – PID $13B5 – Engine Off Time	127

3.2.151	TPSR – PID $13C6 – Throttle Actuator Duty Cycle	127

3.2.152	EPSR – PID $13C7 – Crank Position Sensor Decode Mode	128

3.2.153	GENR – PID $13C8 – Generator L-Terminal Duty Cycle	128

3.2.154	PHSR – PID $13E1 – Intake Cam Phase Bank 1 Control Circuit	128

3.2.155	PHSR – PID $13E2 – Exhaust Cam Phase Bank 1 Control Circuit	129

3.2.156	PHSR – PID $13E3 – Intake Cam Phase Bank 2 Control Circuit	129

3.2.157	PHSR – PID $13E4 – Exhaust Cam Phase Bank 2 Control Circuit	130

3.2.158	VSPR – PID $13EA – Vehicle Speed High Resolution Output Control Circuit	130

3.2.159	IFRR – PID $13F1 – Filtered MAP Model 1 Residual	131

3.2.160	IFRR – PID $13F2 – Filtered MAP Model 2 Residual	131

3.2.161	IFRR – PID $13F3 – Filtered MAF Residual	131

3.2.162	IFRR – PID $13F4 – Filtered Throttle Position Residual	131

3.2.163	SPKR – PID $1400 – Ignition Coil 1 Control Circuit	132

3.2.164	SPKR – PID $1401 – Ignition Coil 2 Control Circuit	132

3.2.165	SPKR – PID $1402 – Ignition Coil 3 Control Circuit	133

3.2.166	SPKR – PID $1403 – Ignition Coil 4 Control Circuit	133

3.2.167	SPKR – PID $1404 – Ignition Coil 5 Control Circuit	134

3.2.168	SPKR – PID $1405 – Ignition Coil 6 Control Circuit	134

3.2.169	SPKR – PID $1406 – Ignition Coil 7 Control Circuit	135

3.2.170	SPKR – PID $1407 – Ignition Coil 8 Control Circuit	135

3.2.171	FULR – PID $1408 – Fuel Injector 1 Control Circuit	136

3.2.172	FULR – PID $1409 – Fuel Injector 2 Control Circuit	136

3.2.173	FULR – PID $140A – Fuel Injector 3 Control Circuit	137

3.2.174	FULR – PID $140B – Fuel Injector 4 Control Circuit	137

3.2.175	FULR – PID $140C – Fuel Injector 5 Control Circuit	138

3.2.176	FULR – PID $140D – Fuel Injector 6 Control Circuit	138

3.2.177	FULR – PID $140E – Fuel Injector 7 Control Circuit	139

3.2.178	FULR – PID $140F – Fuel Injector 8 Control Circuit	139

3.2.179	EVPR – PID $1410 – Canister Purge Control Circuit	140

3.2.180	EVPR – PID $1412 – Canister Vent Control Circuit	140

3.2.181	RINR – PID $1413 – Reverse Inhibit Solenoid Control Circuit	141

3.2.182	SKSR – PID $1414 – Skip Shift Solenoid Control Circuit	141

3.2.183	FANR – PID $141D – Fan Relay 1 / Cooling Fan PWM Control Circuit	142

3.2.184	FANR – PID $141E – Fan Relay 2 Control Circuit	142

3.2.185	EPSR – PID $1420 – Primary Cam Position Sensor Engine Speed	142

3.2.186	EPSR – PID $1421 – Crank Position Sensor Medium Resolution Engine Speed	143

3.2.187	OXYR – PID $146A – Oxygen Sensor Heater Bank 1 Sensor 1 Control Circuit	143

3.2.188	OXYR – PID $146B – Oxygen Sensor Heater Bank 1 Sensor 2 Control Circuit	144

3.2.189	OXYR – PID $146C – Oxygen Sensor Heater Bank 2 Sensor 1 Control Circuit	144

3.2.190	OXYR – PID $146D – Oxygen Sensor Heater Bank 2 Sensor 2 Control Circuit	145

3.2.191	EOPR – PID $1470 – Engine Oil Pressure	145

3.2.192	VLTR – PID $1478 – Reference Voltage A	145

3.2.193	VLTR – PID $1479 – Reference Voltage B	145

3.2.194	FANR – PID $147B – Fan Relay 3 Control Circuit	146

3.2.195	OXYR – PID $147F – Oxygen Sensor Heater Duty Cycle Bank 1 Sensor 1	146

3.2.196	OXYR – PID $1480 – Oxygen Sensor Heater Duty Cycle Bank 2 Sensor 1	146

3.2.197	OXYR – PID $1481 – Oxygen Sensor Heater Current Bank 1 Sensor 1	146

3.2.198	OXYR – PID $1482 – Oxygen Sensor Heater Current Bank 1 Sensor 2	147

3.2.199	OXYR – PID $1484 – Oxygen Sensor Heater Current Bank 2 Sensor 1	147

3.2.200	OXYR – PID $1485 – Oxygen Sensor Heater Current Bank 2 Sensor 2	147

3.2.201	EVPR – PID $1489 – Fuel Tank Vapor Temperature	147

3.2.202	PMDR – PID $148C – Powertrain Relay Control Circuit	148

3.2.203	PMDR – PID $148D – Powertrain Relay Voltage	148

3.2.204	FPMR – PID $148E – Variable Speed Fuel Pump Diagnostic Duty Cycle	148

3.2.205	FLVR – PID $1502 – Secondary Fuel Pump Control Circuit	149

3.2.206	GENR – PID $1503 – Generator L-Terminal Control Circuit	149

3.2.207	TPSR – PID $150A – Maximum Throttle Position	150

3.2.208	CRZR – PID $150C – Cruise Control Status	150

3.2.209	BBVR – PID $150D – Brake Booster Vacuum Pressure	151

3.2.210	ACCR – PID $150E – Air Conditioning Disengage History 1	152

3.2.211	ACCR – PID $150F – Air Conditioning Disengage History 2	153

3.2.212	ACCR – PID $1510 – Air Conditioning Disengage History 3	154

3.2.213	ACCR – PID $1511 – Air Conditioning Disengage History 4	155

3.2.214	AIRR – PID $1517 – Secondary Air Injection Diagnostic Test State	156

3.2.215	AIRR – PID $1518 – Secondary Air Injection System Pressure Error Bank 1	156

3.2.216	AIRR – PID $1519 – Secondary Air Injection System Pressure Error Bank 2	156

3.2.217	AIRR – PID $151A – Secondary Air Injection Normalized System Pressure Bank 1	157

3.2.218	AIRR – PID $151B – Secondary Air Injection Normalized System Pressure Bank 2	157

3.2.219	AIRR – PID $151C – Secondary Air Injection System Pressure Bank 1	157

3.2.220	AIRR – PID $151D – Secondary Air Injection System Pressure Bank 2	157

3.2.221	ACCR – PID $151F – Air Conditioning Disengage History 5	158

3.2.222	ACCR – PID $1520 – Air Conditioning Disengage History 6	159

3.2.223	ACCR – PID $1521 – Air Conditioning Disengage History 7	160

3.2.224	ACCR – PID $1522 – Air Conditioning Disengage History 8	161

3.2.225	FLVR – PID $152C – Fuel Level Sensor Power Supply Percent On-Time	161

3.2.226	CDAR – PID $1539 – Displacement on Demand Solenoid A Control Circuit	162

3.2.227	CDAR – PID $153A – Displacement on Demand Solenoid B Control Circuit	162

3.2.228	CDAR – PID $153B – Displacement on Demand Solenoid C Control Circuit	163

3.2.229	CDAR – PID $153C – Displacement on Demand Solenoid D Control Circuit	163

3.2.230	EOTR – PID $153E – Engine Oil Temperature from Sensor	164

3.2.231	AIRR – PID $1549 – Secondary Air Injection System Pressure A/D Bank 1	164

3.2.232	AIRR – PID $1550 – Secondary Air Injection System Pressure A/D Bank 2	164

3.2.233	RVSR – PID $1551 – Remote Vehicle Start Disable History 1	165

3.2.234	RVSR – PID $1552 – Remote Vehicle Start Disable History 2	166

3.2.235	RVSR – PID $1553 – Remote Vehicle Start Disable History 3	167

3.2.236	RVSR – PID $1554 – Remote Vehicle Start Disable History 4	168

3.2.237	RVSR – PID $1555 – Remote Vehicle Start Disable History 5	169

3.2.238	RVSR – PID $1556 – Remote Vehicle Start Disable History 6	170

3.2.239	RVSR – PID $1557 – Remote Vehicle Start Disable History 7	171

3.2.240	RVSR – PID $1558 – Remote Vehicle Start Disable History 8	172

3.2.241	IMOR – PID $1559 – Powertrain / Immobilizer Response Status	173

3.2.242	IMOR – PID $155A – Immobilizer Status	174

3.2.243	IMOR – PID $155B – Immobilizer Security Code Status	175

3.2.244	IMOR – PID $155C – Autolearn Counter	175

3.2.245	IMOR – PID $155D – Autolearn Delay Timer	175

3.2.246	IMOR – PID $155E – Tool Learn Delay Timer	175

3.2.247	IMOR – PID $155F – Immobilizer Pre Release Time	176

3.2.248	AIRR – PID $1560 – Secondary Air Injection Diagnostic Results	176

3.2.249	EOPR – PID $1561 – Predicted Engine Oil Pressure	177

3.2.250	EOPR – PID $1563 – Filtered Engine Oil Pressure Weighted Residual	177

3.2.251	ACCR – PID $1564 – Air Conditioning Refrigerant High Side Fluid Pressure	177

3.2.252	EVPR – PID $1565 – Evaporative Emissions Service Bay Test Denial Reason	177

3.2.253	EVPR – PID $1566 – Engine Off Natural Vacuum Service Bay Test Abort Reason	178

3.2.254	EVPR – PID $1567 – Engine Off Natural Vacuum Service Test Denial Reason	178

3.2.255	EVPR – PID $1568 – Engine Off Natural Vacuum Service Bay Test Result	178

3.2.256	EVPR – PID $1569 – Remaining Distance to Enable Engine Off Natural Vacuum Service Bay Test	179

3.2.257	EVPR – PID $156A – Remaining Engine Run Time to Enable Engine Off Natural Vacuum Service Bay Test	179

3.2.258	FADR – PID $156C – Fuel Adjustment System Diagnostic State	179

3.2.259	FADR – PID $156D – Fuel Adjustment System Diagnostic Intrusive Test State	179

3.2.260	FADR – PID $156E – Filtered Non Purge Long Term Fuel Trim State Bank 1	180

3.2.261	FADR – PID $156F – Filtered Non Purge Long Term Fuel Trim State Bank 2	180

3.2.262	FADR – PID $1570 – Filtered Short Term Fuel Trim Bank 1	180

3.2.263	FADR – PID $1571 – Filtered Short Term Fuel Trim Bank 2	180

3.2.264	FADR – PID $1572 – Filtered Long Term Fuel Trim Bank 1	181

3.2.265	FADR – PID $1573 – Filtered Long Term Fuel Trim Bank 2	181

3.2.266	FADR – PID $1574 – Filtered Non Purge Long Term Fuel Trim Bank 1	181

3.2.267	FADR – PID $1575 – Filtered Non Purge Long Term Fuel Trim Bank 2	181

3.2.268	FADR – PID $1576 – Long Term Fuel Trim Rich Threshold	182

3.2.269	FADR – PID $1577 – Non Purge Long Term Fuel Trim Rich Threshold	182

3.2.270	FADR – PID $1578 – Long Term Fuel Trim Lean Threshold	182

3.2.271	FADR – PID $1579 – Fuel Adjustment System Diagnostic Status	183

3.2.272	KNKR – PID $157A – Cylinders for Which Knock Is Detected	184

3.2.273	MSFR – PID $1582 – Injectors Disabled for Misfire Catalyst Converter Protection	184

3.2.274	EITR – PID $1626 – Minimum Intake Air Temperature	185

3.2.275	FANR – PID $162B – Actual Fan Speed	185

3.2.276	FANR – PID $163F – Desired Fan Speed	185

3.2.277	FANR – PID $1641 – Percent Fan Commanded	185

3.2.278	IMOR – PID $1643 – Immobilizer Security Status	186

3.2.279	IMOR – PID $1644 – Powertrain Immobilizer Security Status	188

3.2.280	ECTR – PID $1652 – High Sided Coolant Rationality Ignition Off Time	189

3.2.281	PHSR – PID $166A – Exhaust Cam Phase Duty Cycle Bank 1	189

3.2.282	PHSR – PID $166B – Intake Cam Phase Duty Cycle Bank 1	189

3.2.283	PHSR – PID $166C – Exhaust Cam Phase Duty Cycle Bank 2	189

3.2.284	PHSR – PID $166D – Intake Cam Phase Duty Cycle Bank 2	189

3.2.285	OXYR – PID $1676 – Oxygen Sensor Heater Duty Cycle Bank 2 Sensor 2	190

3.2.286	OXYR – PID $1677 – Oxygen Sensor Heater Duty Cycle Bank 1 Sensor 2	190

3.2.287	PHSR – PID $168D – Exhaust Cam Phase Angle Actual Bank 1	190

3.2.288	PHSR – PID $168E – Intake Cam Phase Angle Actual Bank 1	190

3.2.289	PHSR – PID $168F – Exhaust Cam Phase Angle Actual Bank 2	190

3.2.290	PHSR – PID $1690 – Intake Cam Phase Angle Actual Bank 2	191

3.2.291	PHSR – PID $1691 – Exhaust Cam Phase Angle Desired Bank 1	191

3.2.292	PHSR – PID $1692 – Intake Cam Phase Angle Desired Bank 1	191

3.2.293	PHSR – PID $1693 – Exhaust Cam Phase Angle Desired Bank 2	191

3.2.294	PHSR – PID $1694 – Intake Cam Phase Angle Desired Bank 2	191

3.2.295	PHSR – PID $1695 – Exhaust Cam Phase Angle Error Bank 1	192

3.2.296	PHSR – PID $1696 – Intake Cam Phase Angle Error Bank 1	192

3.2.297	PHSR – PID $1697 – Exhaust Cam Phase Angle Error Bank 2	192

3.2.298	PHSR – PID $1698 – Intake Cam Phase Angle Error Bank 2	192

3.2.299	EPSR – PID $169A – Exhaust Cam Position Sensor Pulse Counter Bank 1	192

3.2.300	EPSR – PID $169B – Intake Cam Position Sensor Pulse Counter Bank 1	193

3.2.301	EPSR – PID $169C – Exhaust Cam Position Sensor Pulse Counter Bank 2	193

3.2.302	EPSR – PID $169D – Intake Cam Position Sensor Pulse Counter Bank 2	193

3.2.303	TFTR – PID $1940 – Transmission Oil Temperature	193

3.2.304	TISR – PID $1941 – Transmission Input Speed	193

3.2.305	TOSR – PID $1942 – Transmission Output Speed	194

3.2.306	SPRR – PID $194B – Transmission 4-5 Upshift Time Error	194

3.2.307	SHTR – PID $194C – Transmission 4-5 Upshift Time	194

3.2.308	TRGR – PID $194F – Transmission PRNDL Range	194

3.2.309	TRGR – PID $1951 – Transmission Range Input Status	195

3.2.310	TRTR – PID $195B – Diagnostic Transmission Ratio	195

3.2.311	FWDR – PID $195D – Transfer Case Ratio	195

3.2.312	TCCR – PID $1970 – Torque Converter Clutch PWM Duty Cycle	196

3.2.313	TPCR – PID $1972 – Transmission Line Pressure PWM Duty Cycle	196

3.2.314	AWDR – PID $1974 – All Wheel Drive Enable Duty Cycle	196

3.2.315	TCCR – PID $1975 – Transmission Component Slipping Counter	196

3.2.316	TRTR – PID $1976 – Modeled Torque Converter Speed Ratio	196

3.2.317	TUDR – PID $197E – Transmission Tap Up/Down Status	197

3.2.318	TCCR – PID $1991 – Torque Converter Clutch Slip	198

3.2.319	SHTR – PID $1992 – Transmission Upshift Time	198

3.2.320	SHTR – PID $1993 – Transmission 1-2 Upshift Time	198

3.2.321	SHTR – PID $1994 – Transmission 2-3 Upshift Time	198

3.2.322	SHTR – PID $1995 – Transmission 3-4 Upshift Time	198

3.2.323	SPRR – PID $1996 – Transmission Upshift Time Error	199

3.2.324	SPRR – PID $1997 – Transmission 1-2 Upshift Time Error	199

3.2.325	SPRR – PID $1998 – Transmission 2-3 Upshift Time Error	199

3.2.326	SPRR – PID $1999 – Transmission 3-4 Upshift Time Error	199

3.2.327	SHPR – PID $199A – Transmission Commanded Gear	199

3.2.328	SPRR – PID $199B – Transmission Upshift Pressure Adapt Cell Index	200

3.2.329	TPCR – PID $199E– Transmission Line Pressure Actual Current	200

3.2.330	TPCR – PID $199F – Transmission Line Pressure Desired Current	200

3.2.331	TRTR – PID $19A1 – Transmission Gear Ratio	200

3.2.332	TBNR – PID $19D4 – Turbine Speed	200

3.2.333	TRTR – PID $1A01 – Torque Converter Speed Ratio	201

3.2.334	TCCR – PID $1A18 – Torque Converter Clutch Pressure	201

3.2.335	TRTR – PID $1A1F – Transmission Clutch Slip	201

3.2.336	TCCR – PID $1A26 – Torque Converter Clutch Slip Error	201

3.2.337	ETQR – PID $1A2D – Engine Actual Steady State Torque	201

3.2.338	TRTR – PID $1A88 – Transmission Modeled Gear Ratio	202

3.2.339	AWDR – PID $1A8E – All Wheel Drive System Status	202

3.2.340	MAPR – PID $2000 – Intake Manifold Absolute Pressure A/D	203

3.2.341	MAPR – PID $2001 – Intake Manifold Vacuum Pressure	203

3.2.342	MAPR – PID $2002 – Supercharger Inlet Absolute Pressure	203

3.2.343	MAPR – PID $2003 – Supercharger Inlet Absolute Pressure A/D	204

3.2.344	MAPR – PID $2004 – Supercharger Inlet Vacuum Pressure	204

3.2.345	EITR – PID $2005 – Power-Up Intake Air Temperature	204

3.2.346	EITR – PID $2006 – Intake Air Temperature 2	204

3.2.347	AAPR – PID $2007 – Barometric Pressure A/D	204

3.2.348	IMTR – PID $2008 – Intake Manifold Tuning Valve Control Circuit	205

3.2.349	IMTR – PID $2009 – Intake Manifold Tuning Vane Position Duty Cycle	205

3.2.350	IMTR – PID $200A – Intake Manifold Tuning Status	206

3.2.351	BSTR – PID $200B – Supercharger Boost Solenoid Duty Cycle	206

3.2.352	BSTR – PID $200C – Supercharger Boost Solenoid Control Circuit	207

3.2.353	BSTR – PID $200D – Intercooler Pump Control Circuit	207

3.2.354	CDAR – PID $200E – Distance With Cylinders Deactivated	208

3.2.355	CDAR – PID $200F – Distance With All Cylinders Activated	208

3.2.356	CDAR – PID $2010 – Displacement On Demand Transitions	208

3.2.357	CDAR – PID $2011 – Displacement on Demand Inhibit Reasons 1	209

3.2.358	CDAR – PID $2012 – Displacement on Demand Inhibit Reasons 2	210

3.2.359	CDAR – PID $2013 – Displacement on Demand Reactivation History 1	211

3.2.360	CDAR – PID $2014 – Displacement on Demand Reactivation History 2	212

3.2.361	CDAR – PID $2015 – Displacement on Demand Reactivation History 3	214

3.2.362	CDAR – PID $2016 – Displacement on Demand Reactivation History 4	215

3.2.363	CDAR – PID $2017 – Displacement on Demand Reactivation History 5	217

3.2.364	CDAR – PID $2018 – Displacement on Demand Reactivation History 6	218

3.2.365	CDAR – PID $2019 – Displacement on Demand Reactivation History 7	220

3.2.366	CDAR – PID $201A – Displacement on Demand Reactivation History 8	222

3.2.367	MTCR – PID $201B – Clutch Pedal Position Sensor A/D	223

3.2.368	MTCR – PID $201C – Clutch Pedal Fully Applied Learned Position	223

3.2.369	MTCR – PID $201D – Clutch Pedal Fully Applied Position Learn Status	224

3.2.370	MTCR – PID $201E – Clutch Pedal Released Learned Position	224

3.2.371	MTCR – PID $201F – Clutch Pedal Displacement	224

3.2.372	DFIR – PID $2020 – Malfunction Indicator Lamp Status	224

3.2.373	RTDR – PID $2021 – Powertrain Induced Chassis Pitch	225

3.2.374	TCSR – PID $2022 – Traction Control System Requested Torque Control Mode	225

3.2.375	GENR – PID $2023 – Regulated Voltage Control Override Source	226

3.2.376	VLTR – PID $2024 – Buffered 5 Volt Supply Status	226

3.2.377	E85R – PID $2025 – Fuel Composition Status	227

3.2.378	RINR – PID $2026 – Reverse Inhibit System Status	227

3.2.379	EFVR – PID $2027 – Exhaust Pressure Regulator Valve Control Circuit	228

3.2.380	EFVR – PID $2028 – Bank 1 Exhaust Back Pressure A/D	228

3.2.381	EFVR – PID $2029 – Bank 1 Exhaust Back Pressure	228

3.2.382	EFVR – PID $202A – Bank 2 Exhaust Back Pressure A/D	229

3.2.383	EFVR – PID $202B – Bank 2 Exhaust Back Pressure	229

3.2.384	OXYR – PID $202C – Air Fuel Imbalance Monitor Filtered Ratio Bank 1	229

3.2.385	OXYR – PID $202D – Air Fuel Imbalance Monitor Filtered Ratio Bank 2	229

3.2.386	OXYR – PID $202E – Air Fuel Imbalance Monitor Maximum Filtered Ratio Bank 1	229

3.2.387	OXYR – PID $202F – Air Fuel Imbalance Monitor Maximum Filtered Ratio Bank 2	229

3.2.388	MEMR – PID $2032 – Processor Fault Reason Code	230

3.2.389	MEMR – PID $2033 – Main Processor NVM Fault Status	230

3.2.390	MEMR – PID $2034 – Main Processor RAM Fault Source Address	231

3.2.391	MEMR – PID $2035 – Processor ROM Fault Source	231

3.2.392	VSPR – PID $2049 – ECU Odometer Status	231

3.2.393	RBSR – PID $204A – Main Processor Total Running Resets	233

3.2.394	RBSR – PID $204B – Main Processor Maximum Running Resets Between Power-up Resets	233

3.2.395	TOSR – PID $204C – Replicated Transmission Output Speed	233

3.2.396	TRQR – PID $204D – Transmission Torque Request Limited Reason	233

3.2.397	TOSR – PID $2050 – Transfer Case Output Speed	234

3.2.398	KNKR – PID $2051 – Knock System Status Sensor 1	234

3.2.399	KNKR – PID $2052 – Knock System Status Sensor 2	234

3.2.400	TPSR – PID $2066 – Learned Airflow Variation Algorithm Compensation	235

3.2.401	CATR – PID $206C – Catalyst Monitor Diagnostic Status Bank 1	235

3.2.402	CATR – PID $206D – Catalyst Monitor Diagnostic Status Bank 2	235

3.2.403	CATR – PID $206E – Catalyst Monitor Diagnostic Status	236

3.2.404	ENGR – PID $2070 – Reduced Engine Power Indication History	236

3.2.405	E85R – PID $209B – Recommended Maximum Ethanol Concentration When Recommended Ethanol Concentration Exceeded	236

3.2.406	E85R – PID $209C – Currently Recommended Maximum Ethanol Concentration	237

3.2.407	E85R – PID $209D – Actual Ethanol Composition When Recommended Ethanol Concentration Exceeded	237

3.2.408	E85R – PID $209E – Ambient Air Temperature When Recommended Ethanol Concentration Exceeded	237

3.2.409	E85R – PID $209F – Ignition Cycles Since Recommended Ethanol Concentration Exceeded	237

3.2.410	E85R – PID $20A0 – Distance Since Recommended Ethanol Concentration Exceeded	238

3.2.411	E85R – PID $20A1 – Fuel Consumed Since Recommended Ethanol Concentration Exceeded	238

3.2.412	E85R – PID $20A2 – Refueling Events Since Recommended Ethanol Concentration Exceeded	238

3.2.413	E85R – PID $20A3 – Fuel Composition Sensor Frequency	238

3.2.414	DMDR – PID $20D7 – Economy Mode Switch Status	238

3.2.415	DMDR – PID $20D8 – Economy Mode Switch A/D	239

3.2.416	SHPR – PID $2800 – Transmission Shift Solenoid A Control Circuit	240

3.2.417	SHPR – PID $2801 – Transmission Shift Solenoid B Control Circuit	240

3.2.418	SHPR – PID $2802 – Transmission Shift Solenoid C Control Circuit	241

3.2.419	D32R – PID $2803 – Transmission 3-2 Downshift Solenoid Control Circuit	241

3.2.420	HSDR – PID $2804 – Transmission High Side Driver 1 Control Circuit	242

3.2.421	HSDR – PID $2805 – Transmission High Side Driver 2 Control Circuit	242

3.2.422	TCCR – PID $2806 – Torque Converter Clutch Enable Control Circuit	243

3.2.423	TCCR – PID $2807 – Torque Converter Clutch Control Solenoid Control Circuit	243

3.2.424	TPCR – PID $2808 – Transmission Line Pressure Control Circuit	244

3.2.425	TCCR – PID $2809 – Torque Converter Clutch Reference Slip	244

3.2.426	SHTR – PID $280A – Transmission 5-6 Upshift Time	244

3.2.427	SPRR – PID $280B – Transmission 5-6 Upshift Time Error	244

3.2.428	TCCR – PID $280C – Torque Converter Clutch Status	245

3.2.429	TFTR – PID $280D – Transmission Control Module Substrate Temperature	245

3.2.430	TOSR – PID $280E – Replicated Transmission Output Speed Output Control Circuit	246

3.2.431	TBPR – PID $280F – Transmission Base Pattern	246

3.2.432	TPSR – PID $2810 – Accelerator Effective Position	247

3.2.433	TPCR – PID $2811 – Pressure Control Solenoid 1 Output Status	247

3.2.434	RSCR – PID $2812 – Pressure Control Solenoid 2 Output Status	248

3.2.435	RSCR – PID $2813 – Pressure Control Solenoid 3 Output Status	248

3.2.436	RSCR – PID $2814 – Pressure Control Solenoid 4 Output Status	249

3.2.437	RSCR – PID $2815 – Pressure Control Solenoid 5 Output Status	249

3.2.438	TCCR – PID $2816 – Torque Converter Clutch Pressure Control Solenoid Output Status	250

3.2.439	TPCR – PID $2817 – Pressure Control Solenoid 1 Commanded Pressure	250

3.2.440	RSCR – PID $2818 – Pressure Control Solenoid 2 Commanded Pressure	250

3.2.441	RSCR – PID $2819 – Pressure Control Solenoid 3 Commanded Pressure	250

3.2.442	RSCR – PID $281A – Pressure Control Solenoid 4 Commanded Pressure	251

3.2.443	RSCR – PID $281B – Pressure Control Solenoid 5 Commanded Pressure	251

3.2.444	RSCR – PID $281C – Torque Converter Clutch Pressure Control Solenoid Commanded Pressure	251

3.2.445	DGSR – PID $281D – Transmission Fluid Pressure Switch Status	251

3.2.446	RSSR – PID $281E – Shift Solenoid 1 Control Circuit	252

3.2.447	RSSR – PID $281F – Shift Solenoid 2 Control Circuit	252

3.2.448	RSSR – PID $2820 – Transmission Primary Oncoming Clutch	253

3.2.449	CCER – PID $2821 – Transmission Full Feed Fill Pressure	253

3.2.450	TOSR – PID $2822 – TISS/TOSS Regulated Voltage Supply Status	253

3.2.451	RSCR – PID $2823 – Service Fast Learn Status	254

3.2.452	RSCR – PID $2824 – Transmission Cleaning Procedure Status	254

3.2.453	TFTR – PID $2826 –Transmission Control Module Restart Sensor Temperature	255

3.2.454	APSR – PID $2856 – Accelerator Pedal Fully Applied Learned Position Sensor 1	255

3.2.455	APSR – PID $2857 – Accelerator Pedal Fully Applied Learned Position Sensor 2	255

3.2.456	TUDR – PID $2861 – Transmission Tap Up/Down Switch 2 Status	256

3.2.457	TPCR – PID $2862 – Calculated Line Pressure	256

3.2.458	EVPR – PID $3A00 – Engine Off Natural Vacuum Phase 1 Result Time	256

3.2.459	EVPR – PID $3A01 – Engine Off Natural Vacuum Phase 2 End Time	257

3.2.460	EVPR – PID $3A02 – Engine Off Natural Vacuum EWMA Test Sample Counter	257

3.2.461	EVPR – PID $3A03 – Accumulated Distance Traveled During Previous Key On Cycles	257

3.2.462	EVPR – PID $3A04 – Engine Off Natural Vacuum Corrected Phase 1 End Pressure	257

3.2.463	EVPR – PID $3A05 – Engine Off Natural Vacuum Phase 1 Result Pressure	257

3.2.464	EVPR – PID $3A06 – Engine Off Natural Vacuum Phase 2 Result Vacuum	258

3.2.465	EVPR – PID $3A07 – Engine Running EVAP Diagnostics Fault History	258

3.3	Development PIDs	258

4.	Data Identifiers (DIDs)	259

4.1	Application Specific DIDs	260

4.1.1	VINR – DID $28 – Odometer Vehicle Identification Number	260

4.1.2	MEMR – DID $29 – Primary Calibration Flash History	262

4.1.3	IMOR – DID $30 – Powertrain Immobilizer Configuration	263

4.1.4	IMOR – DID $31 – Powertrain Immobilizer Calibrations	264

4.1.5	RVSR – DID $32 – Remaining Remote Starts	264

4.1.6	SPRR – DID $41 – Transmission 1-2 Upshift Pressure Adapts	265

4.1.7	SPRR – DID $42 – Transmission 2-3 Upshift Pressure Adapts	266

4.1.8	SPRR – DID $43 – Transmission 3-4 Upshift Pressure Adapts	267

4.1.9	SPRR – DID $44 – Transmission 4-5 Upshift Pressure Adapts	268

4.1.10	TPCR – DID $48 – Line Pressure Learned Characteristics	269

4.1.11	TQTR – DID $49 – Torque Converter Learned Characteristics	269

4.1.12	RSCR – DID $4A – Clutch 1 Pressure Learned Characteristics	270

4.1.13	RSCR – DID $4B – Clutch 2 Pressure Learned Characteristics	270

4.1.14	RSCR – DID $4C – Clutch 3 Pressure Learned Characteristics	271

4.1.15	RSCR – DID $4D – Clutch 4 Pressure Learned Characteristics	271

4.1.16	RSCR – DID $4E – Clutch 5 Pressure Learned Characteristics	272

4.1.17	SSPR – DID $50 – Transmission Steady State Pressure Adapts	273

4.1.18	SSPR – DID $51 – Transmission Steady State Pressure Braking Adapts	274

4.1.19	RSCR – DID $52 – Transmission 1-2 Upshift Adaptive Shift Data	275

4.1.20	RSCR – DID $53 – Transmission 2-3 Upshift Adaptive Shift Data	275

4.1.21	RSCR – DID $54 – Transmission 3-4 Upshift Adaptive Shift Data	276

4.1.22	RSCR – DID $55 – Transmission 4-5 Upshift Adaptive Shift Data	277

4.1.23	RSCR – DID $56 – Transmission 5-6 Upshift Adaptive Shift Data	278

4.1.24	RSCR – DID $57 – Transmission D-R Shift Adaptive Shift Data	279

4.1.25	TFQR – DID $5F – Transmission Oil Remaining Life	279

4.1.26	MSFR – DID $6B – Reference Period Compensation Factors	280

4.1.27	EODR – DID $6D – Engine Oil Remaining Life	280

4.1.28	MEMR – DID $75 – First Time Power Up Flag	280

4.2	Corporate Standard DIDs	280

4.2.1	VINR – DID $90 – Vehicle Identification Number	281

4.2.2	MEMR – DID $97 – System Name or Engine Type	282

4.2.3	MEMR – DID $98 – Repair Shop Code or Tester Serial Number	283

4.2.4	MEMR – DID $99 – Programming Date	283

4.2.5	MEMR – DID $9A – Diagnostic Data Identifier	284

4.2.6	OSAR – DID $A0 – Manufacturers Enable Counter	284

4.2.7	CANR – DID $B0 – ECU Diagnostic Address	285

4.2.8	MEMR – DID $B2 – GM Manufacturing Data	285

4.2.9	MEMR – DID $B4 – Manufacturing Traceability Characters	286

4.2.10	MEMR – DID $B5 – GM Broadcast Code	287

4.2.11	MEMR – DID $B6 – GM Target Vehicle	287

4.2.12	MEMR – DID $B7 – GM Software Usage Description	288

4.2.13	MEMR – DID $B8 – GM Bench Verification Information	289

4.2.14	CANR – DID $B9 – Subnet Configuration List – High Speed	290

4.2.15	MEMR/BFPR – DID $C0 – Boot Software Part Number	291

4.2.16	MEMR – DID $C1 - $C8 – Software Module Identifier	292

4.2.17	MCPR – DID $C9 - $CA – Software Module Identifier	294

4.2.18	MEMR – DID $CB – End Model Part Number	295

4.2.19	MEMR – DID $CC – Base Model Part Number	296

4.2.20	MEMR/BFPR – DID $D0– Boot Software Part Number Alpha Code	297

4.2.21	MEMR – DID $D1 - $D8 – Software Module Identifier Alpha Code	298

4.2.22	MCPR – DID $D9 - $DA – Software Module Identifier Alpha Code	299

4.2.23	MEMR – DID $DB – End Model Part Number Alpha Code	299

4.2.24	MEMR – DID $DC – Base Model Part Number Alpha Code	300

4.2.25	VSPR – DID $DF – ECU Odometer Value	300

5.	Control Packet Identifiers (CPIDs)	301

5.1	Device Control Limit Exceeded Return Codes	302

5.1.1	Device Control Timer Time-Out	302

5.2	CPID $10	303

5.2.1	CDAR – Displacement on Demand Solenoid Device Control	303

5.2.1.1	Deactivate Half of the Engine’s Cylinders	305

5.2.1.2	Deactivate One of the Engine’s Cylinders with Engine Running	306

5.2.1.3	Deactivate One of the Engine’s Cylinders with Engine Not Running	307

5.2.2	EPSR – Crank Decode Mode Device Control	308

5.2.3	SPKR – Spark Retard Device Control	308

5.2.4	SPKR – Disable Idle Spark Device Control	309

5.2.5	MSFR – Crankshaft Position Variation Learn Trigger Device Control	309

5.2.6	EFVR – Exhaust Pressure Regulator Valve Solenoid Device Control	310

5.3	CPID $14	311

5.3.1	TPSR – Forced Idle / Wide Open Throttle Self Test Device Control	311

5.3.2	IMTR – Intake Manifold Tuning Valve Device Control	313

5.3.3	BSTR – Intercooler Pump Device Control	313

5.3.4	BSTR – Supercharger Boost Solenoid Device Control	313

5.3.5	TPSR – Throttle Position Device Control	314

5.3.6	IDLR - Idle Engine Speed Device Control	314

5.3.7	TPSR – Learned Airflow Adapts Reset Trigger Device Control	315

5.3.8	TPSR - Electronic Throttle Control Snapshot Trigger Device Control	315

5.3.9	TPSR - Electronic Throttle Control Validation Parameter Reset Trigger Device Control	316

5.4	CPID $18	316

5.4.1	FULR – Fuel Injector Device Control	316

5.4.1.1	Fuel Injector Flow Device Control	317

5.4.1.2	Fuel Injector Disable Device Control	318

5.4.2	FULR – Fuel Closed Loop Device Control	318

5.4.3	FADR – Fuel Trim Learn Device Control	318

5.4.4	FADR – Fuel Trim Reset Trigger Device Control	318

5.4.5	EVPR – Canister Purge Device Control	319

5.4.6	EVPR – Canister Vent Device Control	319

5.4.7	EVPR – Evap Service Test Trigger Device Control	320

5.4.8	FPMR – Fuel Pump Device Control	321

5.4.8.1	Fuel Pump Device Control with Engine Not Running	321

5.4.8.2	Fuel Pump Device Control with Engine Running	322

5.4.9	E85R – Fuel Composition Reset Trigger Device Control	322

5.4.10	FLVR – Secondary Fuel Pump Device Control	323

5.5	CPID $1C	323

5.5.1	PHSR – Exhaust and Intake Cam Phase Solenoid Device Controls	325

5.5.2	AIRR – Secondary Air Injection Diagnostic Service Bay Test Trigger Device Control	325

5.5.3	AIRR – Secondary Air Injection Reaction System Device Control	326

5.5.4	EGRR – Exhaust Gas Recirculation Device Control	326

5.5.5	CATR – Catalyst Diagnostic Service Bay Test Trigger Device Control	327

5.6	CPID $1D	327

5.6.1	PHSR – Exhaust and Intake Cam Phase Device Controls	329

5.7	CPID $1E	329

5.7.1	OXYR – Oxygen Sensor Heater Resistance Reset Trigger Device Control	329

5.7.2	OXYR – Oxygen Sensor Heaters Device Control	330

5.7.2.1	Oxygen Sensor Heater Device Control with Engine Not Running	330

5.7.2.2	Oxygen Sensor Heater Device Control with Engine Running	331

5.8	CPID $20	332

5.8.1	MTCR – Clutch Pedal Fully Applied Position Learn Trigger Device Control	332

5.8.2	ACCR – Air Conditioning Compressor Device Control	333

5.8.3	FANR – Cooling Fan Speed Device Control	333

5.8.4	GENR – Generator L-Terminal Device Control	334

5.8.5	RVSR – Remote Vehicle Start Disable History Reset Trigger Device Control	334

5.8.6	PMDR – Powertrain Relay Device Control	335

5.8.7	RINR – Reverse Inhibit Device Control	335

5.8.7.1	– Reverse Inhibit for Reverse Enable Solenoid Applications	335

5.8.7.2	– Reverse Inhibit for Reverse Inhibit Solenoid Applications	336

5.8.8	RTDR – Vehicle Lift/Dive Device Control	336

5.8.9	SKSR – Skip Shift Device Control	337

5.8.10	STRR – Starter Relay Device Control	337

5.8.11	DFIR – Malfunction Indicator Lamp Device Control	338

5.8.12	CL2R – Class 2 Learned Source Identifiers Reset Trigger Device Control	338

5.8.13	FANR – Cooling Fan Relay Driver Device Control	339

5.9	CPID $28	339

5.9.1	RBSR – Ignition Off Controller Shutdown Trigger Device Control	340

5.9.2	RBSR – Immediate Controller Shutdown Trigger Device Control	341

5.10	CPID $30	341

5.10.1	D32R – Transmission 32 Downshift Device Control	341

5.10.2	HSDR – Transmission High Side Driver 1 Device Control	342

5.10.3	HSDR – Transmission High Side Driver 2 Device Control	342

5.10.4	PTBR – Powertrain Braking Device Control	343

5.10.5	TCCR/RSCR – Torque Converter Clutch System Device Control	343

5.10.6	TPCR – Transmission Line Pressure Current Device Control	344

5.10.7	AWDR – All Wheel Drive Solenoid Device Control	345

5.10.8	SHPR – Transmission Commanded Gear Device Control	345

5.10.9	RSCR / SPRR - Transmission Adapts Preset Trigger Device Control	346

5.10.10	RSSR - Transmission Fast Adapts Reset Trigger Device Control	346

5.10.11	TRNR - Transmission Adapts Reset Trigger Device Control	347

5.10.12	RSCR – Transmission Fast Learn Trigger	347

5.10.12.1	Transmission Fast Learn Trigger for Service	347

5.10.12.2	Transmission Fast Learn Trigger for Assembly Plant	348

5.10.13	RSCR – Transmission Cleaning Procedure Trigger	348

5.11	CPID $34	350

5.11.1	RSCR – Transmission Clutch Solenoids Device Control	350

5.11.2	RSCR – Torque Converter Clutch Solenoid Device Control	351

5.11.3	RSCR – Transmission M3 Clutch Multiplex Solenoid Device Control	351

5.11.4	RSCR – Transmission M2 Clutch / Reverse Multiplex Solenoid Device Control	352

5.12	CPID $38	352

5.12.1	RSCR – Torque Converter Clutch / Pressure Device Control	352

5.12.2	TPCR – Transmission Line Pressure Device Control	353

5.13	CPID $3A	354

5.13.1	RSCR – Transmission Clutch 1 / 2 Pressure Device Control	354

5.14	CPID $3B	354

5.14.1	RSCR – Transmission Clutch 3 / 4 Pressure Device Control	355

5.15	CPID $7D	355

5.15.1	IMOR – Immobilizer Security Code Input Trigger Device Control	355

5.15.2	IMOR – Immobilizer Reset Trigger Device Control	357

5.15.3	IMOR – Immobilizer Deactivation Trigger Device Control	358

5.15.4	IMOR – Immobilizer Reactivation Trigger Device Control	358

5.15.5	IMOR – Immobilizer Powertrain Challenge Transmission Trigger Device Control	359

5.16	CPID $7E	360

5.16.1	IMOR – Immobilizer Security Code Input Trigger Device Control	360

5.16.2	IMOR – Immobilizer Security Code Programming Trigger Device Control	361

5.17	CPID $FE	361

5.17.1	VTDR – Vehicle Security Password Learn Trigger Device Control	361

6.	Diagnostic Trouble Codes (DTCs)	363

7.	Failure Records / Freeze Frame	365

7.1	Corporate Standard Freeze Frame	365

8.	Compliance with Standards	366

8.1	Compliance with GMPT Product Engineering Standards	366

8.2	Compliance with Industry Standards	366

Appendix A - Revision History	367

Appendix B - List of Acronyms	385

Appendix C - Related Documents	389

Table of Figures

Figure 1:  Device Control Timer Example	304

Table of Tables

Table 1:  Legislated Diagnostic Services	25

Table 2:  Diagnostic Services	25

Table 3: Descriptions of Negative Response Return Codes	26

Table 4: Negative Response Return Codes for ClearDiagnosticInformation ($04) Service	28

Table 5: Negative Response Return Codes for RequestVehicleInformation ($09) Service	29

Table 6:  Sub-Functions for InitiateDiagnosticOperation ($10) Service	30

Table 7: Negative Response Return Codes for InitiateDiagnosticOperation ($10) Service	30

Table 8: Sub-Functions for ReadFailureRecordData ($12) Service	31

Table 9: Negative Response Return Codes for ReadFailureRecordData ($12) Service	31

Table 10: Negative Response Return Codes for ReadDataByIdentifier ($1A) Service	32

Table 11: Negative Response Return Codes for ReturnToNormalMode ($20) Service	32

Table 12: Negative Response Return Codes for ReadDataByParameterIdentifier ($22) Service	33

Table 13: Negative Response Return Codes for ReadMemoryByAddress ($23) Service	33

Table 14: Sub-Functions for SecurityAccess ($27) Service	34

Table 15: Negative Response Return Codes for SecurityAccess ($27) Service	34

Table 16: Negative Response Return Codes for DisableNormalComm ($28) Service	35

Table 17: Negative Response Return Codes for DynamicallyDefineMessage ($2C) Service	35

Table 18: Negative Response Return Codes for DefinePIDByAddress ($2D) Service	36

Table 19: Negative Response Return Codes for RequestDownload ($34) Service	37

Table 20: Sub-Functions TransferData ($36) Service	38

Table 21: Negative Response Return Codes for TransferData ($36) Service	38

Table 22: Negative Response Return Codes for WriteDataByIdentifier ($3B) Service	39

Table 23: Negative Response Return Codes for TesterPresent ($3E) Service	39

Table 24: Negative Response Return Codes for ReportProgrammedState ($A2) Service	40

Table 25: Sub-Functions for ProgrammingMode ($A5) Service	40

Table 26: Negative Response Return Codes for ProgrammingMode ($A5) Service	40

Table 27: Sub-Functions for ReadDiagnosticInformation ($A9) Service	41

Table 28: Negative Response Return Codes for ReadDiagnosticInformation ($A9) Service	41

Table 29: Sub-Functions for ReadDataByPacketIdentifier ($AA) Service	42

Table 30: Actual Scheduling Periods for ReadDataByPacketIdentifier ($AA) Service	42

Table 31: Negative Response Return Codes for ReadDataByPacketIdentifier ($AA) Service	42

Table 32: Negative Response Return Codes for DeviceControl ($AE) Service	43

Table 33: PID Ranges	45

Table 34: DID Ranges	261

Table 35: CPID Ranges	303

Table 36: Descriptions of DTC Fault Types	365

Table 37: One Trip MIL Off DTCs	365

Table 38: Special Operating Condition DTCs	366

# Introduction

This document specifies the configuration of GMLAN diagnostic test modes for GMPT controllers using the C core software.

## Applicability

The intended audience for this document includes the algorithm, software, service, assembly verification, and calibration communities.

## Assumptions

It is assumed that the standard data types in [2] shall be used for the scaling / encoding of all signals specified within this document.

# Diagnostic Services

The Legislated diagnostic services in Table 1shall be implemented according to [3].  Additional details (e.g., the set of sub-functions supported, the set of parameter values supported, specific conditions for negative response return codes, etc.) are specified in the following sections.

| Service Identifier | Diagnostic Service | Application Software | Boot Software |
| --- | --- | --- | --- |
| $01 | RequestCurrentPowertrainDiagnosticData | X |  |
| $02 | RequestPowertrainFreezeFrameData | X |  |
| $03 | RequestEmission-RelatedDiagnosticTroubleCodes-Confirmed | X |  |
| $04 | Clear/ResetDiagnosticInformation | X |  |
| $06 | RequestOnBoardMonitoringTestResults | X |  |
| $07 | RequestEmission-RelatedDiagnosticTroubleCodes-Pending | X |  |
| $08 | RequestControlOfOnBoardSystem |  |  |
| $09 | RequestVehicleInformation | X |  |
| $0A | Request Emission-Related Diagnostic Trouble Codes With Permanent Status | X |  |

Table 1:  Legislated Diagnostic Services

The Enhanced diagnostic services in Table 2:  Diagnostic Services

shall be implemented according to [1].  Additional details (e.g., the set of sub-functions supported, the set of parameter values supported, specific conditions for negative response return codes, etc.) are specified in the following sections.

| Service Identifier | Diagnostic Service | Application Software | Boot Software |
| --- | --- | --- | --- |
| $10 | InitiateDiagnosticOperation | X |  |
| $12 | ReadFailureRecordData | X |  |
| $1A | ReadDataByIdentifier | X | X |
| $20 | ReturnToNormalOperation | X | X |
| $22 | ReadDataByParameterIdentifier | X |  |
| $23 | ReadMemoryByAddress | X |  |
| $27 | SecurityAccess | X | X |
| $28 | DisableNormalCommunication | X | X |
| $2C | DynamicallyDefineMessage | X |  |
| $2D | DefinePIDByAddress | X |  |
| $34 | RequestDownload | X | X |
| $36 | TransferData | X | X |
| $3B | WriteDataByIdentifier | X |  |
| $3E | TesterPresent | X | X |
| $7F | NegativeResponse | X | X |
| $A2 | ReportProgrammedState | X | X |
| $A5 | ProgrammingMode | X | X |
| $A9 | ReadDiagnosticInformation | X |  |
| $AA | ReadDataByPacketIdentifier | X |  |
| $AE | DeviceControl | X |  |

Table 2:  Diagnostic Services

## OVIR / BFPR - NegativeResponse ($7F) Service

The values supported for the returnCode parameter of this diagnostic service are specified in Table 3: Descriptions of Negative Response Return Codes

.

If a request is made for a diagnostic service which is not supported or unknown, a negative response return code of ServiceNotSupported ($11) shall be sent.

| Return Code | Description |
| --- | --- |
| $11 | ServiceNotSupported |
| $12 | SubFunctionNotSupported-InvalidFormat |
| $22 | ConditionsNotCorrectOrRequestSequenceError |
| $31 | RequestOutOfRange |
| $35 | InvalidKey |
| $36 | ExceededNumberOfAttempts |
| $37 | RequiredTimeDelayNotExpired |
| $78 | RequestCorrectlyReceived-ResponsePending |
| $81 | SchedulerFull |
| $83 | VoltageOutOfRangeFault (High / Low) |
| $85 | GeneralProgrammingFailure |
| $89 | DeviceTypeError |
| $99 | ReadyForDownload-DTCStored |
| $E3 | DeviceControlLimitsExceeded |

Table 3: Descriptions of Negative Response Return Codes

## ODAR - RequestCurrentPowertrainDiagnosticData ($01) Service

The values supported for the Parameter Identifiers (PIDs) reported by this legislated diagnostic service are specified in Section 3.1, Legislated PIDs.  The individual PID specifications, scaling information, and Generic Scan Tool display formats are found in [3].  Not all PIDs are applicable or supported by all systems.

PID $00 is a bit-encoded PID that indicates, for each module, which PIDs that module supports.  PID $00 must be supported by all modules that respond to a Mode $01 request as defined in this document, because diagnostic tools that conform to SAE J1978 use the presence of a response by the vehicle to this request to determine which protocol is supported for OBD II communications.

The maximum time after a valid request for a supported PID is received and a response is transmitted by the ECU is 50 milliseconds.

There are no conditions for a negative response return code for this diagnostic service.  No response should be generated when a request is made for a PID that is not supported by the ECU.

Additional details can be found in the Implementation Specification for SAE J1979 Test Modes [11].

## FFRR - RequestPowertrainFreezeFrameData ($02) Service

The current direction within GMPT is to implement Service $02 in the PCM/ECM only.  Service $02 should not be supported by a TCM.

The values supported for the Freeze Frame Parameter Identifiers (PIDs) reported by this legislated diagnostic service are specified in the Implementation Specification for SAE J1979 Test Modes [11].  The individual PID specifications, scaling information, and Generic Scan Tool display formats are found in [3].  Not all PIDs are applicable or supported by all systems.

Frame $00 is the only defined Legislative Freeze Frame that must be supported by all ECUs that respond to a Service $02 request as defined in [3].

Frame $00 / PID $00 is a bit-encoded PID that indicates, for each ECU, which Frame $00 PIDs are supported.  Frame $00 / PID $00 must be supported by all ECUs that respond to a Service $02 request as defined in [3].

The maximum time after a valid request for a supported Frame $00 PID is received and a response is transmitted by the ECU is 50 milliseconds.

There are no conditions for a negative response return code for this legislated diagnostic service.  No response should be generated when a request is made for a Freeze Frame or PID that is not supported by the module.

Additional details can be found in the Implementation Specification for SAE J1979 Test Modes [11].

## DFIR - RequestEmission-RelatedDiagnosticTroubleCodes ($03) Service

The purpose of this mode is to obtain the confirmed emission related trouble codes (DTCs) from all emission related controllers.

There are no conditions for a negative response return code for this legislated diagnostic service.

Additional details can be found in the Implementation Specification for SAE J1979 Test Modes [11].

## DRER - ClearDiagnosticInformation ($04) Service

The purpose of this mode is to request to clear or reset all diagnostic information.  This request will initialize both emission and non-emission related powertrain diagnostics which must include all engine, transmission, electronic throttle control, et al. diagnostics.  The list of variables and corresponding initialization values is extensive.  Along with clearing DTCs, freeze frame and failure records Fault ID's must be cleared, diagnostic timers and counters initialized, Mode 06 test results reset, etc.  Some variable initialization is conditional on whether a DTC is stored.  A typical application will likely be required to initialize hundreds of variables.  "Initialization" in general does not mean "clear".

Other manufacturer specific “clearing/resetting” actions may also occur in response to this request message. All ECUs shall respond to this request message with ignition ON regardless of whether or not the engine is running.

If the ignition key is not in the RUN position (as determined by each ECU), the ECU shall send a negative response message with response code $22 - conditionsNotCorrect.

The maximum time after a positive response to complete clearing of all DTC information is 2.0 seconds.

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 4: Negative Response Return Codes for ClearDiagnosticInformation ($04) Service

.

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. - Request message sent with the physically addressed CAN identifier. |
| $22 | ConditionsNotCorrectOrRequestSequenceError - Ignition key not in RUN position. |

Table 4: Negative Response Return Codes for ClearDiagnosticInformation ($04) Service

Additional details can be found in the Implementation Specification for SAE J1979 Test Modes [11].

## ODAR - RequestOnBoardMonitoringTestResults ($06) Service

The current direction within GMPT is to implement Service $06 in the PCM/ECM only.  Service $06 should not be supported by a TCM. The purpose of this service is to request the on-board diagnostic tests supported by the ECU by On-Board Diagnostic Monitor ID (OBDMID).

Positive responses are required for all ranges up to the largest supported OBDMID corresponding to an actual test.

Responses to a Service $06 will include all supported on-board diagnostic Test Identifiers (TIDs) along with their associated Unit and Scaling ID (UASID)

The individual OBDMID, TID, and UASID specifications are found in [10].  Not all OBDMIDs and TIDs are applicable or supported by all systems.

There are no conditions for a negative response return code for this legislated diagnostic service.

Additional details can be found in the Implementation Specification for SAE J1979 Test Modes [11].

## DFIR - RequestPendingEmission-RelatedDiagnosticTroubleCodes ($07) Service

The purpose of this mode is to obtain the emission related DTCs that have failed either this key-on or during the previous trip.  This provides an indication of the DTCs that have failed this ignition cycle or on the prior trip in which they had run.

There are no conditions for a negative response return code for this legislated diagnostic service.

Additional details can be found in the Implementation Specification for SAE J1979 Test Modes [11].

## ODAR - RequestVehicleInformation ($09) Service

The purpose of this mode is to enable the off-board test device to request vehicle specific information such as Vehicle Identification Number (VIN), Calibration IDs, Calibration Verification Numbers, and Rate Based OBD Monitoring Numbers.

The individual InfoType specifications and Generic Scan Tool display formats are found in [3].  Not all InfoTypes are applicable or supported by all systems.  The InfoTypes reported by this service are specified in  the Implementation Specification for SAE J1979 Test Modes [11].

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 5.

| Return Code | Conditions |
| --- | --- |
| $78 | RequestCorrectlyReceived-ResponsePending - Waiting to calculate Calibration Verification Numbers. |

Table 5: Negative Response Return Codes for RequestVehicleInformation ($09) Service

Additional details can be found in the Implementation Specification for SAE J1979 Test Modes [11].

## DFIR – Request Emission-Related Diagnostic Trouble Codes With Permanent Status ($0A) Service

The purpose of this service is to enable the external test equipment to obtain all DTCs with "permanentDTC" status. The confirmed and/or pending DTCs have been previously cleared by a successful execution of a $04 Clear/reset emission-related diagnostic information service but permanent DTCs remain in the non-volatile memory of the server until the appropriate monitors for each DTC have successfully passed.

Service $0A is required for all emissions-related DTCs. The intended use of this data is to prevent vehicles from passing an in-use inspection simply by disconnecting the battery or clearing DTCs with a scan tool prior to the inspection.   The presence of permanent DTCs at an inspection without the MIL illuminated is an indication that a proper repair was not verified by the on-board monitoring system.

Permanent DTCs shall be stored in non-volatile memory. These DTCs cannot be cleared by any test equipment (e.g. on-board tester, off-board tester). The OBD system shall clear these DTCs itself by completing and passing the on-board monitor and also may require some CARB-defined driving cycle. This prevents clearing DTCs simply by disconnecting the battery.

A confirmed DTC shall be stored as a permanent DTC no later than the end of the ignition cycle and subsequently at all times that the confirmed DTC is commanding the Malfunction Indicator on (e.g., for currently failing systems but not during the 40 warm-up cycle self-healing process).

There are no conditions for a negative response return code for this legislated diagnostic service.

Additional details can be found in the Implementation Specification for SAE J1979 Test Modes [11].

## DRER - InitiateDiagnosticOperation ($10) Service

The sub-functions supported for this diagnostic service are specified in Table 6.

| Level | Sub-Function |
| --- | --- |
| $02 | disableAllDTCs |
| $03 | enableDTCsDuringDeviceCntrl |

Table 6:  Sub-Functions for InitiateDiagnosticOperation ($10) Service

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 7: Negative Response Return Codes for InitiateDiagnosticOperation ($10) Service

.

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. - Sub-function not supported. |
| $22 | ConditionsNotCorrectOrRequestSequenceError - A request message is received with sub-function enableDTCsDuringDeviceCntrl ($03) while the DeviceControl ($AE) diagnostic service is active. - A request message is received with sub-function enableDTCsDuringDeviceCntrl ($03) and DTC’s have been disabled by another diagnostic service. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 7: Negative Response Return Codes for InitiateDiagnosticOperation ($10) Service

## FFRR - ReadFailureRecordData ($12) Service

The sub-functions supported for this diagnostic service are specified in Table 8: Sub-Functions for ReadFailureRecordData ($12) Service

.

| Level | Sub-Function |
| --- | --- |
| $01 | readFailureRecordIdentifiers |
| $02 | readFailureRecordParameters |

Table 8: Sub-Functions for ReadFailureRecordData ($12) Service

A positive response to sub-function readFailureRecordIdentifiers ($01) of this diagnostic service shall always have a value of PID ($00) for the failureRecordDataStructureIdentifier parameter.

The content of the failure records supported for this diagnostic service are specified in Section 7.1 Corporate Standard Freeze Frame

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 9: Negative Response Return Codes for ReadFailureRecordData ($12) Service

.

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. - Sub-function not supported. |
| $31 | RequestOutOfRange - Failure record identifier requested is not valid (i.e., does not match a stored identifier in the failure records). |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 9: Negative Response Return Codes for ReadFailureRecordData ($12) Service

## ODAR / BFPR - ReadDataByIdentifier ($1A) Service

The values supported for the dataIdentifier parameter of this diagnostic service are specified in Section 4, Data Identifiers (DIDs) (only DIDs $B0, $C1, and $CC are supported in boot).

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 10: Negative Response Return Codes for ReadDataByIdentifier ($1A) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. |
| $22 | ConditionsNotCorrectOrRequestSequenceError - Not supported. |
| $31 | RequestOutOfRange - Requested DID not supported. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 10: Negative Response Return Codes for ReadDataByIdentifier ($1A) Service

## OVIR / BFPR - ReturnToNormalMode ($20) Service

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 11: Negative Response Return Codes for ReturnToNormalMode ($20) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. |

Table 11: Negative Response Return Codes for ReturnToNormalMode ($20) Service

## ODAR - ReadDataByParameterIdentifier ($22) Service

The values supported for the parameterIdentifier parameter of this diagnostic service are specified in Section 3, Parameter Identifiers (PIDs).

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 12: Negative Response Return Codes for ReadDataByParameterIdentifier ($22) Service

.

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. |
| $22 | ConditionsNotCorrectOrRequestSequenceError - Not supported. |
| $31 | RequestOutOfRange - Requested PID is not supported (and request message was physically addressed). |
| $78 | RequestCorrectlyReceived-ResponsePending - Sent when positive response may take longer than 100 ms to generate (request for a secondary processor PID). |

Table 12: Negative Response Return Codes for ReadDataByParameterIdentifier ($22) Service

## ODAR - ReadMemoryByAddress ($23) Service

The size of the memoryAddress parameter of this diagnostic service shall be 4 bytes in length.  The maximum value of the memorySize parameter of this diagnostic service shall be 16 bytes.

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 13: Negative Response Return Codes for ReadMemoryByAddress ($23) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. |
| $31 | RequestOutOfRange - Memory address requested is invalid, restricted, or secure while the ECU is locked. - More than 16 bytes requested. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 13: Negative Response Return Codes for ReadMemoryByAddress ($23) Service

## OSAR / BFPR - SecurityAccess ($27) Service

The sub-functions supported for this diagnostic service are specified in Table 14: Sub-Functions for SecurityAccess ($27) Service

.

| Level | Sub-Function |
| --- | --- |
| $01 | SPSrequestSeed |
| $02 | SPSsendKey |
| $03 | DevCtrlrequestSeed |
| $04 | DevCtrlsendKey |

Table 14: Sub-Functions for SecurityAccess ($27) Service

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 15: Negative Response Return Codes for SecurityAccess ($27) Service

.

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. - Sub-function not supported. |
| $22 | ConditionsNotCorrectOrRequestSequenceError - A request message is received with sub-function sendKey ($02 or $04) prior to receiving a request message with sub-function requestSeed ($01 or $03, respectively). - A request message is received with sub-function DevCtrlrequestSeed ($03) while the DeviceControl ($AE) diagnostic service is active. |
| $35 | InvalidKey - Value of the key is not valid. |
| $36 | ExceededNumberOfAttempts - Too many attempts with invalid key values. |
| $37 | RequiredTimeDelayNotExpired - Delay timer is active when a request is received. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 15: Negative Response Return Codes for SecurityAccess ($27) Service

## OVIR / BFPR - DisableNormalComm ($28) Service

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 16: Negative Response Return Codes for DisableNormalComm ($28) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. |
| $22 | ConditionsNotCorrectOrRequestSequenceError - Not supported. |

Table 16: Negative Response Return Codes for DisableNormalComm ($28) Service

## ODAR - DynamicallyDefineMessage ($2C) Service

The values supported for the DPIDIdentifier parameter of this diagnostic service are $EF to $FE, for a total of 16 dynamic DPIDs.

The PID numbers supported for the PIDData parameter of this diagnostic service are specified in Section 3, Parameter Identifiers (PIDs).

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 17: Negative Response Return Codes for DynamicallyDefineMessage ($2C) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length too short (must be at least 4 bytes). - Request message data length not an even number (each PID is a 2 byte value). - The total number of data bytes requested to be reported in a UUDT message exceeds 7 data bytes. |
| $31 | RequestOutOfRange - Requested PID is not supported. - The dynamic DPID number is not supported. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 17: Negative Response Return Codes for DynamicallyDefineMessage ($2C) Service

## ODAR - DefinePIDByAddress ($2D) Service

The values supported for the parameterIdentifier parameter of this diagnostic service are $FE00 to $FE3F, for a total of 64 main processor dynamic PIDs, and $FF00 to $FF3F, for a total of 64 secondary processor dynamic PIDs (if present).

The size of the memoryAddress parameter of this diagnostic service shall be 4 bytes in length.

The value of the memorySize parameter of this diagnostic service shall be 1, 2, or 4 bytes for main processor dynamic PIDs and shall be 2 bytes for secondary processor dynamic PIDs (if present).

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 18: Negative Response Return Codes for DefinePIDByAddress ($2D) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. |
| $31 | RequestOutOfRange - Memory address requested is invalid, restricted, or secure while the ECU is locked. - The size of a main processor dynamic PID is not 1, 2, or 4 bytes or the size of a secondary processor dynamic PID is not 2 bytes. - The dynamic PID number is not supported. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 18: Negative Response Return Codes for DefinePIDByAddress ($2D) Service

## OFPR / BFPR - RequestDownload ($34) Service

Since encryption and compression techniques are not supported with the TransferData ($36) diagnostic service, the values of the dataFormatIdentifier and unCompressedMemorySize parameters of this diagnostic service shall be ignored.

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 19: Negative Response Return Codes for RequestDownload ($34) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. |
| $22 | ConditionsNotCorrectOrRequestSequenceError - The DisableNormalComm ($28) diagnostic service is not active when a request message is received for this diagnostic service. - The ProgrammingMode ($A5) diagnostic service is not active when a request message is received for this diagnostic service. - The ECU is secure (i.e., has not been unlocked via the SecurityAccess ($27) diagnostic service) when a request message is received for this diagnostic service. - The ECU is in the process of receiving a download, via the TransferData ($36) diagnostic service, when a request message is received for this diagnostic service. |
| $78 | RequestCorrectlyReceived-ResponsePending - Transmitted prior to transitioning to boot (positive response is transmitted from boot). |
| $99 | ReadyForDownload-DTCStored - Not supported. |

Table 19: Negative Response Return Codes for RequestDownload ($34) Service

## BFPR / OFPR - TransferData ($36) Service

The sub-functions supported for this diagnostic service are specified in Table 20: Sub-Functions TransferData ($36) Service

| Level | Sub-Function |
| --- | --- |
| $00 | download |
| $80 | downloadAndExecuteOrExecute |

Table 20: Sub-Functions TransferData ($36) Service

The size of the startingAddress parameter of this diagnostic service shall be 4 bytes in length.

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 21: Negative Response Return Codes for TransferData ($36) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Not supported. |
| $22 | ConditionsNotCorrectOrRequestSequenceError - RequestDownload ($34) service has not been received prior to the TransferData ($36) service. - The order of software and/or calibration parts downloaded is invalid |
| $31 | RequestOutOfRange - Not supported. |
| $78 | RequestCorrectlyReceived-ResponsePending - Transmitted after each TransferData ($36) message (positive response is transmitted after the data is processed). |
| $83 | VoltageOutOfRangeFault (High / Low) - Not supported. |
| $85 | GeneralProgrammingFailure - Write or erase error during programming. |
| $89 | DeviceTypeError - Not supported. |

Table 21: Negative Response Return Codes for TransferData ($36) Service

## ODAR - WriteDataByIdentifier ($3B) Service

The values supported for the dataIdentifier parameter of this diagnostic service are specified in Section 4, Data Identifiers (DIDs).

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 22: Negative Response Return Codes for WriteDataByIdentifier ($3B) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect (correct number of data bytes not provided with the requested DID). |
| $22 | ConditionsNotCorrectOrRequestSequenceError - Not supported. |
| $31 | RequestOutOfRange - Requested DID not supported or read-only. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 22: Negative Response Return Codes for WriteDataByIdentifier ($3B) Service

## OVIR / BFPR - TesterPresent ($3E) Service

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 23: Negative Response Return Codes for TesterPresent ($3E) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. |

Table 23: Negative Response Return Codes for TesterPresent ($3E) Service

## BFPR / OFPR - ReportProgrammedState ($A2) Service

The values supported for the programmedState parameter of this diagnostic service are fully programmed ($00) and op s/w present, cal data missing ($02).

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 24: Negative Response Return Codes for ReportProgrammedState ($A2) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 24: Negative Response Return Codes for ReportProgrammedState ($A2) Service

## OFPR / BFPR - ProgrammingMode ($A5) Service

The sub-functions supported for this diagnostic service are specified in Table 25: Sub-Functions for ProgrammingMode ($A5) Service

| Level | Sub-Function |
| --- | --- |
| $01 | requestProgrammingMode |
| $03 | enableProgrammingMode |

Table 25: Sub-Functions for ProgrammingMode ($A5) Service

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 26: Negative Response Return Codes for ProgrammingMode ($A5) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. - Sub-function not supported. |
| $22 | ConditionsNotCorrectOrRequestSequenceError - A request message is received with sub-function enableProgrammingMode ($03) prior to receiving a request message with sub-function requestProgrammingMode ($01). - DisableNormalComm ($28) service has not been received prior to the ProgrammingMode ($A5) service. - Programming mode is already active. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 26: Negative Response Return Codes for ProgrammingMode ($A5) Service

## DFIR - ReadDiagnosticInformation ($A9) Service

The sub-functions supported for this diagnostic service are specified in Table 27: Sub-Functions for ReadDiagnosticInformation ($A9) Service

| Level | Sub-Function |
| --- | --- |
| $80 | readStatusOfDTCByNumber |
| $81 | readStatusOfDTCByStatusMask |
| $82 | sendOnChangeDTCCount |

Table 27: Sub-Functions for ReadDiagnosticInformation ($A9) Service

The values supported for the DTCMaskRecord parameter of sub-function readStatusOfDTCByNumber ($80) of this diagnostic service are specified in Section 6, Diagnostic Trouble Codes (DTCs).

The DTC count for sub-function sendOnChangeDTCCount ($82) of this diagnostic service is updated at an interval of 1.0 second.

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 28: Negative Response Return Codes for ReadDiagnosticInformation ($A9) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. - Sub-function not supported. |
| $31 | RequestOutOfRange - DTC and fault type requested with sub-function readStatusOfDTCByNumber ($80) is not valid. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |

Table 28: Negative Response Return Codes for ReadDiagnosticInformation ($A9) Service

## ODAR - ReadDataByPacketIdentifier ($AA) Service

The sub-functions supported for this diagnostic service are specified in Table 29: Sub-Functions for ReadDataByPacketIdentifier ($AA) Service

| Level | Sub-Function |
| --- | --- |
| $00 | stopSending |
| $01 | sendOneResponse |
| $02 | scheduleAtSlowRate |
| $03 | scheduleAtMediumRate |
| $04 | scheduleAtFastRate |

Table 29: Sub-Functions for ReadDataByPacketIdentifier ($AA) Service

The values supported for each element of the request_DPIDs parameter of this diagnostic service are $EF to $FE, for a total of 16 dynamic DPIDs.  Static DPIDs are not supported.

The ECU shall support up to 5 DPID numbers in a request for sub-function sendOneResponse ($01) of this diagnostic service.

The ECU shall have a Periodic DPID Scheduler (PDS) with up to 16 DPIDs.  The ECU’s actual scheduling rates for the PDS are specified in Table 30: Actual Scheduling Periods for ReadDataByPacketIdentifier ($AA) Service

| Sub-Function | Actual Scheduling Period |
| --- | --- |
| scheduleAtSlowRate ($02) | 1000.0 ms |
| scheduleAtMediumRate ($03) | 200.0 ms |
| scheduleAtFastRate ($04) | 25.0 ms |

Table 30: Actual Scheduling Periods for ReadDataByPacketIdentifier ($AA) Service

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 31: Negative Response Return Codes for ReadDataByPacketIdentifier ($AA) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. - Sub-function not supported. - Number of DPIDs requested in a one shot is greater than 5. - Request contains more than 16 DPIDs to be scheduled. |
| $31 | RequestOutOfRange - DPID requested is not supported or has not yet been defined. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |
| $81 | SchedulerFull - Request would result in more than 16 DPIDs being scheduled. |

Table 31: Negative Response Return Codes for ReadDataByPacketIdentifier ($AA) Service

## ODCR - DeviceControl ($AE) Service

The values supported for the CPIDNumber parameter of this diagnostic service are specified in Section 5, Control Packet Identifiers (CPIDs).

The conditions for each of the negative response return codes for this diagnostic service are documented in Table 32: Negative Response Return Codes for DeviceControl ($AE) Service

| Return Code | Conditions |
| --- | --- |
| $12 | SubFunctionNotSupported-InvalidFormat - Request message data length incorrect. |
| $31 | RequestOutOfRange - CPID requested is not supported. |
| $78 | RequestCorrectlyReceived-ResponsePending - Not supported. |
| $E3 $xx $yy | DeviceControlLimitsExceeded - Device control exited due to internal protection algorithms. A device control limits exceeded return code ($xx $yy) is appended to the negative response to indicate which limit was exceeded. The device control limits exceeded return codes supported for each device control are specified in Section 5, Control Packet Identifiers (CPIDs). |

Table 32: Negative Response Return Codes for DeviceControl ($AE) Service

# Parameter Identifiers (PIDs)

PIDs are divided into several ranges that determine their usage and the ownership of their requirements, as shown in Table 33: PID Ranges

Definitions for legislated PIDs, corporate PIDs, and development PIDs are provided in the following sections.  Definitions for controller specific PIDs must be provided in the associated controller specific diagnostic CTS.

| Range | Usage |
| --- | --- |
| $0000 to $3FFF | Powertrain PIDs |
| $0000 to $00FF | Legislated PIDs (see Section 3.1) |
| $0100 to $0FFF | Reserved |
| $1000 to $2FFF | Powertrain Corporate Standard PIDs (see Section 3.2) |
| $3000 to $3FFF | Powertrain Controller Specific PIDs |
| $4000 to $7FFF | Chassis PIDs (Reserved) |
| $8000 to $BFFF | Body PIDs (Reserved) |
| $C000 to $FFFF | Development PIDs |
| $C000 to $FBFF | Reserved |
| $FC00 to $FCFF | Main Processor Development PIDs (see Section 3.3) |
| $FD00 to $FDFF | Secondary Processor Development PIDs (see Section 3.3) |
| $FE00 to $FEFF | Secondary Processor Dynamic PIDs |
| $FF00 to $FFFF | Main Processor Dynamic PIDs |

Table 33: PID Ranges

## Legislated PIDs

The legislated PIDs below are also specified within [3].  In the event of a conflict between this document and [3], the later shall take precedence. Refer to [11] (Implementation Specification for SAE J1979 Test Modes) for the GM interpretation of the PIDs required to be supported.

### ODAR – PID $0000 – Parameter Identifiers Supported - $01 to $20

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0000 | Parameter Identifiers Supported - $01 to $20 | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| PID $01 Supported | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $02 Supported | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $03 Supported | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $04 Supported | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $05 Supported | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $06 Supported | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $07 Supported | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $08 Supported | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $09 Supported | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $0A Supported | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $0B Supported | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $0C Supported | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $0D Supported | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $0E Supported | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $0F Supported | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $10 Supported | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $11 Supported | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $12 Supported | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $13 Supported | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $14 Supported | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $15 Supported | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $16 Supported | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $17 Supported | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $18 Supported | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $19 Supported | 3 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $1A Supported | 3 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $1B Supported | 3 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $1C Supported | 3 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $1D Supported | 3 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $1E Supported | 3 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $1F Supported | 3 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $20 Supported | 3 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID indicates which legislated PIDs, within the range of $01 to $20, are supported.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### DFIR – PID $0001 – Emission Related Monitor Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0001 | Emission Related Monitor Status | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Malfunction Indicator Lamp Status | 0 | 7 | 1 | ENM | N/A | $0=Off; $1=On |
| Number of Emission Related DTCs | 0 | 6 | 7 | UNM | 0 - 127 | E = N * 1 |
| Reserved | 1 | 7 | 1 |  |  |  |
| Comprehensive Component Monitoring Status | 1 | 6 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Fuel System Monitoring Status | 1 | 5 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Misfire Monitoring Status | 1 | 4 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Reserved | 1 | 3 | 1 |  |  |  |
| Comprehensive Component Monitoring Supported | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Fuel System Monitoring Supported | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Misfire Monitoring Supported | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Gas Recirculation System Monitoring Supported | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Heater Monitoring Supported | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Monitoring Supported | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Air Conditioning System Refrigerant Monitoring Supported | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Secondary Air System Monitoring Supported | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Evaporative System Monitoring Supported | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Heated Catalyst Monitoring Supported | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Catalyst Monitoring Supported | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Gas Recirculation System Monitoring Status | 3 | 7 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Oxygen Sensor Heater Monitoring Status | 3 | 6 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Oxygen Sensor Monitoring Status | 3 | 5 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Air Conditioning System Refrigerant Monitoring Status | 3 | 4 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Secondary Air System Monitoring Status | 3 | 3 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Evaporative System Monitoring Status | 3 | 2 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Heated Catalyst Monitoring Status | 3 | 1 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Catalyst Monitoring Status | 3 | 0 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |

This PID contains the following information associated with the status of the legislated emission related monitors.

The status of the malfunction indicator lamp (MIL).  Note that the MIL status shall indicate “Off” during the key on, engine off bulb check unless the MIL has also been commanded “On” for a detected malfunction.

The number of emission related DTCs stored in the ECU.

The set of continuous monitors (comprehensive component, fuel system, and misfire) and non-continuous monitors (exhaust gas recirculation system, oxygen sensor heater, oxygen sensor, air conditioning system refrigerant, secondary air system, evaporative system, heated catalyst, catalyst) supported and the status of each monitor since DTCs were cleared.

Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### FFRR – PID $0002 – Freeze Frame Diagnostic Trouble Code

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0002 | Freeze Frame Diagnostic Trouble Code | 2 | UNM | 0 - 65535 | E = N * 1 |

This PID contains the diagnostic trouble code (DTC) that caused the freeze frame to be stored.  The mapping of values to DTCs is specified by [4].  A value of zero indicates that a freeze frame has not been stored.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### FULR – PID $0003 – Fuel System Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0003 | Fuel System Status | 2 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reserved | 0 | 5 | 1 |  |  |  |
| Bank 1 Closed Loop - Alternate Control | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 1 Open Loop - System Fault | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 1 Open Loop - Driving Conditions | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 1 Closed Loop | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 1 Open Loop - Minimum Criteria for Closed Loop Not Met | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 1 | 7 | 1 |  |  |  |
| Reserved | 1 | 6 | 1 |  |  |  |
| Reserved | 1 | 5 | 1 |  |  |  |
| Bank 2 Closed Loop - Alternate Control | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 2 Open Loop - System Fault | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 2 Open Loop - Driving Conditions | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 2 Closed Loop | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 2 Open Loop - Minimum Criteria for Closed Loop Not Met | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains the status of each bank of the closed loop fuel control system.  For each fuel control bank, exactly one signal within this PID shall have a value of True.  The value of the PID for each fuel control bank should equal 0 when the engine is not running.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### MAFR – PID $0004 – Calculated Engine Load

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0004 | Calculated Engine Load | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the calculated engine load, mass airflow as a percentage of the maximum mass airflow, where the maximum mass airflow is determined at wide open throttle (WOT) and the current operating conditions (ambient temperature, ambient pressure, and engine speed).  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### ECTR – PID $0005 – Engine Coolant Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0005 | Engine Coolant Temperature | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the undefaulted engine coolant temperature measured by a sensor.  If the data used to compute engine coolant temperature is obtained from a serial data signal, then, if the signal is not received, the value of PID $0005 shall be set to $00 = -40 °C.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### FADR – PID $0006 – Short Term Fuel Trim Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0006 | Short Term Fuel Trim Bank 1 | 1 | UNM | -100 - 99.21875 % | E = N * 100 / 128 - 100 |

This PID contains the short term closed loop fuel correction for bank 1.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### FADR – PID $0007 – Long Term Fuel Trim Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0007 | Long Term Fuel Trim Bank 1 | 1 | UNM | -100 - 99.21875 % | E = N * 100 / 128 - 100 |

This PID contains the long term fuel correction for bank 1.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### FADR – PID $0008 – Short Term Fuel Trim Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0008 | Short Term Fuel Trim Bank 2 | 1 | UNM | -100 - 99.21875 % | E = N * 100 / 128 - 100 |

This PID contains the short term closed loop fuel correction for bank 2.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### FADR – PID $0009 – Long Term Fuel Trim Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0009 | Long Term Fuel Trim Bank 2 | 1 | UNM | -100 - 99.21875 % | E = N * 100 / 128 - 100 |

This PID contains the long term fuel correction for bank 2.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### FRPR – PID $000A – Fuel Rail Pressure (Gauge)

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $000A | Fuel Rail Pressure (Gauge) | 1 | UNM | 0 - 765 kPa | E = N * 3 |

This PID displays fuel rail pressure at the engine when the reading is referenced to atmosphere (gauge pressure).  If the data used to compute fuel pressure is obtained from a serial data signal, then, if the signal is not received, the value of PID $000A shall be set to $00 = 0 kPaG.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### MAPR – PID $000B – Intake Manifold Absolute Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $000B | Intake Manifold Absolute Pressure | 1 | UNM | 0 - 255 kPa | E = N * 1 |

This PID contains the unfiltered engine intake manifold absolute pressure (MAP).  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EPSR – PID $000C – Engine Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $000C | Engine Speed | 2 | UNM | 0 - 16383.75 rpm | E = N * 0.25 |

This PID contains the undefaulted engine speed determined from the low resolution (LORES) period of the crank position sensor.  If the data used to compute engine speed is obtained from a serial data signal, then, if the signal is not received, the value of PID $000C shall be set to $FF = 16383.75 rpm.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### VSPR – PID $000D – Vehicle Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $000D | Vehicle Speed | 1 | UNM | 0 - 255 kph | E = N * 1 |

This PID contains the undefaulted vehicle speed calculated from the transmission output speed.  If the data used to compute vehicle speed is obtained from a serial data signal, then, if the signal is not received, the value of PID $000D shall be set to $FF = 255.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### SPKR – PID $000E – Spark Advance

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $000E | Spark Advance | 1 | UNM | -64 - 63.5 ° | E = N * 0.5 - 64 |

This PID contains the final spark advance in terms of a crankshaft angle.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EITR – PID $000F – Intake Air Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $000F | Intake Air Temperature | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the undefaulted intake air temperature measured by a sensor that is typically integrated with the MAF sensor and located near the throttle body.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### MAFR – PID $0010 – MAF Sensor Mass Airflow

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0010 | MAF Sensor Mass Airflow | 2 | UNM | 0 - 655.35 g/s | E = N * 0.01 |

This PID contains the mass airflow measured by a MAF sensor.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### TPSR – PID $0011 – Absolute Throttle Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0011 | Absolute Throttle Position | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the absolute throttle position (no correction based upon learned minimum position) from throttle position sensor 1.  If the data used to compute absolute throttle position is obtained from a serial data signal, then, if the signal is not received, the value of PID $0011 shall be set to $00 = 0 %.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### AIRR – PID $0012 – Commanded Secondary Air Injection Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0012 | Commanded Secondary Air Injection Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Air Diverted to Atmosphere / Pump Off | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Air Diverted Downstream of First Catalyst | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Air Diverted Upstream of First Catalyst | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains the commanded state of the secondary air injection (SAI) system.  When the SAI system is active, the signal Air Diverted Upstream of First Catalyst shall be set to True, and the other signals within this PID shall be set to False.  When the SAI system is not active, the signal Air Diverted to Atmosphere / Pump Off shall be set to True, and the other signals within this PID shall be set to False.  Note that the signal Air Diverted Downstream of First Catalyst is always set to False.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### OXYR – PID $0013 – Location of Oxygen Sensors

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0013 | Location of Oxygen Sensors | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Bank 2 Sensor 4 Present | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 2 Sensor 3 Present | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 2 Sensor 2 Present | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 2 Sensor 1 Present | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 1 Sensor 4 Present | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 1 Sensor 3 Present | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 1 Sensor 2 Present | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 1 Sensor 1 Present | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID indicates the locations of the set of oxygen sensors that are present on the vehicle.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### OXYR – PID $0014 – Oxygen Sensor Voltage Bank 1 Sensor 1 / Short Term Fuel Trim Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0014 | Oxygen Sensor Voltage Bank 1 Sensor 1 / Short Term Fuel Trim Bank 1 | 2 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Oxygen Sensor Voltage | 0 | 7 | 8 | UNM | 0 - 1.275 V | E = N * 0.005 |
| Short Term Fuel Trim | 1 | 7 | 8 | UNM | -100 - 99.21875 % | E = N * 100 / 128 - 100 |

This PID contains the oxygen sensor voltage for bank 1 sensor 1 and the short term fuel correction based upon feedback from this sensor.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### OXYR – PID $0015 – Oxygen Sensor Voltage Bank 1 Sensor 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0015 | Oxygen Sensor Voltage Bank 1 Sensor 2 | 2 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Oxygen Sensor Voltage | 0 | 7 | 8 | UNM | 0 - 1.275 V | E = N * 0.005 |
| Short Term Fuel Trim | 1 | 7 | 8 | UNM | -100 - 99.21875 % | E = N * 100 / 128 - 100 |

This PID contains the oxygen sensor voltage for bank 1 sensor 2 (the short term fuel trim signal shall always contain its maximum representable value since short term fuel correction is not based upon feedback from this sensor).  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### OXYR – PID $0018 – Oxygen Sensor Voltage Bank 2 Sensor 1 / Short Term Fuel Trim Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0018 | Oxygen Sensor Voltage Bank 2 Sensor 1 / Short Term Fuel Trim Bank 2 | 2 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Oxygen Sensor Voltage | 0 | 7 | 8 | UNM | 0 - 1.275 V | E = N * 0.005 |
| Short Term Fuel Trim | 1 | 7 | 8 | UNM | -100 - 99.21875 % | E = N * 100 / 128 - 100 |

This PID contains the oxygen sensor voltage for bank 2 sensor 1 and the short term fuel correction based upon feedback from this sensor.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### OXYR – PID $0019 – Oxygen Sensor Voltage Bank 2 Sensor 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0019 | Oxygen Sensor Voltage Bank 2 Sensor 2 | 2 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Oxygen Sensor Voltage | 0 | 7 | 8 | UNM | 0 - 1.275 V | E = N * 0.005 |
| Short Term Fuel Trim | 1 | 7 | 8 | UNM | -100 - 99.21875 % | E = N * 100 / 128 - 100 |

This PID contains the oxygen sensor voltage for bank 2 sensor 2 (the short term fuel trim signal shall always contain its maximum representable value since short term fuel correction is not based upon feedback from this sensor).  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### DFIR – PID $001C – On Board Diagnostic System Type

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $001C | On Board Diagnostic System Type | 1 | ENM | N/A | $01=OBD II (CARB); $02=OBD (Federal EPA); $03=OBD and OBD II; $04=OBD I; $05=Not OBD Compliant; $06=EOBD; $07=EOBD and OBD II; $08=EOBD and OBD; $09=EOBD, OBD, and OBD II; $0A=JOBD; $0B=JOBD and OBD II; $0C=JOBD and EOBD; $0D=JOBD, EOBD, and OBD II; $0E=ISO/SAE reserved; $0F=ISO/SAE reserved; $10=ISO/SAE reserved; $11=Engine Manufacturer Diagnostics (EMD); $12=Engine Manufacturer Diagnostics Enhanced (EMD+); $13=Heavy Duty On-Board Diagnostics (Child/Partial); $14=Heavy Duty On-Board Diagnostics; $15=World Wide Harmonized OBD; $16=ISO/SAE reserved; $17=Heavy Duty Euro OBD Stage I without NOx control; $18=Heavy Duty Euro OBD Stage I with NOx control; $19=Heavy Duty Euro OBD Stage II without NOx control; $1A=Heavy Duty Euro OBD Stage II with NOx control; $1B=ISO/SAE reserved; $1C=Brazil OBD Phase 1; $1D=Brazil OBD Phase 2 |

This PID contains the on board diagnostic (OBD) system type required by worldwide OBD regulations.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### PTOR – PID $001E – Power Take Off Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $001E | Power Take Off Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Power Take Off Active | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains an indication that power take off (PTO) is active.  If the data used to compute the power take off status is obtained from a serial data signal, then, if the signal is not received, the value of PID $001E shall be set to 0000 0000 (binary) = False for bit 0.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EPSR – PID $001F – Engine Run Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $001F | Engine Run Time | 2 | UNM | 0 - 65535 s | E = N * 1 |

This PID contains the time that the engine has been running.  The value of this PID shall freeze if an engine stall occurs and shall be reset to zero at power-up (Run/Crank transition from low to high).  If the data used to compute engine run time is obtained from a serial data signal, then, if the signal is not received, the value of PID $001F shall be set to $FF = 65535 s.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### ODAR – PID $0020 – Parameter Identifiers Supported - $21 to $40

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0020 | Parameter Identifiers Supported - $21 to $40 | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| PID $21 Supported | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $22 Supported | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $23 Supported | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $24 Supported | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $25 Supported | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $26 Supported | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $27 Supported | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $28 Supported | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $29 Supported | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $2A Supported | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $2B Supported | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $2C Supported | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $2D Supported | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $2E Supported | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $2F Supported | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $30 Supported | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $31 Supported | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $32 Supported | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $33 Supported | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $34 Supported | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $35 Supported | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $36 Supported | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $37 Supported | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $38 Supported | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $39 Supported | 3 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $3A Supported | 3 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $3B Supported | 3 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $3C Supported | 3 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $3D Supported | 3 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $3E Supported | 3 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $3F Supported | 3 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $40 Supported | 3 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID indicates which legislated PIDs, within the range of $21 to $40, are supported.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### VSPR – PID $0021 – Distance With Malfunction Indicator Lamp On

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0021 | Distance With Malfunction Indicator Lamp On | 2 | UNM | 0 - 65535 km | E = N * 1 |

This PID contains the accumulated distance driven with the malfunction indicator lamp (MIL) on.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EGRR – PID $002C – Exhaust Gas Recirculation Desired Pintle Lift

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $002C | Exhaust Gas Recirculation Desired Pintle Lift | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the exhaust gas recirculation (EGR) desired pintle lift as a percentage of maximum pintle lift.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EGRR – PID $002D – Exhaust Gas Recirculation Error

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $002D | Exhaust Gas Recirculation Error | 1 | UNM | -100 - 99.21875 % | E = N * 100 / 128 - 100 |

This PID contains the exhaust gas recirculation (EGR) error, the difference between the actual and desired EGR pintle lift, as a percentage of the desired EGR pintle lift.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EVPR – PID $002E – Canister Purge Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $002E | Canister Purge Duty Cycle | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the commanded duty cycle for the canister purge output.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### FLVR – PID $002F – Total Fuel Level Percentage

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $002F | Total Fuel Level Percentage | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the undefaulted, total fuel level as a percentage of the rated (advertised) capacity of the fuel tank(s).  If the data used to compute fuel level is obtained from a serial data signal, then, if the signal is not received, the value of PID $002F shall be set to $00=0 %.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### DRER – PID $0030 – Warm-Up Cycles Since Code Clear

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0030 | Warm-Up Cycles Since Code Clear | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of OBD defined warm-up cycles since the last time codes were cleared with a scan tool or, when applicable, with a battery disconnect.  Once this counter reaches its maximum value, it will stop counting and will remain at its maximum value until codes are cleared.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### VSPR – PID $0031 – Distance Since Code Clear

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0031 | Distance Since Code Clear | 2 | UNM | 0 - 65535 km | E = N * 1 |

This PID contains the accumulated distance driven since the last code clear or memory reset.  Note that this distance is not reset by a natural code clear (i.e., 40 warm up cycles without a failure being reported).  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EVPR – PID $0032 – Fuel Tank Vapor Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0032 | Fuel Tank Vapor Pressure | 2 | SNM | -8192 - 8191.75 Pa | E = N * 0.25 |

This PID contains the undefaulted fuel tank vapor pressure measured by a sensor.  If the data used to compute fuel tank vapor pressure is obtained from a serial data signal, then if the signal is not received, the value of PID $0032 shall be set to $8000 = -8192 Pa.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### AAPR – PID $0033 – Barometric Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0033 | Barometric Pressure | 1 | UNM | 0 - 255 kPa | E = N * 1 |

This PID contains the ambient air (barometric) pressure.  Ambient air pressure is determined directly from a barometric pressure sensor in applications where this sensor is available and is estimated (as a function of MAP and other parameters) in all other applications.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EGTR – PID $003C – Catalyst Temperature Bank 1 Sensor 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $003C | Catalyst Temperature Bank 1 Sensor 1 | 2 | UNM | -40 - 6513.5 °C | E = N * 0.1 - 40 |

This PID contains the estimated catalyst temperature for bank 1 sensor 1 (as a function of engine speed, engine airflow, and fuel ethanol percentage) for use in the oxygen sensor and catalyst monitor diagnostic algorithms.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EGTR – PID $003D – Catalyst Temperature Bank 2 Sensor 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $003D | Catalyst Temperature Bank 2 Sensor 1 | 2 | UNM | -40 - 6513.5 °C | E = N * 0.1 - 40 |

This PID contains the estimated catalyst temperature for bank 2 sensor 1 (as a function of engine speed, engine airflow, and fuel ethanol percentage) for use in the oxygen sensor and catalyst monitor diagnostic algorithms.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### ODAR – PID $0040 – Parameter Identifiers Supported - $41 to $60

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0040 | Parameter Identifiers Supported - $41 to $60 | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| PID $41 Supported | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $42 Supported | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $43 Supported | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $44 Supported | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $45 Supported | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $46 Supported | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $47 Supported | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $48 Supported | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $49 Supported | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $4A Supported | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $4B Supported | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $4C Supported | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $4D Supported | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $4E Supported | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $4F Supported | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $50 Supported | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $51 Supported | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $52 Supported | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $53 Supported | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $54 Supported | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $55 Supported | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $56 Supported | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $57 Supported | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $58 Supported | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $59 Supported | 3 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $5A Supported | 3 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $5B Supported | 3 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $5C Supported | 3 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $5D Supported | 3 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $5E Supported | 3 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $5F Supported | 3 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $60 Supported | 3 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID indicates which legislated PIDs, within the range of $41 to $60, are supported.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### DFIR – PID $0041 – Emission Related Monitor Status This Driving Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0041 | Emission Related Monitor Status This Driving Cycle | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 8 |  |  |  |
| Reserved | 1 | 7 | 1 |  |  |  |
| Comprehensive Component Monitoring Status This Cycle | 1 | 6 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Fuel System Monitoring Status This Cycle | 1 | 5 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Misfire Monitoring Status This Cycle | 1 | 4 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Reserved | 1 | 3 | 1 |  |  |  |
| Comprehensive Component Monitoring Enabled For This Cycle | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Fuel System Monitoring Enabled For This Cycle | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Misfire Monitoring Enabled For This Cycle | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Gas Recirculation and/or Variable Valve Timing System Monitoring Enabled For This Cycle | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Heater Monitoring Enabled For This Cycle | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Monitoring Enabled For This Cycle | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Air Conditioning System Refrigerant Monitoring Enabled For This Cycle | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Secondary Air System Monitoring Enabled For This Cycle | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Evaporative System Monitoring Enabled For This Cycle | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Heated Catalyst Monitoring Enabled For This Cycle | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Catalyst Monitoring Enabled For This Cycle | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Gas Recirculation and/or Variable Valve Timing System Monitoring Status This Cycle | 3 | 7 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Oxygen Sensor Heater Monitoring Status This Cycle | 3 | 6 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Oxygen Sensor Monitoring Status This Cycle | 3 | 5 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Air Conditioning System Refrigerant Monitoring Status This Cycle | 3 | 4 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Secondary Air System Monitoring Status This Cycle | 3 | 3 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Evaporative System Monitoring Status This Cycle | 3 | 2 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Heated Catalyst Monitoring Status This Cycle | 3 | 1 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |
| Catalyst Monitoring Status This Cycle | 3 | 0 | 1 | ENM | N/A | $0=Monitor Complete; $1=Monitor Not Complete |

This PID contains the enable and completion status during the current driving / monitoring cycle of each continuous legislated emission related monitor (comprehensive component, fuel system, and misfire) and non-continuous legislated emission related monitor (exhaust gas recirculation system, oxygen sensor heater, oxygen sensor, air conditioning system refrigerant, secondary air system, evaporative system, heated catalyst, catalyst).  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### PMDR – PID $0042 – Control Module Voltage

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0042 | Control Module Voltage | 2 | UNM | 0 - 65.535 V | E = N * 0.001 |

This PID contains the voltage of the power input to the control module.  Since battery voltage is not directly measured by powertrain controllers, switched battery voltage, available from the Run/Crank input to the controller, shall be used to populate this PID.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### MAFR – PID $0043 – Absolute Engine Load

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0043 | Absolute Engine Load | 2 | UNM | 0 - 25700 % | E = N * 100 / 255 |

This PID contains the absolute engine load, air mass per cylinder as a percentage of the maximum air mass per cylinder, where the maximum air mass per cylinder is determined at wide open throttle (WOT), standard temperature and pressure (STP), and 100% volumetric efficiency.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### FULR – PID $0044 – Equivalence Ratio

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0044 | Equivalence Ratio | 2 | UNM | 0 - 1.999969482422 | E = N / 32768 |

This PID contains the commanded equivalence ratio.  The equivalence ratio should be reported as 1.0 when PID $0003 indicates that the vehicle is closed loop.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### TPSR – PID $0045 – Relative Throttle Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0045 | Relative Throttle Position | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the relative throttle position (corrected based upon its learned minimum position) from throttle position sensor 1.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EVPR – PID $0046 – Ambient Air Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0046 | Ambient Air Temperature | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID shall display the estimated ambient air temperature.  The variable used for the estimated ambient air temperature shall be the variable that estimates the ambient air temperature by using minimum intake air temperature sampled at a calibrated number of counts.  If the data used to compute outside air temperature is obtained from a serial data signal, then, if the signal is not received, the value of PID $0046 shall be set to $00 = -40 °C.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### TPSR – PID $0047 – Absolute Throttle Position B

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0047 | Absolute Throttle Position B | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the absolute throttle position (no correction based upon learned minimum position) from throttle position sensor 2.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### APSR – PID $0049 – Accelerator Pedal Position D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0049 | Accelerator Pedal Position D | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the absolute accelerator pedal position (no correction or normalization based upon learned minimum or maximum positions) from accelerator pedal position sensor 1.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### APSR – PID $004A – Accelerator Pedal Position E

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $004A | Accelerator Pedal Position E | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the absolute accelerator pedal position (no correction or normalization based upon learned minimum or maximum positions) from accelerator pedal position sensor 2.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### TPSR – PID $004C – Commanded Throttle Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $004C | Commanded Throttle Position | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the commanded throttle position used to control the throttle motor, which is determined from the arbitration of the various requests for the authority to control the throttle, including remedial actions.  A value of 0% indicates that closed throttle is commanded, and a value of 100% indicates that wide open throttle (WOT) is commanded.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### FULR – PID $0051 – Type of fuel currently being utilized by the vehicle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0051 | Type of fuel currently being utilized by the vehicle | 1 | ENM | N/A | $01= Gasoline/petrol; $02= Methanol; $03= Ethanol; $04= Diesel; $05= Liquefied Petroleum Gas (LPG); $06= Compressed Natural Gas (CNG); $07= Propane; $08= Battery/electric; $09= Bi-fuel vehicle using gasoline; $0A= Bi-fuel vehicle using methanol; $0B= Bi-fuel vehicle using ethanol; $0C= Bi-fuel vehicle using LPG; $0D= Bi-fuel vehicle using CNG; $0E= Bi-fuel vehicle using propane; $0F= Bi-fuel vehicle using battery; $10-$FF= ISO/SAE reserved |

This PID contains the type of fuel currently being utilized by the vehicle.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### E85R – PID $0052 – Alcohol Fuel Percentage

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0052 | Alcohol Fuel Percentage | 1 | UNM | 0 to 99.609375 % | E = N * 100 / 256 |

PID $0052 shall indicate the percentage of alcohol contained in ethanol or methanol fuels, if utilized. Alcohol percentage can be determined using a sensor or can be inferred by the fuel control software.  If alcohol percentage is determined using a sensor, PID $0052 must display the undefaulted representation of that sensor.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### ODAR – PID $0060 – Parameter Identifiers Supported - $61 to $80

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0060 | Parameter Identifiers Supported - $61 to $80 | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| PID $61 Supported | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $62 Supported | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $63 Supported | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $64 Supported | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $65 Supported | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $66 Supported | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $67 Supported | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $68 Supported | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $69 Supported | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $6A Supported | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $6B Supported | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $6C Supported | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $6D Supported | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $6E Supported | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $6F Supported | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $70 Supported | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $71 Supported | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $72 Supported | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $73 Supported | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $74 Supported | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $75 Supported | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $76 Supported | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $77 Supported | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $78 Supported | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $79 Supported | 3 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $7A Supported | 3 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $7B Supported | 3 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $7C Supported | 3 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $7D Supported | 3 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $7E Supported | 3 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $7F Supported | 3 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $80 Supported | 3 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID indicates which legislated PIDs, within the range of $61 to $80, are supported.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EITR – PID $0068 – Intake Air Temperature Sensor

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0068 | Intake Air Temperature Sensor | 7 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| IAT Bank 2, Sensor 3 supported | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| IAT Bank 2, Sensor 2 supported | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| IAT Bank 2, Sensor 1 supported | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| IAT Bank 1, Sensor 3 supported | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| IAT Bank 1, Sensor 2 supported | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| IAT Bank 1, Sensor 1 supported | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Intake Air Temperature Bank 1, Sensor 1 | 1 | 7 | 8 | UNM | -40 to 215 °C | E = N * 1 - 40 |
| Intake Air Temperature Bank 1, Sensor 2 | 2 | 7 | 8 | UNM | -40 to 215 °C | E = N * 1 - 40 |
| Intake Air Temperature Bank 1, Sensor 3 | 3 | 7 | 8 | UNM | -40 to 215 °C | E = N * 1 - 40 |
| Intake Air Temperature Bank 2, Sensor 1 | 4 | 7 | 8 | UNM | -40 to 215 °C | E = N * 1 - 40 |
| Intake Air Temperature Bank 2, Sensor 2 | 5 | 7 | 8 | UNM | -40 to 215 °C | E = N * 1 - 40 |
| Intake Air Temperature Bank 2, Sensor 3 | 6 | 7 | 8 | UNM | -40 to 215 °C | E = N * 1 - 40 |

This PID contains the undefaulted intake air temperature measured by up to six sensors as follows.

IAT Bank 1, Sensor 1 shall display intake air temperature, if utilized by the control module strategy.  IAT may be obtained directly from a sensor, or may be inferred by the control strategy using other sensor inputs.

IAT Bank 1, Sensor 2 shall display intake air temperature, if utilized by the control module strategy.

IAT Bank 1, Sensor 3 shall display intake air temperature, if utilized by the control module strategy.

IAT Bank 2, Sensor 1 shall display intake air temperature, if utilized by the control module strategy.

IAT Bank 2, Sensor 2 shall display intake air temperature, if utilized by the control module strategy.

IAT Bank 2, Sensor 3 shall display intake air temperature, if utilized by the control module strategy.

Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### EPSR – PID $007F – Engine Run Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $007F | Engine Run Time | 13 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Total Run Time With PTO Active supported | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Total Idle Run Time supported | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Total Engine Run Time supported | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Total Engine Run Time | 1 | 7 | 32 | UNM | 0 to 4,294,967,295 s | E = N * 1 |
| Total Idle Run Time | 5 | 7 | 32 | UNM | 0 to 4,294,967,295 s | E = N * 1 |
| Total Run Time With PTO Active | 9 | 7 | 32 | UNM | 0 to 4,294,967,295 s | E = N * 1 |

This PID contains information for total engine run time, run time at idle, and run time with PTO active.  Each timer is described below.

Total Engine Run Time shall display the total engine run time for the life of vehicle.  Total Engine Run Time shall increment while the engine is running. It shall freeze if the engine stalls.  Total Engine Run Time shall never be reset to zero.

Total Idle Run Time shall display the total engine idle time for the life of vehicle.  Total Idle Run Time shall increment while the engine is running at closed throttle/closed pedal and vehicle speed is less than 5 kph.  It shall freeze if the engine stalls or the engine is no longer at idle.  Total Idle Run Time shall never be reset to zero.

Total Run Time With PTO Active shall display the total engine run time with PTO engaged for the life of vehicle.  Total Run Time With PTO Active shall increment while the engine is running with PTO engaged.  It shall freeze if the engine stalls.  Total Run Time With PTO Active shall never be reset to zero.

Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### ODAR – PID $0080 – Parameter Identifiers Supported - $81 to $A0

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $0080 | Parameter Identifiers Supported - $81 to $A0 | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| PID $81 Supported | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $82 Supported | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $83 Supported | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $84 Supported | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $85 Supported | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $86 Supported | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $87 Supported | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $88 Supported | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $89 Supported | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $8A Supported | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $8B Supported | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $8C Supported | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $8D Supported | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $8E Supported | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $8F Supported | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $90 Supported | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $91 Supported | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $92 Supported | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $93 Supported | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $94 Supported | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $95 Supported | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $96 Supported | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $97 Supported | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $98 Supported | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $99 Supported | 3 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $9A Supported | 3 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $9B Supported | 3 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $9C Supported | 3 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $9D Supported | 3 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $9E Supported | 3 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $9F Supported | 3 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $A0 Supported | 3 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID indicates which legislated PIDs, within the range of $81 to $A0, are supported.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### ODAR – PID $00A0 – Parameter Identifiers Supported - $A1 to $C0

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $00A0 | Parameter Identifiers Supported - $A1 to $C0 | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| PID $A1 Supported | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $A2 Supported | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $A3 Supported | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $A4 Supported | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $A5 Supported | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $A6 Supported | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $A7 Supported | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $A8 Supported | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $A9 Supported | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $AA Supported | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $AB Supported | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $AC Supported | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $AD Supported | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $AE Supported | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $AF Supported | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $B0 Supported | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $B1 Supported | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $B2 Supported | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $B3 Supported | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $B4 Supported | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $B5 Supported | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $B6 Supported | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $B7 Supported | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $B8 Supported | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $B9 Supported | 3 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $BA Supported | 3 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $BB Supported | 3 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $BC Supported | 3 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $BD Supported | 3 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $BE Supported | 3 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $BF Supported | 3 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $C0 Supported | 3 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID indicates which legislated PIDs, within the range of $A1 to $C0, are supported.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### ODAR – PID $00C0 – Parameter Identifiers Supported - $C1 to $E0

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $00C0 | Parameter Identifiers Supported - $C1 to $E0 | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| PID $C1 Supported | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $C2 Supported | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $C3 Supported | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $C4 Supported | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $C5 Supported | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $C6 Supported | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $C7 Supported | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $C8 Supported | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $C9 Supported | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $CA Supported | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $CB Supported | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $CC Supported | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $CD Supported | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $CE Supported | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $CF Supported | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $D0 Supported | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $D1 Supported | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $D2 Supported | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $D3 Supported | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $D4 Supported | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $D5 Supported | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $D6 Supported | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $D7 Supported | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $D8 Supported | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $D9 Supported | 3 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $DA Supported | 3 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $DB Supported | 3 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $DC Supported | 3 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $DD Supported | 3 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $DE Supported | 3 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $DF Supported | 3 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $E0 Supported | 3 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID indicates which legislated PIDs, within the range of $C1 to $E0, are supported.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

### ODAR – PID $00E0 – Parameter Identifiers Supported - $E1 to $FF

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $00E0 | Parameter Identifiers Supported - $E1 to $FF | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| PID $E1 Supported | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $E2 Supported | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $E3 Supported | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $E4 Supported | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $E5 Supported | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $E6 Supported | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $E7 Supported | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $E8 Supported | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $E9 Supported | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $EA Supported | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $EB Supported | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $EC Supported | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $ED Supported | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $EE Supported | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $EF Supported | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $F0 Supported | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $F1 Supported | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $F2 Supported | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $F3 Supported | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $F4 Supported | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $F5 Supported | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $F6 Supported | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $F7 Supported | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PID $F8 Supported | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| PID $F9 Supported | 3 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PID $FA Supported | 3 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PID $FB Supported | 3 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PID $FC Supported | 3 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PID $FD Supported | 3 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PID $FE Supported | 3 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PID $FF Supported | 3 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 3 | 0 | 1 |  |  |  |

This PID indicates which legislated PIDs, within the range of $E1 to $FF, are supported.  Additional PID information and usage examples are detailed in SAE J1979 and ISO 15031-5.

## Corporate Standard PIDs

The corporate standard PIDs below are also specified within [5].  In the event of a conflict between this document and [5], the former shall take precedence.

### TPSR – PID $1121 – Electronic Throttle Control Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1121 | Electronic Throttle Control Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Throttle Position Sensors 1 and 2 Disagree | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Accelerator Pedal Position Sensors 1 and 2 Disagree | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Reduced Engine Power | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Stall Forced | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Default Throttle Authority | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Wide Open Throttle Commanded | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Wide Open Throttle Self Test Failed | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Wide Open Throttle Self Test Complete | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the electronic throttle control system.  The content of each of these signals is described below.

Throttle Position Sensors 1 and 2 Disagree indicates that the (corrected) throttle positions 1 and 2 are currently out of correlation, that a fault has been detected (and latched) based upon the (corrected) throttle positions 1 and 2 being out of correlation, or that a fault has been detected (and latched) based upon the detection of a short between TPS 1 and 2.

Accelerator Pedal Position Sensors 1 and 2 Disagree indicates that the (corrected) accelerator pedal positions 1 and 2 are currently out of correlation, that a fault has been detected (and latched) based upon the (corrected) accelerator pedal positions 1 and 2 being out of correlation, or that a fault has been detected (and latched) based upon the detection of a short between PPS 1 and 2.

Reduced Engine Power indicates that engine power is being limited due to an ETC related failure and that the reduced power indication is on.

Engine Stall Forced indicates that the engine is being or has been shutdown (i.e., forced to stall) due to an ETC related failure and that the starting disabled indication is on.

Default Throttle Authority indicates that power has been removed from the throttle actuator, resulting in default / pop-up throttle authority, due to an inability to actuate the throttle appropriately.

Wide Open Throttle Commanded indicates that the desired throttle position exceeds a WOT threshold (selected specifically for service).

Wide Open Throttle Self Test Failed indicates that the WOT self test device control has failed due to the throttle position failing to reach a WOT threshold within a calibratable duration.

Wide Open Throttle Self Test Complete indicates that the forced idle and WOT self test device controls have completed.

### FANR – PID $1122 – Cooling Fan Control Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1122 | Cooling Fan Control Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fan Relay 1 On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Fan Relay 2 On | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Fan Relay 3 On | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the cooling fan control system.  This PID is only applicable to discrete cooling fan electrical architectures.  The content of each of these signals is described below.

Fan Relay 1 On, Fan Relay 2 On, and Fan Relay 3 On indicate that fan relays 1, 2, and 3, respectively, are commanded on.  For single speed (Off / On) discrete cooling fan electrical architectures, a value of False will always be provided for the signals Fan Relay 2 On and Fan Relay 3 On.  For two speed (Off / Low / High) and three speed (Off / Low / Medium / High) discrete cooling fan electrical architectures, a value of False will always be provided for the signal Fan Relay 3 On.  Up to seven speeds may be supported when all three fan relay outputs are used.  In cooling fan electrical architectures from PPEI 3.x or later, only two fan relay outputs are supported, and a value of False will always be provided for the signal Fan Relay 3 On.

### ACCR – PID $1123 – Air Conditioning Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1123 | Air Conditioning Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Air Conditioning Compressor On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Air Conditioning Compressor Wide Open Throttle Disable | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Air Conditioning Compressor Pressure Disable | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Air Conditioning Request On | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Air Conditioning High Side Pressure Switch Active | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Air Conditioning Low Side Pressure Cycling Switch Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Air Conditioning Recirculation On | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the air conditioning (A/C) system.  The content of each of these signals is described below.

Air Conditioning Compressor On indicates that the A/C compressor output is commanded on.

Air Conditioning Compressor Wide Open Throttle Disable indicates that the A/C compressor is being disabled during a wide open throttle maneuver in order to attain increased vehicle performance.

Air Conditioning Compressor Pressure Disable indicates that the A/C compressor is being disabled due to an A/C high side pressure condition in order to protect the A/C compressor.

Air Conditioning Request On indicates that the A/C request received over serial communication from the platform electronics is requesting the A/C compressor to be commanded on.  In applications using an A/C electrical architecture that precedes any version of PPEI, this signal indicates that the A/C request hardwire input to the controller is active.

Air Conditioning High Side Pressure Switch Active indicates that the A/C high side pressure switch hardwire input to the controller is active.  The A/C high side pressure switch hardwire input is only supported in A/C electrical architectures that precede any version of PPEI.  A value of False will always be provided for this signal in all other A/C electrical architectures.

Air Conditioning Low Side Pressure Cycling Switch Active indicates that the A/C low side pressure cycling switch hardwire input to the controller is active.  The A/C low side pressure cycling switch hardwire input is only supported in A/C electrical architectures that precede any version of PPEI.  A value of False will always be provided for this signal in all other A/C electrical architectures.

Air Conditioning Recirculation On indicates that the A/C recirculation hardwire output from the controller is commanded on.  The A/C recirculation hardwire output is only supported in A/C electrical architectures that precede any version of PPEI.  A value of False will always be provided for this signal in all other A/C electrical architectures.

### BRKR – PID $1124 – Brake Pedal Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1124 | Brake Pedal Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Brake Pedal Initial Travel Achieved | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Brake Pedal Initial Travel Achieved Valid | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Brake Pedal Moderate Travel Achieved | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Brake Pedal Moderate Travel Achieved Valid | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Brake Pedal Apply Active | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Cruise/ETC/TCC Brake Switch Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Brake Extended Travel Switch Active | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the brake pedal system.  The content of each of these signals is described below.

Brake Pedal Initial Travel Achieved indicates the undefaulted determination of brake pedal travel beyond a top-of-travel threshold by the platform brake sensing module and is received over serial communication.  In brake apply sensing system (BASS) brake pedal electrical architectures, this signal is based upon the brake pedal position passing a top-of-travel threshold.  In discrete brake pedal electrical architectures from PPEI 3.x or later, this signal is based upon the state of the redundant brake switch.  A value of False will always be provided for this signal in applications using other brake pedal electrical architectures.

Brake Pedal Initial Travel Achieved Valid indicates that no failures have been detected in the determination of the Brake Pedal Initial Travel Achieved signal within the platform brake sensing module as received over serial communication.  In BASS brake pedal electrical architectures, this signal is based upon diagnosis of the brake pedal position sensor.  In discrete brake pedal electrical architectures from PPEI 3.x or later, this signal is based upon diagnosis of the redundant brake switch.  A value of True will always be provided for this signal in applications using other brake pedal electrical architectures.

Brake Pedal Moderate Travel Achieved indicates the undefaulted determination of brake pedal travel beyond a moderate / extended travel threshold by the platform brake sensing module and is received over serial communication.  In BASS brake pedal electrical architectures, this signal is based upon the brake pedal position passing a moderate travel threshold.  In discrete brake pedal electrical architectures from PPEI 3.x or later with the optional moderate travel brake switch, this signal is based upon the state of the moderate travel brake switch.  A value of False will always be provided for this signal in applications using other brake pedal electrical architectures.

Brake Pedal Moderate Travel Achieved Valid indicates that no failures have been detected in the determination of the Brake Pedal Moderate Travel Achieved signal within the platform brake sensing module as received over serial communication.  In BASS brake pedal electrical architectures, this signal is based upon diagnosis of the brake pedal position sensor.  In discrete brake pedal electrical architectures from PPEI 3.x or later with the optional moderate travel brake switch, this signal is based upon diagnosis of the moderate travel brake switch.  A value of True will always be provided for this signal in applications using other brake pedal electrical architectures.

Brake Pedal Apply Active indicates that the undebounced / undefaulted state of the discrete brake pedal apply input to the controller indicates that the brake is applied.  In BASS brake pedal electrical architectures, this signal is generated by the platform brake sensing module.  In discrete brake pedal electrical architectures, this signal is generated by the brake lamp switch.

Cruise/ETC/TCC Brake Switch Active indicates that the undebounced / undefaulted state of the redundant discrete brake switch input to the controller indicates that the brake is applied.  The redundant discrete brake switch hardwire input is only supported in discrete brake pedal electrical architectures that precede PPEI 3.x.  A value of False will always be provided for this signal in all other brake pedal electrical architectures.

Brake Extended Travel Switch Active indicates that the undebounced / undefaulted state of the extended travel brake switch input to the ECM indicates that the brake is applied beyond a moderate / extended travel threshold.  The extended travel brake switch hardwire input is only supported in discrete brake pedal electrical architectures that precede PPEI 3.x, within which it is optional.  A value of False will always be provided for this signal when the extended travel brake switch input is not present.

### TRNR – PID $1125 – Manual Transmission Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1125 | Manual Transmission Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reverse Inhibit Active | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Manual Transmission Reverse Switch Active | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Upshift Indication On | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Skip Shift Indication On | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Skip Shift Solenoid On | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Clutch Pedal Top of Travel Achieved | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Clutch Pedal Bottom of Travel Achieved | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the manual transmission system.  The content of each of these signals is described below.

Reverse Inhibit Active indicates that the driver is inhibited from shifting into reverse.  A value of False will always be provided for this signal when the reverse inhibit solenoid is not present.

Manual Transmission Reverse Switch On indicates that the reverse switch hardwire input to the ECM is active.  The reverse switch hardwire input is only supported in manual transmission electrical architectures from PPEI 3.x or later.  A value of False will always be provided for this signal in applications using a manual transmission electrical architecture that precedes PPEI 3.x.

Upshift Indication On indicates that the transmission upshift indication is being requested on over serial communication in order to recommend that the driver shift up to a higher gear to improve fuel economy.

Skip Shift Indication On indicates that the transmission skip shift indication is being requested on over serial communication in order to inform the driver that shifting up to one of the next highest gears is inhibited (i.e., the driver is forced to skip one or more gears when performing an upshift).  A value of False will always be provided for this signal when the skip shift solenoid is not present.

Skip Shift Solenoid On indicates that the skip shift solenoid is commanded on, inhibiting the driver from shifting up to one of the next highest gears.  A value of False will always be provided for this signal when the skip shift solenoid is not present.

Clutch Pedal Top of Travel Achieved indicates the undefaulted determination of clutch pedal travel beyond a top of travel threshold.  In clutch apply sensing system (CASS) electrical architectures, this signal is based upon the clutch pedal position passing a top of travel threshold.  In discrete clutch switch electrical architectures, this signal is based upon the state of the top of travel clutch switch.

Clutch Pedal Bottom of Travel Achieved indicates the undefaulted determination of clutch pedal travel beyond a bottom of travel threshold.  In CASS electrical architectures, this signal is based upon the clutch pedal position passing a bottom of travel threshold.  In discrete clutch switch electrical architectures, this signal is based upon the state of the clutch start switch.

### OXYR – PID $1126 – Oxygen Sensor Heater Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1126 | Oxygen Sensor Heater Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Oxygen Sensor Heater On Bank 1 Sensor 1 | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Heater On Bank 2 Sensor 1 | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Heater On Bank 1 Sensor 2 | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Heater On Bank 2 Sensor 2 | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the oxygen sensor heater system.  The content of each of these signals is described below.

Oxygen Sensor Heater On Bank 1 Sensor 1, Oxygen Sensor Heater On Bank 2 Sensor 1, Oxygen Sensor Heater On Bank 1 Sensor 2, and Oxygen Sensor Heater On Bank 2 Sensor 2 indicate that the oxygen sensor heaters associated with the indicated banks and sensors are commanded on (for PWM controlled heaters, a heater is considered to be commanded on if its duty cycle is not zero).  Values of False will always be provided for these signals when the oxygen sensor heaters associated with the indicated banks and sensors are not present.

### EODR – PID $1127 – Engine Oil Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1127 | Engine Oil Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Engine Oil Level Low | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Oil Level Low Debounced | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Oil Level Low Indication On | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Oil Pressure Low | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Oil Pressure Low Debounced | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Oil Pressure Low Indication On | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Oil Change Indication On | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the engine oil system.  The content of each of these signals is described below.

Engine Oil Level Low indicates that the undebounced state of the engine oil level switch input to the ECM indicates a low engine oil level condition.

Engine Oil Level Low Debounced indicates that the debounced state of the engine oil level switch input to the ECM indicates a low engine oil level condition.

Engine Oil Level Low Indication On indicates that the low engine oil level indication is being requested on over serial communication in order to inform the driver of a low engine oil level condition.

Engine Oil Pressure Low indicates that the undebounced state of the engine oil pressure switch input to the ECM indicates a low engine oil pressure condition.  A value of False will always be provided for this signal when the engine oil pressure switch is not present (i.e., in applications that determine low engine oil pressure based upon the engine oil pressure sensor).

Engine Oil Pressure Low Debounced indicates that the debounced state of the engine oil pressure switch input to the ECM indicates a low engine oil pressure condition.  A value of False will always be provided for this signal when the engine oil pressure switch is not present (i.e., in applications that determine low engine oil pressure based upon the engine oil pressure sensor).

Engine Oil Pressure Low Indication On indicates that the low engine oil pressure indication is being requested on over serial communication in order to inform the driver of a low engine oil pressure condition.  This signal is based upon either the engine oil pressure switch or the engine oil pressure sensor.

Engine Oil Change Indication On indicates that the change engine oil indication is being requested on over serial communication in order to inform the driver that the engine oil needs to be changed.  This signal is based upon either the engine oil life algorithm or the engine oil condition sensor.

### EVPR – PID $1128 – Evaporative Emissions Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1128 | Evaporative Emissions Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Canister Vent Open | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Cold Start Conditions Met | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Volatility Level | 0 | 1 | 2 | ENM | N/A | $0=Low; $1=Moderate; $2=High |

This PID contains several signals associated with the status of the evaporative emissions system.  The content of each of these signals is described below.

Canister Vent Open indicates that the canister vent is commanded open.

Cold Start Conditions Met indicates that the conditions used to identify a cold start (based on the difference between the intake air temperature and engine coolant temperature at engine start) have been met.

Fuel Volatility Level contains the classification of the volatility of the fuel based upon the integration of the fuel tank pressure with the canister vent open.

### FWDR – PID $1129 – Four Wheel Drive Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1129 | Four Wheel Drive Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Four Wheel Drive Low Active | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Four Wheel Drive Low Fault | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Four Wheel Drive Mode Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Rear Axle Low Active | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the four wheel drive system.  The content of each of these signals is described below.

Four Wheel Drive Low Active indicates that the transfer case is in its low range in applications that are equipped with a transfer case capable of providing range selection (a value of False will be provided for this signal in all other applications).  In manual transfer case applications, where a transfer case control module (TCCM) is not present, this signal is based upon the four wheel drive low hardwire input from the transfer case.  In applications with a TCCM that precede PPEI 3.x, this signal is based upon the four wheel drive low hardwire input from the TCCM.  In applications with a TCCM from PPEI 3.x or later, this signal is received over serial communication from the TCCM.

Four Wheel Drive Low Fault indicates that a failure in the determination of the transfer case range has been detected (and a corresponding DTC has been stored) within the TCCM in applications that are equipped with a transfer case capable of providing range selection (a value of False will be provided for this signal in all other applications).  In applications with a TCCM from PPEI 3.x or later, this signal is based upon failure information received over serial communication from the TCCM.  A value of False will be provided for this signal in all other applications.

Four Wheel Drive Mode Active indicates that the secondary axle is engaged in applications that are equipped with part time four wheel drive (a value of False will be provided for this signal in all other applications, including all wheel drive and full-time four wheel drive applications).  In manual transfer case applications, this signal is based upon the secondary axle feedback (four wheel drive mode) hardwire input from the front axle switch.  In applications with a TCCM, this signal is received over serial communication from the TCCM.

Rear Axle Low Active indicates that the rear axle is in its low range in applications that are equipped with a two speed rear axle (a value of False will be provided for this signal in all other applications).

### ENGR – PID $1130 – Power Mode / Starter Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1130 | Power Mode / Starter Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Crank Request Input Active | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Accessory Input Active | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Powertrain Relay On | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Starter Relay On | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Remote Vehicle Start Request | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Power Take Off Remote Engine Start Request | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Automatic Transmission Park/Neutral Input Active | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the power mode and starter systems.  The content of each of these signals is described below.

Crank Request Input Active indicates that either an existing hardwire is active or a serial data Crank Request is received.

Accessory Input Active indicates that the accessory hardwire input to the controller is active in applications using a power mode electrical architecture from PPEI 2.x or later.  In applications using a power mode electrical architecture that precedes PPEI 2.x, this signal indicates that the unlock hardwire input to the controller is active.

Powertrain Relay On indicates that the powertrain relay output driver from the ECM is commanded on.

Starter Relay On indicates the current commanded state of the starter relay.

Remote Vehicle Start Request indicates the state of the request received over serial communication from the remote vehicle start (RVS) system to start the vehicle in remote mode.

Power Take Off Remote Engine Start Request indicates the state of the request received over serial communication from the power take off (PTO) system to start the engine in remote mode.

The Automatic Transmission Park/Neutral Input Active signal is a normally open switch that is connected to ground when the transmission is in Park or Neutral.  The ECM shall read this signal with an input that is pulled up to the ignition voltage.  This switch input is provided to ensure that the engine can be started safely regardless of the communications status with the TCM. This switch input is the source of the transmission range determination for the starter control on IMS equipped transmissions.

### ENGR – PID $1131 – Fuel System Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1131 | Fuel System Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Trim Learn Enabled | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Closed Loop Active | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Power Enrichment Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Hot Coolant Enrichment Active | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Decel Fuel Cut Off Active | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Pump On | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Secondary Fuel Pump On | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Enrichment Catalyst Protection Active | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the fuel system.  The content of each of these signals is described below.

Fuel Trim Learn Enabled indicates that proper conditions exist to learn long term fuel corrections.

Fuel Closed Loop Active indicates that oxygen sensor 1 of each bank is being used as feedback to adjust the amount of fuel delivered to the engine.

Power Enrichment Active indicates that the conditions to enable power enrichment are active and that a richer than stoichiometric air/fuel ratio is being commanded in order to maximize power output.

Hot Coolant Enrichment Active indicates that the conditions to enable hot coolant enrichment are active and that a richer than stoichiometric air/fuel ratio is being commanded in order to help cool the engine when a system malfunction is causing the engine to run at extreme temperatures.

Decel Fuel Cut Off Active indicates that the conditions to enable DFCO are active and that torque is being reduced by turning off one or more fuel injectors and/or retarding spark.

Fuel Pump On indicates that the fuel pump is commanded on.

Secondary Fuel Pump On indicates that the secondary fuel pump is commanded on.  The secondary fuel pump output is only supported in dual fuel tank applications with an electronically controlled transfer pump.  A value of False will always be provided for this signal in other applications.

Fuel Enrichment Catalyst Protection Active indicates fuel is being added to cool the catalytic converter.

### CDAR – PID $1133 – Displacement on Demand Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1133 | Displacement on Demand Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Displacement on Demand Active | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Displacement on Demand Solenoid A On | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Displacement on Demand Solenoid B On | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Displacement on Demand Solenoid C On | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Displacement on Demand Solenoid D On | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Fuel Saver Mode Active | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the displacement on demand system.  The content of each of these signals is described below.

Displacement on Demand Active indicates that half of the engine’s cylinders have been deactivated.

Displacement on Demand Solenoid A On, Displacement on Demand Solenoid B On, Displacement on Demand Solenoid C On, and Displacement on Demand Solenoid D On indicate the actual states of the displacement on demand solenoid outputs.

Fuel Saver Mode Active indicates that the Displacement on Demand (aka Active Fuel Management) window of operation has been changed according to a driver selected calibration to improve fuel economy.

### IFRR – PID $1134 – Intake Flow Rationality Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1134 | Intake Flow Rationality Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| MAF Residual Out of Tolerance | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| MAP Model 1 Residual Out of Tolerance | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| MAP Model 2 Residual Out of Tolerance | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Throttle Position Residual Out of Tolerance | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Commanded Cylinder Deactivation Failed | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the intake flow rationality diagnostic.  The content of each of these signals is described below.

MAF Residual Out of Tolerance indicates that the filtered MAF residual (the difference between the measured mass airflow and the mass airflow predicted from the throttle model) exceeds a failure threshold.

MAP Model 1 Residual Out of Tolerance indicates that the filtered MAP model 1 residual (the difference between the measured intake manifold pressure and the intake manifold pressure predicted from the intake manifold model using the mass airflow predicted from the throttle model) exceeds a failure threshold.

MAP Model 2 Residual Out of Tolerance indicates that the filtered MAP model 2 residual (the difference between the measured intake manifold pressure and the intake manifold pressure predicted from the intake manifold model using the measured mass airflow) exceeds a failure threshold.

Throttle Position Residual Out of Tolerance indicates that the filtered throttle position residual (the product of the MAF residual and the MAP model 1 residual) exceeds a failure threshold.

Commanded Cylinder Deactivation Failed indicates that one or more cylinders failed to deactivate when commanded to do so by DoD.

### PTOR – PID $1135 – Power Take Off / Fast Idle Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1135 | Power Take Off / Fast Idle Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Power Take Off / Fast Idle Active | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Fast Idle Switch Active | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Fast Idle Switch Fault | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Power Take Off Enable Switch Active | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Power Take Off Set 1 Switch Active | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Power Take Off Set 2 Switch Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Power Take Off Switch Fault | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Power Take Off Remote Engine Shutdown Request | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the power take off (PTO) and fast idle systems.  The content of each of these signals is described below.

Power Take Off / Fast Idle Active indicates that PTO or fast idle engine speed control is active.

Fast Idle Switch Active indicates the state of the fast idle switch.  In fast idle electrical architectures from PPEI 3.x or later, this signal is based upon the state of the fast idle switch received over serial communication from the platform electronics.  In fast idle electrical architectures that precede any version of PPEI, this signal is based upon the hardwire fast idle switch input to the ECM.  A value of False will always be provided for this signal when fast idle is not present.

Fast Idle Switch Fault indicates that a failure of the fast idle switch has been detected (and a corresponding DTC has been stored) within the platform electronics.  In fast idle electrical architectures from PPEI 3.x or later, this signal is based upon the failure information for the fast idle switch received over serial communication from the platform electronics.  In fast idle electrical architectures that precede any version of PPEI, a value of False will always be provided for this signal.  A value of False will always be provided for this signal when fast idle is not present.

Power Take Off Enable Switch Active indicates the state of the PTO enable switch.  In PTO electrical architectures from PPEI 3.x or later, this signal is based upon the state of the PTO enable switch received over serial communication from the platform electronics.  In PTO electrical architectures that precede any version of PPEI, this signal is based upon the hardwire PTO enable switch input to the ECM.  A value of False will always be provided for this signal when PTO is not present.

Power Take Off Set 1 Switch Active and Power Take Off Set 2 Switch Active indicate the states of the PTO set 1 switch and PTO set 2 switch, respectively.  In PTO electrical architectures from PPEI 3.x or later, these signals are based upon the states of the PTO set 1 switch and PTO set 2 switch received over serial communication from the platform electronics.  In PTO electrical architectures that precede any version of PPEI, these signals are based upon the hardwire PTO set 1 switch and PTO set 2 switch inputs to the ECM.  Values of False will always be provided for these signals when PTO is not present.

Power Take Off Switch Fault indicates that a failure of the PTO switches has been detected (and a corresponding DTC has been stored) within the platform electronics.  In PTO electrical architectures from PPEI 3.x or later, this signal is based upon the failure information for the PTO switches received over serial communication from the platform electronics.  In PTO electrical architectures that precede any version of PPEI, a value of False will always be provided for this signal.  A value of False will always be provided for this signal when PTO is not present.

Power Take Off Remote Engine Shutdown Requested indicates that the PTO remote engine shutdown request hardwire input to the ECM is active.  A value of False will always be provided for this signal when PTO is not present.

### PMDR – PID $1141 – Run/Crank Voltage

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1141 | Run/Crank Voltage | 1 | UNM | 0 - 25.5 V | E = N * 0.1 |

This PID contains the voltage of the Run/Crank input to the controller.

### ACCR – PID $1144 – Air Conditioning Refrigerant High Side Fluid Pressure A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1144 | Air Conditioning Refrigerant High Side Fluid Pressure A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the air conditioning refrigerant high side fluid pressure analog input as a percentage of its reference voltage.

### OXYR – PID $1145 – Oxygen Sensor Voltage Bank 1 Sensor 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1145 | Oxygen Sensor Voltage Bank 1 Sensor 1 | 1 | UNM | 0 - 1.106770833 V | E = N / 230.4 |

This PID contains the oxygen sensor voltage for bank 1 sensor 1.

### OXYR – PID $1146 – Oxygen Sensor Voltage Bank 1 Sensor 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1146 | Oxygen Sensor Voltage Bank 1 Sensor 2 | 1 | UNM | 0 - 1.106770833 V | E = N / 230.4 |

This PID contains the oxygen sensor voltage for bank 1 sensor 2.

### OXYR – PID $1148 – Oxygen Sensor Voltage Bank 2 Sensor 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1148 | Oxygen Sensor Voltage Bank 2 Sensor 1 | 1 | UNM | 0 - 1.106770833 V | E = N / 230.4 |

This PID contains the oxygen sensor voltage for bank 2 sensor 1.

### OXYR – PID $1149 – Oxygen Sensor Voltage Bank 2 Sensor 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1149 | Oxygen Sensor Voltage Bank 2 Sensor 2 | 1 | UNM | 0 - 1.106770833 V | E = N / 230.4 |

This PID contains the oxygen sensor voltage for bank 2 sensor 2.

### EGRR – PID $114B – Exhaust Gas Recirculation Pintle Position A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $114B | Exhaust Gas Recirculation Pintle Position A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the exhaust gas recirculation (EGR) pintle position analog input as a percentage of its reference voltage.

### GENR – PID $114D – Generator F-Terminal Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $114D | Generator F-Terminal Duty Cycle | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the filtered generator F-terminal duty cycle.

### EVPR – PID $114E – Fuel Tank Vapor Pressure A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $114E | Fuel Tank Vapor Pressure A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the fuel tank vapor pressure analog input as a percentage of its reference voltage.

### EGRR – PID $1152 – Exhaust Gas Recirculation Actual Pintle Lift

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1152 | Exhaust Gas Recirculation Actual Pintle Lift | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the exhaust gas recirculation (EGR) actual pintle lift as a percentage of maximum pintle lift.

### EOTR – PID $1154 – Estimated Engine Oil Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1154 | Estimated Engine Oil Temperature | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the estimated (modeled) engine oil temperature (as a function of engine coolant temperature and other parameters).

### FLVR – PID $1155 – Fuel Level Sensor A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1155 | Fuel Level Sensor A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the unfiltered fuel level sensor analog input for the primary fuel tank as a percentage of its reference voltage.

### BBVR – PID $1157 – Brake Booster Vacuum Pressure A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1157 | Brake Booster Vacuum Pressure A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the brake booster vacuum pressure analog input as a percentage of its reference voltage.

### TCSR – PID $1158 – Requested Torque Input

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1158 | Requested Torque Input | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the engine torque requested by the traction control system, as a percentage of maximum engine torque, via the requested torque PWM input to the ECM from the ABS/TCS module.

### EOPR – PID $115C – Engine Oil Pressure A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $115C | Engine Oil Pressure A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the engine oil pressure analog input as a percentage of its reference voltage.

### EPSR – PID $115D – Crank Position Sensor Pulse Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $115D | Crank Position Sensor Pulse Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains a rolling count of the number of crank position sensor pulses.  A change in the value of this PID indicates crank position sensor activity.

### EPSR – PID $115E – Primary Cam Position Sensor Pulse Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $115E | Primary Cam Position Sensor Pulse Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains a rolling count of the number of primary cam position sensor pulses.  A change in the value of this PID indicates primary cam position sensor activity.

### EPSR – PID $115F – Crankshaft Resynchronization Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $115F | Crankshaft Resynchronization Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains a rolling count of the number of times that crankshaft synchronization has been lost (at normal engine speeds).

### ECTR – PID $116F – Power-Up Engine Coolant Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $116F | Power-Up Engine Coolant Temperature | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the engine coolant temperature recorded at power-up (Run/Crank transition from low to high).

### EGRR – PID $1171 – Exhaust Gas Recirculation Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1171 | Exhaust Gas Recirculation Duty Cycle | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded duty cycle for the exhaust gas recirculation (EGR) output.

### TCSR – PID $1175 – Delivered Torque Output

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1175 | Delivered Torque Output | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the estimated engine torque delivered by the powertrain, as a percentage of maximum engine torque, that is provided to the ABS/TCS module via the delivered torque PWM output from the ECM.

### FPMR – PID $1178 – Fuel Pump Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1178 | Fuel Pump Duty Cycle | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded duty cycle for the fuel pump output.  In applications with a constant speed fuel pump, the duty cycle is commanded to values of 0% and 100% in order to command the fuel pump relay off and on, respectively.  In applications with a variable speed fuel pump, the duty cycle is used to provide a control request to the variable speed fuel pump control module.

### MSFR – PID $1186 – Misfire Catalyst Pass History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1186 | Misfire Catalyst Pass History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of 100 engine cycle test intervals for which misfire activity was below the catalyst damage threshold since a misfire failure has been reported.  This counter is maintained such that it is also halved whenever the set of misfire history counters for the individual cylinders are halved in order to prevent overflow.

### MSFR – PID $1187 – Misfire Catalyst Fail History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1187 | Misfire Catalyst Fail History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of 100 engine cycle test intervals for which misfire activity was high enough to cause catalyst damage since a code clear.  This counter is maintained such that it is also halved whenever the set of misfire history counters for the individual cylinders are halved in order to prevent overflow.

### MAFR – PID $1188 – Engine Load

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1188 | Engine Load | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the engine load, air mass per cylinder as a percentage of the maximum air mass per cylinder, where the maximum air mass per cylinder is determined at wide open throttle (WOT) and conditions for maximum air density (minimum possible ambient temperature, minimum possible ambient pressure, and maximum possible volumetric efficiency).

### MSFR – PID $1189 – Misfire Failure Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1189 | Misfire Failure Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Misfire Catalyst Failure Status | 0 | 7 | 4 | ENM | N/A | $0=Fault; $1=No Fault; $2=Abnormal Speed |
| Misfire Emission Failure Status | 0 | 3 | 4 | ENM | N/A | $0=Fault; $1=No Fault; $2=Abnormal Speed |

This PID indicates whether misfire activity over the most recent 100 engine cycle test interval is high enough to cause catalyst damage or an emission failure.

### MSFR – PID $118A – Misfire Catalyst Exceedences Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $118A | Misfire Catalyst Exceedences Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of misfire catalyst exceedences observed over the current key cycle.

### MSFR – PID $118B – Misfire Emission Exceedences Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $118B | Misfire Emission Exceedences Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of misfire emission exceedences observed over the current key cycle.

### MSFR – PID $118C – Maximum Misfire Catalyst Exceedences Count

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $118C | Maximum Misfire Catalyst Exceedences Count | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the maximum number of misfire catalyst exceedences observed over a key cycle since a code clear.

### MSFR – PID $118D – Maximum Misfire Emission Exceedences Count

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $118D | Maximum Misfire Emission Exceedences Count | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the maximum number of misfire emission exceedences observed over a key cycle since a code clear.

### FADR – PID $1190 – Long Term Fuel Correction Index

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1190 | Long Term Fuel Correction Index | 1 | UNM | 0 - 255 | E = N * 1 |

This PID contains the index of the entry within the array of long term fuel corrections that is active based upon the current operating conditions (MAP, engine speed, purge duty cycle, A/C compressor state, and transmission range).

### TRQR – PID $119C – Torque Management Spark Retard

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $119C | Torque Management Spark Retard | 1 | UNM | 0 - 89.6484375 ° | E = N * 90 / 256 |

This PID contains the total spark retard (in terms of a crankshaft angle) from mean best torque (MBT) requested by the torque management algorithm.

### EPSR – PID $11A1 – Engine Run Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11A1 | Engine Run Time | 2 | UNM | 0 - 65535 s | E = N * 1 |

This PID contains the time that the engine has been running.  This PID have a value of zero when the engine is not running.

### KNKR – PID $11A6 – Maximum Knock Fast Spark Retard

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11A6 | Maximum Knock Fast Spark Retard | 1 | UNM | 0 - 22.412109375 ° | E = N * 22.5 / 256 |

This PID contains the maximum knock fast spark retard (in terms of a crankshaft angle) for all cylinders over the last engine cycle.

### MAFR – PID $11AC – Speed Density Mass Airflow

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11AC | Speed Density Mass Airflow | 2 | UNM | 0 - 511.9921875 g/s | E = N / 128 |

This PID contains the mass airflow determined from the speed density (MAP based) calculation of air mass per cylinder.

### EGRR – PID $11BB – Exhaust Gas Recirculation Closed Valve Pintle Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11BB | Exhaust Gas Recirculation Closed Valve Pintle Position | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the learned exhaust gas recirculation (EGR) closed valve pintle position as a percentage of the reference voltage for the EGR pintle position analog input.

### EGRR – PID $11BD – Exhaust Gas Recirculation Flow Test Trip Sample Count

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11BD | Exhaust Gas Recirculation Flow Test Trip Sample Count | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of samples in the exhaust gas recirculation (EGR) flow test for the current trip.

### EGRR – PID $11C1 – Exhaust Gas Recirculation Pintle Lift Error

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11C1 | Exhaust Gas Recirculation Pintle Lift Error | 1 | SNM | -100 - 99.21875 % | E = N * 100 / 128 |

This PID contains the difference between the desired and actual exhaust gas recirculation (EGR) pintle lift as a percentage of maximum pintle lift.

### IDLR – PID $11C9 – Desired Idle Engine Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11C9 | Desired Idle Engine Speed | 1 | UNM | 0 - 2040 rpm | E = N * 8 |

This PID contains the desired idle engine speed after the application of all limits and corrections.

### MSFR – PID $11E4 – Cylinder Mode Highest Peak

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11E4 | Cylinder Mode Highest Peak | 2 | UNM | 0 - 65535 us | E = N * 1 |

This PID contains the highest delta time (indicating the highest crankshaft deceleration) observed over the most recent engine cycle when the misfire diagnostic is operating in cylinder mode.  This value is compared to a threshold to determine if misfire has occurred during the most recent engine cycle.

### MSFR – PID $11E5 – Cylinder Mode Misfire Threshold

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11E5 | Cylinder Mode Misfire Threshold | 2 | UNM | 0 - 65535 us | E = N * 1 |

This PID contains the threshold used to determine if misfire has occurred during the most recent engine cycle when the misfire diagnostic is operating in cylinder mode.  This threshold is a function of engine speed, engine load, and the state of the displacement on demand system.

### MSFR – PID $11E6 – Revolution Mode Balanced Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11E6 | Revolution Mode Balanced Time | 2 | UNM | 0 - 65535 us | E = N * 1 |

This PID contains the balanced time (indicating the relative intensity of any engine speed imbalance) calculated over the most recent engine cycle when the misfire diagnostic is operating in revolution mode.  This value is compared to a threshold to determine if misfire has occurred during the most recent engine cycle.

### MSFR – PID $11E8 – Misfire Delay Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11E8 | Misfire Delay Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the total number of engine cycles remaining until the misfire diagnostic is enabled (the misfire diagnostic is only enabled when this counter is zero).  This counter is set to a calibration when conditions are present that could cause false misfire detection and is decremented on each subsequent engine cycle until it reaches a value of zero.

### MSFR – PID $11EA – Cylinder 5 Misfire Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11EA | Cylinder 5 Misfire Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 5 within the current 100 engine cycle test interval.  The set of misfire counters for the individual cylinders are maintained such that if a single misfire is detected over an engine cycle, the misfire counter for the associated cylinder is incremented by two, and if two misfires are detected over an engine cycle, the misfire counters for each of the associated cylinders is incremented by one.

### MSFR – PID $11EB – Cylinder 6 Misfire Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11EB | Cylinder 6 Misfire Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 6 within the current 100 engine cycle test interval.  The set of misfire counters for the individual cylinders are maintained such that if a single misfire is detected over an engine cycle, the misfire counter for the associated cylinder is incremented by two, and if two misfires are detected over an engine cycle, the misfire counters for each of the associated cylinders is incremented by one.

### MSFR – PID $11EC – Cylinder 7 Misfire Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11EC | Cylinder 7 Misfire Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 7 within the current 100 engine cycle test interval.  The set of misfire counters for the individual cylinders are maintained such that if a single misfire is detected over an engine cycle, the misfire counter for the associated cylinder is incremented by two, and if two misfires are detected over an engine cycle, the misfire counters for each of the associated cylinders is incremented by one.

### MSFR – PID $11ED – Cylinder 8 Misfire Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11ED | Cylinder 8 Misfire Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 8 within the current 100 engine cycle test interval.  The set of misfire counters for the individual cylinders are maintained such that if a single misfire is detected over an engine cycle, the misfire counter for the associated cylinder is incremented by two, and if two misfires are detected over an engine cycle, the misfire counters for each of the associated cylinders is incremented by one.

### MSFR – PID $11F2 – Misfire Abnormal Speed Ratio

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11F2 | Misfire Abnormal Speed Ratio | 1 | UNM | 0 - 255 | E = N * 1 |

This PID contains the ratio of the number engine cycles which appear to have exceeded the misfire threshold due to abnormal crankshaft speed variations to those which appear to have exceeded the misfire threshold due to a true misfire over the most recent 100 engine cycle test interval.

### MSFR – PID $11F4 – Misfire Catalyst Failure Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11F4 | Misfire Catalyst Failure Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of the 16 most recent 100 engine cycle test intervals for which misfire activity was high enough to cause catalyst damage.

### MSFR – PID $11F5 – Maximum Misfire Catalyst Failure Count

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11F5 | Maximum Misfire Catalyst Failure Count | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the maximum value of the misfire catalyst failure counter observed since a code clear.

### MSFR – PID $11F6 – Misfire Emission Failure Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11F6 | Misfire Emission Failure Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of the 16 most recent 100 engine cycle test intervals for which misfire activity was high enough to cause an emission failure.

### MSFR – PID $11F7 – Maximum Misfire Emission Failure Count

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11F7 | Maximum Misfire Emission Failure Count | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the maximum value of the misfire emission failure counter observed since a code clear.

### MSFR – PID $11F8 – Cylinder 5 Misfire History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11F8 | Cylinder 5 Misfire History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 5 since a misfire failure has been reported (determined by accumulating the values of the associated misfire counter at the conclusion of each 100 engine cycle test interval).  The set of misfire history counters for the individual cylinders are maintained such that if the addition of the misfire counter to the associated misfire history counter would cause overflow for any cylinder, the misfire history counters for all cylinders are halved prior to the addition of the misfire counter.

### MSFR – PID $11F9 – Cylinder 6 Misfire History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11F9 | Cylinder 6 Misfire History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 6 since a misfire failure has been reported (determined by accumulating the values of the associated misfire counter at the conclusion of each 100 engine cycle test interval).  The set of misfire history counters for the individual cylinders are maintained such that if the addition of the misfire counter to the associated misfire history counter would cause overflow for any cylinder, the misfire history counters for all cylinders are halved prior to the addition of the misfire counter.

### MSFR – PID $11FA – Cylinder 7 Misfire History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11FA | Cylinder 7 Misfire History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 7 since a misfire failure has been reported (determined by accumulating the values of the associated misfire counter at the conclusion of each 100 engine cycle test interval).  The set of misfire history counters for the individual cylinders are maintained such that if the addition of the misfire counter to the associated misfire history counter would cause overflow for any cylinder, the misfire history counters for all cylinders are halved prior to the addition of the misfire counter.

### MSFR – PID $11FB – Cylinder 8 Misfire History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $11FB | Cylinder 8 Misfire History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 8 since a misfire failure has been reported (determined by accumulating the values of the associated misfire counter at the conclusion of each 100 engine cycle test interval).  The set of misfire history counters for the individual cylinders are maintained such that if the addition of the misfire counter to the associated misfire history counter would cause overflow for any cylinder, the misfire history counters for all cylinders are halved prior to the addition of the misfire counter.

### MSFR – PID $1200 – Total Misfire Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1200 | Total Misfire Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the total number of misfires detected for all cylinders over the most recent 100 engine cycle test interval (determined by summing the misfire counters for the individual cylinders at the conclusion of the 100 engine cycle test interval).

### MSFR – PID $1201 – Cylinder 1 Misfire History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1201 | Cylinder 1 Misfire History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 1 since a misfire failure has been reported (determined by accumulating the values of the associated misfire counter at the conclusion of each 100 engine cycle test interval).  The set of misfire history counters for the individual cylinders are maintained such that if the addition of the misfire counter to the associated misfire history counter would cause overflow for any cylinder, the misfire history counters for all cylinders are halved prior to the addition of the misfire counter.

### MSFR – PID $1202 – Cylinder 2 Misfire History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1202 | Cylinder 2 Misfire History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 2 since a misfire failure has been reported (determined by accumulating the values of the associated misfire counter at the conclusion of each 100 engine cycle test interval).  The set of misfire history counters for the individual cylinders are maintained such that if the addition of the misfire counter to the associated misfire history counter would cause overflow for any cylinder, the misfire history counters for all cylinders are halved prior to the addition of the misfire counter.

### MSFR – PID $1203 – Cylinder 3 Misfire History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1203 | Cylinder 3 Misfire History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 3 since a misfire failure has been reported (determined by accumulating the values of the associated misfire counter at the conclusion of each 100 engine cycle test interval).  The set of misfire history counters for the individual cylinders are maintained such that if the addition of the misfire counter to the associated misfire history counter would cause overflow for any cylinder, the misfire history counters for all cylinders are halved prior to the addition of the misfire counter.

### MSFR – PID $1204 – Cylinder 4 Misfire History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1204 | Cylinder 4 Misfire History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 4 since a misfire failure has been reported (determined by accumulating the values of the associated misfire counter at the conclusion of each 100 engine cycle test interval).  The set of misfire history counters for the individual cylinders are maintained such that if the addition of the misfire counter to the associated misfire history counter would cause overflow for any cylinder, the misfire history counters for all cylinders are halved prior to the addition of the misfire counter.

### MSFR – PID $1205 – Cylinder 2 Misfire Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1205 | Cylinder 2 Misfire Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 2 within the current 100 engine cycle test interval.  The set of misfire counters for the individual cylinders are maintained such that if a single misfire is detected over an engine cycle, the misfire counter for the associated cylinder is incremented by two, and if two misfires are detected over an engine cycle, the misfire counters for each of the associated cylinders is incremented by one.

### MSFR – PID $1206 – Cylinder 1 Misfire Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1206 | Cylinder 1 Misfire Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 1 within the current 100 engine cycle test interval.  The set of misfire counters for the individual cylinders are maintained such that if a single misfire is detected over an engine cycle, the misfire counter for the associated cylinder is incremented by two, and if two misfires are detected over an engine cycle, the misfire counters for each of the associated cylinders is incremented by one.

### MSFR – PID $1207 – Cylinder 3 Misfire Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1207 | Cylinder 3 Misfire Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 3 within the current 100 engine cycle test interval.  The set of misfire counters for the individual cylinders are maintained such that if a single misfire is detected over an engine cycle, the misfire counter for the associated cylinder is incremented by two, and if two misfires are detected over an engine cycle, the misfire counters for each of the associated cylinders is incremented by one.

### MSFR – PID $1208 – Cylinder 4 Misfire Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1208 | Cylinder 4 Misfire Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of misfires detected for cylinder 4 within the current 100 engine cycle test interval.  The set of misfire counters for the individual cylinders are maintained such that if a single misfire is detected over an engine cycle, the misfire counter for the associated cylinder is incremented by two, and if two misfires are detected over an engine cycle, the misfire counters for each of the associated cylinders is incremented by one.

### MSFR – PID $1227 – Misfire Emission Fail History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1227 | Misfire Emission Fail History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of 100 engine cycle test intervals for which misfire activity was high enough to cause an emission failure since a code clear.  This counter is maintained such that it is also halved whenever the set of misfire history counters for the individual cylinders are halved in order to prevent overflow.

### MSFR – PID $1228 – Misfire Emission Pass History Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1228 | Misfire Emission Pass History Counter | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of 100 engine cycle test intervals for which misfire activity was below the emission failure threshold since a misfire failure has been reported.  This counter is maintained such that it is also halved whenever the set of misfire history counters for the individual cylinders are halved in order to prevent overflow.

### MSFR – PID $1229 – Revolution Mode Misfire Threshold

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1229 | Revolution Mode Misfire Threshold | 2 | UNM | 0 - 65535 us | E = N * 1 |

This PID contains the threshold used to determine if misfire has occurred during the most recent engine cycle when the misfire diagnostic is operating in revolution mode.  This threshold is a function of engine speed and engine load.

### MSFR – PID $122A – Misfire Engine Cycle Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $122A | Misfire Engine Cycle Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of engine cycles completed within the current 100 engine cycle test interval for the misfire diagnostic.

### DRER – PID $1232 – Emissions Warm-Up Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1232 | Emissions Warm-Up Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of OBD defined warm-up cycles since the last emissions related (fault type A or B) fail report or MIL request.  Emissions related DTCs are cleared from memory when this counter reaches 40.

### DRER – PID $1233 – Non Emissions Warm-Up Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1233 | Non Emissions Warm-Up Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of OBD defined warm-up cycles since the last non emissions related (fault type C) fail report.  Non emissions related DTCs are cleared from memory when this counter reaches 40.

### VSPR – PID $1234 – Distance Since Code Clear

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1234 | Distance Since Code Clear | 2 | UNM | 0 - 327675 km | E = N * 5 |

This PID contains the accumulated distance driven since the last code clear or memory reset.  Note that this distance is not reset by a natural code clear (i.e., 40 warm up cycles without a failure being reported).

### FFRR – PID $1235 – Distance at First Failure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1235 | Distance at First Failure | 2 | UNM | 0 - 327675 km | E = N * 5 |

This PID contains the value of PID $1234 at the time that a failure record was first captured for a diagnostic failure.  This PID is only meaningful within the context of a failure record or freeze frame.

### FFRR – PID $1236 – Distance at Last Failure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1236 | Distance at Last Failure | 2 | UNM | 0 - 327675 km | E = N * 5 |

This PID contains the value of PID $1234 at the time that a failure record was last captured for a diagnostic failure.  This PID is only meaningful within the context of a failure record or freeze frame.

### FFRR – PID $1237 – Failure Record Fail Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1237 | Failure Record Fail Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of ignition cycles with a failure reported since the first ignition cycle with a failure reported.  This PID is only meaningful within the context of a failure record or freeze frame.

### FFRR – PID $1238 – Failure Record Pass Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1238 | Failure Record Pass Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of ignition cycles with a pass reported and no failures reported since the first ignition cycle with a failure reported.  This PID is only meaningful within the context of a failure record or freeze frame.

### FFRR – PID $1239 – Failure Record Not Run Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1239 | Failure Record Not Run Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of ignition cycles without a pass or a failure reported since the first ignition cycle with a failure reported.  This PID is only meaningful within the context of a failure record or freeze frame.

### EVPR – PID $12AB – Evaporative Emissions Test Result

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12AB | Evaporative Emissions Test Result | 1 | ENM | N/A | $00=No Fault; $01=Canister Vent Restriction; $02=Purge Valve Leak; $03=Small Leak; $04=Weak Vacuum; $05=No Test Result |

This PID contains the result of the most recently completed evaporative emissions diagnostic test.

### APSR – PID $12B0 – Arbitrated Accelerator Pedal Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12B0 | Arbitrated Accelerator Pedal Position | 1 | UNM | 0 - 255 % | E = N * 1 |

This PID contains the accelerator pedal position resulting directly from the arbitration of the accelerator pedal position sensors (prior to any compensation to determine driver intent).

### TPSR – PID $12B1 – Arbitrated Throttle Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12B1 | Arbitrated Throttle Position | 2 | UNM | 0 - 1279.98046875 % | E = N * 5 / 256 |

This PID contains the throttle position resulting directly from the arbitration of the (corrected) throttle position sensors.

### APSR – PID $12B4 – Accelerator Pedal Position Sensor 1 Displacement

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12B4 | Accelerator Pedal Position Sensor 1 Displacement | 1 | UNM | 0 - 255 % | E = N * 1 |

This PID contains the displacement determined from accelerator pedal position sensor 1 (corrected and normalized based upon its learned minimum and maximum positions).

### APSR – PID $12B5 – Accelerator Pedal Position Sensor 2 Displacement

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12B5 | Accelerator Pedal Position Sensor 2 Displacement | 1 | UNM | 0 - 255 % | E = N * 1 |

This PID contains the displacement determined from accelerator pedal position sensor 2 (corrected and normalized based upon its learned minimum and maximum positions).

### TPSR – PID $12B7 – Throttle Position Sensor 1 Displacement

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12B7 | Throttle Position Sensor 1 Displacement | 1 | UNM | 0 - 255 % | E = N * 1 |

This PID contains the displacement determined from throttle position sensor 1 (corrected based upon its learned minimum position).

### TPSR – PID $12B8 – Throttle Position Sensor 2 Displacement

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12B8 | Throttle Position Sensor 2 Displacement | 1 | UNM | 0 - 255 % | E = N * 1 |

This PID contains the displacement determined from throttle position sensor 2 (corrected based upon its learned minimum position).

### TPSR – PID $12B9 – Throttle Position Sensor 2 A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12B9 | Throttle Position Sensor 2 A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the throttle position sensor 2 analog input as a percentage of its reference voltage.

### TPSR – PID $12BA – Throttle Position Sensor 1 A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12BA | Throttle Position Sensor 1 A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the throttle position sensor 1 analog input as a percentage of its reference voltage.

### APSR – PID $12BC – Accelerator Pedal Position Sensor 2 A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12BC | Accelerator Pedal Position Sensor 2 A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the accelerator pedal position sensor 2 analog input as a percentage of its reference voltage.

### APSR – PID $12BD – Accelerator Pedal Position Sensor 1 A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12BD | Accelerator Pedal Position Sensor 1 A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the accelerator pedal position sensor 1 analog input as a percentage of its reference voltage.

### FULR – PID $12C3 – Fuel Injector Pulse Width Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12C3 | Fuel Injector Pulse Width Bank 1 | 2 | UNM | 0 - 984.6 ms | E = N / 66.56 |

This PID contains the fuel injector pulse width (on-time) commanded to each cylinder on bank 1.

### FULR – PID $12C4 – Fuel Injector Pulse Width Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12C4 | Fuel Injector Pulse Width Bank 2 | 2 | UNM | 0 - 984.6 ms | E = N / 66.56 |

This PID contains the fuel injector pulse width (on-time) commanded to each cylinder on bank 2.

### KNKR – PID $12D9 – Total Knock Spark Retard

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12D9 | Total Knock Spark Retard | 1 | UNM | 0 - 44.82421875 ° | E = N * 45 / 256 |

This PID contains the total knock spark retard (in terms of a crankshaft angle) requested by the knock system.  The total knock spark retard includes the burst knock spark retard, the maximum knock fast spark retard, and the maximum knock balancing spark retard.

### MAFR – PID $12DA – Mass Airflow Sensor Frequency

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12DA | Mass Airflow Sensor Frequency | 2 | UNM | 0 - 65535 Hz | E = N * 1 |

This PID contains the frequency of the mass airflow (MAF) sensor input.

### RBSR – PID $12DE – Main Processor Reset Source

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12DE | Main Processor Reset Source | 1 | ENM | N/A | $00=Power up $02=External watchdog $03=Internal watchdog $04=Unhandled exception $05-FF=Controller-specific exception |

This PID contains the reset source for the latest reset in the main processor.

### MAFR – PID $12E0 – Corrected Mass Airflow

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12E0 | Corrected Mass Airflow | 2 | UNM | 0 - 2559.9609375 g/s | E = N / 25.6 |

This PID contains the mass airflow determined from the corrected calculation of air mass per cylinder.

### RBSR – PID $12E7 – Main Processor Running Reset Source Address

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12E7 | Main Processor Running Reset Source Address | 4 | UNM | 0 - $FFFFFFFF | E = N |

This PID contains the reset source address for the latest running reset in the main processor.

### VTDR – PID $12EE – Vehicle Theft Deterrent Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12EE | Vehicle Theft Deterrent Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Vehicle Theft Deterrent Password Learn Enabled | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Vehicle Theft Deterrent Fail Enable Active | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Vehicle Theft Deterrent Auto Learn Timer or Seed and Key Timer Enabled | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Vehicle Theft Deterrent Fuel Disabled Until Ignition Off | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Vehicle Theft Deterrent Fuel Disabled | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Vehicle Theft Deterrent Fuel Continue | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the vehicle theft deterrent (VTD) system.  This PID is only applicable to NAO VTD electrical architectures that precede PPEI 3.x.  The content of each of these signals is described below.

Vehicle Theft Deterrent Password Learn Enabled indicates that learning of the VTD password received from the platform electronics is enabled.  VTD password learning is initially enabled in the assembly plant (prior to the manufacturer’s enable counter being set to zero) and may be reenabled via either the auto learn procedure or the seed and key procedure.

Vehicle Theft Deterrent Fail Enable Active indicates that a serial communication failure has been detected after the receipt of the correct VTD password.  This indication is used to allow the vehicle to start on future ignition cycles in the presence of a serial communication failure.

Vehicle Theft Deterrent Auto Learn Timer or Seed and Key Timer Enabled indicates that either the auto learn timer (used in the auto learn procedure to enable VTD password learning) or the seed and key timer (used in the seed and key procedure to enable VTD password learning) is active.

Vehicle Theft Deterrent Fuel Disabled Until Ignition Off indicates the VTD system has disabled fuel for the remainder of the ignition cycle.

Vehicle Theft Deterrent Fuel Disabled indicates that the VTD system has disabled fuel.

Vehicle Theft Deterrent Fuel Continue indicates that the VTD system has decided not to disable fuel for the remainder of the ignition cycle based upon the receipt of the correct VTD password.

### MSFR – PID $12EF – Crank Angle Sensing Error Learn Attempts

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12EF | Crank Angle Sensing Error Learn Attempts | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of attempts to learn the crank angle sensing error (CASE) during the current key cycle.

### MSFR – PID $12F0 – Crank Angle Sensing Error Learn Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12F0 | Crank Angle Sensing Error Learn Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Crank Angle Sensing Error Learned This Key On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Crank Angle Sensing Error Learn In Progress | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Cylinder Compensation Factors Out of Range | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Opposing Cylinder Compensation Factors Out of Range | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Sum of Cylinder Compensation Factors Out of Range | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Crank Pulse Count Error | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the crank angle sensing error (CASE) learn algorithm.  The content of each of these signals is described below.

Crank Angle Sensing Error Learned This Key On indicates that the crank angle sensing errors have been learned during the current key cycle.

Crank Angle Sensing Error Learn In Progress indicates that the calculation and learning of the crank angle sensing errors is enabled and in progress.

Cylinder Compensation Factors Out of Range indicates that one or more of the compensation factors calculated for the individual cylinders is too large.

Opposing Cylinder Compensation Factors Out of Range indicates that the difference in the compensation factors calculated for one or more pairs of opposing cylinders is too large.

Sum of Cylinder Compensation Factors Out of Range indicates that the sum of the compensation factors calculated for the individual cylinders is too large.

Crank Pulse Count Error indicates that the number of crank pulses observed over an engine cycle did not equal the number of cylinders for the engine at some point during the CASE learn cycle.

### EPSR – PID $12F5 – Crank Position Sensor Low Resolution Period

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12F5 | Crank Position Sensor Low Resolution Period | 2 | UNM | 0 - 999.9847412109 ms | E = N * 1000 / 65536 |

This PID contains the time between the two most recent crank position sensor low resolution (LORES) pulses.

### MSFR – PID $12FF – Misfire Diagnostic Engine Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $12FF | Misfire Diagnostic Engine Speed | 2 | UNM | 0 - 12799.8046875 rpm | E = N * 0.1953125 |

This PID contains the engine speed used within the misfire diagnostic.  This engine speed  is determined from the minimum LORES period observed over the previous engine cycle and, thus, is the maximum engine speed observed over the previous engine cycle.

### MSFR – PID $1300 – Misfire Diagnostic Engine Load

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1300 | Misfire Diagnostic Engine Load | 2 | UNM | 0 - 1279.98046875 % | E = N * 0.01953125 |

This PID contains the engine load used within the misfire diagnostic (calculated as the ratio of the mass of air predicted to be in the cylinder two firing events into the future to the maximum possible mass of air in the cylinder based on the current ambient air pressure and charge temperature).

### EPSR – PID $130E – Previous Crank Position Sensor Low Resolution Period

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $130E | Previous Crank Position Sensor Low Resolution Period | 2 | UNM | 0 - 999.9847412109 ms | E = N * 1000 / 65536 |

This PID contains the time between the two crank position sensor low resolution (LORES) pulses that occurred just prior to the most recent LORES pulse.

### EPSR – PID $130F – Cylinder Identifier

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $130F | Cylinder Identifier | 1 | ENM | N/A | $00=Cylinder A; $01=Cylinder B; $02=Cylinder C; $03=Cylinder D; $04=Cylinder E; $05=Cylinder F; $06=Cylinder G; $07=Cylinder H |

This PID contains the identifier for the cylinder that will experience top dead center (TDC) compression within the current LORES period (the LORES event typically occurs 60 to 75 degrees before TDC).

### EVPR – PID $1310 – Evaporative Emissions Service Bay Test Result

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1310 | Evaporative Emissions Service Bay Test Result | 1 | ENM | N/A | $00=No Result; $01=Passed; $02=Aborted; $03=Failed - Large Leak; $04=Failed - Small Leak; $05=Failed - Vent System Performance; $06=Failed - System Flow During Non Purge; $07=Engine Run Test Complete with No Failure |

This PID contains the result of the evaporative emissions diagnostic service bay test.

### EVPR – PID $1311 – Evaporative Emissions Service Bay Test Abort Reason

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1311 | Evaporative Emissions Service Bay Test Abort Reason | 1 | ENM | N/A | $00=Test Not Aborted; $01=Lost Enable Conditions; $02=Small Leak Test Time Expired; $03=Vehicle Not at Rest |

This PID contains the reason that the evaporative emissions diagnostic service bay test was aborted.

### EVPR – PID $1312 – Evaporative Emissions Service Bay Test Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1312 | Evaporative Emissions Service Bay Test Status | 1 | ENM | N/A | $00=Waiting for Purge Enable; $01=Evaporative Emissions Service Bay Test Running; $02=Evaporative Emissions Service Bay Test Complete; $03=Drive Vehicle; $04=Ready to Run Engine Off Test; $05=EONV Diagnostic Executing; $06=EONV Service Bay Test Complete; $07=Performing Refueling Rationality |

This PID contains the status of the evaporative emissions diagnostic and engine off natural vacuum (EONV) diagnostic service bay tests.

### CRZR – PID $1315 – Cruise Disengage History 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1315 | Cruise Disengage History 1 | 1 | ENM | N/A | $00=Brake Pedal Applied; $01=Brake Pedal Before Cruise; $02=Cancel Switch Active; $03=Clutch Pedal Applied; $04=Coast Disengage; $05=Coast Below Low Speed; $06=ETC Inhibit; $07=High Acceleration; $08=High Deceleration; $09=High Vehicle Speed; $0A=Illegal Cruise Mode; $0B=Low Vehicle Speed; $0C=None; $0D=Cruise Switch Off; $0E=Speed Difference High; $0F=Invalid Cruise Tap Down; $10=Control Module Inhibit; $11=R/A and S/C Switches Pressed Together; $12=TCS Active; $13=Speed Difference Low; $14=Device Control; $15=Memory Error; $16=Engine Run Time; $17=Engine Speed; $18=DTC Active; $19=Engine Overspeed Protection Active; $1A=First Gear Disable; $1B=Accelerator Pedal Override; $1C=Low Ignition Voltage; $1D=Transmission Not in Gear; $1E=Vehicle Overspeed Protection Active; $1F=Analog Cruise Switch Invalid; $20=High Ignition Voltage; $21=Non Driven Wheel Speed Greater; $22=Driven Wheel Speed Greater; $23=Cruise Execution Error; $24=Cruise Switch Serial Communication Fault; $25=Engine Metal Overtemperature Protection Active; $26=VSES Active; $27=Rear Axle Low; $28=Transmission Gear Fault; $29=Brake Signal Serial Communication Fault; $2A=Brake Pedal Fault; $2B=Park Brake Active; $2C=ACC Inhibit; $2D=ACC Automatic Braking Failed; $2E=Vehicle Speed Control Type Error; $2F=ACC Command Serial Communication Fault; $30=Power Take Off Active; $31=ECM Running Reset |

This PID contains one of the last eight reasons that powertrain has disengaged cruise control.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CRZR – PID $1316 – Cruise Disengage History 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1316 | Cruise Disengage History 2 | 1 | ENM | N/A | $00=Brake Pedal Applied; $01=Brake Pedal Before Cruise; $02=Cancel Switch Active; $03=Clutch Pedal Applied; $04=Coast Disengage; $05=Coast Below Low Speed; $06=ETC Inhibit; $07=High Acceleration; $08=High Deceleration; $09=High Vehicle Speed; $0A=Illegal Cruise Mode; $0B=Low Vehicle Speed; $0C=None; $0D=Cruise Switch Off; $0E=Speed Difference High; $0F=Invalid Cruise Tap Down; $10=Control Module Inhibit; $11=R/A and S/C Switches Pressed Together; $12=TCS Active; $13=Speed Difference Low; $14=Device Control; $15=Memory Error; $16=Engine Run Time; $17=Engine Speed; $18=DTC Active; $19=Engine Overspeed Protection Active; $1A=First Gear Disable; $1B=Accelerator Pedal Override; $1C=Low Ignition Voltage; $1D=Transmission Not in Gear; $1E=Vehicle Overspeed Protection Active; $1F=Analog Cruise Switch Invalid; $20=High Ignition Voltage; $21=Non Driven Wheel Speed Greater; $22=Driven Wheel Speed Greater; $23=Cruise Execution Error; $24=Cruise Switch Serial Communication Fault; $25=Engine Metal Overtemperature Protection Active; $26=VSES Active; $27=Rear Axle Low; $28=Transmission Gear Fault; $29=Brake Signal Serial Communication Fault; $2A=Brake Pedal Fault; $2B=Park Brake Active; $2C=ACC Inhibit; $2D=ACC Automatic Braking Failed; $2E=Vehicle Speed Control Type Error; $2F=ACC Command Serial Communication Fault; $30=Power Take Off Active; $31=ECM Running Reset |

This PID contains one of the last eight reasons that powertrain has disengaged cruise control.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CRZR – PID $1317 – Cruise Disengage History 3

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1317 | Cruise Disengage History 3 | 1 | ENM | N/A | $00=Brake Pedal Applied; $01=Brake Pedal Before Cruise; $02=Cancel Switch Active; $03=Clutch Pedal Applied; $04=Coast Disengage; $05=Coast Below Low Speed; $06=ETC Inhibit; $07=High Acceleration; $08=High Deceleration; $09=High Vehicle Speed; $0A=Illegal Cruise Mode; $0B=Low Vehicle Speed; $0C=None; $0D=Cruise Switch Off; $0E=Speed Difference High; $0F=Invalid Cruise Tap Down; $10=Control Module Inhibit; $11=R/A and S/C Switches Pressed Together; $12=TCS Active; $13=Speed Difference Low; $14=Device Control; $15=Memory Error; $16=Engine Run Time; $17=Engine Speed; $18=DTC Active; $19=Engine Overspeed Protection Active; $1A=First Gear Disable; $1B=Accelerator Pedal Override; $1C=Low Ignition Voltage; $1D=Transmission Not in Gear; $1E=Vehicle Overspeed Protection Active; $1F=Analog Cruise Switch Invalid; $20=High Ignition Voltage; $21=Non Driven Wheel Speed Greater; $22=Driven Wheel Speed Greater; $23=Cruise Execution Error; $24=Cruise Switch Serial Communication Fault; $25=Engine Metal Overtemperature Protection Active; $26=VSES Active; $27=Rear Axle Low; $28=Transmission Gear Fault; $29=Brake Signal Serial Communication Fault; $2A=Brake Pedal Fault; $2B=Park Brake Active; $2C=ACC Inhibit; $2D=ACC Automatic Braking Failed; $2E=Vehicle Speed Control Type Error; $2F=ACC Command Serial Communication Fault; $30=Power Take Off Active; $31=ECM Running Reset |

This PID contains one of the last eight reasons that powertrain has disengaged cruise control.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CRZR – PID $1318 – Cruise Disengage History 4

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1318 | Cruise Disengage History 4 | 1 | ENM | N/A | $00=Brake Pedal Applied; $01=Brake Pedal Before Cruise; $02=Cancel Switch Active; $03=Clutch Pedal Applied; $04=Coast Disengage; $05=Coast Below Low Speed; $06=ETC Inhibit; $07=High Acceleration; $08=High Deceleration; $09=High Vehicle Speed; $0A=Illegal Cruise Mode; $0B=Low Vehicle Speed; $0C=None; $0D=Cruise Switch Off; $0E=Speed Difference High; $0F=Invalid Cruise Tap Down; $10=Control Module Inhibit; $11=R/A and S/C Switches Pressed Together; $12=TCS Active; $13=Speed Difference Low; $14=Device Control; $15=Memory Error; $16=Engine Run Time; $17=Engine Speed; $18=DTC Active; $19=Engine Overspeed Protection Active; $1A=First Gear Disable; $1B=Accelerator Pedal Override; $1C=Low Ignition Voltage; $1D=Transmission Not in Gear; $1E=Vehicle Overspeed Protection Active; $1F=Analog Cruise Switch Invalid; $20=High Ignition Voltage; $21=Non Driven Wheel Speed Greater; $22=Driven Wheel Speed Greater; $23=Cruise Execution Error; $24=Cruise Switch Serial Communication Fault; $25=Engine Metal Overtemperature Protection Active; $26=VSES Active; $27=Rear Axle Low; $28=Transmission Gear Fault; $29=Brake Signal Serial Communication Fault; $2A=Brake Pedal Fault; $2B=Park Brake Active; $2C=ACC Inhibit; $2D=ACC Automatic Braking Failed; $2E=Vehicle Speed Control Type Error; $2F=ACC Command Serial Communication Fault; $30=Power Take Off Active; $31=ECM Running Reset |

This PID contains one of the last eight reasons that powertrain has disengaged cruise control.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CRZR – PID $1319 – Cruise Disengage History 5

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1319 | Cruise Disengage History 5 | 1 | ENM | N/A | $00=Brake Pedal Applied; $01=Brake Pedal Before Cruise; $02=Cancel Switch Active; $03=Clutch Pedal Applied; $04=Coast Disengage; $05=Coast Below Low Speed; $06=ETC Inhibit; $07=High Acceleration; $08=High Deceleration; $09=High Vehicle Speed; $0A=Illegal Cruise Mode; $0B=Low Vehicle Speed; $0C=None; $0D=Cruise Switch Off; $0E=Speed Difference High; $0F=Invalid Cruise Tap Down; $10=Control Module Inhibit; $11=R/A and S/C Switches Pressed Together; $12=TCS Active; $13=Speed Difference Low; $14=Device Control; $15=Memory Error; $16=Engine Run Time; $17=Engine Speed; $18=DTC Active; $19=Engine Overspeed Protection Active; $1A=First Gear Disable; $1B=Accelerator Pedal Override; $1C=Low Ignition Voltage; $1D=Transmission Not in Gear; $1E=Vehicle Overspeed Protection Active; $1F=Analog Cruise Switch Invalid; $20=High Ignition Voltage; $21=Non Driven Wheel Speed Greater; $22=Driven Wheel Speed Greater; $23=Cruise Execution Error; $24=Cruise Switch Serial Communication Fault; $25=Engine Metal Overtemperature Protection Active; $26=VSES Active; $27=Rear Axle Low; $28=Transmission Gear Fault; $29=Brake Signal Serial Communication Fault; $2A=Brake Pedal Fault; $2B=Park Brake Active; $2C=ACC Inhibit; $2D=ACC Automatic Braking Failed; $2E=Vehicle Speed Control Type Error; $2F=ACC Command Serial Communication Fault; $30=Power Take Off Active; $31=ECM Running Reset |

This PID contains one of the last eight reasons that powertrain has disengaged cruise control.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CRZR – PID $131A – Cruise Disengage History 6

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $131A | Cruise Disengage History 6 | 1 | ENM | N/A | $00=Brake Pedal Applied; $01=Brake Pedal Before Cruise; $02=Cancel Switch Active; $03=Clutch Pedal Applied; $04=Coast Disengage; $05=Coast Below Low Speed; $06=ETC Inhibit; $07=High Acceleration; $08=High Deceleration; $09=High Vehicle Speed; $0A=Illegal Cruise Mode; $0B=Low Vehicle Speed; $0C=None; $0D=Cruise Switch Off; $0E=Speed Difference High; $0F=Invalid Cruise Tap Down; $10=Control Module Inhibit; $11=R/A and S/C Switches Pressed Together; $12=TCS Active; $13=Speed Difference Low; $14=Device Control; $15=Memory Error; $16=Engine Run Time; $17=Engine Speed; $18=DTC Active; $19=Engine Overspeed Protection Active; $1A=First Gear Disable; $1B=Accelerator Pedal Override; $1C=Low Ignition Voltage; $1D=Transmission Not in Gear; $1E=Vehicle Overspeed Protection Active; $1F=Analog Cruise Switch Invalid; $20=High Ignition Voltage; $21=Non Driven Wheel Speed Greater; $22=Driven Wheel Speed Greater; $23=Cruise Execution Error; $24=Cruise Switch Serial Communication Fault; $25=Engine Metal Overtemperature Protection Active; $26=VSES Active; $27=Rear Axle Low; $28=Transmission Gear Fault; $29=Brake Signal Serial Communication Fault; $2A=Brake Pedal Fault; $2B=Park Brake Active; $2C=ACC Inhibit; $2D=ACC Automatic Braking Failed; $2E=Vehicle Speed Control Type Error; $2F=ACC Command Serial Communication Fault; $30=Power Take Off Active; $31=ECM Running Reset |

This PID contains one of the last eight reasons that powertrain has disengaged cruise control.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CRZR – PID $131B – Cruise Disengage History 7

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $131B | Cruise Disengage History 7 | 1 | ENM | N/A | $00=Brake Pedal Applied; $01=Brake Pedal Before Cruise; $02=Cancel Switch Active; $03=Clutch Pedal Applied; $04=Coast Disengage; $05=Coast Below Low Speed; $06=ETC Inhibit; $07=High Acceleration; $08=High Deceleration; $09=High Vehicle Speed; $0A=Illegal Cruise Mode; $0B=Low Vehicle Speed; $0C=None; $0D=Cruise Switch Off; $0E=Speed Difference High; $0F=Invalid Cruise Tap Down; $10=Control Module Inhibit; $11=R/A and S/C Switches Pressed Together; $12=TCS Active; $13=Speed Difference Low; $14=Device Control; $15=Memory Error; $16=Engine Run Time; $17=Engine Speed; $18=DTC Active; $19=Engine Overspeed Protection Active; $1A=First Gear Disable; $1B=Accelerator Pedal Override; $1C=Low Ignition Voltage; $1D=Transmission Not in Gear; $1E=Vehicle Overspeed Protection Active; $1F=Analog Cruise Switch Invalid; $20=High Ignition Voltage; $21=Non Driven Wheel Speed Greater; $22=Driven Wheel Speed Greater; $23=Cruise Execution Error; $24=Cruise Switch Serial Communication Fault; $25=Engine Metal Overtemperature Protection Active; $26=VSES Active; $27=Rear Axle Low; $28=Transmission Gear Fault; $29=Brake Signal Serial Communication Fault; $2A=Brake Pedal Fault; $2B=Park Brake Active; $2C=ACC Inhibit; $2D=ACC Automatic Braking Failed; $2E=Vehicle Speed Control Type Error; $2F=ACC Command Serial Communication Fault; $30=Power Take Off Active; $31=ECM Running Reset |

This PID contains one of the last eight reasons that powertrain has disengaged cruise control.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CRZR – PID $131C – Cruise Disengage History 8

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $131C | Cruise Disengage History 8 | 1 | ENM | N/A | $00=Brake Pedal Applied; $01=Brake Pedal Before Cruise; $02=Cancel Switch Active; $03=Clutch Pedal Applied; $04=Coast Disengage; $05=Coast Below Low Speed; $06=ETC Inhibit; $07=High Acceleration; $08=High Deceleration; $09=High Vehicle Speed; $0A=Illegal Cruise Mode; $0B=Low Vehicle Speed; $0C=None; $0D=Cruise Switch Off; $0E=Speed Difference High; $0F=Invalid Cruise Tap Down; $10=Control Module Inhibit; $11=R/A and S/C Switches Pressed Together; $12=TCS Active; $13=Speed Difference Low; $14=Device Control; $15=Memory Error; $16=Engine Run Time; $17=Engine Speed; $18=DTC Active; $19=Engine Overspeed Protection Active; $1A=First Gear Disable; $1B=Accelerator Pedal Override; $1C=Low Ignition Voltage; $1D=Transmission Not in Gear; $1E=Vehicle Overspeed Protection Active; $1F=Analog Cruise Switch Invalid; $20=High Ignition Voltage; $21=Non Driven Wheel Speed Greater; $22=Driven Wheel Speed Greater; $23=Cruise Execution Error; $24=Cruise Switch Serial Communication Fault; $25=Engine Metal Overtemperature Protection Active; $26=VSES Active; $27=Rear Axle Low; $28=Transmission Gear Fault; $29=Brake Signal Serial Communication Fault; $2A=Brake Pedal Fault; $2B=Park Brake Active; $2C=ACC Inhibit; $2D=ACC Automatic Braking Failed; $2E=Vehicle Speed Control Type Error; $2F=ACC Command Serial Communication Fault; $30=Power Take Off Active; $31=ECM Running Reset |

This PID contains one of the last eight reasons that powertrain has disengaged cruise control.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### FLVR – PID $131D – Fuel Tank Rated Capacity

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $131D | Fuel Tank Rated Capacity | 2 | UNM | 0 - 1023.984375 l | E = N * 0.015625 |

This PID contains the rated (advertised) capacity of the fuel tank(s).

### TPSR – PID $131E – Desired Throttle Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $131E | Desired Throttle Position | 2 | UNM | 0 - 1279.98046875 % | E = N * 5 / 256 |

This PID contains the desired throttle position used to control the throttle motor, which is determined from the arbitration of the various requests for the authority to control the throttle, including remedial actions.

### APSR – PID $131F – Accelerator Pedal Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $131F | Accelerator Pedal Position | 2 | UNM | 0 - 1279.98046875 % | E = N * 5 / 256 |

This PID contains the driver intent accelerator pedal position resulting from the arbitration of the accelerator pedal position sensors and the application of minimum pedal learn floormat protection and deadband and hysteresis compensation.

### FLVR – PID $132A – Total Fuel Volume

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $132A | Total Fuel Volume | 2 | UNM | 0 - 1023.984375 l | E = N * 0.015625 |

This PID contains the filtered, total fuel volume contained in the fuel tank(s).

### FLVR – PID $1338 – Fuel Level Sensor A/D Secondary Tank

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1338 | Fuel Level Sensor A/D Secondary Tank | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the unfiltered fuel level sensor analog input for the secondary fuel tank as a percentage of its reference voltage.

### CATR – PID $1354 – Catalyst Monitor Diagnostic Test Counter Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1354 | Catalyst Monitor Diagnostic Test Counter Bank 1 | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of iterations of the catalyst monitor diagnostic test that have been performed since a code clear for bank 1 (used to determine if a sufficient number of tests have run to report a passing condition).

### CATR – PID $1355 – Catalyst Monitor Diagnostic Test Counter Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1355 | Catalyst Monitor Diagnostic Test Counter Bank 2 | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of iterations of the catalyst monitor diagnostic test that have been performed since a code clear for bank 2 (used to determine if a sufficient number of tests have run to report a passing condition).

### EGRR – PID $135D – Exhaust Gas Recirculation Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $135D | Exhaust Gas Recirculation Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Exhaust Gas Recirculation Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Exhaust Gas Recirculation Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Exhaust Gas Recirculation Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Exhaust Gas Recirculation Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the exhaust gas recirculation (EGR) output (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0403.

### DFIR – PID $135F – Malfunction Indicator Lamp Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $135F | Malfunction Indicator Lamp Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Malfunction Indicator Lamp Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Malfunction Indicator Lamp Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Malfunction Indicator Lamp Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Malfunction Indicator Lamp Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the malfunction indicator lamp (MIL) output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0650.

### EPSR – PID $1360 – Tachometer Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1360 | Tachometer Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Tachometer Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Tachometer Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Tachometer Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Tachometer Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the tachometer output (the output is considered to be commanded on if the engine control module is in a full state of synchronization to the cam and crank shaft sensors, resulting in a non zero frequency) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0654.

### AIRR – PID $1365 – Secondary Air Injection Valve Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1365 | Secondary Air Injection Valve Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Secondary Air Injection Valve Commanded Open | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Secondary Air Injection Valve Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Secondary Air Injection Valve Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Secondary Air Injection Valve Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the secondary air injection valve output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0412.

### AIRR – PID $1366 – Secondary Air Injection Pump Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1366 | Secondary Air Injection Pump Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Secondary Air Injection Pump Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Secondary Air Injection Pump Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Secondary Air Injection Pump Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Secondary Air Injection Pump Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the secondary air injection pump output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0418.

### VSPR – PID $1367 – Vehicle Speed Output Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1367 | Vehicle Speed Output Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Vehicle Speed Output Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Vehicle Speed Output Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Vehicle Speed Output Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Vehicle Speed Output Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the 4K PPM (4,000 pulse per mile) vehicle speed output (the output is considered to be commanded on if vehicle speed is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0608.

### TCSR – PID $1369 – Delivered Torque Output Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1369 | Delivered Torque Output Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Delivered Torque Output Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Delivered Torque Output Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Delivered Torque Output Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Delivered Torque Output Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the delivered torque PWM output from the ECM to the ABS/TCS module (the delivered torque PWM output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P1689.

### RTDR – PID $136A – Vehicle Lift / Dive Output Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $136A | Vehicle Lift / Dive Output Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Vehicle Lift / Dive Output Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Vehicle Lift / Dive Output Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Vehicle Lift / Dive Output Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Vehicle Lift / Dive Output Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the vehicle lift / dive output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P1652.

### STRR – PID $136B – Starter Relay Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $136B | Starter Relay Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Starter Relay Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Starter Relay Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Starter Relay Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Starter Relay Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the starter relay output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0615.

### FPMR – PID $136C – Fuel Pump Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $136C | Fuel Pump Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Pump Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fuel Pump Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Pump Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Pump Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fuel pump output (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0230.

### ACCR – PID $136D – Air Conditioning Compressor Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $136D | Air Conditioning Compressor Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Air Conditioning Compressor Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Air Conditioning Compressor Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Air Conditioning Compressor Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Air Conditioning Compressor Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the air conditioning (A/C) compressor output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P0646, P0645, and P0647, respectively.

### FANR – PID $13AF – Cooling Fan PWM Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13AF | Cooling Fan PWM Duty Cycle | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the commanded duty cycle for the cooling fan PWM output for electric PWM or engine driven (EV or MRF) fan systems.

### TPSR – PID $13B1 – Throttle Position Sensor 1 Learned Minimum

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13B1 | Throttle Position Sensor 1 Learned Minimum | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the minimum position learned for throttle position sensor 1 when the throttle is closed.

### TPSR – PID $13B2 – Throttle Position Sensor 2 Learned Minimum

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13B2 | Throttle Position Sensor 2 Learned Minimum | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the minimum position learned for throttle position sensor 2 when the throttle is closed.

### EPSR – PID $13B5 – Engine Off Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13B5 | Engine Off Time | 2 | UNM | 0 - 65535 s | E = N * 1 |

This PID contains the time since the engine has been running (includes the time that the controller has been shutdown).

### TPSR – PID $13C6 – Throttle Actuator Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13C6 | Throttle Actuator Duty Cycle | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the commanded duty cycle for the throttle actuator output.

### EPSR – PID $13C7 – Crank Position Sensor Decode Mode

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13C7 | Crank Position Sensor Decode Mode | 1 | ENM | N/A | $07=60X-2 Threshold; $08=60X-2 Percentage; $FF=Disable |

This PID contains the method used to decode the crank position sensor signal in order to determine crankshaft synchronization.

### GENR – PID $13C8 – Generator L-Terminal Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13C8 | Generator L-Terminal Duty Cycle | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded generator L-terminal duty cycle.

### PHSR – PID $13E1 – Intake Cam Phase Bank 1 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13E1 | Intake Cam Phase Bank 1 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Intake Cam Phase Bank 1 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Intake Cam Phase Bank 1 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Intake Cam Phase Bank 1 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Intake Cam Phase Bank 1 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the intake cam phase output for bank 1 (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0010.

### PHSR – PID $13E2 – Exhaust Cam Phase Bank 1 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13E2 | Exhaust Cam Phase Bank 1 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Exhaust Cam Phase Bank 1 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Exhaust Cam Phase Bank 1 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Exhaust Cam Phase Bank 1 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Exhaust Cam Phase Bank 1 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the exhaust cam phase output for bank 1 (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0013.

### PHSR – PID $13E3 – Intake Cam Phase Bank 2 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13E3 | Intake Cam Phase Bank 2 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Intake Cam Phase Bank 2 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Intake Cam Phase Bank 2 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Intake Cam Phase Bank 2 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Intake Cam Phase Bank 2 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the intake cam phase output for bank 2 (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0020.

### PHSR – PID $13E4 – Exhaust Cam Phase Bank 2 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13E4 | Exhaust Cam Phase Bank 2 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Exhaust Cam Phase Bank 2 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Exhaust Cam Phase Bank 2 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Exhaust Cam Phase Bank 2 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Exhaust Cam Phase Bank 2 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the exhaust cam phase output for bank 2 (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0023.

### VSPR – PID $13EA – Vehicle Speed High Resolution Output Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13EA | Vehicle Speed High Resolution Output Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Vehicle Speed High Resolution Output Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Vehicle Speed High Resolution Output Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Vehicle Speed High Resolution Output Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Vehicle Speed High Resolution Output Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the 128K PPM (128,000 pulse per mile) high resolution vehicle speed output (the output is considered to be commanded on if vehicle speed is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0609.

### IFRR – PID $13F1 – Filtered MAP Model 1 Residual

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13F1 | Filtered MAP Model 1 Residual | 2 | SNM | -256 - 255.9921875 kPa | E = N / 128 |

This PID contains the filtered MAP model 1 residual (the difference between the measured intake manifold pressure and the intake manifold pressure predicted from the intake manifold model using the mass airflow predicted from the throttle model) used within the intake flow rationality diagnostic.

### IFRR – PID $13F2 – Filtered MAP Model 2 Residual

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13F2 | Filtered MAP Model 2 Residual | 2 | SNM | -256 - 255.9921875 kPa | E = N / 128 |

This PID contains the filtered MAP model 2 residual (the difference between the measured intake manifold pressure and the intake manifold pressure predicted from the intake manifold model using the measured mass airflow) used within the intake flow rationality diagnostic.

### IFRR – PID $13F3 – Filtered MAF Residual

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13F3 | Filtered MAF Residual | 2 | SNM | -512 - 511.984375 g/s | E = N / 64 |

This PID contains the filtered MAF residual (the difference between the measured mass airflow and the mass airflow predicted from the throttle model) used within the intake flow rationality diagnostic.

### IFRR – PID $13F4 – Filtered Throttle Position Residual

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $13F4 | Filtered Throttle Position Residual | 2 | SNM | -8192 - 8191.75 kPa g/s | E = N / 4 |

This PID contains the filtered throttle position residual (the product of the MAF residual and the MAP model 1 residual) used within the intake flow rationality diagnostic.

### SPKR – PID $1400 – Ignition Coil 1 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1400 | Ignition Coil 1 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Ignition Coil 1 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Ignition Coil 1 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 1 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 1 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the ignition coil 1 output (the output is considered to be commanded on if the ignition output is enabled while the engine is in sync and running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0351.

### SPKR – PID $1401 – Ignition Coil 2 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1401 | Ignition Coil 2 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Ignition Coil 2 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Ignition Coil 2 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 2 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 2 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the ignition coil 2 output (the output is considered to be commanded on if the ignition output is enabled while the engine is in sync and running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0352.

### SPKR – PID $1402 – Ignition Coil 3 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1402 | Ignition Coil 3 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Ignition Coil 3 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Ignition Coil 3 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 3 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 3 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the ignition coil 3 output (the output is considered to be commanded on if the ignition output is enabled while the engine is in sync and running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0353.

### SPKR – PID $1403 – Ignition Coil 4 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1403 | Ignition Coil 4 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Ignition Coil 4 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Ignition Coil 4 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 4 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 4 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the ignition coil 4 output (the output is considered to be commanded on if the ignition output is enabled while the engine is in sync and running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0354.

### SPKR – PID $1404 – Ignition Coil 5 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1404 | Ignition Coil 5 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Ignition Coil 5 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Ignition Coil 5 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 5 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 5 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the ignition coil 5 output (the output is considered to be commanded on if the ignition output is enabled while the engine is in sync and running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0355.

### SPKR – PID $1405 – Ignition Coil 6 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1405 | Ignition Coil 6 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Ignition Coil 6 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Ignition Coil 6 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 6 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 6 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the ignition coil 6 output (the output is considered to be commanded on if the ignition output is enabled while the engine is in sync and running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0356.

### SPKR – PID $1406 – Ignition Coil 7 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1406 | Ignition Coil 7 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Ignition Coil 7 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Ignition Coil 7 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 7 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 7 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the ignition coil 7 output (the output is considered to be commanded on if the ignition output is enabled while the engine is in sync and running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0357.

### SPKR – PID $1407 – Ignition Coil 8 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1407 | Ignition Coil 8 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Ignition Coil 8 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Ignition Coil 8 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 8 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Ignition Coil 8 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the ignition coil 8 output (the output is considered to be commanded on if the ignition output is enabled while the engine is in sync and running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0358.

### FULR – PID $1408 – Fuel Injector 1 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1408 | Fuel Injector 1 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Injector 1 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fuel Injector 1 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 1 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 1 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fuel injector 1 output (the output is considered to be commanded on if the injector is enabled while the engine is running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0201.

### FULR – PID $1409 – Fuel Injector 2 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1409 | Fuel Injector 2 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Injector 2 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fuel Injector 2 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 2 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 2 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fuel injector 2 output (the output is considered to be commanded on if the injector is enabled while the engine is running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0202.

### FULR – PID $140A – Fuel Injector 3 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $140A | Fuel Injector 3 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Injector 3 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fuel Injector 3 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 3 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 3 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fuel injector 3 output (the output is considered to be commanded on if the injector is enabled while the engine is running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0203.

### FULR – PID $140B – Fuel Injector 4 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $140B | Fuel Injector 4 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Injector 4 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fuel Injector 4 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 4 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 4 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fuel injector 4 output (the output is considered to be commanded on if the injector is enabled while the engine is running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0204.

### FULR – PID $140C – Fuel Injector 5 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $140C | Fuel Injector 5 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Injector 5 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fuel Injector 5 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 5 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 5 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fuel injector 5 output (the output is considered to be commanded on if the injector is enabled while the engine is running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0205.

### FULR – PID $140D – Fuel Injector 6 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $140D | Fuel Injector 6 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Injector 6 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fuel Injector 6 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 6 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 6 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fuel injector 6 output (the output is considered to be commanded on if the injector is enabled while the engine is running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0206.

### FULR – PID $140E – Fuel Injector 7 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $140E | Fuel Injector 7 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Injector 7 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fuel Injector 7 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 7 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 7 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fuel injector 7 output (the output is considered to be commanded on if the injector is enabled while the engine is running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0207.

### FULR – PID $140F – Fuel Injector 8 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $140F | Fuel Injector 8 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Injector 8 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fuel Injector 8 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 8 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fuel Injector 8 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fuel injector 8 output (the output is considered to be commanded on if the injector is enabled while the engine is running or cranking) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0208.

### EVPR – PID $1410 – Canister Purge Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1410 | Canister Purge Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Canister Purge Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Canister Purge Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Canister Purge Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Canister Purge Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the canister purge output (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0443.

### EVPR – PID $1412 – Canister Vent Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1412 | Canister Vent Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Canister Vent Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Canister Vent Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Canister Vent Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Canister Vent Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the canister vent output (the canister vent is commanded open when the output is commanded off) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0449.

### RINR – PID $1413 – Reverse Inhibit Solenoid Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1413 | Reverse Inhibit Solenoid Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reverse Inhibit Solenoid Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reverse Inhibit Solenoid Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Reverse Inhibit Solenoid Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Reverse Inhibit Solenoid Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the reverse inhibit solenoid output:

Reverse is inhibited when the output is commanded off in Reverse Enable applications.

Reverse is inhibited when the output is commanded on in Reverse Inhibit applications.

This PID also contains the status of the reverse inhibit solenoid associated short to ground, open circuit, and short to power tests used to report DTC P0801.

### SKSR – PID $1414 – Skip Shift Solenoid Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1414 | Skip Shift Solenoid Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Skip Shift Solenoid Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Skip Shift Solenoid Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Skip Shift Solenoid Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Skip Shift Solenoid Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the skip shift solenoid output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P080C, P0803, and P080D, respectively.

### FANR – PID $141D – Fan Relay 1 / Cooling Fan PWM Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $141D | Fan Relay 1 / Cooling Fan PWM Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fan Relay 1 / Cooling Fan PWM Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fan Relay 1 / Cooling Fan PWM Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fan Relay 1 / Cooling Fan PWM Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fan Relay 1 / Cooling Fan PWM Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fan relay 1 / cooling fan PWM output (the cooling fan PWM output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0480.

### FANR – PID $141E – Fan Relay 2 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $141E | Fan Relay 2 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fan Relay 2 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fan Relay 2 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fan Relay 2 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fan Relay 2 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fan relay 2 output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0481.

### EPSR – PID $1420 – Primary Cam Position Sensor Engine Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1420 | Primary Cam Position Sensor Engine Speed | 2 | UNM | 0 - 65535 rpm | E = N * 1 |

This PID contains the engine speed determined from the period of the primary cam position sensor.

### EPSR – PID $1421 – Crank Position Sensor Medium Resolution Engine Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1421 | Crank Position Sensor Medium Resolution Engine Speed | 2 | UNM | 0 - 65535 rpm | E = N * 1 |

This PID contains the engine speed determined from the medium resolution (MEDRES) period of the crank position sensor.

### OXYR – PID $146A – Oxygen Sensor Heater Bank 1 Sensor 1 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $146A | Oxygen Sensor Heater Bank 1 Sensor 1 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Oxygen Sensor Heater Bank 1 Sensor 1 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Oxygen Sensor Heater Bank 1 Sensor 1 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Oxygen Sensor Heater Bank 1 Sensor 1 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Oxygen Sensor Heater Bank 1 Sensor 1 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the oxygen sensor heater output for bank 1 sensor 1 (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0030.

### OXYR – PID $146B – Oxygen Sensor Heater Bank 1 Sensor 2 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $146B | Oxygen Sensor Heater Bank 1 Sensor 2 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Oxygen Sensor Heater Bank 1 Sensor 2 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Oxygen Sensor Heater Bank 1 Sensor 2 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Oxygen Sensor Heater Bank 1 Sensor 2 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Oxygen Sensor Heater Bank 1 Sensor 2 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the oxygen sensor heater output for bank 1 sensor 2 (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0036.

### OXYR – PID $146C – Oxygen Sensor Heater Bank 2 Sensor 1 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $146C | Oxygen Sensor Heater Bank 2 Sensor 1 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Oxygen Sensor Heater Bank 2 Sensor 1 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Oxygen Sensor Heater Bank 2 Sensor 1 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Oxygen Sensor Heater Bank 2 Sensor 1 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Oxygen Sensor Heater Bank 2 Sensor 1 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the oxygen sensor heater output for bank 2 sensor 1 (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0050.

### OXYR – PID $146D – Oxygen Sensor Heater Bank 2 Sensor 2 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $146D | Oxygen Sensor Heater Bank 2 Sensor 2 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Oxygen Sensor Heater Bank 2 Sensor 2 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Oxygen Sensor Heater Bank 2 Sensor 2 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Oxygen Sensor Heater Bank 2 Sensor 2 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Oxygen Sensor Heater Bank 2 Sensor 2 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the oxygen sensor heater output for bank 2 sensor 2 (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0056.

### EOPR – PID $1470 – Engine Oil Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1470 | Engine Oil Pressure | 1 | UNM | 0 - 1020 kPa | E = N * 4 |

This PID contains the undefaulted, unfiltered engine oil pressure measured by a sensor.

### VLTR – PID $1478 – Reference Voltage A

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1478 | Reference Voltage A | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the Vref 1 value from the MCP.

### VLTR – PID $1479 – Reference Voltage B

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1479 | Reference Voltage B | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the Vref 2 value from the MCP.

### FANR – PID $147B – Fan Relay 3 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $147B | Fan Relay 3 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Fan Relay 3 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Fan Relay 3 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fan Relay 3 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Fan Relay 3 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the fan relay 3 output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0482.

### OXYR – PID $147F – Oxygen Sensor Heater Duty Cycle Bank 1 Sensor 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $147F | Oxygen Sensor Heater Duty Cycle Bank 1 Sensor 1 | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the commanded duty cycle for the oxygen sensor heater output for bank 1 sensor 1.

### OXYR – PID $1480 – Oxygen Sensor Heater Duty Cycle Bank 2 Sensor 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1480 | Oxygen Sensor Heater Duty Cycle Bank 2 Sensor 1 | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the commanded duty cycle for the oxygen sensor heater output for bank 2 sensor 1.

### OXYR – PID $1481 – Oxygen Sensor Heater Current Bank 1 Sensor 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1481 | Oxygen Sensor Heater Current Bank 1 Sensor 1 | 2 | UNM | 0 - 1310.7 A | E = N * 0.02 |

This PID contains the current feedback for the oxygen sensor heater output for bank 1 sensor 1.

### OXYR – PID $1482 – Oxygen Sensor Heater Current Bank 1 Sensor 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1482 | Oxygen Sensor Heater Current Bank 1 Sensor 2 | 2 | UNM | 0 - 1310.7 A | E = N * 0.02 |

This PID contains the current feedback for the oxygen sensor heater output for bank 1 sensor 2.

### OXYR – PID $1484 – Oxygen Sensor Heater Current Bank 2 Sensor 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1484 | Oxygen Sensor Heater Current Bank 2 Sensor 1 | 2 | UNM | 0 - 1310.7 A | E = N * 0.02 |

This PID contains the current feedback for the oxygen sensor heater output for bank 2 sensor 1.

### OXYR – PID $1485 – Oxygen Sensor Heater Current Bank 2 Sensor 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1485 | Oxygen Sensor Heater Current Bank 2 Sensor 2 | 2 | UNM | 0 - 1310.7 A | E = N * 0.02 |

This PID contains the current feedback for the oxygen sensor heater output for bank 2 sensor 2.

### EVPR – PID $1489 – Fuel Tank Vapor Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1489 | Fuel Tank Vapor Temperature | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the undefaulted fuel tank vapor temperature measured by a sensor.

### PMDR – PID $148C – Powertrain Relay Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $148C | Powertrain Relay Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Powertrain Relay Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Powertrain Relay Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Powertrain Relay Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Powertrain Relay Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the powertrain relay output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0685.

### PMDR – PID $148D – Powertrain Relay Voltage

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $148D | Powertrain Relay Voltage | 1 | UNM | 0 - 25.5 V | E = N * 0.1 |

This PID contains the voltage of the powertrain relay feedback to the controller.

### FPMR – PID $148E – Variable Speed Fuel Pump Diagnostic Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $148E | Variable Speed Fuel Pump Diagnostic Duty Cycle | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the duty cycle of the PWM feedback from the variable speed fuel pump control module, which indicates the diagnostic status of the module.

### FLVR – PID $1502 – Secondary Fuel Pump Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1502 | Secondary Fuel Pump Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Secondary Fuel Pump Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Secondary Fuel Pump Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Secondary Fuel Pump Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Secondary Fuel Pump Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the secondary fuel pump output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P2632.

### GENR – PID $1503 – Generator L-Terminal Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1503 | Generator L-Terminal Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Generator L-Terminal Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Generator L-Terminal Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Generator L-Terminal Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Generator L-Terminal Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the generator L-terminal output (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0621.

In generator electrical architectures that support regulated voltage control (RVC), the generator L-terminal duty cycle is only commanded to zero if the engine is not running.  In generator electrical architectures that do not support RVC, the generator L-terminal is commanded off and on by commanding duty cycles of 0% and 100%, respectively.

When the Generator L-Terminal Short to Ground Test Status has a value of Failed, the generator L-terminal has either shorted to ground or has been pulled to ground by the generator (the generator pulls the L-terminal to ground when the generator has detected an internal failure).

### TPSR – PID $150A – Maximum Throttle Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $150A | Maximum Throttle Position | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the maximum throttle position observed during the wide open throttle (WOT) self test device control.

### CRZR – PID $150C – Cruise Control Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $150C | Cruise Control Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Cruise Control Switch Fault | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Cruise Control Active | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Cruise Control Cancel Switch Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Cruise Control Speed Decrease Switch Active | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Cruise Control Speed Increase Switch Active | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Cruise Control Resume Switch Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Cruise Control Set Switch Active | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Cruise Control On Switch Active | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the cruise control system.  The content of each of these signals is described below.

Cruise Control Switch Fault indicates that a failure of the cruise control switches has been detected (and a corresponding DTC has been stored) within the platform electronics.  In GMLAN cruise control electrical architectures, this signal is based upon the failure information for the cruise control switches received over serial communication from the platform electronics.  A value of False will always be provided for this signal in Class 2 cruise control electrical architectures.

Cruise Control Active indicates that the cruise control system is in an active cruise control mode (i.e., the vehicle’s speed is being controlled).

Cruise Control Cancel Switch Active indicates the state of the cruise control cancel switch.  In cruise control electrical architectures from PPEI 3.x or later, this signal is based upon the state of the cruise control cancel switch received over serial communication from the platform electronics.  In cruise control electrical architectures that precede PPEI 3.x, this signal is based upon the state of the cruise control cancel request, which is based upon the state of the cruise control cancel switch as well as other criteria, received over serial communication from the platform electronics.

Cruise Control Speed Decrease Switch Active indicates the state of the cruise control speed decrease switch.  In cruise control electrical architectures from PPEI 3.x or later, this signal is based upon the state of the cruise control speed decrease switch received over serial communication from the platform electronics.  In cruise control electrical architectures that precede PPEI 3.x, a value of False will always be provided for this signal.

Cruise Control Speed Increase Switch Active indicates the state of the cruise control speed increase switch.  In cruise control electrical architectures from PPEI 3.x or later, this signal is based upon the state of the cruise control speed increase switch received over serial communication from the platform electronics.  In cruise control electrical architectures that precede PPEI 3.x, a value of False will always be provided for this signal.

Cruise Control Resume Switch Active indicates the state of the cruise control resume switch.  In GMLAN cruise control electrical architectures, this signal is based upon the state of the cruise control resume switch received over serial communication from the platform electronics.  In Class 2 cruise control electrical architectures, this signal is based upon the hardwire cruise switch input to the ECM.

Cruise Control Set Switch Active indicates the state of the cruise control set switch.  In GMLAN cruise control electrical architectures, this signal is based upon the state of the cruise control set switch received over serial communication from the platform electronics.  In Class 2 cruise control electrical architectures, this signal is based upon the hardwire cruise switch input to the ECM.

Cruise Control On Switch Active indicates the state of the cruise control on switch.  In GMLAN cruise control electrical architectures, this signal is based upon the state of the cruise control on switch received over serial communication from the platform electronics.  In Class 2 cruise control electrical architectures, this signal is based upon the hardwire cruise switch input to the ECM.

### BBVR – PID $150D – Brake Booster Vacuum Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $150D | Brake Booster Vacuum Pressure | 1 | UNM | 10.3321 - 104.4280 kPa | E = (N + 28) / 2.71 |

This PID contains the undefaulted, unfiltered brake booster vacuum pressure measured by a sensor.

### ACCR – PID $150E – Air Conditioning Disengage History 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $150E | Air Conditioning Disengage History 1 | 1 | ENM | N/A | $00=None; $01=AC Protection – High Side Fluid Pressure; $02=AC Protection – Engine Speed; $03=AC Protection – Battery Voltage; $04=Engine Stall Prevention; $05=Engine Speed Stabilization; $06=Wide Open Throttle; $07=Vehicle Launch Performance; $08=High Coolant Temperature; $09=Reserved; $0A=Reserved; $0B=Reserved; $0C=Reserved; $0D=Reserved; $0E=AC Relay Fault |

This PID contains one of the last eight reasons that powertrain has disengaged the air conditioning compressor.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.  If powertrain has already commanded disengagement of the air conditioning compressor for a reason other than a platform request, and a new disengagement reason exists, the new reason is placed in the history buffer.

### ACCR – PID $150F – Air Conditioning Disengage History 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $150F | Air Conditioning Disengage History 2 | 1 | ENM | N/A | $00=None; $01=AC Protection – High Side Fluid Pressure; $02=AC Protection – Engine Speed; $03=AC Protection – Battery Voltage; $04=Engine Stall Prevention; $05=Engine Speed Stabilization; $06=Wide Open Throttle; $07=Vehicle Launch Performance; $08=High Coolant Temperature; $09=Reserved; $0A=Reserved; $0B=Reserved; $0C=Reserved; $0D=Reserved; $0E=AC Relay Fault |

This PID contains one of the last eight reasons that powertrain has disengaged the air conditioning compressor.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.  If powertrain has already commanded disengagement of the air conditioning compressor for a reason other than a platform request, and a new disengagement reason exists, the new reason is placed in the history buffer.

### ACCR – PID $1510 – Air Conditioning Disengage History 3

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1510 | Air Conditioning Disengage History 3 | 1 | ENM | N/A | $00=None; $01=AC Protection – High Side Fluid Pressure; $02=AC Protection – Engine Speed; $03=AC Protection – Battery Voltage; $04=Engine Stall Prevention; $05=Engine Speed Stabilization; $06=Wide Open Throttle; $07=Vehicle Launch Performance; $08=High Coolant Temperature; $09=Reserved; $0A=Reserved; $0B=Reserved; $0C=Reserved; $0D=Reserved; $0E=AC Relay Fault |

This PID contains one of the last eight reasons that powertrain has disengaged the air conditioning compressor.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.  If powertrain has already commanded disengagement of the air conditioning compressor for a reason other than a platform request, and a new disengagement reason exists, the new reason is placed in the history buffer.

### ACCR – PID $1511 – Air Conditioning Disengage History 4

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1511 | Air Conditioning Disengage History 4 | 1 | ENM | N/A | $00=None; $01=AC Protection – High Side Fluid Pressure; $02=AC Protection – Engine Speed; $03=AC Protection – Battery Voltage; $04=Engine Stall Prevention; $05=Engine Speed Stabilization; $06=Wide Open Throttle; $07=Vehicle Launch Performance; $08=High Coolant Temperature; $09=Reserved; $0A=Reserved; $0B=Reserved; $0C=Reserved; $0D=Reserved; $0E=AC Relay Fault |

This PID contains one of the last eight reasons that powertrain has disengaged the air conditioning compressor.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.  If powertrain has already commanded disengagement of the air conditioning compressor for a reason other than a platform request, and a new disengagement reason exists, the new reason is placed in the history buffer.

### AIRR – PID $1517 – Secondary Air Injection Diagnostic Test State

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1517 | Secondary Air Injection Diagnostic Test State | 1 | ENM | N/A | $00=SAI Diagnostic Disabled; $01=Phase 1 – SAI On; $02=Phase 2 – SAI Valve Shut; $03=Phase 3 – SAI Pump Off; $04=SAI Diagnostic Test Complete |

This PID contains the current state of the secondary air injection (SAI) diagnostic.  These states are interpreted in the following manner.

| State | Description |
| --- | --- |
| SAI Diagnostic Disabled | The secondary air injection diagnostic is disabled. |
| Phase 1 – SAI On | The secondary air injection on test (incorrect airflow) is in progress. |
| Phase 2 – SAI Valve Shut | The secondary air injection valve shut test (valve stuck open) is in progress. |
| Phase 3 – SAI Pump Off | The secondary air injection pump off test (pump stuck open) is in progress. |
| SAI Diagnostic Test Complete | The secondary air injection diagnostic has been completed or aborted. |

### AIRR – PID $1518 – Secondary Air Injection System Pressure Error Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1518 | Secondary Air Injection System Pressure Error Bank 1 | 2 | SNM | -25 - 24.99923706055 kPa | E = N * 25 / 32768 |

This PID contains the weighted average pressure error (difference between the predicted and actual secondary air injection system pressure) for bank 1 during the secondary air injection on test (phase 1).

### AIRR – PID $1519 – Secondary Air Injection System Pressure Error Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1519 | Secondary Air Injection System Pressure Error Bank 2 | 2 | SNM | -25 - 24.99923706055 kPa | E = N * 25 / 32768 |

This PID contains the weighted average pressure error (difference between the predicted and actual secondary air injection system pressure) for bank 2 during the secondary air injection on test (phase 1).

### AIRR – PID $151A – Secondary Air Injection Normalized System Pressure Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $151A | Secondary Air Injection Normalized System Pressure Bank 1 | 1 | UNM | 0 - 24.90234375 kPa | E = N * 25 / 256 |

This PID contains the normalized secondary air injection system pressure for bank 1.  The normalized pressure is the filtered pressure minus the baseline pressure (the ambient air pressure measured by the sensor).

### AIRR – PID $151B – Secondary Air Injection Normalized System Pressure Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $151B | Secondary Air Injection Normalized System Pressure Bank 2 | 1 | UNM | 0 - 24.90234375 kPa | E = N * 25 / 256 |

This PID contains the normalized secondary air injection system pressure for bank 2.  The normalized pressure is the filtered pressure minus the baseline pressure (the ambient air pressure measured by the sensor).

### AIRR – PID $151C – Secondary Air Injection System Pressure Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $151C | Secondary Air Injection System Pressure Bank 1 | 1 | UNM | 40 - 149.5703125 kPa | E = N * (110 / 256) + 40 |

This PID contains the unfiltered secondary air injection system pressure for bank 1.

### AIRR – PID $151D – Secondary Air Injection System Pressure Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $151D | Secondary Air Injection System Pressure Bank 2 | 1 | UNM | 40 - 149.5703125 kPa | E = N * (110 / 256) + 40 |

This PID contains the unfiltered secondary air injection system pressure for bank 2.

### ACCR – PID $151F – Air Conditioning Disengage History 5

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $151F | Air Conditioning Disengage History 5 | 1 | ENM | N/A | $00=None; $01=AC Protection – High Side Fluid Pressure; $02=AC Protection – Engine Speed; $03=AC Protection – Battery Voltage; $04=Engine Stall Prevention; $05=Engine Speed Stabilization; $06=Wide Open Throttle; $07=Vehicle Launch Performance; $08=High Coolant Temperature; $09=Reserved; $0A=Reserved; $0B=Reserved; $0C=Reserved; $0D=Reserved; $0E=AC Relay Fault |

This PID contains one of the last eight reasons that powertrain has disengaged the air conditioning compressor.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.  If powertrain has already commanded disengagement of the air conditioning compressor for a reason other than a platform request, and a new disengagement reason exists, the new reason is placed in the history buffer.

### ACCR – PID $1520 – Air Conditioning Disengage History 6

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1520 | Air Conditioning Disengage History 6 | 1 | ENM | N/A | $00=None; $01=AC Protection – High Side Fluid Pressure; $02=AC Protection – Engine Speed; $03=AC Protection – Battery Voltage; $04=Engine Stall Prevention; $05=Engine Speed Stabilization; $06=Wide Open Throttle; $07=Vehicle Launch Performance; $08=High Coolant Temperature; $09=Reserved; $0A=Reserved; $0B=Reserved; $0C=Reserved; $0D=Reserved; $0E=AC Relay Fault |

This PID contains one of the last eight reasons that powertrain has disengaged the air conditioning compressor.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.  If powertrain has already commanded disengagement of the air conditioning compressor for a reason other than a platform request, and a new disengagement reason exists, the new reason is placed in the history buffer.

### ACCR – PID $1521 – Air Conditioning Disengage History 7

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1521 | Air Conditioning Disengage History 7 | 1 | ENM | N/A | $00=None; $01=AC Protection – High Side Fluid Pressure; $02=AC Protection – Engine Speed; $03=AC Protection – Battery Voltage; $04=Engine Stall Prevention; $05=Engine Speed Stabilization; $06=Wide Open Throttle; $07=Vehicle Launch Performance; $08=High Coolant Temperature; $09=Reserved; $0A=Reserved; $0B=Reserved; $0C=Reserved; $0D=Reserved; $0E=AC Relay Fault |

This PID contains one of the last eight reasons that powertrain has disengaged the air conditioning compressor.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.  If powertrain has already commanded disengagement of the air conditioning compressor for a reason other than a platform request, and a new disengagement reason exists, the new reason is placed in the history buffer.

### ACCR – PID $1522 – Air Conditioning Disengage History 8

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1522 | Air Conditioning Disengage History 8 | 1 | ENM | N/A | $00=None; $01=AC Protection – High Side Fluid Pressure; $02=AC Protection – Engine Speed; $03=AC Protection – Battery Voltage; $04=Engine Stall Prevention; $05=Engine Speed Stabilization; $06=Wide Open Throttle; $07=Vehicle Launch Performance; $08=High Coolant Temperature; $09=Reserved; $0A=Reserved; $0B=Reserved; $0C=Reserved; $0D=Reserved; $0E=AC Relay Fault |

This PID contains one of the last eight reasons that powertrain has disengaged the air conditioning compressor.  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.  If powertrain has already commanded disengagement of the air conditioning compressor for a reason other than a platform request, and a new disengagement reason exists, the new reason is placed in the history buffer.

### FLVR – PID $152C – Fuel Level Sensor Power Supply Percent On-Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $152C | Fuel Level Sensor Power Supply Percent On-Time | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the percentage of time that the power supply for the fuel level sensor(s) is on.  This percentage is minimized when the fuel has high ethanol content and is maximized when the fuel has low ethanol content.

### CDAR – PID $1539 – Displacement on Demand Solenoid A Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1539 | Displacement on Demand Solenoid A Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Displacement on Demand Solenoid A Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Displacement on Demand Solenoid A Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Displacement on Demand Solenoid A Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Displacement on Demand Solenoid A Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the displacement on demand solenoid A output along with the status of its associated short to ground, open circuit, and short to power tests used to report a DTC for the appropriate cylinder number.

### CDAR – PID $153A – Displacement on Demand Solenoid B Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $153A | Displacement on Demand Solenoid B Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Displacement on Demand Solenoid B Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Displacement on Demand Solenoid B Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Displacement on Demand Solenoid B Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Displacement on Demand Solenoid B Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the displacement on demand solenoid B output along with the status of its associated short to ground, open circuit, and short to power tests used to report a DTC for the appropriate cylinder number.

### CDAR – PID $153B – Displacement on Demand Solenoid C Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $153B | Displacement on Demand Solenoid C Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Displacement on Demand Solenoid C Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Displacement on Demand Solenoid C Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Displacement on Demand Solenoid C Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Displacement on Demand Solenoid C Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the displacement on demand solenoid C output along with the status of its associated short to ground, open circuit, and short to power tests used to report a DTC for the appropriate cylinder number.

### CDAR – PID $153C – Displacement on Demand Solenoid D Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $153C | Displacement on Demand Solenoid D Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Displacement on Demand Solenoid D Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Displacement on Demand Solenoid D Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Displacement on Demand Solenoid D Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Displacement on Demand Solenoid D Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the displacement on demand solenoid D output along with the status of its associated short to ground, open circuit, and short to power tests used to report a DTC for the appropriate cylinder number.

### EOTR – PID $153E – Engine Oil Temperature from Sensor

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $153E | Engine Oil Temperature from Sensor | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the undefaulted engine oil temperature as measured by a sensor connected to the ECM.

### AIRR – PID $1549 – Secondary Air Injection System Pressure A/D Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1549 | Secondary Air Injection System Pressure A/D Bank 1 | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the secondary air injection system pressure analog input for bank 1 as a percentage of its reference voltage.

### AIRR – PID $1550 – Secondary Air Injection System Pressure A/D Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1550 | Secondary Air Injection System Pressure A/D Bank 2 | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the secondary air injection system pressure analog input for bank 2 as a percentage of its reference voltage.

### RVSR – PID $1551 – Remote Vehicle Start Disable History 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1551 | Remote Vehicle Start Disable History 1 | 1 | ENM | N/A | $00=None; $01=Engine Speed High; $02=Engine Over Temperature; $03=Crank Aborted; $04=Engine Oil Pressure Low; $05=Maximum Crank Time Exceeded; $06=Malfunction Indicator Lamp Illuminated; $07=RVS Backup Timer Expired; $08=Accelerator Pedal Position High; $09=Transmission Range Not Park; $0A=Vehicle Speed Not Zero; $0B=Vehicle Theft Disable; $0C=Transfer Case In Neutral; $0D=Remaining RVS Count Equals Zero; $0E=Low Fuel Level |

This PID contains one of the last eight reasons that powertrain has disabled remote vehicle start (RVS).  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### RVSR – PID $1552 – Remote Vehicle Start Disable History 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1552 | Remote Vehicle Start Disable History 2 | 1 | ENM | N/A | $00=None; $01=Engine Speed High; $02=Engine Over Temperature; $03=Crank Aborted; $04=Engine Oil Pressure Low; $05=Maximum Crank Time Exceeded; $06=Malfunction Indicator Lamp Illuminated; $07=RVS Backup Timer Expired; $08=Accelerator Pedal Position High; $09=Transmission Range Not Park; $0A=Vehicle Speed Not Zero; $0B=Vehicle Theft Disable; $0C=Transfer Case In Neutral; $0D=Remaining RVS Count Equals Zero; $0E=Low Fuel Level |

This PID contains one of the last eight reasons that powertrain has disabled remote vehicle start (RVS).  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### RVSR – PID $1553 – Remote Vehicle Start Disable History 3

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1553 | Remote Vehicle Start Disable History 3 | 1 | ENM | N/A | $00=None; $01=Engine Speed High; $02=Engine Over Temperature; $03=Crank Aborted; $04=Engine Oil Pressure Low; $05=Maximum Crank Time Exceeded; $06=Malfunction Indicator Lamp Illuminated; $07=RVS Backup Timer Expired; $08=Accelerator Pedal Position High; $09=Transmission Range Not Park; $0A=Vehicle Speed Not Zero; $0B=Vehicle Theft Disable; $0C=Transfer Case In Neutral; $0D=Remaining RVS Count Equals Zero; $0E=Low Fuel Level |

This PID contains one of the last eight reasons that powertrain has disabled remote vehicle start (RVS).  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### RVSR – PID $1554 – Remote Vehicle Start Disable History 4

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1554 | Remote Vehicle Start Disable History 4 | 1 | ENM | N/A | $00=None; $01=Engine Speed High; $02=Engine Over Temperature; $03=Crank Aborted; $04=Engine Oil Pressure Low; $05=Maximum Crank Time Exceeded; $06=Malfunction Indicator Lamp Illuminated; $07=RVS Backup Timer Expired; $08=Accelerator Pedal Position High; $09=Transmission Range Not Park; $0A=Vehicle Speed Not Zero; $0B=Vehicle Theft Disable; $0C=Transfer Case In Neutral; $0D=Remaining RVS Count Equals Zero; $0E=Low Fuel Level |

This PID contains one of the last eight reasons that powertrain has disabled remote vehicle start (RVS).  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### RVSR – PID $1555 – Remote Vehicle Start Disable History 5

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1555 | Remote Vehicle Start Disable History 5 | 1 | ENM | N/A | $00=None; $01=Engine Speed High; $02=Engine Over Temperature; $03=Crank Aborted; $04=Engine Oil Pressure Low; $05=Maximum Crank Time Exceeded; $06=Malfunction Indicator Lamp Illuminated; $07=RVS Backup Timer Expired; $08=Accelerator Pedal Position High; $09=Transmission Range Not Park; $0A=Vehicle Speed Not Zero; $0B=Vehicle Theft Disable; $0C=Transfer Case In Neutral; $0D=Remaining RVS Count Equals Zero; $0E=Low Fuel Level |

This PID contains one of the last eight reasons that powertrain has disabled remote vehicle start (RVS).  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### RVSR – PID $1556 – Remote Vehicle Start Disable History 6

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1556 | Remote Vehicle Start Disable History 6 | 1 | ENM | N/A | $00=None; $01=Engine Speed High; $02=Engine Over Temperature; $03=Crank Aborted; $04=Engine Oil Pressure Low; $05=Maximum Crank Time Exceeded; $06=Malfunction Indicator Lamp Illuminated; $07=RVS Backup Timer Expired; $08=Accelerator Pedal Position High; $09=Transmission Range Not Park; $0A=Vehicle Speed Not Zero; $0B=Vehicle Theft Disable; $0C=Transfer Case In Neutral; $0D=Remaining RVS Count Equals Zero; $0E=Low Fuel Level |

This PID contains one of the last eight reasons that powertrain has disabled remote vehicle start (RVS).  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### RVSR – PID $1557 – Remote Vehicle Start Disable History 7

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1557 | Remote Vehicle Start Disable History 7 | 1 | ENM | N/A | $00=None; $01=Engine Speed High; $02=Engine Over Temperature; $03=Crank Aborted; $04=Engine Oil Pressure Low; $05=Maximum Crank Time Exceeded; $06=Malfunction Indicator Lamp Illuminated; $07=RVS Backup Timer Expired; $08=Accelerator Pedal Position High; $09=Transmission Range Not Park; $0A=Vehicle Speed Not Zero; $0B=Vehicle Theft Disable; $0C=Transfer Case In Neutral; $0D=Remaining RVS Count Equals Zero; $0E=Low Fuel Level |

This PID contains one of the last eight reasons that powertrain has disabled remote vehicle start (RVS).  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### RVSR – PID $1558 – Remote Vehicle Start Disable History 8

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1558 | Remote Vehicle Start Disable History 8 | 1 | ENM | N/A | $00=None; $01=Engine Speed High; $02=Engine Over Temperature; $03=Crank Aborted; $04=Engine Oil Pressure Low; $05=Maximum Crank Time Exceeded; $06=Malfunction Indicator Lamp Illuminated; $07=RVS Backup Timer Expired; $08=Accelerator Pedal Position High; $09=Transmission Range Not Park; $0A=Vehicle Speed Not Zero; $0B=Vehicle Theft Disable; $0C=Transfer Case In Neutral; $0D=Remaining RVS Count Equals Zero; $0E=Low Fuel Level |

This PID contains one of the last eight reasons that powertrain has disabled remote vehicle start (RVS).  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### IMOR – PID $1559 – Powertrain / Immobilizer Response Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1559 | Powertrain / Immobilizer Response Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Powertrain Authentication Status | 0 | 7 | 2 | ENM | N/A | $0=Unknown; $1=Incorrect; $2=Negative; $3=Valid |
| Powertrain Response Source | 0 | 5 | 2 | ENM | N/A | $0=Normal Start; $1=Remote Vehicle Start; $2=PTO Remote Start |
| Immobilizer Response Status | 0 | 3 | 2 | ENM | N/A | $0=No Information; $1=In Process; $2=Negative Response; $3=Valid |
| Immobilizer Response Source | 0 | 1 | 2 | ENM | N/A | $0=Normal Start; $1=Remote Vehicle Start; $2=PTO Remote Start |

This PID contains several signals associated with the status of the powertrain response and the immobilizer response within the PPEI 3 immobilizer system.  The content of each of these signals is described below.

Powertrain Authentication Status indicates the result of the comparison between the last received immobilizer response and the expected response calculated by powertrain.

Powertrain Response Source indicates the source used to calculate the powertrain response.

Immobilizer Response Status indicates the status of the immobilizer response received over serial communication from the immobilizer module.

Immobilizer Response Source indicates the source used to calculate the immobilizer response.

### IMOR – PID $155A – Immobilizer Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $155A | Immobilizer Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Learning Enabled | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Secret Information Programmed | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Powertrain Challenge Valid | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Powertrain Immobilizer Status | 0 | 2 | 3 | ENM | N/A | $0=Standby; $1=Immobilized at Startup; $2=Immobilized Due to No Response; $3=Immobilized Due to Negative Response; $4=Immobilized Due to Incorrect Response; $5=Post Release; $6=Pre Release; $7=Released |

This PID contains several signals associated with the status of the PPEI 3 immobilizer system.  The content of each of these signals is described below.

Learning Enabled indicates that learning of immobilizer secret information is enabled.

Secret Information Programmed indicates that the immobilizer secret information (e.g., powertrain pre release password, powertrain secret key) has been programmed.

Powertrain Challenge Valid indicates that the currently calculated powertrain challenge is valid.

Powertrain Immobilizer Status indicates the current state of the immobilizer algorithm.

### IMOR – PID $155B – Immobilizer Security Code Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $155B | Immobilizer Security Code Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Security Code Lockout Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Security Code Programmed | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Security Code Accepted | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the security code associated with the PPEI 3 immobilizer system.  The content of each of these signals is described below.

Security Code Lockout Active indicates that the security code has not passed and lockout is active.

Security Code Programmed indicates that the security code has been programmed.

Security Code Accepted indicates that the received security code is correct.

### IMOR – PID $155C – Autolearn Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $155C | Autolearn Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of autolearn cycles that have been successfully completed within the PPEI 3 immobilizer system.

### IMOR – PID $155D – Autolearn Delay Timer

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $155D | Autolearn Delay Timer | 1 | UNM | 0 - 2550 s | E = N * 10 |

This PID contains the time elapsed within the current autolearn cycle within the PPEI 3 immobilizer system.

### IMOR – PID $155E – Tool Learn Delay Timer

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $155E | Tool Learn Delay Timer | 1 | UNM | 0 - 2550 s | E = N * 10 |

This PID contains the time elapsed since the receipt of the correct security code within the PPEI 3 immobilizer system.

### IMOR – PID $155F – Immobilizer Pre Release Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $155F | Immobilizer Pre Release Time | 1 | UNM | 0 - 25.5 s | E = N * 0.1 |

This PID contains the elapsed authentication time that engine operation has been enabled by the pre release password functionality within the PPEI 3 immobilizer system.

### AIRR – PID $1560 – Secondary Air Injection Diagnostic Results

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1560 | Secondary Air Injection Diagnostic Results | 1 | ENM | N/A | $00=Test in Progress; $01=Test Aborted; $02=Test Passed; $03=Test Failed - SAI Incorrect Airflow; $04=Test Failed - SAI Valve Stuck Open; $05=Test Failed - SAI Pump Stuck On; $06=No Result; $07=Error |

This PID contains the results of the secondary air injection (SAI) diagnostic for use by the service and inspection and maintenance communities.  The PID provides information from the on-board diagnostic to aid a technician in diagnosing the system or determining why the diagnostic has not provided a result.  The diagnostic results are interpreted in the following manner.

| State | Description |
| --- | --- |
| Test in Progress | The SAI diagnostic is in progress. |
| Test Aborted | The SAI diagnostic has been aborted due to active status of one or more faults. |
| Test Passed | The SAI diagnostic test has been completed and has passed all tests. |
| Test Failed - SAI Incorrect Airflow | The SAI diagnostic has detected an incorrect secondary airflow failure condition. |
| Test Failed - SAI Valve Stuck Open | The SAI diagnostic has detected a valve stuck open failure condition. |
| Test Failed - SAI Pump Stuck On | The SAI diagnostic has detected a pump stuck on failure condition. |
| No Result | The SAI diagnostic was not completed due to inadequate test conditions. |
| Error | The SAI diagnostic cannot determine the proper result to report. |

### EOPR – PID $1561 – Predicted Engine Oil Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1561 | Predicted Engine Oil Pressure | 1 | UNM | 0 - 1020 kPa | E = N * 4 |

This PID contains the engine oil pressure predicted by an empirical model, as a function of engine oil temperature and engine speed, for use within the engine oil pressure rationality diagnostic.

### EOPR – PID $1563 – Filtered Engine Oil Pressure Weighted Residual

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1563 | Filtered Engine Oil Pressure Weighted Residual | 2 | SNM | -1024 - 1023.96875 kPa | E = N * 0.03125 |

This PID contains the filtered engine oil pressure weighted residual (the difference between the measured and the predicted engine oil pressure) used within the engine oil pressure rationality diagnostic.

### ACCR – PID $1564 – Air Conditioning Refrigerant High Side Fluid Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1564 | Air Conditioning Refrigerant High Side Fluid Pressure | 1 | UNM | 0 - 3570 kPa | E = N * 14 |

This PID contains the undefaulted air conditioning refrigerant high side fluid pressure measured by a sensor.

### EVPR – PID $1565 – Evaporative Emissions Service Bay Test Denial Reason

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1565 | Evaporative Emissions Service Bay Test Denial Reason | 1 | ENM | N/A | $00=Test Not Denied; $01=Test Not Available; $02=IM Readiness Set; $03=Engine Running; $04=Coolant Temperature High; $05=Enable Criteria Not Met; $06=EONV Malfunction Code Present |

This PID contains the reason that entry into the evaporative emissions diagnostic service bay test was denied.

### EVPR – PID $1566 – Engine Off Natural Vacuum Service Bay Test Abort Reason

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1566 | Engine Off Natural Vacuum Service Bay Test Abort Reason | 1 | ENM | N/A | $00=Test Not Aborted; $01=System Voltage Low; $02=Fuel Tank Vapor Temperature Out of Range; $03=Fuel Volatility High; $04=Refueling Event Detected; $05=Fuel Level Out Of Range |

This PID contains the reason that the engine off natural vacuum (EONV) diagnostic service bay test was aborted.

### EVPR – PID $1567 – Engine Off Natural Vacuum Service Test Denial Reason

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1567 | Engine Off Natural Vacuum Service Test Denial Reason | 1 | ENM | N/A | $00=Test Not Denied; $01=General Enable Criteria Not Met; $02=Distance Enable Criteria Not Met; $03=Engine Run Time Enable Criteria Not Met; $04=Ambient Temperature High; $05=Ambient Temperature Low; $06=Fuel Level Out Of Range; $07=System Voltage Low |

This PID indicates the enable criteria that was not met which resulted in the denial of the request to run the service bay test

### EVPR – PID $1568 – Engine Off Natural Vacuum Service Bay Test Result

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1568 | Engine Off Natural Vacuum Service Bay Test Result | 1 | ENM | N/A | $00=No Result; $01=Passed; $02=Failed; $03=Aborted |

This PID contains the result of the engine off natural vacuum (EONV) diagnostic service bay test.

### EVPR – PID $1569 – Remaining Distance to Enable Engine Off Natural Vacuum Service Bay Test

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1569 | Remaining Distance to Enable Engine Off Natural Vacuum Service Bay Test | 2 | UNM | 0 - 6553.5 km | E = N * 0.1 |

This PID contains the remaining distance the vehicle must be driven in order to enable the engine off natural vacuum (EONV) diagnostic service bay test.

### EVPR – PID $156A – Remaining Engine Run Time to Enable Engine Off Natural Vacuum Service Bay Test

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $156A | Remaining Engine Run Time to Enable Engine Off Natural Vacuum Service Bay Test | 2 | UNM | 0 - 65536 s | E = N * 1 |

This PID contains the remaining time that the engine must be run in order to enable the engine off natural vacuum (EONV) diagnostic service bay test.

### FADR – PID $156C – Fuel Adjustment System Diagnostic State

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $156C | Fuel Adjustment System Diagnostic State | 1 | ENM | N/A | $00=Disabled; $01=Monitor; $02=Run Purge Test; $03=Report |

This PID contains the current state of the fuel adjustment system diagnostic.

### FADR – PID $156D – Fuel Adjustment System Diagnostic Intrusive Test State

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $156D | Fuel Adjustment System Diagnostic Intrusive Test State | 1 | ENM | N/A | $00=Inhibit; $01=Request; $02=Close Purge; $03=Accumulate; $04=Return to Normal; $05=Wait While Purging ; $06=Extended Purging |

This PID contains the current state of the fuel adjustment system diagnostic intrusive excess purge vapor test, which turns purge off to determine if a rich condition has been caused by excess purge vapor or a truly rich fuel system.

### FADR – PID $156E – Filtered Non Purge Long Term Fuel Trim State Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $156E | Filtered Non Purge Long Term Fuel Trim State Bank 1 | 1 | ENM | N/A | $00=Inhibit; $01=Reset; $02=Initialize; $03=Accumulate |

This PID contains the current state of the filtered non purge long term closed loop fuel correction for bank 1 that is accumulated while purge is off.

### FADR – PID $156F – Filtered Non Purge Long Term Fuel Trim State Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $156F | Filtered Non Purge Long Term Fuel Trim State Bank 2 | 1 | ENM | N/A | $00=Inhibit; $01=Reset; $02=Initialize; $03=Accumulate |

This PID contains the current state of the filtered non purge long term closed loop fuel correction for bank 2 that is accumulated while purge is off.

### FADR – PID $1570 – Filtered Short Term Fuel Trim Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1570 | Filtered Short Term Fuel Trim Bank 1 | 1 | UNM | 0 - 1.9921875 | E = N / 128 |

This PID contains the filtered short term closed loop fuel correction for bank 1, which is filtered using a first-order lag filter, also known as an exponentially weighted moving average (EWMA).  This value is compared to thresholds to determine if the fuel adjustment system diagnostic is passing or failing or if an excess purge vapor test should be executed.

### FADR – PID $1571 – Filtered Short Term Fuel Trim Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1571 | Filtered Short Term Fuel Trim Bank 2 | 1 | UNM | 0 - 1.9921875 | E = N / 128 |

This PID contains the filtered short term closed loop fuel correction for bank 2, which is filtered using a first-order lag filter, also known as an exponentially weighted moving average (EWMA).  This value is compared to thresholds to determine if the fuel adjustment system diagnostic is passing or failing or if an excess purge vapor test should be executed.

### FADR – PID $1572 – Filtered Long Term Fuel Trim Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1572 | Filtered Long Term Fuel Trim Bank 1 | 1 | UNM | 0 - 1.9921875 | E = N / 128 |

This PID contains the filtered long term closed loop fuel correction for bank 1 (accumulated while the excess purge vapor test is inhibited), which is filtered using a first-order lag filter, also known as an exponentially weighted moving average (EWMA).  This value is compared to thresholds to determine if the fuel adjustment system diagnostic is passing or failing or if an excess purge vapor test should be executed.

### FADR – PID $1573 – Filtered Long Term Fuel Trim Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1573 | Filtered Long Term Fuel Trim Bank 2 | 1 | UNM | 0 - 1.9921875 | E = N / 128 |

This PID contains the filtered long term closed loop fuel correction for bank 2 (accumulated while the excess purge vapor test is inhibited), which is filtered using a first-order lag filter, also known as an exponentially weighted moving average (EWMA).  This value is compared to thresholds to determine if the fuel adjustment system diagnostic is passing or failing or if an excess purge vapor test should be executed.

### FADR – PID $1574 – Filtered Non Purge Long Term Fuel Trim Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1574 | Filtered Non Purge Long Term Fuel Trim Bank 1 | 1 | UNM | 0 - 1.9921875 | E = N / 128 |

This PID contains the filtered non purge long term closed loop fuel correction for bank 1 (accumulated while purge is off), which is filtered using a first-order lag filter, also known as an exponentially weighted moving average (EWMA).  This value is compared to thresholds to determine if the fuel adjustment system diagnostic is passing, failing, or indeterminate during normal purge-off operation as well as during the intrusive excess purge vapor test.

### FADR – PID $1575 – Filtered Non Purge Long Term Fuel Trim Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1575 | Filtered Non Purge Long Term Fuel Trim Bank 2 | 1 | UNM | 0 - 1.9921875 | E = N / 128 |

This PID contains the filtered non purge long term closed loop fuel correction for bank 2 (accumulated while purge is off), which is filtered using a first-order lag filter, also known as an exponentially weighted moving average (EWMA).  This value is compared to thresholds to determine if the fuel adjustment system diagnostic is passing, failing, or indeterminate during normal purge-off operation as well as during the intrusive excess purge vapor test.

### FADR – PID $1576 – Long Term Fuel Trim Rich Threshold

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1576 | Long Term Fuel Trim Rich Threshold | 1 | UNM | 0 - 1.9921875 | E = N / 128 |

This PID contains the rich threshold, determined as a function of the ethanol composition of the fuel, to which the filtered long term closed loop fuel correction for each bank is compared with purge on.  If the filtered long term closed loop fuel correction for either bank exceeds this threshold with purge on, the fuel system may be failing due to rich fueling, in which case an excess purge vapor test is requested to determine if the fuel system is truly rich or rich due to excess purge vapor conditions.

### FADR – PID $1577 – Non Purge Long Term Fuel Trim Rich Threshold

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1577 | Non Purge Long Term Fuel Trim Rich Threshold | 1 | UNM | 0 - 1.9921875 | E = N / 128 |

This PID contains the rich threshold, determined as a function of the ethanol composition of the fuel, to which the filtered non purge long term closed loop fuel correction for each bank is compared with purge off.  If the filtered non purge long term closed loop fuel correction for either bank exceeds this threshold with purge off, the fuel system is failing due to rich fueling, as opposed to excess purge vapor conditions.

### FADR – PID $1578 – Long Term Fuel Trim Lean Threshold

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1578 | Long Term Fuel Trim Lean Threshold | 1 | UNM | 0 - 1.9921875 | E = N / 128 |

This PID contains the lean threshold, determined as a function of the ethanol composition of the fuel, to which the filtered long term closed loop fuel correction for each bank is compared.  If the filtered long term closed loop fuel correction for either bank exceeds this threshold, the fuel system is failing due to lean fueling.

### FADR – PID $1579 – Fuel Adjustment System Diagnostic Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1579 | Fuel Adjustment System Diagnostic Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Intrusive Excess Purge Vapor Test Enabled | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reporting Criteria Met | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| General Engine Accumulation Criteria Met | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 2 Specific Accumulation Criteria Met | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Bank 1 Specific Accumulation Criteria Met | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Diagnostic Enable Criteria Met | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the fuel adjustment system diagnostic.  The content of each of these signals is described below.

Intrusive Excess Purge Vapor Test Enabled indicates that the criteria to enable the intrusive excess purge vapor test have been met.

Reporting Criteria Met indicates that the criteria for enabling the use of the accumulated fuel trim data for diagnosis of a lean or rich fueling condition have been met.

General Engine Accumulation Criteria Met indicates that the non bank specific criteria for enabling the accumulation of fuel trim data have been met (the fuel trim data for both banks is not accumulated when this indicator has a value of False).

Bank 1 Specific Accumulation Criteria Met and Bank 2 Specific Accumulation Criteria Met indicate that the bank specific criteria for enabling the accumulation of fuel trim data for the respective bank have been met (the fuel trim data for each bank is not accumulated when its respective indicator has a value of False).

Diagnostic Enable Criteria Met indicates that the criteria for enabling the fuel adjustment system diagnostic have been met and that the diagnostic is enabled.

### KNKR – PID $157A – Cylinders for Which Knock Is Detected

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $157A | Cylinders for Which Knock Is Detected | 2 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Knock Detected for Cylinder 1 | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Knock Detected for Cylinder 2 | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Knock Detected for Cylinder 3 | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Knock Detected for Cylinder 4 | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Knock Detected for Cylinder 5 | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Knock Detected for Cylinder 6 | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Knock Detected for Cylinder 7 | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Knock Detected for Cylinder 8 | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 1 | 7 | 1 |  |  |  |
| Reserved | 1 | 6 | 1 |  |  |  |
| Reserved | 1 | 5 | 1 |  |  |  |
| Reserved | 1 | 4 | 1 |  |  |  |
| Reserved | 1 | 3 | 1 |  |  |  |
| Reserved | 1 | 2 | 1 |  |  |  |
| Reserved | 1 | 1 | 1 |  |  |  |
| Reserved | 1 | 0 | 1 |  |  |  |

This PID indicates the set of cylinders for which knock has been detected over a calibratable sample period (typically one second).  The indicator associated with each cylinder is set to False at the beginning of each sample period in order to allow this PID to reflect the recent status of knock detection for an individual cylinder.

### MSFR – PID $1582 – Injectors Disabled for Misfire Catalyst Converter Protection

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1582 | Injectors Disabled for Misfire Catalyst Converter Protection | 2 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Injector Disabled for Cylinder 1 | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Injector Disabled for Cylinder 2 | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Injector Disabled for Cylinder 3 | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Injector Disabled for Cylinder 4 | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Injector Disabled for Cylinder 5 | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Injector Disabled for Cylinder 6 | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Injector Disabled for Cylinder 7 | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Injector Disabled for Cylinder 8 | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 1 | 7 | 1 |  |  |  |
| Reserved | 1 | 6 | 1 |  |  |  |
| Reserved | 1 | 5 | 1 |  |  |  |
| Reserved | 1 | 4 | 1 |  |  |  |
| Reserved | 1 | 3 | 1 |  |  |  |
| Reserved | 1 | 2 | 1 |  |  |  |
| Reserved | 1 | 1 | 1 |  |  |  |
| Reserved | 1 | 0 | 1 |  |  |  |

This PID indicates the set of injectors that have been disabled by the misfire catalyst converter protection algorithm.

### EITR – PID $1626 – Minimum Intake Air Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1626 | Minimum Intake Air Temperature | 1 | UNM | -40 - 151.25 °C | E = N * 0.75 - 40 |

This PID contains the minimum intake air temperature (measured by a sensor located near the throttle body) observed over the current ignition cycle.

### FANR – PID $162B – Actual Fan Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $162B | Actual Fan Speed | 1 | UNM | 0 - 4080 rpm | E = N * 16 |

This PID contains the measured fan speed for engine driven (EV or MRF) fan systems.

### FANR – PID $163F – Desired Fan Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $163F | Desired Fan Speed | 1 | UNM | 0 - 4080 rpm | E = N * 16 |

This PID contains the desired fan speed for engine driven (EV or MRF) fan systems.

### FANR – PID $1641 – Percent Fan Commanded

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1641 | Percent Fan Commanded | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the amount of fan power commanded as a percentage of the total fan power available.  (Note: This PID will not indicate actual fan speed for discrete fan systems when using the Cooling Fan Relay Driver Device Control.)

### IMOR – PID $1643 – Immobilizer Security Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1643 | Immobilizer Security Status | 2 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Vehicle Immobilized by ICM | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| ICM Identification Completed | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Unknown Transponder | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Incorrect Transponder | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Transponder Identification Active | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Environment Identification Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Security Code Programmed in ICM | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 0 | 1 |  |  |  |
| Powertrain Device Unknown | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 1 | 6 | 1 |  |  |  |
| Reserved | 1 | 5 | 1 |  |  |  |
| Reserved | 1 | 4 | 1 |  |  |  |
| Environment Device 4 Unknown | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Environment Device 3 Unknown | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Environment Device 2 Unknown | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Environment Device 1 Unknown | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the GMIO immobilizer system within the immobilizer control module (ICM).  The content of each of these signals is described below.

Vehicle Immobilized by ICM indicates that the ICM has decided to immobilize the vehicle.  In this case, the immobilizer response will not be release from the ICM.

ICM Identification Completed indicates that the vehicle environment identification process within the ICM has completed successfully and the immobilizer response is valid.

Unknown Transponder indicates that the transponder is unknown or has not yet been evaluated.

Incorrect Transponder indicates that the transponder crypt identification has failed or has not yet been evaluated.

Transponder Identification Active indicates that the transponder identification process is active within the ICM.  This indicator will be cleared if the identification has completed successfully or has failed.  The transponder identification includes the fixcode and challenge - response identification.

Environment Identification Active indicates that the vehicle environment identification process is active within the ICM.  This indicator is the signal for the identification devices to identify themselves to the immobilizer .  If this indicator is set, the immobilizer response has not yet been released and is, therefore, invalid.  This indicator will be cleared if the environment identification has completed successfully or has failed.

Security Code Programmed in ICM indicates that the ICM security code has been programmed (i.e., the immobilizer device is locked).

Powertrain Device Unknown indicates that the ICM has not received the powertrain identifier or has received the wrong powertrain identifier.

Environment Device 1 Unknown, Environment Device 2 Unknown, Environment Device 3 Unknown, and Environment Device 4 Unknown indicate that the associated device is unknown or its identification is pending to the ICM.

### IMOR – PID $1644 – Powertrain Immobilizer Security Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1644 | Powertrain Immobilizer Security Status | 2 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Security Level | 0 | 7 | 4 | UNM | 0 - 15 | E = N * 1 |
| Reserved | 0 | 3 | 1 |  |  |  |
| Security Code Lockout Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Security Code Accepted | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Security Code Programmed | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Free Start Active | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Vehicle Immobilized | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Post Release Passive | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Post Release Active | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Release | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Pre Release | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Immobilizer Function in Reset | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Immobilizer Secret Keys Not Programmed | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the GMIO immobilizer system within the engine control module (ECM).  The content of each of these signals is described below.

Security Level contains the current immobilizer security level.  If the correct security code is entered, the security level will remain at or be reset to a level of 10.  If an incorrect security code is entered, the security level will be set to a level of 9.

Security Code Lockout Active indicates that the security code has not passed and lockout is active.

Security Code Accepted indicates that the received security code is correct.

Security Code Programmed indicates that the security code has been programmed.

Free Start Active indicates that the free start function is active.

Vehicle Immobilized indicates that the immobilizer algorithm is in either the immobilized active or immobilized passive state (i.e., vehicle has been immobilized).

Post Release Passive indicates that the immobilizer algorithm is in the post release passive state.

Post Release Active indicates that the immobilizer algorithm is in the post release active state.

Release indicates that the immobilizer algorithm is in the release state.

Pre Release indicates that the immobilizer algorithm is in the pre release active.

Immobilizer Function in Reset indicates that the immobilizer function within the ECM is in a reset state (required to program immobilizer secret keys).

Immobilizer Secret Keys Not Programmed indicates that the immobilizer secret keys have not been programmed into memory.

### ECTR – PID $1652 – High Sided Coolant Rationality Ignition Off Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1652 | High Sided Coolant Rationality Ignition Off Time | 2 | UNM | 0 - 65535 s | E = N * 1 |

This PID contains the time that the ignition has been off prior to the result of the high sided coolant rationality diagnostic being reported.

After the engine controller has been flashed, this PID will read zero and will be updated only if the High Side Coolant Rationality test is enabled.  The HSCR enable criteria is based on engine soak time, engine coolant temperature and engine intake air temperature.

### PHSR – PID $166A – Exhaust Cam Phase Duty Cycle Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $166A | Exhaust Cam Phase Duty Cycle Bank 1 | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded duty cycle for the exhaust cam phase output for bank 1.

### PHSR – PID $166B – Intake Cam Phase Duty Cycle Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $166B | Intake Cam Phase Duty Cycle Bank 1 | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded duty cycle for the intake cam phase output for bank 1.

### PHSR – PID $166C – Exhaust Cam Phase Duty Cycle Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $166C | Exhaust Cam Phase Duty Cycle Bank 2 | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded duty cycle for the exhaust cam phase output for bank 2.

### PHSR – PID $166D – Intake Cam Phase Duty Cycle Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $166D | Intake Cam Phase Duty Cycle Bank 2 | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded duty cycle for the intake cam phase output for bank 2.

### OXYR – PID $1676 – Oxygen Sensor Heater Duty Cycle Bank 2 Sensor 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1676 | Oxygen Sensor Heater Duty Cycle Bank 2 Sensor 2 | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the commanded duty cycle for the oxygen sensor heater output for bank 2 sensor 2.

### OXYR – PID $1677 – Oxygen Sensor Heater Duty Cycle Bank 1 Sensor 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1677 | Oxygen Sensor Heater Duty Cycle Bank 1 Sensor 2 | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the commanded duty cycle for the oxygen sensor heater output for bank 1 sensor 2.

### PHSR – PID $168D – Exhaust Cam Phase Angle Actual Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $168D | Exhaust Cam Phase Angle Actual Bank 1 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the position of the exhaust camshaft for bank 1 in terms of degrees of camshaft rotation (retard) from the park position (a value of zero represents the park position).

### PHSR – PID $168E – Intake Cam Phase Angle Actual Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $168E | Intake Cam Phase Angle Actual Bank 1 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the position of the intake camshaft for bank 1 in terms of degrees of camshaft rotation (advance) from the park position (a value of zero represents the park position).

### PHSR – PID $168F – Exhaust Cam Phase Angle Actual Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $168F | Exhaust Cam Phase Angle Actual Bank 2 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the position of the exhaust camshaft for bank 2 in terms of degrees of camshaft rotation (retard) from the park position (a value of zero represents the park position).

### PHSR – PID $1690 – Intake Cam Phase Angle Actual Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1690 | Intake Cam Phase Angle Actual Bank 2 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the position of the intake camshaft for bank 2 in terms of degrees of camshaft rotation (advance) from the park position (a value of zero represents the park position).

### PHSR – PID $1691 – Exhaust Cam Phase Angle Desired Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1691 | Exhaust Cam Phase Angle Desired Bank 1 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the set point for the position of the exhaust camshaft for bank 1 in terms of degrees of camshaft rotation (retard) from the park position.

### PHSR – PID $1692 – Intake Cam Phase Angle Desired Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1692 | Intake Cam Phase Angle Desired Bank 1 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the set point for the position of the intake camshaft for bank 1 in terms of degrees of camshaft rotation (advance) from the park position.

### PHSR – PID $1693 – Exhaust Cam Phase Angle Desired Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1693 | Exhaust Cam Phase Angle Desired Bank 2 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the set point for the position of the exhaust camshaft for bank 2 in terms of degrees of camshaft rotation (retard) from the park position.

### PHSR – PID $1694 – Intake Cam Phase Angle Desired Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1694 | Intake Cam Phase Angle Desired Bank 2 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the set point for the position of the intake camshaft for bank 2 in terms of degrees of camshaft rotation (advance) from the park position.

### PHSR – PID $1695 – Exhaust Cam Phase Angle Error Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1695 | Exhaust Cam Phase Angle Error Bank 1 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the absolute difference between the desired and actual positions of the exhaust camshaft for bank 1 in terms of degrees of camshaft rotation.

### PHSR – PID $1696 – Intake Cam Phase Angle Error Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1696 | Intake Cam Phase Angle Error Bank 1 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the absolute difference between the desired and actual positions of the intake camshaft for bank 1 in terms of degrees of camshaft rotation.

### PHSR – PID $1697 – Exhaust Cam Phase Angle Error Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1697 | Exhaust Cam Phase Angle Error Bank 2 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the absolute difference between the desired and actual positions of the exhaust camshaft for bank 2 in terms of degrees of camshaft rotation.

### PHSR – PID $1698 – Intake Cam Phase Angle Error Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1698 | Intake Cam Phase Angle Error Bank 2 | 1 | UNM | 0 - 51 ° | E = N * 0.2 |

This PID contains the absolute difference between the desired and actual positions of the intake camshaft for bank 2 in terms of degrees of camshaft rotation.

### EPSR – PID $169A – Exhaust Cam Position Sensor Pulse Counter Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $169A | Exhaust Cam Position Sensor Pulse Counter Bank 1 | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains a rolling count of the number of exhaust cam position sensor pulses for bank 1.  A change in the value of this PID indicates exhaust cam position sensor activity for bank 1.

### EPSR – PID $169B – Intake Cam Position Sensor Pulse Counter Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $169B | Intake Cam Position Sensor Pulse Counter Bank 1 | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains a rolling count of the number of intake cam position sensor pulses for bank 1.  A change in the value of this PID indicates intake cam position sensor activity for bank 1.

### EPSR – PID $169C – Exhaust Cam Position Sensor Pulse Counter Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $169C | Exhaust Cam Position Sensor Pulse Counter Bank 2 | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains a rolling count of the number of exhaust cam position sensor pulses for bank 2.  A change in the value of this PID indicates exhaust cam position sensor activity for bank 2.

### EPSR – PID $169D – Intake Cam Position Sensor Pulse Counter Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $169D | Intake Cam Position Sensor Pulse Counter Bank 2 | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains a rolling count of the number of intake cam position sensor pulses for bank 2.  A change in the value of this PID indicates intake cam position sensor activity for bank 2.

### TFTR – PID $1940 – Transmission Oil Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1940 | Transmission Oil Temperature | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the undefaulted, unfiltered transmission oil / fluid temperature measured by a sensor.  In automatic transmission applications, this PID is based upon the transmission oil temperature sensor input to the TCM.  In manual transmission applications with a transmission oil temperature sensor, this PID is based upon the transmission oil temperature sensor input to the ECM.

### TISR – PID $1941 – Transmission Input Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1941 | Transmission Input Speed | 2 | UNM | 0 - 8191.875 rpm | E = N * 0.125 |

This PID contains the transmission input speed (the speed at the input of the gear box) determined from the speed measured by the transmission input speed (TIS) sensor with the application of factors to account for the location of the sensor (e.g., before the sprocket, after the overdrive, etc.).

### TOSR – PID $1942 – Transmission Output Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1942 | Transmission Output Speed | 2 | UNM | 0 - 8191.875 rpm | E = N * 0.125 |

This PID contains the transmission output speed (the speed at the output of the gear box) determined from the speed measured by the transmission output speed (TOS) sensor with the application of factors to account for the location of the sensor (e.g., after the gear box, after the final chain, after the transfer case, after the final drive, etc.).  In applications that include both a transmission output speed sensor and a transfer case speed sensor, this PID will apply specifically to the output of the transmission.

### SPRR – PID $194B – Transmission 4-5 Upshift Time Error

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $194B | Transmission 4-5 Upshift Time Error | 1 | SNM | -3.2 - 3.175 s | E = N * 0.025 |

This PID contains the difference between the desired and actual shift times for the most recent transmission 4-5 upshift, which is used to modify the transmission 4-5 upshift pressure adapts.

### SHTR – PID $194C – Transmission 4-5 Upshift Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $194C | Transmission 4-5 Upshift Time | 1 | UNM | 0 - 6.375 s | E = N * 0.025 |

This PID contains the duration of the ratio change portion of the most recent transmission 4-5 upshift.

### TRGR – PID $194F – Transmission PRNDL Range

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $194F | Transmission PRNDL Range | 1 | ENM | N/A | $00=Illegal; $01=Park; $02=Reverse; $04=Neutral; $06=Drive 6; $08=Drive 5; $10=Drive 4; $20=Drive 3; $40=Drive 2; $80=Drive 1 |

This PID contains the transmission range determined directly from the decoding of the PRNDL A, B, C, and P inputs that are based upon either an internal mode switch (IMS) or a neutral safety backup (NSBU) switch.

### TRGR – PID $1951 – Transmission Range Input Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1951 | Transmission Range Input Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| PRNDL Inputs Illegal | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| PRNDL P Input Active | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| PRNDL C Input Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PRNDL B Input Active | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| PRNDL A Input Active | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| PSM C Input Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| PSM B Input Active | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| PSM A Input Active | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the transmission range inputs.  The content of each of these signals is described below.

PSM A Input Active, PSM B Input Active, and PSM C Input Active indicate the states of the pressure switch manifold (PSM) inputs.  In drive reverse indicator (DRI) applications, only the PSM A Input Active and PSM B Input Active signals are supported, and a value of False will be provided for the signal PSM C Input Active.

PRNDL A Input Active, PRNDL B Input Active, PRNDL C Input Active, and PRNDL P Input Active indicate the states of the PRNDL A, B, C, and P inputs, respectively, that are based upon either an internal mode switch (IMS) or a neutral safety backup (NSBU) switch.

PRNDL Inputs Illegal indicates that the decoding of the PRNDL A, B, C, and P inputs resulted in an illegal transmission range.

### TRTR – PID $195B – Diagnostic Transmission Ratio

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $195B | Diagnostic Transmission Ratio | 2 | UNM | 0 - 7.999877929688 | E = N / 8192 |

This PID contains the undefaulted ratio of transmission input speed (the speed at the input of the gear box) to transmission output speed (the speed at the output of the gear box).  If the transmission input speed (TIS) sensor is located after the overdrive, the overdrive is not included in this ratio.

### FWDR – PID $195D – Transfer Case Ratio

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $195D | Transfer Case Ratio | 2 | UNM | 0 - 15.99975585938 | E = N / 4096 |

This PID contains the estimated transfer case ratio for applications that are equipped with a transfer case capable of providing range selection (e.g., four wheel drive low).

### TCCR – PID $1970 – Torque Converter Clutch PWM Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1970 | Torque Converter Clutch PWM Duty Cycle | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded duty cycle for the torque converter clutch (TCC) PWM output determined as a function of TCC capacity.

### TPCR – PID $1972 – Transmission Line Pressure PWM Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1972 | Transmission Line Pressure PWM Duty Cycle | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded duty cycle for the transmission line pressure PWM output determined as a function of the error between the desired and actual transmission line pressure currents.

### AWDR – PID $1974 – All Wheel Drive Enable Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1974 | All Wheel Drive Enable Duty Cycle | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded duty cycle for the all wheel drive enable output.

### TCCR – PID $1975 – Transmission Component Slipping Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1975 | Transmission Component Slipping Counter | 1 | UNM | 0 - 255 counts | E = N * 1 |

This PID contains the number of times that excessive torque converter clutch (TCC) slip has been detected when TCC is commanded on (on mode or lock on mode) during the current test interval for the transmission component slipping diagnostic (DTC P1870).

### TRTR – PID $1976 – Modeled Torque Converter Speed Ratio

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1976 | Modeled Torque Converter Speed Ratio | 1 | UNM | 0 - 1.9921875 | E = N / 128 |

This PID contains the ratio of the modeled speed at the output of the torque converter (difference between engine speed and the modeled torque converter slip) to the speed at the input of the torque converter (engine speed) for transmissions that do not have a transmission input speed (TIS) sensor.

### TUDR – PID $197E – Transmission Tap Up/Down Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $197E | Transmission Tap Up/Down Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission Tap Up/Down Switch Invalid | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Driver Shift Control Present | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Electronic Range Selection Present | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 4 | 1 |  |  |  |
| Transmission Electronic Range Selection Active | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Tap Down Switch Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Tap Up Switch Active | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Driver Shift Control Active | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the transmission driver shift control and transmission electronic range selection systems.  The content of each of these signals is described below.

Transmission Tap Up/Down Switch Invalid indicates that a failure condition associated with the tap up/down switch has been detected.  In electrical architectures that precede PPEI 3.x, this signal is based upon the tap up/down switch hardwire input to the controller.  In electrical architectures from PPEI 3.x or later, this signal is based upon the tap up/down switch information received over serial communication from the platform electronics.

Transmission Driver Shift Control Present indicates that the selection calibration used to enable/disable the driver shift control feature is set to enable this feature via a request received over serial communication from the platform electronics.

Transmission Electronic Range Selection Present indicates that the selection calibration used to enable/disable the electronic range selection feature is set to enable this feature via a request received over serial communication from the platform electronics.

Transmission Electronic Range Selection Active indicates that the tap up/down switch is being used to request an increase or decrease in the transmission drive range.

Transmission Tap Down Switch Active indicates that a driver request to decrease the transmission drive range or transmission gear is active.  In electrical architectures that precede PPEI 3.x, this signal is based upon the tap up/down switch hardwire input to the controller.  In electrical architectures from PPEI 3.x or later, this signal is based upon the tap up/down switch information received over serial communication from the platform electronics.

Transmission Tap Up Switch Active indicates that a driver request to increase the transmission drive range or transmission gear is active.  In electrical architectures that precede PPEI 3.x, this signal is based upon the tap up/down switch hardwire input to the controller.  In electrical architectures from PPEI 3.x or later, this signal is based upon the tap up/down switch information received over serial communication from the platform electronics.

Transmission Driver Shift Control Active indicates that the tap up/down switch is being used to request an increase or decrease in the transmission gear.

### TCCR – PID $1991 – Torque Converter Clutch Slip

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1991 | Torque Converter Clutch Slip | 2 | SNM | -4096 - 4095.875 rpm | E = N * 0.125 |

This PID contains the torque converter clutch (TCC) slip determined as the difference between the speed at the input of the torque converter (engine speed) and the speed at the output of the torque converter (turbine speed).

### SHTR – PID $1992 – Transmission Upshift Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1992 | Transmission Upshift Time | 1 | UNM | 0 - 6.375 s | E = N * 0.025 |

This PID contains the duration of the ratio change portion of the most recent transmission upshift.

### SHTR – PID $1993 – Transmission 1-2 Upshift Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1993 | Transmission 1-2 Upshift Time | 1 | UNM | 0 - 6.375 s | E = N * 0.025 |

This PID contains the duration of the ratio change portion of the most recent transmission 1-2 upshift.

### SHTR – PID $1994 – Transmission 2-3 Upshift Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1994 | Transmission 2-3 Upshift Time | 1 | UNM | 0 - 6.375 s | E = N * 0.025 |

This PID contains the duration of the ratio change portion of the most recent transmission 2-3 upshift.

### SHTR – PID $1995 – Transmission 3-4 Upshift Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1995 | Transmission 3-4 Upshift Time | 1 | UNM | 0 - 6.375 s | E = N * 0.025 |

This PID contains the duration of the ratio change portion of the most recent transmission 3-4 upshift.

### SPRR – PID $1996 – Transmission Upshift Time Error

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1996 | Transmission Upshift Time Error | 1 | SNM | -3.2 - 3.175 s | E = N * 0.025 |

This PID contains the difference between the desired and actual shift times for the most recent transmission upshift, which is used to modify the transmission upshift pressure adapts.

### SPRR – PID $1997 – Transmission 1-2 Upshift Time Error

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1997 | Transmission 1-2 Upshift Time Error | 1 | SNM | -3.2 - 3.175 s | E = N * 0.025 |

This PID contains the difference between the desired and actual shift times for the most recent transmission 1-2 upshift, which is used to modify the transmission 1-2 upshift pressure adapts.

### SPRR – PID $1998 – Transmission 2-3 Upshift Time Error

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1998 | Transmission 2-3 Upshift Time Error | 1 | SNM | -3.2 - 3.175 s | E = N * 0.025 |

This PID contains the difference between the desired and actual shift times for the most recent transmission 2-3 upshift, which is used to modify the transmission 2-3 upshift pressure adapts.

### SPRR – PID $1999 – Transmission 3-4 Upshift Time Error

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1999 | Transmission 3-4 Upshift Time Error | 1 | SNM | -3.2 - 3.175 s | E = N * 0.025 |

This PID contains the difference between the desired and actual shift times for the most recent transmission 3-4 upshift, which is used to modify the transmission 3-4 upshift pressure adapts.

### SHPR – PID $199A – Transmission Commanded Gear

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $199A | Transmission Commanded Gear | 1 | ENM | N/A | $01=First Gear; $02=Second Gear; $03=Third Gear; $04=Fourth Gear; $05=Fifth Gear; $06=Sixth Gear |

This PID contains the gear that is currently commanded by the transmission, which will change state at the beginning of a transmission gear shift event.

### SPRR – PID $199B – Transmission Upshift Pressure Adapt Cell Index

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $199B | Transmission Upshift Pressure Adapt Cell Index | 1 | UNM | 0 - 255 | E = N * 1 |

This PID contains the index of the entry within the array of transmission upshift pressure adapts that is closest to the shift torque for the most recent transmission upshift.

### TPCR – PID $199E– Transmission Line Pressure Actual Current

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $199E | Transmission Line Pressure Actual Current | 1 | UNM | 0 - 4.98046875 A | E = N * 5 / 256 |

This PID contains the actual (measured) transmission line pressure current.

### TPCR – PID $199F – Transmission Line Pressure Desired Current

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $199F | Transmission Line Pressure Desired Current | 1 | UNM | 0 - 4.98046875 A | E = N * 5 / 256 |

This PID contains the desired transmission line pressure current determined from the desired transmission line pressure and other criteria.

### TRTR – PID $19A1 – Transmission Gear Ratio

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $19A1 | Transmission Gear Ratio | 1 | UNM | 0 - 3.984375 | E = N / 64 |

This PID contains the ratio of transmission input speed (the speed at the input of the gear box) to transmission output speed (the speed at the output of the gear box).

### TBNR – PID $19D4 – Turbine Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $19D4 | Turbine Speed | 2 | UNM | 0 - 8191.875 rpm | E = N * 0.125 |

This PID contains the turbine speed (the speed at the output of the torque converter and the input of the sprocket) determined from the division of the transmission input speed (the speed at the output of sprocket and the input to the gear box) by the sprocket ratio (the ratio of the number of teeth of the drive sprocket to the number of teeth of the driven sprocket).  If the transmission is not equipped with an input chain (e.g., a rear wheel drive transmission), the sprocket ratio is unity.

### TRTR – PID $1A01 – Torque Converter Speed Ratio

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1A01 | Torque Converter Speed Ratio | 2 | UNM | 0 - 7.999877929688 | E = N / 8192 |

This PID contains the ratio of the speed at the output of the torque converter (turbine speed) to the speed at the input of the torque converter (engine speed).

### TCCR – PID $1A18 – Torque Converter Clutch Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1A18 | Torque Converter Clutch Pressure | 2 | UNM | 0 - 262140 kPa | E = N * 4 |

This PID contains the desired torque converter clutch (TCC) pressure based upon the current operating mode of the electronically controlled converter clutch (ECCC) algorithm.

### TRTR – PID $1A1F – Transmission Clutch Slip

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1A1F | Transmission Clutch Slip | 2 | SNM | -4096.0 - 4095.875 rpm | E = N * 0.125 |

This PID contains the transmission clutch slip determined as the difference between transmission input speed (the speed at the input of the gear box) and the product of transmission output speed (the speed at the output of the gear box) and the calibrated gear box ratio for the current commanded gear.  If the transmission input speed (TIS) sensor is located after the overdrive, and the current commanded gear is fourth gear, the transmission clutch slip is determined as the difference between the speed after the overdrive and the product of the transmission output speed and the calibrated gear box ratio for third gear.

### TCCR – PID $1A26 – Torque Converter Clutch Slip Error

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1A26 | Torque Converter Clutch Slip Error | 2 | SNM | -4096 - 4095.875 rpm | E = N * 0.125 |

This PID contains the torque converter clutch (TCC) slip error determined as the difference between the absolute value of TCC slip and the TCC reference slip.

### ETQR – PID $1A2D – Engine Actual Steady State Torque

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1A2D | Engine Actual Steady State Torque | 2 | SNM | -8192 - 8191.75 Nm | E = N * 0.25 |

This PID contains the engine torque based upon the airflow through the engine and the torque losses due to spark retard and fuel shutoff.  Since this is a steady state engine torque, it does not include the inertial component of engine torque.

### TRTR – PID $1A88 – Transmission Modeled Gear Ratio

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1A88 | Transmission Modeled Gear Ratio | 2 | UNM | 0 - 15.99975585938 | E = N / 4096 |

This PID contains the ratio of the modeled speed at the input of the gear box (difference between engine speed and the modeled torque converter slip multiplied by the sprocket ratio) to transmission output speed (the speed at the output of the gear box) for transmissions that do not have a transmission input speed (TIS) sensor.

### AWDR – PID $1A8E – All Wheel Drive System Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $1A8E | All Wheel Drive System Status | 2 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Brake System Fault Disable | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Mini Spare Tire Lost Communication Disable | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Energy Reduction Software Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| PTO Torque Management Active | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Mini Spare Tire Lost Communication Disable | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Temperature Limiting Software Disable | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| ODM Short to Power/Ground Fault Detected | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| ODM Open Fault Detected | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Overtemperature Delay Active | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| All Wheel Drive Disabled | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Mini Spare Tire Disable | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Voltage Disable | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Overtemperature Disable | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Wheel Speed Fault Disable | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Overspeed Disable | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| All Wheel Drive Present | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains several signals associated with the status of the all wheel drive (AWD) system.  The content of each of these signals is described below.

Brake System Fault Disable indicates that AWD is disabled due to a brake system (ABS) failure.

Mini Spare Tire Lost Communication Disable indicates that AWD is disabled for the remainder of the driving cycle due to the loss of the serial communication providing the mini spare tire status.

Energy Reduction Software Active indicates that the energy reduction software (ERS) is requesting a gear override.

PTO Torque Management Active indicates that the gearbox torque is greater than the PTO Torque Limit

Temperature Limiting Software Disable indicates that AWD is disabled due to the temperature limiting software (TLS) being active.

ODM Short to Power/Ground Fault Detected indicates status of the short to ground and short to power tests used to report DTC P1728.

ODM Open Fault Detected indicates status of the open circuit tests used to report DTC P1728.

Overtemperature Delay Active indicates that the AWD system is waiting to re-enable following a rear drive module (RDM) overtemperature condition.

All Wheel Drive Disabled indicates that AWD is currently disabled (i.e., the all wheel drive enable output is commanded off).

Mini Spare Tire Disable indicates that AWD is disabled for the remainder of the driving cycle due to the use of a mini spare tire.

Voltage Disable indicates that AWD is disabled due to an ignition voltage out of range condition.

Overtemperature Disable indicates that AWD is disabled due to a RDM overtemperature condition.

Wheel Speed Fault Disable indicates that AWD is disabled due to a wheel speed failure.

Overspeed Disable indicates that AWD is disabled due to vehicle speed being too high or due to a vehicle speed fault.

All Wheel Drive Present indicates that the vehicle is equipped with AWD.

### MAPR – PID $2000 – Intake Manifold Absolute Pressure A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2000 | Intake Manifold Absolute Pressure A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the intake manifold absolute pressure (MAP) analog input as a percentage of its reference voltage.

### MAPR – PID $2001 – Intake Manifold Vacuum Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2001 | Intake Manifold Vacuum Pressure | 1 | SNM | -128 - 127 kPa | E = N * 1 |

This PID contains the engine intake manifold vacuum pressure calculated as the difference between ambient air pressure (barometric pressure) and intake manifold absolute pressure (MAP).

### MAPR – PID $2002 – Supercharger Inlet Absolute Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2002 | Supercharger Inlet Absolute Pressure | 1 | UNM | 0 - 255 kPa | E = N * 1 |

This PID contains the unfiltered supercharger inlet absolute pressure (SCIAP).

### MAPR – PID $2003 – Supercharger Inlet Absolute Pressure A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2003 | Supercharger Inlet Absolute Pressure A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the supercharger inlet absolute pressure (SCIAP) analog input as a percentage of its reference voltage.

### MAPR – PID $2004 – Supercharger Inlet Vacuum Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2004 | Supercharger Inlet Vacuum Pressure | 1 | UNM | 0 - 255 kPa | E = N * 1 |

This PID contains the supercharger inlet vacuum pressure calculated as the difference between ambient air pressure (barometric pressure) and supercharger inlet absolute pressure (SCIAP).

### EITR – PID $2005 – Power-Up Intake Air Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2005 | Power-Up Intake Air Temperature | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the intake air temperature (measured by a sensor located near the throttle body) recorded at power-up (Run/Crank transition from low to high).

### EITR – PID $2006 – Intake Air Temperature 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2006 | Intake Air Temperature 2 | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the undefaulted intake air temperature measured by a sensor that is located within the engine intake manifold.

### AAPR – PID $2007 – Barometric Pressure A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2007 | Barometric Pressure A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the barometric pressure analog input as a percentage of its reference voltage.

### IMTR – PID $2008 – Intake Manifold Tuning Valve Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2008 | Intake Manifold Tuning Valve Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Intake Manifold Tuning Valve Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Intake Manifold Tuning Valve Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Intake Manifold Tuning Valve Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Intake Manifold Tuning Valve Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the intake manifold tuning valve output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0660.

### IMTR – PID $2009 – Intake Manifold Tuning Vane Position Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2009 | Intake Manifold Tuning Vane Position Duty Cycle | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the duty cycle of the PWM feedback from the intake manifold tuning electronics, which represents the position of the intake manifold tuning vane.

### IMTR – PID $200A – Intake Manifold Tuning Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $200A | Intake Manifold Tuning Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Intake Manifold Tuning Valve Commanded Closed | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Intake Manifold Tuning Valve Commanded On | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Intake Manifold Tuning Hard Stop Vane Position Learn Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Intake Manifold Tuning Vane Position | 0 | 1 | 2 | ENM | N/A | $0=Open; $1=Intermediate; $2=Closed |

This PID contains several signals associated with the status of the intake manifold tuning system.  The content of each of these signals is described below.

Intake Manifold Tuning Valve Commanded Closed indicates that the intake manifold tuning valve is commanded closed in order to prevent the manifold runners from communicating.

Intake Manifold Tuning Valve Commanded On indicates that the intake manifold tuning valve output is commanded on (a calibration is used to select whether this results in the valve being commanded closed or open for a particular application).

Intake Manifold Tuning Hard Stop Vane Position Learn Active indicates that the intake manifold tuning electronics are currently learning the hard stop position for the intake manifold tuning vane.

Intake Manifold Tuning Vane Position indicates whether the intake manifold tuning vane is in an open, closed, or intermediate position based upon the PWM feedback from the intake manifold tuning electronics.

### BSTR – PID $200B – Supercharger Boost Solenoid Duty Cycle

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $200B | Supercharger Boost Solenoid Duty Cycle | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the commanded duty cycle for the supercharger boost solenoid output.

### BSTR – PID $200C – Supercharger Boost Solenoid Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $200C | Supercharger Boost Solenoid Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Supercharger Boost Solenoid Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Supercharger Boost Solenoid Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Supercharger Boost Solenoid Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Supercharger Boost Solenoid Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the supercharger boost solenoid output (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0033.

### BSTR – PID $200D – Intercooler Pump Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $200D | Intercooler Pump Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Intercooler Pump Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Intercooler Pump Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Intercooler Pump Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Intercooler Pump Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the intercooler pump output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P023A.

### CDAR – PID $200E – Distance With Cylinders Deactivated

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $200E | Distance With Cylinders Deactivated | 2 | UNM | 0 - 1023.984375 km | E = N * 0.015625 |

This PID contains the accumulated distance driven with half of the engine’s cylinders deactivated during the current key cycle.

### CDAR – PID $200F – Distance With All Cylinders Activated

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $200F | Distance With All Cylinders Activated | 2 | UNM | 0 - 1023.984375 km | E = N * 0.015625 |

This PID contains the accumulated distance driven with all of the engine’s cylinders activated during the current key cycle.

### CDAR – PID $2010 – Displacement On Demand Transitions

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2010 | Displacement On Demand Transitions | 2 | UNM | 0 - 65535 counts | E = N * 1 |

This PID contains the number of times that displacement on demand has been activated (i.e., has transitioned from all of the engine’s cylinders activated to half of the engine’s cylinders deactivated) during the current key cycle.

### CDAR – PID $2011 – Displacement on Demand Inhibit Reasons 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2011 | Displacement on Demand Inhibit Reasons 1 | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Engine Manifold Vacuum Pressure Low | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Brake Vacuum Pressure Low | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Accelerator Pedal Position Increase High | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Accelerator Pedal Position High | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Ignition Voltage Out of Range | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Oil Pressure Out of Range | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Oil Temperature Out of Range | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Coolant Temperature Out of Range | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Speed Out of Range | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Gear Incorrect | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Range Incorrect | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Gear Shift in Progress | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| All Cylinders Activated via Device Control | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Minimum Time with All Cylinders Activated Not Met | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Maximum Deactivation Time Exceeded | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Oil Aeration Present | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Decel Fuel Cutoff Active | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Shut-Off Stability Timer Active | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Heater Performance | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| AC Clutch Being Engaged | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| POSD Intrusive Active | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| In 4WD Low State | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Vehicle Speed Too Low | 2 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Tip In Bump Active | 2 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Catalyst Temperature Low | 3 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| ETC Disable Time | 3 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Ethanol Learn In Progress | 3 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Ethanol Concentration Too High | 3 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 3 | 3 | 1 |  |  |  |
| Reserved | 3 | 2 | 1 |  |  |  |
| Reserved | 3 | 1 | 1 |  |  |  |
| Reserved | 3 | 0 | 1 |  |  |  |

This PID contains the set of general operating conditions that are inhibiting deactivation of half of the engine’s cylinders.

### CDAR – PID $2012 – Displacement on Demand Inhibit Reasons 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2012 | Displacement on Demand Inhibit Reasons 2 | 4 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Manifold Absolute Pressure Sensor Fault | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Brake Vacuum Pressure Sensor Fault | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Electronic Throttle Control Reduced Engine Power Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Oil Pressure Sensor Fault | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Coolant Temperature Sensor Fault | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Intake Air Temperature Sensor Fault | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Vehicle Speed Fault | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Displacement on Demand Solenoid Driver Fault | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Misfire Detected | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Crank Sensor Fault | 1 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Cam Sensor Fault | 1 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Powertrain Relay Fault | 1 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Brake Torque Management Active (Obsolete) | 1 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Axle Torque Limiting Active (Obsolete) | 1 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Metal Overtemperature Protection Active | 1 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Catalyst Converter Overtemperature Protection Active | 1 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Piston Protection Active | 2 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Hot Coolant Mode Active | 2 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Overspeed Protection Active | 2 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Cylinder Deactivation System Fault | 2 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Torque Model Inaccurate Fault | 2 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Invalid 4WD Low State | 2 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 2 | 1 | 1 |  |  |  |
| Reserved | 2 | 0 | 1 |  |  |  |
| Reserved | 3 | 7 | 1 |  |  |  |
| Reserved | 3 | 6 | 1 |  |  |  |
| Reserved | 3 | 5 | 1 |  |  |  |
| Reserved | 3 | 4 | 1 |  |  |  |
| Reserved | 3 | 3 | 1 |  |  |  |
| Reserved | 3 | 2 | 1 |  |  |  |
| Reserved | 3 | 1 | 1 |  |  |  |
| Reserved | 3 | 0 | 1 |  |  |  |

This PID contains the set of fault conditions and protection modes that are inhibiting deactivation of half of the engine’s cylinders.

### CDAR – PID $2013 – Displacement on Demand Reactivation History 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2013 | Displacement on Demand Reactivation History 1 | 1 | ENM | N/A | $00=None; $01=Engine Manifold Vacuum Pressure Low; $02=Brake Vacuum Pressure Low; $03=Accelerator Pedal Position Increase High; $04=Accelerator Pedal Position High; $05=Ignition Voltage Out of Range; $06=Engine Oil Pressure Out of Range; $07=Engine Oil Temperature Out of Range; $08=Engine Coolant Temperature Out of Range; $09=Engine Speed Out of Range; $0A=Transmission Gear Incorrect; $0B=Transmission Range Incorrect; $0C=Transmission Gear Shift in Progress; $0D=All Cylinders Activated via Device Control; $0E=Minimum Time with All Cylinders Activated Not Met; $0F=Maximum Deactivation Time Exceeded; $10=Engine Oil Aeration Present; $11=Decel Fuel Cutoff Active; $12=Fuel Shut-Off Stability Timer Active; $13=Heater Performance; $30=Manifold Absolute Pressure Sensor Fault; $31=Brake Vacuum Pressure Sensor Fault; $32=Electronic Throttle Control Reduced Engine Power Active; $33=Engine Oil Pressure Sensor Fault; $34=Engine Coolant Temperature Sensor Fault; $35=Intake Air Temperature Sensor Fault; $36=Vehicle Speed Fault; $37=Displacement on Demand Solenoid Driver Fault; $38=Engine Misfire Detected; $39=Crank Sensor Fault; $3A=Cam Sensor Fault; $3B=Powertrain Relay Fault; $3E=Engine Metal Overtemperature Protection Active; $3F=Catalyst Converter Overtemperature Protection Active; $40=Piston Protection Active; $41=Hot Coolant Mode Active; $42=Engine Overspeed Protection Active; $43=AC Clutch Being Engaged; $44=Post O2 Diagnostic Intrusive Active; $45=In 4WD Low; $46=Vehicle Speed Too Low; $47=Tip In Bump Active; $48=Catalyst Temperature Low; $49=Cylinder Deactivation System Fault; $4A=Torque Model Inaccurate Fault; $4B=Invalid 4WD Low State; $4C=ETC Disable Time; $4E=Ethanol Learn In Progress; $4F=Ethanol Concentration Too High |

This PID contains one of the last eight reasons for the reactivation of all engine cylinders.  (Deactivate = half-cylinder mode, Reactivate = all-cylinder mode)  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CDAR – PID $2014 – Displacement on Demand Reactivation History 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2014 | Displacement on Demand Reactivation History 2 | 1 | ENM | N/A | $00=None; $01=Engine Manifold Vacuum Pressure Low; $02=Brake Vacuum Pressure Low; $03=Accelerator Pedal Position Increase High; $04=Accelerator Pedal Position High; $05=Ignition Voltage Out of Range; $06=Engine Oil Pressure Out of Range; $07=Engine Oil Temperature Out of Range; $08=Engine Coolant Temperature Out of Range; $09=Engine Speed Out of Range; $0A=Transmission Gear Incorrect; $0B=Transmission Range Incorrect; $0C=Transmission Gear Shift in Progress; $0D=All Cylinders Activated via Device Control; $0E=Minimum Time with All Cylinders Activated Not Met; $0F=Maximum Deactivation Time Exceeded; $10=Engine Oil Aeration Present; $11=Decel Fuel Cutoff Active; $12=Fuel Shut-Off Stability Timer Active; $13=Heater Performance; $30=Manifold Absolute Pressure Sensor Fault; $31=Brake Vacuum Pressure Sensor Fault; $32=Electronic Throttle Control Reduced Engine Power Active; $33=Engine Oil Pressure Sensor Fault; $34=Engine Coolant Temperature Sensor Fault; $35=Intake Air Temperature Sensor Fault; $36=Vehicle Speed Fault; $37=Displacement on Demand Solenoid Driver Fault; $38=Engine Misfire Detected; $39=Crank Sensor Fault; $3A=Cam Sensor Fault; $3B=Powertrain Relay Fault; $3E=Engine Metal Overtemperature Protection Active; $3F=Catalyst Converter Overtemperature Protection Active; $40=Piston Protection Active; $41=Hot Coolant Mode Active; $42=Engine Overspeed Protection Active; $43=AC Clutch Being Engaged; $44=Post O2 Diagnostic Intrusive Active; $45=In 4WD Low; $46=Vehicle Speed Too Low; $47=Tip In Bump Active; $48=Catalyst Temperature Low; $49=Cylinder Deactivation System Fault; $4A=Torque Model Inaccurate Fault; $4B=Invalid 4WD Low State; $4C=ETC Disable Time; $4E=Ethanol Learn In Progress; $4F=Ethanol Concentration Too High |

This PID contains one of the last eight reasons for the reactivation of all engine cylinders.  (Deactivate = half-cylinder mode, Reactivate = all-cylinder mode)  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CDAR – PID $2015 – Displacement on Demand Reactivation History 3

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2015 | Displacement on Demand Reactivation History 3 | 1 | ENM | N/A | $00=None; $01=Engine Manifold Vacuum Pressure Low; $02=Brake Vacuum Pressure Low; $03=Accelerator Pedal Position Increase High; $04=Accelerator Pedal Position High; $05=Ignition Voltage Out of Range; $06=Engine Oil Pressure Out of Range; $07=Engine Oil Temperature Out of Range; $08=Engine Coolant Temperature Out of Range; $09=Engine Speed Out of Range; $0A=Transmission Gear Incorrect; $0B=Transmission Range Incorrect; $0C=Transmission Gear Shift in Progress; $0D=All Cylinders Activated via Device Control; $0E=Minimum Time with All Cylinders Activated Not Met; $0F=Maximum Deactivation Time Exceeded; $10=Engine Oil Aeration Present; $11=Decel Fuel Cutoff Active; $12=Fuel Shut-Off Stability Timer Active; $13=Heater Performance; $30=Manifold Absolute Pressure Sensor Fault; $31=Brake Vacuum Pressure Sensor Fault; $32=Electronic Throttle Control Reduced Engine Power Active; $33=Engine Oil Pressure Sensor Fault; $34=Engine Coolant Temperature Sensor Fault; $35=Intake Air Temperature Sensor Fault; $36=Vehicle Speed Fault; $37=Displacement on Demand Solenoid Driver Fault; $38=Engine Misfire Detected; $39=Crank Sensor Fault; $3A=Cam Sensor Fault; $3B=Powertrain Relay Fault; $3E=Engine Metal Overtemperature Protection Active; $3F=Catalyst Converter Overtemperature Protection Active; $40=Piston Protection Active; $41=Hot Coolant Mode Active; $42=Engine Overspeed Protection Active; $43=AC Clutch Being Engaged; $44=Post O2 Diagnostic Intrusive Active; $45=In 4WD Low; $46=Vehicle Speed Too Low; $47=Tip In Bump Active; $48=Catalyst Temperature Low; $49=Cylinder Deactivation System Fault; $4A=Torque Model Inaccurate Fault; $4B=Invalid 4WD Low State; $4C=ETC Disable Time; $4E=Ethanol Learn In Progress; $4F=Ethanol Concentration Too High |

This PID contains one of the last eight reasons for the reactivation of all engine cylinders.  (Deactivate = half-cylinder mode, Reactivate = all-cylinder mode)  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CDAR – PID $2016 – Displacement on Demand Reactivation History 4

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2016 | Displacement on Demand Reactivation History 4 | 1 | ENM | N/A | $00=None; $01=Engine Manifold Vacuum Pressure Low; $02=Brake Vacuum Pressure Low; $03=Accelerator Pedal Position Increase High; $04=Accelerator Pedal Position High; $05=Ignition Voltage Out of Range; $06=Engine Oil Pressure Out of Range; $07=Engine Oil Temperature Out of Range; $08=Engine Coolant Temperature Out of Range; $09=Engine Speed Out of Range; $0A=Transmission Gear Incorrect; $0B=Transmission Range Incorrect; $0C=Transmission Gear Shift in Progress; $0D=All Cylinders Activated via Device Control; $0E=Minimum Time with All Cylinders Activated Not Met; $0F=Maximum Deactivation Time Exceeded; $10=Engine Oil Aeration Present; $11=Decel Fuel Cutoff Active; $12=Fuel Shut-Off Stability Timer Active; $13=Heater Performance; $30=Manifold Absolute Pressure Sensor Fault; $31=Brake Vacuum Pressure Sensor Fault; $32=Electronic Throttle Control Reduced Engine Power Active; $33=Engine Oil Pressure Sensor Fault; $34=Engine Coolant Temperature Sensor Fault; $35=Intake Air Temperature Sensor Fault; $36=Vehicle Speed Fault; $37=Displacement on Demand Solenoid Driver Fault; $38=Engine Misfire Detected; $39=Crank Sensor Fault; $3A=Cam Sensor Fault; $3B=Powertrain Relay Fault; $3E=Engine Metal Overtemperature Protection Active; $3F=Catalyst Converter Overtemperature Protection Active; $40=Piston Protection Active; $41=Hot Coolant Mode Active; $42=Engine Overspeed Protection Active; $43=AC Clutch Being Engaged; $44=Post O2 Diagnostic Intrusive Active; $45=In 4WD Low; $46=Vehicle Speed Too Low; $47=Tip In Bump Active; $48=Catalyst Temperature Low; $49=Cylinder Deactivation System Fault; $4A=Torque Model Inaccurate Fault; $4B=Invalid 4WD Low State; $4C=ETC Disable Time; $4E=Ethanol Learn In Progress; $4F=Ethanol Concentration Too High |

This PID contains one of the last eight reasons that for the reactivation of all engine cylinders.  (Deactivate = half-cylinder mode, Reactivate = all-cylinder mode)  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CDAR – PID $2017 – Displacement on Demand Reactivation History 5

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2017 | Displacement on Demand Reactivation History 5 | 1 | ENM | N/A | $00=None; $01=Engine Manifold Vacuum Pressure Low; $02=Brake Vacuum Pressure Low; $03=Accelerator Pedal Position Increase High; $04=Accelerator Pedal Position High; $05=Ignition Voltage Out of Range; $06=Engine Oil Pressure Out of Range; $07=Engine Oil Temperature Out of Range; $08=Engine Coolant Temperature Out of Range; $09=Engine Speed Out of Range; $0A=Transmission Gear Incorrect; $0B=Transmission Range Incorrect; $0C=Transmission Gear Shift in Progress; $0D=All Cylinders Activated via Device Control; $0E=Minimum Time with All Cylinders Activated Not Met; $0F=Maximum Deactivation Time Exceeded; $10=Engine Oil Aeration Present; $11=Decel Fuel Cutoff Active; $12=Fuel Shut-Off Stability Timer Active; $13=Heater Performance; $30=Manifold Absolute Pressure Sensor Fault; $31=Brake Vacuum Pressure Sensor Fault; $32=Electronic Throttle Control Reduced Engine Power Active; $33=Engine Oil Pressure Sensor Fault; $34=Engine Coolant Temperature Sensor Fault; $35=Intake Air Temperature Sensor Fault; $36=Vehicle Speed Fault; $37=Displacement on Demand Solenoid Driver Fault; $38=Engine Misfire Detected; $39=Crank Sensor Fault; $3A=Cam Sensor Fault; $3B=Powertrain Relay Fault; $3E=Engine Metal Overtemperature Protection Active; $3F=Catalyst Converter Overtemperature Protection Active; $40=Piston Protection Active; $41=Hot Coolant Mode Active; $42=Engine Overspeed Protection Active; $43=AC Clutch Being Engaged; $44=Post O2 Diagnostic Intrusive Active; $45=In 4WD Low; $46=Vehicle Speed Too Low; $47=Tip In Bump Active; $48=Catalyst Temperature Low; $49=Cylinder Deactivation System Fault; $4A=Torque Model Inaccurate Fault; $4B=Invalid 4WD Low State; $4C=ETC Disable Time; $4E=Ethanol Learn In Progress; $4F=Ethanol Concentration Too High |

This PID contains one of the last eight reasons for the reactivation of all engine cylinders.  (Deactivate = half-cylinder mode, Reactivate = all-cylinder mode)  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CDAR – PID $2018 – Displacement on Demand Reactivation History 6

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2018 | Displacement on Demand Reactivation History 6 | 1 | ENM | N/A | $00=None; $01=Engine Manifold Vacuum Pressure Low; $02=Brake Vacuum Pressure Low; $03=Accelerator Pedal Position Increase High; $04=Accelerator Pedal Position High; $05=Ignition Voltage Out of Range; $06=Engine Oil Pressure Out of Range; $07=Engine Oil Temperature Out of Range; $08=Engine Coolant Temperature Out of Range; $09=Engine Speed Out of Range; $0A=Transmission Gear Incorrect; $0B=Transmission Range Incorrect; $0C=Transmission Gear Shift in Progress; $0D=All Cylinders Activated via Device Control; $0E=Minimum Time with All Cylinders Activated Not Met; $0F=Maximum Deactivation Time Exceeded; $10=Engine Oil Aeration Present; $11=Decel Fuel Cutoff Active; $12=Fuel Shut-Off Stability Timer Active; $13=Heater Performance; $30=Manifold Absolute Pressure Sensor Fault; $31=Brake Vacuum Pressure Sensor Fault; $32=Electronic Throttle Control Reduced Engine Power Active; $33=Engine Oil Pressure Sensor Fault; $34=Engine Coolant Temperature Sensor Fault; $35=Intake Air Temperature Sensor Fault; $36=Vehicle Speed Fault; $37=Displacement on Demand Solenoid Driver Fault; $38=Engine Misfire Detected; $39=Crank Sensor Fault; $3A=Cam Sensor Fault; $3B=Powertrain Relay Fault; $3E=Engine Metal Overtemperature Protection Active; $3F=Catalyst Converter Overtemperature Protection Active; $40=Piston Protection Active; $41=Hot Coolant Mode Active; $42=Engine Overspeed Protection Active; $43=AC Clutch Being Engaged; $44=Post O2 Diagnostic Intrusive Active; $45=In 4WD Low; $46=Vehicle Speed Too Low; $47=Tip In Bump Active; $48=Catalyst Temperature Low; $49=Cylinder Deactivation System Fault; $4A=Torque Model Inaccurate Fault; $4B=Invalid 4WD Low State; $4C=ETC Disable Time; $4E=Ethanol Learn In Progress; $4F=Ethanol Concentration Too High |

This PID contains one of the last eight reasons for the reactivation of all engine cylinders.  (Deactivate = half-cylinder mode, Reactivate = all-cylinder mode)  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CDAR – PID $2019 – Displacement on Demand Reactivation History 7

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2019 | Displacement on Demand Reactivation History 7 | 1 | ENM | N/A | $00=None; $01=Engine Manifold Vacuum Pressure Low; $02=Brake Vacuum Pressure Low; $03=Accelerator Pedal Position Increase High; $04=Accelerator Pedal Position High; $05=Ignition Voltage Out of Range; $06=Engine Oil Pressure Out of Range; $07=Engine Oil Temperature Out of Range; $08=Engine Coolant Temperature Out of Range; $09=Engine Speed Out of Range; $0A=Transmission Gear Incorrect; $0B=Transmission Range Incorrect; $0C=Transmission Gear Shift in Progress; $0D=All Cylinders Activated via Device Control; $0E=Minimum Time with All Cylinders Activated Not Met; $0F=Maximum Deactivation Time Exceeded; $10=Engine Oil Aeration Present; $11=Decel Fuel Cutoff Active; $12=Fuel Shut-Off Stability Timer Active; $13=Heater Performance; $30=Manifold Absolute Pressure Sensor Fault; $31=Brake Vacuum Pressure Sensor Fault; $32=Electronic Throttle Control Reduced Engine Power Active; $33=Engine Oil Pressure Sensor Fault; $34=Engine Coolant Temperature Sensor Fault; $35=Intake Air Temperature Sensor Fault; $36=Vehicle Speed Fault; $37=Displacement on Demand Solenoid Driver Fault; $38=Engine Misfire Detected; $39=Crank Sensor Fault; $3A=Cam Sensor Fault; $3B=Powertrain Relay Fault; $3E=Engine Metal Overtemperature Protection Active; $3F=Catalyst Converter Overtemperature Protection Active; $40=Piston Protection Active; $41=Hot Coolant Mode Active; $42=Engine Overspeed Protection Active; $43=AC Clutch Being Engaged; $44=Post O2 Diagnostic Intrusive Active; $45=In 4WD Low; $46=Vehicle Speed Too Low; $47=Tip In Bump Active; $48=Catalyst Temperature Low; $49=Cylinder Deactivation System Fault; $4A=Torque Model Inaccurate Fault; $4B=Invalid 4WD Low State; $4C=ETC Disable Time; $4E=Ethanol Learn In Progress; $4F=Ethanol Concentration Too High |

This PID contains one of the last eight reasons for the reactivation of all engine cylinders.  (Deactivate = half-cylinder mode, Reactivate = all-cylinder mode)  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### CDAR – PID $201A – Displacement on Demand Reactivation History 8

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $201A | Displacement on Demand Reactivation History 8 | 1 | ENM | N/A | $00=None; $01=Engine Manifold Vacuum Pressure Low; $02=Brake Vacuum Pressure Low; $03=Accelerator Pedal Position Increase High; $04=Accelerator Pedal Position High; $05=Ignition Voltage Out of Range; $06=Engine Oil Pressure Out of Range; $07=Engine Oil Temperature Out of Range; $08=Engine Coolant Temperature Out of Range; $09=Engine Speed Out of Range; $0A=Transmission Gear Incorrect; $0B=Transmission Range Incorrect; $0C=Transmission Gear Shift in Progress; $0D=All Cylinders Activated via Device Control; $0E=Minimum Time with All Cylinders Activated Not Met; $0F=Maximum Deactivation Time Exceeded; $10=Engine Oil Aeration Present; $11=Decel Fuel Cutoff Active; $12=Fuel Shut-Off Stability Timer Active; $13=Heater Performance; $30=Manifold Absolute Pressure Sensor Fault; $31=Brake Vacuum Pressure Sensor Fault; $32=Electronic Throttle Control Reduced Engine Power Active; $33=Engine Oil Pressure Sensor Fault; $34=Engine Coolant Temperature Sensor Fault; $35=Intake Air Temperature Sensor Fault; $36=Vehicle Speed Fault; $37=Displacement on Demand Solenoid Driver Fault; $38=Engine Misfire Detected; $39=Crank Sensor Fault; $3A=Cam Sensor Fault; $3B=Powertrain Relay Fault; $3E=Engine Metal Overtemperature Protection Active; $3F=Catalyst Converter Overtemperature Protection Active; $40=Piston Protection Active; $41=Hot Coolant Mode Active; $42=Engine Overspeed Protection Active; $43=AC Clutch Being Engaged; $44=Post O2 Diagnostic Intrusive Active; $45=In 4WD Low; $46=Vehicle Speed Too Low; $47=Tip In Bump Active; $48=Catalyst Temperature Low; $49=Cylinder Deactivation System Fault; $4A=Torque Model Inaccurate Fault; $4B=Invalid 4WD Low State; $4C=ETC Disable Time; $4E=Ethanol Learn In Progress; $4F=Ethanol Concentration Too High |

This PID contains one of the last eight reasons for the reactivation of all engine cylinders.  (Deactivate = half-cylinder mode, Reactivate = all-cylinder mode)  A FIFO history buffer, which retains its contents over a battery disconnect, is used to store these reasons with reason 1 containing the most recent reason.  If the most recent reason in the history buffer is the same as the current reason, the history buffer is not updated.

### MTCR – PID $201B – Clutch Pedal Position Sensor A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $201B | Clutch Pedal Position Sensor A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the clutch pedal position sensor analog input as a percentage of its reference voltage.

### MTCR – PID $201C – Clutch Pedal Fully Applied Learned Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $201C | Clutch Pedal Fully Applied Learned Position | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the position learned for the clutch pedal position sensor when the clutch pedal is fully applied during the associated learn procedure.

### MTCR – PID $201D – Clutch Pedal Fully Applied Position Learn Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $201D | Clutch Pedal Fully Applied Position Learn Status | 1 | ENM | N/A | $00=Not Attempted; $01=In Progress; $02=Successful; $03=Unsuccessful - Low; $04=Unsuccessful - High; $05=Unsuccessful - Moving |

This PID contains the status of the procedure used to learn the clutch pedal fully applied position.

### MTCR – PID $201E – Clutch Pedal Released Learned Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $201E | Clutch Pedal Released Learned Position | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the position learned for the clutch pedal position sensor when the clutch pedal is released.

### MTCR – PID $201F – Clutch Pedal Displacement

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $201F | Clutch Pedal Displacement | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the displacement of the clutch pedal (corrected and normalized based upon its learned released and fully applied positions), where 0 % corresponds to the clutch pedal being released and 100% corresponds to the clutch pedal being fully applied.

### DFIR – PID $2020 – Malfunction Indicator Lamp Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2020 | Malfunction Indicator Lamp Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Malfunction Indicator Lamp Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| MIL On Due to Detected Fault | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Bulb Check MIL Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains signals associated with the status of the malfunction indicator lamp.  More than one of these signals can be active at the same time.  The content of each of these signals is described below.

Malfunction Indicator Lamp Commanded On contains the commanded state of the malfunction indicator lamp (MIL) output.

MIL On Due to Detected Fault indicates that the malfunction indicator lamp is being commanded on due to the failure of a diagnostic test.

Bulb Check MIL Active indicates that the malfunction indicator lamp is being commanded on due to a power up or a repower condition.

### RTDR – PID $2021 – Powertrain Induced Chassis Pitch

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2021 | Powertrain Induced Chassis Pitch | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Powertrain Induced Chassis Pitch | 0 | 7 | 2 | ENM | N/A | $0=Neutral; $1=Lift; $2=Dive |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 2 |  |  |  |
| Reserved | 0 | 2 | 2 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 2 |  |  |  |

This PID keeps track of the commanded state of the RTD Lift/Dive algorithm.

### TCSR – PID $2022 – Traction Control System Requested Torque Control Mode

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2022 | Traction Control System Requested Torque Control Mode | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Normal | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Drop Throttle | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| TCS Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| TCS Active and Drop Throttle | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| EDC Active | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the Requested Torque Signal and the Traction Control System Data Control Signal inputs from the ABS/TCS module.  The content of each of these signals is described below.

Normal Torque Control Mode is indicated when the Requested Torque Signal indicates > 90% duty cycle while the Traction Control System Data Control Signal input is Inactive (high).

Drop Throttle Control Mode is indicated when the Requested Torque Signal indicates > 90% duty cycle while the Traction Control System Data Control Signal input is Active (asserted low).

TCS Active Torque Control Mode is indicated when the Requested Torque Signal is within the TCS region (40% to 90% duty cycle) while the Traction Control System Data Control Signal input is Inactive (high).

TCS Active and Drop Throttle Control Mode is indicated when the Requested Torque Signal is within the TCS region (40% to 90% duty cycle) while the Traction Control System Data Control Signal input is Active (asserted low).

EDC Active Control Mode is indicated when the Requested Torque Signal is within the EDC region (10% to 35% duty cycle) while the Traction Control System Data Control Signal input is Active (asserted low).

### GENR – PID $2023 – Regulated Voltage Control Override Source

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2023 | Regulated Voltage Control Override Source | 1 | ENM | N/A | $00= None; $01= Device Control; $02= Fuel Demand; $03= OBD Diagnostics; $04= Airflow Measurement; $05= Cylinder Deactivation; $06= Transmission Control Module Request; |

This PID contains the source of the Regulated Voltage Control (RVC) override command.  RVC lowers system operating voltage to reduce system electrical load and improve fuel economy performance. The RVC command will be overridden if the voltage commanded is below the minimum system voltage required to ensure that the fuel demand, Powertrain OBD diagnostics, airflow measurement, cylinder deactivation, or Transmission Control Module functions will execute properly.  The RVC function can also be overridden through device controls.

### VLTR – PID $2024 – Buffered 5 Volt Supply Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2024 | Buffered 5 Volt Supply Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| V5B1 Signal Out of Range | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| V5B2 Signal Out of Range | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains signals associated with the status of the Buffered 5 Volt Supplies.

V5B1 Signal Out of Range indicates that the buffered 5 volt supply 1 reading (within the main processor) is currently out of range.  This parameter must not latch or include any MCP V5B information.

V5B2 Signal Out of Range indicates that the buffered 5 volt supply 2 reading (within the main processor) is currently out of range.  This parameter must not latch or include any MCP V5B information.

### E85R – PID $2025 – Fuel Composition Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2025 | Fuel Composition Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Recommended Ethanol Concentration Exceeded | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Composition Learn Status Active | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains signals associated with the status of the fuel composition system.  The content of each of these signals is described below.

Recommended Ethanol Concentration Exceeded indicates that the fuel alcohol percentage has exceeded the recommended value for vehicle conditions.  A true value for this bit will latch until a mode $04 code clear is issued to the controller.  PIDs $209B and $209D - $20A2 will also latch or trigger on this event.

Fuel Composition Learn Active indicates that a refueling event has been detected to cause the fuel composition to be relearned.

### RINR – PID $2026 – Reverse Inhibit System Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2026 | Reverse Inhibit Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reverse Inhibit Supported | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reverse Enable Application | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Reverse Inhibit Application | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reverse Inhibit Active | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains signals associated with the status of the reverse inhibit system.  The content of each of these signals is described below.

Reverse Inhibit Supported indicates that the reverse inhibit solenoid is present on the vehicle.

Reverse Enable Application indicates that the driver is inhibited from shifting into reverse when the Reverse Inhibit output is commanded Off.

Reverse Inhibit Application indicates that the driver is inhibited from shifting into reverse when the Reverse Inhibit output is commanded On.

Reverse Inhibit Active indicates that the driver is inhibited from shifting into reverse (regardless of the reverse enable / inhibit mechanization) .  A value of False will always be provided for this signal when the reverse inhibit solenoid is not present.

### EFVR – PID $2027 – Exhaust Pressure Regulator Valve Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2027 | Exhaust Pressure Regulator Valve Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Exhaust Pressure Regulator Valve Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Exhaust Pressure Regulator Valve Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Exhaust Pressure Regulator Valve Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Exhaust Pressure Regulator Valve Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the exhaust Pressure Regulator valve output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P0475.

### EFVR – PID $2028 – Bank 1 Exhaust Back Pressure A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2028 | Bank 1 Exhaust Back Pressure A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the bank 1 exhaust back pressure analog input as a percentage of its reference voltage.

### EFVR – PID $2029 – Bank 1 Exhaust Back Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2029 | Bank 1 Exhaust Back Pressure | 1 | SNM | -128 – 127 kPa | E = N * 1 |

Bank 1 exhaust back pressure is determined directly from a bank 1 exhaust back pressure sensor in applications where this sensor is available.

### EFVR – PID $202A – Bank 2 Exhaust Back Pressure A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $202A | Bank 2 Exhaust Back Pressure A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the bank 2 exhaust back pressure analog input as a percentage of its reference voltage.

### EFVR – PID $202B – Bank 2 Exhaust Back Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $202B | Bank 2 Exhaust Back Pressure | 1 | SNM | -128 – 127 kPa | E = N * 1 |

Bank 2 exhaust back pressure is determined directly from a bank 2 exhaust back pressure sensor in applications where this sensor is available.

### OXYR – PID $202C – Air Fuel Imbalance Monitor Filtered Ratio Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $202C | Air Fuel Imbalance Monitor Filtered Ratio Bank 1 | 2 | UNM | -16 – 15.999 | E = N / 2048 - 16 |

This PID contains the air fuel imbalance monitor filtered results of the long-term length calculation for Bank 1.

### OXYR – PID $202D – Air Fuel Imbalance Monitor Filtered Ratio Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $202D | Air Fuel Imbalance Monitor Filtered Ratio Bank 2 | 2 | UNM | -16 – 15.999 | E = N / 2048 - 16 |

This PID contains the air fuel imbalance monitor filtered results of the long-term length calculation for Bank 2.

### OXYR – PID $202E – Air Fuel Imbalance Monitor Maximum Filtered Ratio Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $202E | Air Fuel Imbalance Monitor Maximum Filtered Ratio Bank 1 | 2 | UNM | -16 – 15.999 | E = N / 2048 - 16 |

This PID contains the maximum air fuel imbalance monitor filtered results of the long-term length calculation for Bank 1 during the trip after first reporting has occurred.

### OXYR – PID $202F – Air Fuel Imbalance Monitor Maximum Filtered Ratio Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $202F | Air Fuel Imbalance Monitor Maximum Filtered Ratio Bank 2 | 2 | UNM | -16 – 15.999 | E = N / 2048 - 16 |

This PID contains the maximum air fuel imbalance monitor filtered results of the long-term length calculation for Bank 2 during the trip after first reporting has occurred.

### MEMR – PID $2032 – Processor Fault Reason Code

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2032 | Processor Fault Reason Code | 1 | ENM | N/A | $00= No Fault; $01= MHC State of Health Fault; $02= Reserved; $03= Reserved; $04= Reserved; $05= MAIN Detected SPI Fault; $06= Reserved; $07= Sensor Min Learn Corruption Fault; $08= Processor Clock Fault; $09= 12.5 ms Loop Completion Time Fault; $0A= 25 ms Loop Completion Time Fault; $0B= 50 ms Loop Completion Time Fault; $0C= 100 ms Loop Completion Time Fault; $0D= 250 ms Loop Completion Time Fault; $0E= RAM Diagnostic Completion Time Fault; $0F= ROM Diagnostic Completion Time Fault; $10= MHC ALU Fault; $11= MHC Configuration Register Fault; $12= MHC Stack Fault; $13= MHC Sequence Fault; $14= MHC Detected MAIN Processor Fault; $15= MHC Detected SPI Fault; $16= Desired Throttle Rationality Fault; $17= Reserved; $18= Reserved; $19= Reserved; $1A= Reserved; $1B= Reserved; $1C= Reserved; $1D= Reserved; $1E= Reserved; $1F= Reserved; $20= MAIN ALU Fault; $21= MAIN Configuration Register Fault |

This PID contains the source of the P0606 Diagnostic Trouble Code.

### MEMR – PID $2033 – Main Processor NVM Fault Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |

| $2033 | Main Processor NVM Fault Status | 2 | PKT | N/A | N/A |
| --- | --- | --- | --- | --- | --- |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Static NVM Read Status | 0 | 7 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Cumulative NVM Read Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Adaptable NVM Read Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Static NVM Checksum Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Cumulative NVM Checksum Status | 1 | 7 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Adaptable NVM Checksum Status | 1 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Reserved | 1 | 3 | 2 |  |  |  |
| Reserved | 1 | 1 | 2 |  |  |  |

This PID contains several signals associated with the status of the main processor NVM read and checksum tests.

### MEMR – PID $2034 – Main Processor RAM Fault Source Address

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2034 | Main Processor RAM Fault Source Address | 4 | UNM | 0 - $FFFFFFFF | E = N |

This PID contains the source address for the latest RAM fault in the main processor.

### MEMR – PID $2035 – Processor ROM Fault Source

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2035 | Processor ROM Fault Source | 1 | ENM | N/A | $00-FD=Main processor module ID of failed checksum region $FE=Secondary processor ROM fault $FF=No checksum faults |

This PID contains the Read Only Memory fault source in the processor.

### VSPR – PID $2049 – ECU Odometer Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2049 | ECU Odometer Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| ECU Odometer Supported | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| ECU Odometer Programmed | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| ECU Odometer VIN Programmed | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains status bits associated with the ECU odometer strategy.  The content of each of these signals is described below.

ECU Odometer Supported indicates if the ECU supports the odometer strategy as detailed in the “Global A Electrical Architecture Vehicle Odometer Sense & Display” functional requirements specification.

ECU Odometer Programmed indicates if the ECU odometer value (DID $DF) has been programmed.  This bit will be used as part of the odometer strategy logic to determine if the odometer VIN value needs to be programmed.

ECU Odometer VIN Programmed indicates if the ECU odometer VIN value has been programmed.  The ECU odometer VIN is stored in DID $28, which is separate from the standard DID $90 VIN value.  This bit will be used as part of the odometer strategy logic to determine if the odometer VIN value needs to be programmed.

### RBSR – PID $204A – Main Processor Total Running Resets

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $204A | Main Processor Total Running Resets | 2 | UNM | 0 – 65,535 | E = N * 1 |

This PID contains the total number of running resets that have occurred since last reprogramming event.  This PID is intended for use by engineering only.  This PID should not be displayed on any off-board device designed for Service.  This PID shall be included in the Controller State of Health Freeze Frame.

### RBSR – PID $204B – Main Processor Maximum Running Resets Between Power-up Resets

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $204B | Main Processor Maximum Running Resets Between Power-up Resets | 2 | UNM | 0 – 65,535 | E = N * 1 |

This PID contains the maximum number of running resets between power up resets, and since last reprogramming event.  This PID is intended for use by engineering only.  This PID should not be displayed on any off-board device designed for Service.  This PID shall be included in the Controller State of Health Freeze Frame.

### TOSR – PID $204C – Replicated Transmission Output Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $204C | Replicated Transmission Output Speed | 2 | UNM | 0 - 8191.875 rpm | E = N * 0.125 |

This PID contains the Replicated Transmission Output Speed (Replicated TOS) received from the Transmission Control Module.  Replicated TOS results from hardware in the TCM that “re-transmits” the Transmission Output Speed Sensor pulses to the ECM.

### TRQR – PID $204D – Transmission Torque Request Limited Reason

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $204D | Transmission Torque Request Limited Reason | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Minimum Idle Area | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Spark Advance | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Throttle Position Security | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Engine Minimum Torque | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Throttle Arbitration | 0 | 3 | 1 | BLN | N/A | $1=True; $0=False |
| Spark Arbitration | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Arbitration | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID indicates the existing condition in the ECM causing the transmission torque request to be limited.

### TOSR – PID $2050 – Transfer Case Output Speed

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2050 | Transfer Case Output Speed | 2 | UNM | 0 - 8191.875 rpm | E = N * 0.125 |

This PID contains the transfer case output speed (the speed at the output of the transfer case) determined from the speed measured by the transfer case speed sensor (TCSS).

### KNKR – PID $2051 – Knock System Status Sensor 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2051 | Knock System Status Sensor 1 | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Knock Circuit Test Status | 0 | 7 | 2 | ENM | N/A | $0=Passed; $1=Failed; $2=Incomplete |
| Knock Sensor 1 Circuit Fault | 0 | 5 | 2 | ENM | N/A | $0=Passed; $1=Failed; $2=Incomplete |
| Knock Sensor 1 Signal Low | 0 | 3 | 2 | ENM | N/A | $0=Passed; $1=Failed; $2=Incomplete |
| Knock Sensor 1 Signal High | 0 | 1 | 2 | ENM | N/A | $0=Passed; $1=Failed; $2=Incomplete |

This PID contains several signals associated with the status of the knock system.  The content of each of these signals is described below.

Knock Circuit Test Status indicates the current status of fault conditions within the electronic circuits used to perform knock detection.

Knock Sensor 1 Circuit Fault indicates knock sensor 1 circuit faults other than stuck low/high.

Knock Sensor 1 Signal Low indicates the knock sensor 1 signal low fault test status.

Knock Sensor 1 Signal High indicates the knock sensor 1 signal high fault test status.

### KNKR – PID $2052 – Knock System Status Sensor 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2052 | Knock System Status Sensor 2 | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Knock Sensor 2 Circuit Fault | 0 | 5 | 2 | ENM | N/A | $0=Passed; $1=Failed; $2=Incomplete |
| Knock Sensor 2 Signal Low | 0 | 3 | 2 | ENM | N/A | $0=Passed; $1=Failed; $2=Incomplete |
| Knock Sensor 2 Signal High | 0 | 1 | 2 | ENM | N/A | $0=Passed; $1=Failed; $2=Incomplete |

This PID contains several signals associated with the status of the knock system.  The content of each of these signals is described below.

Knock Sensor 2 Circuit Fault indicates knock sensor 2 circuit faults other than stuck low/high.

Knock Sensor 2 Signal Low indicates the knock sensor 2 signal low fault test status.

Knock Sensor 2 Signal High indicates the knock sensor 2 signal high fault test status.

### TPSR – PID $2066 – Learned Airflow Variation Algorithm Compensation

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2066 | Learned Airflow Variation Algorithm Compensation | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the percent compensation of the Learned Airflow Variation Algorithm.  This value indicates the current amount of airflow compensation in response to engine intake airflow restrictions (i.e. throttle body coking).  It can also be used as feedback for the Learned Airflow Adapts Reset Trigger Device Control.

### CATR – PID $206C – Catalyst Monitor Diagnostic Status Bank 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $206C | Catalyst Monitor Diagnostic Status Bank 1 | 1 | ENM | N/A | $00=No Decision; $01=Failed; $02=Passed; |

This PID indicates the status of the catalyst monitor diagnostic for bank 1.  Multiple tests may run before the diagnostic reports a “Pass” or “Fail” and will be tracked by PID $1354 “Catalyst Monitor Diagnostic Test Counter Bank 1”.  When this PID reports a “Pass” or “Fail”, the diagnostic is complete for the current trip and will not attempt any further tests on bank 1.

### CATR – PID $206D – Catalyst Monitor Diagnostic Status Bank 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $206D | Catalyst Monitor Diagnostic Status Bank 2 | 1 | ENM | N/A | $00=No Decision; $01=Failed; $02=Passed; |

This PID indicates the status of the catalyst monitor diagnostic for bank 2.  Multiple tests may run before the diagnostic reports a “Pass” or “Fail” and will be tracked by PID $1355 “Catalyst Monitor Diagnostic Test Counter Bank 2”.  When this PID reports a “Pass” or “Fail”, the diagnostic is complete for the current trip and will not attempt any further tests on bank 2.

### CATR – PID $206E – Catalyst Monitor Diagnostic Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $206E | Catalyst Monitor Diagnostic Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Catalyst Monitor Test Running | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Off Idle Conditions Met | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID indicates the status of the catalyst monitor diagnostic.

Catalyst Monitor Test Running indicates when the catalyst monitor diagnostic is actively running a test.  This condition will only be true when the vehicle is at idle.  When Catalyst Monitor Test Running transitions from “True” to “False” with the vehicle at idle, PIDs $1354 & $1355 “Catalyst Monitor Diagnostic Test Counter Bank 1 & 2” can be referenced along with PIDs $206C & $206D “Catalyst Monitor Diagnostic Status Bank 1 & 2” to determine the results of the test.

Off Idle Conditions Met indicates if the off idle conditions for the catalyst monitor diagnostic have been met.  Once the Off Idle Conditions Met bit equals “True”, the vehicle can return to idle and allow the catalyst monitor diagnostic test to run.

### ENGR – PID $2070 – Reduced Engine Power Indication History

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2070 | Reduced Engine Power Indication History | 1 | ENM | N/A | $01=ETC Fault; $02=EV Fan Fault; $03=Engine Metal Ovrtemp; $04=Engine Oil Overtemp; $05=OnStar Speed Limiting |

This PID contains the last reason that powertrain has illuminated the Reduced Engine Power (REP) indication.

### E85R – PID $209B – Recommended Maximum Ethanol Concentration When Recommended Ethanol Concentration Exceeded

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $209B | Recommended Maximum Ethanol Concentration When Recommended Ethanol Concentration Exceeded | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID indicates the recommended fuel alcohol percentage for vehicle conditions at the most recent time that the Recommended Ethanol Concentration Exceeded bit became true.  This PID value will latch until a mode $04 code clear is issued to the controller.  If a Recommended Ethanol Concentration Exceeded event has not taken place (either initially or after a mode $04), this PID shall default to zero.

### E85R – PID $209C – Currently Recommended Maximum Ethanol Concentration

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $209C | Currently Recommended Maximum Ethanol Concentration | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID indicates the recommended maximum fuel alcohol percentage for current vehicle conditions.

### E85R – PID $209D – Actual Ethanol Composition When Recommended Ethanol Concentration Exceeded

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $209D | Actual Ethanol Composition When Recommended Ethanol Concentration Exceeded | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID indicates the actual fuel alcohol percentage at the most recent time that the Recommended Ethanol Concentration Exceeded bit became true.  In vehicles with an ethanol concentration sensor, this will be a measured value.  In vehicles without a physical sensor, this will be a calculated value.  This PID will latch until a mode $04 code clear is issued to the controller.  If a Recommended Ethanol Concentration Exceeded event has not taken place (either initially or after a mode $04), this PID shall default to zero.

### E85R – PID $209E – Ambient Air Temperature When Recommended Ethanol Concentration Exceeded

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $209E | Ambient Air Temperature When Recommended Ethanol Concentration Exceeded | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID indicates the estimated ambient air temperature at the most recent time that the Recommended Ethanol Concentration Exceeded bit became true.  This PID value will latch until a mode $04 code clear is issued to the controller.  If a Recommended Ethanol Concentration Exceeded event has not taken place (either initially or after a mode $04), this PID shall default to zero.

### E85R – PID $209F – Ignition Cycles Since Recommended Ethanol Concentration Exceeded

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $209F | Ignition Cycles Since Recommended Ethanol Concentration Exceeded | 1 | UNM | 0 – 255 counts | E = N * 1 |

This PID indicates the number of ignitions cycles since the most recent time that the Recommended Ethanol Concentration Exceeded bit became true.  This PID value will reset when a mode $04 code clear is issued to the controller.  If a Recommended Ethanol Concentration Exceeded event has not taken place (either initially or after a mode $04), this PID shall default to zero.

### E85R – PID $20A0 – Distance Since Recommended Ethanol Concentration Exceeded

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $20A0 | Distance Since Recommended Ethanol Concentration Exceeded | 2 | UNM | 0 - 327675 km | E = N * 5 |

This PID indicates the accumulated distance driven since the most recent time that the Recommended Ethanol Concentration Exceeded bit became true.  This PID value will reset when a mode $04 code clear is issued to the controller.  If a Recommended Ethanol Concentration Exceeded event has not taken place (either initially or after a mode $04), this PID shall default to zero.

### E85R – PID $20A1 – Fuel Consumed Since Recommended Ethanol Concentration Exceeded

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $20A1 | Fuel Consumed Since Recommended Ethanol Concentration Exceeded | 1 | UNM | 0 – 255 liters | E = N * 1 |

This PID indicates the amount of fuel consumed since the most recent time that the Recommended Ethanol Concentration Exceeded bit became true.  This PID value will reset when a mode $04 code clear is issued to the controller.  If a Recommended Ethanol Concentration Exceeded event has not taken place (either initially or after a mode $04), this PID shall default to zero.

### E85R – PID $20A2 – Refueling Events Since Recommended Ethanol Concentration Exceeded

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $20A2 | Refueling Events Since Recommended Ethanol Concentration Exceeded | 1 | UNM | 0 – 255 counts | E = N * 1 |

This PID indicates the number of times the vehicle has been refueled since the most recent time that the Recommended Ethanol Concentration Exceeded bit became true.  This PID value will reset when a mode $04 code clear is issued to the controller.  If a Recommended Ethanol Concentration Exceeded event has not taken place (either initially or after a mode $04), this PID shall default to zero.

### E85R – PID $20A3 – Fuel Composition Sensor Frequency

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $20A3 | Fuel Composition Sensor Frequency | 1 | UNM | 0 – 255 Hz | E = N * 1 |

This PID contains the frequency of the fuel composition sensor input.  This value can be decoded to determine fuel composition or sensor fault status, depending on the value.

### DMDR – PID $20D7 – Economy Mode Switch Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $20D7 | Economy Mode Switch Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Economy Mode Driver Request | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Economy Mode Indication On | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Economy Mode Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Economy Mode Switch State | 0 | 4 | 3 | ENM | N/A | $0=Fault Low; $1=On; $2=Indeterminate; $3=Off; $4=Fault High |
| Economy Mode Switch Both States Latched | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains signals associated with the status of the economy mode switch.  The content of each of these signals is described below.

Economy Mode Driver Request indicates if the driver of the vehicle has requested economy mode by pressing the economy mode switch.

Economy Mode Indication On indicates the status of the serial data signal used to illuminate the economy mode indicator lamp.

Economy Mode Active indicates if the economy mode algorithm has activated economy mode.

Economy Mode Switch State indicates the raw economy mode switch state information.

Economy Mode Switch Both States Latched indicates true once both the on and off switch states are observed and will remain latched for the current ignition cycle.  This bit will reset to false once the power mode transitions to off.

### DMDR – PID $20D8 – Economy Mode Switch A/D

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $20D8 | Economy Mode Switch A/D | 1 | UNM | 0 - 100 % 5V | E = N * 100 / 255 |

This PID contains the economy mode switch analog input as a percentage of its reference voltage.

### SHPR – PID $2800 – Transmission Shift Solenoid A Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2800 | Transmission Shift Solenoid A Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission Shift Solenoid A Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Transmission Shift Solenoid A Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission Shift Solenoid A Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission Shift Solenoid A Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the transmission shift solenoid A output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P0973 and P0974.

### SHPR – PID $2801 – Transmission Shift Solenoid B Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2801 | Transmission Shift Solenoid B Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission Shift Solenoid B Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Transmission Shift Solenoid B Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission Shift Solenoid B Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission Shift Solenoid B Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the transmission shift solenoid B output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P0976 and P0977.

### SHPR – PID $2802 – Transmission Shift Solenoid C Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2802 | Transmission Shift Solenoid C Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission Shift Solenoid C Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Transmission Shift Solenoid C Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission Shift Solenoid C Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission Shift Solenoid C Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the transmission shift solenoid C output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P0979 and P0980.

### D32R – PID $2803 – Transmission 3-2 Downshift Solenoid Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2803 | Transmission 3-2 Downshift Solenoid Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission 3-2 Downshift Solenoid Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Transmission 3-2 Downshift Solenoid Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission 3-2 Downshift Solenoid Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission 3-2 Downshift Solenoid Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the transmission 3-2 downshift solenoid output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P0787 and P0788.

### HSDR – PID $2804 – Transmission High Side Driver 1 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2804 | Transmission High Side Driver 1 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission High Side Driver 1 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Transmission High Side Driver 1 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission High Side Driver 1 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission High Side Driver 1 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the transmission high side driver 1 output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P1831 and P1832.

### HSDR – PID $2805 – Transmission High Side Driver 2 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2805 | Transmission High Side Driver 2 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission High Side Driver 2 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Transmission High Side Driver 2 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission High Side Driver 2 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission High Side Driver 2 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the transmission high side driver 2 output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P1833 and P1834.

### TCCR – PID $2806 – Torque Converter Clutch Enable Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2806 | Torque Converter Clutch Enable Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Torque Converter Clutch Enable Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Torque Converter Clutch Enable Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Torque Converter Clutch Enable Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Torque Converter Clutch Enable Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the torque converter clutch (TCC) enable output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P2769 and P2770.

### TCCR – PID $2807 – Torque Converter Clutch Control Solenoid Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2807 | Torque Converter Clutch Control Solenoid Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Torque Converter Clutch Control Solenoid Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Torque Converter Clutch Control Solenoid Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Torque Converter Clutch Control Solenoid Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Torque Converter Clutch Control Solenoid Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the torque converter clutch (TCC) control solenoid output (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P2763 and P2764.

### TPCR – PID $2808 – Transmission Line Pressure Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2808 | Transmission Line Pressure Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission Line Pressure Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Transmission Line Pressure Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission Line Pressure Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Transmission Line Pressure Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the transmission line pressure output (the output is considered to be commanded on if its duty cycle is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P0960, P0962, and P0963.

### TCCR – PID $2809 – Torque Converter Clutch Reference Slip

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2809 | Torque Converter Clutch Reference Slip | 2 | UNM | 0 - 8191.875 rpm | E = N * 0.125 |

This PID contains the torque converter clutch (TCC) reference slip, the desired slip when the electronically controlled converter clutch (ECCC) algorithm is operating in on mode, determined as a function of commanded gear, turbine speed, engine torque, and other parameters.

### SHTR – PID $280A – Transmission 5-6 Upshift Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $280A | Transmission 5-6 Upshift Time | 1 | UNM | 0 - 6.375 s | E = N * 0.025 |

This PID contains the duration of the ratio change portion of the most recent transmission 5-6 upshift.

### SPRR – PID $280B – Transmission 5-6 Upshift Time Error

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $280B | Transmission 5-6 Upshift Time Error | 1 | SNM | -3.2 - 3.175 s | E = N * 0.025 |

This PID contains the difference between the desired and actual shift times for the most recent transmission 5-6 upshift, which is used to modify the transmission 5-6 upshift pressure adapts.

### TCCR – PID $280C – Torque Converter Clutch Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $280C | Torque Converter Clutch Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Torque Converter Clutch Control Solenoid Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Torque Converter Clutch Enable Commanded On | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Torque Converter Clutch Release Switch Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the torque converter clutch (TCC) system.  The content of each of these signals is described below.

Torque Converter Clutch Control Solenoid Commanded On indicates that the TCC control solenoid output is commanded on (the output is considered to be commanded on if its duty cycle is not zero).

Torque Converter Clutch Enable Commanded On indicates that the TCC enable output is commanded on.

Torque Converter Clutch Release Switch Active indicates the debounced state of the TCC release switch (TCC is not applied when the release switch is active).

### TFTR – PID $280D – Transmission Control Module Substrate Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $280D | Transmission Control Module Substrate Temperature | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the temperature measured by a sensor located on the substrate of the transmission control module, which is used to determine when special actions are required to protect control module components from an overtemperature condition.

### TOSR – PID $280E – Replicated Transmission Output Speed Output Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $280E | Replicated Transmission Output Speed Output Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Replicated Transmission Output Speed Output Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Replicated Transmission Output Speed Output Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Replicated Transmission Output Speed Output Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Replicated Transmission Output Speed Output Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the replicated transmission output speed (TOS) output (the output is considered to be commanded on if TOS is not zero) along with the status of its associated short to ground, open circuit, and short to power tests used to report DTC P1729.

### TBPR – PID $280F – Transmission Base Pattern

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $280F | Transmission Base Pattern | 1 | ENM | N/A | $00=Tap Up/Down Detent; $01=Tap Up/Down; $02=Hot Mode Tap Up/Down; $03=Hot Mode Detent; $04=Hot Mode; $05=Hot Engine Detent; $06=Switch A Detent; $07=Switch A; $08=Switch B Detent; $09=Switch B; $0A=Default Detent; $0B=Default $0C = 4WD Low Detent $0D = Hot Mode Manual Up / Manual Down $0E = Manual Up / Manual Down $0F = Tap Up / Tap Down Cruise |

This PID contains the current transmission base pattern determined from the transmission tap up/down status, detent status, hot mode determination, hot engine determination, and driver selected patterns.  Calibrations are used to map the switch A and switch B patterns to the appropriate driver selected patterns (e.g., cruise, trailer, sport, winter, etc.) for a particular application.

### TPSR – PID $2810 – Accelerator Effective Position

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2810 | Accelerator Effective Position | 1 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

This PID contains the accelerator effective position, which represents the driver’s intended request for torque or acceleration.  It is determined from the maximum of the driver requests to open the throttle (e.g., accelerator pedal, cruise control, PTO, etc.) and is normalized to 0 to 100 %, where 0 % represents an idle or coast request and 100% represents a request for wide open throttle (WOT).  It is the primary indication of accelerator position / throttle position used by transmission control algorithms and, thus, is often referred to as transmission throttle position.  In the event that the engine is operating in a forced idle mode due to an accelerator pedal position fault, the accelerator effective position will have a value of 0 %, corresponding to an idle request.

On Heavy Duty Truck applications with ETC and incorporating a maximum vehicle speed within the normal operating region, this signal may be limited by the ECM with a bit selectable option.  If the limiting option is selected and the vehicle speed is above the maximum vehicle speed, the Accelerator Effective Position will be replaced by the "Back Calculated Pedal" position calculated for the current operating condition.  This option is intended only to be used by heavy duty truck applications to allow proper transmission shifting during vehicle speed governed conditions.

### TPCR – PID $2811 – Pressure Control Solenoid 1 Output Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2811 | Pressure Control Solenoid 1 Output Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Pressure Control Solenoid 1 Stuck Low Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Pressure Control Solenoid 1 Stuck High Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Pressure Control Solenoid 1 Rationality Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the status of the pressure control solenoid (PCS) 1 output’s stuck low, stuck high, and rationality tests used to report DTCs P0962, P0963, and P0961, respectively.

### RSCR – PID $2812 – Pressure Control Solenoid 2 Output Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2812 | Pressure Control Solenoid 2 Output Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Pressure Control Solenoid 2 Stuck Low Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Pressure Control Solenoid 2 Stuck High Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Pressure Control Solenoid 2 Rationality Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the status of the pressure control solenoid (PCS) 2 output’s stuck low, stuck high, and rationality tests used to report DTCs P0966, P0967, and P0965, respectively.

### RSCR – PID $2813 – Pressure Control Solenoid 3 Output Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2813 | Pressure Control Solenoid 3 Output Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Pressure Control Solenoid 3 Stuck Low Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Pressure Control Solenoid 3 Stuck High Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Pressure Control Solenoid 3 Rationality Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the status of the pressure control solenoid (PCS) 3 output’s stuck low, stuck high, and rationality tests used to report DTCs P0970, P0971, and P0969, respectively.

### RSCR – PID $2814 – Pressure Control Solenoid 4 Output Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2814 | Pressure Control Solenoid 4 Output Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Pressure Control Solenoid 4 Stuck Low Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Pressure Control Solenoid 4 Stuck High Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Pressure Control Solenoid 4 Rationality Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the status of the pressure control solenoid (PCS) 4 output’s stuck low, stuck high, and rationality tests used to report DTCs P2720, P2721, and P2719, respectively.

### RSCR – PID $2815 – Pressure Control Solenoid 5 Output Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2815 | Pressure Control Solenoid 5 Output Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Pressure Control Solenoid 5 Stuck Low Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Pressure Control Solenoid 5 Stuck High Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Pressure Control Solenoid 5 Rationality Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the status of the pressure control solenoid (PCS) 5 output’s stuck low, stuck high, and rationality tests used to report DTCs P2729, P2730, and P2728, respectively.

### TCCR – PID $2816 – Torque Converter Clutch Pressure Control Solenoid Output Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2816 | Torque Converter Clutch Pressure Control Solenoid Output Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Torque Converter Clutch Pressure Control Solenoid Stuck Low Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Torque Converter Clutch Pressure Control Solenoid Stuck High Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Torque Converter Clutch Pressure Control Solenoid Rationality Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the status of the torque converter clutch (TCC) pressure control solenoid (PCS) output’s stuck low, stuck high, and rationality tests used to report DTCs P2763, P2764, and P2762, respectively.

### TPCR – PID $2817 – Pressure Control Solenoid 1 Commanded Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2817 | Pressure Control Solenoid 1 Commanded Pressure | 2 | UNM | 0 - 4095.9375 kPa | E = N * 0.0625 |

This PID contains the commanded pressure for pressure control solenoid (PCS) 1.

### RSCR – PID $2818 – Pressure Control Solenoid 2 Commanded Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2818 | Pressure Control Solenoid 2 Commanded Pressure | 2 | UNM | 0 - 4095.9375 kPa | E = N * 0.0625 |

This PID contains the commanded pressure for pressure control solenoid (PCS) 2.

### RSCR – PID $2819 – Pressure Control Solenoid 3 Commanded Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2819 | Pressure Control Solenoid 3 Commanded Pressure | 2 | UNM | 0 - 4095.9375 kPa | E = N * 0.0625 |

This PID contains the commanded pressure for pressure control solenoid (PCS) 3.

### RSCR – PID $281A – Pressure Control Solenoid 4 Commanded Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $281A | Pressure Control Solenoid 4 Commanded Pressure | 2 | UNM | 0 - 4095.9375 kPa | E = N * 0.0625 |

This PID contains the commanded pressure for pressure control solenoid (PCS) 4.

### RSCR – PID $281B – Pressure Control Solenoid 5 Commanded Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $281B | Pressure Control Solenoid 5 Commanded Pressure | 2 | UNM | 0 - 4095.9375 kPa | E = N * 0.0625 |

This PID contains the commanded pressure for pressure control solenoid (PCS) 5.

### RSCR – PID $281C – Torque Converter Clutch Pressure Control Solenoid Commanded Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $281C | Torque Converter Clutch Pressure Control Solenoid Commanded Pressure | 2 | UNM | 0 - 4095.9375 kPa | E = N * 0.0625 |

This PID contains the commanded pressure for the torque converter clutch (TCC) pressure control solenoid (PCS).

### DGSR – PID $281D – Transmission Fluid Pressure Switch Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $281D | Transmission Fluid Pressure Switch Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Pressure Switch 1 Active | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Pressure Switch 2 Active | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Pressure Switch 3 Active | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Pressure Switch 4 Active | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains the status of the individual transmission fluid pressure switches.  An active pressure switch indicates that the associated clutch has been exhausted while an inactive pressure switch indicates that pressure is applied to the associated clutch.

### RSSR – PID $281E – Shift Solenoid 1 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $281E | Shift Solenoid 1 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Shift Solenoid 1 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Shift Solenoid 1 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Shift Solenoid 1 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Shift Solenoid 1 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the shift solenoid 1 (mode 2 solenoid) output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P0973 and P0974.

### RSSR – PID $281F – Shift Solenoid 2 Control Circuit

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $281F | Shift Solenoid 2 Control Circuit | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Shift Solenoid 2 Commanded On | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Shift Solenoid 2 Short to Ground Test Status | 0 | 5 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Shift Solenoid 2 Open Circuit Test Status | 0 | 3 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |
| Shift Solenoid 2 Short to Power Test Status | 0 | 1 | 2 | ENM | N/A | $0=Failed; $1=Indeterminate; $3=Passed |

This PID contains the commanded state of the shift solenoid 2 (mode 3 solenoid) output along with the status of its associated short to ground, open circuit, and short to power tests used to report DTCs P0976 and P0977.

### RSSR – PID $2820 – Transmission Primary Oncoming Clutch

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2820 | Transmission Primary Oncoming Clutch | 1 | ENM | N/A | $01 - C1 Clutch (C1234) $02 - C2 Clutch (C456) $03 - C3 Clutch (C35R) $04 - C4 Clutch (CB26) $05 - C5 Clutch (CBR1) $06 - No Clutch |

This PID indicates the transmission primary oncoming clutch (the clutch being filled).

### CCER – PID $2821 – Transmission Full Feed Fill Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2821 | Transmission Full Feed Fill Pressure | 2 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |

The PID contains the commanded transmission full feed clutch fill pressure.

### TOSR – PID $2822 – TISS/TOSS Regulated Voltage Supply Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2822 | TISS/TOSS Regulated Voltage Supply Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| TISS/TOSS Regulated Voltage Supply Status | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Reserved | 0 | 1 | 1 |  |  |  |
| Reserved | 0 | 0 | 1 |  |  |  |

Indicates the regulated TISS/TOSS supply voltage is outside of the normal operating range as specified in the operating specifications. This PID does not indicate whether the fault is external or internal to the TCM. Measurements are made by TCM internal components.

### RSCR – PID $2823 – Service Fast Learn Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2823 | Service Fast Learn Status | 1 | ENM | N/A | $00=Fast Learn Disabled; $01= Fast Learn Completed; $02=Service Brake Must be Applied; $03=Sump Temp Below Operating Range; $04= Sump Temp Above Operating Range; $05=Park/Neutral Must Be Selected; $06=Drive Must Be Selected; $07= Reverse Must Be Selected; $08=Max Throttle/Eng Speed Exceeded; $09=Low Engine Speed Detected; $0A=Max Output Speed Exceeded; $0B= Trans Purge in Progress; $0C=Drive Calibration test in Progress; $0D= Drive Calibration test Complete; $0E=Reverse Calibration test in Progress; $0F=Abort-Test Criteria Not Met $10=Abort-Test Failed |

This PID contains the status of the Service Fast Learn procedure.

### RSCR – PID $2824 – Transmission Cleaning Procedure Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2824 | Transmission Cleaning Procedure Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | 0 | 7 | 1 |  |  |  |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Transmission Cleaning Procedure complete | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Cleaning Procedure aborted | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Cleaning Procedure in progress | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |

This PID contains the status of the transmission cleaning procedure.

### TFTR – PID $2826 –Transmission Control Module Restart Sensor Temperature

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2826 | Transmission Control Module Restart Sensor Temperature | 1 | UNM | -40 - 215 °C | E = N * 1 - 40 |

This PID contains the actual temperature measured by a secondary sensor located on the substrate of the transmission control module, which is used to determine when the transmission control module should restart after a hot mode shutdown.

### APSR – PID $2856 – Accelerator Pedal Fully Applied Learned Position Sensor 1

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2856 | Accelerator Pedal Fully Applied Learned Position Sensor 1 | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the learned maximum (fully applied) accelerator pedal position relating to absolute accelerator pedal position (no correction or normalization based upon learned minimum or maximum positions) from accelerator pedal position sensor 1.

### APSR – PID $2857 – Accelerator Pedal Fully Applied Learned Position Sensor 2

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2857 | Accelerator Pedal Fully Applied Learned Position Sensor 2 | 1 | UNM | 0 - 100 % | E = N * 100 / 255 |

This PID contains the learned maximum (fully applied) accelerator pedal position relating to absolute accelerator pedal position (no correction or normalization based upon learned minimum or maximum positions) from accelerator pedal position sensor 2.

### TUDR – PID $2861 – Transmission Tap Up/Down Switch 2 Status

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2861 | Transmission Tap Up/Down Switch 2 Status | 1 | PKT | N/A | N/A |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission Tap Up/Down Switch 2 Invalid | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 6 | 1 |  |  |  |
| Reserved | 0 | 5 | 1 |  |  |  |
| Reserved | 0 | 4 | 1 |  |  |  |
| Reserved | 0 | 3 | 1 |  |  |  |
| Transmission Tap Down Switch 2 Active | 0 | 2 | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Tap Up Switch 2 Active | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 0 | 1 |  |  |  |

This PID contains several signals associated with the status of the transmission driver shift control and transmission electronic range selection systems.  The content of each of these signals is described below.

Transmission Tap Up/Down Switch 2 Invalid indicates that a failure condition associated with the tap up/down switch 2 has been detected.  In electrical architectures that precede PPEI 3.x, this signal is based upon the tap up/down switch hardwire input to the controller.  In electrical architectures from PPEI 3.x or later, this signal is based upon the tap up/down switch information received over serial communication from the platform electronics.

Transmission Tap Down Switch 2 Active indicates that a driver request to decrease the transmission drive range or transmission gear is active as requested using the optional secondary TUTD switch.  This signal is based upon the tap up/down switch information received over serial communication from the platform electronics in Global A and later electrical architectures.

Transmission Tap Up Switch 2 Active indicates that a driver request to increase the transmission drive range or transmission gear is active as requested using the optional secondary TUTD switch.  This signal is based upon the tap up/down switch information received over serial communication from the platform electronics in Global A and later electrical architectures.

### TPCR – PID $2862 – Calculated Line Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $2862 | Calculated Line Pressure | 2 | UNM | 0 – 4095.9375 kPa | E = N / 16 |

This PID contains the expected real line pressure after accounting for pump loss factors.

### EVPR – PID $3A00 – Engine Off Natural Vacuum Phase 1 Result Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $3A00 | Engine Off Natural Vacuum Phase 1 Result Time | 2 | UNM | 0 - 3276.75 s | E = N * 0.05 |

This PID captures the Engine Off Natural Vacuum (EONV) Phase 1 time result.  (Engineering use only)

### EVPR – PID $3A01 – Engine Off Natural Vacuum Phase 2 End Time

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $3A01 | Engine Off Natural Vacuum Phase 2 End Time | 2 | UNM | 0 - 3276.75 s | E = N * 0.05 |

This PID captures the time at the end of Engine Off Natural Vacuum (EONV) Phase 2.  (Engineering use only)

### EVPR – PID $3A02 – Engine Off Natural Vacuum EWMA Test Sample Counter

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $3A02 | Engine Off Natural Vacuum EWMA Test Sample Counter | 2 | UNM | 0 – 65535 counts | E = N * 1 |

This PID keeps track of how many times the 0.020 inch Exponentially Weighted Moving Average (EWMA) parameter has been updated.  (Engineering use only)

### EVPR – PID $3A03 – Accumulated Distance Traveled During Previous Key On Cycles

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $3A03 | Accumulated Distance Traveled During Previous Key On Cycles | 4 | UNM | 0 - 1048575.999755859375 km | E = N / 4096 |

This PID stores the total accumulated distance occurring with the engine running above a minimum vehicle speed saved at shutdown.  (Engineering use only)

### EVPR – PID $3A04 – Engine Off Natural Vacuum Corrected Phase 1 End Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $3A04 | Engine Off Natural Vacuum Corrected Phase 1 End Pressure | 2 | SNM | -32 - 31.9990234375 in. H20 | E = N / 1024 |

This PID indicates the pressure at the end of Phase 1, corrected for fuel volatility.  (Engineering use only)

### EVPR – PID $3A05 – Engine Off Natural Vacuum Phase 1 Result Pressure

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $3A05 | Engine Off Natural Vacuum Phase 1 Result Pressure | 2 | SNM | -32 - 31.9990234375 in. H20 | E = N / 1024 |

This PID indicates the pressure at the end of Phase 1.  (Engineering use only)

### EVPR – PID $3A06 – Engine Off Natural Vacuum Phase 2 Result Vacuum

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $3A06 | Engine Off Natural Vacuum Phase 2 Result Vacuum | 2 | SNM | -32 - 31.9990234375 in. H20 | E = N / 1024 |

This PID indicates the vacuum level at the end of Phase 2.  (Engineering use only)

### EVPR – PID $3A07 – Engine Running EVAP Diagnostics Fault History

| PID | Signal | Length (Bytes) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- |
| $3A07 | Engine Running EVAP Diagnostics Fault History | 1 | ENM | N/A | $00=No Fault; $01=Canister Vent Restriction; $02=Purge Valve Leak; $03=Small Leak; $04=Weak Vacuum; $05=No Test Result |

This PID keeps track of the test result from the previous trip where an Engine Running EVAP Diagnostics (EVPD) test was completed.  (Engineering use only)

## Development PIDs

To be determined in a future version of this document.

# Data Identifiers (DIDs)

DIDs are divided into several ranges that determine their usage and the ownership of their requirements, as shown in Table 34: DID Ranges

.  Definitions for application specific DIDs and corporate standard DIDs are provided in the following sections.  Definitions for controller specific DIDs must be provided in the associated controller specific diagnostic CTS.

| Range | Usage |
| --- | --- |
| $00 | Reserved |
| $01 to $0F | Controller Specific DIDs |
| $10 to $7F | Application Specific DIDs (see Section 4.1) |
| $80 to $FF | Corporate Standard DIDs (see Section 4.2) |

Table 34: DID Ranges

## Application Specific DIDs

### VINR – DID $28 – Odometer Vehicle Identification Number

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $28 | Odometer Vehicle Identification Number | 17 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| VIN character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 4 | 3 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 5 | 4 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 6 | 5 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 7 | 6 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 8 | 7 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 9 | 8 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 10 | 9 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 11 | 10 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 12 | 11 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 13 | 12 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 14 | 13 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 15 | 14 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 16 | 15 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 17 | 16 | 7 | 8 | ASCII | N/A | N/A |

This DID contains the Odometer Vehicle Identification Number (VIN).  The data content shall be specified by the vehicle manufacturer.  If the Odometer VIN has not yet been programmed into the ECU's permanent memory, those locations shall be either “$00” or “$FF”.

The Odometer VIN is to be used as part of the odometer strategy as a “write once” DID to store the Vehicle Identification Number (VIN) in the ECM.  The odometer strategy is detailed in the “Global A Electrical Architecture Vehicle Odometer Sense & Display” functional requirements specification.

When the Manufacturing Enable Counter (MEC) is non-zero, the controller will populate the value in DID $28 so that it will mirror the value stored in DID $90 “Vehicle Identification Number”.  Any read of DID $28 while the MEC is non-zero will show its current contents.

When the MEC is equal to zero and DID $28 has been programmed with a valid VIN (defined as no characters equal to “$00” or “$FF”), the value of DID $28 shall be locked and the “ECU Odometer VIN Programmed” bit in PID $2049 “ECU Odometer Status” shall be set to true.

When the MEC is equal to zero and DID $28 has not been programmed (as in the case of a service controller) an update will be allowed.  During an ignition cycle when DID $28 has not been programmed, the controller will populate the value in DID $28 so that it will mirror the value stored in DID $90 throughout the current ignition cycle.  Any read of DID $28 during the same ignition cycle as the DID $28 update will report back a value of “$FF” in all bytes.  (This behavior is intended to prevent someone from entering a VIN for the purpose of an odometer update and then returning the VIN to an invalid number so as not to lock the DID.)  Once a valid VIN is stored in DID $28 and the ignition key is cycled off, the value of DID $28 shall be locked and the “ECU Odometer VIN Programmed” bit in PID $2049 “ECU Odometer Status” shall be set to true.  Any read of DID $28 on any subsequent ignition cycles will show the actual contents of the DID.

Example:

| Position | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VIN | W | 0 | L | 0 | 0 | 0 | 0 | 3 | 6 | T | 1 | 0 | 0 | 0 | 0 | 8 | 5 |
| Hex value | 57 | 30 | 4C | 30 | 30 | 30 | 30 | 33 | 36 | 54 | 31 | 30 | 30 | 30 | 30 | 38 | 35 |

### MEMR – DID $29 – Primary Calibration Flash History

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $29 | Primary Calibration Flash History | 92 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Number of Entries | 0 | 7 | 8 | UNM | 0 – 255 | E = N * 1 |
| History Buffer Locked | 1 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 1 | 6 | 1 |  |  |  |
| Reserved | 1 | 5 | 1 |  |  |  |
| Reserved | 1 | 4 | 1 |  |  |  |
| Reserved | 1 | 3 | 1 |  |  |  |
| Reserved | 1 | 2 | 1 |  |  |  |
| Reserved | 1 | 1 | 1 |  |  |  |
| Reserved | 1 | 0 | 1 |  |  |  |
| Position 0 - Part Number | 2 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 0 - Calibration Verification Number | 6 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 0 - Programming Count | 10 | 7 | 8 | UNM | 0 – 255 counts | E = N * 1 |
| Position 1 - Part Number | 11 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 1 - Calibration Verification Number | 15 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 1 - Programming Count | 19 | 7 | 8 | UNM | 0 – 255 counts | E = N * 1 |
| Position 2 - Part Number | 20 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 2 - Calibration Verification Number | 24 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 2 - Programming Count | 28 | 7 | 8 | UNM | 0 – 255 counts | E = N * 1 |
| Position 3 - Part Number | 29 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 3 - Calibration Verification Number | 33 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 3 - Programming Count | 37 | 7 | 8 | UNM | 0 – 255 counts | E = N * 1 |
| Position 4 - Part Number | 38 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 4 – Calibration Verification Number | 42 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 4 - Programming Count | 46 | 7 | 8 | UNM | 0 – 255 counts | E = N * 1 |
| Position 5 - Part Number | 47 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 5 - Calibration Verification Number | 51 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 5 - Programming Count | 55 | 7 | 8 | UNM | 0 – 255 counts | E = N * 1 |
| Position 6 - Part Number | 56 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 6 - Calibration Verification Number | 60 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 6 - Programming Count | 64 | 7 | 8 | UNM | 0 – 255 counts | E = N * 1 |
| Position 7 - Part Number | 65 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 7 - Calibration Verification Number | 69 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 7 - Programming Count | 73 | 7 | 8 | UNM | 0 – 255 counts | E = N * 1 |
| Position 8 – Part Number | 74 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 8 - Calibration Verification Number | 78 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 8 - Programming Count | 82 | 7 | 8 | UNM | 0 – 255 counts | E = N * 1 |
| Position 9 - Part Number | 83 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 9 - Calibration Verification Number | 87 | 7 | 32 | UNM | 0 – 4,294,967,295 | E = N * 1 |
| Position 9 - Programming Count | 91 | 7 | 8 | UNM | 0 – 255 counts | E = N * 1 |

This DID contains the history buffer entries for the primary calibration part in the controller.  The first entry tells how many of the 10 possible locations are valid for this particular controller.  The History Buffer Locked signal will be set to true when the buffer has been locked against further updates due to detection of a non-production calibration. The remaining entries in the DID contain the flash history for the primary calibration.

### IMOR – DID $30 – Powertrain Immobilizer Configuration

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $30 | Powertrain Immobilizer Configuration | 4 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Remote Vehicle Start Present | 0 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Power Take Off Remote Start Present | 0 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Free Starts Allowed | 0 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Immobilizer Deactivated | 0 | 4 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 0 | 3 | 1 |  |  |  |
| Reserved | 0 | 2 | 1 |  |  |  |
| Seed and Key Required | 0 | 1 | 1 | BLN | N/A | $1=True; $0=False |
| Autolearn Enabled | 0 | 0 | 1 | BLN | N/A | $1=True; $0=False |
| Autolearn Count Threshold | 1 | 7 | 8 | UNM | 0 - 255 counts | E = N * 1 |
| Autolearn Delay Time Threshold | 2 | 7 | 8 | UNM | 0 - 2550 s | E = N * 10 |
| Tool Learn Delay Time Threshold | 3 | 7 | 8 | UNM | 0 - 2550 s | E = N * 10 |

This DID contains several signals associated with the powertrain immobilizer configuration within the PPEI 3 immobilizer system.  The content of each of these signals is described below.

Remote Vehicle Start Present indicates that remote vehicle start (RVS) functionality within the immobilizer algorithm is enabled.

Power Take Off Remote Start Present indicates that power take off (PTO) remote start functionality within the immobilizer algorithm is enabled.

Free Starts Allowed indicates that free start functionality is enabled (i.e., engine operation will be enabled regardless of the authentication state while the manufacturer’s enable counter (MEC) is not zero).

Immobilizer Deactivated indicates that the immobilizer algorithm is disabled.

Seed and Key Required indicates that the seed and key procedure must be performed prior to inputting the security code to enable learning.

Autolearn Enabled indicates that the ability to enable learning without a service tool is enabled.

Autolearn Count Threshold contains the number of autolearn cycles required to enable learning.

Autolearn Delay Time Threshold contains the time required to complete an autolearn cycle.

Tool Learn Delay Time Threshold contains the time required to wait after the receipt of the correct security code to enable learning.

### IMOR – DID $31 – Powertrain Immobilizer Calibrations

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $31 | Powertrain Immobilizer Calibrations | 7 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Incorrect Pre Release Password Time | 0 | 7 | 8 | UNM | 0 - 25.5 s | E = N * 0.1 |
| Immobilizer Engine Speed Threshold | 1 | 7 | 8 | UNM | 0 - 1020 rpm | E = N * 4 |
| Initial Release Time | 2 | 7 | 8 | UNM | 0 - 25.5 s | E = N * 0.1 |
| Maximum Authentication Time | 3 | 7 | 8 | UNM | 0 - 25.5 s | E = N * 0.1 |
| Post Release Duration | 4 | 7 | 8 | UNM | 0 - 25.5 s | E = N * 0.1 |
| Push Start Engine Speed Threshold | 5 | 7 | 8 | UNM | 0 - 1020 rpm | E = N * 4 |
| Remote Start Response Delay | 6 | 7 | 8 | UNM | 0 - 25.5 s | E = N * 0.1 |

This DID contains several signals associated with the calibration of the PPEI 3 immobilizer system.  The content of each of these signals is described below.

Incorrect Pre Release Password Time contains the maximum time allowed to continue receiving incorrect passwords after initial receipt of an incorrect password prior to disabling crank.

Immobilizer Engine Speed Threshold contains the threshold below which engine speed must lie in order to extend the authentication time during challenge / response authentication.

Initial Release Time contains the maximum authentication time that engine operation is enabled by the pre release password functionality.

Maximum Authentication Time contains the maximum authentication time regardless of the state of engine operation.

Post Release Duration contains the maximum time that post release functionality may be active.

Push Start Engine Speed Threshold contains the minimum engine speed indicating a push start of a manual transmission vehicle.

Remote Start Response Delay contains the maximum time allowed to authenticate remote start.

### RVSR – DID $32 – Remaining Remote Starts

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $32 | Remaining Remote Starts | 1 | UNM | 0 - 255 counts | E = N * 1 | R/W |

This DID contains the number of remote starts remaining before full theft authentication is required.  The data contained in this DID is decremented each time the engine is running due to a remote vehicle start and is reset to a calibration each time theft passes full authentication.

This DID may be used to increase the number of remote starts remaining before full theft authentication is required in order to facilitate fob testing.  However, write access for this DID shall only be available when the manufacturer’s enable counter (MEC) is not zero.

### SPRR – DID $41 – Transmission 1-2 Upshift Pressure Adapts

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $41 | Transmission 1-2 Upshift Pressure Adapts | 17 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission 1-2 Upshift Pressure Adapt Cell 0 | 0 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 1 | 1 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 2 | 2 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 3 | 3 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 4 | 4 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 5 | 5 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 6 | 6 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 7 | 7 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 8 | 8 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 9 | 9 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 10 | 10 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 11 | 11 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 12 | 12 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 13 | 13 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 14 | 14 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 15 | 15 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 1-2 Upshift Pressure Adapt Cell 16 | 16 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |

This DID contains the transmission 1-2 upshift pressure adapts as a function of shift torque.

### SPRR – DID $42 – Transmission 2-3 Upshift Pressure Adapts

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $42 | Transmission 2-3 Upshift Pressure Adapts | 17 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission 2-3 Upshift Pressure Adapt Cell 0 | 0 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 1 | 1 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 2 | 2 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 3 | 3 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 4 | 4 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 5 | 5 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 6 | 6 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 7 | 7 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 8 | 8 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 9 | 9 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 10 | 10 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 11 | 11 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 12 | 12 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 13 | 13 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 14 | 14 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 15 | 15 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 2-3 Upshift Pressure Adapt Cell 16 | 16 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |

This DID contains the transmission 2-3 upshift pressure adapts as a function of shift torque.

### SPRR – DID $43 – Transmission 3-4 Upshift Pressure Adapts

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $43 | Transmission 3-4 Upshift Pressure Adapts | 17 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission 3-4 Upshift Pressure Adapt Cell 0 | 0 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 1 | 1 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 2 | 2 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 3 | 3 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 4 | 4 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 5 | 5 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 6 | 6 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 7 | 7 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 8 | 8 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 9 | 9 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 10 | 10 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 11 | 11 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 12 | 12 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 13 | 13 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 14 | 14 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 15 | 15 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 3-4 Upshift Pressure Adapt Cell 16 | 16 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |

This DID contains the transmission 3-4 upshift pressure adapts as a function of shift torque.

### SPRR – DID $44 – Transmission 4-5 Upshift Pressure Adapts

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $44 | Transmission 4-5 Upshift Pressure Adapts | 17 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission 4-5 Upshift Pressure Adapt Cell 0 | 0 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 1 | 1 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 2 | 2 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 3 | 3 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 4 | 4 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 5 | 5 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 6 | 6 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 7 | 7 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 8 | 8 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 9 | 9 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 10 | 10 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 11 | 11 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 12 | 12 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 13 | 13 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 14 | 14 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 15 | 15 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |
| Transmission 4-5 Upshift Pressure Adapt Cell 16 | 16 | 7 | 8 | SNM | -256 - 254 kPa | E = N * 2 |

This DID contains the transmission 4-5 upshift pressure adapts as a function of shift torque.

### TPCR – DID $48 – Line Pressure Learned Characteristics

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $48 | Line Pressure Learned Characteristics | 12 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Minimum Line Pressure | 0 | 7 | 16 | UNM | 0 - 4095.9375 kPa | E = N * 0.0625 |
| Engine Speed to Achieve First Maximum Line Pressure Point | 2 | 7 | 16 | UNM | 0 - 8191.875 rpm | E = N * 0.125 |
| Minimum Line Pressure Knee Engine Speed | 4 | 7 | 16 | UNM | 0 - 8191.875 rpm | E = N * 0.125 |
| Maximum Line Pressure | 6 | 7 | 16 | UNM | 0 - 4095.9375 kPa | E = N * 0.0625 |
| Engine Speed to Achieve Last Minimum Line Pressure Point | 8 | 7 | 16 | UNM | 0 - 8191.875 rpm | E = N * 0.125 |
| Maximum Line Pressure Knee Engine Speed | 10 | 7 | 16 | UNM | 0 - 8191.875 rpm | E = N * 0.125 |

This DID is used to set various line pressure characteristics that are learned during C2C final test.

### TQTR – DID $49 – Torque Converter Learned Characteristics

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $49 | Torque Converter Learned Characteristics | 5 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission Torque Converter K Factor – Low Torque | 0 | 7 | 16 | UNM | 0 - 1023.984375 | E = N * 0.015625 |
| Transmission Torque Converter K Factor – High Torque | 2 | 7 | 16 | UNM | 0 - 1023.984375 | E = N * 0.015625 |
| Transmission EOL Program Complete | 4 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 4 | 6 | 1 |  |  |  |
| Reserved | 4 | 5 | 1 |  |  |  |
| Reserved | 4 | 4 | 1 |  |  |  |
| Reserved | 4 | 3 | 1 |  |  |  |
| Reserved | 4 | 2 | 1 |  |  |  |
| Reserved | 4 | 1 | 1 |  |  |  |
| Reserved | 4 | 0 | 1 |  |  |  |

This DID is used to set various Transmission characteristics that are learned during C2C final test and in plant testing.

Transmission Torque Converter K-Factor represents the K-Factor of the transmission torque converter as measured at the EOL transmission assembly for both low torque low engine speed, and high torque high engine speed.

Transmission EOL Program Complete Flag indicates the EOL programming is complete.

### RSCR – DID $4A – Clutch 1 Pressure Learned Characteristics

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $4A | Clutch 1 Pressure Learned Characteristics | 15 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Final Test Clutch Pressure Gain | 0 | 7 | 16 | UNM | 0 - 3.999938964844 kPa/Nm | E = N / 16384 |
| Final Test Clutch Pressure Offset | 2 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Final Test Clutch Fill Time | 4 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |
| Final Test Clutch Full Feed Fill Pressure | 6 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Fill Time | 8 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |
| Service Fast Learn Clutch Full Feed Fill Pressure | 10 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Pressure Offset | 12 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Fill Complete | 14 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Service Fast Learn Clutch Full Feed Fill Complete | 14 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Service Fast Learn Clutch Pressure Offset Calculation Complete | 14 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 14 | 4 | 1 |  |  |  |
| Reserved | 14 | 3 | 1 |  |  |  |
| Reserved | 14 | 2 | 1 |  |  |  |
| Reserved | 14 | 1 | 1 |  |  |  |
| Reserved | 14 | 0 | 1 |  |  |  |

This DID is used to set various clutch characteristics (for clutch 1) that are learned during C2C final test and the Service Fast Learn (SFL) procedure.  The write length for DID $4A should not include the SFL data and therefore be only 8 bytes in length.  For reading, all 15 bytes should be sent by the controller.

### RSCR – DID $4B – Clutch 2 Pressure Learned Characteristics

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $4B | Clutch 2 Pressure Learned Characteristics | 15 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Final Test Clutch Pressure Gain | 0 | 7 | 16 | UNM | 0 - 3.999938964844 kPa/Nm | E = N / 16384 |
| Final Test Clutch Pressure Offset | 2 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Final Test Clutch Fill Time | 4 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |
| Final Test Clutch Full Feed Fill Pressure | 6 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Fill Time | 8 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |
| Service Fast Learn Clutch Full Feed Fill Pressure | 10 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Pressure Offset | 12 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Fill Complete | 14 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Service Fast Learn Clutch Full Feed Fill Complete | 14 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Service Fast Learn Clutch Pressure Offset Calculation Complete | 14 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 14 | 4 | 1 |  |  |  |
| Reserved | 14 | 3 | 1 |  |  |  |
| Reserved | 14 | 2 | 1 |  |  |  |
| Reserved | 14 | 1 | 1 |  |  |  |
| Reserved | 14 | 0 | 1 |  |  |  |

This DID is used to set various clutch characteristics (for clutch 2) that are learned during C2C final test and the Service Fast Learn (SFL) procedure.  The write length for DID $4B should not include the SFL data and therefore be only 8 bytes in length.  For reading, all 15 bytes should be sent by the controller.

### RSCR – DID $4C – Clutch 3 Pressure Learned Characteristics

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $4C | Clutch 3 Pressure Learned Characteristics | 15 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Final Test Clutch Pressure Gain | 0 | 7 | 16 | UNM | 0 - 3.999938964844 kPa/Nm | E = N / 16384 |
| Final Test Clutch Pressure Offset | 2 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Final Test Clutch Fill Time | 4 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |
| Final Test Clutch Full Feed Fill Pressure | 6 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Fill Time | 8 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |
| Service Fast Learn Clutch Full Feed Fill Pressure | 10 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Pressure Offset | 12 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Fill Complete | 14 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Service Fast Learn Clutch Full Feed Fill Complete | 14 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Service Fast Learn Clutch Pressure Offset Calculation Complete | 14 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 14 | 4 | 1 |  |  |  |
| Reserved | 14 | 3 | 1 |  |  |  |
| Reserved | 14 | 2 | 1 |  |  |  |
| Reserved | 14 | 1 | 1 |  |  |  |
| Reserved | 14 | 0 | 1 |  |  |  |

This DID is used to set various clutch characteristics (for clutch 3) that are learned during C2C final test and the Service Fast Learn (SFL) procedure.  The write length for DID $4C should not include the SFL data and therefore be only 8 bytes in length.  For reading, all 15 bytes should be sent by the controller.

### RSCR – DID $4D – Clutch 4 Pressure Learned Characteristics

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $4D | Clutch 4 Pressure Learned Characteristics | 15 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Final Test Clutch Pressure Gain | 0 | 7 | 16 | UNM | 0 - 3.999938964844 kPa/Nm | E = N / 16384 |
| Final Test Clutch Pressure Offset | 2 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Final Test Clutch Fill Time | 4 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |
| Final Test Clutch Full Feed Fill Pressure | 6 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Fill Time | 8 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |
| Service Fast Learn Clutch Full Feed Fill Pressure | 10 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Pressure Offset | 12 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Fill Complete | 14 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Service Fast Learn Clutch Full Feed Fill Complete | 14 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Service Fast Learn Clutch Pressure Offset Calculation Complete | 14 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 14 | 4 | 1 |  |  |  |
| Reserved | 14 | 3 | 1 |  |  |  |
| Reserved | 14 | 2 | 1 |  |  |  |
| Reserved | 14 | 1 | 1 |  |  |  |
| Reserved | 14 | 0 | 1 |  |  |  |

This DID is used to set various clutch characteristics (for clutch 4) that are learned during C2C final test and the Service Fast Learn (SFL) procedure.  The write length for DID $4D should not include the SFL data and therefore be only 8 bytes in length.  For reading, all 15 bytes should be sent by the controller.

### RSCR – DID $4E – Clutch 5 Pressure Learned Characteristics

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $4E | Clutch 5 Pressure Learned Characteristics | 15 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Final Test Clutch Pressure Gain | 0 | 7 | 16 | UNM | 0 - 3.999938964844 kPa/Nm | E = N / 16384 |
| Final Test Clutch Pressure Offset | 2 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Final Test Clutch Fill Time | 4 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |
| Final Test Clutch Full Feed Fill Pressure | 6 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Fill Time | 8 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |
| Service Fast Learn Clutch Full Feed Fill Pressure | 10 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Pressure Offset | 12 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Service Fast Learn Clutch Fill Complete | 14 | 7 | 1 | BLN | N/A | $1=True; $0=False |
| Service Fast Learn Clutch Full Feed Fill Complete | 14 | 6 | 1 | BLN | N/A | $1=True; $0=False |
| Service Fast Learn Clutch Pressure Offset Calculation Complete | 14 | 5 | 1 | BLN | N/A | $1=True; $0=False |
| Reserved | 14 | 4 | 1 |  |  |  |
| Reserved | 14 | 3 | 1 |  |  |  |
| Reserved | 14 | 2 | 1 |  |  |  |
| Reserved | 14 | 1 | 1 |  |  |  |
| Reserved | 14 | 0 | 1 |  |  |  |

This DID is used to set various clutch characteristics (for clutch 5) that are learned during C2C final test and the Service Fast Learn (SFL) procedure.  The write length for DID $4E should not include the SFL data and therefore be only 8 bytes in length.  For reading, all 15 bytes should be sent by the controller.

### SSPR – DID $50 – Transmission Steady State Pressure Adapts

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $50 | Transmission Steady State Pressure Adapts | 13 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reverse Steady State Pressure Adapt | 0 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| First Gear TCC Off Steady State Pressure Adapt | 1 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| First Gear TCC On Steady State Pressure Adapt | 2 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Second Gear TCC Off Steady State Pressure Adapt | 3 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Second Gear TCC On Steady State Pressure Adapt | 4 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Third Gear TCC Off Steady State Pressure Adapt | 5 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Third Gear TCC On Steady State Pressure Adapt | 6 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Fourth Gear TCC Off Steady State Pressure Adapt | 7 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Fourth Gear TCC On Steady State Pressure Adapt | 8 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Fifth Gear TCC Off Steady State Pressure Adapt | 9 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Fifth Gear TCC On Steady State Pressure Adapt | 10 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Sixth Gear TCC Off Steady State Pressure Adapt | 11 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Sixth Gear TCC On Steady State Pressure Adapt | 12 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |

This DID contains the transmission steady state pressure adapts.

### SSPR – DID $51 – Transmission Steady State Pressure Braking Adapts

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $51 | Transmission Steady State Pressure Braking Adapts | 13 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reverse Steady State Pressure Braking Adapt | 0 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| First Gear TCC Off Steady State Pressure Braking Adapt | 1 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| First Gear TCC On Steady State Pressure Braking Adapt | 2 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Second Gear TCC Off Steady State Pressure Braking Adapt | 3 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Second Gear TCC On Steady State Pressure Braking Adapt | 4 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Third Gear TCC Off Steady State Pressure Braking Adapt | 5 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Third Gear TCC On Steady State Pressure Braking Adapt | 6 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Fourth Gear TCC Off Steady State Pressure Braking Adapt | 7 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Fourth Gear TCC On Steady State Pressure Braking Adapt | 8 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Fifth Gear TCC Off Steady State Pressure Braking Adapt | 9 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Fifth Gear TCC On Steady State Pressure Braking Adapt | 10 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Sixth Gear TCC Off Steady State Pressure Braking Adapt | 11 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |
| Sixth Gear TCC On Steady State Pressure Braking Adapt | 12 | 7 | 8 | UNM | 0 - 510 kPa | E = N * 2 |

This DID contains the transmission steady state pressure braking adapts.

### RSCR – DID $52 – Transmission 1-2 Upshift Adaptive Shift Data

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $52 | Transmission 1-2 Upshift Adaptive Shift Data | 26 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission 1-2 Upshift Pattern 0 Oncoming Clutch Volume | 0 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission 1-2 Upshift Pattern 0 Oncoming Pressure Region 1 | 1 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 0 Oncoming Pressure Region 2 | 3 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 0 Oncoming Pressure Region 3 | 5 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 0 Offgoing Pressure Region 1 | 7 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 0 Offgoing Pressure Region 2 | 9 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 0 Offgoing Pressure Region 3 | 11 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 1 Oncoming Clutch Volume | 13 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission 1-2 Upshift Pattern 1 Oncoming Pressure Region 1 | 14 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 1 Oncoming Pressure Region 2 | 16 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 1 Oncoming Pressure Region 3 | 18 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 1 Offgoing Pressure Region 1 | 20 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 1 Offgoing Pressure Region 2 | 22 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 1-2 Upshift Pattern 1 Offgoing Pressure Region 3 | 24 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |

This DID contains shift pattern 0 and shift pattern 1 transmission 1-2 upshift oncoming and offgoing pressure and clutch volume shift data used for adaptive purposes.  The clutch pressure data is indicated for torque regions 1, 2, and 3.

### RSCR – DID $53 – Transmission 2-3 Upshift Adaptive Shift Data

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $53 | Transmission 2-3 Upshift Adaptive Shift Data | 26 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission 2-3 Upshift Pattern 0 Oncoming Clutch Volume | 0 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission 2-3 Upshift Pattern 0 Oncoming Pressure Region 1 | 1 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 0 Oncoming Pressure Region 2 | 3 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 0 Oncoming Pressure Region 3 | 5 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 0 Offgoing Pressure Region 1 | 7 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 0 Offgoing Pressure Region 2 | 9 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 0 Offgoing Pressure Region 3 | 11 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 1 Oncoming Clutch Volume | 13 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission 2-3 Upshift Pattern 1 Oncoming Pressure Region 1 | 14 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 1 Oncoming Pressure Region 2 | 16 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 1 Oncoming Pressure Region 3 | 18 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 1 Offgoing Pressure Region 1 | 20 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 1 Offgoing Pressure Region 2 | 22 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 2-3 Upshift Pattern 1 Offgoing Pressure Region 3 | 24 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |

This DID contains shift pattern 0 and shift pattern 1 transmission 2-3 upshift oncoming and offgoing pressure and clutch volume shift data used for adaptive purposes.  The clutch pressure data is indicated for torque regions 1, 2, and 3.

### RSCR – DID $54 – Transmission 3-4 Upshift Adaptive Shift Data

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $54 | Transmission 3-4 Upshift Adaptive Shift Data | 26 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission 3-4 Upshift Pattern 0 Oncoming Clutch Volume | 0 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission 3-4 Upshift Pattern 0 Oncoming Pressure Region 1 | 1 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 0 Oncoming Pressure Region 2 | 3 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 0 Oncoming Pressure Region 3 | 5 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 0 Offgoing Pressure Region 1 | 7 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 0 Offgoing Pressure Region 2 | 9 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 0 Offgoing Pressure Region 3 | 11 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 1 Oncoming Clutch Volume | 13 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission 3-4 Upshift Pattern 1 Oncoming Pressure Region 1 | 14 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 1 Oncoming Pressure Region 2 | 16 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 1 Oncoming Pressure Region 3 | 18 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 1 Offgoing Pressure Region 1 | 20 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 1 Offgoing Pressure Region 2 | 22 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 3-4 Upshift Pattern 1 Offgoing Pressure Region 3 | 24 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |

This DID contains shift pattern 0 and shift pattern 1 transmission 3-4 upshift oncoming and offgoing pressure and clutch volume shift data used for adaptive purposes.  The clutch pressure data is indicated for torque regions 1, 2, and 3.

### RSCR – DID $55 – Transmission 4-5 Upshift Adaptive Shift Data

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $55 | Transmission 4-5 Upshift Adaptive Shift Data | 26 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission 4-5 Upshift Pattern 0 Oncoming Clutch Volume | 0 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission 4-5 Upshift Pattern 0 Oncoming Pressure Region 1 | 1 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 0 Oncoming Pressure Region 2 | 3 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 0 Oncoming Pressure Region 3 | 5 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 0 Offgoing Pressure Region 1 | 7 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 0 Offgoing Pressure Region 2 | 9 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 0 Offgoing Pressure Region 3 | 11 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 1 Oncoming Clutch Volume | 13 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission 4-5 Upshift Pattern 1 Oncoming Pressure Region 1 | 14 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 1 Oncoming Pressure Region 2 | 16 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 1 Oncoming Pressure Region 3 | 18 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 1 Offgoing Pressure Region 1 | 20 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 1 Offgoing Pressure Region 2 | 22 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 4-5 Upshift Pattern 1 Offgoing Pressure Region 3 | 24 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |

This DID contains shift pattern 0 and shift pattern 1 transmission 4-5 upshift oncoming and offgoing pressure and clutch volume shift data used for adaptive purposes.  The clutch pressure data is indicated for torque regions 1, 2, and 3.

### RSCR – DID $56 – Transmission 5-6 Upshift Adaptive Shift Data

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $56 | Transmission 5-6 Upshift Adaptive Shift Data | 26 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission 5-6 Upshift Pattern 0 Oncoming Clutch Volume | 0 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission 5-6 Upshift Pattern 0 Oncoming Pressure Region 1 | 1 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 0 Oncoming Pressure Region 2 | 3 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 0 Oncoming Pressure Region 3 | 5 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 0 Offgoing Pressure Region 1 | 7 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 0 Offgoing Pressure Region 2 | 9 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 0 Offgoing Pressure Region 3 | 11 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 1 Oncoming Clutch Volume | 13 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission 5-6 Upshift Pattern 1 Oncoming Pressure Region 1 | 14 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 1 Oncoming Pressure Region 2 | 16 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 1 Oncoming Pressure Region 3 | 18 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 1 Offgoing Pressure Region 1 | 20 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 1 Offgoing Pressure Region 2 | 22 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission 5-6 Upshift Pattern 1 Offgoing Pressure Region 3 | 24 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |

This DID contains shift pattern 0 and shift pattern 1 transmission 5-6 upshift oncoming and offgoing pressure and clutch volume shift data used for adaptive purposes.  The clutch pressure data is indicated for torque regions 1, 2, and 3.

### RSCR – DID $57 – Transmission D-R Shift Adaptive Shift Data

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $57 | Transmission D-R Shift Adaptive Shift Data | 6 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission D-R Oncoming Clutch Volume | 0 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission D-R Minimum Oncoming Clutch Volume | 1 | 7 | 8 | UNM | 0 - 255 mL | E = N |
| Transmission D-R Oncoming Clutch Pressure | 2 | 7 | 16 | SNM | -4096 - 4095.875 kPa | E = N * 0.125 |
| Transmission D-R Oncoming Fill Delay | 4 | 7 | 16 | SNM | -32 - 31.9990234375 s | E = N / 1024 |

This DID indicates the clutch volume, minimum clutch volume, oncoming clutch pressure, and oncoming fill delay for the oncoming clutch during drive to reverse gear transition events.

### TFQR – DID $5F – Transmission Oil Remaining Life

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $5F | Transmission Oil Remaining Life | 1 | UNM | 0 - 99.609375 % | E = N * 100/256 | R/W |

This DID contains the transmission oil remaining life determined from an algorithm based upon the transmission oil / fluid temperature.  This DID may be used to reset the transmission oil remaining life.

### MSFR – DID $6B – Reference Period Compensation Factors

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $6B | Reference Period Compensation Factors | 8 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Reference Period Compensation Factor A | 0 | 7 | 16 | UNM | 0 - 1.999969482422 | E = N * 1 / 32768 |
| Reference Period Compensation Factor B | 2 | 7 | 16 | UNM | 0 - 1.999969482422 | E = N * 1 / 32768 |
| Reference Period Compensation Factor C | 4 | 7 | 16 | UNM | 0 - 1.999969482422 | E = N * 1 / 32768 |
| Reference Period Compensation Factor D | 6 | 7 | 16 | UNM | 0 - 1.999969482422 | E = N * 1 / 32768 |

This DID contains the crank angle sensing error (CASE) learn reference period compensation factor for each cylinder pair.

### EODR – DID $6D – Engine Oil Remaining Life

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $6D | Engine Oil Remaining Life | 1 | UNM | 0 - 99.609375 % | E = N * 100/256 | R/W |

This DID contains the engine oil remaining life determined from an algorithm based upon the effective engine revolutions and the distance driven since the last oil change.  This DID may be used to reset the engine oil remaining life.

### MEMR – DID $75 – First Time Power Up Flag

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $75 | First Time Power Up Flag | 2 | UNM | 0 - 65535 | E = N * 1 | R/W |

This DID allows the reset of the single NVM Static variable that serves as the 'first time initialization' flag.  During NVM initialization, before anything else takes place, this 'first init' flag will be checked.  If the state of the flag indicates that this is the first initialization of the controller then invoke TSKR functions and then set the flag to NOT first initialization state.  The states of this flag are encoded to avoid selecting values that might occur naturally in memory ( ie, FF, 00, AA, 55).  The initial value of this flag is specified within the EBOM.

## Corporate Standard DIDs

The corporate standard DIDs below are also specified within [1].  In the event of a conflict between this document and [1], the later shall take precedence.

### VINR – DID $90 – Vehicle Identification Number

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $90 | VehicleIdentificationNumber (VIN) | 17 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| VIN character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 4 | 3 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 5 | 4 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 6 | 5 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 7 | 6 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 8 | 7 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 9 | 8 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 10 | 9 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 11 | 10 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 12 | 11 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 13 | 12 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 14 | 13 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 15 | 14 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 16 | 15 | 7 | 8 | ASCII | N/A | N/A |
| VIN character position 17 | 16 | 7 | 8 | ASCII | N/A | N/A |

This DID contains the Vehicle Identification Number.  The data content shall be specified by the vehicle manufacturer.  If the VIN has not yet been programmed into the ECU's permanent memory, those locations shall be either '$00' or '$FF'.

* When the Manufacturing Enable Counter (MEC) is equal to zero, a DID $90 write may only be performed under two circumstances:

1) When preceded by a ClearDiagnosticInformation ($04) Service during the same ignition cycle

- or -

2) When the controller has been programmed within the last three ignition cycles

There are no DID $90 write restrictions when the MEC is non-zero.

Example:

| Position | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VIN | W | 0 | L | 0 | 0 | 0 | 0 | 3 | 6 | T | 1 | 0 | 0 | 0 | 0 | 8 | 5 |
| Hex value | 57 | 30 | 4C | 30 | 30 | 30 | 30 | 33 | 36 | 54 | 31 | 30 | 30 | 30 | 30 | 38 | 35 |

### MEMR – DID $97 – System Name or Engine Type

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $97 | SystemNameOrEngineType (SNOET) | <20 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| System Name or Engine Type character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 4 | 3 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 5 | 4 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 6 | 5 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 7 | 6 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 8 | 7 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 9 | 8 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 10 | 9 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 11 | 10 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 12 | 11 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 13 | 12 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 14 | 13 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 15 | 14 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 16 | 15 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 17 | 16 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 18 | 17 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 19 | 18 | 7 | 8 | ASCII | N/A | N/A |
| System Name or Engine Type character position 20 | 19 | 7 | 8 | ASCII | N/A | N/A |

This DID contains the SystemNameOrEngineType which identifies the electronic system name (e.g. Automatic Transmission with 2 Ltr engine: GS820 X20XEV) or engine type (X30XE) installed.  The data shall always be of type ASCII.  The length is variable (depends on length of string) but shall not exceed 20 characters.

Example:

| Position | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNOET | G | S | 8 | 2 | 0 |  | X | 2 | 0 | X | E | V | … |
| Hex Value | 47 | 53 | 38 | 32 | 30 | 20 | 58 | 32 | 30 | 58 | 45 | 56 | … |

### MEMR – DID $98 – Repair Shop Code or Tester Serial Number

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $98 | RepairShopCodeOrTesterSerialNumber (RSCOTSN) | 10 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Repair Shop or Tester Serial # character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| Repair Shop or Tester Serial # character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| Repair Shop or Tester Serial # character position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |
| Repair Shop or Tester Serial # character position 4 | 3 | 7 | 8 | ASCII | N/A | N/A |
| Repair Shop or Tester Serial # character position 5 | 4 | 7 | 8 | ASCII | N/A | N/A |
| Repair Shop or Tester Serial # character position 6 | 5 | 7 | 8 | ASCII | N/A | N/A |
| Repair Shop or Tester Serial # character position 7 | 6 | 7 | 8 | ASCII | N/A | N/A |
| Repair Shop or Tester Serial # character position 8 | 7 | 7 | 8 | ASCII | N/A | N/A |
| Repair Shop or Tester Serial # character position 9 | 8 | 7 | 8 | ASCII | N/A | N/A |
| Repair Shop or Tester Serial # character position 10 | 9 | 7 | 8 | ASCII | N/A | N/A |

This DID contains the RepairShopCodeOrTesterSerialNumber which identifies the dealer’s code or the tester’s serial number. This ASCII string is programmed into the ECU’s memory during the last programming session (SPS) at the dealer site.

Example:

| Position | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | --- |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSCOSN | S | N | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | --- |
| Hex Value | 58 | 31 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | --- |

### MEMR – DID $99 – Programming Date

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $99 | ProgrammingDate (PD) | 4 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Programming Date position 1 | 0 | 7 | 8 | BCD | 0 - 99 | N/A |
| Programming Date position 2 | 1 | 7 | 8 | BCD | 0 - 99 | N/A |
| Programming Date position 3 | 2 | 7 | 8 | BCD | 0 - 99 | N/A |
| Programming Date position 4 | 3 | 7 | 8 | BCD | 0 - 99 | N/A |

This DID contains the ProgrammingDate of the last programming event (SPS) at the dealer site.  This BCD value was programmed into the ECU’s memory during the last programming session (SPS).

Example:

| Position | 1 | 2 | 3 | 4 | Description |
| --- | --- | --- | --- | --- | --- |
| PD (BCD) | 20 | 00 | 01 | 22 | 2000 = Year; 01 = Month; 22 = Day |
| Hex Value | 20 | 00 | 01 | 22 |  |

### MEMR – DID $9A – Diagnostic Data Identifier

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $9A | DiagnosticDataIdentifier (DDI) | 2 | PKT | N/A | N/A | R/W* |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Diagnostic Data ID Digit 1 | 0 | 7 | 8 | UNM | 0 - $FF | E = N |
| Diagnostic Data ID Digit 2 | 1 | 7 | 8 | UNM | 0 - $FF | E = N |

This DID contains  the DiagnosticDataIdentifier which identifies a supplier and system specific diagnostic data stream and shall be used by diagnostic testers to interpret a diagnostic data stream. The data format of the DDI is defined as two (2) bytes  where the most significant byte of the DDI identifies the system by a system code and  the least significant byte identifies the system data stream version number.  For an SPS_TYPE_B or SPS_TYPE_C ECU, the value of the least significant byte of the DDI shall always be $00 (provided that the DID is supported in a node which is not completely programmed).

Example:

| Position | 1 | 2 | Description |
| --- | --- | --- | --- |
| DDI | 02 | 03 | System Code e.g. $02 = Simtec MS56.5 |
| Hex Value | 02 | 03 | Diagnostic Data Stream Version Code e.g. $03 = version 03 |

### OSAR – DID $A0 – Manufacturers Enable Counter

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $A0 | manufacturers_enable_counter (MEC) | 1 | PKT | N/A | N/A | R/W1 |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Manufacturers Enable Counter | 0 | 7 | 8 | UNM | 0 - $FF | E = N |

This DID contains  the MEC which is used when determining the current status of ECU security.

W1 = A node shall not allow the value of the MEC to change once it becomes $00, unless SecurityAccess ($27) is successfully initiated.  The ability to allow writing a new value to the MEC for any specific ECU which supports the MEC shall be negotiated by representatives from Service and Assembly Verification, and the responsible DRE, and documented in the CTS..

Example:

| Position | 1 | Description |
| --- | --- | --- |
| MEC | 254 | $FE - $01: ECU’s security system not armed |
| Hex value | FE | $00: ECU’s security system armed |

### CANR – DID $B0 – ECU Diagnostic Address

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $B0 | ECUDiagnosticAddress (ECUADDR) | 1 | PKT | N/A | N/A | R/W3 |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| ECUDiagnosticAddress | 0 | 7 | 8 | UNM | 0 - $FF | E = N |

This DID contains the diagnostic address of an ECU.

W3 = This DID is usually read only and supported in boot software.  An ECU may choose to allow this DID to be written when more than 1 of a given hardware device can exist on a single vehicle.

Examples:

| Position | 1 | Description |
| --- | --- | --- |
| ECUADDR | 17 | ECM1 ECUDiagnosticAddress |
| Hex value | 11 |  |
| ECUADDR | 18 | ECM2 ECUDiagnosticAddress |
| Hex value | 12 |  |
| ECUADDR | 24 | TCM1 ECUDiagnosticAddress |
| Hex value | 18 |  |
| ECUADDR | 26 | TCCM ECUDiagnosticAddress |
| Hex value | 1A |  |

### MEMR – DID $B2 – GM Manufacturing Data

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $B2 | GM ManufacturingData (GMMD) | 5 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| GM Manufacturing Data Byte 1 | 0 | 7 | 8 | UNM | 0 - $FF | E = N |
| GM Manufacturing Data Byte 2 | 1 | 7 | 8 | UNM | 0 - $FF | E = N |
| GM Manufacturing Data Byte 3 | 2 | 7 | 8 | UNM | 0 - $FF | E = N |
| GM Manufacturing Data Byte 4 | 3 | 7 | 8 | UNM | 0 - $FF | E = N |
| GM Manufacturing Data Byte 5 | 4 | 7 | 8 | UNM | 0 - $FF | E = N |

This DID contains 5 bytes of ManufacturingData which is both written and read by tooling in the vehicle assembly plant during the production process.  This area in product memory is used as a scratch pad for assembly tooling to pass test results back and forth from the various testers used in the vehicle assembly process.  The ECU memory used for this DID shall be permanent  memory (e.g. flash or EEPROM).

Example:

| Position | 1 | 2 | 3 | 4 | 5 | --- |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | --- |
|  |  |  |  |  |  | --- |

### MEMR – DID $B4 – Manufacturing Traceability Characters

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $B4 | ManufacturingTraceabilityCharacters (MTC) | 16 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Component ID character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| Component ID character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| Part Number/Broadcast Code character position 1 | 2 | 7 | 8 | ASCII | N/A | N/A |
| Part Number/Broadcast Code character position 2 | 3 | 7 | 8 | ASCII | N/A | N/A |
| Part Number/Broadcast Code character position 3 | 4 | 7 | 8 | ASCII | N/A | N/A |
| Part Number/Broadcast Code character position 4 | 5 | 7 | 8 | ASCII | N/A | N/A |
| Supplier Code character position | 6 | 7 | 8 | ASCII | N/A | N/A |
| Traceability Number character position 1 | 7 | 7 | 8 | ASCII | N/A | N/A |
| Traceability Number character position 2 | 8 | 7 | 8 | ASCII | N/A | N/A |
| Traceability Number character position 3 | 9 | 7 | 8 | ASCII | N/A | N/A |
| Traceability Number character position 4 | 10 | 7 | 8 | ASCII | N/A | N/A |
| Traceability Number character position 5 | 11 | 7 | 8 | ASCII | N/A | N/A |
| Traceability Number character position 6 | 12 | 7 | 8 | ASCII | N/A | N/A |
| Traceability Number character position 7 | 13 | 7 | 8 | ASCII | N/A | N/A |
| Traceability Number character position 8 | 14 | 7 | 8 | ASCII | N/A | N/A |
| Traceability Number character position 9 | 15 | 7 | 8 | ASCII | N/A | N/A |

This DID contains the data necessary to meet the component traceability requirements as specified in GMW 4710GS and GM1737. The length of this DID is variable but shall not exceed 16 characters.  The data is stored in ASCII format and is broken up into 4 fields.  A description of each field follows:

Component Identifier (Comp Id)

The component Identifier is a 2 byte field (starting at byte 1 of the DID) that is used to indicate the component type.  A master list of Component Identifiers can be found in Appendix 1 of GM1737.

Part Number / Broadcast Code (PN / BC)

This is a 4 byte field (starting at byte 3 of the DID) and contains either the ECU Broadcast Code or the last 4 digits of the ECU part number.  GM1737 specifies this requirement.

Supplier Code (sup)

This is a one byte field occupying the 7th byte of the DID that identifies the supplier and manufacturing location of the ECU. . The Master List of Supplier Codes is located in Appendix 2 of the GM1737 document.

Traceability Number

This field is used to uniquely identify a specific ECU.  The field starts at byte 8 of the DID and is 9 bytes in length.  The content is assigned by the module manufacturer and must consist of enough information to uniquely identify an ECU.  Examples of the type of data included in this field are Julian Date of production, shift build data, production assembly line information and ECU serial number.  If less than 9 characters are required to uniquely identify an ECU, then the trailing characters shall be filled with spaces (0x20).

Example:

| Position | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | --- |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MTC | A | S | 7 | 0 | 8 | 1 | K | 1 | 5 | 0 | 6 | 1 | 0 | 0 | 0 | F |  |
| Hex value | 41 | 53 | 37 | 30 | 38 | 31 | 4B | 31 | 35 | 30 | 36 | 31 | 30 | 30 | 30 | 46 | --- |
| Field Desc. | Comp Id |  | PN / BC |  |  |  | sup | Traceability Number |  |  |  |  |  |  |  |  | --- |

### MEMR – DID $B5 – GM Broadcast Code

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $B5 | GM Broadcast Code | 4 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| GM Broadcast Code Position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| GM Broadcast Code Position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| GM Broadcast Code Position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |
| GM Broadcast Code Position 4 | 3 | 7 | 8 | ASCII | N/A | N/A |

This DID contains 4 alpha characters which represent the ECU broadcast code.  The broadcast code is unique for each combination of hardware, software, and calibrations for an ECU.

Example:

| Position | 1 | 2 | 3 | 4 | --- |
| --- | --- | --- | --- | --- | --- |
| Broadcast Code | A | B | C | D | --- |
| Hex value | 41 | 42 | 43 | 44 | --- |

### MEMR – DID $B6 – GM Target Vehicle

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $B6 | GM Target Vehicle (GMTV) | 5 | PKT | N/A | N/A | R/W4 |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Vehicle Model Year | 0 | 7 | 8 | UNM | See description below | See description below |
| Vehicle Make | 1 | 7 | 8 | ASCII | N/A | N/A |
| Vehicle Car Line | 2 | 7 | 8 | ASCII | N/A | N/A |
| Engine RPO Code character position 1 | 3 | 7 | 8 | ASCII | N/A | N/A |
| Engine RPO Code character position 2 | 4 | 7 | 8 | ASCII | N/A | N/A |

This DID contains 5 bytes of data used to identify the vehicle that the ECU is targeted for.  The data is broken down as follows:

Byte 1 (Model Year)

This byte is an USN value that represents the target model year using the year 1900 as the base year.  To represent the 2000 model year, the value of byte 1 would be 100 or ($64).  This is derived from 1900 + 100 = 2000.  If the module supports multiple model years, then this byte is filled in with $23 (ASCII equivalent to ‘#’).

Byte 2 (Vehicle Make)

This byte is the ASCII representation of the Vehicle Make as Identified in the Vehicle Identification Number (VIN). For example, Cadillac = 6 or $36.  If the part is used in multiple vehicle makes, then a value of $23 (ASCII ‘#’) is used.

Byte 3 (Vehicle Car Line)

This byte is an ASCII character representing the vehicle car line (e.g. “J” for Chevy Cavalier).  If the module can be used in multiple car lines, then a value of $23 (ASCII ‘#’) is used.

Bytes 4 and 5 (Engine RPO)

These two bytes contain the  ASCII representation of the last two characters of the engine Regular Production Option (RPO). The first character of an engine RPO is constant (always “L”) and is therefore not stored.  For the L67 engine, the value for these two bytes would be ‘6’ = $36, and ‘7’ = $37.  If the module is to be used with multiple engines, then a $23 (ASCII ‘#’) is put in both bytes.  If the module can be used with any engine type, then $2A  (ASCII ‘*’) is put in both bytes.

W4 = This DID may be written with service $3B at the discretion of the platform.

Example:

| Position | 1 | 2 | 3 | 4 | 5 | --- |
| --- | --- | --- | --- | --- | --- | --- |
| GMTV | 2002 | 1 | J | D | 9 | For 2002 Chevy Cavalier with LD9 engine |
| HEX | 66 | 31 | 4A | 44 | 39 | --- |

### MEMR – DID $B7 – GM Software Usage Description

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $B7 | GM Software Usage Description (GMSUD) | 5 | PKT | N/A | N/A | R/W5 |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Program Phase | 0 | 7 | 8 | ASCII | N/A | N/A |
| Software/Calibration Release Type | 1 | 7 | 8 | UNM | $DD, $DA, $AD, or $AA | E = N |
| Software Release Number | 2 | 7 | 8 | UNM | $01 - $FF | E = N |
| PROM Identifier character position 1 | 3 | 7 | 8 | UNM | N/A | N/A |
| PROM Identifier character position 2 | 4 | 7 | 8 | UNM | N/A | N/A |

This DID contains 5 bytes of data used to identify the intended usage of ECU development software.  The data is broken down as follows:

Byte 1 (Program Phase)

This byte is contains an ASCII  value that represents the program phase that the software is target for (e.g. B = Beta, G = Gamma, etc).

Byte 2 (Software Release Type / Calibration Release Type)

This byte is stored in USN format where the upper nibble represents the software release type and the lower nibble represents the calibration release type.  A value of  $D = development, and $A = approved for release by supplier.

Byte 3 (Software Release Number)

This byte is stored in USN format and represents the release number of the ECU software set.  Software revision numbers shall start at $01 and increment with each release.

Bytes 4 and 5 (PROM Identifier)

These two bytes are stored in USN format and are used to provide a unique representation of a software version (e.g. 2000 NAO V6 PCM with level 2400 software = $09 $60).

W5 = This DID may be written with service $3B at the discretion of the platform.

Example:

| Position | 1 | 2 | 3 | 4 | 5 | For Beta release with approved software and |
| --- | --- | --- | --- | --- | --- | --- |
| GMSUD | B | AA | 2 | 2400 |  | calibrations. 2nd software release and software |
| HEX | 42 | AA | 02 | 09 | 60 | revision level = 2400 |

### MEMR – DID $B8 – GM Bench Verification Information

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $B8 | GM Bench Verification Information (GMBVI) | 5 | PKT | N/A | N/A | R/W6 |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Verification Initials character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| Verification Initials character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| Verification Date character position 1 | 2 | 7 | 8 | UNM | N/A | N/A |
| Verification Date character position 2 | 3 | 7 | 8 | UNM | N/A | N/A |
| Verification Date character position 3 | 4 | 7 | 8 | UNM | N/A | N/A |

This DID contains 5 bytes of data used to provide information about bench verification during the development process.  This DID is made up of the following components:

Byte 1 and 2 (Verification Initials)

These 2 bytes are the ASCII representation of the first and last initials of the individual who authorized this software release by checking a sample version on the software integration bench.

Bytes 3, 4 and 5 (Verification Date)

The data stored in bytes 3,4 and 5 are saved in USN format and represent the date that the bench verification took place.  The data is arranged in the following format:  YY/MM/DD, where the Year (YY) will use 1900 as a base and be stored in byte 3.  Month (MM) is stored in byte 4, and the day (DD) is stored in byte 5.

W6 = This DID may be written with service $3B at the discretion of the platform.

Example:

| Position | 1 | 2 | 3 | 4 | 5 | --- |
| --- | --- | --- | --- | --- | --- | --- |
| GMBVI | J | D | 2001 | 2 | 3 | John Doe performed Bench Verification on Feb 3rd, 2001 |
| HEX | 4A | 44 | 65 | 02 | 03 | --- |

### CANR – DID $B9 – Subnet Configuration List – High Speed

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $B9 | Subnet_Config_List_HighSpeed (SCLHS) | >1 | PKT | N/A | N/A | R/W |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| High Speed Subnet ECU List 1 | 0 | 7 | 8 | BIN | N/A | N/A |
| High Speed Subnet ECU List 2 | 1 | 7 | 8 | BIN | N/A | N/A |
| - | - | - | - | - | - | - |

This DID contains the SCLHS. Each bit represents one ECU (with unique Diagnostic Address, see Appendix D in reference document [1]) that can exist on the high speed subnet. A logic '1' indicates that a given ECU is present on the vehicle. The bit mapping of ECUs must be documented in a CTS, SSTS, or supplemental diagnostic specification.

Example:

ECM	Engine Control Module

| Byte number | 0 (Read/Write) |  |  | 1 (Read/Write) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit location | 7 | 6 to 1 | 0 | 7 | 6 to 1 | 0 | --- |
| Bit state | 0 | … | 1 | 0 | … | 0 | --- |
| DA (hex) J3200 | --- | … | 11 | --- | … | --- | --- |
| ECU name | --- | … | ECM | --- | … | --- | --- |

### MEMR/BFPR – DID $C0 – Boot Software Part Number

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $C0 | BootSoftwarePartNumber (BSPN) | 4 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Boot Software Part Number Position 1 | 0 | 7 | 8 | UNM | $00 - $FF | E = N |
| Boot Software Part Number Position 2 | 1 | 7 | 8 | UNM | $00 - $FF | E = N |
| Boot Software Part Number Position 3 | 2 | 7 | 8 | UNM | $00 - $FF | E = N |
| Boot Software Part Number Position 4 | 3 | 7 | 8 | UNM | $00 - $FF | E = N |

This DID contains BootSoftwarePartNumber information which is a 4 byte unsigned numeric representation of the 8 digit GM part number assigned to the boot software of the ECU.  If a Boot Software Alpha Code is used, then the Alpha Code associated with this part number shall be stored in dataIdentifier $D0.

Example:

| Position | 1 | 2 | 3 | 4 | --- |
| --- | --- | --- | --- | --- | --- |
| BootSoftwarePartNumber | 16265965 |  |  |  | --- |
| Hex value | 00 | F8 | 32 | ED | --- |

### MEMR – DID $C1 - $C8 – Software Module Identifier

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $C1 - $C8 | SoftwareModuleIdentifier (SWMI) | 4 / 5-16 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Software Module 1 Identifier character position 1 | 0 | 7 | 8 | UNM | N/A | N/A |
| Software Module 1 Identifier character position 2 | 1 | 7 | 8 | UNM | N/A | N/A |
| Software Module 1 Identifier character position 3 | 2 | 7 | 8 | UNM | N/A | N/A |
| Software Module 1 Identifier character position 4 | 3 | 7 | 8 | UNM | N/A | N/A |
| or |  |  |  |  |  |  |
| Software Module 1 ASCII Identifier character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 ASCII Identifier character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 ASCII Identifier character position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 ASCII Identifier character position 4 | 3 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 ASCII Identifier character position 5 | 5 | 7 | 8 | ASCII | N/A | N/A |
| - | - | - | - | - | - | - |

DID’s ranging from $C1 to $CA contain Software Module Identifier (SWMI) information used to uniquely identify a given software or calibration module.  The SWMI shall be included in each software module that can be separately programmed into the ECU’s permanent memory via the SPS. Individual software modules can contain Boot software, Operational software, or Calibration data.

The SWMI information shall be stored using one of 2 allowed formats.  The first format consists of a 4 byte UNM representation of the 8 digit GM part number assigned to the software or calibration module.  The second allowed format consists of an ASCII representation of either an assigned GM part number or the base name of the file as released by the module supplier. If the ASCII format is used, then the part number/file name shall contain a minimum of 5 characters and a maximum of 16.

The length of the data reported in the response message for a given SWMI is used by the tool to determine which of the 2 formats is being used.  A given ECU shall use the same format for all supported SWMI.

If an Alpha Code (or Design Level Suffix) is supported, then this information shall be stored in ASCII format in a dataIdentifier with the upper nibble set to $D and the lower nibble the same as the lower nibble of the dataIdentifier that contains the corresponding SWMI. In other words, if a SWMI is stored in dataIdentifier $C1, then the corresponding Alpha Code (or Design Level Suffix) shall be stored in dataIdentifier $D1

Each SWMI has a corresponding dataIdentifier associated with it.  ECU’s are only required to support the DID’s which correspond to SWMI’s valid for that ECU.  The relationship between the SWMI and DID number is as follows:

- $C1: SoftwareModule_01_Identifier

- $C2: SoftwareModule_02_Identifier

- $C3: SoftwareModule_03_Identifier

- $C4: SoftwareModule_04_Identifier

- $C5: SoftwareModule_05_Identifier

- $C6: SoftwareModule_06_Identifier

- $C7: SoftwareModule_07_Identifier

- $C8: SoftwareModule_08_Identifier

There are a maximum of ten (10) software modules possible with DID’s $C1 - $CA.  ECUs that have a combined number of software and calibration modules that exceed 10 shall also support DID $DD.  DID $DD provides a way for the test tool to determine which DID’s contain the SWMI and SWMIAC for the 11th through the nth software/calibration module(s).

Example:

| Position | 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- | --- |
| SWMI | 16265965 |  |  |  |  |
| Hex value | 00 | F8 | 32 | ED |  |

### MCPR – DID $C9 - $CA – Software Module Identifier

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $C9 - $CA | SoftwareModuleIdentifier (SWMI) | 4 / 5-16 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Software Module 1 Identifier character position 1 | 0 | 7 | 8 | UNM | N/A | N/A |
| Software Module 1 Identifier character position 2 | 1 | 7 | 8 | UNM | N/A | N/A |
| Software Module 1 Identifier character position 3 | 2 | 7 | 8 | UNM | N/A | N/A |
| Software Module 1 Identifier character position 4 | 3 | 7 | 8 | UNM | N/A | N/A |
| or |  |  |  |  |  |  |
| Software Module 1 ASCII Identifier character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 ASCII Identifier character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 ASCII Identifier character position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 ASCII Identifier character position 4 | 3 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 ASCII Identifier character position 5 | 5 | 7 | 8 | ASCII | N/A | N/A |
| - | - | - | - | - | - | - |

DID’s ranging from $C1 to $CA contain Software Module Identifier (SWMI) information used to uniquely identify a given software or calibration module.  The SWMI shall be included in each software module that can be separately programmed into the ECU’s permanent memory via the SPS. Individual software modules can contain Boot software, Operational software, or Calibration data.

The SWMI information shall be stored using one of 2 allowed formats.  The first format consists of a 4 byte UNM representation of the 8 digit GM part number assigned to the software or calibration module.  The second allowed format consists of an ASCII representation of either an assigned GM part number or the base name of the file as released by the module supplier. If the ASCII format is used, then the part number/file name shall contain a minimum of 5 characters and a maximum of 16.

The length of the data reported in the response message for a given SWMI is used by the tool to determine which of the 2 formats is being used.  A given ECU shall use the same format for all supported SWMI.

If an Alpha Code (or Design Level Suffix) is supported, then this information shall be stored in ASCII format in a dataIdentifier with the upper nibble set to $D and the lower nibble the same as the lower nibble of the dataIdentifier that contains the corresponding SWMI. In other words, if a SWMI is stored in dataIdentifier $C1, then the corresponding Alpha Code (or Design Level Suffix) shall be stored in dataIdentifier $D1

Each SWMI has a corresponding dataIdentifier associated with it.  ECU’s are only required to support the DID’s which correspond to SWMI’s valid for that ECU.  The relationship between the SWMI and DID number is as follows:

- $C9: SoftwareModule_09_Identifier

- $CA: SoftwareModule_010_Identifier

There are a maximum of ten (10) software modules possible with DID’s $C1 - $CA.  ECUs that have a combined number of software and calibration modules that exceed 10 shall also support DID $DD.  DID $DD provides a way for the test tool to determine which DID’s contain the SWMI and SWMIAC for the 11th through the nth software/calibration module(s).

Example:

| Position | 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- | --- |
| SWMI | 16265965 |  |  |  |  |
| Hex value | 00 | F8 | 32 | ED |  |

### MEMR – DID $CB – End Model Part Number

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $CB | EndModelPartNumber (EMPN) | 4 | PKT | N/A | N/A | R/W7 |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| End Model Part Number position 1 | 0 | 7 | 8 | UNM | $00 - $FF | E = N |
| End Model Part Number position 2 | 1 | 7 | 8 | UNM | $00 - $FF | E = N |
| End Model Part Number position 3 | 2 | 7 | 8 | UNM | $00 - $FF | E = N |
| End Model Part Number position 4 | 3 | 7 | 8 | UNM | $00 - $FF | E = N |

This DID is used to identify the part number that represents the combination of hardware/software/calibrations present in the ECU as it is received in the vehicle assembly plant. This part number is also used in service to uniquely identify the combination of hardware/software/calibrations programmed into the ECU at the time the part is ordered.  Converting the 4 byte USN value to decimal provides the 8 digit part number assigned by the division. If an Alpha Code is used, then the Alpha Code associated with this part number shall be stored in dataIdentifier $DB.

W7 = This DID may or may not be updated at the conclusion of a programming event based on divisional practices.  This DID is not required to be writeable if divisional practices do not require this part number to be updated after module programming.

Example:

| Position | 1 | 2 | 3 | 4 | --- |
| --- | --- | --- | --- | --- | --- |
| EndModelPartNumber | 16265965 |  |  |  | --- |
| Hex value | 00 | F8 | 32 | ED | --- |

### MEMR – DID $CC – Base Model Part Number

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $CC | BaseModelPartNumber (BMPN) | 4 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Base Model Part Number position 1 | 0 | 7 | 8 | UNM | $00 - $FF | E = N |
| Base Model Part Number position 2 | 1 | 7 | 8 | UNM | $00 - $FF | E = N |
| Base Model Part Number position 3 | 2 | 7 | 8 | UNM | $00 - $FF | E = N |
| Base Model Part Number position 4 | 3 | 7 | 8 | UNM | $00 - $FF | E = N |

This DID contains BaseModelPartNumber information which is used during SPS programming to identify a unique combination of ECU hardware and all non-reprogrammable software (e.g. Boot software).  The BMPN must be stored in the ECU in a non-erasable memory area (e.g. boot sector of flash, or EEPROM). It shall not be possible to change the BMPN as part of a programming session or through diagnostic services when a programming session is not active.  The BMPN of a service replacement  ECU shall be identical to that of a “Production ECU” if both ECU’s have the same hardware and non-reprogrammable software. If an Alpha Code is used, then the Alpha Code associated with this part number shall be stored in dataIdentifier $DC.

Example:

| Position | 1 | 2 | 3 | 4 | --- |
| --- | --- | --- | --- | --- | --- |
| BaseModelPartNumber | 16265965 |  |  |  | --- |
| Hex value | 00 | F8 | 32 | ED | --- |

### MEMR/BFPR – DID $D0– Boot Software Part Number Alpha Code

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $D0 | BootSoftwarePartNumberAlphaCode | 2 - 3 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Boot Software Part Number Alpha Code Position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| Boot Software Part Number Alpha Code Position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| Boot Software Part Number Alpha Code Position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |

This DID contains the 2 or 3 character representation of the Alpha Code (or Design Level suffix) associated with the BootSoftwarePartNumber (stored in dataIdentifier $C0)

Example:

| Position | 1 | 2 |  |
| --- | --- | --- | --- |
| Alpha Code | RS |  |  |
| Hex value | 52 | 53 |  |

### MEMR – DID $D1 - $D8 – Software Module Identifier Alpha Code

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $D1 - $D8 | SoftwareModuleIdentifierAlphaCode (SWMIAC) | 2 - 3 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Software Module 1 Alpha Code Identifier character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 Alpha Code Identifier character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 Alpha Code Identifier character position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |

DID’s ranging from $D1 to $DA contain a 2 or 3 character representation of the Alpha Code (or Design Level suffix) for the corresponding Software Module Identifier (SWMI).  The SWMI’s occupy the dataIdentifier range $C1 to $CA.  Reference the SWMI DID description for the numerical relationship between the DID’s used for the SWMI and the corresponding SWMIAC.

- $D1: SoftwareModule_01_AlphaCode

- $D2: SoftwareModule_02_AlphaCode

- $D3: SoftwareModule_03_AlphaCode

- $D4: SoftwareModule_04_AlphaCode

- $D5: SoftwareModule_05_AlphaCode

- $D6: SoftwareModule_06_AlphaCode

- $D7: SoftwareModule_07_AlphaCode

- $D8: SoftwareModule_08_AlphaCode

The SoftwareModuleIdentifier Alpha Code shall be specified by the vehicle manufacturer.

Example:

| Position | 1 | 2 |  |
| --- | --- | --- | --- |
| Alpha Code | RS |  |  |
| Hex value | 52 | 53 |  |

### MCPR – DID $D9 - $DA – Software Module Identifier Alpha Code

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $D9 - $DA | SoftwareModuleIdentifierAlphaCode (SWMIAC) | 2 - 3 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Software Module 1 Alpha Code Identifier character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 Alpha Code Identifier character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| Software Module 1 Alpha Code Identifier character position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |

DID’s ranging from $D1 to $DA contain a 2 or 3 character representation of the Alpha Code (or Design Level suffix) for the corresponding Software Module Identifier (SWMI).  The SWMI’s occupy the dataIdentifier range $C1 to $CA.  Reference the SWMI DID description for the numerical relationship between the DID’s used for the SWMI and the corresponding SWMIAC.

- $D9: SoftwareModule_09_AlphaCode

- $DA: SoftwareModule_10_AlphaCode

The SoftwareModuleIdentifier Alpha Code shall be specified by the vehicle manufacturer.

Example:

| Position | 1 | 2 |  |
| --- | --- | --- | --- |
| Alpha Code | RS |  |  |
| Hex value | 52 | 53 |  |

### MEMR – DID $DB – End Model Part Number Alpha Code

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $DB | EndModelPartNumberAlphaCode | 2 - 3 | PKT | N/A | N/A | R/W8 |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| End Model Part Number Alpha Code character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| End Model Part Number Alpha Code character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| End Model Part Number Alpha Code character position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |

This DID contains the 2 or 3 character representation of the Alpha Code (or Design Level suffix) associated with the EndModelPartNumber (stored in dataIdentifier $CB)

W8 = This DID may or may not be updated at the conclusion of a programming event based on divisional practices.  This DID is not required to be writeable if divisional practices do not require this part number to be updated after module programming.

Example:

| Position | 1 | 2 | --- |
| --- | --- | --- | --- |
| EndModelPartNumberAlphaCode | RS |  | --- |
| Hex value | 52 | 53 | --- |

### MEMR – DID $DC – Base Model Part Number Alpha Code

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $DC | BaseModelPartNumberAlphaCode | 2 - 3 | PKT | N/A | N/A | R |

| Signal | Start Byte | Start Bit | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| Base Model Part Number Alpha Code character position 1 | 0 | 7 | 8 | ASCII | N/A | N/A |
| Base Model Part Number Alpha Code character position 2 | 1 | 7 | 8 | ASCII | N/A | N/A |
| Base Model Part Number Alpha Code character position 3 | 2 | 7 | 8 | ASCII | N/A | N/A |

This DID contains the 2 or 3 character representation of the Alpha Code (or Design Level suffix) associated with the BaseModelPartNumber (stored in dataIdentifier $CC)

Example:

| Position | 1 | 2 | --- |
| --- | --- | --- | --- |
| BaseModelPartNumberAlphaCode | RS |  | --- |
| Hex value | 52 | 53 | --- |

### VSPR – DID $DF – ECU Odometer Value

| DID | Signal | Length (Bytes) | Data Type | Range | Conversion | Access |
| --- | --- | --- | --- | --- | --- | --- |
| $DF | ECU Odometer Value | 4 | UNM | 0 - 67,108,863.984375 km | E = N / 64 | R/W |

This DID contains the ECU odometer value determined from accumulation of Vehicle Speed Sensor (VSS) pulses, converted to kilometers, and maintained for the life of the vehicle.  This DID shall allow unrestricted reading but shall allow restricted access for writing, based on the vehicle odometer strategy detailed in the “Global A Electrical Architecture Vehicle Odometer Sense & Display” functional requirements specification.

# Control Packet Identifiers (CPIDs)

CPIDs are divided into several ranges that determine their usage and the ownership of their requirements, as shown in Table 35: CPID Ranges

Definitions for individual CPIDs and their associated device controls are provided in the following sections.

| Range | Usage |
| --- | --- |
| $00 | Reserved – Cancel Device Controls |
| $01 to $7F | Application Specific CPIDs |
| $80 to $FE | Corporate Standard CPIDs |
| $FF | Reserved |

Table 35: CPID Ranges

## Device Control Limit Exceeded Return Codes

The values supported for device control limit exceeded return codes are specified in the Device Limit Exceeded Document found on the Service and Parts Operations web page (http://spo.gm.com/dept/eng/nade/general/index.html)  The definitions of the individual device controls in the following sections specify the set of device control limits to be applied for service and dynamic vehicle test (DVT).  The service device control limits shall be applied unless they have been bypassed via the SecurityAccess ($27) diagnostic service, in which case the DVT device control limits shall be applied.

### Device Control Timer Time-Out

Some device controls support a device control timer time-out as one of their device control limits (return code $0001).  The behavior of a timer used to support this device control limit is illustrated in Figure 1.  The device control may only be enabled continuously for a limited amount of time (typically 30 seconds), after which it must be disabled for a limited duration (typically 60 seconds) before it may be enabled again.  Device control limit exceeded return code $0001 is reported when the device control is first disabled and if requests to enable the device control occur during the limited duration that it is disabled.

Figure 1:  Device Control Timer Example

## CPID $10

| CPID $10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Crank Decode Mode Enable | Crankshaft Position Variation Learn Trigger | Spark Retard Enable | Disable Idle Spark Enable | Crank Decode Mode |  | Engine Sync Mode |  |
| Byte 1 |  |  |  |  |  |  |  |  |
| Byte 2 | Spark Retard Angle |  |  |  |  |  |  |  |
| Byte 3 | Exhaust Pressure Regulator Valve Enable | Exhaust Pressure Regulator Valve On |  | Displacement on Demand Solenoid Enable | Displacement on Demand Solenoid A On | Displacement on Demand Solenoid B On | Displacement on Demand Solenoid C On | Displacement on Demand Solenoid D On |
| Byte 4 |  |  |  |  |  |  |  |  |

### CDAR – Displacement on Demand Solenoid Device Control

This device control is used to override the state of the displacement on demand solenoid outputs.  This device control may be used to activate all of the engine’s cylinders (all solenoids commanded off), deactivate half of the engine’s cylinders (all solenoids commanded on), or deactivate one of the engine’s cylinders (exactly one solenoid commanded on).  Each of these types of device control has its own set of limit checks, and different limit checks are used when the engine is and is not running for the deactivation of one of the engine’s cylinders.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Displacement on Demand Solenoid Enable | 1 | BLN | N/A | $1=True; $0=False |
| Displacement on Demand Solenoid A On | 1 | BLN | N/A | $1=True; $0=False |
| Displacement on Demand Solenoid B On | 1 | BLN | N/A | $1=True; $0=False |
| Displacement on Demand Solenoid C On | 1 | BLN | N/A | $1=True; $0=False |
| Displacement on Demand Solenoid D On | 1 | BLN | N/A | $1=True; $0=False |

If (Displacement on Demand Solenoid Enable is True)

If ((Displacement on Demand Solenoid A On is False) AND 
     (Displacement on Demand Solenoid B On is False) AND 
     (Displacement on Demand Solenoid C On is False) AND 
     (Displacement on Demand Solenoid D On is False))

All of the engine’s cylinders shall be activated by commanding displacement on demand solenoids A, B, C, and D off.

ElseIf ((Displacement on Demand Solenoid A On is True) AND 
            (Displacement on Demand Solenoid B On is True) AND 
            (Displacement on Demand Solenoid C On is True) AND 
            (Displacement on Demand Solenoid D On is True))

Manage the request to deactivate half of the engine’s cylinders as described in Section 5.2.1.1.

Else

Clear the indication that the DoD solenoid to command on has been found.

Clear the indication that too many DoD solenoids are requested to be commanded on.

If (Displacement on Demand Solenoid A On is True)

If (the indication that the DoD solenoid to command on has been found is set)

Set the indication that too many DoD solenoids are requested to be commanded on.

Else

Set the DoD solenoid to command on to displacement on demand solenoid A.

Set the indication that the DoD solenoid to command on has been found.

EndIf

EndIf

If (Displacement on Demand Solenoid B On is True)

If (the indication that the DoD solenoid to command on has been found is set)

Set the indication that too many DoD solenoids are requested to be commanded on.

Else

Set the DoD solenoid to command on to displacement on demand solenoid B.

Set the indication that the DoD solenoid to command on has been found.

EndIf

EndIf

If (Displacement on Demand Solenoid C On is True)

If (the indication that the DoD solenoid to command on has been found is set)

Set the indication that too many DoD solenoids are requested to be commanded on.

Else

Set the DoD solenoid to command on to displacement on demand solenoid C.

Set the indication that the DoD solenoid to command on has been found.

EndIf

EndIf

If (Displacement on Demand Solenoid D On is True)

If (the indication that the DoD solenoid to command on has been found is set)

Set the indication that too many DoD solenoids are requested to be commanded on.

Else

Set the DoD solenoid to command on to displacement on demand solenoid D.

Set the indication that the DoD solenoid to command on has been found.

EndIf

EndIf

If (the indication that too many DoD solenoids are requested to be commanded on is set)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

If (the engine is running)

Manage the request to deactivate one of the engine’s cylinders with the engine running as described in Section5.2.1.2.

Else

Manage the request to deactivate one of the engine’s cylinders with the engine not running as described in Section5.2.1.3.

EndIf

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0312 | Invalid displacement on demand device control request. | Y | Y |

#### Deactivate Half of the Engine’s Cylinders

This device control is used to deactivate half of the engine’s cylinders by commanding all of the displacement on demand solenoid outputs on.

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

Half of the engine’s cylinders shall be deactivated by commanding displacement on demand solenoids A, B, C, and D on.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $000C | Maximum deactivation time exceeded. | Y | Y |
| $0010 | Engine speed out of range. - Engine speed does not lie between calibratable lower and upper thresholds. | Y | Y |
| $0308 | Fault active. - Crank sensor fault. - Cam sensor fault. - Engine oil pressure sensor fault. - Powertrain relay fault. - Intake air temperature sensor fault (service only) (E21240) - Brake vacuum pressure sensor fault (service only). - Displacement on demand solenoid driver fault (service only). - Electronic throttle control reduced engine power active (service only). - Engine coolant temperature sensor fault (service only). - Manifold absolute pressure sensor fault (service only). - Vehicle speed fault (service only). | Y | Y |
| $030A | Accelerator pedal position high. - Accelerator pedal effective position exceeds a calibratable threshold. | Y | N |
| $030B | Piston protection active. | Y | N |
| $030C | Engine oil temperature out of range. - Engine oil temperature does not lie between calibratable lower and upper thresholds. | Y | N |
| $030D | Engine oil pressure out of range. - Engine oil pressure does not lie between calibratable lower and upper thresholds. | Y | Y |
| $030F | Engine oil aeration present. | Y | N |
| $0310 | Engine metal overtemperature protection active. | Y | N |
| $0311 | Accelerator pedal position increase high. - Change in accelerator pedal effective position exceeds a calibratable threshold. | Y | N |
| $0315 | Catalyst converter overtemperature protection active. | Y | N |
| $0316 | Brake vacuum pressure low. - Brake vacuum pressure falls below a calibratable threshold. | Y | N |
| $0318 | Axle torque limiting active. | Y | N |
| $0319 | Brake torque management active. | Y | N |
| $0601 | Engine coolant temperature too high. - Hot coolant mode active. | Y | N |
| $0604 | Engine not running. | Y | N |
| $0A05 | Engine speed too high. - Engine overspeed protection active. | Y | Y |
| $0E02 | Vehicle speed not zero. | Y | N |
| $2102 | Transmission gear incorrect. - Disabled in transmission gears selected via calibration. | Y | N |
| $2103 | Transmission range incorrect. - Disabled in transmission ranges selected via calibration. | Y | N |
| $5003 | Ignition voltage out of range. - Powertrain relay voltage does not lie between calibratable lower and upper thresholds. | Y | N |
| $031D | Engine coolant temperature out of range. - Engine coolant temperature does not lie between calibratable lower and upper thresholds. | Y | N |

#### Deactivate One of the Engine’s Cylinders with Engine Running

This device control is used to deactivate one of the engine’s cylinders with the engine running by commanding the appropriate displacement on demand solenoid output on.

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

Exactly one of the engine’s cylinders shall be deactivated by commanding the appropriate displacement on demand solenoid on (while commanding all remaining displacement on demand solenoids off).

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $000C | Maximum deactivation time exceeded. | Y | Y |
| $0010 | Engine speed out of range. - Engine speed does not lie between calibratable lower and upper thresholds. | Y | Y |
| $0308 | Fault active. - Crank sensor fault. - Cam sensor fault. - Engine oil pressure sensor fault. - Powertrain relay fault. - Intake air temperature sensor fault (E21240) - Brake vacuum pressure sensor fault. - Displacement on demand solenoid driver fault. - Electronic throttle control reduced engine power active. - Engine coolant temperature sensor fault. - Manifold absolute pressure sensor fault. - Vehicle speed fault. | Y | Y |
| $030A | Accelerator pedal position high. - Accelerator pedal effective position exceeds a calibratable threshold. | Y | Y |
| $030B | Piston protection active. | Y | Y |
| $030C | Engine oil temperature out of range. - Engine oil temperature does not lie between calibratable lower and upper thresholds. | Y | Y |
| $030D | Engine oil pressure out of range. - Engine oil pressure does not lie between calibratable lower and upper thresholds. | Y | Y |
| $030F | Engine oil aeration present. | Y | Y |
| $0310 | Engine metal overtemperature protection active. | Y | Y |
| $0311 | Accelerator pedal position increase high. - Change in accelerator pedal effective position exceeds a calibratable threshold. | Y | Y |
| $0315 | Catalyst converter overtemperature protection active. | Y | Y |
| $0316 | Brake vacuum pressure low. - Brake vacuum pressure falls below a calibratable threshold. | Y | Y |
| $0318 | Axle torque limiting active. | Y | Y |
| $0319 | Brake torque management active. | Y | Y |
| $0601 | Engine coolant temperature too high. - Hot coolant mode active. | Y | Y |
| $0604 | Engine not running. | Y | Y |
| $0A05 | Engine speed too high. - Engine overspeed protection active. | Y | Y |
| $0E02 | Vehicle speed not zero. | Y | Y |
| $2102 | Transmission gear incorrect. - Disabled in transmission gears selected via calibration. | Y | Y |
| $2103 | Transmission range incorrect. - Disabled in transmission ranges selected via calibration. | Y | Y |
| $5003 | Ignition voltage out of range. - Powertrain relay voltage does not lie between calibratable lower and upper thresholds. | Y | Y |
| $031D | Engine coolant temperature out of range. - Engine coolant temperature does not lie between calibratable lower and upper thresholds. | Y | Y |

#### Deactivate One of the Engine’s Cylinders with Engine Not Running

This device control is used to deactivate one of the engine’s cylinders with the engine not running by commanding the appropriate displacement on demand solenoid output on.

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

Exactly one of the engine’s cylinders shall be deactivated by commanding the appropriate displacement on demand solenoid on (while commanding all remaining displacement on demand solenoids off).

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0201 | Engine speed not zero. | Y | Y |
| $0308 | Fault active. - Powertrain relay fault. | Y | Y |
| $0E02 | Vehicle speed not zero. | Y | Y |
| $0E03 | Transmission not in Park or Neutral. | Y | Y |
| $5003 | Ignition voltage out of range. - Powertrain relay voltage does not lie between calibratable lower and upper thresholds. | Y | Y |

### EPSR – Crank Decode Mode Device Control

This device control is used to override the crank decode mode and the engine sync mode.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Crank Decode Mode Enable | 1 | BLN | N/A | $1=True; $0=False |
| Crank Decode Mode | 2 | ENM | N/A | $0=60X-2 Threshold; $1=60X-2 Percentage; $3=Disable |
| Engine Sync Mode | 2 | ENM | N/A | $0=Crank and Primary Cam; $1=Primary Cam Backup; $2=Primary and Secondary Cam Backup; $3=Engine Sync Disabled |

If (Crank Decode Mode Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The crank decode mode and engine sync mode shall be commanded to the values requested in Crank Decode Mode and Engine Sync Mode, respectively.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $1701 | Engine is running. | Y | N |

### SPKR – Spark Retard Device Control

This device control is used to retard the spark angle from the software calculated spark angle.  For example, if the software calculated spark angle was 15 degrees advanced and this device control requested a 3 degree retard, the commanded spark angle that would be delivered would be 12 degrees advanced.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Spark Retard Enable | 1 | BLN | N/A | $1=True; $0=False |
| Spark Retard Angle | 8 | UNM | 0.0 - 89.6484375 deg | E = N * 0.3515625 |

If (Spark Retard Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The spark angle delivered shall be adjusted by the requested spark retard angle in Spark Retard Angle.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0001 | Generic Device Control Timer (30 second enable, 60 second disable) (This limit only applies while the engine is off-idle) | Y | N |
| $0701 | Requested spark retard is too large | Y | N |

### SPKR – Disable Idle Spark Device Control

This device control is used to disable the portion of the spark control algorithm that adjusts spark angle to correct the difference between commanded idle engine speed and actual idle engine speed.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Disable Idle Spark Enable | 1 | BLN | N/A | $1=True; $0=False |

If (Disable Idle Spark Enable is True)

Disable Idle Spark

Else

Return to normal control.

EndIf

### MSFR – Crankshaft Position Variation Learn Trigger Device Control

This device control is used to activate the crankshaft position variation learn (CASE) algorithm which calculates the reference pulse correction factors for the misfire diagnostic.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Crankshaft Position Variation Learn Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Crankshaft Position Variation Learn Trigger is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

If	(the crankcase position variation learn algorithm has run and is complete)

Then

Return to normal control.

Else

The crankcase position variation learn algorithm shall be enabled

Endif

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description (Non Two-mode hybrid vehicles) | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0010 | RPM out of range | Y | N |
| $0406 | Incorrect Power Mode | Y | Y |
| $1101 | The vehicle is not in park or neutral | Y | Y |
| $1102 | A diagnostic fault is currently active - Crank sensor faults - Cam sensor faults - 5v reference faults - Fan speed faults - Coolant temp sensor fault - Trans engaged fault - Manual trans TOSS fault | Y | Y |
| $1103 | Coolant temperature is not in acceptable range | Y | Y |
| $1104 | Too many failed attempts to learn CASE this power up | Y | Y |
| $1105 | CASE already learned this power up | Y | Y |
| $1106 | CASE not supported in this controller | Y | Y |
| $1108 | PTO is enabled | Y | Y |

| Return Code | Device Control Limit Exceeded Description (Two-mode hybrid vehicles) | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0010 | RPM out of range | Y | N |
| $0406 | Incorrect Power Mode | Y | Y |
| $1101 | The vehicle is not in park or neutral | Y | Y |
| $1102 | A diagnostic fault is currently active - Hood switch faults - HCP communication faults - Crank sensor faults - Cam sensor faults - 5v reference faults - Fan speed faults - Coolant temp sensor fault - Trans engaged fault - Manual trans TOSS fault - Brake fault | Y | Y |
| $1103 | Coolant temperature is not in acceptable range | Y | Y |
| $1104 | Too many failed attempts to learn CASE this power up | Y | Y |
| $1105 | CASE already learned this power up | Y | Y |
| $1106 | CASE not supported in this controller | Y | Y |
| $1107 | Brake pedal is not being applied | Y | N |
| $1108 | PTO is enabled | Y | Y |
| $7002 | Hood is ajar | Y | Y |

### EFVR – Exhaust Pressure Regulator Valve Solenoid Device Control

This device control is used to verify functionality of the Exhaust Pressure Regulator Valve by allowing the Exhaust Pressure Regulator Valve output to be commanded to a specific state.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Exhaust Pressure Regulator Valve Enable | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Pressure Regulator Valve On | 1 | BLN | N/A | $1=True; $0=False |

If (Exhaust Pressure Regulator Valve Enable is True)

If (Exhaust Pressure Regulator Valve Off is True)

The Exhaust Pressure Regulator Valve output shall be commanded Off

Else

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The Exhaust Pressure Regulator Valve output shall be commanded On

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0401 | Vehicle Speed not zero | Yes | No |
| $0402 | Vehicle Speed fault exists | Yes | No |

## CPID $14

| CPID $14 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Throttle Position Enable | Idle Engine Speed Enable |  | Learned Airflow Adapts Reset Trigger | Electronic Throttle Control Snapshot Trigger | Electronic Throttle Control Validation Parameter Reset Trigger | Forced Idle Enable | Wide Open Throttle Self Test Enable |
| Byte 1 | Throttle Position |  |  |  |  |  |  |  |
| Byte 2 | Idle Engine Speed |  |  |  |  |  |  |  |
| Byte 3 | Supercharger Boost Solenoid Enable |  |  |  | Intake Manifold Tuning Valve Enable | Intake Manifold Tuning Valve Closed | Intercooler Pump Enable | Intercooler Pump On |
| Byte 4 | Supercharger Boost Solenoid Duty Cycle |  |  |  |  |  |  |  |

### TPSR – Forced Idle / Wide Open Throttle Self Test Device Control

The forced idle device control is used to prevent the accelerator pedal contribution to throttle area from being added to the idle contribution to throttle area, effectively forcing idle.  This device control allows the accelerator pedal to be moved to its maximum position during DVT, in order to learn the maximum accelerator pedal position, without affecting engine operation.

The wide open throttle (WOT) self test device control is used to limit the duration of time that the throttle spends at WOT while the accelerator pedal is being held at its maximum position.  This device control allows the throttle to be moved to WOT during DVT, in order to learn the maximum throttle position, while minimally affecting engine operation.  This is accomplished by preventing the accelerator pedal contribution to throttle area from being added to the idle contribution to throttle area, effectively forcing idle, once WOT is achieved (i.e., the throttle position exceeds a WOT threshold) or once the time allowed to achieve WOT has expired.

When terminating these device controls, the learned minimum accelerator pedal position must be reset to 100% in order to prevent a sudden increase in the accelerator pedal contribution to throttle area.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Forced Idle Enable | 1 | BLN | N/A | $1=True; $0=False |
| Wide Open Throttle Self Test Enable | 1 | BLN | N/A | $1=True; $0=False |

If ((Forced Idle Enable is True) OR (Wide Open Throttle Self Test Enable is True))

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

If (the indication that this device control is enabled is set)

Terminate this device control (as described below).

Else

Return to normal control.

EndIf

Else

If ((the indication that this device control is enabled is not set) AND 
     (engine speed is at idle) AND (accelerator pedal position is at idle))

Set the indication that this device control is enabled.

Clear the indication that the WOT self test device control failed.

Clear the indication that this device control is complete.

EndIf

If (the indication that this device control is enabled is set)

If (Forced Idle Enable is True)

Prevent the accelerator pedal contribution to throttle area from being added to the idle contribution to throttle area.

ElseIf (Wide Open Throttle Self Test Enable is True)

If (throttle position exceeds a calibratable threshold)

Terminate this device control (as described below).

ElseIf (the WOT self test device control has been enabled longer than a calibratable duration)

Set the indication that the WOT self test device control failed.

Terminate this device control (as described below).

Else

Allow the accelerator pedal contribution to throttle area to be added to the idle contribution to throttle area.

EndIf

EndIf

Else

Continue normal control.

EndIf

EndIf

Else

If (the indication that this device control is enabled is set)

Terminate this device control (as described below).

Else

Return to normal control.

EndIf

EndIf

The following actions shall be performed upon the termination of this device control:

Prevent the accelerator pedal contribution to throttle area from being added to the idle contribution to throttle area.

Reset the learned minimum accelerator pedal position to 100%.

Set the indication that this device control is complete.

Clear the indication that this device control is enabled.

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0004 | Invalid device control selected (used if trying to access a device control in service which is only allowed for assembly line testing). | Y | N |
| $0401 | Vehicle speed not zero | N | Y |
| $0E03 | Transmission not in Park or Neutral. | N | Y |
| $1603 | Failure detected within the throttle control system. | N | Y |

### IMTR – Intake Manifold Tuning Valve Device Control

This device control is used to override the state of the intake manifold tuning valve output.  A calibration is used to select whether commanding the intake manifold tuning valve output on or off results in the valve being commanded closed or open for a particular application.  The intake manifold tuning valve is commanded closed in order to prevent the manifold runners from communicating.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Intake Manifold Tuning Valve Enable | 1 | BLN | N/A | $1=True; $0=False |
| Intake Manifold Tuning Valve Closed | 1 | BLN | N/A | $1=True; $0=False |

If (Intake Manifold Tuning Valve Enable is True)

The intake manifold tuning valve output shall be commanded to the value required to result in the position requested in Intake Manifold Tuning Valve Closed.

Else

Return to normal control.

EndIf

### BSTR – Intercooler Pump Device Control

This device control is used to override the state of the intercooler pump output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Intercooler Pump Enable | 1 | BLN | N/A | $1=True; $0=False |
| Intercooler Pump On | 1 | BLN | N/A | $1=True; $0=False |

If (Intercooler Pump Enable is True)

The intercooler pump output shall be commanded to the value requested in Intercooler Pump On.

Else

Return to normal control.

EndIf

### BSTR – Supercharger Boost Solenoid Device Control

This device control is used to override the state of the supercharger boost solenoid output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Supercharger Boost Solenoid Enable | 1 | BLN | N/A | $1=True; $0=False |
| Supercharger Boost Solenoid Duty Cycle | 8 | UNM | 0 - 99.609375 % | E = N * 100 / 256 |

If (Supercharger Boost Solenoid Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The supercharger boost solenoid output shall be commanded to the value requested in Supercharger Boost Solenoid Duty Cycle.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $031B | Protection mode active – desired boost unattainable. | Y | N |

### TPSR – Throttle Position Device Control

This device control is used to override the commanded throttle position.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Throttle Position Enable | 1 | BLN | N/A | $1=True; $0=False |
| Throttle Position | 8 | UNM | 0.0 - 99.609375 % | E = N * 0.390625 |

If (Throttle Position Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The throttle position shall be commanded to the position requested in Throttle Position.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $1601 | Engine speed or vehicle speed too high | Y | Y |

### IDLR - Idle Engine Speed Device Control

This device control is used to override the idle engine speed.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Idle Engine Speed Enable | 1 | BLN | N/A | $1=True; $0=False |
| Idle Engine Speed | 8 | UNM | 0.0 - 2040.0 rpm | E = N * 8.0 |

If (Idle Engine Speed Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The idle engine speed shall be commanded to the speed requested in Idle Engine Speed.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0401 | Vehicle speed not zero. | Y | N |
| $0402 | Vehicle speed fault. | Y | N |
| $0903 | Transmission not in park or neutral | Y | N |
| $2B00 | Transmission gear data is not valid (auto or manual trans) | Y | N |

### TPSR – Learned Airflow Adapts Reset Trigger Device Control

This device control is used to reset the learned airflow adaptive values to their initial values.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Learned Airflow Adapts Reset Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Learned Airflow Adapts Reset Trigger is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

ElseIf (the learned airflow adaptive values are already reset)

Return to normal control.

Else

Reset the learned airflow adaptive values.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0201 | Engine Speed Not Zero | Y | N |
| $0401 | Vehicle Speed Not Zero | Y | N |

### TPSR - Electronic Throttle Control Snapshot Trigger Device Control

This device control is used to trigger a snapshot of throttle control parameters that will later be read out of the controller and analyzed by off-board processing tools.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Electronic Throttle Control Snapshot Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Electronic Throttle Control Snapshot Trigger is True)

If (the snapshot of electronic throttle control parameters is complete)

Return to normal control.

Else

Perform the snapshot of the electronic throttle control parameters.

EndIf

Else

Return to normal control.

EndIf

### TPSR - Electronic Throttle Control Validation Parameter Reset Trigger Device Control

This device control is used to trigger a reset of the ETC long term measurement and statistical parameters typically monitored during development and validation.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Electronic Throttle Control Validation Parameter Reset Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Electronic Throttle Control Validation Parameter Reset Trigger is True)

If (the reset is complete)

Return to normal control.

Else

Perform the reset of the ETC validation parameters.

EndIf

Else

Return to normal control.

EndIf

## CPID $18

| CPID $18 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Fuel Injector Enable | Fuel Injector On | Fuel Closed Loop Enable | Fuel Closed Loop |  | Fuel Trim Reset Trigger | Fuel Trim Learn Enable | Fuel Trim Learn On |
| Byte 1 | Fuel Injector Identifier |  |  |  |  |  |  |  |
| Byte 2 | Canister Purge Enable | Evap Service Test Trigger | Canister Vent Enable | Canister Vent Open | Fuel Pump Enable | Fuel Comp Reset Trigger | Secondary Fuel Pump Enable | Secondary Fuel Pump On |
| Byte 3 | Canister Purge Duty Cycle |  |  |  |  |  |  |  |
| Byte 4 | Fuel Pump Duty Cycle |  |  |  |  |  |  |  |

### FULR – Fuel Injector Device Control

This device control is used to override the state of the fuel injector outputs.  This device control may be used to trigger a single injector to be commanded on for a short duration in order to test the flow of the injector or to disable injectors in order to cut off fuel to the associated cylinders.  Each of these types of device control has its own set of limit checks.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Fuel Injector Enable | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Injector On | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Injector Identifier | 8 | PKT | N/A | N/A |
| Cylinder A (most significant bit) | 1 | BLN | N/A | $1=True; $0=False |
| Cylinder B | 1 | BLN | N/A | $1=True; $0=False |
| Cylinder C | 1 | BLN | N/A | $1=True; $0=False |
| Cylinder D | 1 | BLN | N/A | $1=True; $0=False |
| Cylinder E | 1 | BLN | N/A | $1=True; $0=False |
| Cylinder F | 1 | BLN | N/A | $1=True; $0=False |
| Cylinder G | 1 | BLN | N/A | $1=True; $0=False |
| Cylinder H (least significant bit) | 1 | BLN | N/A | $1=True; $0=False |

If (Fuel Injector Enable is True)

If (Fuel Injector On is True)

Manage the fuel injector flow device control as described in Section5.4.1.1.

Else

Manage the fuel injector disable device control as described in Section5.4.1.2.

EndIf

Else

Return to normal control.

EndIf

#### Fuel Injector Flow Device Control

This device control is used to trigger a single injector to be commanded on for a duration of 500 ms (via approximately 100, 5 ms pulses) in order to test the flow of the injector.

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

If (fuel injector output specified by the Fuel Injector Identifier has been commanded on for at least 500 ms)

Return to normal control.

Else

The fuel injector output specified by the Fuel Injector Identifier shall be commanded on.

EndIf

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0201 | Engine speed not zero. | Y | N |
| $0202 | Injector already tested. | Y | N |
| $0203 | Injector not valid. | Y | Y |
| $0204 | Injector flow test in progress. | Y | N |
| $0205 | More than one injector selected. | Y | N |
| $0206 | Fuel disabled by theft deterrent system. | Y | N |

#### Fuel Injector Disable Device Control

This device control is used to disable injectors in order to cut off fuel to the associated cylinders.

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The fuel injector output specified by the Fuel Injector Identifier shall be commanded off.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0001 | Generic device control timer time-out active (30 second enable, 60 second disable). | Y | N |
| $0301 | Injector not valid. | Y | Y |
| $0302 | Too many injectors to be disabled simultaneously. | Y | N |
| $0303 | Injector flow test in progress. | Y | Y |
| $000B | Displacement on demand active. | Y | Y |

### FULR – Fuel Closed Loop Device Control

This device control is used to enable or disable closed loop fuel.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Fuel Closed Loop Enable | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Closed Loop | 1 | BLN | N/A | $1=True; $0=False |

If (Fuel Closed Loop Enable is True)

Closed loop fuel shall be commanded to the requested state in Fuel Closed Loop.

Else

Return to normal control.

EndIf

### FADR – Fuel Trim Learn Device Control

This device control is used to enable or disable fuel trim learning.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Fuel Trim Learn Enable | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Trim Learn On | 1 | BLN | N/A | $1=True; $0=False |

If (Fuel Trim Learn Enable is True)

The fuel trim learn shall be commanded to the requested state in Fuel Trim Learn On

Else

Return to normal control.

EndIf

### FADR – Fuel Trim Reset Trigger Device Control

This device control is used to reset the learned fuel trim values to their initial values.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Fuel Trim Reset Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Fuel Trim Reset Trigger is True)

If (the learned fuel trim values are reset)

Return to normal control.

Else

Reset the learned fuel trim values

EndIf

Else

Return to normal control.

EndIf

### EVPR – Canister Purge Device Control

This device control is used to verify functionality of the fuel vapor purge system by allowing the Canister Purge output to be commanded to a specific duty cycle.  The software calculated duty cycle should be available to be monitored via a parameter identifier as feedback for the device control override.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Canister Purge Enable | 1 | BLN | N/A | $1=True; $0=False |
| Canister Purge Duty Cycle | 8 | UNM | 0.0 - 99.609375 % | E = N * 0.390625 |

If (Canister Purge Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The canister purge duty cycle shall be commanded to the requested duty cycle in Canister Purge Duty Cycle.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0F01 | Vacuum Level too high in purge system | Y | Y |
| $0F02 | Failure detected in purge system | Y | Y |
| $0F03 | EVAP system test in progress | Y | N |

### EVPR – Canister Vent Device Control

This device control is used to either open or close the canister vent.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Canister Vent Enable | 1 | BLN | N/A | $1=True; $0=False |
| Canister Vent Open | 1 | BLN | N/A | $1=True; $0=False |

If (Canister Vent Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The canister vent shall be commanded to the requested state in Canister Vent Open

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $1001 | Vacuum Level too high in purge system | Y | Y |
| $1002 | Failure detected in purge system | Y | Y |
| $0F03 | EVAP system test in progress | Y | N |

### EVPR – Evap Service Test Trigger Device Control

This device control is used to activate the evaporative emissions service bay test.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Evap Service Test Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Evap Service Test Trigger is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

If (the Evap Service Bay Test is complete)

Return to normal control.

Else

The Evap Service Bay Test shall be enabled

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $1301 | The service bay test is not available in the controller. | Y | Y |
| $1302 | The Evap IM flag is already set. | Y | Y |
| $1303 | The engine is running. | Y | Y |
| $1304 | The coolant temperature is too high. | Y | Y |
| $1305 | Other application specific enable criteria has not been met. | Y | Y |

### FPMR – Fuel Pump Device Control

This device control is used to override the state of the fuel pump output. The device control is a service diagnostic aid that is used to determine if the fuel pump system is functioning properly. This device control may be used to operate a variable speed fuel pump at a range of duty cycle or simply on and off, as it would be in a relay controlled fuel pump. Each of these types of device control has its own set of limit checks, and different limit checks are used when the engine is and is not running for the operation of the fuel pump.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Fuel Pump Enable | 1 | BLN | N/A | $1=True; $0=False |
| Fuel Pump Duty Cycle | 8 | UNM | 0.0 - 99.609375 % | E = N * 100 / 256 |

#### Fuel Pump Device Control with Engine Not Running

With the engine off, this device control is intended to be used as a discrete control for circuit or pressure diagnosis. The fuel pump will be controlled in a similar manner as a relay controlled fuel pump. With the engine off, the Fuel Pump can not be commanded on for more than 3 seconds.

If   (Fuel Pump Enable is True)

If   (the engine is running)

Manage the request to control the Fuel Pump as described in Section 5.4.8.2

Else

If   (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The fuel pump output shall be commanded to the value requested in Fuel Pump Duty Cycle.

(Typical values will be 100% for on and 0% for off)

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $5003 | Voltage Out of Range System voltage is above or below a calibrated value | Y | N |
| $0008 | The Output Control timer has expired. Please Wait. The fuel pump must not be commanded on (Duty Cycle > 0.0) continuously for more than 3 seconds with engine not running | Y | N |

#### Fuel Pump Device Control with Engine Running

With the engine running, this device control is used to operate the fuel pump at the various duty cycles available to the engine controller. Fuel pump speed device control will be used for fuel system pressure and noise diagnosis/troubleshooting.  With the engine running, the Fuel Pump can not be commanded on (Duty Cycle > 0.0) for more than 30 seconds.

If (Fuel Pump Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The fuel pump output shall be commanded to the value requested in Fuel Pump Duty Cycle.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0401 | Vehicle speed detected - Vehicle must not be moving | Y | N |
| $0402 | Vehicle speed DTC present, output control not allowed - DTCs for VSS or TOSS | Y | N |
| $5003 | Voltage Out of Range - System voltage is above or below a calibrated value | Y | N |
| $0008 | The Output Control timer has expired. Please Wait. - The fuel pump must not be commanded on (Duty Cycle > 0.0) continuously for more than 30 seconds with engine running | Y | N |

### E85R – Fuel Composition Reset Trigger Device Control

This device control is used to reset the learned ethanol content of the fuel.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Fuel Comp Reset Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Fuel Comp Reset Trigger is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

If (Ethanol Learn is in progress).

Disable Ethanol Learn.

EndIf

If (the reset is complete)

Return to normal control.

Else

Perform the reset of the Fuel Composition parameters.

EndIf

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0004 | Invalid device control selected | Y | N |

### FLVR – Secondary Fuel Pump Device Control

This device control is used to override the state of the electric secondary transfer pump output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Secondary Fuel Pump Enable | 1 | BLN | N/A | $1=True; $0=False |
| Secondary Fuel Pump On | 1 | BLN | N/A | $1=True; $0=False |

If (secondary fuel pump enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The Secondary Fuel Pump shall be commanded to the state indicated by Secondary Fuel Pump On.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0401 | Vehicle speed detected - Vehicle must not be moving | Y | N |
| $0403 | Fuel pump: time limit exceeded. | Y | N |
| $0406 | Incorrect power mode. | Y | Y |
| $0407 | Vehicle not configured with an electric secondary transfer fuel pump | Y | N |

## CPID $1C

| CPID $1C |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Exhaust Gas Recirculation Enable |  | Secondary Air Injection System Enable | Secondary Air Injection Pump On | Secondary Air Injection Valve On | Secondary Air Injection Diagnostic Service Bay Test Trigger | Catalyst Diagnostic Service Bay Test Trigger |  |
| Byte 1 | Exhaust Gas Recirculation Pintle Lift |  |  |  |  |  |  |  |
| Byte 2 | Exhaust Cam Phase Solenoid Bank 1 Enable | Exhaust Cam Phase Solenoid Bank 1 On | Exhaust Cam Phase Solenoid Bank 2 Enable | Exhaust Cam Phase Solenoid Bank 2 On | Intake Cam Phase Solenoid Bank 1 Enable | Intake Cam Phase Solenoid Bank 1 On | Intake Cam Phase Solenoid Bank 2 Enable | Intake Cam Phase Solenoid Bank 2 On |
| Byte 3 |  |  |  |  |  |  |  |  |
| Byte 4 |  |  |  |  |  |  |  |  |

### PHSR – Exhaust and Intake Cam Phase Solenoid Device Controls

These device controls are used to override the states of the exhaust and intake cam phase solenoid outputs for banks 1 and 2.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Exhaust Cam Phase Solenoid Bank 1 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Cam Phase Solenoid Bank 1 On | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Cam Phase Solenoid Bank 2 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Cam Phase Solenoid Bank 2 On | 1 | BLN | N/A | $1=True; $0=False |
| Intake Cam Phase Solenoid Bank 1 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Intake Cam Phase Solenoid Bank 1 On | 1 | BLN | N/A | $1=True; $0=False |
| Intake Cam Phase Solenoid Bank 2 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Intake Cam Phase Solenoid Bank 2 On | 1 | BLN | N/A | $1=True; $0=False |

If (Exhaust/Intake Cam Phase Solenoid Bank N Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The exhaust/intake cam phase solenoid output for bank N shall be commanded to the value requested in Exhaust/Intake Cam Phase Solenoid Bank N On (i.e., 100% duty cycle or 0% duty cycle).

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0001 | Device control timer time-out active (5 second enable, 60 second disable). | Y | N |
| $0004 | Invalid device control selected (vehicle not equipped with camshaft position actuators). | Y | N |
| $0201 | Engine speed not zero. | Y | N |

### AIRR – Secondary Air Injection Diagnostic Service Bay Test Trigger Device Control

This device control is used to trigger the secondary air injection (SAI) diagnostic service bay test, which causes the on-board SAI diagnostic to execute until it is completed or aborted.  The SAI diagnostic consists of three phases, SAI on test, SAI valve shut test, and SAI pump off test, all of which are triggered by this device control.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Secondary Air Injection Diagnostic Service Bay Test Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Secondary Air Injection Diagnostic Service Bay Test Trigger is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The secondary air injection diagnostic service bay test shall be triggered.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0001 | Device control timer time-out active | Y | Y |
| $0002 | Device control not supported. | Y | Y |
| $000A | DTC present. - Intake Air Temperature (IAT) Sensor Circuit Low - Intake Air Temperature (IAT) Sensor Circuit High - Intake Air Temperature (IAT) Sensor Performance - Secondary Air Injection (AIR) Solenoid Control Circuit Bank 1 - Secondary Air Injection (AIR) Pump Control Circuit Bank 1 - ECM/PCM Internal Engine Off Timer Performance | Y | Y |
| $0201 | Engine speed not zero. | Y | Y |
| $0601 | Coolant temperature too high | Y | Y |
| $0E02 | Vehicle speed not zero. | Y | Y |
| $0E03 | Transmission not in Park or Neutral. | Y | Y |
| $5003 | Ignition voltage out of range. | Y | Y |

### AIRR – Secondary Air Injection Reaction System Device Control

This device control is used to either turn the AIR Pump and/or AIR Valve on or off.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Secondary Air Injection Reaction System Enable | 1 | BLN | N/A | $1=True; $0=False |
| Secondary Air Injection Reaction Pump On | 1 | BLN | N/A | $1=True; $0=False |
| Secondary Air Injection Reaction Valve On | 1 | BLN | N/A | $1=True; $0=False |

If  (Secondary Air Injection Reaction System Enable is True)

If  (Secondary Air Injection Reaction Pump On is True OR Secondary Air Injection Reaction Valve On is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The AIR Pump and Valve shall be commanded to the states requested by Secondary Air Injection Reaction Pump On and Secondary Air Injection Reaction Valve On

Endif

Else

Secondary Air Injection Reaction Pump On is False AND Secondary Air Injection Reaction Valve On is False  (The AIR Pump and Valve shall be commanded off)

Endif

Else

Return to normal control

Endif

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0001 | Generic Device Control Timer (30 second enable, 60 second disable) | Y | N |
| $1103 | Engine coolant temperature low | Y | N |

### EGRR – Exhaust Gas Recirculation Device Control

This device control is used to verify functionality of the Exhaust Gas Recirculation (EGR) system by allowing the Exhaust Gas Recirculation output to be commanded to a specific lift.  The software calculated pintle lift should be available to be monitored via a parameter identifier as feedback for the device control override.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Exhaust Gas Recirculation Enable | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Gas Recirculation Pintle Lift | 8 | UNM | 0.0 - 99.609375 % | E = N * 0.390625 |

If  (Exhaust Gas Recirculation Enable is True)

If  (the engine is not running)

The pintle lift shall be commanded to the requested pintle lift in Exhaust Gas Recirculation Pintle Lift

Else

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The pintle lift shall be commanded to the requested pintle lift in Exhaust Gas Recirculation Pintle Lift

Endif

Endif

Else

Return to normal control

Endif

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0001 | Generic Device Control Timer (30 second enable, 60 second disable) | Y | N |

### CATR – Catalyst Diagnostic Service Bay Test Trigger Device Control

This device control is used to activate the catalyst diagnostic service bay test.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Catalyst Diagnostic Service Bay Test Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Catalyst diagnostic service bay test trigger is True)

If (Catalyst diagnostic service bay test is complete)

Return to normal control.

Else

Catalyst diagnostic service bay test shall be triggered.

EndIf

Else

Return to normal control.

EndIf

## CPID $1D

| CPID $1D |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Exhaust Cam Phase Bank 1 Enable | Exhaust Cam Phase Bank 2 Enable | Intake Cam Phase Bank 1 Enable | Intake Cam Phase Bank 2 Enable |  |  |  |  |
| Byte 1 | Exhaust Cam Phase Angle Bank 1 |  |  |  |  |  |  |  |
| Byte 2 | Exhaust Cam Phase Angle Bank 2 |  |  |  |  |  |  |  |
| Byte 3 | Intake Cam Phase Angle Bank 1 |  |  |  |  |  |  |  |
| Byte 4 | Intake Cam Phase Angle Bank 2 |  |  |  |  |  |  |  |

### PHSR – Exhaust and Intake Cam Phase Device Controls

These device controls are used to override the states of the exhaust and intake cam phase outputs for banks 1 and 2.  The commanded camshaft positions are expressed in terms of degrees of camshaft rotation (retard for exhaust camshafts, advance for intake camshafts) from the park position (a camshaft position of zero represents the park position).

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Exhaust Cam Phase Bank 1 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Cam Phase Angle Bank 1 | 8 | UNM | 0 - 51 ° | E = N * 0.2 |
| Exhaust Cam Phase Bank 2 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Exhaust Cam Phase Angle Bank 2 | 8 | UNM | 0 - 51 ° | E = N * 0.2 |
| Intake Cam Phase Bank 1 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Intake Cam Phase Angle Bank 1 | 8 | UNM | 0 - 51 ° | E = N * 0.2 |
| Intake Cam Phase Bank 2 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Intake Cam Phase Angle Bank 2 | 8 | UNM | 0 - 51 ° | E = N * 0.2 |

If (Exhaust/Intake Cam Phase Bank N Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The exhaust/intake cam phase output for bank N shall be commanded to the value requested in Exhaust/Intake Cam Phase Angle Bank N.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0001 | Device control timer time-out active (30 second enable, 60 second disable). | Y | N |
| $0004 | Invalid device control selected (vehicle not equipped with camshaft position actuators). | Y | N |
| $0009 | Commanded camshaft position out of range. | Y | N |
| $0010 | Engine speed out of range. | Y | N |
| $0308 | Fault active (engine position sensing system fault or camshaft position to crankshaft position correlation fault). | Y | N |
| $030C | Engine oil temperature out of range. | Y | N |

## CPID $1E

| CPID $1E |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Oxygen Sensor Heater Bank 1 Sensor 1 Enable | Oxygen Sensor Heater Bank 2 Sensor 1 Enable | Oxygen Sensor Heater Bank 1 Sensor 2 Enable | Oxygen Sensor Heater Bank 2 Sensor 2 Enable | Oxygen Sensor Heater Resistance Reset Trigger |  |  |  |
| Byte 1 | Oxygen Sensor Heater Bank 1 Sensor 1 Duty Cycle |  |  |  |  |  |  |  |
| Byte 2 | Oxygen Sensor Heater Bank 2 Sensor 1 Duty Cycle |  |  |  |  |  |  |  |
| Byte 3 | Oxygen Sensor Heater Bank 1 Sensor 2 Duty Cycle |  |  |  |  |  |  |  |
| Byte 4 | Oxygen Sensor Heater Bank 2 Sensor 2 Duty Cycle |  |  |  |  |  |  |  |

### OXYR – Oxygen Sensor Heater Resistance Reset Trigger Device Control

This device control is used to trigger the reset of the learned resistances for the oxygen sensor heaters following a replacement in service.  The learned resistances are reset to a calibration used to prevent overheating of the sensors until more accurate resistances may be learned following an extended engine off time (typically 12 or more hours).

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Oxygen Sensor Heater Resistance Reset Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Oxygen Sensor Heater Resistance Reset Trigger is True)

The reset of the learned resistances for all oxygen sensor heaters shall be triggered.

EndIf

### OXYR – Oxygen Sensor Heaters Device Control

This device control is used to override the duty cycle of the oxygen sensor heaters.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Oxygen Sensor Heater Bank 1 Sensor 1 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Heater Bank 1 Sensor 1 Duty Cycle | 8 | UNM | 0.0 - 99.609375 % | E = N * 0.390625 |
| Oxygen Sensor Heater Bank 2 Sensor 1 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Heater Bank 2 Sensor 1 Duty Cycle | 8 | UNM | 0.0 - 99.609375 % | E = N * 0.390625 |
| Oxygen Sensor Heater Bank 1 Sensor 2 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Heater Bank 1 Sensor 2 Duty Cycle | 8 | UNM | 0.0 - 99.609375 % | E = N * 0.390625 |
| Oxygen Sensor Heater Bank 2 Sensor 2 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Oxygen Sensor Heater Bank 2 Sensor 2 Duty Cycle | 8 | UNM | 0.0 - 99.609375 % | E = N * 0.390625 |

#### Oxygen Sensor Heater Device Control with Engine Not Running

With the engine off, this device control is intended to be used as a discrete control for circuit diagnosis. With the engine off, none of the oxygen sensor heaters can be commanded on for more than 3.0 seconds, after which the selected oxygen sensor heater must be disabled for a limited duration (typically 60 seconds) before it may be enabled again.  Device control limit exceeded return code $0008 is reported when the device control is first disabled and if requests to enable the device control occur during the limited duration that it is disabled.

For all applicable oxygen sensor heaters:

If (Oxygen Sensor Heater Bank N Sensor Y Enable is True)

If   (the engine is running)

Manage the request to control the Oxygen Sensor Heaters as described in Oxygen Sensor Heater Device Control with Engine Running Below

Else

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The oxygen sensor heater output for bank N sensor Y shall be commanded to the value requested in Oxygen Sensor Heater Bank N Sensor Y Duty Cycle. (Typical values will be 100% for on and 0% for off)

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0307 | Intake Air Temperature out of range | Y | N |
| $1805 | Ignition Voltage too high during O2 Heater device control. | Y | N |
| $0008 | The Output Control timer has expired. Please Wait. - The selected oxygen sensor heater must not be commanded on (Duty Cycle > 0.0) continuously for more than 3.0 seconds with engine not running. | Y | Y |

#### Oxygen Sensor Heater Device Control with Engine Running

With the engine running, this device control is used to operate the oxygen sensor heaters at the various duty cycles available to the engine controller.

For all applicable oxygen sensor heaters:

If (Oxygen Sensor Heater Bank N Sensor Y Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The oxygen sensor heater output for bank N sensor Y shall be commanded to the value requested in Oxygen Sensor Heater Bank N Sensor Y Duty Cycle.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $1802 | O2 Heater: O2 Heaters are over temperature | Y | N |
| $1805 | Ignition Voltage too high during O2 Heater device control. | Y | N |

## CPID $20

| CPID $20 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Malfunction Indicator Lamp Enable | Malfunction Indicator Lamp On | Air Conditioning Compressor Enable | Air Conditioning Compressor On | Generator L-Terminal Enable | Generator L-Terminal On | Powertrain Relay Enable | Powertrain Relay On |
| Byte 1 | Cooling Fan Speed Enable | Class 2 Learned Source Identifiers Reset Trigger | Reverse Inhibit Enable | Reverse Inhibit On | Skip Shift Enable | Skip Shift On | Vehicle Lift/Dive Enable | Vehicle Lift/Dive On |
| Byte 2 | Cooling Fan Speed |  |  |  |  |  |  |  |
| Byte 3 | Cooling Fan Relay Enable | Cooling Fan Relay 1 On | Cooling Fan Relay 2 On | Cooling Fan Relay 3 On | Starter Relay Enable | Starter Relay On |  | Remote Vehicle Start Disable History Reset Trigger |
| Byte 4 |  |  |  |  |  |  |  | Clutch Pedal Fully Applied Position Learn Trigger |

### MTCR – Clutch Pedal Fully Applied Position Learn Trigger Device Control

This device control is used to trigger the procedure to learn the clutch pedal fully applied position.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Clutch Pedal Fully Applied Position Learn Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Clutch Pedal Fully Applied Position Learn Trigger is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The procedure to learn the clutch pedal fully applied position shall be triggered.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0201 | Engine speed not zero. | Y | N |
| $0308 | Fault active. - Clutch pedal position sensor fault active. - Buffered 5V supply fault active. | Y | Y |
| $0309 | Learn procedure already performed this key cycle. | Y | N |
| $0401 | Vehicle speed not zero. | Y | N |

### ACCR – Air Conditioning Compressor Device Control

This device control is used to override the state of the air conditioning compressor output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Air Conditioning Compressor Enable | 1 | BLN | N/A | $1=True; $0=False |
| Air Conditioning Compressor On | 1 | BLN | N/A | $1=True; $0=False |

If (Air Conditioning Compressor Enable is True)

If (Air Conditioning Compressor On is False)

The air conditioning compressor output shall be commanded off.

Else

If (the engine is not running)

The air conditioning compressor output shall be commanded on.

Else

If (the air conditioning compressor output has been commanded on for less than 5 seconds)

If (the device control limits associated with return codes of $0A05 or $0A06 are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The air conditioning compressor output shall be commanded on.

EndIf

Else

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The air conditioning compressor output shall be commanded on.

EndIf

EndIf

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0A03 | Air conditioning refrigerant high side fluid pressure out of range. | Y | N |
| $0A04 | Air conditioning compressor commanded off by platform. | Y | N |
| $0A05 | Engine speed out of range. | Y | N |
| $0A06 | Ignition voltage out of range. | Y | N |
| $0A07 | Air conditioning refrigerant high side fluid pressure fault. | Y | N |

### FANR – Cooling Fan Speed Device Control

This device control is used to override the state of the cooling fan control output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Cooling Fan Speed Enable | 1 | BLN | N/A | $1=True; $0=False |
| Cooling Fan Speed | 8 | UNM | 0.0 - 99.609375 % | E = N * 0.390625 |

If (Cooling Fan Speed Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The cooling fan speed shall be commanded to the value requested in Cooling Fan Speed.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0601 | Engine coolant temperature too high. | Y | N |
| $031A | Commanded fan speed too low. | Y | N |

### GENR – Generator L-Terminal Device Control

This device control is used to override the state of the generator L-terminal output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Generator L-Terminal Enable | 1 | BLN | N/A | $1=True; $0=False |
| Generator L-Terminal On | 1 | BLN | N/A | $1=True; $0=False |

If (Generator L-Terminal Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The generator L-terminal output shall be commanded to the state requested in Generator L-Terminal On.

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0008 | The Output Control Timer Has Expired. | Y | N |
| $0E01 | Engine is not running - must be running to turn off L-terminal. | Y | N |
| $0E02 | Vehicle speed is not zero - vehicle must not be moving. | Y | N |
| $0E03 | Transmission range is not in Park or Neutral. | Y | N |

### RVSR – Remote Vehicle Start Disable History Reset Trigger Device Control

This device control is used to trigger the reset of the last eight reasons that powertrain has disabled remote vehicle start (RVS).

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Remote Vehicle Start Disable History Reset Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Remote Vehicle Start Disable History Reset Trigger is True)

The reset of the last eight reasons that powertrain has disabled RVS shall be triggered.

EndIf

### PMDR – Powertrain Relay Device Control

This device control is used to override the state of the Powertrain Relay.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Powertrain Relay Enable | 1 | BLN | N/A | $1=True; $0=False |
| Powertrain Relay On | 1 | BLN | N/A | $1=True; $0=False |

If (Powertrain Relay Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The Powertrain Relay shall be commanded to the state requested in Powertrain Relay On

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0201 | Engine is rotating | Y | N |
| $1600 | Engine rotating data is not valid | Y | N |
| $0401 | Vehicle speed is not equal to zero | Y | N |
| $0402 | Vehicle speed data is not valid | Y | N |

### RINR – Reverse Inhibit Device Control

This device control is used to override the state of the Reverse Inhibit output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Reverse Inhibit Enable | 1 | BLN | N/A | $1=True; $0=False |
| Reverse Inhibit On | 1 | BLN | N/A | $1=True; $0=False |

If (the vehicle is equipped with a reverse enable solenoid (reverse is inhibited with solenoid commanded off))

Manage the request to inhibit reverse as described in Section 5.8.7.1.

Else (for reverse inhibit applications (reverse is inhibited with solenoid commanded on))

Manage the request to inhibit reverse as described in Section 5.8.7.2.

EndIf

#### – Reverse Inhibit for Reverse Enable Solenoid Applications

If (Reverse Inhibit Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The reverse inhibit output shall be commanded to the state requested in Reverse Inhibit On

If Reverse Inhibit On = True

De-energize Reverse Inhibit Output (command solenoid off to inhibit reverse).

Else

Energize Reverse Inhibit Output (command solenoid on to allow reverse)

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0004 | Invalid device control selected. | Y | Y |
| $0401 | Vehicle speed is not equal to zero | Y | N |
| $0402 | Vehicle speed data is not valid | Y | N |

#### – Reverse Inhibit for Reverse Inhibit Solenoid Applications

If (Reverse Inhibit Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The reverse inhibit output shall be commanded to the state requested in Reverse Inhibit On

If Reverse Inhibit On = True

Energize Reverse Inhibit Output (command solenoid on to inhibit reverse).

Else

De-energize Reverse Inhibit Output (command solenoid off to allow reverse)

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0004 | Invalid device control selected. | Y | Y |
| $0401 | Vehicle speed is not equal to zero | Y | N |
| $0402 | Vehicle speed data is not valid | Y | N |

### RTDR – Vehicle Lift/Dive Device Control

This device control is used to override the state of the Vehicle Lift/Dive output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Vehicle Lift/Dive Enable | 1 | BLN | N/A | $1=True; $0=False |
| Vehicle Lift/Dive On | 1 | BLN | N/A | $1=True; $0=False |

If (Vehicle Lift/Dive Enable is True)

The vehicle lift/dive output shall be commanded to the state requested in Vehicle Lift/Dive On

Else

Return to normal control.

EndIf

### SKSR – Skip Shift Device Control

This device control is used to override the state of the Skip Shift output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Skip Shift Enable | 1 | BLN | N/A | $1=True; $0=False |
| Skip Shift On | 1 | BLN | N/A | $1=True; $0=False |

If (Skip Shift Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The skip shift output shall be commanded to the state requested in Skip Shift On

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0001 | Generic Device Control Timer (30 second enable, 60 second disable) | Y | N |

### STRR – Starter Relay Device Control

This device control is used to override the state of the Starter Relay output.  Fuel and spark are disabled to prevent the engine from starting if the starter relay output is commanded on.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Starter Relay Enable | 1 | BLN | N/A | $1=True; $0=False |
| Starter Relay On | 1 | BLN | N/A | $1=True; $0=False |

If (Starter Relay Enable is True)

If (starter relay output is currently commanded off)

If (Starter Relay On is True)

If (Limits $1600, $0201, $0206, $2B00, $0903, $2C07, $0402, $0401, $0405, $1603, and $0008 in the following table are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The starter relay output shall be commanded on

EndIf

Else

The starter relay output shall be commanded off

EndIf

Else

If (Starter Relay On is True)

If (Limits $2B00, $0903, $2C07, $0207, $0402, $0401, $0405, $1603, and $0008 in the following table are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The starter relay output shall be commanded on

(fuel and spark control are also disabled to prevent the engine from starting)

EndIf

Else

The starter relay output shall be commanded off

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0201 | Engine is rotating | Y | N |
| $1600 | Engine rotating data is not valid | Y | N |
| $0206 | Theft system inhibiting starter enable | Y | N |
| $2B00 | Transmission gear data is not valid (automatic or manual transmission) | Y | N |
| $0903 | Transmission is in gear (automatic transmission) | Y | N |
| $2C07 | Transmission is in gear (manual transmission) | Y | N |
| $0405 | Run/Crank voltage is too low | Y | N |
| $1603 | ETC system inhibiting starter enable | Y | N |
| $0008 | Not enough time has elapsed since the last time the starter has been enabled via device control | Y | N |
| $0207 | No engine rotation has been detected after starter has been enabled | Y | N |
| $0401 | Vehicle speed is not equal to zero | Y | N |
| $0402 | Vehicle speed data is not valid | Y | N |

### DFIR – Malfunction Indicator Lamp Device Control

This device control is used to either turn the Malfunction Indicator Lamp (MIL) on or off.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Malfunction Indicator Lamp Enable | 1 | BLN | N/A | $1=True; $0=False |
| Malfunction Indicator Lamp On | 1 | BLN | N/A | $1=True; $0=False |

If (Malfunction Indicator Lamp Enable is True)

If (Malfunction Indicator Lamp On is False)

The Malfunction Indicator Lamp shall be commanded off

Else

The Malfunction Indicator Lamp shall be commanded on

EndIf

Else

Return to normal control.

EndIf

### CL2R – Class 2 Learned Source Identifiers Reset Trigger Device Control

This device control is used to reset the Class 2 Message Handler "learned" source ID's if maintained in non-volatile memory (memory which may be retained when battery voltage is removed from the powertrain controller).  This function is used when transferring a powertrain controller between vehicles that have different platform option content.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Class 2 Learned Source Identifiers Reset Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Class 2 Learned Source Identifiers Reset Trigger is True)

If (the source identifiers are reset)

Return to normal control.

Else

Reset the source identifiers

EndIf

Else

Return to normal control.

EndIf

### FANR – Cooling Fan Relay Driver Device Control

This device control is used to override the state of the Discrete Cooling Fan Relay outputs.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Cooling Fan Relay Enable | 1 | BLN | N/A | $1=True; $0=False |
| Cooling Fan Relay 1 On | 1 | BLN | N/A | $1=True; $0=False |
| Cooling Fan Relay 2 On | 1 | BLN | N/A | $1=True; $0=False |
| Cooling Fan Relay 3 On | 1 | BLN | N/A | $1=True; $0=False |

If (Cooling Fan Relay Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The cooling fan relay 1 output shall be commanded to the state requested in Cooling Fan Relay 1 On

The cooling fan relay 2 output shall be commanded to the state requested in Cooling Fan Relay 2 On

The cooling fan relay 3 output shall be commanded to the state requested in Cooling Fan Relay 3 On

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0601 | Engine coolant temperature too high. | Y | N |

## CPID $28

| CPID $28 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Ignition Off Controller Shutdown Trigger | Immediate Controller Shutdown Trigger |  |  |  |  |  |  |
| Byte 1 |  |  |  |  |  |  |  |  |
| Byte 2 |  |  |  |  |  |  |  |  |
| Byte 3 |  |  |  |  |  |  |  |  |
| Byte 4 |  |  |  |  |  |  |  |  |

### RBSR – Ignition Off Controller Shutdown Trigger Device Control

This device control is used to trigger a controller shutdown once the Run/Crank input is low in order to force a write of the RAM copy of NVM parameters to flash memory.  When the Run/Crank input is low, the ignition key has transitioned out of the Run position and into either the Accessory or Off position.  This device control will be sent at the successful completion of a DVT test in the assembly plant.  If power is being supplied to the controller (i.e., if the Accessory input is high), the controller will reset immediately following the shutdown sequence.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Ignition Off Controller Shutdown Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Ignition Off Controller Shutdown Trigger is True)

Set an indication that a controller shutdown should be performed (once the Run/Crank input is low).

EndIf

If (an indication that a controller shutdown should be performed has been set) AND (the Run/Crank input is low)

Trigger controller shutdown.

EndIf

### RBSR – Immediate Controller Shutdown Trigger Device Control

This device control is used to trigger a controller shutdown immediately in order to force a write of the RAM copy of NVM parameters to flash memory.  This device control will be sent following a reprogramming event in the assembly plant.  If the controller is an ECM, the immediate controller shutdown will not be allowed if the engine is rotating.  If power is being supplied to the controller (i.e., if the Run/Crank or Accessory inputs are high), the controller will reset immediately following the shutdown sequence.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Immediate Controller Shutdown Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Immediate Controller Shutdown Trigger is True)

If (the controller is an ECM) AND (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Else

Trigger controller shutdown.

EndIf

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0201 | Engine speed not zero. | Y | Y |

## CPID $30

| CPID $30 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Transmission Commanded Gear Enable | Transmission Commanded Gear |  |  | Transmission Line Pressure Current Enable | Torque Converter Clutch System Enable | Torque Converter Clutch Control On | Torque Converter Clutch Enable On |
| Byte 1 | Transmission Line Pressure Current |  |  |  |  |  |  |  |
| Byte 2 | Transmission Adapts Reset Trigger | Transmission Adapts Preset Trigger | Transmission 32 Downshift Enable | Transmission 32 Downshift On | Transmission High Side Driver 1 Enable | Transmission High Side Driver 1 On | Transmission High Side Driver 2 Enable | Transmission High Side Driver 2 On |
| Byte 3 | Powertrain Braking Enable | Powertrain Braking On | All Wheel Drive Solenoid Enable | All Wheel Drive Solenoid On | Transmission Fast Adapts Reset Trigger | Transmission Fast Learn Trigger | Transmission Cleaning Procedure Trigger |  |
| Byte 4 |  |  |  |  |  |  |  |  |

### D32R – Transmission 32 Downshift Device Control

This device control is used to override the state of the Transmission 3-2 Downshift output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission 32 Downshift Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission 32 Downshift On | 1 | BLN | N/A | $1=True; $0=False |

If (Transmission 32 Downshift Enable is True)

If (engine is not running)

The Transmission 3-2 Downshift output shall be commanded to the state requested in Transmission 32 Downshift On

Else

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The Transmission 3-2 Downshift output shall be commanded to the state requested in Transmission 32 Downshift On

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2B01 | Transmission is not in park or neutral | Y | N |

### HSDR – Transmission High Side Driver 1 Device Control

This device control is used to override the state of the Transmission High Side Driver 1 output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission High Side Driver 1 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission High Side Driver 1 On | 1 | BLN | N/A | $1=True; $0=False |

If (Transmission High Side Driver 1 Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission high side driver 1 output shall be commanded to the state requested in Transmission High Side Driver 1 On

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2D01 | Engine is running | Y | N |

### HSDR – Transmission High Side Driver 2 Device Control

This device control is used to override the state of the Transmission High Side Driver 2 output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission High Side Driver 2 Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission High Side Driver 2 On | 1 | BLN | N/A | $1=True; $0=False |

If (Transmission High Side Driver 2 Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission high side driver 2 output shall be commanded to the state requested in Transmission High Side Driver 2 On

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2D01 | Engine is running | Y | N |

### PTBR – Powertrain Braking Device Control

This device control is used to override the state of the Powertrain Braking output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Powertrain Braking Enable | 1 | BLN | N/A | $1=True; $0=False |
| Powertrain Braking On | 1 | BLN | N/A | $1=True; $0=False |

If (Powertrain Braking Enable is True)

If (engine is not running)

The powertrain braking output shall be commanded to the state requested in Powertrain Braking On

Else

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The powertrain braking output shall be commanded to the state requested in Powertrain Braking On

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2802 | Engine speed is too high | Y | N |

### TCCR/RSCR – Torque Converter Clutch System Device Control

This device control is used to either turn the Torque Converter Clutch (TCC) Enable and/or TCC Control solenoids on or off.  For Freewheel applications use TCCR ring and for Clutch-to-Clutch applications, use RSCR ring.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Torque Converter Clutch System Enable | 1 | BLN | N/A | $1=True; $0=False |
| Torque Converter Clutch Enable On | 1 | BLN | N/A | $1=True; $0=False |
| Torque Converter Clutch Control On | 1 | BLN | N/A | $1=True; $0=False |

If (Torque Converter Clutch System Enable is True)

If (engine is not running)

The TCC enable and TCC control solenoid shall be commanded to the states requested by Torque Converter Clutch Enable On and Torque Converter Clutch Control On

Else

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The TCC enable and TCC control solenoid shall be commanded to the states requested by Torque Converter Clutch Enable On and Torque Converter Clutch Control On

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2903 | When the enable solenoid is commanded ON, the control solenoid must be set to maximum capacity | Y | N |
| $2904 | TCC system may not be commanded off while in Hot Mode | Y | N |
| $2905 | TCC system may not be commanded off for more than 5 minutes | Y | N |
| $4002 | Engine speed Low. | Y | N |
| $3D02 | Vehicle speed too low. | Y | N |

### TPCR – Transmission Line Pressure Current Device Control

This device control is used to override the current to the line pressure solenoid.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Line Pressure Current Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Line Pressure Current | 8 | UNM | 0.0 - 1.2451171875 A | E = N * 0.0048828125 |

If (Transmission Line Pressure Current Enable is True)

If (engine is not running)

If (limit $2A01 in the table below is exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission line pressure current shall be commanded to the current requested in Transmission Line Pressure Current.

EndIf

Else

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission line pressure current shall be commanded to the current requested in Transmission Line Pressure Current.

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2A01 | Requested current is not in allowable range | Y | N |
| $2A02 | Transmission fault is present | Y | N |
| $2A03 | Requested current is too high when in gear | Y | N |
| $2A04 | Engine speed too high (> 1500 rpm) when in park or neutral | Y | N |

### AWDR – All Wheel Drive Solenoid Device Control

This device control is used to override the state of the All Wheel Drive solenoid.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| All Wheel Drive Solenoid Enable | 1 | BLN | N/A | $1=True; $0=False |
| All Wheel Drive Solenoid On | 1 | BLN | N/A | $1=True; $0=False |

If (All Wheel Drive Solenoid Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The All Wheel Drive solenoid shall be commanded to the state requested in All Wheel Drive Solenoid On

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2A02 | Invalid transmission gear (actual gear not equal to commanded gear) | Y | N |
| $2B01 | Transmission is not in park or neutral | Y | N |

### SHPR – Transmission Commanded Gear Device Control

This device control is used to override the state of the transmission commanded gear.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Commanded Gear Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Commanded Gear | 3 | ENM | N/A | $1=First Gear $2=Second Gear $3=Third Gear $4=Fourth Gear $5=Fifth Gear $6=Sixth Gear |

If (Transmission Commanded Gear Enable is True)

If (engine is not running)

The transmission commanded gear shall be set to the gear requested in Transmission Commanded Gear

Else

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission commanded gear shall be set to the gear requested in Transmission Commanded Gear

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2801 | Vehicle speed is too high | Y | N |
| $2802 | Engine speed is too high | Y | N |
| $2803 | A non-sequential gear change is requested | Y | N |
| $2804 | Vehicle speed is too high for a the downshift requested | Y | N |
| $2805 | Requested gear is greater than the selected PRNDL range allows | Y | N |
| $3D02 | Vehicle speed is too low | Y | N |

### RSCR / SPRR - Transmission Adapts Preset Trigger Device Control

This device control is used to trigger the transmission adaptive parameters to be preset to their initial values.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Adapts Preset Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Transmission Adapts Preset Trigger is True)

If (the preset is complete)

Return to normal control.

Else

Perform the preset of the Transmission Adaptive parameters.

EndIf

Else

Return to normal control.

EndIf

### RSSR - Transmission Fast Adapts Reset Trigger Device Control

This device control is used to trigger the reset of the transmission fast adaptive parameters.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Fast Adapts Reset Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Transmission Fast Adapts Reset Trigger is True)

If (the reset is complete)

Return to normal control.

Else

Perform the reset of the Transmission Fast Adaptive parameters.

EndIf

Else

Return to normal control.

EndIf

### TRNR - Transmission Adapts Reset Trigger Device Control

This device control is used to trigger the reset of the transmission adaptive parameters.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Adapts Reset Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Transmission Adapts Reset Trigger is True)

If (the reset is complete)

Return to normal control.

Else

Perform the reset of the Transmission Adaptive parameters.

EndIf

Else

Return to normal control.

EndIf

### RSCR – Transmission Fast Learn Trigger

This device control is used to trigger the transmission fast learn algorithm.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Fast Learn Trigger | 1 | BLN | N/A | $1=True; $0=False |

#### Transmission Fast Learn Trigger for Service

If   (Transmission Fast Learn Trigger is True)

If   (the vehicle is in the vehicle assembly plant)

Manage the request to trigger the Transmission Fast Learn as described in Section 5.10.12.2

Else

If   (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The Service transmission fast learn algorithm shall be enabled

Endif

Endif

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0604 | Engine not running. | Y | N |
| $2A02 | Transmission fault present. - Service Brake Faults of any kind - Engine Speed Faults - Transmission Range fault - Output speed fault - Input speed fault - Transmission temperature fault - Throttle position fault | Y | N |

#### Transmission Fast Learn Trigger for Assembly Plant

If   (Transmission Fast Learn Trigger is True)

If   (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The Assembly Plant transmission fast learn algorithm shall be enabled

Endif

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0604 | Engine not running. | Y | N |
| $2A02 | Transmission fault present. - Engine Speed Faults - Transmission Range fault - Output speed fault - Input speed fault - Transmission temperature fault - Throttle position fault | Y | N |

### RSCR – Transmission Cleaning Procedure Trigger

This device control is used to trigger the transmission cleaning service procedure algorithm.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Cleaning Procedure Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Transmission Cleaning Procedure Trigger is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

If (the Service transmission cleaning procedure algorithm has run and is complete)

Then

Return to normal control.

Else

The Service transmission cleaning procedure algorithm shall be enabled

Endif

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $0604 | Engine not running. | Y | N |
| $0E03 | Transmission not in park or neutral. | Y | N |
| $2801 | Vehicle speed too high. | Y | N |
| $2802 | Engine speed too high. | Y | N |
| $2E06 | Transmission oil temperature out of range | Y | N |
| $2A02 | Transmission fault present. - Engine Speed Faults - Transmission Range fault - Output speed fault - Transmission temperature fault - Throttle position fault | Y | N |

## CPID $34

| CPID $34 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Transmission Clutch 1 Solenoid Enable | Transmission Clutch 1 Solenoid On | Transmission Clutch 2 Solenoid Enable | Transmission Clutch 2 Solenoid On | Transmission Clutch 3 Solenoid Enable | Transmission Clutch 3 Solenoid On | Transmission Clutch 4 Solenoid Enable | Transmission Clutch 4 Solenoid On |
| Byte 1 | Torque Converter Clutch Solenoid Enable | Torque Converter Clutch Solenoid On | Transmission M2 Clutch / Reverse Multiplex Solenoid Enable | Transmission M2 Clutch / Reverse Multiplex Solenoid On | Transmission M3 Clutch Multiplex Solenoid Enable | Transmission M3 Clutch Multiplex Solenoid On |  |  |
| Byte 2 |  |  |  |  |  |  |  |  |
| Byte 3 |  |  |  |  |  |  |  |  |
| Byte 4 |  |  |  |  |  |  |  |  |

### RSCR – Transmission Clutch Solenoids Device Control

This device control is used to override the state of the transmission clutch solenoids.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Clutch 1 Solenoid Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 1 Solenoid On | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 2 Solenoid Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 2 Solenoid On | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 3 Solenoid Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 3 Solenoid On | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 4 Solenoid Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 4 Solenoid On | 1 | BLN | N/A | $1=True; $0=False |

For Transmission Clutch 1 to N:

If (Transmission Clutch N Solenoid Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission clutch N Solenoid state shall be commanded to the state requested in Transmission Clutch N Solenoid On

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2A02 | Transmission fault is present. | Y | N |
| $2B01 | Transmission not in park or neutral. | Y | N |

### RSCR – Torque Converter Clutch Solenoid Device Control

This device control is used to override the state of the Torque Converter Clutch solenoid.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Torque Converter Clutch Solenoid Enable | 1 | BLN | N/A | $1=True; $0=False |
| Torque Converter Clutch Solenoid On | 1 | BLN | N/A | $1=True; $0=False |

If (Torque Converter Clutch Solenoid Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The torque converter clutch solenoid shall be commanded to the state requested in Torque Converter Clutch Solenoid On

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2A02 | Transmission fault is present. | Y | N |
| $2B01 | Transmission not in park or neutral. | Y | N |

### RSCR – Transmission M3 Clutch Multiplex Solenoid Device Control

This device control is used to override the state of the Transmission M3 Clutch Multiplex solenoid.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission M3 Clutch Multiplex Solenoid Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission M3 Clutch Multiplex Solenoid On | 1 | BLN | N/A | $1=True; $0=False |

If (Transmission M3 Clutch Multiplex Solenoid Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission clutch multiplex solenoid shall be commanded to the state requested in Transmission  M3 Clutch Multiplex Solenoid On

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2A02 | Transmission fault is present. | Y | N |
| $2B01 | Transmission not in park or neutral. | Y | N |

### RSCR – Transmission M2 Clutch / Reverse Multiplex Solenoid Device Control

This device control is used to override the state of the Transmission M2 Clutch / Reverse Multiplex solenoid.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission M2 Clutch / Reverse Multiplex Solenoid Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission M2 Clutch / Reverse Multiplex Solenoid On | 1 | BLN | N/A | $1=True; $0=False |

If (Transmission M2 Clutch / Reverse Multiplex Solenoid Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The Transmission M2 clutch / reverse multiplex solenoid shall be commanded to the state requested in Transmission M2 Clutch / Reverse Multiplex Solenoid On

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2A02 | Transmission fault is present. | Y | N |
| $2B01 | Transmission not in park or neutral. | Y | N |

## CPID $38

| CPID $38 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Torque Converter Clutch Pressure Enable | Transmission Line Pressure Enable |  |  |  |  |  |  |
| Byte 1 | Torque Converter Clutch Pressure (upper byte) |  |  |  |  |  |  |  |
| Byte 2 | Torque Converter Clutch Pressure (lower byte) |  |  |  |  |  |  |  |
| Byte 3 | Transmission Line Pressure (upper byte) |  |  |  |  |  |  |  |
| Byte 4 | Transmission Line Pressure (lower byte) |  |  |  |  |  |  |  |

### RSCR – Torque Converter Clutch / Pressure Device Control

This device control is used to override the pressure of the Torque Converter Clutch output.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Torque Converter Clutch Pressure Enable | 1 | BLN | N/A | $1=True; $0=False |
| Torque Converter Clutch Pressure | 16 | UNM | 0.0 to 4095.9375 kPa | E = N * 0.0625 |

If (Torque Converter Clutch Pressure Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The torque converter clutch Pressure shall be commanded to the pressure requested in Torque Converter Clutch Pressure

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2A02 | Transmission fault is present. | Y | N |
| $2B01 | Transmission not in park or neutral. | Y | N |

### TPCR – Transmission Line Pressure Device Control

This device control is used to override the pressure of the line pressure solenoid.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Line Pressure Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Line Pressure | 16 | UNM | 0.0 to 4095.9375 kPa | E = N * 0.0625 |

If (Transmission Line Pressure Enable is True)

If (engine is not running)

If (limit $2A01 in the table below is exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission line pressure shall be commanded to the pressure requested in Transmission Line Pressure.

EndIf

Else

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission line pressure shall be commanded to the pressure requested in Transmission Line Pressure.

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2A01 | Requested current is not in allowable range | Y | N |
| $2A02 | Transmission fault is present | Y | N |
| $2A03 | Requested current is too high when in gear | Y | N |
| $2A04 | Engine speed too high (> 1500 rpm) when in park or neutral | Y | N |

## CPID $3A

| CPID $3A |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Transmission Clutch 1 Pressure Enable | Transmission Clutch 2 Pressure Enable |  |  |  |  |  |  |
| Byte 1 | Transmission Clutch 1 Pressure (upper byte) |  |  |  |  |  |  |  |
| Byte 2 | Transmission Clutch 1 Pressure (lower byte) |  |  |  |  |  |  |  |
| Byte 3 | Transmission Clutch 2 Pressure (upper byte) |  |  |  |  |  |  |  |
| Byte 4 | Transmission Clutch 2 Pressure (lower byte) |  |  |  |  |  |  |  |

### RSCR – Transmission Clutch 1 / 2 Pressure Device Control

This device control is used to override the pressure of the transmission clutch 1 and 2 outputs.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Clutch 1 Pressure Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 1 Pressure | 16 | UNM | 0.0 to 4095.9375 kPa | E = N * 0.0625 |
| Transmission Clutch 2 Pressure Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 2 Pressure | 16 | UNM | 0.0 to 4095.9375 kPa | E = N * 0.0625 |

For transmission clutch 1 and 2:

If (Transmission Clutch 1 / 2 Pressure Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission clutch 1 / 2 pressure shall be commanded to the pressure requested in Transmission Clutch 1 / 2 Pressure

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2A02 | Transmission fault is present. | Y | N |
| $2B01 | Transmission not in park or neutral. | Y | N |

## CPID $3B

| CPID $3B |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Transmission Clutch 3 Pressure Enable | Transmission Clutch 4 Pressure Enable |  |  |  |  |  |  |
| Byte 1 | Transmission Clutch 3 Pressure (upper byte) |  |  |  |  |  |  |  |
| Byte 2 | Transmission Clutch 3 Pressure (lower byte) |  |  |  |  |  |  |  |
| Byte 3 | Transmission Clutch 4 Pressure (upper byte) |  |  |  |  |  |  |  |
| Byte 4 | Transmission Clutch 4 Pressure (lower byte) |  |  |  |  |  |  |  |

### RSCR – Transmission Clutch 3 / 4 Pressure Device Control

This device control is used to override the pressure of the transmission clutch 3 and 4 outputs.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Transmission Clutch 3 Pressure Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 3 Pressure | 16 | UNM | 0.0 to 4095.9375 kPa | E = N * 0.0625 |
| Transmission Clutch 4 Pressure Enable | 1 | BLN | N/A | $1=True; $0=False |
| Transmission Clutch 4 Pressure | 16 | UNM | 0.0 to 4095.9375 kPa | E = N * 0.0625 |

For transmission clutch 3 and 4:

If (Transmission Clutch 3 / 4 Pressure Enable is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

The transmission clutch 3 / 4 pressure shall be commanded to the pressure requested in Transmission Clutch 3 / 4 Pressure

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $2A02 | Transmission fault is present. | Y | N |
| $2B01 | Transmission not in park or neutral. | Y | N |

## CPID $7D

| CPID $7D |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Immobilizer Security Code Input Trigger |  |  |  | Immobilizer Reset Trigger | Immobilizer Deactivation Trigger | Immobilizer Reactivation Trigger | Immobilizer Powertrain Challenge Transmission Trigger |
| Byte 1 | Immobilizer Security Code |  |  |  |  |  |  |  |
| Byte 2 |  |  |  |  |  |  |  |  |
| Byte 3 |  |  |  |  |  |  |  |  |
| Byte 4 |  |  |  |  |  |  |  |  |

### IMOR – Immobilizer Security Code Input Trigger Device Control

This device control is used to verify the internally stored security code for the GMIO immobilizer algorithm.  The security code is entered into the tester when this device control is commanded and is compared to the stored security code.  If the comparison fails, the device control limit exceeded return code $FE05 (Theft Deterrent: Security Code Input Incorrect) will be reported.  Device control limit exceeded return codes will also be reported if the engine is running or the security code function is locked.  The lack of a negative response with a device control limit exceeded return code indicates that the correct security code was entered.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Immobilizer Security Code Input Trigger | 1 | BLN | N/A | $1=True; $0=False |
| Immobilizer Security Code | 32 | UNM | 0 - 4294967295 | E = N |

If (Immobilizer Security Code Input Trigger is True)

Compare Immobilizer Security Code to the stored security code.

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Else

No action (the lack of a negative response with a device control limit exceeded return code indicates that the correct security code was entered).

EndIf

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $FE01 | Theft Deterrent: Engine is Running | Y | Y |
| $FE03 | Theft Deterrent: Security Code Function Locked | Y | Y |
| $FE05 | Theft Deterrent: Security Code Input Incorrect | Y | Y |

### IMOR – Immobilizer Reset Trigger Device Control

This device control is used to allow programming of immobilizer secret information for the GMIO immobilizer algorithm.  The security code is entered into the tester when this device control is commanded and is compared to the stored security code.  If the comparison fails, the device control limit exceeded return code $FE05 (Theft Deterrent: Security Code Input Incorrect) will be reported.  Device control limit exceeded return codes will also be reported if the engine is running or the security code function is locked.  When this device control is accepted, the security code is reset, and learning is enabled for the immobilizer secret information.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Immobilizer Reset Trigger | 1 | BLN | N/A | $1=True; $0=False |
| Immobilizer Security Code | 32 | UNM | 0 - 4294967295 | E = N |

If (Immobilizer Reset Trigger is True)

Compare Immobilizer Security Code to the stored security code.

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Else

Enable learning for the immobilizer secret information.

Reset the stored security code.

EndIf

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $FE01 | Theft Deterrent: Engine is Running | Y | Y |
| $FE03 | Theft Deterrent: Security Code Function Locked | Y | Y |
| $FE05 | Theft Deterrent: Security Code Input Incorrect | Y | Y |

### IMOR – Immobilizer Deactivation Trigger Device Control

This device control is used to deactivate the immobilizer functionality for the GMIO immobilizer algorithm.  The security code is entered into the tester when this device control is commanded and is compared to the stored security code.  If the comparison fails, the device control limit exceeded return code $FE05 (Theft Deterrent: Security Code Input Incorrect) will be reported.  If the immobilizer secret information has been programmed, or the ECM is not identified as a development unit (via an immobilizer calibration set to a “secret value”), the device control limit exceeded return code $FE07 (Theft Deterrent: Immobilizer Function Deactivation Not Allowed via Device Control) will be reported.  Device control limit exceeded return codes will also be reported if the engine is running or the security code function is locked.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Immobilizer Deactivation Trigger | 1 | BLN | N/A | $1=True; $0=False |
| Immobilizer Security Code | 32 | UNM | 0 - 4294967295 | E = N |

If (Immobilizer Deactivation Trigger is True)

Compare Immobilizer Security Code to the stored security code.

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Else

Deactivate immobilizer functionality.

EndIf

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $FE01 | Theft Deterrent: Engine is Running | Y | Y |
| $FE03 | Theft Deterrent: Security Code Function Locked | Y | Y |
| $FE05 | Theft Deterrent: Security Code Input Incorrect | Y | Y |
| $FE07 | Theft Deterrent: Immobilizer Function Deactivation Not Allowed via Device Control | Y | Y |

### IMOR – Immobilizer Reactivation Trigger Device Control

This device control is used to reactivate the immobilizer functionality for the GMIO immobilizer algorithm if the immobilizer functionality was previously deactivated via device control.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Immobilizer Reactivation Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Immobilizer Reactivation Trigger is True)

Reactivate immobilizer functionality.

EndIf

### IMOR – Immobilizer Powertrain Challenge Transmission Trigger Device Control

This device control is used to transmit the current powertrain challenge to the immobilizer module for the GMIO immobilizer algorithm.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Immobilizer Powertrain Challenge Transmission Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Immobilizer Powertrain Challenge Transmission Trigger is True)

Transmit powertrain challenge.

EndIf

## CPID $7E

| CPID $7E |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Immobilizer Security Code Input Trigger | Immobilizer Security Code Programming Trigger |  |  |  |  |  |  |
| Byte 1 | Immobilizer Security Code |  |  |  |  |  |  |  |
| Byte 2 |  |  |  |  |  |  |  |  |
| Byte 3 |  |  |  |  |  |  |  |  |
| Byte 4 |  |  |  |  |  |  |  |  |

### IMOR – Immobilizer Security Code Input Trigger Device Control

This device control is used to trigger the PPEI 3 immobilizer algorithm to transition into the learning enabled state (for NAO applications, the PPEI 3 immobilizer algorithm transitions into the learning enabled state after 10 minutes while, for non-NAO applications, the PPEI 3 immobilizer algorithm transitions into the learning enabled state immediately).  The learning enabled state allows the PPEI 3 immobilizer algorithm to transmit secret information to the immobilizer module and receive secret information from the immobilizer module.  The learning enabled state also allows a new security code to be programmed.

When this device control is commanded, a security code is entered into the tester and compared to the stored security code.  If the comparison fails, the device control limit exceeded return code $FE05 (Theft Deterrent: Security Code Input Incorrect) will be reported.  Device control limit exceeded return codes will also be reported if the controller is locked or the security code function is locked.  When this device control is accepted, the PPEI 3 immobilizer algorithm transitions into the learning enabled state.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Immobilizer Security Code Input Trigger | 1 | BLN | N/A | $1=True; $0=False |
| Immobilizer Security Code | 32 | UNM | 0 - 4294967295 | E = N |

If (Immobilizer Security Code Input Trigger is True)

Compare Immobilizer Security Code to the stored security code.

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Else

Transition the PPEI 3 immobilizer algorithm into the learning enabled state.

EndIf

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $FE02 | Theft Deterrent: Controller is Locked | Y | Y |
| $FE03 | Theft Deterrent: Security Code Function Locked | Y | Y |
| $FE05 | Theft Deterrent: Security Code Input Incorrect | Y | Y |

### IMOR – Immobilizer Security Code Programming Trigger Device Control

This device control is used to program the security code for the PPEI 3 immobilizer algorithm.  The security code is entered into the tester when this device control is commanded.  If the PPEI 3 immobilizer algorithm is not in a learning enabled state, the device control limit exceeded return code $FE04 (Theft Deterrent: Learning Not Enabled) will be reported.  A device control limit exceeded return code will also be reported if security code programming is not supported (for NAO applications, security code programming is not supported while, for non-NAO applications, security code programming is supported).  When this device control is accepted, the security code is programmed.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Immobilizer Security Code Programming Trigger | 1 | BLN | N/A | $1=True; $0=False |
| Immobilizer Security Code | 32 | UNM | 0 - 4294967295 | E = N |

If (Immobilizer Security Code Programming Trigger is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Else

Program the security code.

EndIf

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $FE04 | Theft Deterrent: Learning Not Enabled | Y | Y |
| $FE06 | Theft Deterrent: Security Code Programming Not Enabled | Y | Y |

## CPID $FE

| CPID $FE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| Byte 0 | Vehicle Security Password Learn Trigger |  |  |  |  |  |  |  |
| Byte 1 |  |  |  |  |  |  |  |  |
| Byte 2 |  |  |  |  |  |  |  |  |
| Byte 3 |  |  |  |  |  |  |  |  |
| Byte 4 |  |  |  |  |  |  |  |  |

### VTDR – Vehicle Security Password Learn Trigger Device Control

This device control is used to activate the password learn algorithm to learn a new password from the VTD module.  Needed if installing a different controller into the vehicle.

| Signal | Length (Bits) | Data Type | Range | Conversion |
| --- | --- | --- | --- | --- |
| Vehicle Security Password Learn Trigger | 1 | BLN | N/A | $1=True; $0=False |

If (Vehicle Security Password Learn Trigger is True)

If (any of the device control limits in the table below are exceeded)

Report the appropriate device control limit exceeded return code to the tester.

Return to normal control.

Else

If (VTD password learning is complete)

Return to normal control.

Else

VTD password learning shall be enabled

EndIf

EndIf

Else

Return to normal control.

EndIf

| Return Code | Device Control Limit Exceeded Description | Active for Service | Active for DVT |
| --- | --- | --- | --- |
| $FE01 | The engine is running | Y | Y |
| $FE02 | The controller is locked | Y | Y |

# Diagnostic Trouble Codes (DTCs)

All of the indicators within the DTC status byte (i.e., the value provided for the DTCStatusByte parameter of a positive response to sub-function readStatusOfDTCByNumber ($80) or sub-function readStatusOfDTCByStatusMask ($81) of the ReadDiagnosticInformation ($A9) service) shall be implemented according to Appendix E of [1].  Additional details of the implementation of these indicators are specified below.

The behavior of the indicators within the DTC status byte is controlled by the light enable and fault type calibrations associated with each DTC.  The recommended values for these calibrations for individual DTCs are specified in [6].

The boolean light enable calibration is used to enable the setting of the warningIndicatorRequestedState indicator.  If the light enable calibration for a DTC is true, the associated warningIndicatorRequestedState indicator shall be set upon the reporting of a failure that results in the associated historyDTC indicator being set.  If the light enable calibration for a DTC is false, the associated warningIndicatorRequestedState indicator shall always be cleared.

The fault type calibration may assume the values described in Table 36: Descriptions of DTC Fault Types

.  It is used to enable (type A, B, and C) or disable (type X) DTCs, to differentiate emission related (type A and B) and non emission related (type C) DTCs, and to determine whether to set the historyDTC indicator for a DTC on the first trip (type A and C) or the second consecutive trip (type B) during which a failure is reported.

| Fault Type | Description |
| --- | --- |
| A | Emission related DTC The historyDTC and (if enabled) warningIndicatorRequestedState indicators shall be set on the first trip during which a failure is reported. |
| B | Emission related DTC The historyDTC and (if enabled) warningIndicatorRequestedState indicators shall be set on the second consecutive trip during which a failure is reported. |
| C | Non emission related DTC The historyDTC and (if enabled) warningIndicatorRequestedState indicators shall be set on the first trip during which a failure is reported. |
| X | DTC not supported |

Table 36: Descriptions of DTC Fault Types

For emission related (type A and B) DTCs, the associated warningIndicatorRequestedState indicator shall be cleared after the third consecutive trip during which a pass condition is reported and a failure is not reported.  For non emission related (type C) DTCs, the associated warningIndicatorRequestedState indicator shall be cleared after the first trip during which a pass condition is reported and a failure is not reported.

The behavior of the set of emission related DTCs shown in Table 37: One Trip MIL Off DTCs

, one trip MIL off DTCs, differs slightly from that of other emission related DTCs.  For these DTCs, the associated warningIndicatorRequestedState indicator shall be cleared after the first trip during which a pass condition is reported and a failure is not reported.

| DTC | Description |
| --- | --- |
| P0455 | Evaporative Emission System Large Leak Detected |

Table 37: One Trip MIL Off DTCs

The behavior of the set of type B emission related DTCs shown in Table 38: Special Operating Condition DTCs

, special operating condition DTCs, differs slightly from that of other type B emission related DTCs.  For these DTCs, the associated historyDTC and (if enabled) warningIndicatorRequestedState indicators shall be set on the second, but not necessarily consecutive, trip during which a failure is reported if the operating conditions (engine speed, engine load, and warm up status) under which the failure was reported on the first trip and second trip match.  In addition, the associated warningIndicatorRequestedState indicator shall be cleared after the third consecutive trip during which the operating conditions under which the stored failure was reported are matched, a pass condition is reported, and a failure is not reported.

| DTC | Description |
| --- | --- |
| P0171 | Fuel Trim System Lean Bank 1 |
| P0172 | Fuel Trim System Rich Bank 1 |
| P0174 | Fuel Trim System Lean Bank 2 |
| P0175 | Fuel Trim System Rich Bank 2 |
| P0300 | Engine Misfire Detected |

Table 38: Special Operating Condition DTCs

In addition to the use of the ClearDiagnosticInformation ($04) diagnostic service to reset the indicators within the DTC status byte as described in [1], an automatic code clear function (based upon the completion of a number of warm up cycles without a failure being reported) shall be supported.  An automatic code clear for a DTC shall affect any failure records associated with the DTC and all indicators within the associated DTC status byte in the same manner as the ClearDiagnosticInformation ($04) diagnostic service with the exception of the testNotPassedSinceDTCCleared indicator, which shall be unaffected by an automatic code clear.

The automatic code clear function shall be implemented as two independent functions: one for all emission related (type A and B) DTCs and another for all non emission related (type C) DTCs.  An automatic code clear shall be performed for all emission related DTCs upon the completion of 40 warm up cycles without an emission related failure being reported.  An automatic code clear shall be performed for all non emission related DTCs upon the completion of 40 warm up cycles without a non emission related failure being reported.

# Failure Records / Freeze Frame

- Mode 02 is used to obtain OBD required (legislated) freeze frame data

- OBD required freeze frame data is a subset of the FFRR freeze frame

- Only PIDs $0000 to $00FF can be requested

- Only the ECM  need support Mode 02

- Mode 12 is used to obtain both corporate standard freeze frame data and failure record data

- All PIDs ($0000 to $FFFF) can be delivered

- ECM, TCM, and TCCM can support Mode 12

## Corporate Standard Freeze Frame

Refer to [7] and [8] for the rules for PID selection to create an application specific freeze frame PID List.

# Compliance with Standards

## Compliance with GMPT Product Engineering Standards

This document is in compliance with the following standards.

| Document Number | Version | Document Title |
| --- | --- | --- |
| TL.07.0043 | R08 | Document Format Standard |

## Compliance with Industry Standards

No applicable industry standards were identified for this document.

Appendix A - Revision History

| Revision | Date | Author | Description | EACR Number(s) |
| --- | --- | --- | --- | --- |
| 08.10.31.00 | 04/15/09 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.10.30.00 | 02/27/09 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.10.29.00 | 01/09/09 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.10.28.00 | 12/12/08 | Matt Kaufer | Added support for the following PID: - $0068 – Intake Air Temperature Sensor | E41384-AI01 |
| 08.10.27.00 | 10/17/08 | Matt Kaufer | Added support for the following PID: - $007F – Engine Run Time | E40828-AI01 |
| 08.10.26.00 | 08/22/08 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.10.25.00 | 06/16/08 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.10.24.00 | 05/22/08 | Matt Kaufer | Added support for the following DID: - $29 – Primary Calibration Flash History | E39264-AI01 |
| 08.10.24.00 | 05/22/08 | Matt Kaufer | Updated description for the following PID: - $1414 – Skip Shift Solenoid Control Circuit | E38479-AI02 |
| 08.10.23.00 | 03/31/08 | Matt Kaufer | Added support for the following PIDs: - $209B – Recommended Maximum Ethanol Concentration When Recommended Ethanol Concentration Exceeded - $209C – Currently Recommended Maximum Ethanol Concentration - $209D – Actual Ethanol Composition When Recommended Ethanol Concentration Exceeded - $209E – Ambient Air Temperature When Recommended Ethanol Concentration Exceeded - $209F – Ignition Cycles Since Recommended Ethanol Concentration Exceeded - $20A0 – Distance Since Recommended Ethanol Concentration Exceeded - $20A1 – Fuel Consumed Since Recommended Ethanol Concentration Exceeded - $20A2 – Refueling Events Since Recommended Ethanol Concentration Exceeded - $20A3 – Fuel Composition Sensor Frequency Updated descriptions for the following PIDs: - $0052 – Alcohol Fuel Percentage - $2025 – Fuel Composition Status Removed support for the following PID: - $139C – Fuel Composition Sensor Frequency | E32904-AI03 |
| 08.10.22.00 | 02/22/08 | Matt Kaufer | Added support for the following PIDs: - $20D7 – Economy Mode Switch Status - $20D8 – Economy Mode Switch A/D | E34745-AI17 |
| 08.10.22.00 | 02/22/08 | Matt Kaufer | (Pulled from 08.09.22.00 – 02/22/08) Updated description for the following device control: - CPID $10 - Crankshaft Position Variation Learn Trigger Device Control | E36560-AI01 |
| 08.10.21.00 | 02/19/08 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.10.20.00 | 02/14/08 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.10.19.00 | 11/21/07 | Matt Kaufer | Updated description for the following PID: - $001C – On Board Diagnostic System Type | E34784-AI03 |
| 08.10.19.00 | 11/21/07 | Matt Kaufer | (Pulled from 08.09.19.00 – 11/21/07) Added support for the following PIDs: - $3A00 – Engine Off Natural Vacuum Phase 1 Result Time - $3A01 – Engine Off Natural Vacuum Phase 2 End Time - $3A02 – Engine Off Natural Vacuum EWMA Test Sample Counter - $3A03 – Accumulated Distance Traveled During Previous Key On Cycles - $3A04 – Engine Off Natural Vacuum Corrected Phase 1 End Pressure - $3A05 – Engine Off Natural Vacuum Phase 1 Result Pressure - $3A06 – Engine Off Natural Vacuum Phase 2 Result Vacuum - $3A07 – Engine Running EVAP Diagnostics Fault History | E35976-AI01 |
| 08.10.19.00 | 11/21/07 | Matt Kaufer | (Pulled from 08.09.19.00 – 11/21/07) Updated descriptions for the following PIDs: - $150E – Air Conditioning Disengage History 1 - $150F – Air Conditioning Disengage History 2 - $1510 – Air Conditioning Disengage History 3 - $1511 – Air Conditioning Disengage History 4 - $151F – Air Conditioning Disengage History 5 - $1520 – Air Conditioning Disengage History 6 - $1521 – Air Conditioning Disengage History 7 - $1522 – Air Conditioning Disengage History 8 | E35614-AI01 |
| 08.10.19.00 | 11/21/07 | Matt Kaufer | (Pulled from 08.09.19.00 – 11/21/07) Added support for the following DID: - $28 – Odometer Vehicle Identification Number | E30769-AI04 |
| 08.10.19.00 | 11/21/07 | Matt Kaufer | (Pulled from 08.09.19.00 – 11/21/07) Added support for the following PID: - $2049 – ECU Odometer Status Added support for the following DID: - $DF – ECU Odometer Value | E27009-AI01 |
| 08.10.18.00 | 09/28/07 | Matt Kaufer | Updated description for the following DID: - $90 – Vehicle Identification Number | E34196-AI04 |
| 08.10.18.00 | 09/28/07 | Matt Kaufer | Series 08.10.18.00 document created from 08.09.18.00 document. | N/A |
| 08.09.18.00 | 09/28/07 | Matt Kaufer | Added support for the following PID: - $2861 – Transmission Tap Up/Down Switch 2 Status | E35406-AI01 |
| 08.09.18.00 | 09/28/07 | Matt Kaufer | Added support for the following PID: - $2862 – Calculated Line Pressure | E34141-AI03 |
| 08.09.17.00 | 07/12/07 | Matt Kaufer | (Pulled from 08.08.23.00 – 07/12/07) Updated descriptions for the following service modes: - RequestCurrentPowertrainDiagnosticData ($01) Service - RequestPowertrainFreezeFrameData ($02) Service - RequestEmission-RelatedDiagnosticTroubleCodes ($03) Service - ClearDiagnosticInformation ($04) Service - RequestOnBoardMonitoringTestResults ($06) Service - RequestPendingEmission-RelatedDiagnosticTroubleCodes ($07) Service - RequestVehicleInformation ($09) Service Updated additional sections: - Appendix C - Related Documents - Legislated Freeze Frame - Corporate Standard Freeze Frame - Legislated On-Board Monitor IDs - Legislated On-Board Device Control Test IDs - Legislated Vehicle Information Info Types - Device Control Limit Exceeded Return Codes - Device Control Timer Time-Out | E35237-AI01 |
| 08.09.17.00 | 07/12/07 | Matt Kaufer | (Pulled from 08.08.23.00 – 07/12/07) Updated description for the following device control: - CPID $20 – Clutch Pedal Fully Applied Position Learn Trigger Device Control | E35234-AI01 |
| 08.09.16.00 | 06/14/07 | Matt Kaufer | (Pulled from 08.08.22.00 – 06/14/07) Updated description for the following PID: - $2070 – Reduced Engine Power Indication History | E33371-AI16 |
| 08.09.16.00 | 06/14/07 | Matt Kaufer | (Pulled from 08.08.22.00 – 06/14/07) Added support for the following DID: - $75 – First Time Power Up Flag | E32844-AI01 |
| 08.09.16.00 | 06/14/07 | Matt Kaufer | (Pulled from 08.08.22.00 – 06/14/07) Updated description for the following service mode: - $0A - Request Emission-Related Diagnostic Trouble Codes With Permanent Status Service | E32366-AI11 |
| 08.09.15.00 | 04/12/07 | Matt Kaufer | Updated description for the following PID: - $1133 – Displacement on Demand Status | E33051-AI01 |
| 08.09.15.00 | 04/12/07 | Matt Kaufer | (Pulled from 08.08.21.00 – 04/12/07) Added support for the following service mode: - $0A - Request Emission-Related Diagnostic Trouble Codes With Permanent Status after a Clear/Reset Emission-Related Diagnostic Information Service | E32366-AI09 |
| 08.09.14.00 | 04/03/07 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.09.13.00 | 03/30/07 | Matt Kaufer | (Pulled from 08.08.19.00 – 02/08/07) Added support for the following PID: - $2070 – Reduced Engine Power Indication History | E32253-AI04 |
| 08.09.12.00 | 03/30/07 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.09.11.00 | 02/06/07 | Matt Kaufer | Updated descriptions for the following PIDs: - $1551 – Remote Vehicle Start Disable History 1 - $1552 – Remote Vehicle Start Disable History 2 - $1553 – Remote Vehicle Start Disable History 3 - $1554 – Remote Vehicle Start Disable History 4 - $1555 – Remote Vehicle Start Disable History 5 - $1556 – Remote Vehicle Start Disable History 6 - $1557 – Remote Vehicle Start Disable History 7 - $1558 – Remote Vehicle Start Disable History 8 | E29348-AI01 |
| 08.09.11.00 | 02/06/07 | Matt Kaufer | Series 08.09.11.00 document created from 08.08.17.00 document. | N/A |
| 08.08.17.00 | 11/09/06 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.08.16.00 | 10/30/06 | Matt Kaufer | Updated description for the following PID: - $2810 – Accelerator Effective Position | E30669-AI01 |
| 08.08.15.00 | 10/11/06 | Matt Kaufer | Updated descriptions for the following PIDs: - $2011 – Displacement on Demand Inhibit Reasons 1 - $2013 – Displacement on Demand Reactivation History 1 - $2014 – Displacement on Demand Reactivation History 2 - $2015 – Displacement on Demand Reactivation History 3 - $2016 – Displacement on Demand Reactivation History 4 - $2017 – Displacement on Demand Reactivation History 5 - $2018 – Displacement on Demand Reactivation History 6 - $2019 – Displacement on Demand Reactivation History 7 - $201A – Displacement on Demand Reactivation History 8 | E30455-AI01 |
| 08.08.15.00 | 10/11/06 | Matt Kaufer | Added support for the following PIDs: - $1354 - Catalyst Monitor Diagnostic Test Counter Bank 1 - $1355 - Catalyst Monitor Diagnostic Test Counter Bank 2 - $206C – Catalyst Monitor Diagnostic Status Bank 1 - $206D – Catalyst Monitor Diagnostic Status Bank 2 - $206E – Catalyst Monitor Diagnostic Status | E29705-AI01 |
| 08.08.14.00 | 09/26/06 | Matt Kaufer | Added support for the following PIDs: - $2856 – Accelerator Pedal Fully Applied Learned Position Sensor 1 - $2857 – Accelerator Pedal Fully Applied Learned Position Sensor 2 | E26255-AI01 |
| 08.08.13.00 | 06/26/06 | Matt Kaufer | (Pulled from 08.07.14.02 – 06/26/06) Update description for the following device control: - CPID $14 “Idle Adapts Reset Trigger Device Control” becomes CPID $14 “Learned Airflow Adapts Reset Trigger Device Control” Added support for the following PID: - $2066 – Learned Airflow Variation Algorithm Compensation | E29665-AI05 |
| 08.08.12.01 | 06/22/06 | Matt Kaufer | (Pulled from 08.05.17.01 – 06/22/06) Updated description for the following PID: - $1124 – Brake Pedal Status | E30138-AI01 |
| 08.08.12.01 | 06/22/06 | Matt Kaufer | (Pulled from 08.07.11.01 – 06/22/06) Added descriptions for the following PIDs - $2820 – Transmission Primary Oncoming Clutch - $2821 – Transmission Full Feed Fill Pressure Corrected typos for the following PIDs - $2033 - Main Processor NVM Fault Status | N/A |
| 08.08.12.00 | 03/23/06 | Matt Kaufer | Updated descriptions of the following PIDs: - $1154 – Estimated Engine Oil Temperature - $153E – Engine Oil Temperature from Sensor | E28780-AI01 |
| 08.08.12.00 | 03/23/06 | Matt Kaufer | Added support for the following PID: - $2826 –Transmission Control Module Restart Sensor Temperature | E23816-AI01 |
| 08.08.11.00 | 03/22/06 | Matt Kaufer | Document version incremented to match software cadence. No other changes made. | N/A |
| 08.08.10.00 | 03/22/06 | Matt Kaufer | Removed support for the following PID: - $11E9 – Misfire Status per Engine Cycle | E28345-AI01 |
| 08.08.10.00 | 03/22/06 | Matt Kaufer | (Pulled from 08.07.11.00 – 03/21/06) Updated description for the following PIDs: - $2011 - Displacement on Demand Inhibit Reasons 1 - $2012 - Displacement on Demand Inhibit Reasons 2 - $2013 - Displacement on Demand Reactivation History 1 - $2014 - Displacement on Demand Reactivation History 2 - $2015 - Displacement on Demand Reactivation History 3 - $2016 - Displacement on Demand Reactivation History 4 - $2017 - Displacement on Demand Reactivation History 5 - $2018 - Displacement on Demand Reactivation History 6 - $2019 - Displacement on Demand Reactivation History 7 - $201A - Displacement on Demand Reactivation History 8 | E27871-AI01 |
| 08.08.09.00 | 03/22/06 | Matt Kaufer | Corrected a number of typos for the following PIDs and DIDs: - $2033 – Main Processor NVM Fault Status - $2035 – Processor ROM Fault Source - $57 – Transmission D-R Shift Adaptive Shift Data - $97 – System Name or Engine Type - $98 – Repair Shop Code or Tester Serial Number | N/A |
| 08.08.08.00 | 11/23/05 | Matt Kaufer | Removed support for the following diagnostic service: - RequestControlOfOnBoardSystem ($08) Service | E26327-AI02 |
| 08.08.07.00 | 11/22/05 | Matt Kaufer | Removed support for the following PID: - $1562 – Knock System Status | E25942-AI04 |
| 08.08.07.00 | 11/22/05 | Matt Kaufer | Updated descriptions of the following PIDs: - $1551 – Remote Vehicle Start Disable History 1 - $1552 – Remote Vehicle Start Disable History 2 - $1553 – Remote Vehicle Start Disable History 3 - $1554 – Remote Vehicle Start Disable History 4 - $1555 – Remote Vehicle Start Disable History 5 - $1556 – Remote Vehicle Start Disable History 6 - $1557 – Remote Vehicle Start Disable History 7 - $1558 – Remote Vehicle Start Disable History 8 | E24336-AI01 |
| 08.08.07.00 | 11/22/05 | Matt Kaufer | Updated the descriptions of the following PIDs: (The actual change was implemented with 08.08.03 software, but the documentation change was not identified until 08.08.07.) - $1315 – Cruise Disengage History 1 - $1316 – Cruise Disengage History 2 - $1317 – Cruise Disengage History 3 - $1318 – Cruise Disengage History 4 - $1319 – Cruise Disengage History 5 - $131A – Cruise Disengage History 6 - $131B – Cruise Disengage History 7 - $131C – Cruise Disengage History 8 | E23138-AI01 |
| 08.08.07.00 | 11/22/05 | Matt Kaufer | Corrected a number of typos for the following PIDs, DIDs, and CPID: - PIDs: $1128, $1130, $1131, $11F8, $11F9, $11FA, $11FB, $1201, $1202, $1203, $1204, $12DE, $13C7 - DIDs: $B2, $B4, $B5, $B6, $B7, $B8, $B9, $C1-C8, $C9-CA, $CC, $D1-D8, $D9-DA, $DB, $DC - CPID: $30 “Transmission Commanded Gear Device Control” | N/A |
| 08.08.06.00 | 08/02/05 | Matt Kaufer | Updated description of the following PID: - $1942 - Transmission Output Speed Added support for the following PID: - $2050 - Transfer Case Output Speed | E26195-AI03 |
| 08.08.06.00 | 08/02/05 | Matt Kaufer | Updated description for the following diagnostic service: - ClearDiagnosticInformation ($04) Service | E25958-AI03 |
| 08.08.06.00 | 08/02/05 | Matt Kaufer | Updated enumerations for the following PID: - $001C – On Board Diagnostic System Type | E25951-AI06 |
| 08.08.06.00 | 08/02/05 | Matt Kaufer | Updated descriptions for the following PIDs: - $000A – Fuel Rail Pressure (Gauge) - $000D – Vehicle Speed - $002F – Total Fuel Level Percentage - $0032 – Fuel Tank Vapor Pressure | E25949-AI01 |
| 08.08.06.00 | 08/02/05 | Matt Kaufer | Updated description of the CPID $1C - Secondary Air Injection Reaction System Device Control as follows: - Add return code $0001 – Device control timer time-out active - Add return code description for $000A DTC Present “ECM/PCM Internal Engine Off Timer Performance” | E24949-AI01 |
| 08.08.06.00 | 08/02/05 | Matt Kaufer | Added support for the following PID: - $204D – Transmission Torque Request Limited Reason | E25095-AI01 |
| 08.08.06.00 | 08/02/05 | Matt Kaufer | (Pulled from 08.05.13.01 – 08/01/05) Updated description for the following PID: - $1641 – Percent Fan Commanded | E26285-AI01 |
| 08.08.06.00 | 08/02/05 | Matt Kaufer | (Pulled from 08.05.13.01 – 08/01/05) Added support for the following PIDs: - $2051 – Knock System Status Sensor 1 - $2052 – Knock System Status Sensor 2 | E25942-AI03 |
| 08.08.06.00 | 08/02/05 | Matt Kaufer | (Pulled from 08.05.13.01 – 08/01/05) Updated description for the following PID: - $1652 - High Sided Coolant Rationality Ignition Off Time | E26286-AI01 |
| 08.08.05.01 | 08/01/05 | Matt Kaufer | Updated enumerations for the following PIDs: - $1315 – Cruise Disengage History 1 - $1316 – Cruise Disengage History 2 - $1317 – Cruise Disengage History 3 - $1318 – Cruise Disengage History 4 - $1319 – Cruise Disengage History 5 - $131A – Cruise Disengage History 6 - $131B – Cruise Disengage History 7 - $131C – Cruise Disengage History 8 | E25781-AI01 |
| 08.08.05.01 | 08/01/05 | Matt Kaufer | (Pulled from 08.07.02.01 – 07/29/05) Added support for the following DIDs: - $C0 – Boot Software Part Number - $D0 – Boot Software Part Number Alpha Code | E24655-AI03 |
| 08.08.05.00 | 05/03/05 | Matt Kaufer | Removed support of the following CPID: - $14 - Idle Adapts Reset Trigger Device Control (Byte 0, Bit 4) | E24062-AI02 |
| 08.08.05.00 | 05/03/05 | Matt Kaufer | Updated description of the following CPID: - $1C - Secondary Air Injection Reaction System Device Control – Add return code $1103 - Engine coolant temperature low. | E25258-AI02 |
| 08.08.05.00 | 05/03/05 | Matt Kaufer | Updated descriptions for the following DIDs: - $4A – Clutch 1 Pressure Learned Characteristics - $4B – Clutch 2 Pressure Learned Characteristics - $4C – Clutch 3 Pressure Learned Characteristics - $4D – Clutch 4 Pressure Learned Characteristics - $4E – Clutch 5 Pressure Learned Characteristics DID write length = 8 bytes, read length = 15 bytes | E26025 –AI02 |
| 08.08.04.00 | 03/31/05 | Matt Kaufer | Added support for the following PIDs: - $202C – Air Fuel Imbalance Monitor Filtered Ratio Bank 1 - $202D – Air Fuel Imbalance Monitor Filtered Ratio Bank 2 - $202E – Air Fuel Imbalance Monitor Maximum Filtered Ratio Bank 1 - $202F – Air Fuel Imbalance Monitor Maximum Filtered Ratio Bank 2 | E24638-AI01 |
| 08.08.04.00 | 03/29/05 | Matt Kaufer | Updated description and limits for the following CPID: - $14 - IDLR - Idle Engine Speed Device Control | E24591-AI01 |
| 08.08.04.00 | 03/29/05 | Matt Kaufer | (Pulled from 08.07.05 – 03/29/05) Added support for the following PID: - $204C – Replicated Transmission Output Speed | E24833-AI01 |
| 08.08.04.00 | 03/29/05 | Matt Kaufer | (Pulled from 08.05.11.00 – 03/02/05) Corrected the descriptions of the following PIDs: - $0007 – Removed “with compensation for the effects of purging” - $0009 – Removed “with compensation for the effects of purging” | E25738-AI01 |
| 08.08.03.00 | 03/21/05 | Matt Kaufer | Updated the description for the following CPID: - $1E – Oxygen Sensor Heater Device Control | E24794-AI01 |
| 08.08.03.00 | 03/21/05 | Matt Kaufer | (Pulled from 08.07.04.01 – 03/15/05) Added support for the following legislated PID: - $000A – Fuel Rail Pressure (Gauge) | E25086-AI01 |
| 08.08.03.00 | 03/21/05 | Matt Kaufer | (Pulled from 08.07.04.00 – 02/22/05 Added support for the following PIDs: - $204A – Main Processor Total Running Resets - $204B – Main Processor Maximum Running Resets Between Power-up Resets | E24135-AI03 |
| 08.08.03.00 | 03/21/05 | Matt Kaufer | (Pulled from 08.05.10.00 - 12/16/04) Updated the description of the following CPID: - $30 – Transmission Commanded Gear Device Control - Add return code $3D02 - Vehicle Speed is too Low | E25189-AI01 |
| 08.08.02.00 | 12/16/04 | Kyle FitzGerald | Added support for the following DID: - $B5 – GM Broadcast Code | E21230-AI02 |
| 08.08.02.00 | 12/16/04 | Kyle FitzGerald | Removed support for the following PIDs: - $12DF - Secondary Processor Reset Source - $12E8 - Secondary Processor Running Reset Source Address - Both PIDs delayed until 11.9 | E24046-AI04 |
| 08.08.02.00 | 12/16/04 | Kyle FitzGerald | Added support for the following PID: - $2032 - Processor Fault Reason Code | E23854-AI03 |
| 08.08.02.00 | 12/16/04 | Kyle FitzGerald | Added support for the following PIDs: - $2033 - Main Processor NVM Fault Status - $2035 - Processor ROM Fault Source | E23629-AI04 |
| 08.08.02.00 | 12/16/04 | Kyle FitzGerald | Added support for the following PID: - $2034 - Main Processor RAM Fault Source Address | E23768-AI04 |
| 08.08.02.00 | 12/16/04 | Kyle FitzGerald | Added support for the following PIDs: - $12DE - Main Processor Reset Source - $12DF - Secondary Processor Reset Source - $12E7 - Main Processor Running Reset Source Address - $12E8 - Secondary Processor Running Reset Source Address | E24046-AI03 |
| 08.08.01.00 | 10/5/04 | Kyle FitzGerald | (Pulled from 08.07.00.00) Updated the description of the following PID: - $0044 – Equivalence Ratio – add requirement for PID $0003 indicating closed loop fuel control | E22365-AI03 |
| 08.08.01.00 | 10/5/04 | Kyle FitzGerald | (Pulled from 08.07.00.00) Updated the description of the following PID: - $0003 – Fuel System Status – add requirement for engine not running | E22365-AI03 |
| 08.08.01.00 | 10/5/04 | Kyle FitzGerald | (Pulled from 08.07.00.00) Added support for the following PID: - $0051 - Type of fuel currently being utilized by the vehicle | E22365-AI03 |
| 08.08.01.00 | 10/5/04 | Kyle FitzGerald | (Pulled from 08.07.00.00) Added support for the following PIDs: - $2027 - Exhaust Pressure Regulator Valve Control Circuit - $2028 - Bank 1 Exhaust Back Pressure A/D - $2029 - Bank 1 Exhaust Back Pressure - $202A - Bank 2 Exhaust Back Pressure A/D - $202B - Bank 2 Exhaust Back Pressure | E22739-AI03 |
| 08.08.01.00 | 10/5/04 | Kyle FitzGerald | (Pulled from 08.07.00.00) Updated the description of the following CPID: - $10 – Add Exhaust Pressure Regulator Valve Enable and Control | E22739-AI03 |
| 08.08.01.00 | 10/5/04 | Kyle FitzGerald | (Pulled from 08.07.00.00) Updated the description of the following Services: - $22 – Read Data By Parameter Identifier – add physical addressing requirement - $AA – Read Data By Packet Identifier – change medium periodic scheduling rate from 300 to 200 ms | E22905-AI03 |
| 08.08.01.00 | 10/5/04 | Kyle FitzGerald | (Pulled from 08.07.00.00) Eliminated support for the following PIDs: - $12EA – Auto Learn Counter - $12EB – Auto Learn Timer - $12EC – Seed and Key Timer | E22863-AI01 |
| 08.08.01.00 | 10/5/04 | Kyle FitzGerald | (Pulled from 08.07.00.00) Eliminated support for the following PIDs: - $11A5 – Purge Learn Multiplier Bank 1 - $1354 – Catalyst Monitor Diagnostic Test Counter Bank 1 - $1355 – Catalyst Monitor Diagnostic Test Counter Bank 2 - $13D9 – Exhaust Gas Recirculation Flow Test On Time Count - $13EB – Oxygen Storage Capacity Time Difference Fail Threshold Bank 1 - $13EC – Oxygen Storage Capacity Time Difference Fail Threshold Bank 2 - $13ED – Oxygen Storage Capacity Time Difference Bank 1 - $13EE – Oxygen Storage Capacity Time Difference Bank 2 - $13EF – Filtered Oxygen Storage Capacity Time Difference Bank 1 - $13F0 – Filtered Oxygen Storage Capacity Time Difference Bank 2 - $1448 – Catalyst Monitor Average Airflow - $1452 – Purge Learn Multiplier Bank 2 | E22862-AI01 |
| 08.08.01.00 | 10/1/04 | Kyle FitzGerald | Added support for the following PID: - $2022 – Traction Control System Requested Torque Control Mode | E23639 Reference: E21291-AI06 |
| 08.05.08.00 | 9/27/04 | Kyle FitzGerald | Added support for the following PIDs: - $1478 – Reference Voltage A - $1479 – Reference Voltage B | E24447-AI01 |
| 08.05.08.00 | 9/27/04 | Kyle FitzGerald | Updated the description of the following CPIDs: - $30 – Add return codes for Vehicle Speed Low and Engine Speed Low - $34 – Remove “Reverse Clutch” from TCC/Reverse Clutch Solenoid Device Control description - $34 – Change Torque Converter Clutch to Transmission M2 Clutch in the TCC/Reverse Multiplex Solenoid Device Control description - $34 – Added M3 to the Transmission Clutch Multiplex Solenoid Device Control description - $38 – Remove “Reverse Clutch” from TCC/Reverse Clutch Pressure Device Control description | E24332-AI01 |
| 08.05.08.00 | 9/7/04 | Kyle FitzGerald | Updated the description of the following DID: - $49 – Torque Converter Learned Characteristics – Add signals | E24334-AI01 |
| 08.05.08.00 | 9/7/04 | Kyle FitzGerald | Updated the description of the following PID: - $280F – Transmission Base Pattern – Add states $0C - $0F | E24058-AI02 |
| 08.05.07.00 | 8/16/04 | Kyle FitzGerald | Added support for the following PID: - $2824 – Transmission Cleaning Procedure Status Updated the description of the following CPID: - $30 – Add Transmission Cleaning Procedure Trigger – add new return code $2E06 | E23841-AI02 |
| 08.05.07.00 | 8/16/04 | Kyle FitzGerald | Updated the description of the following CPID: - $30 – Transmission Fast Learn Trigger – separate into service and in-plant modes – update return codes Updated the description of the following PID: - $2823 - Service Fast Learn Status – redefined states | E23799-AI01 |
| 08.05.07.00 | 8/16/04 | Kyle FitzGerald | Updated the description of the following PIDs: - $1157 – Brake Booster Vacuum Pressure A/D – change the Ring from BTRR to BBVR - $150D – Brake Booster Vacuum Pressure – change the Ring from BTRR to BBVR | E20027-AI02 |
| 08.05.07.00 | 7/20/04 | Kyle FitzGerald | Updated the description of the following PID: - $1130 – Power Mode / Starter Status, – change the description of the Crank Active signal to include Serial Data status | E23978-AI01 |
| 08.05.06.00 | 7/12/04 | Kyle FitzGerald | Updated the description of the following DIDs: - $4A - $4E – Clutch 1- 5 Pressure Learned Characteristics, – add Final Test Clutch Full Feed Fill Pressure | E23130-AI01 |
| 08.05.06.00 | 7/12/04 | Kyle FitzGerald | Updated the description of the following DIDs: - $4A - $4E – Clutch 1- 5 Pressure Learned Characteristics, – change units on Final Test Clutch Pressure Gains and Offsets | E23818-AI01 |
| 08.05.06.00 | 7/12/04 | Kyle FitzGerald | Updated the description of the following CPID: - $34 – Transmission Clutch Solenoids, Torque Converter Clutch / Reverse Clutch Solenoid, Transmission Clutch Multiplex Solenoid and Torque Converter Clutch / Reverse Multiplex Solenoid Device Control, – change device control limits | E23268-AI01 |
| 08.05.06.00 | 7/12/04 | Kyle FitzGerald | Updated the description of the following CPID: - $38 – Torque Converter Clutch / Reverse Clutch Pressure Device Control, – change device control limits | E23268-AI01 |
| 08.05.06.00 | 7/12/04 | Kyle FitzGerald | Updated the description of the following CPID: - $3A – Transmission Clutch 1 / 2 Pressure Device Control, – change device control limits | E23268-AI01 |
| 08.05.06.00 | 7/12/04 | Kyle FitzGerald | Updated the description of the following CPID: - $3B – Transmission Clutch 3 / 4 Pressure Device Control, – change device control limits | E23268-AI01 |
| 08.05.05.00 | 7/12/04 | Kyle FitzGerald | Added support for the following PID: - $2026 - Reverse Inhibit System Status | E23456-AI01 |
| 08.05.05.00 | 7/12/04 | Kyle FitzGerald | Updated the description of the following PID: - $1413 - Reverse Inhibit Solenoid Control Circuit - Cover Reverse Enable and Reverse Inhibit applications and emphasize DTC P0801 should be tied to short to power | E23456-AI01 |
| 08.05.05.00 | 7/12/04 | Kyle FitzGerald | Updated the description of the following CPID: - $20 – Reverse Inhibit Device Control – split device control description between Reverse Enable and Reverse Inhibit applications: - Reverse Enable = Reverse is inhibited with output de-energized - Reverse Inhibit = Reverse is inhibited with output energized. - $20 – Reverse Inhibit Device Control – Add device control limit exceeded description for applications where the Reverse Inhibit function is not present (use $0004). | E23456-AI01 |
| 08.05.04.01 | 5/26/04 | Kyle FitzGerald | Updated the description of the following CPID: - $18 – Canister Purge Device Control – Add Retrun Code $0F03 Active Status for Service and DVT - $18 – Canister Vent Device Control – Add Retrun Code $0F03 Active Status for Service and DVT |  |
| 08.05.04.00 | 5/25/04 | Kyle FitzGerald | Updated the description of the following PID: - $1517 – Removed the SAI Diagnostic Test Results Reported State | E23517-AI01 |
| 08.05.04.00 | 5/24/04 | Kyle FitzGerald | Added support for the following PID: - $2823 - Service Fast Learn Status Updated the description of the following CPID: - $30 – Add Transmission Fast Learn Trigger Updated the description of the following DIDs: - $4A – $4E – Add Service Fast Learn data to the End of Line test data | E22761-AI01 |
| 08.05.04.00 | 5/24/04 | Kyle FitzGerald | Added support for the following PID: - $2822 - TISS/TOSS Regulated Voltage Supply Status | E23283-AI01 |
| 08.05.04.00 | 5/24/04 | Kyle FitzGerald | Added support for the following Return Code - $0F03 - EVAP system test in progress Updated the description of the following CPID: - $18 – Canister Purge Device Control – Add Retrun Code $0F03 - $18 – Canister Vent Device Control – Add Retrun Code $0F03 | E23343-AI01 |
| 08.05.04.00 | 5/07/04 | Kyle FitzGerald | Updated the description of the following PID: - $0013 – Correct the ordering of the PID Signals |  |
| 08.05.04.00 | 5/07/04 | Kyle FitzGerald | Eliminated support for the following PID: - $12F3 – Fuel Alcohol Content Updated the description of the following CPID: - $18 – Fuel Comp Learn Trigger changed to Fuel Comp Reset Trigger – update description Updated the description of the following PID: - $2025 – Eliminate reference to Fuel Comp Learn Device Control | E23560-AI01 |
| 08.05.03.00 | 4/06/04 | Kyle FitzGerald | Updated the description of the following CPID: - $18 – Add description of Fuel Composition Learn Trigger Device Control Added support for the following PID: - $2025 – Fuel Composition Learn Status | E22455-AI01 |
| 08.05.03.00 | 4/06/04 | Kyle FitzGerald | Added support for the following PID: - $2024 – Buffered 5 Volt Supply Status | E21402-AI01 |
| 08.05.03.00 | 4/06/04 | Kyle FitzGerald | Updated the description of the following PID: - $2023 – Regulated Voltage Control Override Source | E21988-AI04 |
| 08.05.03.00 | 4/06/04 | Kyle FitzGerald | Added support for the following PID: - $0052 – Alcohol Fuel Percentage | E22990-AI01 |
| 08.05.02.00 | 3/12/04 | Kyle FitzGerald | Eliminated support for the following unused (by Service or DVT) PIDs: - $19A0 – Transmission Upshift Delay Time - $19CC – Transmission Steady State Slip Counter - $19D0 – Transmission Steady State Pressure Mode - $1A13 – Torque Converter Clutch Mode - $1A16 – Torque Converter Clutch Base Operation Pressure - $1A19 – Transmission Engine Torque Reduction - $1A23 – Garage Shift Pulse Pressure - $1A24 – Garage Shift Pressure - $1A2A – Shift Torque - $1A2B – Gear Box Torque - $1A2C – Turbine Torque - $1A2E – Transmission Line Pressure - $1A2F – Transmission Upshift Pressure Adapt Cell Value - $1A30 – Transmission Steady State Pressure Adapt Cell Value - $1A31 – Torque Converter Clutch On Ramp Pressure - $1A33 – Garage Shift Pulse Time - $1A34 – Garage Shift Delay Time - $1A35 – Torque Converter Clutch Capacity - $1A36 – Throttle Position at Start of 1-2 Upshift - $1A37 – Throttle Position at Start of 2-3 Upshift - $1A38 – Throttle Position at Start of 3-4 Upshift - $1A39 – Transmission Line Pressure Source - $1A3B – Throttle Position at Start of 4-5 Upshift - $1A7B – Transmission Engine Torque Request | E22864-AI01 |
| 08.05.01.00 | 2/23/04 | Kyle FitzGerald | Updated the description of the following PIDs: - $281E – Changed Ring Owner to RSSR - $281F – Changed Ring Owner to RSSR | E22774-AI01 |
| 08.05.02.00 | 2/23/04 | Kyle FitzGerald | Added the description of the following PID: - $2023 – Regulated Voltage Control Override Source | E21988-AI01 |
| 08.05.02.00 | 2/23/04 | Kyle FitzGerald | Added the description of the following PID: - $2022 – Traction Control System Requested Torque Control Mode | E21291-AI06 |
| 08.05.02.00 | 1/23/04 | Kyle FitzGerald | Updated the description of the following PID: - $1A8E – All Wheel Drive System Status – to add PTO Torque Management Active Status | E22589-AI01 |
| 08.05.01.00 | 1/21/04 | Kyle FitzGerald | Updated the description of the following PID: - $1A8E – All Wheel Drive System Status – to swap Byte 0 and Byte 1 contents to match implementation - $1A8E - Added description of Open and Short Faults detected | E22600-AI01 |
| 08.05.01.00 | 1/20/04 | Kyle FitzGerald | Updated the description of the following PID: - $1A1F – Transmission Clutch Slip – to adjust the Data Type and Range to match the software implementation | E22298-AI01 |
| 08.05.01.00 | 1/20/04 | Kyle FitzGerald | Updated the description of the following CPIDs: - $1C – Add return code $0601 to Secondary Air Injection Diagnostic Service Bay Test Trigger Device Control | E22405-AI01 |
| 08.05.01.00 | 12/15/03 | Kyle FitzGerald | Updated the description of the following CPIDs: - $34 – Add missing return codes - $38 – Add missing return codes - $3A – Add missing return codes - $3B – Add missing return codes | N/A |
| 08.05.01.00 | 12/12/03 | Kyle FitzGerald | Added the description of the following DID: - $B0 – ECU Diagnostic Address | N/A |
| 08.05.01.00 | 12/9/03 | Kyle FitzGerald | Updated the description of the following PID: - $1130 – Power Mode/Starter Status – to add Automatic Transmission Park/Neutral Input Active | E22368-AI01 |
|  |  |  | Added support for the following PID: - $2021 – Powertrain Induced Chassis Pitch | E22369-AI01 |
|  |  |  | Eliminated support for the following unused (by Service or DVT) Device Control Timer PIDs: - $13E5 – Spark Retard - $13E6 – Secondary AIR Injection System - $13E7 – Fuel Injector Disable - $13E8 – Exhaust Gas Recirculation - $1699 – Cam Phase | E22370-AI01 |
|  |  |  | Added support for the following PIDs: - $2820 – Transmission Primary Oncoming Clutch - $2821 – Transmission Full Feed Fill Pressure | E22282-AI01 |
|  |  |  | Updated the description of the following DID: - $57 – Transmission D-R Shift Adaptive Shift Data – to remove Transmission Full Feed Fill Pressure | E22282-AI01 |
| 08.05.00.00 | 11/18/03 | Kyle FitzGerald | Added support for the following Legislated PID: - $0046 – Ambient Air Temperature | E21805-AI01 |
|  |  |  | Eliminated support for the following PIDs: - $1653 – High Sided Coolant Rationality Temperature Difference | E21638-AI01 |
|  |  |  | Updated the descriptions for the following PIDs: - $1131 – Fuel System Status | E20404-AI01 |
|  |  |  | Added support for the following PIDs: - $2020 – Create new PID for malfunction indicator lamp | E20403-AI01 |
|  |  |  | Updated the description of CPID $18 | E20614-AI01 |
| 08.04.03 | 11/05/03 | Kyle FitzGerald | Added description of DIDs $52 - $57 | E21041-AI01 |
| 08.04.02 | 11/05/03 | Kyle FitzGerald | Updated the description of PID $1567 | E21376-AI01 |
|  | 11/05/03 | Kyle FitzGerald | Updated the description of PID $1134 | E20798-AI01 |
|  | 11/05/03 | Kyle FitzGerald | Added EACR E21240 as implemented in version 08.04.01 to CPID $10 description | E18458-AI02 |
|  | 11/05/03 | Kyle FitzGerald | Added description of Corporate DIDs $90, $97, $98, $99, $9A, $A0, $B2, $B4, $B6, $B7, $B8, $B9, $C1-$C8, $C9-$CA, $CB, $CC, $D1-$D8, $D9-$DA, $DB, and $DC | E16480-AI03 |
| 08.04.01 | 08/30/03 | Kyle FitzGerald | Updated list of Return Codes – Add $031D, $0403, $0406, $0407, and $1801. Remove $0306, $030E, $0313, $0314, $0317, $031C, and $0802. | E16480-AI02 |
| 08.04.01 | 08/30/03 | Kyle FitzGerald | Added description of CPID $FE | E16480-AI02 |
| 08.04.01 | 08/30/03 | Kyle FitzGerald | Added description of CPID $7E | E16480-AI02 E16447-AI01 |
| 08.04.01 | 08/30/03 | Kyle FitzGerald | Added description of CPID $7D | E16480-AI02 E16447-AI01 |
| 08.04.01 | 08/30/03 | Kyle FitzGerald | Added description of CPID $3B | E16480-AI02 |
| 08.04.01 | 08/30/03 | Kyle FitzGerald | Added description of CPID $3A | E16480-AI02 |
| 08.04.01 | 08/30/03 | Kyle FitzGerald | Added description of CPID $38 | E16480-AI02 |
| 08.04.01 | 08/30/03 | Kyle FitzGerald | Added description of CPID $34 | E16480-AI02 |
| 08.04.01 | 08/30/03 | Kyle FitzGerald | Added description of CPID $30 | E16480-AI02 E11697-AI02 |
| 08.04.01 | 08/29/03 | Kyle FitzGerald | Added description of CPID $28 | E16480-AI02 E13388-AI03 |
| 08.04.01 | 08/29/03 | Kyle FitzGerald | Added description of CPID $20 | E16480-AI02 E16823-AI02 |
| 08.04.01 | 08/28/03 | Kyle FitzGerald | Added description of CPID $1E | E16480-AI02 E18916-AI01 E11697-AI02 |
| 08.04.01 | 08/28/03 | Kyle FitzGerald | Added description of CPID $1D | E16480-AI02 E17801-AI02 E11697-AI02 |
| 08.04.01 | 08/28/03 | Kyle FitzGerald | Added description of CPID $1C | E16480-AI02 E17801-AI02 E16463-AI04 E11697-AI02 |
| 08.04.01 | 08/27/03 | Kyle FitzGerald | Added description of CPID $18 | E16480-AI02 E11697-AI02 |
| 08.04.01 | 08/27/03 | Kyle FitzGerald | Added description of CPID $14 | E16480-AI02 E18198-AI02 E18199-AI02 E20037-AI02 E13863-AI03 E13525-AI02 |
| 08.04.01 | 08/21/03 | Kyle FitzGerald | Added description of CPID $10 | E16480-AI02 E14658-AI02 E16823-AI02 E13667-AI01 E11697-AI02 E21240-AI01 |
| 08.04.01 | 08/18/03 | Kyle FitzGerald | Added support for the following legislated diagnostic services: - $01 – RequestCurrentPowertrainDiagnosticData - $02 – RequestPowertrainFreezeFrameData - $03 – RequestEmission-RelatedDiagnosticTroubleCodes-Confirmed - $06 – RequestOn-BoardMonitoringTestResults - $07 – RequestEmission-RelatedDiagnosticTroubleCodes-Pending - $08 – RequestControlOfOn-BoardSystem - $09 – RequestVehicleInformation Added reference [9], [10] and [11] to Appendix C | E21029-AI01 |
| 08.04.01 | 08/18/03 | Kyle FitzGerald | Updated cover sheet to indicate new document ownership |  |
| 08.04.00 | 06/19/03 | Trenton Haines | Eliminated support for the following PIDs: - $11A3 – Estimated Catalyst Converter Temperature | E20647-AI01 |
|  |  |  | Eliminated support for the following PIDs: - $11E7 – Wheel Sensor Rough Road Magnitude | E20867-AI01 |
| 08.03.01 | 06/19/03 | Trenton Haines | Updated the descriptions for the following PIDs: - $1941 – Transmission Input Speed - $1942 – Transmission Output Speed | E20721-AI01 |
|  |  |  | Added support for the following PIDs: - $200E – Distance With Cylinders Deactivated - $200F – Distance With All Cylinders Activated - $2010 – Displacement On Demand Transitions | E19280-AI01 |
|  |  |  | Added support for the following PIDs: - $2011 – Displacement on Demand Inhibit Reasons 1 - $2012 – Displacement on Demand Inhibit Reasons 2 - $2013 – Displacement on Demand Inhibit History 1 - $2014 – Displacement on Demand Inhibit History 2 - $2015 – Displacement on Demand Inhibit History 3 - $2016 – Displacement on Demand Inhibit History 4 - $2017 – Displacement on Demand Inhibit History 5 - $2018 – Displacement on Demand Inhibit History 6 - $2019 – Displacement on Demand Inhibit History 7 - $201A – Displacement on Demand Inhibit History 8 Eliminated support for the following PIDs: - $153D – Displacement on Demand Inhibit Reason | E18458-AI01 |
|  |  |  | Added support for the following PIDs: - $201B – Clutch Pedal Position Sensor A/D - $201C – Clutch Pedal Fully Applied Learned Position - $201D – Clutch Pedal Fully Applied Position Learn Status - $201E – Clutch Pedal Released Learned Position - $201F – Clutch Pedal Displacement Updated the descriptions for the following PIDs: - $1125 – Manual Transmission Status | E20105-AI01 |
|  |  |  | Added support for the following PIDs: - $2811 – Pressure Control Solenoid 1 Output Status - $2812 – Pressure Control Solenoid 2 Output Status - $2813 – Pressure Control Solenoid 3 Output Status - $2814 – Pressure Control Solenoid 4 Output Status - $2815 – Pressure Control Solenoid 5 Output Status - $2816 – Torque Converter Clutch Pressure Control Solenoid Output Status | E20297-AI01 |
|  |  |  | Added support for the following PIDs: - $2817 – Pressure Control Solenoid 1 Commanded Pressure - $2818 – Pressure Control Solenoid 2 Commanded Pressure - $2819 – Pressure Control Solenoid 3 Commanded Pressure - $281A – Pressure Control Solenoid 4 Commanded Pressure - $281B – Pressure Control Solenoid 5 Commanded Pressure - $281C – Torque Converter Clutch Pressure Control Solenoid Commanded Pressure | E20301-AI01 |
|  |  |  | Added support for the following PIDs: - $281D – Transmission Fluid Pressure Switch Status | E20411-AI01 |
|  |  |  | Added support for the following PIDs: - $281E – Shift Solenoid 1 Control Circuit - $281F – Shift Solenoid 2 Control Circuit | E20304-AI01 |
| 08.03.00 | 04/29/03 | Trenton Haines | Added support for the following diagnostic services: - $22 – ReadDataByParameterIdentifier - $2D – DefinePIDByAddress Changed the size of the memoryAddress parameter for Mode $23 from 3 bytes to 4 bytes for consistency with Mode $2D. Eliminated support for the use of Mode $23 to retrieve data from the secondary processor. Eliminated support for DPID $EE from Modes $2C and $AA. Updated the table of PID ranges. | E20041-AI01 |
|  |  |  | Added support for the following PIDs: - $1178 – Fuel Pump Duty Cycle - $148E – Variable Speed Fuel Pump Diagnostic Duty Cycle | E20104-AI01 |
|  |  |  | Eliminated support for the following PIDs: - $1A20 – Transmission 1-2 Detent Upshift Maximum Engine Speed - $1A21 – Transmission 2-3 Detent Upshift Maximum Engine Speed - $1A22 – Transmission 3-4 Detent Upshift Maximum Engine Speed | E20241-AI01 |
|  |  |  | Added support for the following PIDs: - $2000 – Intake Manifold Absolute Pressure A/D - $2001 – Intake Manifold Vacuum Pressure - $2002 – Supercharger Inlet Absolute Pressure - $2003 – Supercharger Inlet Absolute Pressure A/D - $2004 – Supercharger Inlet Vacuum Pressure Eliminated support for the following PIDs: - $1142 – Intake Manifold Absolute Pressure - $11AB – Intake Manifold Vacuum | E20034-AI01 |
|  |  |  | Added support for the following PIDs: - $2005 – Power-Up Intake Air Temperature - $2006 – Intake Air Temperature 2 Eliminated support for the following PIDs: - $160A – Power-Up Intake Air Temperature Updated the descriptions for the following PIDs: - $000F – Intake Air Temperature - $1626 – Minimum Intake Air Temperature | E20035-AI01 |
|  |  |  | Added support for the following PIDs: - $2007 – Barometric Pressure A/D Updated the descriptions for the following PIDs: - $0033 – Barometric Pressure | E20036-AI01 |
|  |  |  | Added support for the following PIDs: - $2008 – Intake Manifold Tuning Valve Control Circuit - $2009 – Intake Manifold Tuning Vane Position Duty Cycle - $200A – Intake Manifold Tuning Status | E20037-AI01 |
|  |  |  | Added support for the following PIDs: - $200B – Supercharger Boost Solenoid Duty Cycle - $200C – Supercharger Boost Solenoid Control Circuit - $200D – Intercooler Pump Control Circuit | E20103-AI02 |
|  |  |  | Added support for the following PIDs: - $2809 – Torque Converter Clutch Reference Slip | E20242-AI01 |
|  |  |  | Added support for the following PIDs: - $280A – Transmission 5-6 Upshift Time - $280B – Transmission 5-6 Upshift Time Error | E20243-AI01 |
|  |  |  | Added support for the following PIDs: - $280C – Torque Converter Clutch Status | E20244-AI01 |
|  |  |  | Added support for the following PIDs: - $280D – Transmission Control Module Substrate Temperature | E20296-AI01 |
|  |  |  | Added support for the following PIDs: - $280E – Replicated Transmission Output Speed Output Control Circuit | E20307-AI01 |
|  |  |  | Added support for the following PIDs: - $280F – Transmission Base Pattern Eliminated support for the following PIDs: - $1973 – Transmission Shift Pattern | E20282-AI01 |
|  |  |  | Added support for the following PIDs: - $2810 – Accelerator Effective Position | E20491-AI01 |
|  |  |  | Added overview for CPIDs, device controls, and device control limit exceeded return codes. | E16480-AI04 |
| 08.02.01 | 03/28/03 | Trenton Haines | Added support for the following PIDs: - $2800 – Transmission Shift Solenoid A Control Circuit - $2801 – Transmission Shift Solenoid B Control Circuit - $2802 – Transmission Shift Solenoid C Control Circuit - $2803 – Transmission 3-2 Downshift Solenoid Control Circuit - $2804 – Transmission High Side Driver 1 Control Circuit - $2805 – Transmission High Side Driver 2 Control Circuit - $2806 – Torque Converter Clutch Enable Control Circuit - $2807 – Torque Converter Clutch Control Solenoid Control Circuit - $2808 – Transmission Line Pressure Control Circuit). | E20038-AI01 |
| PCC.08.02.00 | 02/20/03 | Trenton Haines | Created initial document. | E16480-AI01 |

Appendix B - List of Acronyms

The following acronyms are used throughout this document.

| ABS | Antilock Brake System |
| --- | --- |
| A/C | Air Conditioning |
| ASC | ASCII Encoded |
| AWD | All Wheel Drive |
| BASS | Brake Apply Sensing System |
| BCD | Binary Coded Decimal (Numeric Signal) |
| BLN | Boolean |
| C2C | Clutch to Clutch |
| CAN | Controller Area Network |
| CARB | California Air Resource Board |
| CASS | Clutch Apply Sensing System |
| CASE | Crank Angle Sensing Error |
| CPID | Control Packet Identifier |
| CTS | Component Technical Specification |
| DID | Data Identifier |
| DPID | Data Packet Identifier |
| DRI | Drive Reverse Indicator |
| DTC | Diagnostic Trouble Code |
| E | Engineering Units |
| EACR | Enhancement and Anomaly Change Record |
| ECCC | Electronically Controlled Converter Clutch |
| ECM | Engine Control Module |
| ECU | Electronic Control Unit |
| EGR | Exhaust Gas Recirculation |
| ENM | Enumerated |
| EONV | Engine Off Natural Vacuum |
| EPA | Environmental Protection Agency |
| ERS | Energy Reduction Software |
| ETEI | Engine Control Module – Transmission Control Module Electrical Interface |
| EV | Electro Viscous |
| EWMA | Exponentially Weighted Moving Average |
| GM | General Motors |
| GMLAN | General Motors Local Area Network |
| GMPT | General Motors Powertrain |
| ICM | Immobilizer Control Module |
| IMS | Internal Mode Switch |
| ISO | The International Organization for Standardization |
| LORES | Low Resolution |
| MAF | Mass Airflow |
| MAP | Manifold Absolute Pressure |
| MBT | Mean Best Torque |
| MEC | Manufacturers Enable Counter |
| MEDRES | Medium Resolution |
| MRF | Magneto Rheological Fan |
| MIL | Malfunction Indicator Lamp |
| N | Numerical Units |
| NSBU | Neutral Safety Backup |
| OBD | On Board Diagnostic |
| OSC | Oxygen Storage Capacity |
| PCS | Pressure Control Solenoid |
| PDS | Periodic DPID Scheduler |
| PID | Parameter Identifier |
| PKT | Packeted |
| PPEI | Platform-Powertrain Electrical Interface |
| PPM | Pulse Per Mile |
| PRNDL | Park Reverse Neutral Drive Low |
| PSM | Pressure Switch Manifold |
| PTO | Power Take Off |
| PWM | Pulse Width Modulation |
| RDM | Rear Drive Module |
| RVC | Regulated Voltage Control |
| RVS | Remote Vehicle Start |
| SAI | Secondary Air Injection |
| SCIAP | Supercharger Inlet Absolute Pressure |
| SNM | 2´s Complement Signed Numeric |
| SPS | Service Programming System |
| STP | Standard Temperature and Pressure |
| SVS | Service Vehicle Soon |
| TCC | Torque Converter Clutch |
| TCCM | Transfer Case Control Module |
| TCM | Transmission Control Module |
| TIS | Transmission Input Speed |
| TLS | Temperature Limiting Software |
| TOS | Transmission Output Speed |
| UNM | Unsigned Numeric |
| USDT | Unacknowledged Segmented Data Transfer |
| UUDT | Unacknowledged Unsegmented Data Transfer |
| VTD | Vehicle Theft Deterrent |
| WOT | Wide Open Throttle |

The following acronyms are used to identify specific rings throughout this document.

| AAPR | Ambient Air Pressure (Barometric Pressure) |
| --- | --- |
| ACCR | Air Conditioning Compressor |
| AIRR | Air Injection Reaction (Secondary Air Injection) |
| APSR | Accelerator Position |
| AWDR | All Wheel Drive (Versatrak) |
| BFPR | Boot Flash Programming |
| BRKR | Brakes |
| BSTR | Boost |
| BTRR | Brake Torque Reduction |
| CATR | Catalyst Monitor |
| CCTR | Catalyst Converter Temperature Estimation |
| CDAR | Cylinder Deactivation (Displacement on Demand) |
| CRZR | Cruise Control |
| DFIR | Diagnostic Fault Information |
| DGSR | Diagnostic Gear Scheduling |
| DRER | Diagnostic Reset and Enable |
| E85R | Ethanol 85 (Fuel Composition) |
| ECTR | Engine Coolant Temperature |
| EGRR | Exhaust Gas Recirculation |
| EGTR | Exhaust Gas Temperature Estimation |
| EITR | Engine Intake Temperatures |
| ENGR | General Engine |
| EODR | Engine Oil Displays (Level, Life, Temperature) |
| EOPR | Engine Oil Pressure |
| EOTR | Engine Oil Temperature Estimation |
| EPSR | Engine Position |
| ETQR | Engine Torque Estimation |
| EVPR | Evaporative Emissions |
| FADR | Fuel Adjustment |
| FANR | Cooling Fans |
| FFRR | Freeze Frame / Failure Record |
| FLVR | Fuel Level |
| FPMR | Fuel Pump |
| FULR | Fuel |
| FWDR | Four Wheel Drive |
| GENR | Generator |
| GSPR | Garage Shift Pressure |
| IDLR | Idle |
| IFRR | Intake Airflow Rationality Diagnostic |
| IMOR | Immobilizer |
| IMTR | Intake Manifold Tuning |
| KNKR | Knock |
| MAFR | Mass Airflow |
| MAPR | Manifold Absolute Pressure |
| MEMR | Memory |
| MSFR | Misfire |
| MTCR | Manual Transmission Clutch Pedal |
| ODAR | Off Vehicle Data Access |
| ODCR | Off Vehicle Device Control |
| OFPR | Off Vehicle Flash Programming (Application) |
| OSAR | Off Vehicle Security Access |
| OVIR | Off Vehicle Infrastructure |
| OXYR | Exhaust Oxygen Sensors |
| PHSR | Cam Phasing |
| PMDR | Power Moding |
| PTOR | Power Take Off |
| RINR | Reverse Inhibit |
| RTDR | Real Time Damping (Lift / Dive) |
| RSCR | Range Shift Control |
| RVSR | Remote Vehicle Start |
| SHPR | Shift Point |
| SHTR | Shift Timing |
| SKSR | Skip Shift |
| SPKR | Spark |
| SPRR | Shift Pressure |
| SSPR | Steady State Pressure |
| STRR | Starter Control |
| TBNR | Turbine Speed |
| TCCR | Torque Converter Clutch |
| TCSR | Traction Control |
| TFQR | Transmission Fluid (Oil) Quality (Life) |
| TFTR | Transmission Fluid (Oil) Temperature |
| TISR | Transmission Input Speed |
| TOSR | Transmission Output Speed |
| TPCR | Transmission Line Pressure Control |
| TPSR | Throttle Position |
| TQTR | Transmission Torque Estimation |
| TRGR | Transmission Range |
| TRNR | General Transmission |
| TRQR | Engine Torque Control |
| TRTR | Transmission Ratio |
| TTQR | Transmission Torque Management |
| TUDR | Tap Up / Tap Down |
| VSPR | Vehicle Speed |
| VTDR | Vehicle Theft Deterrent |

Appendix C - Related Documents

The following documents are either referenced by this document or contain related information.  These documents may provide useful information to the reader and should be reviewed for compatibility in the event that this document (or any of the related documents) undergoes revision.

|  | Document Number | Version | Document Title |
| --- | --- | --- | --- |
| [1] | GMW 3110 | 1.4 | GMLAN Enhanced Diagnostic Test Mode Specification http://www.ived.gm.com/electrical/warren/ssltexpt/gmlan/specs/index.html |
| [2] | GMW 3203 | 0.2 | GMLAN Signal and Frame Definition Guidelines Section 3, Standard Data Types http://www.ived.gm.com/electrical/warren/ssltexpt/gmlan/specs/index.html |
| [3] | ISO 15031-5 |  | Road Vehicles - Communication Between Vehicle and External Equipment - Part 5: Emission-Related Diagnostic Services |
| [4] | ISO 15031-6 |  | Road Vehicles - Communication Between Vehicle and External Equipment - Part 6: Diagnostic Trouble Code Definitions |
| [5] | corp_pid.xls |  | Parameter Identification (PID) List (All) http://so.gm.com/product/tech2/tech2a.htm |
| [6] | GM_Master_DTC_List.xls | PCC.08.02.00 | DTC Master List http://powertrain.gm.com/engineer/controls/eis/algo_private/inf/dfir/index.htm |
| [7] | ffmaster.txt | PT1#08.02.00 | Freeze Frame / Failure Record Master List http://powertrain.gm.com/engineer/controls/eis/algo_private/inf/ffrr/index.htm |
| [8] | FreezeFrameFailureRecordList.xls |  | Freeze Frame Failure Record List.xls |
| [9] | ISO 15031-4 |  | Road Vehicles - Communication Between Vehicle and External Equipment - Part 5: External Test Equipment |
| [10] | ovirkmode06tmp.doc | 08.02.00 | OBD Mode 6 Test Value and Limit Template |
| [11] | ovirkgmptJ1979.doc | 08.02.00 | Implementation Specification for SAE J1979 Test Modes http://powertrain.gm.com/engineer/controls/eis/algo_private/inf/ovir/index.htm |