Customer Product Specification

Commissioning and Test on Vehicle

MK100 ESC

T6 – SOP KW17/2015

Caddy 4 - SOP KW22/2015

Amarok GP - SOP KW22/2016

The reproduction, distribution and utilization of this document as well as the disclosure of its contents to third parties without express authorization are prohibited. Offenders will be held liable for the payment of damages. All rights reserved in the event of the grant of a patent, utility model or design.

| No guarantee can be given in respect of the translation. In all cases the latest German-Language version of this standard shall be taken as authoritative. | NOTE NO. | DATE | STANDARDS DEPT. | OPERATING DEPT. | CHECKED BY |
| --- | --- | --- | --- | --- | --- |
| ©Continental AG. 2016 |  |  |  | CEMARC Roth |  |

82.9212-0000.4/800/01 “doc”

SUMMARY

Revisions...................................................................................................................................... 4

1 Application Field........................................................................................................................ 5

2 Other Applicable Documents.................................................................................................... 5

3 General Notes… ........................................................................................................................ 6

3.1 Deviations from this Specification.......................................................................................... 6

3.2 List of Abbreviations............................................................................................................... 6

3.3 Term Definitions..................................................................................................................... 7

3.4 Copyright Notice….................................................................................................................. 7

4 Specific Test Conditions ……….. ................................................................................................. 8

4.1 Electric / Electronic................................................................................................................. 8

4.1.1 Hydraulic.............................................................................................................................. 8

4.1.2 Test Temperature................................................................................................................ 8

4.1.3 External Wheel Drive… ....................................................................................................... 8

4.2 Test Device Requirements...................................................................................................... 9

4.3 Deviations from this Specification.......................................................................................... 9

5 Sensor Signal Test in Pre-Assembly......................................................................................... 10

5.1 Sensor Signal Evaluation....................................................................................................... 10

5.2 Test with Low Input Voltage…………………. ............................................................................ 10

5.3 Signal Quality Check…… ....................................................................................................... 11

6 Filling and After Work Deareration......................................................................................... 12

7 Commissioning and Test on Completely Assembled Vehicle.................................................. 13

7.1 Basic Configurations and Sensor Comparison...................................................................... 13

7.1.1 Control Module Coding (T6, Caddy 4, Amarok)................................................................. 13

7.1.2 Basic Configuration of Brake Pressure Encoder (T6, Caddy 4, Amarok)............................ 13

7.1.3 Basic Configuration of Steering Angle Encoder (T6, Amarok)........................................... 13

7.1.4 Basic Configuration of ESC-Sensorik (T6, Caddy 4, Amarok)............................................. 14

7.1.5 Basic Configuration of Tire Control Monitor (if installed: T6, Caddy 4)............................. 14

7.2 Static Tests…. ....................................................................................................................... 15

7.2.1 ECU Switch-On Test und ABS Self-Test.............................................................................. 15

7.2.2 Electronic Controller Identification…................................................................................. 15

7.2.3 Brake Light Sensor Test..................................................................................................... 16

7.2.4 Test of the ESC-Off Button and of the ASR/ESC and ESC-Off Warning Lights.................... 16

7.3 Sensor and Control Cursor Tests.......................................................................................... 17

7.3.1 Steering Angle Sensor Test................................................................................................ 17

7.3.2 Zero Point Accuracy Test of the Steering Rate Sensor...................................................... 17

7.3.3 Zero Point Accuracy Test of the Transverse Acceleration Sensor..................................... 17

7.3.4 Zero Point Accuracy Test of the Lengthwise Acceleration Sensor.................................... 18

7.3.5 Zero Point Accuracy Test of the Brake Pressure Encoder.................................................. 18

7.3.6 Emergency Brake Monitor Function Test.......................................................................... 19

7.3.7 Brake Light Activation........................................................................................................ 19

8 Dynamic Tests……. .................................................................................................................. 20

8.1 Manual Hydraulic Incorrect Connection Test (Research Shop, Customer Service).............. 20

8.2 Hydraulic Incorrect Connection Test on the Roll Test Bench for ESC................................... 20

8.2.1 Individual Wheel Test Process…........................................................................................ 22

8.2.2 Individual Wheel Evaluation of the Process Values........................................................... 23

8.2.3 Adjustment Instructions.................................................................................................... 24

8.3 Tooth Pitch Test…................................................................................................................. 25

9 Valve Calibration in ESC Unit (only for develoment and/or repair purposes)......................... 26

9.1 Equipment Requirements.. .................................................................................................. 26

9.1.1 Electric / Electronic............................................................................................................ 26

9.1.2 Hydraulic............................................................................................................................ 26

9.1.3 Vehicle............................................................................................................................... 26

9.1.4 Temperature...................................................................................................................... 27

9.2 ESC Valve Calibration in the Vehicle..................................................................................... 27

9.2.1 Determination of an HECU Calibration Status................................................................... 28

9.2.2 Inlet Valve Calibration…. ................................................................................................... 29

9.2.3 Separation Valve Calibration............................................................................................. 29

9.2.4 Troubleshooting................................................................................................................ 29

Revisions

| Version | Changed by | Changed on | Revision notes |
| --- | --- | --- | --- |
| $1.1 | Walter Barth | 05.03.2013 | Initial Issue, issued by VW MQB MK 100 ESC (status 02.15.2013 |
| $1.2 | Walter Barth | 08.21.2013 | Review and small corrections in F55 Sample Map |

PREVIOUS ISSUES

None

1 Application Field

This test specification applies to the electronic brake system of the vehicle designs T6 (7E0/7E5), Caddy 4 (2K5) and Amarok GP (2H0). It contains tests for the respective electronic ESC brake system, which are recommended to ensure functional interchangeability for installation of the system in a vehicle.

The test steps arrangement in the assembly line and in stationary test equipment remains at the vehicle manufacturer’s discretion.

2 Other Applicable Documents

520 24.70 Sheet.117	Filling Standard for MK100 ESC

Lamp Matrix of the Brake System Warning / Monitoring Concept

VN75xGP2_VN357GP2_VW_Kodiertabelle_V_0_5_1.xlsx

Diagnosis Specification

DVR26_BV_Brake1UDS_VN75X_final // Version 007003

10.0211-0070.6 Hydraulic Connection Diagram MK100 ESC

10.0800-1239.6 Electric Connection Diagram ER100 ESC

3 General Notes

3.1 Deviations from this Specification

Depending on the assembly site, the arrangement of the respective test technique applied, the customer’s internal test performance requirements as cycle time, assembly line layout, equipment, etc., individual tests described in this specification may be modified or omitted.

Deviations regarding the recommended test methods for individual tests are allowed, provided that use of the modified test will yield comparable test results. Should line-end test equipment not be available, Continental Automotive Systems will not be able to check other test methods than those contained in the prescribed specification.

Should the vehicle manufacturer decide to deviate from the recommended test procedures, the Continental Automotive Systems customer service will be thankful for being informed about such event. Only in this case, a competent technical series follow-up by Continental Automotive Systems will be possible.

3.2 Abbreviation List

UB - Battery voltage	( Batterie Spannung )

BLS - Break Light Switch	( Bremslichtschalter )

BSW – Brake disc cleaner	( Bremsscheibenwischer )

BWE - Fluid level warning indicator	( Behälterwarneinrichtung )

BWL - Brake Warning Lamp	( Bremsenwarnleuchte )

CAN - Controller Area Network	( Controller Netzwerk )

DDS - Deflation Detection System	( Reifendruckverlust-Erkennung = RKA)

DSR - Driver Steering Recommendation	( Fahrer Lenkempfehlung )

ECU - Electronic Controller Unit	( Elektronisches Steuergerät )

EMV - Electromagnetic Compatibility (EMC)	( Elektromagnetische Verträglichkeit )

ESD - Electrostatic discharge	( Elektrostatische Entladung )

EPS - Electric Power Steering	( Elektrische Servolenkung )

EUV - Electric Shuttle Valve	( Elektrisches Umschaltventil )

fs – Signal frequency wheel speed sensor	( Signalfrequenz Radsensor )

GND – Ground	( Masse )

HCU - Hydraulic Control Unit	( Hydraulische Regeleinheit )

Is - Sensor current	( Sensorstrom )

IH - Sensor current high	( Sensorstrom hoch )

IL - Sensor current low	( Sensorstrom unten )

IFL - Inlet Valve Front Left	( Einlassventil vorne links )

IFR - Inlet Valve Front Right	( Einlassventil vorne rechts )

IRL - Inlet Valve Rear Left	( Einlassventil hinten links )

IRR - Inlet Valve Rear Right	( Einlassventil hinten rechts )

LWS - Steering wheel angle sensor	( Lenkwinkelsensor )

MAXSS - Max. Sensor Output Voltage	( max. Grenzwert for die Ausgabe der Sensorspannung )

MINSS - Min. Sensor Output Voltage	( min. Grenzwert for die Ausgabe der Sensorspannung )

MSR - Engine Drag Control	( Motor-Schleppmomentenregelung )

MWB – Measured values block	( Messwerteblock )

NC - Normally Closed Solenoid Valves	( Stromlos Geschlossen Magnetventile )

NO - Normally Opened Solenoid Valves	( Stromlos Offen Magnetventile )

OFL - Outlet Valve Front Left	( Auslassventil vorne links )

OFR - Outlet Valve Front Right	( Auslassventil vorne rechts )

ORL - Outlet Valve Rear Left	( Auslassventil hinten links )

ORR - Outlet Valve Rear Right	( Auslassventil hinten rechts )

PIN - Connector terminal	( Steckerklemme )

PYA - Pressure Yaw Rate Acceleration	( im Hydroaggregat integrierte ins ESC-Sensorik )

3.3 Term Definitions

Calibration:

If a systematic deviation of a sensor signal is determined regarding the measured value, without the sensor itself being changed. Such deviation is due to design or manufacturing reasons. The deviation will be captured by the Electronic Controller. The measurement value deviation is then corrected electronically through a calculated calibration factor.

3.4 Copyright Notice

ALL RIGHTS RESERVED

PROPERTY OF CONTINENTAL TEVES AG & CO. OHG

Any reproduction, disclosure or use of this information is strictly forbidden without previous written authorization by Continental. However, this does not apply should the reproduction, disclosure or use be made with respect to proper business relationship. If and only when third parties are involved (as for ex. subcontracting, supplementary help), the reproduction, disclosure or use will be allowed provided the business partner submits a respective in-house keeping obligation. Continental reserves all applicable rights worldwide, also in the case of the placement of commercial property rights.

4 Specific Test Conditions

4.1 Electric / Electronic

The tests must be carried out with a ready for operation Electronic Controller in the diagnosis mode. This requires:

- Completely assembled ESC components

- Wiring connector connected to the Electronic Controller

- The controller voltage must be between 10.0 V and 16.0 V

- The external power source must remain on after the Controller work cycle has ended.

- The test device must be connected to the Electronic Controller through the serial interface

- The ignition must be kept on during the entire test period (in some test steps may be necessary to start the vehicle engine)

- The Electronic Controller’s failure memory must be cleared (in vehicles equipped with ESC System, the failure memory must be cleared after the steering angle sensor calibration).

- The Electronic Controller must be coded.

Caution: The electronic system can be damaged with the presence of ESD.

4.1.1 Hydraulic

The ESC hydraulic system must be operational, this means a thoroughly connected, desired and filled-up brake system.

Caution: The hydraulic system can be damaged if the pump runs dry.

Important: Assembly instruction for filling the brake system: In some tests there is a brake pressure build-up in the wheels. During such periods, no work must be carried out in the brake system.

4.1.2 Test Temperature

Piping temperature on the piping being tested +10°C up to +50°C

Ambient temperature +10°C up to +40°Cmag

4.1.3 External Wheel Drive

During the tests the wheels can be driven externally

a) through a roll test equipment for

- Electric incorrect connection test

- Hydraulic incorrect connection test

- Signal evaluation

- Brake pressure measurement in individual wheels

b) manually, on the elevator (only for incorrect connection test).

The procedures for the different test methods must be referred to in the respective test step.

4.2 Test Device Requirements

- The test device must be connected to an ISO14229 Diagnosis Interface.

- The evaluation and/or analysis of the data transmitted by the Electronic Controller during the respective test is carried out by the test device.

4.3 Deviations from this Specification

This specification establishes possible vehicle test procedures using the Continental Skid Control System. Depending on the test site and on the arrangement of the respective test technique, some of the test steps prescribed here can be omitted (see Table).

| Test Step | Need |
| --- | --- |
| Sensor signal test in pre-assembly | Needed if no tooth pitch test was carried out during ZP8 |
| Commissioning and tests in completely assembled vehicle | Needed |
| Dynamic tests | Needed |

5 Sensor Signal Test in Pre-assembly

5.1 Sensor Signal Evaluation

Test feature:

The square-shaped signal of the active sensors is detected and saved as binary values in the work memory of the microprocessor system.

The signal frequency created by the sensor element is therefore correct and allowable if the value in Hz equals the (dimensionless) count of the encoder pitch (pole pairs), which crosses the sensor element in the period of 1 second.

The signal frequency range is of:

0 Hz < fs < 2,500 Hz

Test condition:

The wheel hub can be freely rotated. The sensor signal is detected by an appropriate measuring system and evaluated. If nothing is determined in a specific test point, the following test conditions must be ensured:

Test temperature:

Ambient temperature (23°C ± 5°C)

Power source:

12.0 V (= direct current)

Terminal voltage UB:

12.0 V ± 0.1 V (= direct current)

The rotation direction of the wheel and/or of the magnetized encoder is of free choice.

The following test connections must be provided for the operation of the active sensors:

5.2 Test with Low Input Voltage

Test feature:

The sensor power source voltage falls to 4.5V. Under this condition, it must be checked whether all pole pairs can be detected.

Test method:

This test must be carried out according to the prescribed conditions. If a rotating encoder is used, the signal of current IS must be taken as a time function. Forty-eight pole pairs must be detected, distributed over the encoder ring range.

Test condition:

Operating voltage: UB = 4.2 V ± 0.1 V, direct current

Encoder frequency: 100 Hz ± 2%

Specified value:

All pole pairs are later detected

5.3 Signal Quality Check

Test feature:

The active sensors are supplied with voltage and, during the operation, deliver a signal current which frequency depends on the speed and which float between two constant levels depending on the magnetization direction of the encoder plates. Both current levels must be checked.

Test method:

This test must be carried out according to the prescribed conditions. If a rotating encoder is used, the signal of current IS must be taken as a time function. The maximum and minimum signal current must be checked against the specified values given below.

Test condition:

Operating voltage: UB=12.0 V ± 0.1 V direct current

Encoder frequency: 100 Hz ± 2%

Specified values:

IH = 11.8 mA - 16.8 mA

IL = 5.9 mA - 8.4 mA

Figure 1: Signal quality of the active sensor

6 Filling and After Work Deareration

Refer to the accompanying document ATE N 520 24.70 Page 117

a) Chapter 5.5 for high vacuum filling with available diagnosis routine

b) Chapter 6.5 for after work dearation with available diagnosis routine

7 Commissioning and Test on Completely Assembled Vehicle

7.1 Basic Configurations and Sensor Comparison

7.1.1 Control Module Coding (T6, Caddy 4, Amarok)

Test feature:

Only the current official release table must be used for parameter coding during production. The release table part number and date must be taken from the parts list and/or the installation standard (VBV).

Test condition:

Ignition ON, Electronic Controller initialized

Test procedure:

| Preparation | Diagnosis Service | Values |
| --- | --- | --- |
| Write VIN | [$2E, $F190] | After VBV |
| Control Module coding | [$2E, $0600] | After VBV |

Result:

In case of successful coding, the failure entry „Control Module incorrectly coded (byte coding) no indication“ is deleted from the failure memory.

7.1.2 Basic Configuration of Brake Pressure Encoder (T6, Caddy 4, Amarok)

Test feature:

The pressure sensor offset value is saved in the Electronic Controller.

Test condition:

Ignition ON, Electronic Controller initialized

Brake pedal not actuated

Test procedure:

Carry out the pressure sensor zero point check.

| Preparation | Diagnosis Service | Routine result for successful performance |
| --- | --- | --- |
| Calibration | [$31,$01,$040A] | [$71, $03, $040A, $02, $FFFF] |

7.1.3 Basic Configuration of Steering Angle Encoder (T6, Amarok)

Test feature:

The steering angle encoder offset value is saved in the Electronic Controller.

Test condition:

Ignition ON, Electronic Controller initialized

Brake pedal not actuated

Test procedure:

Carry out the pressure sensor zero point check.

| Preparation | Diagnosis Service | Routine result for successful performance |
| --- | --- | --- |
| Calibration | [$31,$01,$0416] | [$71, $03, $0416, $02, $FFFF] |

Result:

In case of successful calibration, the failure message „No or incorrect basic configuration/adaptation of steering angle ESP encoder” is reset.

7.1.4 Basic Configuration of the ESC Sensorik (T6, Caddy 4, Amarok)

Test feature:

The offset value of the lengthwise and traverse acceleration sensor regarding the zero position is saved in the Electronic Controller.

Test condition:

Ignition ON, Electronic Controller initialized

Vehicle stopped over a horizontal surface

Test procedure:

Carry out the zero point check of the lengthwise and traverse acceleration sensors.

In case of successful calibration, the failure entries „No or incorrect basic configuration of the lengthwise acceleration sensor “ and „ No or incorrect basic configuration of the transverse acceleration sensor “ are deleted from the failure memory.

Test result:

| Preparation | Diagnosis Service | Routine result for successful performance |
| --- | --- | --- |
| Calibration | [$31, $01, $0409] | [$71, $03, $0409, $02, $FFFF] |

7.1.5 Basic Configuration of Tire Control Monitor (if installed: T6, Caddy 4)

Test feature:

The Tire Control Monitor must be reset to the manufacturing configuration.

Test condition:

Ignition ON, Electronic Controller initialized

Test procedure:

Activate the basic configuration „Reset the Tire Control Monitor to the manufacturing configuration“ in the ESC Control Module.

Test result:

| Preparation | Diagnosis Service | Routine result for successful performance |
| --- | --- | --- |
| Resetting to the manufacturing configuration | [$31, $01,$0413] | [$71, $03, $0413, $02, $FFFF] |

7.2 Static Tests

7.2.1 ECU Switch-On Test und ABS Self-Test

Test feature:

Visual test to check if the Warning and Operation Lights (WL) turn off when the ignition is turned on and turn on again after the controller initialization phase (time of approx. 2 sec.).

Test method:

Visual lamp control

Test condition:

Ignition ON

Parking brake NOT actuated and/or hand brake not pulled.

Result:

1. After the ignition is turned on, the following warning lights turn on:

|  | T6 | Caddy 4 | Amarok GP |
| --- | --- | --- | --- |
| Brake warning light | ON | ON | ON |
| ABS warning light | ON | ON | ON |
| ASR/ESC warning light | ON | ON | ON |
| ESC off warning light | OFF | OFF | OFF |
| RKA light | ON | ON | ------- |
| HDC light | OFF | ------- | ------- |

2. Warning lights turn on after the self-test within 2s.

7.2.2 Electronic Controller Identification

Test feature:

Visual test to check if the Warning Lights (WL) turn off when the ignition is turned on and turn on again after the controller initialization phase (time of approx. 2 sec.).

Test condition:

Ignition ON, Electronic Controller in diagnosis mode

Test method:

Selection of the test program according to the serially downloaded data

Test procedure:

After sending the request block "Read Control Module identification " the Electronic Controller answers with the requested identification data.

Result:

Control Module identification data according to VBV.

7.2.3 Brake Light Sensor Test

Test feature:

The brake light switch function is tested. This switch is certainly no longer directly connected to the ESC Control Module, but handled through CAN messages.

Test condition:

Ignition ON

Test method:

The states "Brake pedal not actuated" and/or "Brake pedal actuated" match the respective bit codes emitted by the Electronic Controller, which are interpreted by the Test and/or Communication system employed.

Test procedure:

Actuate the brake pedal and/or loosen it again.

Read the measurement value block.

Test result:

| Diagnosis Service | [$22,$1820] CAN input signal Byte 3 – Bit 4-5 |
| --- | --- |
| Brake actuated | $01 |
| Brake not actuated | $00 |
| Failure | $02 |

7.2.4 Test of the ESC-Off button and of the ASR/ESC and ESC-Off warning lights

Test feature:

The ASR and/or ESC-ON/OFF button and the ASR and/or ESC warning lights are checked.

Test condition:

Ignition ON, Electronic Controller initialized (all warning lights off)

Test method:

Actuation of the ASR and/or ESC ON/OFF button and visual lamp control.

Test result:

| Diagnosis Service | [$22,$1814] Button Byte 3 – Bit 6 and 7 |
| --- | --- |
| Button actuated | $01 |
| Button not actuated | $00 |
| Failure | $02 |
| Button obstructed | $03 |

7.3 Sensor and Control Cursor Tests

7.3.1 Steering Angle Sensor Test

Test feature:

Signal reception check from the steering angle sensor

Test condition:

- Ignition ON, Electronic Controller initialized

- Steering angle sensor initialized

- Steering wheel on the straight ahead position

Test procedure:

Read sensor signal.

Test result:

| Preparation | Diagnosis Service | Values |
| --- | --- | --- |
| Read sensor signal | [$22, $1804] Skid control system Sensorik Byte 13 and 14 | Move to 0o |

7.3.2 Zero Point Accuracy Test of the Steering Rate Sensor

Test feature:

The steering rate sensor function is checked.

Test condition:

Vehicle stopped over a horizontal surface

Ignition ON, Electronic Controller initialized

Test procedure:

Read the sensor signal raw values with the function “Read measurement values of steering angle, transverse acceleration and steering rate sensor".

Result:

The measured steering rate with the vehicle stopped must not exceed 3.5°/s.

| Preparation | Diagnosis Service | Values |
| --- | --- | --- |
| Read sensor signal | [$22, $1804] Skid control system Sensorik Byte 3 and 4 | -3.5o ……. 3.5o |

7.3.3 Zero Point Accuracy Test of the Transverse Acceleration Sensor

Test feature:

The transverse acceleration sensor function is checked.

Test condition:

Vehicle stopped over a horizontal surface, Ignition ON, Electronic Controller initialized

Test procedure:

Read the sensor signal raw values with the function “Read measurement values of steering angle, transverse acceleration and steering rate sensor".

Specified value:

The measured transverse acceleration with the vehicle stopped must not exceed + 1.5 m/s2. The measurement value information is given in absolute values, which must be calculated as follows:

| Preparation | Diagnosis Service | Values |
| --- | --- | --- |
| Read sensor measurement value | [$22, $1804] Skid control system Sensorik Byte 5 and 6 | -1.5 m/s2 ……. 1.5 m/s2 |

7.3.4 Zero Point Accuracy Test of the Lengthwise Acceleration Sensor

Test feature: The lengthwise acceleration sensor function is checked.

Test condition: Vehicle stopped over a horizontal surface, Ignition ON, Electronic Controller initialized.

Test procedure: Read the sensor signal raw values with the function “Read measurement values of the lengthwise acceleration sensor ".

Result: The measured transverse acceleration with the vehicle stopped must not exceed  + 1.5 m/s2. The measurement value information is given in absolute values, which must be calculated as follows:

| Preparation | Diagnosis Service | Values |
| --- | --- | --- |
| Read sensor signal | [$22, $1804] Skid control system Sensorik Byte 9 and 10 | -1.5 m/s2 ……. 1.5 m/s2 |

7.3.5 Zero Point Accuracy Test of the Brake Pressure Encoder

Test feature:

The pressure sensor function is checked.

Test condition:

Ignition ON, Electronic Controller initialized, brake pedal not actuated

Test procedure:

Read the sensor signal raw values with the function “Read the pressure sensor measurement values".

Result:

The pressure measured on the depressurized brake system must not exceed + 7.0 bar.

| Preparation | Diagnosis Service | Values |
| --- | --- | --- |
| Read sensor signal | [$22, $1816] Other sensors Byte 4 and 5 | + 7.0 bar ……. 7.0 bar |

7.3.6 Emergency Brake Monitor Function Test

Test feature:

The Emergency Brake Monitor Function is checked.

Test condition:

Ignition ON, Electronic Controller initialized,

Diagnosis communication to the external Diagnosis Tester active

Controller coded

Test procedure:

| Preparation | Diagnosis Service | Routine result for successful performance |
| --- | --- | --- |
| Activate Emergency Brake Monitor | [$31,$01,$0415] | [$71, $03, $0415, $01, $0001 or $FFFF] and/or optical control of the warning blinking system |
| Stop Emergency Brake Monitor | [$31,$02,$0415] | [$71, $03, $0415, $02, $0001 or $FFFF] and/or optical control of the warning blinking system |

7.3.7 Brake Light Activation

Test feature:

The brake light activation by the position cursor test must be checked.

Test condition:

Vehicle stopped, Ignition ON, vehicle in Diagnosis Mode, valid access permission code (numeric)

Test procedure:

Activate the brake lights through the diagnosis service a) with the respective selectable activation time. After the adjusted time for the brake light activation is elapsed, the brake lights turn on again.

Alternatively, deactivate the brake lights through the diagnosis service b) deactivate.

| Preparation | Diagnosis Service | Result |
| --- | --- | --- |
| Activate Brake Light | [$2F, $03, $0265, $FF, $000001] | Brake Lights Activated |
| Deactivate Brake Light | [$2F, $03, $0265, $FF, $000000] | Brake Lights Deactivated |

8 Dynamic Tests

8.1 Hydraulic Incorrect Connection Test by Hand (research shop, customer service)

Test feature:

After installing the HCU in the vehicle, it must be ensured, for correct operation, that the arrangement of the hydraulic connections to the wheel brakes is correct.

Test condition:

Ignition on, engine running,

Electronic Controller in Diagnosis Mode for End of Line Test

Test method:

Prevention of pressure build-up in the driven wheel. Sequential procedure for each individual wheel.

Test procedure:

1. Start the individual wheel test using the respective diagnosis service

2. Actuation and keeping the brake pedal actuated

3. Braking torque test at the selected wheel. It is necessary for a helper to rotate the respective wheel manually and confirm that it remains free to rotate. Block the other three wheels.

4. End the individual wheel test using the respective diagnosis service

Result:

| Wheel | Diagnosis Service | Result |
| --- | --- | --- |
| VL | [$2F, $02 58, $03, $3C, $00 00 01] | Wheel turns freely |
| VR | [$2F, $02 5A, $03, $3C, $00 00 01] | Wheel turns freely |
| HL | [$2F, $02 5C, $03, $3C, $00 00 01] | Wheel turns freely |
| HR | [$2F, $02 5E, $03, $3C, $00 00 01] | Wheel turns freely |

8.2 Hydraulic Incorrect Connection Test on the Roll Test Equipment for ESC

Test feature:

The function of the ABS valves and hydraulic connection piping to the wheel ring are checked for leaks and correct arrangement.

Test condition:

Ignition on, engine running, Electronic Controller in Diagnosis Mode for End of Line Test

Test method:

Activate the valves and pump and evaluate using the test equipment computer.

Test procedure:

The diagnosis service, better described in the following chapter, has to be configured individually for each wheel on the rolls and started in a free choice sequence. An automatic procedure is started according to figure 2.

- Note:

The service can be interrupted by actuating the brake pedal.

- Note:

The incorrect connection routine makes not plausible the behavior of the other three wheels.

Should there actually be an incorrect connection of the hydraulic piping, this must be acknowledged by the test equipment.

Figure 2: Incorrect connection test – Line end procedure ESC

Legend:

Geschwindigkeit = Speed

Druckpuls = Pressure pulse

ΔV nach Druckabbau = ΔV after pressure release

Druckabbau = Pressure release

Zeit = Time

Anlegepuls = Applied pulse

Pumpe = Pump

Ein = On

Aus = Off

8.2.1 Individual Wheel Test Procedure

Start with the Diagnosis Service: [$31, $01,$03AF, W T1 T2 MP T3 dV_high dV_low T4]

Therefore:

| Variable | Description | Conversion |
| --- | --- | --- |
| W | Wheel indicator with value range [$01…$04] Here the routine is informed about which wheel is to be tested. | $01= front left $02= front right $03= rear left $04= rear right |
| T1 | Applied pulse with value range [0ms …400ms] and/or [$00 … $28] The first applied pulse is used to overcome the free play, so further pressure pulses can already operate in a „pre-stressed“ situation. The time for such pressure pulse is therefore normally longer than for all following pulses. | 1 LSB = 10 ms |
| T2 | Pressure pulse with value range [0ms …200ms] and/or [$00 …$14] Depending on the brake configuration, as for ex., volume capacity, it is possible to adjust here the pressure pulse’s pulse width. | 1 LSB = 10 ms |
| MP | Max. quantity of pressure pulses with value range [$00…$0A] Diagnosis Service interruption criteria. When the adjusted number of pressure pulses is reached, the Diagnosis Service is interrupted with not OK. | $01 = 1 … $0A = 10 |
| T3 | Setting time in value range [0ms …1000ms] and/or [$00 …$64] The time period after the pressure pulse is used, among others, to dissipate the system reaction to the pressure build-up, and also for the measurement of the speed reduction. | 1 LSB = 10 ms |
| dv_high dv_low | Speed reduction in value range [0.1km/h…..20 km/h] and/or [$0A …$07D0] Test criterion for the Diagnosis Service. When the adjusted speed reduction is reached, no other pressure relief pulse is created and the routine ends with the pressure relief. | 1 LSB = 0.01 km/h |
| T4 | Waiting time after pressure relief for the measurement of speed control (remaining braking torque) [0ms …1000ms] and/or [$00 …$64] The time period after the pressure relief is used, among others, to dissipate the system reaction in order to maintain the speed value after brake release. | 1 LSB = 10 ms |

Test result:

The Diagnosis Service ends, after the normal end of cycle, with the respective Status message, which is better described in the next Chapter.

8.2.2 Process Value Evaluation for Individual Wheels

With Diagnosis Service: [$31, $03,$03AF] can receive evaluation data from the Control Module after the conclusion of a wheel test.

The measurement data evaluation must be performed in the test equipment. The measurement values received are the following:

| Data Byte | Description | Conversion |
| --- | --- | --- |
| 1 | Routine status | 0x01 = Routine running 0x02 = Routine not running or stopped 0x03 = Routine stopped |
| 2 | Routine result | $0000 = Result with errors $0001 = No result available $FFFF = Correct result |
| 3 |  |  |
| 4 | Speed limit reached | 0x00 = Correct 0x0F = Error, routine stopped 0xFD = Routine running and/or not started 0xFE = Brake pedal actuated 0xFF = Speed reduction not reached |
| 5 | Quantity of necessary pressure pulses | $01 = 1 … $0A = 10 |
| 6 | Speed reduction after applied pulse | 1 LSB = 0,01 km/h |
| 7 |  |  |
| 8 | Speed reduction after pulse 1 | 1 LSB = 0,01 km/h |
| 9 |  |  |
| 10 | Speed reduction after pulse 2 | 1 LSB = 0,01 km/h |
| 11 |  |  |
| 12 | Speed reduction after pulse 3 | 1 LSB = 0,01 km/h |
| 13 |  |  |
| 14 | Speed reduction after pulse 4 | 1 LSB = 0,01 km/h |
| 15 |  |  |
| 16 | Speed reduction after pulse 5 | 1 LSB = 0,01 km/h |
| 17 |  |  |
| 18 | Speed reduction after pulse 6 | 1 LSB = 0,01 km/h |
| 19 |  |  |
| 20 | Speed reduction after pulse 7 | 1 LSB = 0,01 km/h |
| 21 |  |  |
| 22 | Speed reduction after pulse 8 | 1 LSB = 0,01 km/h |
| 23 |  |  |
| 24 | Speed reduction after pulse 9 | 1 LSB = 0,01 km/h |
| 25 |  |  |
| 26 | Speed reduction after pulse 10 | 1 LSB = 0,01 km/h |
| 27 |  |  |
| 28 | Speed reduction after pressure reduction | 1 LSB = 0,01 km/h |
| 29 |  |  |
| 30 | Failure status | 0x00 = No error 0x01 = Stopped due to ECU error 0x02 = reserved 0x03 = Wrong parameter sent 0x04 = Stopped by Diagnosis Service 0x05 = Brake pedal actuated |

8.2.3 Adjustment Instructions

Applied pulse T1:

Time T1 must be optimized in the process, so a speed reduction after the applied pulse of [0km/h….3km/h] will be determined as an average over more vehicles. Different brake configurations (16“, 17“, etc.) must be here considered.

A so „pre-tensioned“ brake system  should have a relatively linear behavior in the pressure relief behavior regarding the following pressure pulses.

Recommended start values:

T1(FA): 120 ms

T1(RA): 100 ms

Pressure pulse T2:

Depending on the brake configuration, the pressure relief behavior will be somewhat different. However, different pressure reduction gradients will be observed at the front and/or rear axles – they are slightly different:

The following assumptions were established in the automobile sector:

RA: approx. 2.7 bar/10 ms

FA: approx. 2,2 bar/10 ms

Recommended start values for the pressure pulse period:

T2(FA) = T2(RA) = 20 ms for a pressure step of approx. 5 bar per pulse

Figure 3: Pressure relief gradient (typical) for MK100 ESC

Legend:

Typical pressure relief behavior in 1 wheel RT 90oC (tire temp.)

Hinterachse = Rear axle

Vorderachse = Front axle

Setting time T3:

For the setting time, test equipment-specific influences are decisive (type of roll adjustment, roll own friction, etc.).

Recommended start values taken from actual values of other customer projects:

T3: 300 ms

Waiting time T4:

For the setting time, test equipment-specific influences are decisive (type of roll adjustment, roll own friction, etc.).

Recommended start values taken from actual values of other customer projects:

T4: 300 ms

Speed Reduction dV:

Test equipment-specific influences are also decisive here (type of roll adjustment, roll own friction, etc.). Here it is possible to determine an appropriate project-specific value that will be reached with all brake configurations within the allowable 10 pressure pulses.

8.3 Tooth Pitch Test

Test feature:

If no checking of the sensor function is made during the axle assembly, according to Chap. 5, or a supplement of such test, the correct total clearance of the connected encoder wheel <-> rotation sensor <-> ESC Control Module can be checked.

Test condition:

Ignition on, engine running,

Electronic Controller in Diagnosis Mode for End of Line Test

Test method:

Roll operation, reading and evaluating of diagnosis measurement values.

Test procedure:

v = 10…30km/h, forward, constant speed. Reading of measurement values for the rotation sensor.

Test result:

| Preparation | Diagnosis Service | Response Values |
| --- | --- | --- |
| Tooth Pitch Test Results FL Tooth Pitch Test Results FR Tooth Pitch Test Results RL Tooth Pitch Test Results RR | [$22, $1800] [$22, $1801] [$22, $1802] [$22, $1803] | Average pitch value Minimum pitch value Maximum pitch value respectively individual for FL, FR, RL, RR |

The pitch value response values average/minimum/maximum provide measurement values in the physical operation range between 0 ms < Value < 6.55 ms.

With

p = Normal accuracy of the roll test equipment in % and/or 1/100

A maximum deviation of± (5% + p) is established as the test tolerance.

This results in the following tolerances for the Tooth Pitch Test.

Minimum pitch value ≥ Average pitch value * (0,95 – p)

Maximum pitch value ≥ Average pitch value * (1,05 + p)

These values depend on the selected roll speed as well as, eventually, the axle configuration – mainly the quantity of pole pitches of the installed encoder wheel, which therefore shall be determined and established during production.

9 Valve Calibration in ESC Unit (only for develoment and/or repair purposes)

9.1 Equipment Requirements

9.1.1 Electric / Electronic

- The tests must be carried out on a ready for operation Electronic Controller in Diagnosis Mode. This requires:

- The electronic ESC components to be completely assembled.

- The wiring connector is connected to the Electronic Controller.

- The Electronic Controller is coded.

- The power source voltage at the controller lies between 11.0V and 16.0V.

- Overlapping alternating voltage of the power source +/- 2.5V.

- Transient switch on/ switch off voltage peaks and voltage interruptions of > 0,5 Volt are allowed for a maximum of 300 µsec at switch on/ switch off of consumers.

- The resulting total voltage, including the overlapping alternating voltage / Peaks/

- Interruptions must be within the defined power source voltage ranges at all times.

- The limiting conditions for the power source voltage are valid for both stopped and running engine as well.

- Note:

If possible, it is better to turn off all consumers before calibration. The switching status of the consumers must not be changed during calibration.

The ignition must be off during the connection and/or disconnection of the ECU connector!

9.1.2 Hydraulic

The hydraulic system must be ready for operation, this means having the brake system completely connected, filled, deaerated and positively checked for leaks.

Caution:

The hydraulic system can be damaged if the pump runs dry!

9.1.3 Vehicle

Because no brake pressure can be created in the brake system during calibration, the vehicle must be secured against moving. The parking brake must be actuated. If this is not possible, the vehicle must be secured by braking wedges.

9.1.4 Temperature

HECU temperature: +10°C to +70°C

The respective temperature must be held constant during calibration.

- Note:

Temperatures under +10°C are not allowed because the brake fluid viscosity has a significant influence on the calibration. (The vehicle must be kept for at least two hours in a heated shop if minus temperatures took place during the night!)

9.2 ESC-Valve Calibration in the Vehicle

Calibration task: Valve calibration is necessary to ensure a correct pressure adjustment during normal braking.

Calibration method:

The characteristic curve is programmed for the analogical operation of the intake valve and the isolation valve. Therefore, the valve opening point is determined in a depressurized state.

The calibration is carried out in two independent steps. In case of single activation the sequence has no importance.

Calibration conditions:

- Brake system is completely assembled, filled and without leaks (see filling instructions).

- Ignition ON, Electronic Controller initialized.

- Basic encoder configuration (zero point adjustment) for brake pressure available.

- Diagnosis interface ready for communication.

- Voltage peaks must be avoided and during the calibration procedure no electric consumer shall be turned on or off

- Wheels must not rotate.

- Work step monitoring is carried out by the shop diagnosis equipment.

- Note:

No work on the brake system shall take place during valve calibration!

- Note:

The routine status can be checked at any moment through the services "Intake valve calibration status ", and/or "Isolation valve calibration status" (regardless if any calibration routine was started).

9.2.1 Determination of HECU Calibration Status

The calibration status can be inquired through a diagnosis message to the status inquiry.

The response values are:

Routine Status,

Routine Result,

Error

with meaning according to the following table:

9.2.1.1 Isolation and Intake Valve calibration OK

| Preparation | Diagnosis Service | Expected Control Module Answer |
| --- | --- | --- |
| Read calibration status | [$31, $03, $0469] | RoutineStatus: 0x00 RoutineResult: 0x02 or 0x03 Error: 0x00 |

Vehicle is ready for the ABS/ESC operation.

9.2.1.2 Valve Calibration not OK

| Preparation | Diagnosis Service | Expected Control Module Answer |
| --- | --- | --- |
| Read calibration status | [$31, $03, $0469] | RoutineStatus: 0x00 RoutineResult: 0x01, 0x04 , 0x05 Error: 0x20 – 0xC0 |

In this state, an active valve calibration failure entry is made in the failure memory. The vehicle is not ready for ESC operation.

Valve calibration must be performed. Depending on the repair note shown in the failure memory entry „Brake control module: Valve calibration failure“, the Intake Valve and/or the ASR Isolation Valve calibration, or both, must be performed.

9.2.2 Intake Valve Calibration

| Preparation | Diagnosis Service | Expected Control Module Answer |
| --- | --- | --- |
| Intake valve calibration | [$31, $01, $0469, $05] | Positive response |

9.2.3 Isolation Valve Calibration

| Preparation | Diagnosis Service | Expected Control Module Answer |
| --- | --- | --- |
| Intake valve calibration | [$31, $03, $0469, $0F] | Positive response |

9.2.4 Troubleshooting

| Error code | Solution |
| --- | --- |
| 0x20 – velocity too high | Check/Correct speed signal evaluation. Place the vehicle in secure stopped condition. |
| 0x30 – pressure too low | Check the encoder brake pressure signal emission with brake pedal not actuated. If necessary, carry out basic configuration again (zero point adjustment). |
| 0x50 – KL30 voltage too low | Ensure sufficient power source voltage. If necessary, leave engine running with assistance or provide temporary supplementary external power source. |
| 0x60 – temperature too high | Let hydraulic unit cool until the ESC temperature signal shows <70°C. |
| 0x70 – temperature too low | Heat hydraulic unit or, if necessary, carry out a new ignition if brake pressure encoder was adjusted in the same ignition cycle and/or in the same Diagnosis Session. |