### C.2.4 DID 0x0203-Engine Temperature

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| Engine Temperature | 1 | °C | 0.75 | 0…FEH | -48…142.5 |

### Offset:-48

### C.2.14 DID 0x020D-Intake Air Temperature

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| Intake Air Temperature | 1 | °C | 0.75 | 0…FEH | -48…142.5 |

### Offset:-48

### C.2.90 DID 0x02F8-V_TPS_1

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| V_TPS_1 | 2 | V | 0.0001526 | 8000H…7 FFFH | -5…4.99985 |

当ECU反馈的X（十六进制值）：

X>=8000，Y=（X-FFFF）*0.0001526

X<8000，Y=X*0.0001526

### C.2.11 DID 0x020A-Battery Voltage

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| Battery Voltage | 1 | V | 0.1015625 | 0…FFH | 0…25.8984375 |

### Offset:0

### C.2.9 DID 0x0208-Vehicle Speed

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| Vehicle Speed | 1 | km/h | 1 | 0…FFH | 0…255 |

### Offset:0

### C.2.5 DID 0x0204-Engine Speed

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| Engine Speed | 2 | rpm | 1 | 0…1FE0H | 0…8160 |

### Offset:0

### C.2.39 DID 0x0228-Intake Manifold Pressure (Absolute)

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| Intake Manifold Pressure (Absolute) | 2 | hPa | 0.082917524 98664835 | 0…FFFFH | 0…5434 |

### Offset:0

### C.2.83 DID 0x025B-Output Duty Cycle for Canister Purge Valve

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| Output Duty Cycle For Canister Purge Valve | 1 | % | 0.390625 | 0..FFH | 0…99.609375 |

### Offset:0

### C.2.86 DID 0x0260-O2 Sensor Voltage Bank1, Sensor 1

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| O2 Sensor Voltage Bank1, Sensor 1 | 2 | V | 0.004882812 5 | 0…3FFH | 0…4.9951171875 |

### Offset:0

### C.2.87 DID 0x0261-O2 Sensor Voltage Bank1, Sensor 2

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| O2 Sensor Voltage Bank1, Sensor 2 | 2 | V | 0.004882812 5 | 0…3FFH | 0…4.9951171875 |

### Offset:0

### C.2.33 DID 0x0222-Real Ignition Angle—点火角度

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| Real Ignition Angle | 1 | °CRK | 0.375 | 0…FFH | -35.625…60 |

### Offset:-35.625

### C.2.210 DID 0x0500-Cruise Status

| Byte No. | Description | Data Type | Value (Hex) |
| --- | --- | --- | --- |
| data #1 | Cruise Status | Hex | 0x00: Cruise OFF 0x01: Cruise ON |

### C.2.211 DID 0x0501- Cruise speed

| Data Name | #of bytes | Unit | Scaling | HEXlimits | Physicallimits |
| --- | --- | --- | --- | --- | --- |
| Cruise speed | 2 | Km/h | 0.01 | 0…FFFFH | 0…655.35 |

### Offset:0

(备注：供应商未按照五菱的精度来开发，昨天测试发现这个问题。现按照0.01处理（原精度为1）)