### C.2.19 DID 0x021E-relative air mass (calc. load value) acc. To SAE J1979

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| relative air mass (calc. load value) acc. To SAE J1979 | 1 | % | 0.390625 | 0 | 0 | 99.609375 |

### C.2.3 DID 0x0203-engine temperature

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| engine temperature | 1 | ℃ | 0.75 | -48 | -48 | 143.25 |

### C.2.46 DID 0x0245-intake manifold pressure (absolute)

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| intake manifold pressure (absolute) | 2 | hPa | 0.0390625 | 0 | 0 | 2559.960938 |

### C.2.4 DID 0x0204-engine speed

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| engine speed | 2 | rpm | 0.25 | 0 | 0 | 16383.75 |

### C.2.102 DID 0x0D01-vehicle speed

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| vehicle speed | 2 | km/h | 0.0078125 | 0 | 0 | 511.99218 |

### C.2.35 DID 0x0236-real ignition angle

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| real ignition angle | 1 | Grad KW | 0.75 | 0 | -96 | 95.25 |

当ECU反馈的X（十六进制值）：

X>=80，Y=（X-FF）*0.75

X<80，Y=X*0.75

### C.2.9 DID 0x020D-intake air temperature

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| intake air temperature | 1 | ℃ | 0.75 | -48 | -48 | 143.25 |

### C.2.37 DID 0x023A-throttle blade position dependent on poti 1

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| throttle blade position dependent on poti 1 | 2 | %DK | 0.024414063 | 0 | -800 | 799.9755859 |

当ECU反馈的X（十六进制值）：

X>=8000，Y=（X-FFFF）*0.024414063

X<8000，Y=X*0.024414063

### C.2.106 DID 0x0F01-Duty cycle for carbon

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| Duty Cycle for Carbon | 1 | % | 0.390625 | 0 | 0 | 99.609375 |

### C.2.86 DID 0x02AA-O2 sensor voltage bank1, sensor 1

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| O2 sensor voltage bank1, sensor 1 | 2 | V | 0.004882813 | 0 | 0 | 319.9951172 |

### C.2.87 DID 0x02AB-O2 sensor voltage bank1, sensor 2

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| O2 sensor voltage bank1, sensor 2 | 2 | V | 0.004882813 | 0 | 0 | 319.9951172 |

### C.2.101 DID 0x0D00-battery voltage

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| battery voltage | 2 | V | 0.000390625 | 0 | 0 | 25.59960938 |

### C.2.127 DID 0x0F16-Cruise status

| Byte No. | Description | Data Type | Value (Hex) |
| --- | --- | --- | --- |
| data #1 | Cruise status | Hex | 0x01：Cruise ON 0x00：Cruise OFF |

### C.2.128 DID 0x0F17-Cruise speed

| Data Name | #of bytes | Unit | Scaling | Offset | Min. | Max. |
| --- | --- | --- | --- | --- | --- | --- |
| Cruise speed | 2 | Km/h | 1 | 1 | 0 | 65535 |