- Test content:

- Constant speed test

- Acceleration test

- Deceleration test

- Function exist test

- Test procedure

- After speedometer test, accelerate from 40km/h to 60km/h.

- Machine prompts driver to start cruise control

- Driver starts cruise control function manually and machine reading cruise speed setting from module

- Machine reading rollers speed to compare with cruise speed reading from module. The tolerance is ±5km/h

- Machine prompts driver to decelerate speed to 50km/h

- Machine reading cruise speed setting has changed to 50km/h

- Machine prompts driver to accelerate speed to 60km/h

- Machine reading cruise speed setting has changed back to 60km/h

- Machine prompts driver to accelerate speed to brake arm speed

- Machine reading cruise control mode has been existed before drag test