### Service Programming System (SPS)

Interpreter

Programmers Reference Manual

Creation Date:	March 1, 1993

Version Date:	June 10, 2014

Authoring Group: GM Customer Care & Aftersales - Product Engineering

Document Name:

“Interpreters_June_10_2014.docx”

The information contained within this document

is proprietary to General Motors Company (GM)

# Table of Contents

Service Programming System (SPS)	1

Table of Contents	2

Overview	6

Utility File Structure	7

Format of Header Information	8

Format of Interpreter Instructions	9

Device (ECU) Specific Control Routines and/or Service Request Data Routines	10

Reprogramming Algorithm Overview	11

Additional Utility File Information - Best Practices and recommendations	12

Programming Storage Buffers – Recommended Usage	16

Interpreter 0 - UART Communications Programming Op-Codes	17

UART Support for No Communication Faults	18

UART Return Type Processing	19

00 Op-Code: Enable Normal Communications	20

01 Op-Code: Request message from the device	21

02 Op-Code: Request memory dump from the device using Mode 2 (Saturn EEPROM)	22

03 Op-Code: Verify programming of a EEPROM device (NAO EEPROM)	24

04 Op-Code: Device control - Mode 4	25

05 Op-Code: Ram download request - Mode 5	27

08 Op-Code: Disable Normal Communications	28

09 Op-Code: Enable Normal Communications	29

0A Op-Code: Clear Trouble Codes - Mode 10	30

0C Op-Code: Program EEPROM - Mode 12	31

0D Op-Code: Perform Security - Mode 13	33

20 Op-Code: Receive a normal mode message from device	34

A3 Op-Code Read Data from Specified Sequential Addresses (Electric Vehicle) Mode 3	35

C0 Op-Code: Download Routine to RAM and Execute using Mode 6	36

C1 Op-Code: Download Routine to RAM and Execute using Mode 16	38

C2 Op-Code: Download Routine to RAM and Save Two Byte Response using Mode 6	40

C3 Op-Code: Download Routine to RAM and Execute using Mode 6 (Electric Vehicle)	41

C4 Op-Code: Download Routine to RAM using Mode 20 (Holden Telematics)	43

C5 Op-Code: Execute Routine using Mode 21 (Holden Telematics)	45

D0 Op-Code: Download Calibration / Software File using Mode 6	46

D1 Op-Code: Download Calibration or Software Module using Mode 16	48

D2 Op-Code: Download VIN using Mode 6 & 16	49

D4 Op-Code: Download Calibration / Software File using Mode 6 (Saturn EEPROM)	51

D5 Op-Code: Download Calibration / Software File using Mode 12 (Electric Vehicle)	53

D6 Op-Code: Erase Flash Block using Mode 22 (Holden Telematics)	54

D7 Op-Code: Download Software Module using Mode 23 (Holden Telematics)	55

D8 Op-Code: Execute Software Module using Mode 24 (Holden Telematics)	57

F0 Op-Code: Wait for Poll from a Device	58

Interpreter 1 - Class 2 Communications Programming Op-Codes	59

Class 2 No Communications Processing	60

Class 2 Response Processing	61

01 Op-Code: Setup Global Variables	63

10 Op-Code: Initiate Diagnostic Operation	64

14 Op-Code: Clear Diagnostic Information	65

20 Op-Code: Return to Normal Mode	66

27 Op-Code: Perform Security - Mode 27	67

28 Op-Code: Disable Normal Communications - Mode 28	70

29 Op-Code: Resume Normal Communications - Mode 29 (Obsolete Use Op-Code 20)	71

31 Op-Code: Start a Diagnostic Routine by Test Number	72

34 Op-Code: Request the Control Module for Block Transfer - Mode 34	73

3B Op-Code: Write block of memory	75

3C Op-Code: Read and verify block of memory	82

3E Op-Code: Modify Block of Memory	84

40 Op-Code: Read/Write Data Buffer	86

52 Op-Code: Write Checksum to Block 82 / 8E	89

72 Op-Code: Set Programming Voltage	91

84 Op-Code: Set Communications Parameters (Add Delay)	92

A0 Op-Code: Request High Speed Communications - Mode A0	93

A1 Op-Code: Begin High Speed Communications - Mode A1	94

A8 Op-Code: GM Propriety Test Mode	95

AE Op-Code: Request Device Control - Mode AE	96

B0 Op-Code: Block Transfer to RAM and Execute - Mode 36	97

B2 Op-Code: Block Transfer to RAM and Save Two Byte Response - Mode 36	100

B3 Op-Code: Block Transfer to Download VIN as a Calibration - Mode 36	101

B4 Op-Code: Block Transfer to Download Calibration File - Mode 36	103

Interpreter 2 KWP2000 Communications Programming Op-Codes	106

KWP2000 No Communications Processing	107

KWP2000 Response Processing	108

01 Op-Code: Setup KWP2000 Programming (Target and Source Bytes)	110

03 Op-Code: Set Number of Repetitions	111

10 Op-Code: SR 10 to Start Diagnostic Session	112

11 Op-Code: SR 11 ECU Reset	114

14 Op-Code: SR 14 Clear Diagnostic Trouble Codes	115

20 Op-Code: SR 20 to Stop Diagnostic Session	116

23 Op-Code: SR 23 to Read Memory by Address and Save Bytes	117

27 Op-Code: SR 27 for Security Access Request	119

30 Op-Code: SR 30 to Input/Output Control by Local Identifier	121

31 Op-Code: SR 31 to Start Routine by Local Identifier	122

32 Op-Code: SR 32 to Stop Routine by Local Identifier	123

33 Op-Code: SR 33 to Request Routine Results by Local Identifier	124

34 Op-Code: SR 34 for Download Request	125

37 Op-Code: SR 37 for Request Transfer Exit	127

38 Op-Code: SR 38 to Start Utility File Routine by Address	128

39 Op-Code: SR 39 to Stop Utility File Routine by Address	130

3A Op-Code: SR 3A to Request Routine Results by Address	132

3B Op-Code: SR 3B to Write Data by Local Identifier	133

3D Op-Code: SR 3D to Write Memory by Address	135

71 Op-Code: SR 31 to Start Routine by Local Identifier and Save 2 Bytes	137

78 Op-Code: SR 38 to Start Utility File Routine by Address and Save 2 Bytes	139

81 Op-Code: SR 81 to Start Communications	141

82 Op-Code: SR 82 to Stop Communications	142

83 Op-Code: SR 83 to Access Communications Parameters	143

84 Op-Code: SR 83 to Set Communications Parameters	145

90 Op-Code: SR 36 to Transfer a Routine to ECU	146

93 Op-Code: SR 36 to Transfer a Calibration File to ECU	149

Interpreter 3 - GMLAN Communications Programming Op-Codes	153

GMLAN No Communications Processing	154

GMLAN Response Processing	155

01 Op-Code: Setup Global Variables	157

10 Op-Code: Mode 10 Initiate Diagnostic Operation	158

14 Op-Code: Mode 04 Clear DTC's	159

1A Op-Code: Mode 1A Read Data by Identifier	160

20 Op-Code: Mode 20 Return To Normal Mode	161

22 Op-Code: Mode 22 Read Data by Parameter Identifier (PID)	162

25 Op-Code: Mode AE Security Code	163

27 Op-Code: Mode 27 Security Access	164

34 Op-Code: Mode 34 Request Download	167

3B Op-Code: Mode 3B Write Data by Identifier	169

84 Op-Code: Set Communications Parameters	172

A2 Op-Code: Mode A2 Report Programmed State and Save Response	173

AA Op-Code: Mode AA Read Data by Packet Identifier	174

AE Op-Code: Mode AE Request Device Control	176

B0 Op-Code: Mode 36 Block Transfer to RAM	177

Interpreter 4 – UDS Communications Programming Op-Codes	180

UDS No Communications Processing	181

UDS Response Processing	182

01 Op-Code: Setup Global Variables	183

10 Op-Code: Mode 10 Diagnostic Session Control	184

11 Op-Code: Mode 11 ECU Reset	185

22 Op-Code: Mode 22 Read Data by Identifier	186

27 Op-Code: Mode 27 Security Access	187

2E Op-Code: Mode 2E Write Data by Identifier	189

3B Op-Code: Mode 2E Write Data by Identifier from Internal Storage Buffer	191

2F Op-Code: Mode 2F Input/Output Control By Identifier (Device Control)	192

31 Op-Code: Mode 31 Routine Control	193

34 Op-Code: Mode 34 Request Download	194

36 Op-Code: Mode 36 Transfer Data	196

37 Op-Code: Mode 37 Request Transfer Exit	198

Communications Independent Programming Op-Codes	199

50 Op-Code: Compare Bytes	200

51 Op-Code: Compare Checksum	201

53 Op-Code: Compare Data	202

54 Op-Code: Change Data	204

55 Op-Code: Evaluate RPO (for use in vehicle assembly plants only)	206

56 Op-Code: Interpreter Identifier	207

E0 Op-Code: End of ‘flash erase’	208

EE Op-Code: End with ERROR	209

F1 Op-Code: Set Global Memory Address	210

F2 Op-Code: Set Global Memory Length	211

F3 Op-Code: Set Global Header Length	212

F4 Op-Code: Ignore Responses for Milliseconds	213

F5 Op-Code: Override the Message Length Value	214

F7 Op-Code: No Operation Op-Code	215

F8 Op-Code: Goto Field continuation	216

FB Op-Code: Set and Decrement Counter	217

FC Op-Code: Delay for ?? Seconds/Minutes	218

FD Op-Code: Reset counter	219

FF Op-Code: End with SUCCESS	220

System Errors and Op-Code Failures	221

Procedure for Adding New Op-Codes or Exceptions to Existing Op-Codes	222

Appendixes	223

Appendix A - UART Interpreter Examples	224

Appendix B - Class 2 Interpreter Examples	225

Appendix C - KWP2000 Interpreter Example	226

Appendix D – GMLAN Utility File Guidelines	228

Revision History (Appendix D:  GMLAN Utility File Guidelines)	229

Assumptions	230

Programming Flow	231

Step $01 – Set Target and Source Address	233

Step $02 “Start”– Perform Seed and Key	234

Step $03 “SetUp” – Prepare to Download Flash Routine #1 from Utility File	235

Step $04 – Download Flash Routine #1 from Utility File	236

Step $05 – Request the Operating Software Part Number	237

Step $06 – Evaluate the Application Software Part Number Value	238

Step $07 “ProgOp” – Set the Global Header Length	239

Step $08 – Prepare to Download the Operating Software	240

Step $09 – Download the Operating Software	241

Step $0A “ProgCal” – Set the Global Header Length	242

Step $0B – Prepare to Download the First Calibration Module	243

Step $0C – Download the First Calibration Module	244

Step $0D – Prepare to Download the Second Calibration Module	245

Step $0E – Download the Second Calibration Module	246

Step $0F “ProgState” – Request the Programmed State	247

Step $10 – Evaluate the Programmed State Value	248

Step $11 “Part 2” – Write the VIN	249

Step $12 – Write the Tester Serial Number	250

Step $13 – Write the Programming Date	251

Step $14 “Retry1” – Initialize Retry counter and decrement	252

Step $15 – Retry1 Delay	253

Step $16 “Retry2” – Initialize Retry counter and decrement	254

Step $17 – Retry2 Delay	255

Step $18 “Retry3” – Initialize Retry counter and decrement	256

Step $19 – Retry3 Delay	257

Step $1A “Retry4” – Initialize Retry counter and decrement	258

Step $1B – Retry4 Delay	259

Step $1C “Success” – Exit Interpreter with Success	260

Step $1D “Error” – Exit Interpreter with Error	261

SPS Editor Output of Interpreter Instructions	262

GMLAN Message Flow Diagrams	263

Appendix E – UDS Utility File Guidelines	266

Appendix F - Revision Log   (Interpreters Document)	267

Appendix G - Related Documents	274

Appendix H – $54 Op-Code Additional Information	275

Appendix I – $55 Op-Code Additional Information - GATS Option String Processing	277

Appendix J – Plant Manufacturing VIT Data	279

# Overview

This manual is designed for developers who will be writing the interpretive sections of the Utility Files and for the system programmers who will be writing the Interpreter software. The purpose of the manual is to explain the Interpreter concept as well as instruct a user exactly how to write interpreter instructions. This document will be updated regularly, so user input is a must for maximum benefit.

The main input to the Interpreter Concept is the Utility File. Step by step instructions on how to reprogram a control module is provided by the Utility File. Utility Files were developed to keep the proliferation of tool reprogramming software to a minimum. The Utility file is viewed as three distinct sections: the Header Information (24 bytes), the Interpreter Instructions (size varies) and the device specific programming routines (a.k.a. Programming Algorithm (PA)). Even though the Utility file is viewed as three distinct parts, it will always be treated as a single entity necessary for reprogramming a Control Module.

The Interpreter Concept uses Interpreters that allow reprogramming support of new products, without having to hard code or create independent software packages. An Interpreter is a module that understands the format of Utility Files as well as the use of each of its Op-Codes. The Interpreter follows the Interpreter Instructions in the Utility files until an exit point is reached. There are two Op-Codes that will end the programming event:

If the EE Op-Code (End with Error) is executed by the interpreter, “Programming has Failed”.

If the FF Op-Code (End with SUCCESS) is executed by the interpreter, “Programming was Successful”.

NOTE: Refer to Communications Independent Programming Op-Codes

# Utility File Structure

The Utility File is one contiguous block of data consisting of three distinct sections as depicted below:

| Header Information (24 bytes) Used to setup the programming session | The Header Information defines information that remains constant during the entire reprogramming event. An example of this information is the "Type of Interpreter", once the software starts using an Interpreter it will not change to an Interpreter using another communications protocol. |
| --- | --- |
| Interpreter Instructions A set of sequentially numbered steps to control the programming session | The Interpreter Instructions are the Op-Codes that guide the terminal application through a reprogramming event. Each instruction line is 16 bytes long and consists of four sections: 1 byte Step Number, 1 byte Op-Code, 4 byte Action Field, and 10 bytes of goto fields. |
| Device (ECU) Specific Control Routines and / or Service Request Data Routine | The Device Specific Control Routines are programming routines used for performing various functions during the reprogramming event. The number of routines varies, depending on how each ECU. Examples of control routines are: erase flash memory, turn on reprogramming voltage, or read flash manufacturer and ID. Service Request Data Routines provide a means to pass additional data in Service Request. Examples of this data are Routine Entry Options and Record Values. |

Even though the Utility File is viewed as three sections, it must be handled as a single file. The following subsections, Format of Header Information, Format of Interpreter Instructions and Format of Device Specific Programming Routines, provide greater insight to the design of the three sections of the Utility File.  The routine section of the utility file is an optional section and is controller specific.

## Format of Header Information

The Utility Files will have the following header information starting at byte 00:

| 2 bytes $00-$01 | Checksum of the Utility File | Not used by interpreter, left here for commonality |
| --- | --- | --- |
| 2 bytes $02-$03 | Module ID of the Utility File | 0x0000 is the module ID for the Utility File |
| 4 bytes $04-$07 | Part Number of Utility File | Not used by interpreter, left here for commonality |
| 2 bytes $08-$09 | Utility File Design Level Suffix | Not used by interpreter, left here for commonality |
| 2 bytes $0A-$0B | Type of Header/Utility | 0x0000 for a standard single section file Header/Utility. |
|  |  | 0xXXXX starting step number for the second section. |
| 2 bytes $0C-$0D | Type of Interpreter | 00 for UART communications. 01 for Class 2 communications. 02 for KW2000 communications 03 for GMLAN communications 04 for UDS communications |
| 2 bytes $0E-$0F | Offset to routine section from byte $00 | The length of the Header plus the Interpretive code (programming instructions) section. Required even without routine data to indicate end of interpreter instructions. |
| 2 bytes $10-$11 | Type of addressing (2,3 or 4 byte) | 02 for two byte addressing or 03 for three byte addressing or 04 for four byte addressing |
| 4 bytes $12-$15 | Data address information | Data address information |
| 2 bytes $16-$17 | Number of data bytes in a message | The number of data bytes to build into each data packet. This may be over written by Op-Code instructions Note: This field is not used for UDS. UDS will determine this length based on the pos. resp. data from the $34 Request (max is 4095) – See UDS OpCode $36. |

Example Header Information

790E00000000000000000000000100A80002000020000040 (ASCII Hex Format)

| Checksum | Module ID | Part # | Design Level Suffix | Header/Utility Type | Interpreter Type | Offset | Type of Addressing | Data Address | # Of Data Bytes in Message |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 790E | 0000 | 00000000 | 0000 | 0000 | 0001 | 00A8 | 0002 | 00002000 | 0040 |

## Format of Interpreter Instructions

Each line of interpreter instructions is 16 bytes long and defined as follows:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| ## | ## | ## ## ## ## | ## ## ## ## ## ## ## ## ## ## |

Any position not used in an instruction line must be filled with zero. All numbers in the interpreter instructions must be entered as hexadecimal values. (Note: All entries made to a utility file must be in caps)

Step Field (1 byte - valid steps 01 - FF)

The Step is a sequential number, increasing by one, indicating the order of the instruction file. The Step is used as a reference for goto field branching in the instructions. The first step number in the interpretive code must be 1 (0x01) and the last step cannot be greater than 255 (0xFF). An Interpreter error will occur if a step zero (0x00) is encountered.

Op-Code Field (1 byte)

Op-Codes are assigned values that identify what type of function/service the service tool should perform.

Action Fields (4 bytes)

The Action Fields are used as parameters or indicating exceptions. By setting appropriate exceptions in the Action fields, the behavior of the Op-Code can be modified. All unused Action Fields shall be set to 0x00.

GOTO Fields (5 - 2 byte pairs)

The Goto Fields are used to determine the next step. Goto fields are defined as pairs. For Op-Codes that send a message to a control module based on the responses returned from the control module, the first byte is the response byte and the second byte is the interpreter step that is executed next. Comparison of the Goto Field pairs is done from left to right and will stop when the first matching code is found.

The Goto Field pair is read as follows:

On response return code ?? - Then_- Goto step ??

Two specific numbers FD and FF are defined to handle response codes / events.

In case of a no communications fault event the FD code will determine the next step.

The FF code is read as "ANY".  If there is a response code or event return code that is not listed in the Goto Field pairs, then the FF “Goto Field” code will determine the next step.

For Op-Codes which send a message to a control module, the FF code must be in the last Goto Field pair to handle all possible response codes / events.

NOTE:

In the case of negative response processing, it is possible to receive a negative response code that is equal to the expected service identification response for a positive response message.  To resolve this issue, it is necessary to put a duplicate response code in the goto fields: the first goto response will be for positive response messages and the second will handle the negative response messages.

For a more detailed description, see the “Response processing” section of this document.

The response return codes are the control module responses to a transmitted message. Not all Op-Codes send a message to a control module, some perform other functions (i.e. Op-Code 50 and 51). The first byte of the goto pair should be zero for Op-Codes not requiring a response from the module.

If more than five Goto Fields are needed for an interpreter instruction, do not close the step with an 'FF'. If the tool does not see an 'FF' return code and a return code has not been found, the tool continues searching for a return code in the Goto Fields of the next step in the interpreter instructions. The 'F8' Op-Code must be used on the goto continuation line (for more information see 'F8' Op-Code description).

# Device (ECU) Specific Control Routines and/or Service Request Data Routines

The Device (ECU) Specific Control Routines are pieces of code that are transferred to an ECU.  The suppliers or the Flash vendors develop the control routines.  Examples of Control Routines are: erase flash memory, checksum memory, or reset ECU.  The user has the option to include Control Routines in the Utility File or to embed the routines in the ECU's memory.

For certain protocol implementation, the user is not allowed to include Control Routines in the Utility File, the routines must be embedded in the ECU’s memory.

This entire section of the Utility File is optional.  If this section is omitted all required Control Routines shall be embedded in the ECU's memory and therefore no user optional data shall be needed for any Service Requests.  If this section is in the Utility File, it shall be formatted as follows:

The routines in the Utility File must be laid out as follows:

| 4 bytes | Address information |
| --- | --- |
| 2 byte | Length of the routine 1 (not including address and length) |
| X bytes | Routine 1 data |
| 4 bytes | Address information |
| 2 byte | Length of the routine 2 (not including address and length) |
| Y bytes | Routine 2 data |
| : | : |
| 4 bytes | Address information |
| 2 byte | Length of the routine N (not including address and length) |
| Z bytes | Routine N data. |

The routines are one contiguous block of data. They are separated only for presentation purposes.

The address can be used by the tool as a destination address (Note:  The address information could also come from the global address or the header address or might be not used at all.).  The length is used to locate the routines in the routine(s) portion of the Utility File. The length of a routine is the total number of data bytes not including the 2 byte length and the 4 byte address.

The first routine in the routine section will be referenced as routine number 1. The routine numbers are not included in the routines section of the Utility File. However, the routine number will be used in the Op-Code to call the routine. The tool will internally keep track of the routine numbers. If an Op-Code calls for routine 3, the tool will skip over the first two routines and return the data in the third routine. The tool will determine where each routine is located by using the routine lengths that precede each routine located in the routines section of the Utility File.

# Reprogramming Algorithm Overview

The Reprogramming Algorithm is not required to be part of the Utility File. An application may store its Reprogramming Algorithm within the controller software instead of downloading them into RAM from the Utility File (routine section). Applications using Flash devices that do not store the Reprogramming Algorithm in the Utility File may copy their Flash I/O routines from controller software to RAM prior to reprogramming. This is necessary because it is not possible to execute from one Flash segment while trying to erase or write to another in the same Flash device.

Using the Utility File to download the Reprogramming Algorithm offers a way to keep the reprogramming capability out of the controller software. This eliminates the danger of being able to write and erase controller software while executing normal operation controller software. The Reprogramming Algorithm is downloaded to the controller by the reprogramming tool per the Programming Instructions. The Reprogramming Algorithm is executed by the controller and should reside in Keep-Alive Memory if available.

The Reprogramming Algorithm portion of the Utility File is flash device and hardware dependent. There is no distinction between subroutines that are hardware dependent and those that are flash device dependent.

If additional suppliers exist for a reprogrammable controller or components of a controller, the Utility File must include all the necessary Reprogramming Algorithms for all suppliers of the controller and/or components.

It is typically preferred to keep the size of the Reprogramming Algorithm as small as possible when the Reprogramming Algorithm is part of the Utility File. This may be necessary due to a lack of RAM available for reprogramming or Flash reprogramming event time limitations. Reprogramming requires the use of RAM that does not conflict with locations used in the normal operation download itself to execute the Reprogramming Algorithm and to buffer the new software and/or calibrations that will be reprogrammed. The ideal situation is to have a contiguous RAM region for each purpose, which takes advantage of the maximum downloaded message size. The Reprogramming Algorithm should be as efficient and fast as possible because of time constraints (i.e. an assembly plant reprogramming environment).

The Reprogramming Algorithm typically consists of the following 5 basic functions:

- Executive Routine -

The communications executive logic coordinates the reprogramming event. It has knowledge of the size of the data and addresses to be reprogrammed. It coordinates error handling of the Flash and Communications I/O routines and determines when the reprogramming event is complete.

- Communications I/O Routines -

The Communications I/O Routines consist of the minimum amount of I/O support that is required for reprogramming. This piece is typically a subset of the normal operation Communication I/O routines. When designing the Communication I/O routines, it is possible to make assumptions on what part of the Communications I/O software must be supported since the Programming Instructions will be written in conjunction with the Reprogramming Algorithm.

- Reprogrammable Device I/O Erase/Write Routines -

The Reprogrammable Device I/O Erase and Write Routines allow access to the reprogrammable device and include functions such as turning programming voltage on and off, erasing, writing, and checking the erase status of the memory. Error checking of these functions is performed to inform the Executive Routine whether or not to continue Reprogramming.

- Checksum Verify Routine -

Checksums are typically calculated to verify that each block was transferred into RAM properly. At the end of reprogramming a section, the memory that has been written to is typically check summed to ensure that it was written successfully.

- Exit Routine -

The Exit Routine ensures RAM is left in the erased state that is acceptable to the operational software’s initialization logic. The communications hardware may need to be reconfigured to normal operation conditions. At the end, a power up reset is typically invoked to put the Controller back into its normal operational state.

# Additional Utility File Information - Best Practices and recommendations

All Communication Protocols:

- There shall be no additional steps between the interpreter instructions and the beginning of the routine section (routine #1).

- The only protocols that support two part utility files are GMLAN CAN and UDS.  All other protocols shall have the utility file header bytes $0A-$0B set to $0000.

- To calculate the offset to the routine section, first multiply the number of interpreter instructions by 16 (number of data bytes in an interpreter instruction) and then add 24 (number of data bytes in the utility file header).  All values should be calculated as decimal values and the sum should be converted to hexadecimal and placed into bytes $0E-$0F of the utility file header. If the SPS Editor program is used to create the utility file, the program will calculate the offset.

- Any interpreter instruction that sends a message on the vehicle communication bus shall implement the FD (no communications fault) and FF (unexpected response) return codes in the Goto section of the interpreter instruction.

S1130000071AC10100005A08FD09FF1B00000000FF

- Each interpreter instruction that uses Op-Code 27 (security access) shall use the same security algorithm value for a given device ID.

- Op-Code FC (Delay) has a setting for seconds or minutes (AC3).  Care should be taken to ensure that the setting is set to seconds for most programming events.  The minute’s feature is primarily used for vehicle theft deterrent re-learns.

- It is recommended that re-try logic be built into the utility file using a combination of the FC (delay) and FB (set and decrement counter) Op-Codes.

S113000003FC0200000000040000000000000000FF

S113000004FB010600000002001B000000000000FF

- When using the FB (set and decrement counter) Op-Code, the value of the Loop limit (AC1) must be > 1 as the counter value is decremented prior to processing the Goto instructions.

S113000012FB0103000000010010000000000000FF

- Care should be taken to reference Op-Code information by communication protocol as many of the Op-Codes are used in multiple protocols and the action field requirements vary by protocol.

- If an interpreter instruction is removed from a utility file, the offset to the routine and the Goto fields must be updated to reflect this modification.

- No extra bytes after the last routine sections allowed.

UART Protocol:

TBD

Class 2 Protocol:

- Op-Code 29 (mode $29) is obsolete and should be replaced with Op-Code 20 (mode $20) in any new utility files.  Op-Code $29 is supported for legacy issues only.

- For part number comparisons using Op-Code 3B (mode $3B) other than the VIN, the first goto field shall indicate a matching part number, the second goto field will provide the path to follow when the part numbers differ.

Keyword 2000 Protocol:

- There shall be a delay after ECU Reset and a start/stop communication to assure that the controller is still OK after Reset

- Seed &Key: Error code 37 is handled internally (external loop not necessary)

- Op-Code FC Delay for Seconds/Minutes KWP 2000 requires AC1=01 to send Tester Present (AC1=00 Default Tester Present is only for UART and GMLAN)

(continued)

GMLAN CAN Protocol:

- All GMLAN CAN Utility files shall have Op-Code 01 as the first interpreter instruction.

- All SPS_TYPE_A and SPS_TYPE_B ECU's shall support mode $1A DID $B0 (ECU diagnostic address). See GMW3110 specification.

- All SPS_TYPE_C ECU’s shall enable and support two special case CANId’s as defined in GMW3110 chapter 11.

- Items 1,2 and 3 allow the programming code to map the utility file device ID to the ECU request and response CANId’s.

- Any interpreter instruction which needs to sent after a ECU reset shall be implemented in part two of the utility file. The utility file header bytes $0A-$0B shall be set to the interpreter instruction number for the beginning of part two.  A FF Op-Code is required before the beginning of part two of the utility file.

- Only the GMLAN CAN protocol supports 4 byte addressing.

- GMLAN CAN utilizes pre utility (programming setup) and post utility (programming conclusion) file processes as documented in GMW3110.

Pre Utility file process example for HS GMLAN CAN:

<04 00 00 01 01 00 00 00 00 FE 3E

<04 00 00 01 01 00 00 00 00 FE 1A B0

>04 00 00 00 00 00 00 07 E8 00 5A B0 11

<04 00 00 01 01 00 00 00 00 FE 10 02

>04 00 00 00 00 00 00 07 E8 00 50

<04 00 00 01 01 00 00 00 00 FE 28

>04 00 00 00 00 00 00 07 E8 00 68

<04 00 00 01 01 00 00 00 00 FE A2

>04 00 00 00 00 00 00 07 E8 00 E2 00

<04 00 00 01 01 00 00 00 00 FE A5 01

>04 00 00 00 00 00 00 07 E8 00 E5

<04 00 00 01 01 00 00 00 00 FE A5 03

Post Utility file process example for HS GMLAN CAN:

<04 00 00 01 01 00 00 00 00 FE 20

>04 00 00 00 00 00 00 06 44 00 60

<04 00 00 01 01 00 00 00 00 FE 1A B0

>04 00 00 00 00 00 00 07 E8 00 5A B0 11

Certain Op-Codes commonly used in Class 2 are not defined for GMLAN CAN as they are part of the pre and post utility file processes and run automatically.

- GMLAN CAN supports Op-Code 53 Compare Data functions.  In combination with Op-Code 1A, Op-Code 53 can be used to read the operational software part number and program operational software and calibrations or program only calibrations.

S1130000071AC10100005A08FD09FF1B00000000FF

S1130000085301010000000A000B000000000000FF

- For Single Wire GMLAN CAN, the pre utility file process as defined in GMW3110 handles the high voltage wake up message.

- If the VIN, Repair Shop Code, Programming Date or End Model Part Number are written to the ECU, it is recommended that this be done in part two of the utility file after programming and ECU reset.  Repair shop code and Programming Date should be stored in flash memory.

(continued)

- The utility file needs to comprehend how the ECU reacts to a request for a part number if the part is not fully programmed.  For example, some ECU’s when running out of boot will go to a fixed memory location and report out the contents of those memory locations as the part number.  If the data file had been previously successfully programmed the reported data is correct.  If programming was interrupted you may get bogus data (e.g. FF FF FF FF) or you may get a valid part number even though the complete data file had not been programmed.  This can occur if the boot code retrieves the part number from the memory where the GMLAN header is written and programming had been interrupted after the header data was written to flash.  There are several ways to get around this.  One would be to have the boot check the validity of that particular data file and reject the part number request with reject code $31 if the data file was not fully programmed.  The utility file could then be setup to jump to the appropriate line to reprogram the partially programmed data component.  A second method to get around this is to use a combination of $A2 and $50 Op-Codes.  The $A2 Op-Code will retrieve the programmed state of the ECU and dump it into one of the utility file buffers.  The $50 Op-Code can then be used to verify that software is programmed before proceeding to the Op-Codes to check the software part numbers.  If the software were not programmed then the utility file would jump directly to program op-software.  If software is programmed then check the part numbers to see if the software needs to be updated (see above use of the $1A and $53 Op-Codes).  If the software part numbers match then the utility file can jump to just program the calibration section.  Note also that the use of the $50 Op-Code needs to comprehend whether or not the ECU supports any of the memory failure responses ($5x) that the ECU may send when checking to see if software is present.

- The utility files should be designed to compare part numbers of software components to determine if software and calibrations are to be programmed or just calibrations (see above descriptions on use of Op-Code $1A and Op-Code $53).  It is not necessary (or desirable) to check individual calibration file part numbers in the utility file unless the ECU supports reprogramming of subsets of the calibration data files.  If all calibration data files must be programmed as part of the same programming event it is not necessary to compare these part numbers.

- The module Id field in the $B0 Op-Code must match the module Id field in the drawing file for the data part as well as the module Id field in the WWAPI wrapper.

- The utility file should include an $A2 Op-Code after programming all data files.  Success of the programming event would then be determined based on an $A2 response of $00 (fully programmed).  Putting the $A2 Op-Code at the end ensures that a data size mismatch of the last programmed data file cannot result in a passed programming event and a non-functioning ECU.

- The conclusion of part 1 and part 2 of the utility file are both signified by encountering either the end with success ($FF) Op-Code or end with error ($EE) Op-Code.  There only needs to be one end with success and one end with error Op-Code.  Both parts can conclude by jumping to the same end with success or end with error Op-Code.

- It is not necessary to put Op-Code $01 in part 2 of the utility file.  The value used in part 1 of the utility file will also be used in part 2.

- Global A GMLAN ECUs that employ the Global A Secure Bootloader design (digitally signed OpSoftware and Calibration files) shall attempt to read the ProgrammingErrorCode (DID $F1) prior to exiting the utility file (OpCode EE) for a failed programming event.   The creator of the utility needs to include this ‘read’ logic within the utility file (reading this DID will not happen automatically).

This ProgrammingErrorCode DID response data will contain the details as to why an ECU failed a programming event (Mode $36).  This DID is defined in the ‘GMW3110 v1p6 Addendum’ document.

UDS Protocol:

- All UDS Utility files shall have Op-Code $01 as the first interpreter instruction.

- Any interpreter instruction which needs to be sent after a ECU reset shall be implemented in part two of the utility file. The utility file header bytes $0A-$0B shall be set to the interpreter instruction number for the beginning of part two.  The execution of an FF Op-Code is required before part two of the utility file will be executed.

- UDS supports only 4 byte addressing.

- Pre Processing Logic and Post Processing Logic

(tbd)

Certain Op-Codes commonly used in Class 2 are not defined for UDS as they are part of the pre and post utility file processes and run automatically.

- UDS supports the Op-Code $53 Compare Data function.  In combination with Op-Code $22, Op-Code $53 can be used to compare the Part Number currently in the ECU against the Part Number ‘slated’ to be programmed.  If the Part Numbers match, then skipping of that particular programming piece can be performed (if the ECU supports this feature).

- If the VIN, Repair Shop Code, and/or Programming Date are written to the ECU, it is recommended that this be done in part two of the utility file after programming and an ECU reset.

- The utility files should be designed to compare part numbers of software components to determine if software and calibrations are to be programmed or just calibrations (see above descriptions on use of Op-Code $22 and Op-Code $53).  It is not necessary (or desirable) to check individual calibration file part numbers in the utility file unless the ECU supports reprogramming of subsets of the calibration data files.  If all calibration data files must be programmed as part of the same programming event it is not necessary to compare these part numbers.

- The module ID field in the $36 Op-Code must match the module Id field in the drawing file for the data part as well as the module Id field in the WWAPI wrapper.

- The conclusion of part 1 and part 2 of the utility file are both signified by encountering either the end with success ($FF) Op-Code or end with error ($EE) Op-Code.  There only needs to be one end with success and one end with error Op-Code.  Both parts can conclude by jumping to the same end with success or end with error Op-Code.

- It is not necessary to put Op-Code $01 in part 2 of the utility file.  The value used in part 1 of the utility file will also be used in part 2.  However, the recommendation is that this Op-Code is the first step in Part 2 of the utility file.

- UDS ECUs that employ the Secure Bootloader design (digitally signed OpSoftware and Calibration files) shall attempt to read the ProgrammingErrorCode Data Identifier prior to exiting the utility file (OpCode EE) for a failed programming event.   The creator of the utility needs to include this ‘read’ logic within the utility file (reading this Identifier will not happen automatically).

This ProgrammingErrorCode DID response data will contain the details as to why an ECU failed a programming event (Mode $36).

# Programming Storage Buffers – Recommended Usage

  - There are currently twenty 2-byte storage buffers and twenty 256-byte storage buffers available for use during a programming event.  When referenced within an OpCode, the buffer number is entered in HEX (0x00 – 0x13).

  - The 2-byte buffers were originally designed for use with 2 byte checksums.  In most cases, two bytes of data should be stored into the 2-byte buffers for proper operation.  OpCode $50 is the corresponding “compare data” OpCode for 2-byte buffers.

  - The 256-byte buffers were designed to store data (ex. OpCode $1A), compare data (ex. OpCode $53), change data (ex. OpCode $54) or write data (ex. OpCode $3B). OpCode examples are GMLAN.  These buffers can hold 1-256 bytes of data.

  - The Programming Buffers are automatically initialized to all 00's (cleared) at the beginning of every programming event.

  - The Programming Buffers are not cleared between part 1 and part 2 of a GMLAN programming event.

# Interpreter 0 - UART Communications Programming Op-Codes

## UART Support for No Communication Faults

Supported By:

UART Interpreter 0

Description:

Normal processing of a request message is to re-send the request up to three times if the transmission of a request fails.  However, if the transmission is successful and no controllers respond to the request, then the request will fail with a no communications fault.

The goto fields in the interpreter can be used to force the tool to execute another step rather than re-sending the same ALDL message. This is accomplished by putting a FD in one of the first bytes of a goto field (G0, G2, G4, G6, or G8). The no communications screen will still be displayed but after verification the tool will goto the step specified by the FD rather than re-send the same ALDL message.

The option also exists to have the tool ignore any no communication faults. If this is necessary, it needs to be explained in the device’s reprogramming specification because this processing is handled internal to the tool software.

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | 01 03 F4 00 | AA 09 FD 12 FF 4A 00 00 00 00 |

Pseudo Code:

XX   OP   AC0   AC1   AC2 AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

If the Op-Code (OP) Ignores No Communication Faults

Continue reprogramming process

Else

Display the no communication screen

Verification is received

Check the goto fields for a FD return code (G0, G2, G4, G6, or G8)

If a FD Return Code is Found

Flag appropriate step to execute next and return

Else

Do While No Response

Send the ALDL message again

If This Is the Third Failure

End reprogramming process as a no comm fault

Endif this is the third failure

End do while no response

Endif an FD return code is found

Endif the Op-Code (OP) ignores no communication faults

## UART Return Type Processing

Supported By:

UART Interpreter 0

Description:

Several of the mode 6 and 16 Op-Codes set Return Type Processing and Return Code Position. These pieces of information that allow the tool to decipher the varying responses received from the target address. Return type processing and return code position are tied to a single exception Op-Code logic. The exceptions, when used, are documented with the Op-Codes (i.e. C0 Op-Code). Return Type Processing and Return Code Position are not directly modifiable unless the desired combination already exists for an Op-Code. If the combination does not exist, a new exception will need to be created for the appropriate Op-Code.

Return Code Position - Tells the tool what byte of the response will contain the actual return code. The return code is the value expected in the goto fields to determine the next step to execute.

Return Type Processing - This is used to handle the different formats of responses that result when using Op-Codes. For example, the CO Op-Code could return an AA or 06 response depending on the type of module being reprogrammed.

Pseudo Code:

XX   OP   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Do Return Type Processing Zero

Check response byte 4 for return codes (set return code position to 3)

Expect an AA return code for SUCCESS

Enddo return type processing zero

Do Return Type Processing One

Ignore the response and return

Enddo return type processing one

Do Return Type Processing Two

If Response Byte 3 is the Message Mode

If Response Byte 4 is a 15

Check return code in response byte 5 (anything but 06 is a FAILURE)

Else

Mode number in response byte 3 is SUCCESS

Endif response byte 4 is a 15

Else

No response received from device

Endif response 3 is the message mode

Enddo return type processing is two

Do Return Type Processing Three

If Response Byte 4 is Equal to Block Number (AC1)

return SUCCESS

Else

return FAILURE

Endif response byte 4 is equal to Block Number (AC1)

Enddo return type processing is three

Do Return Type Processing Five (developed for EEPROM applications)

Check response byte 3 for response

If Response Byte 3 is Mode Number

Return SUCCESS

Endif response byte 3 is mode number

Enddo return type processing five

## 00 Op-Code: Enable Normal Communications

Supported By:

UART Interpreter 0

Description:

The 00 Op-Code will build a standard mode 0 message to send to a device.  A mode 0 command will restart devices transmitting which may have been previously inhibited by a mode 8 command.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |
| G0: | 00 - Don't care |

Pseudo Code:

XX     00 AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of One.

Build Mode 0 message as below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 01 | Message length |
| 3 | 00 | Message mode |
| 4 | CS | Message checksum |

Send Mode 0 message

Goto step specified in goto field 1 (G1)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 00 | F4 00 00 00 | 00 02 FF 99 00 00 00 00 00 00 |

## 01 Op-Code: Request message from the device

Supported By:

UART Interpreter 0

Description:

The 01 Op-Code will build a standard mode 1 message to request a data list from a device. Devices may have different data list contents, so check device specifications to determine which data lists to request. Requested data list numbers and exceptions will be used by the tool to process the information returned from the devices.

Note:  The first byte of the Goto Fields is always set to 0x00 because it is not used (don't care).

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | Message number requested |
| AC2: | 00 - Not Used |
| AC3: | 00 - Do not wait for an F0 poll before sending mode 1 request |
|  | 01 - Wait for an F0 poll before sending mode 1 request |
| G0: | 00 - Don't care |

Pseudo Code:

XX   01   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of One

Build Mode 1 message as indicated in the table below:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 02 | Message length |
| 3 | 01 | Message mode |
| 4 | AC1 | Message number requested |
| 5 | CS | Message checksum |

If the Exception Is One (AC3=01)

Wait for an F0 poll before sending message

Endif the exception is one

Send Mode 1 message

Goto UART Return Type Processing

Goto step specified in goto field 1 (G1) (this Op-Code ignores no communication fault)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 1F | 01 | F4 03 00 00 | 00 20 FF 99 00 00 00 00 00 00 |

## 02 Op-Code: Request memory dump from the device using Mode 2 (Saturn EEPROM)

Supported By:

UART Interpreter 0

Description:

The 02 Op-Code will build a standard mode 2 message to request a 64 byte memory dump from a device.  Messages can consist of a single or multiple mode 2s, depending on the action fields. The calculated value is then compared against a previously stored value. In order to work correctly, a D4 Op=Code may need to be run prior to this one in order to create a valid value to compare against.

Warning:

In order for this code to work correctly, a D4 Op-Code may need to be run first.  This will place a valid checksum or CAL ID in global storage against which we will perform a comparison. (Note: This is not necessary if AC3 = 02).

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | High byte of address (needed only if AC3 = 02 or 04) |
| AC2: | Low byte of address (needed only if AC3 = 02 or 04) |
| AC3: | Exceptions. See pseudo code below 00 = multiple block read and checksum calculation. Compare against stored checksum. 01 = Read block and compare first two bytes to global checksum field 02 = Read block and exit – No comparisons 03 = Read block and compare to global CAL ID 04 = Read block and compare ABS controller transmission option byte. |
| G0: | 00 - Don't care |

Pseudo Code:

XX     02 AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing Five

Build Mode 2 message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 03 | Message length |
| 3 | 02 | Message mode |
| 4 | AC1 | High byte of address to read from |
| 5 | AC2 | Low byte of address to read from |
| 6 | CS | Message checksum |

If the Exception is One or Three (AC3=01, AC3=03 or AC3 = 04)

Send Mode 2 and receive response (64 data Bytes)

Compare extracted value to stored value

Endif the exception is one or three

If the Exception is Two (AC3=02)

Send Mode 2 and receive response (64 data Bytes)

Compare extracted value to stored value

End the exception is two

If the Exception is Zero (AC3=0)

While entire calibration has not yet been read from controller

Send Mode 2 with address of next block

Receive response

Perform summation algorithm on new data block

(continued)

End while

Compare calculated checksum to stored checksum

End the exception is zero

Goto UART Response Processing

Goto step specified in goto field 1 (G1)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 1F | 02 | F4 10 F7 00 | 00 20 FF 99 00 00 00 00 00 00 |

## 03 Op-Code: Verify programming of a EEPROM device (NAO EEPROM)

Supported By:

UART Interpreter 0

Description:

The 03 Op-Code will build a standard null mode 3 message to verify the programming of a device. On EEPROM applications, sending a mode 3 with no address (the entire message consists of only the mode number). As part of the functionality of the EEPROM controller, a response (which consists of only the mode number) to this message indicates that the programming process was successful. No response indicates that a problem occurred during reprogramming.

Note:  A mode 3 combined with the proper address will return the prom id of the current calibration. The prom id is the last four digits of the calibration part number. The id is accessed and used by the reprogramming software to select the proper calibration archive file prior to utilization of the utility file and interpreter.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | 00 - Send null mode 3 message |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |
| G0: | 00 - Return code (mode number) |

Pseudo Code:

XX   03   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of Five

Build Null Mode 3 message as indicated in the table below:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 01 | Message length |
| 3 | 03 | Mode number |
| 4 | 00 | Placeholder for checksum |

Send Mode 3 and check for response

Goto UART Return Type Processing

If Response Byte 3 is Message Mode

Indicate success

Else If No Response

Fail and indicate error

Endif response byte 3 is message mode

Goto step specified in goto field 1 (G1)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 0A | 03 | F4 00 00 00 | 03 02 FD 99 FF 99 00 00 00 00 |

## 04 Op-Code: Device control - Mode 4

Supported By:

UART Interpreter 0

Description:

The 04 Op-Code sends an ALDL mode 4 command that sets a bit

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX 04   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of One

Build Mode 4 message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 0D | Message length |
| 3 | 04 | Message mode |
| 4 | 00 | First data byte |
| 5 | 00 | Second data byte |
| 6 | 00 | Third data byte |
| 7 | 00 | Fourth data byte |
| 8 | 01 | Fifth data byte |
| 9 | 00 | Sixth data byte |
| 10 | 00 | Seventh data byte |
| 11 | 00 | Eighth data byte |
| 12 | 00 | Ninth data byte |
| 13 | 00 | Tenth data byte |
| 14 | 00 | Eleventh data byte |
| 15 | 00 | Twelfth data byte |
| 16 | CS | Message checksum |

Send Mode 4 message and receive response

Goto UART Return Type Processing

Build a Mode 5 message as exactly as indicated below (the interpreter builds and sends this message as part of the 04 Op-Code. A separate 05 Op-Code does not necessarily need to follow the 04 Op-Code):

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 01 | Message length |
| 3 | 05 | Message mode |
| 4 | CS | Message checksum |

Use UART Return Type Processing of Zero

Wait for an ‘F0’ poll (equivalent of the option AC1=01 shown in the description of 05 Op-Code)

(continued)

Send the Mode 5 message and receive response

Goto UART Response Processing

Look for response byte 4 in Goto fields

If Response Byte 4 is Found in Goto Fields

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response byte 4 is found in goto field

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 1F | 04 | F4 00 00 00 | 04 20 FF 99 00 00 00 00 00 00 |

## 05 Op-Code: Ram download request - Mode 5

Supported By:

UART Interpreter 0

Description:

The 05 Op-Code will build a standard mode 5 message to send to a device. A mode 5 command will prepare the device to receive executable code from the serial data link. Mode 5 messages are used in conjunction with modes 6 & 16 to download information to devices. The mode 5 command should expect an AA response for success.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | 00 No wait |
|  | 01 Wait for an F0 poll before sending mode 5 request |
|  | 02 Wait for an F0 poll before sending mode 5 message (don't display poll No Comm. failures) |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX   05   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of Zero

Build Mode 5 message as below:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 01 | Message length |
| 3 | 05 | Message mode |
| 4 | CS | Message checksum |

If Exception is One or Two (AC1=01 or AC1=02)

Wait for an F0 poll

Endif exception is one or two

If Exception is Two (AC1=02)

Ignore no communications faults

Endif exception is two

Send Mode 5 message and receive response

Goto UART Return Type Processing

Look for response byte 4 in Goto fields

If Response Byte 4 (AA) is Found in Goto Fields

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response byte 4 is found in goto field

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 05 | F4 01 00 00 | AA 11 05 0D FF 99 00 00 00 00 |

## 08 Op-Code: Disable Normal Communications

Supported By:

UART Interpreter 0

Description:

The 08 Op-Code will build a standard mode 8 message to send to a device.  A mode 8 command will inhibit devices from transmitting data on the serial data link.  Communications will remain inhibited while any ALDL activity exists on the link or the link is inactive for more than five seconds.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Do not wait for an F0 poll before sending mode 8 message Wait for a response from the mode 8 |
|  | 01 - Wait for an F0 poll before sending mode 8 message Wait for a response from the mode 8 |
|  | 02 - Do not wait for an F0 poll before sending mode 8 message Do not wait for a response from the mode 8 |
|  | 03 - Wait for an F0 poll before sending mode 8 message Do not wait for a response from the mode 8 |
| G0: | 00 - Don't care |

Pseudo Code:

XX     08  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

UART Return Type Processing is based on exception.

Build Mode 8 message as below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 01 | Message length |
| 3 | 08 | Message mode |
| 4 | CS | Message checksum |

If the Exception is One or Three (AC3=01 or AC3=03)

If the Device ID is F1

Wait for an F0 poll with Tech 2 as Master (For Electric Vehicle)

Else

Wait for an F0 poll with Tech 2 as Slave (Normal F0 processing)

Endif the device ID is F1

Else

Do not wait for an F0 poll

Endif the exception is one or three

Send Mode 8 message

If the Exception is Zero or One (AC3=00 or AC3=01)

Wait for a response

Verify correct response

Else

Do not wait for a response (ignore)

Endif the exception is zero or one

Goto step specified in the goto fields

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 08 | F4 00 00 00 | 00 02 FF 99 00 00 00 00 00 00 |

## 09 Op-Code: Enable Normal Communications

Supported By:

UART Interpreter 0

Description:

The 09 Op-Code is used to resume normal communications after a 08 (Mode 8) message. The Op-Code will build a Mode 9 message.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |
| G0: | 00 - Don't care |

Pseudo Code:

XX   09   AC0   AC1   AC2   AC3   G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of One.

Build Mode 9 message as below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 01 | Message length |
| 3 | 09 | Message mode |
| 4 | CS | Message checksum |

Send Mode 9 message and receive response.

Goto UART Return Type Processing (Op-Code ignores no communications faults).

Goto step specified in goto field 1 (G1)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 09 | F4 00 00 00 | 00 02 FF 99 00 00 00 00 00 00 |

## 0A Op-Code: Clear Trouble Codes - Mode 10

Supported By:

UART Interpreter 0

Description:

The 0A Op-Code will build both a standard mode 8 and standard mode 10 message to send to a device.  A mode 10 command will clear diagnostic trouble codes for a given device.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |
| G0: | 00 - Don't care |

Pseudo Code:

XX     0A  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of One (ignore).

Build Mode 8 message as below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | F4 | Device ID |
| 2 | 01 | Message length |
| 3 | 08 | Message mode |
| 4 | CS | Message checksum |

Send the Mode 8 response to break normal communications.

Ignore the response.

Build Mode 10 message as below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 01 | Message length |
| 3 | 0A | Message mode |
| 4 | CS | Message checksum |

Send Mode 10 message and ignore the response.

Goto step specified in goto field 1 (G1)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 0A | F4 00 00 00 | 00 02 FF 99 00 00 00 00 00 00 |

## 0C Op-Code: Program EEPROM - Mode 12

Supported By:

UART Interpreter 0

Description:

The 0C Op-Code is used to download data to EEPROM using standard mode 12 ALDL communications. The data that is downloaded is determined by the information contained in the Action Fields. The 0C Op-Code will use return type processing 3 as defined under the Return Code Position and Return Type Processing section of this document.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | Block Number being downloaded if AC2 is not $10, Number of Routine being downloaded if AC2 is $10 |
| AC2: | Data to include in the message |
|  | $00 - Send VIN (17 bytes) |
|  | $01 - Send End Model Part Number (4 bytes) $02 - Reset Manufacturer’s Enable Counter $03 - Write a new key code to the ccu or pcm of the EV1 Electric Vehicle $04 - Write the state of charge information to the EV1 Electric Vehicle PCM $10 - Write Routine including block number as first byte. AC1 contains Routine Number |
| AC3: | 00 - Do not wait for an F0 poll before sending mode 12 message |
|  | 01 - Wait for an F0 poll before sending mode 12 message |

Pseudo Code:

XX     0C AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of Three

Build Mode 12 message as below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | N + 2 | Message length |
| 3 | 0C | Message mode |
| 4 | AC1* | Block number |
| 5 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 5 + N | XX | Data byte N |
| 6 + N | CS | Message checksum |

* Use first routine byte if AC2 is $10

If the Exception is Zero (AC2 is 00)

Copy the VIN into the data area of the message starting at data byte 1

ElseIf the Exception is One (AC2 is 01)

Copy the End Model part number into the data area of the message starting at data byte 1

ElseIf the Exception is Two (AC2 is 02)

Reset the manufacturer’s enable counter to zero

ElseIf the Exception is Three (AC2 is 03) (Used for Electric Vehicle)

If AC0 is 0xF4

Write key code to the PCM (Electric Vehicle)

Else

Write key code to the CCU (Electric Vehicle)

(continued)

ElseIf the Exception is Four (AC2 is 04) (Used for Electric Vehicle)

While state of charge data remains

Create new mode 12 command with appropriate block number

Send the new mode 12 and receive response

Endwhile

ElseIf the Exception is Ten Hex or Sixteen Decimal (AC2 is $10)

Copy Routine specified by AC1 into message. The first byte is used as block number (Request Byte 4) and all following bytes as data (Request Byte 5 and following)

Endif exception processing

If AC3 is 01

Wait for an F0 poll before sending message

Endif AC3 is 01

If AC2 is Not Equal to Four

Send 12 message and receive response

Endif AC2 is not equal to four

Look for response byte 4 in Goto fields (Block Number)

If Matching Block Number is Found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif matching block number is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 0C | F4 01 00 00 | 01 22 FF 99 00 00 00 00 00 00 |

## 0D Op-Code: Perform Security - Mode 13

Supported By:

UART Interpreter 0

Description:

The 0D Op-Code is used to open a control module's memory for reprogramming. The tool uses Action Field zero (AC0) to determine which security algorithm is used.

Action Fields:

| AC0: | Security Algorithm Number (Default security algorithm is 00) |
| --- | --- |
| AC1: | Device ID |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX   0D   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of Zero

Build mode 13 security seed request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC1 | Device ID |
| 2 | 02 | Message Length |
| 3 | 0D | Message mode |
| 4 | 01 | Request number |
| 5 | CS | Message checksum |

Send mode 13 seed request and receive response

Goto UART Return Type Processing

If Both Bytes of the Seed (response bytes 5 & 6) are Zeros

Set security response byte 5 to success (AA)

Indicate successful response

Else

Use specified security algorithm to determine device's key

Build mode 13 security key response as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC1 | Device ID |
| 2 | 04 | Message Length |
| 3 | 0D | Message mode |
| 4 | 02 | Request number |
| 5 | XX | High byte of key |
| 6 | XX | Low byte of key |
| 9 | CS | Message checksum |

Send Mode 13 key response and receive response

Goto UART Return Type Processing

Endif both bytes of the seed (response bytes 5 & 6) are zeros

Look for mode 13 key response byte 5 in goto fields

If Mode 13 Key Response Byte 5 is Found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif Mode 13 Key Response Byte 5 is Found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 0D | 00 F4 00 00 | AA 02 27 22 FF 55 00 00 00 00 |

## 20 Op-Code: Receive a normal mode message from device

Supported By:

UART Interpreter 0

Description:

The 20 Op-Code will eavesdrop on a normal mode conversation and return the first message it finds with the given device ID. This is used to read the current odometer value stored in the Electric Vehicle IPC.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | 00 – Not used |
| AC2: | 00 – Store read odometer value for later comparisons. 01 – Compare value read against odometer value to be written. If old value is <= new value, test passes. |
|  | 02 – Compare value read against odometer value written. If old value == new value, test passes. |
| AC3: | 00 – Not used |
| G0: | 00 – Don’t Care |

Pseudo Code:

XX     20  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of Five

Build Mode 1 message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 01 | Message length |
| 3 | 20 | Message mode |
| 4 | CS | Message checksum |

Send message and receive response (up to 256 Bytes)

Goto UART Response Processing

If the Exception is One or Two (AC2=01 or AC2=02)

Extract odometer value from incoming message

I	Initialize return code to FAILURE

If AC2=00

Store read odometer value in VIT2 Structure

Set return code to SUCCESS

Elseif AC2=01 and received odometer value is <= value in VIT2 structure

Set return code to SUCCESS

Elseif AC2=02 and received odometer value is = value in VIT2 structure

Set return code to SUCCESS

Endif

Endif the exception is one or two

Goto step specified in goto field 1 (G1), this Op-Code ignores no communication faults

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 1F | 20 | 21 00 02 00 | 21 20 FD 99 FF 99 00 00 00 00 |

## A3 Op-Code Read Data from Specified Sequential Addresses (Electric Vehicle) Mode 3

Supported By:

UART Interpreter 0

Description:

The A3 Op-Code is used to read the data from the specified two byte addresses of a controller. It was developed specifically for reprogramming the key code of the CCU/RSA controller of the EV1 electric vehicle. To use the Op-Code, the target controller must have normal communication discontinued.

The addresses will all be sequential starting from the address indicated in the actions fields. The number of addresses can be varied based on the value of AC1.

See the pseudo code for more detailed information concerning Op-Code function.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Number of Addresses to Read (including the address specified in AC2 & AC3) |
| AC2: | High Byte of Starting Address |
| AC3: | Low Byte of Starting Address |

Pseudo Code:

XX   D0   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing Five

Build Mode 03 Address Read as indicated in the table below.

Contents of the XX locations are detailed in the pseudo code following the table.

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 1 + M | Message mode + (AC1 multiplied by 2) |
| 3 | 03 | Message mode |
| 4 | AC | High byte of starting address |
| 5 | XX | Low byte of starting address |
| 6 | XX | High byte of second address |
| 7 | XX | Low byte of second address |
|   |  | High and low byte of each consecutive address to read. |
| 1 + M +1 | CS | 1 byte request checksum |

Build mode 03 message through low byte of starting address.

Combine the high and low byte of starting address into one number.

Increment the starting address by one.

Load the new address value in the next two message bytes.

Continue to increment and load the addresses until number of addresses indicated in AC1 is met.

Send the message and expect a response of 03 in the mode position.

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | A3 | F3 05 1F 20 | 00 09 FD 12 FF 4A 00 00 00 00 |

## C0 Op-Code: Download Routine to RAM and Execute using Mode 6

Supported By:

UART Interpreter 0

Description:

The C0 Op-Code builds and sends an ALDL mode 6 download routine message. It is used to send routines with a prolog to the RAM of the device being reprogrammed. Three different messages can be sent:

1) The address to download to (starting from the highest address working backwards)

2) An optional prolog (utility record parameters indicate if one is sent or not)

3) A portion of the routine.

If the entire routine can be sent in one message, it will be sent without a prolog. If it cannot be sent in one message, then the routine is split evenly and a prolog is sent with each message except for the last one. A C0 Op-Code should always be preceded by a Mode 5 (Op-Code 05) message unless the module is already prepared to accept a mode 6.

Note:	 See Pseudo code for more information on maximum packet sizes.

Action Fields:

| AC0: | Section Number of Routine |
| --- | --- |
| AC1: | Section Number of Prolog or Prolog Value. See Exceptions. |
| AC2: | Device ID |
| AC3: | Exceptions. See Pseudo Code below |

Pseudo Code:

XX   C0   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of Three.

Build Download message as indicated in the table below. Contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC2 | Device ID |
| 2 | 3 + N | Message length |
| 3 | 06 | Message mode |
| 4 | XX | High byte of message address |
| 5 | XX | Low byte of message address |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N | XX | Data byte N (N <= maximum as defined in pseudo code) |
| 7 + N | CS | 1 byte request checksum |

If the Exception is Zero (AC3=00) or the Exception is One (AC3=01)

Expect an AA return code for SUCCESS (set return type processing to 0)

Check response byte 4 for return codes (set return code position to 3)

If AC1 is Not 0

Retrieve section AC1 and assign to prolog

Endif AC1 is Not 0

Retrieve section AC0 and assign to routine

Set maximum packet to 167 (SFI-66U & PCM-66U)

Endif exception is zero or exception is one

If the Exception is Two (AC3=02)

Expect a 06 return code for success (set return type processing to 2)

For failures or last packets expect a 15 in response byte 4 and a return code in response byte 5

Check response byte 3 for message mode (set return code position to 2)

Retrieve section AC0 and assign to routine

Assign value in AC1 as prolog

Set maximum packet to 60 bytes (VCM-1)

Endif the exception is two

If the Exception is Three (AC3=03) - developed for EEPROM applications

Expect an 06 return code for success (set return type processing to 5)

Check response byte 3 for message mode (set return code position to 2)

(continued)

Retrieve section AC0 and assign to routine

Assign value in AC1 as prolog

Set maximum packet to 114 bytes

Endif the exception is three

If Routine Length is Greater than Maximum Packet

# of packets = routine length / (maximum packet - prolog length)

Last message length = remainder of routine length / (maximum packet - prolog length)

Addresses are calculated from highest load address to lowest load address

If Exception is Two or Three (AC3=02 or AC3=03)

Use value in AC1 as the prolog; copy AC1 into data byte 1 of message buffer

Else

Copy prolog into data area of message starting at data byte one

Endif exception is two or three

Do while More Packets

If This is the Last Packet

Change message length to last message length+3

Set the two byte packet address to the low two bytes of the four byte address in the routine

Copy the last message from the routine overlaying the prolog (no prolog on last packet)

Else

Calculate appropriate two byte packet address

Copy calculated packet address into message buffer

Copy a piece of the routine into message buffer

Endif this is the last packet

Send the message and expect a response

If Exception is Two or Three (AC3=02 or AC3=03)

If Message Response was SUCCESS (06 in position 3)

Loop and continue sending packets

Else

Indicate FAIL

Endif message response was a success

Else

If Message Response was SUCCESS (AA in position 4)

Loop and continue sending packets

Else

Indicate FAIL

Endif message response was SUCCESS

Endif exception is two

End do while more packets

Else

Use low two bytes of the four byte address in the routine for message address

Copy routine into message buffer, NO prolog on packet

Send the message and expect a response

Endif routine length is greater than maximum packet

If Exception is One (AC3=01)

Do while response byte 4 is $55

Wait for another response

End do while response is $55

Endif exception is one

Look for response byte 4 in Goto fields

If Response Byte 4 is Found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response byte 4 is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | C0 | 01 39 F4 02 | 06 11 04 0D FD 20 FF 49 00 00 |

## C1 Op-Code: Download Routine to RAM and Execute using Mode 16

Supported By:

UART Interpreter 0

Description:

This Op-Code is an ALDL mode 16 with three byte addressing. It is used to send routines with a prolog to RAM of the device being reprogrammed.

Note: AC1 Op-Code should always follow a Mode 5 (Op-Code 05) message.

Action Fields:

| AC0: | Section Number of Routine |
| --- | --- |
| AC1: | Use value as high byte of prolog |
| AC2: | Use value as low byte of prolog |
| AC3: | Device ID |

Pseudo Code:

XX   C1   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of Two.

Build Mode 16 Download Request as indicated in the table below. Contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC3 | Device ID |
| 2 | 4 + N | Message length |
| 3 | 10 | Message mode |
| 4 | XX | High byte of load address |
| 5 | XX | Middle byte of load address |
| 6 | XX | Low byte of load address |
| 7 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 7 + N | XX | Data byte N (N <= maximum as defined in pseudo code) |
| 8 + N | CS | 1 byte request checksum |

For failures or last packets expect a 15 in response byte 4 and a return code in response byte 5 and Expect a 06 return code for success (set return type processing to 2)

Check response byte 3 for message mode (set return code position to 2)

Retrieve section AC0 and assign to routine

Set maximum packet to 60

If Routine Length is Greater than Maximum Packet

# Of packets = routine length / (maximum packet - prolog length)

Last message length = remainder of routine length / (maximum packet - prolog length)

If remainder of the routine length is 0

Reduce the number of messages sent with a prolog by one

Endif remainder of the routine length is 0

Addresses are calculated from high to low memory so last packet can be executed once downloaded

Copy AC1 into next data byte of data area, this is high byte of prolog

Copy AC2 into next data byte of data area, this is low byte of prolog

Do while More Packets

If This is the Last Packet

Change message length to last message length+4

(continued)

Set three byte message address to the low three bytes of the four byte address in the routine

Copy the last message from the routine overlaying the prolog (no prolog in last packet)

Else

Calculate appropriate two byte packet address for next data packet

Copy calculated packet address into message buffer

Copy a piece of the routine into message buffer

Endif this is the last packet

Send the message and expect a response

Goto UART Return Type Processing

If Message Response was SUCCESS (10 in position 3)

Loop and continue sending packets

Else

Indicate FAIL

Endif message response was SUCCESS

End do while more packets

Else Routine Length is Not Greater than Maximum Packet

Copy routine into message buffer, no prolog in message

Send the message and expect a response

Goto UART Return Type Processing

Endif routine length is greater than maximum packet

Look for response byte 3 in Goto fields

If Response byte 3 is Found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response byte 3 is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | C1 | 01 4E 75 F4 | 10 11 05 0D FF 99 00 00 00 00 |

## C2 Op-Code: Download Routine to RAM and Save Two Byte Response using Mode 6

Supported By:

UART Interpreter 0

Description:

The C2 Op-Code is processed exactly like the C0 Op-Code except for response processing. If a valid response is received from the Mode 6 command, positions 4 and 5 of the response are saved. The Action Field 3 (AC3) is the ID of the bytes saved. The valid ID's are 0x00 thru 0x13.

Note:	Only twenty (20) two byte responses can be saved before ID positions must be reused.

Note:	If the routine can be downloaded in one packet no prolog is need. See the Pseudo code for more information on maximum packet sizes.

Action Fields:

| AC0: | Section Number of Routine |
| --- | --- |
| AC1: | Section Number of Prolog. |
| AC2: | Device ID |
| AC3: | ID of saved bytes (0x00 – 0x13) |

Pseudo Code:

XX   C2   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

The message build and transmit portion of this Op-Code is identical to the C0 Op-Code. The only change is after a successful response is returned from the control module.

If Successful Response is Received

Save two bytes from the response to storage ID indicated by AC3 (bytes 5 & 6 of response are saved)

Endif successful response is received

Look for response byte 4 in Goto fields

If Response Byte 4 is Found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response byte 4 is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | C2 | 04 03 F4 09 | AA 11 05 0D FF 99 00 00 00 00 |

## C3 Op-Code: Download Routine to RAM and Execute using Mode 6 (Electric Vehicle)

Supported By:

UART Interpreter 0

Description:

The C3 Op-Code builds and sends an ALDL mode 6 download routine message. It is used to send routines with a prolog to the RAM of the device being reprogrammed. This Op-Code was developed specifically for the EV1 electric vehicle BTCM controller because the controller does not respond to each download and uses a one byte prolog.

As with the C0 Op-Code, if the entire routine can be sent in one message, it will be sent without a prolog. If it cannot be sent in one message, then the routine is split evenly and a prolog is sent with each message except for the last one. A C3 Op-Code should always be preceded by a Mode 5 (Op-Code 05) message unless the module is already prepared to accept a mode 6.

Note:	 See Pseudo code for more information on maximum packet sizes.

Action Fields:

| AC0: | Section Number of Routine |
| --- | --- |
| AC1: | Prolog Value |
| AC2: | Device ID |
| AC3: | Not Used |

Pseudo Code:

XX   C0   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use UART Return Type Processing of One.

Build Download message as indicated in the table below. Contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC2 | Device ID |
| 2 | 3 + N | Message length |
| 3 | 06 | Message mode |
| 4 | XX | High byte of message address |
| 5 | XX | Low byte of message address |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N | XX | Data byte N (N <= maximum as defined in pseudo code) |
| 7 + N | CS | 1 byte request checksum |

Retrieve section AC0 and assign to routine

Retrieve the address from the routine header

Assign value in AC1 as prolog

Set maximum packet size to 60 bytes

Send the Mode 6 message and do not expect a response.

If Routine Length is Greater than Maximum Packet

# Of packets = routine length / (maximum packet - prolog length)

Last message length = remainder of routine length / (maximum packet - prolog length)

Addresses are calculated from highest load address to lowest load address

Use value in AC1 as the prolog; copy AC1 into data byte 1 of message buffer

Do while More Packets

(continued)

If This is the Last Packet

Change message length to last message length+3

Set the two byte packet address to the low two bytes of the four byte address in the routine

Copy the last message from the routine overlaying the prolog (no prolog on last packet)

Else

Calculate appropriate two byte packet address

Copy calculated packet address into message buffer

Copy a piece of the routine into message buffer DO NOT include a prolog

Endif this is the last packet

Send the message and do not expect a response

Loop and continue sending packets

End do while more packets

Else

Use low two bytes of the four byte address in the routine for message address

Copy routine into message buffer, NO prolog on packet

Send the message do not expect a response

Endif routine length is greater than maximum packet

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | C3 | 01 39 F9 00 | 00 11 FD 20 FF 21 00 00 00 00 |

## C4 Op-Code: Download Routine to RAM using Mode 20 (Holden Telematics)

Supported By:

UART Interpreter 0

Description:

The C4 Op-Code builds and sends ALDL mode 20 messages with three address bytes. It is used to send routines to the RAM of the device being reprogrammed. This Op-Code was developed specifically for the Holden Telematics controller but can be used for any controllers using mode 20 in the same manner.

Note:	 See Pseudo code for more information on maximum packet sizes.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | Routine Number |
| AC2: | Not Used |
| AC3: | Not Used |

Pseudo Code:

XX   C4   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build Mode 20 RAM Download Request as indicated in the table below. Contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 4 + N | Message length |
| 3 | 14 | Message mode |
| 4 | XX | High byte of download address |
| 5 | XX | Middle byte of download address |
| 6 | XX | Low byte of download address |
| 7 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 7 + N | XX | Data byte N (N <= maximum as defined in pseudo code) |
| 8 + N | CS | 1 byte request checksum |

Retrieve section AC1 and assign to routine

Get download address from routine address

Get number of bytes to download from routine size

Get packet length from utility file header

Do while number of bytes to download not yet 0

If number of bytes to download is greater than packet length

Limit packet length to number of bytes to download

Endif number of bytes to download is greater than packet length

Copy download address into message bytes 4.6

Copy next data block from routine into message starting with byte 7, packet length specifies the number of bytes to copy.

Send the message and expect a response

If message response was SUCCESS

If the response byte 3 is $14 (expected mode)

If response byte 4 is $06 (download completed)

Increment the download address by the packet size

Decrement the number of bytes to download by the packet length

(continued)

Else

Break the loop

Endif response byte 4 is $06 (download completed)

Else

Set message response FAILURE

Break the loop

Endif the response byte 3 is $14 (expected mode)

Else

Break the loop

Endif message response was SUCCESS

Enddo while number of bytes to download not yet 0

If message response was SUCCESS

Set response code to response byte 4

Else

Set response code to NO COMM

Endif message response was SUCCESS

Look for response code in Goto fields

If response code is found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response code is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | C4 | E4 03 00 00 | 06 11 15 0D FF 99 00 00 00 00 |

## C5 Op-Code: Execute Routine using Mode 21 (Holden Telematics)

Supported By:

UART Interpreter 0

Description:

The C5 Op-Code builds and sends an ALDL mode 21 message with three address bytes. It is used to start a routine in RAM of the device being reprogrammed. This Op-Code was developed specifically for the Holden Telematics controller but can be used for any controllers using mode 21 in the same manner.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | Routine Number |
| AC2: | Not Used |
| AC3: | Not Used |

Pseudo Code:

XX   C5   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build Mode 21 RAM Execute Request as indicated in the table below. Contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 04 | Message length |
| 3 | 15 | Message mode |
| 4 | XX | High byte of start address |
| 5 | XX | Middle byte of start address |
| 6 | XX | Low byte of start address |
| 7 | CS | 1 byte request checksum |

Retrieve section AC1 and assign to routine

Get start address from routine address

Copy start address into message bytes 4.6

Send the message and expect a response

If message response was SUCCESS

If the response byte 3 is not $15 (expected mode)

Set message response FAILURE

Endif the response byte 3 is not $15 (expected mode)

Endif message response was SUCCESS

If message response was SUCCESS

Set response code to response byte 4

Else

Set response code to NO COMM

Endif message response was SUCCESS

Look for response code in Goto fields

If response code is found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response code is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | C5 | E4 03 00 00 | 06 11 15 0D FF 99 00 00 00 00 |

## D0 Op-Code: Download Calibration / Software File using Mode 6

Supported By:

UART Interpreter 0

Description:

The D0 Op-Code is used to download a data file using mode 6 messages. The device must be prepared to accept a mode 6 message before this Op-Code can be used. The "data address information" in the header, is used for the first packet of data that is downloaded. The additional packet addresses are determined based off the value of the exception field (AC3). The exception values are defined as follows:

00 -	The address will be incremented by the number of bytes that are downloaded in each calibration data packet (i.e. if address 1 = 0x8000 and 0x80 bytes are downloaded in the data packet, then address 2 = 0x8000 + 0x80 = 0x8080).

01 -	The address remains the same for all of the calibration data packets.

02 -	Uses the same functionality of exception 00 but increases the starting position of the calibration data packet 3 bytes (from 16 to 19) to allow for a longer prolog (Developed for EEPROM applications).

See the pseudo code for more detailed information concerning exception function.

Action Fields:

| AC0: | ID of file to download |
| --- | --- |
| AC1: | Section number of Prolog or Prolog Value. See Exceptions. |
| AC2: | Device ID |
| AC3: | Exceptions. See Pseudo Code below |

Pseudo Code:

XX   D0   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Find the file with ID equal to AC0

Load the file into memory

Build Mode 6 Download Request as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC2 | Device ID |
| 2 | 6 + N + M | Message length (N is prolog length, M data bytes in message) |
| 3 | 06 | Message mode |
| 4 | XX | High byte of load address |
| 5 | XX | Low byte of load address |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N + M | XX | Data byte N (N <= maximum as defined in pseudo code) |
| 7 + N + M | CS | 1 byte request checksum |

If the Exception is Zero (AC3=00)

Expect an AA return code for success (set return type processing to 0)

Check response byte 4 for return codes (set return code position to 3)

Retrieve section AC1 and assign to the prolog

Set message length to 6 + prolog length + data packet length

Copy high byte of prolog address into data byte 1 of buffer

Copy low byte of prolog address into data byte 2 of buffer

Copy prolog into buffer starting at byte 3

Copy high byte of data address information from the Header into the data buffer

Copy low byte of data address information from the Header into the data buffer

Endif the exception is zero

(continued)

If the Exception is One (AC3=01)

Expect a 06 return code for success (set return type processing to 2)

For failures or last packets expect a 15 in response byte 4 and a return code in response byte 5

Check response byte 3 for message mode (set return code position to 2)

Set message length to 4 + data packet length

Copy high byte of data address information from the Header into the data buffer byte 1

Copy low byte of data address information from the Header into the data buffer byte 2

Copy AC1 into data buffer byte 3 to be used as a prolog

Endif the exception is one

If the Exception is Two (AC3=02) - (developed for EEPROM applications)

Expect a 06 return code for success (set return type processing to 2)

For failures or last packets expect a 15 in response byte 4 and a return code in response byte 5

Check response byte 3 for message mode (set return code position to 2)

Retrieve section AC1 and assign to the prolog

Set message length to 6 + prolog length + data packet length

Copy high byte of prolog address into data byte 1 of buffer

Copy low byte of prolog address into data byte 2 of buffer

Copy prolog into buffer starting at byte 3

Copy high byte of data address information from the Header into the data buffer

Copy low byte of data address information from the Header into the data buffer

Endif the exception is two

Do while more data to download

Copy M data bytes into message buffer following the front end

Put message checksum at end of data buffer

Send message and receive the response

If Exception is Zero or Two (AC3=00 or AC3=02)

Increment message data address by number of data bytes downloaded in message

Else

If Exception is One (AC3=01)

Use the same address for all data packets

Endif exception is one

Endif exception is zero or two

Update and display percent downloaded

End do while more data to download

Look for response byte 4 in Goto fields

If Response Byte 4 is Found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response byte 4 is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | D0 | 01 03 F4 00 | AA 09 FD 12 FF 4A 00 00 00 00 |

## D1 Op-Code: Download Calibration or Software Module using Mode 16

Supported By:

UART Interpreter 0

Description:

Download a data file using mode 16. The device must be prepared to accept a mode 16 message before this Op-Code can be used.

Action Fields:

| AC0: | ID of file to download |
| --- | --- |
| AC1: | Use value as high byte of prolog |
| AC2: | Use value as low byte of prolog |
| AC3: | Device ID |

Pseudo Code:

XX   D1   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Find the file with ID equal to AC0

Load the file into memory

Expect a 06 return code for success (set return type processing to 2)

For failures or last packets expect a 15 in response byte 4 and a return code in response byte 5

Check response byte 3 for message mode (set return code position to 2)

Build Mode 6 Download Request as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC3 | Device ID |
| 2 | 4 + M | Message length (M is data bytes in message) |
| 3 | 10 | Message mode |
| 4 | XX | High byte of load address |
| 5 | XX | Middle byte of load address |
| 6 | XX | Low byte of load address |
| 7 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 7 + N + M | XX | Data byte M (M <= maximum as defined in pseudo code) |
| 8 + N + M | CS | 1 byte request checksum |

Copy high byte of data load address from the Utility header into message

Copy middle byte of data load address from the Utility header into message

Copy low byte of data load address from the Utility header into message

Copy AC1 into data area of message as the high byte of the prolog into data byte 1

Copy AC2 into data area of message as the high byte of the prolog into data byte 2

Do until all of the data is downloaded

Copy next packet of data into message buffer at data byte 3, packet size is from header

Copy message checksum at end of data area of message

Send message and receive the response

Goto UART Return Type Processing

Update and display percent downloaded

End do when all of the data is downloaded

Look for response byte 3 in Goto fields

If response byte 3 is found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response byte 3 is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 98 | D1 | 02 4E 75 00 | 06 11 05 0D FF 99 00 00 00 00 |

## D2 Op-Code: Download VIN using Mode 6 & 16

Supported By:

UART Interpreter 0

Description:

The D2 Op-Code is used to download the VIN. The device must be prepared to accept a mode 6 or 16 message before this Op-Code can be used. The Op-Code uses mode 6 for two byte addressing and mode 16 for three byte addressing. See Action Fields for more information on type of addressing.

Action Fields:

| AC0: | Module ID for VIN |
| --- | --- |
| AC1: | Prolog routine number for Two byte addressing |
|  | Prolog ID for Three byte addressing |
|  | 00 - Use 4E75 as the prolog |
| AC2: | Device ID |
| AC3: | 00 Two byte addressing - 01 Three byte addressing |

Pseudo Code:

XX   D2   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build Mode 6 or 16 Download Request as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC2 | Device ID |
| 2 | 4 + M | Message length (M is data bytes in message) |
| 3 | 06 or 10 | Message mode |
| 4 | XX | High byte of load address |
| 5 | XX | Middle of low byte of load address |
| 6 | XX | Low byte of load address or data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N + M | XX | Data byte N (N <= maximum as defined in pseudo code) |
| 7 + N + M | CS | 1 byte request checksum |

If the Exception is Zero (AC3=00)

Use two byte addressing

Expect a 06 return code for success (set return type processing to 2)

For failures or last packets expect a 15 in response byte 4 and a return code in response byte 5

Check response byte 3 for message mode (set return code position to 2)

Set message mode to 06

Copy high byte of data load address from the Utility header into message

Copy low byte of data load address from the Utility header into message

Reserve this byte for high byte of module checksum

Reserve this byte for low byte of module checksum

Copy AC0 into data area of message as the module ID

Copy AC1 into data area of message as the prolog

Endif the exception is zero

If the Exception is One (AC3=01)

Use three byte addressing

Set the return type processing to 2

Set the return code position to 3

(continued)

Set message mode to 10

Copy high byte of data load address from the Utility header into message

Copy middle byte of data load address from the Utility header into message

Copy low byte of data load address from the Utility header into message

Reserve this byte for high byte of module checksum

Reserve this byte for low byte of module checksum

Copy AC0 into data area of message as the module ID

If the Prolog Identifier is Zero (AC1=00)

Copy 4E into data area of message as the high byte of the prolog

Copy 75 into data area of message as the low byte of the prolog

Endif the prolog identifier is zero

Endif the exception is one

Copy VIN data into data area of message following the prolog

Calculate module checksum by adding all bytes from module ID through VIN

Copy module checksum into module checksum area of message

Copy message checksum at end of data area of message

Send message and receive the response

Goto UART Return Type Processing

Update and display percent downloaded

If Response Byte in Position 4 is 15

Look for response byte 5 in goto fields

If Response byte 5 is Found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response byte 5 is found

Else

Error invalid response from module

Endif response byte in position 4 is 15

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 98 | D2 | 06 00 F4 01 | 06 99 FD A0 FF A9 00 00 00 00 |

## D4 Op-Code: Download Calibration / Software File using Mode 6 (Saturn EEPROM)

Supported By:

UART Interpreter 0

Description:

The D4 Op-Code is used to download a data file using mode 6 messages.  The device must be prepared to accept a mode 6 message before this Op-Code can be used.  In this particular Op-Code, the mode 5s are sent as part of the D4 logic and do not need to be specified by separate utility file entries.  The "data address information" in the header, is used for the first packet of data that is downloaded, the address will be incremented by the number of bytes that are downloaded in each data packet (i.e. if address 1 = 0x8000 and 0x80 bytes are downloaded in the data packet, then address 2 = 0x8000 + 0x80 = 0x8080).

Warnings:

In order to correctly write a previously calculated checksum to the controller (AC3 = 02), You will first need to run Op-Code D4 with AC3 equal to 0 or 1.  These two routines will store a checksum value to the global checksum area.

In order to correctly write the CAL ID (AC3 = 03) or ABS option byte (AC3 = 05), You will first need to run Op-code D4 with AC3 equal to 0. This will extract the CAL ID and its address from the Saturn calibration file header.

This Op-Code requires two routine blocks to be loaded in the utility file.  The first (AC1) will be the routine data, stored in traditional format. The second (AC1 + 1) will be a header block containing Saturn specific offset information.

EXAMPLE:  If you set AC1 to 7, section 7 will contain your routine and section 8 will contain a block of Saturn specific header data. The next section block available for other Op-Codes will be 9.

Action Fields:

| AC0: | ID of file to download |
| --- | --- |
| AC1: | Section number of routine to download. |
| AC2: | Device ID |
| AC3: | Exceptions. 00 = Send multiple mode 6 messages with each containing a different block of cal data. 01 = Send a single mode 6 which will write “DEAD” to the checksum of the controller 02 = Send a single mode 6 which will write the previously stored global checksum to the controller 03 = Send a single mode 6 which will write the previously stored CAL ID to the controller 04 = Send only the mode 6 file. Do not overlay any additional data. 05 = Send a single mode 6 which will write the traction byte to a Saturn ABS controller. 06 = Send a single mode 6 which will write the VIN. 07 = Send a single mode 6 which will write the odometer (electric vehicle). |

Pseudo Code:

XX     D4   AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

This Op-Code use return type processing one (ignore response).

Find the routine with section equal to AC0 + 1

(This is the header block associated with the routine stored at section AC0)

Extract required information

Find the routine with section equal to AC0

(This is the actual download and run routine)

Load this data into the download request buffer

(continued)

Build Mode 6 Download Request as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC2 | Device ID |
| 2 | 6 + N + M | Message length (N is prolog length, M data bytes in message) |
| 3 | 06 | Message mode |
| 4 | XX | High byte of load address |
| 5 | XX | Low byte of load address |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N + M | XX | Data byte N (N <= maximum as defined in pseudo code) |
| 7 + N + M | CS | 1 byte request checksum. |

If the Exception is One, Two, Three, Four, Five, Six, or Seven (AC3 > 0)

Copy data bytes from appropriate locations

Overlay the copied data bytes into the data portion of the routine

Update size bytes appropriately

Update global checksum if necessary

Send a mode 5 message and receive reply

Fail if error

Send message to write the specified information

Endif the exception is greater than zero processing

If the Exception is Equal to Zero (AC3 = 0)

Find the calibration file with ID equal to AC0

Load the file into memory

Overlay adaptive data from VIT2 to appropriate location in calibration data (if required)

Do while more data to download

Copy M data bytes into message buffer at the appropriate location

If Device ID is F2

Wait for F0 poll with the Tech 2 as Master

else

Wait for F0 poll with the Tech 2 as Slave

Endif device id is F2

Send a mode 8 message

Send a mode 5 message and check response

If the Mode 5 Response is Not Valid

Send the Mode 8 and Mode 5 message up to three more times

If Response is Valid

Continue with Mode 6.

Else

Fail with error.

Endif response is valid

Endif the mode 5 response is not valid

Send the Mode 6 message to write the calibration data

Ignore any response

Increment message data address by number of data bytes downloaded in message

Update and display percent downloaded

End do while more data to download

Update global checksum field for later processing.

Endif the exception is equal to zero

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | D4 | 01 00 F4 00 | 00 09 FD 12 FF 4A 00 00 00 00 |

## D5 Op-Code: Download Calibration / Software File using Mode 12 (Electric Vehicle)

Supported By:

UART Interpreter 0

Description:

The D5 Op-Code is used to download a data file using mode 12 (0C) messages. It was developed specifically for reprogramming the BTCM controller of the EV1 electric vehicle. To perform a download, the device must be prepared to accept a download by receiving a success response from a 05 Op-Code.

The "data address information" in the calibration file, which must use the Saturn calibration file format, is used for the first packet of data that is downloaded. The address will then be incremented by the number of bytes that are downloaded in each calibration data packet (i.e. if address 1 = 0x8000 and 0x80 bytes are downloaded in the data packet, then address 2 = 0x8000 + 0x80 = 0x8080). No prolog is used.

See the pseudo code for more detailed information concerning Op-Code function.

Action Fields:

| AC0: | ID of file to download |
| --- | --- |
| AC1: | Device ID |
| AC2: | Not Used |
| AC3: | Not Used |

Pseudo Code:

XX   D0   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Find the file with ID equal to AC0

Load the file into memory

Build Mode 12 Download Request as indicated in the table below, contents of the XX locations is detailed in the pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC2 | Device ID |
| 2 | 6 + M | Message length (M data bytes in message) |
| 3 | 06 | Message mode |
| 4 | XX | High byte of load address |
| 5 | XX | Low byte of load address |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N + M | XX | Data byte N (N <= maximum as defined in pseudo code) |
| 7 + N + M | CS | 1 byte request checksum |

Copy high byte of data address information from the calibration file header into the data buffer

Copy low byte of data address information from the calibration file header into the data buffer

Expect a 00 in position 3 for success

Do while more data to download

Copy M data bytes into message buffer following the front end

Put message checksum at end of data buffer

Send message and receive the response

Increment message data address by number of data bytes downloaded in message

Update and display percent downloaded

End do while more data to download

Send a Mode FF message; do not expect a response.

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | D5 | 01 F9 00 00 | 00 09 FD 12 FF 4A 00 00 00 00 |

## D6 Op-Code: Erase Flash Block using Mode 22 (Holden Telematics)

Supported By:

UART Interpreter 0

Description:

The D6 Op-Code builds and sends an ALDL mode 22 message. It is used to erase a specified block of flash memory of the device being reprogrammed. This Op-Code was developed specifically for the Holden Telematics controller but can be used for any controllers using mode 22 in the same manner.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | Block Number |
| AC2: | Not Used |
| AC3: | Not Used |

Pseudo Code:

XX   D6   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build Mode 22 Flash Erase Request as indicated in the table below. Contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 4 + N | Message length |
| 3 | 16 | Message mode |
| 4 | XX | Block number |
| 5 | CS | 1 byte request checksum |

Copy AC1 address into message byte 4

Send the message and expect a response

If message response was SUCCESS

If the response byte 3 is not $16 (expected mode)

Set message response FAILURE

Endif the response byte 3 is not $16 (expected mode)

Endif message response was SUCCESS

If message response was SUCCESS

Set response code to response byte 5

Else

Set response code to NO COMM

Endif message response was SUCCESS

Look for response code in Goto fields

If response code is found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response code is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | D6 | E4 01 00 00 | 06 11 15 0D FF 99 00 00 00 00 |

## D7 Op-Code: Download Software Module using Mode 23 (Holden Telematics)

Supported By:

UART Interpreter 0

Description:

The D7 Op-Code builds and sends ALDL mode 23 messages with three address bytes. It is used to send modules to the flash memory of the device being reprogrammed. This Op-Code was developed specifically for the Holden Telematics controller but can be used for any controllers using mode 23 in the same manner.

Note:	 See Pseudo code for more information on maximum packet sizes.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | Calibration Number |
| AC2: | Not Used |
| AC3: | Not Used |

Pseudo Code:

XX   D7   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build Mode 23 Flash Program And Verify Request as indicated in the table below. Contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC0 | Device ID |
| 2 | 4 + N | Message length |
| 3 | 17 | Message mode |
| 4 | XX | High byte of download address |
| 5 | XX | Middle byte of download address |
| 6 | XX | Low byte of download address |
| 7 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 7 + N | XX | Data byte N (N <= maximum as defined in pseudo code) |
| 8 + N | CS | 1 byte request checksum |

Find the file with ID equal to AC1

Load the file into memory

If AC3 is $00

Get download address from utility file header

Else

Get download address from global address (must be set by Op-Code $F1)

Endif AC3 is $00

Get number of bytes to download from file size

Get packet length from utility file header

Do while number of bytes to download not yet 0

If number of bytes to download is greater than packet length

Limit packet length to number of bytes to download

Endif number of bytes to download is greater than packet length

Copy download address into message bytes 4.6

Copy next data block from file into message starting with byte 7, packet length specifies the number of bytes to copy

Send the message and expect a response

(continued)

If message response was SUCCESS

If the response byte 3 is $17 (expected mode)

If response byte 4 is $06 (download completed)

Increment the download address by the packet size

Decrement the number of bytes to download by the packet length

Else

Break the loop

Endif response byte 4 is $06 (download completed)

Else

Set message response FAILURE

Break the loop

Endif the response byte 3 is $17 (expected mode)

Else

Break the loop

Endif message response was SUCCESS

Enddo while number of bytes to download not yet 0

If message response was SUCCESS

Set response code to response byte 4

Else

Set response code to NO COMM

Endif message response was SUCCESS

Look for response code in Goto fields

If response code is found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response code is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | D7 | E4 01 00 00 | 06 11 15 0D FF 99 00 00 00 00 |

## D8 Op-Code: Execute Software Module using Mode 24 (Holden Telematics)

Supported By:

UART Interpreter 0

Description:

The D8 Op-Code builds and sends an ALDL mode 24 message with three address bytes. It is used to start the execution of a software module in the flash memory of the device being reprogrammed. This Op-Code was developed specifically for the Holden Telematics controller but can be used for any controllers using mode 24 in the same manner.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | Start address – high byte |
| AC2: | Start address – mid byte |
| AC3: | Start address – low byte |

Pseudo Code:

XX   D8   AC0   AC1   AC2   AC3    G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build Mode 24 Flash Memory Execute Request as indicated in the table below. Contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AC3 | Device ID |
| 2 | 04 | Message length |
| 3 | 18 | Message mode |
| 4 | XX | High byte of start address |
| 5 | XX | Middle byte of start address |
| 6 | XX | Low byte of start address |
| 7 | CS | 1 byte request checksum |

Copy start address from AC1.AC3 into message bytes 4.6

Send the message and expect a response

If message response was SUCCESS

If the response byte 3 is not $18 (expected mode)

Set message response FAILURE

Endif the response byte 3 is not $18 (expected mode)

Endif message response was SUCCESS

If message response was SUCCESS

Set response code to response byte 4

Else

Set response code to NO COMM

Endif message response was SUCCESS

Look for response code in Goto fields

If response code is found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif response code is found

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | D8 | E4 00 00 00 | 06 11 15 0D FF 99 00 00 00 00 |

## F0 Op-Code: Wait for Poll from a Device

Supported By:

UART Interpreter 0

Description:

The F0 Op-Code waits for a poll from a device.

Action Fields:

| AC0: | Device ID |
| --- | --- |
| AC1: | Number of 100 ms intervals to wait |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |
| G0: | 00 - Don't care |

Pseudo Code:

XX  F0  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Do While Wait Time is Not Greater than (AC1*100ms)

Wait for a poll from device ID AC0

End do

Goto step specified in goto field 1 (G1), this Op-Code ignores no communication faults

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | F0 | F4 05 00 00 | 00 11 FF 99 00 00 00 00 00 00 |

# Interpreter 1 - Class 2 Communications Programming Op-Codes

## Class 2 No Communications Processing

Supported By:

Class 2 Interpreter 1

Description:

Normal processing of a request message is to re-send the request up to three times if the transmission of a request fails.  However, if the transmission is successful and no controllers respond to the request, then the request will fail with a no comm. Fault**.

When writing instructions for a Utility File, the ‘FD’ code can be used to determine the next step to execute after a no comm. fault occurs. The 'FD' processing can be used for any step in the utility file where a no comm. condition can occur**.

** Note:

- There are some Class 2 Op-Codes that send out a request and do not look for a response (i.e. Ignore Response).

- There are some Class 2 Op-Codes that send out a request and don’t care if there are no responses (i.e. Ignore Error).

- There are some Class 2 Op-Codes that do not support the “no comm.” (‘FD’) processing.

These special cases are defined within the individual Class 2 Op-Codes.

Pseudo Code:

XX     OP  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

If “no comm.” occurred

If the Op-Code Ignores No Communication Faults	 (Ignore Error or Ignore Response)

Continue reprogramming event based on specific Op-Code logic

Else

Check goto fields for an FD return code (G0, G2, G4, G6, or G8)

If found (‘FD’)

Flag appropriate step to execute next and return

Else

Check goto fields for an FF return code (G0, G2, G4, G6, or G8)

If found (‘FF’)

Flag appropriate step to execute next and return

Else

End programming with fatal error

Endif

Endif

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 11 | OP | B4 03 10 00 | 65 12 FD 2A FF 4A 00 00 00 00 |

## Class 2 Response Processing

Supported By:

Class 2 Interpreter 1

Description:

The Response Processing defines how the tool deciphers the response(s) returned from a control module after a successful request has been transmitted to the control module **.

The response buffer fields, filled when a controller responds to a message, are identified as follows:

Byte 1 		- Priority and Message Type

Byte 2 		- Target Address 	(typically targeted to the tester device)

Byte 3 		- Source Address 	(typically sent from a control module)

Byte 4		- Message ID 		(typically a positive/negative response ID)

Byte 5 		- Misc. Response Data (N = 4096 bytes max.)

Byte N+5 	- High Byte of Block Transfer Message Checksum

Byte N+6 	- Low Byte of Block Transfer Message Checksum

** Note:

- There are some Class 2 Op-Codes that send out a request and do not look for a response (i.e. Ignore Response).

- There are some Class 2 Op-Codes that send out a request and don’t care if there are no responses (i.e. Ignore Error).

- There are some Class 2 Op-Codes that do not support the “no comm.” (‘FD’) processing.

These special cases are defined within the individual Class 2 Op-Codes.

Pseudo Code:

“Ignore Response” Processing

If the op-code specifies a response processing type of “Ignore Response”

No response messages will be processed (read) from the vehicle communication link.

Response processing will return immediately with “Success” after the request has been transmitted

Endif

“Ignore Error” Processing

If the op-code specifies a response processing type of “Ignore Error”

The response data (one or more messages) WILL be processed (read) from the vehicle communication

link (based on “FE Response Processing” or “Individual Device Response Processing”).

Regardless of whether the response is a positive, negative, or no response is received, response

processing will return with “Success”

Endif

“FE” (all devices) Response Processing

If the request message is targeted to “all” (FE) devices

Process the response from the controller(s) on the vehicle communication link

(only for controllers that respond to the “FE” request)

If a negative (7F) response is processed from one of the “responding” controllers

Return immediately with “failed” (negative response) and the data from the negative response

Endif

Wait for 2 seconds of no responses being processed (idle vehicle communication link)

If after 2 seconds there have been NO responses processed (zero in total)

Return with a “no communication” response

Else

Return with “Success” and the data from the last response processed

Endif

Endif

“Individual Device” Response Processing														(continued)

If the request message is targeted to a “specific“ device (ex. $10, $11, $18, $28, $40, …)

Process the response from the controller on the vehicle communication link

If a positive response was received (Response Message ID = Mode Number + $40)

Return with “Success” and the positive response data

Endif

If a negative response was received (Response Message ID = 0x7F)

If the last byte of the response is a $23 (Routine Not Complete)

Wait up to one more second for an additional (positive/negative) response

If no more responses are received (in the 1 second)

Return with “failed” (negative response) and the data from the (0x7F . . . 0x23) response

Endif

Endif

If the Response Message “target address” does not equal the Request Message “source address”

Keep attempting to process responses from the controller

If no more responses are received (in the remaining amount of time)

Return with “failed” (negative response) and the data from the mis-matched target/source

response

Endif

Endif

Endif

If “no response” was ever received from the individual device

Return with a “No Communication” response

Endif

Special Mode $36 Response Processing

If a positive response was received (Response Message ID = $76)

If “response code” data byte is a $78 or a $6F (Response Data Byte #2 – “action not yet complete”)

Keep attempting to process responses from the controller until a “true” positive or negative is

received

Endif

Endif

Endif

## 01 Op-Code: Setup Global Variables

Supported By:

Class 2 Interpreter 1

Description:

The 01 Op-Code is used to set the global variables used during the programming process. The Op-Code allows the user to alter the global settings any time during the programming event by issuing another 01 Op-Code.

AC0 is used to set the Global Source Address. The Global Source Address is initialized to F0.

AC1 is used to toggle the option to send periodic 3F messages to all devices (target address = FE) during programming. This option should be turned ON when the tool is required to send a periodic 3F message to all devices to prevent all controllers on the link from resuming normal communications during a reprogramming event. If this option is ON, the 3F message will be transmitted at least once every 2.45 seconds. The default setting for this option is ON.

If the default settings are correct, the 01 Op-Code is not required in the Interpreter instructions.

Action Fields:

| AC0: | Global Source Address (Tester ID) |
| --- | --- |
| AC1: | 00 - Do not send periodic 3F messages to all devices 01 - Send periodic 3F messages to all devices during programming |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  01  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Set global source address to value in AC0

If AC1 = 00

Turn periodic 3F transmission to all devices (Target address = "FE") OFF

Else

Turn periodic 3F transmission to all devices (Target address = "FE") ON

Endif AC1 = 00

Goto step specified by goto field one (G1)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 01 | F0 01 00 00 | 00 02 00 00 0 00 00 00 00 00 |

## 10 Op-Code: Initiate Diagnostic Operation

Supported By:

Class 2 Interpreter 1

Description:

Note:	This Op-Code is NOT functional within the software application (i.e.   A request to initiate a Diagnostic Operation Mode will never be sent out over the vehicle communication link.)  The response processing will be based on GOTO field G1.

The 10 Op-Code is used to inform devices on the serial data link that the tool is ready to start diagnostic procedures using the link and the devices on the link may need to alter their normal operation. (This Op-Code is seldom used in programming. It was used only by truck in the 1995 model year.)

AC0 is used to set the Target Device ID.

AC1 is the level of diagnostic operations to be performed. See XDE - 3001B page 93 for a list of level operation codes.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | 00 – Default (See XDE-3001B for details) |
| AC2: | 00 - Not used |
| AC3: | 00 - Not Used |

Build message 10 as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 10 | Initiate Diagnostic Operation Mode |
| 5 | AC1 | Level of diagnostic operation |
| 6 | CS | 1 byte message checksum (handled by the system) |

Pseudo Code:

XX  10  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

N/A - Set the target device ID to value in AC0

N/A – Set the diagnostic operation level (within the request) to AC1

N/A – Send Diagnostic message to inform devices on the link

N/A – Goto Class 2 Response Processing

This Op-Code is NOT supported.  The response processing will be based on GOTO field G1.

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 10 | 10 90 00 00 | 50 02 FF 99 00 00 00 00 00 00 |

## 14 Op-Code: Clear Diagnostic Information

Supported By:

Class 2 Interpreter 1

Description:

The 14 Op-Code builds a standard mode 14 message that initializes all diagnostics and diagnostic information in a device. Diagnostic information is, but is not limited to, diagnostic trouble codes (DTC) and corresponding DTC status, freeze frame data or other on-board test results that may be stored as a result of diagnostic activity.

This Op-Code will ignore the error of “no response” being received from the request. If this condition occurs, then Goto Step G1 will be executed.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | 00 – Not Used |
| AC2: | 00 – Not Used |
| AC3: | 00 – Not Used |

Pseudo Code:

XX  14  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Build Return to Normal Mode message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 14 | Clear Diagnostic Information Mode |
| 5 | CS | 1 byte message checksum (handled by the system) |

Send Mode 14 message and receive the response

Goto Class 2 Response processing

If No response was received

Goto step G1

Else

Look for “response byte 4” in goto fields

If Response Byte 4 is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 1F | 14 | 10 00 00 00 | 54 20 7F 1B FF 24 00 00 00 00 |

## 20 Op-Code: Return to Normal Mode

Supported By:

Class 2 Interpreter 1

Description:

The 20 Op-Code builds a standard mode 20 message to return a device to Normal Mode operating conditions.

This Op-Code will ignore the error of “no response” being received from the request. If this condition occurs, then Goto Step G1 will be executed.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  20  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Build Return to Normal Mode message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 20 | Return to Normal Communication Mode |
| 5 | CS | 1 byte message checksum (handled by the system) |

Send Mode 20 message and receive the response

Goto Class 2 Response processing

Set the link to “Normal Speed” 10400 baud

Wait 1000 milliseconds

If No response was received

Goto step G1

Else

Look for “response byte 4” in goto fields

If Response Byte 4 is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 1F | 20 | 10 00 00 00 | 60 20 7F 1C FF 24 00 00 00 00 |

## 27 Op-Code: Perform Security - Mode 27

Supported By:

Class 2 Interpreter 1

Description:

The 27 Op-Code is used to open a control module's memory for reprogramming. The procedure number, located in Action Field one (AC1), identifies which security algorithm to use for unlocking the control module for programming.

A Mode $28 (Disable Normal Communications) is sent out to all devices ($FE) prior to performing the security request (Mode $27) unless AC3 equals 01.  This request is sent out as a precautionary measure to ensure that normal communications on the link has been stopped.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | 00 Default Security - Ones Complement |
|  | ?? Production Security |
| AC2: | 00 - Not Used |
| AC3: | 01 - Do not send out a Mode $28 (Disable Normal Communications) request to all devices prior to the Mode $27. 00,02-FF - Send out the Mode $28. |

Pseudo Code:

XX  27  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

If AC3 is not equal to 01 (send a Disable Normal Communications request)

Set the Target Device ID to $FE (All devices)

Send out a Mode $28 request (Disable Normal Communications)

Set the Target Device ID to AC0

Part 1 – Request Seed

Build Security Access message 1 as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 27 | Security access message mode |
| 5 | 01 | Security access request number |
| 6 | CS | 1 byte message checksum (handled by the system) |

Send Security Access message 1 to request seed and receive response

If Message 1 Response Is Positive

If Message 1 Response Contains a Seed

If Both Bytes of the Seed (response bytes 6 & 7) Are Zeros (controller already unlocked)

Set “response code” to ‘34’

Done - Goto “Process Response Code”

Else

If Security Algorithm (AC1) Not Yet Used

Use specified security algorithm (AC1) to determine device's key

Goto “Part 2 – Send Key”

Else

Use one's complement of seed to determine device's key

Goto “Part 2 – Send Key”

EndIf

EndIf

Else

Set “response code” to the last response byte (byte 6)

EndIf

Else

If Message 1 Response Is Negative															(continued)

Set “response code” to negative response code (last response byte)

Else

If No Response to Message 1 is received

Set “response code” to NO Response (‘FD’)

Done - Goto “Process Response Code”

Else

Return with the failed error

Done - Goto “Process Response Code”

EndIf

EndIf

Endif

If Security Key Was Not Determined and the Response from Message 1 reflects a “Time Delay Not Expired”

If Message 1 Was Not Yet Sent 2 Times

Wait 10 seconds (with tester present messages)

Re-execute the “Part 1 – Request Seed” logic

Else

Return with the failed error

Done - Goto “Process Response Code”

Endif

Else

Return with the failed error

Done - Goto “Process Response Code”

Endif

Part 2 – Send Key

Build Security Access message 2 as indicated in the table below (when a Seed has been received)

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 27 | Security access message mode |
| 5 | 02 | Security access request number |
| 7 | XX | High byte of key |
| 8 | XX | Low byte of key |
| 9 | CS | 1 byte message checksum (handled by the system) |

Send Security Access message 2 key response and receive response

If Message 2 Response Is Positive

If Response Byte 6 Is 34 (Box is now Unlocked)

Set “response code” to ‘34’

Done - Goto “Process Response Code”

Else

If Response Byte 6 Is 35 (Invalid Key)

If Running on DPS Tool And One's Complement Not Yet Used

Reset message 1 counter and try the One’s Complement

Re-execute the “Part 1 – Request Seed” logic

Else

Return with the failed error (Response Byte 6)

Done - Goto “Process Response Code”

Endif

Else

Return with the failed error (Response Byte 6)

Done - Goto “Process Response Code”

Endif

Endif

Else

If Message 2 Response Is Negative

Set “response code” to negative response code (Response Byte 4 = 0x7F)

Done - Goto “Process Response Code” 													(continued)

Else

If No Response to Message 2 is received

Set “response code” to NO Response (‘FD’)

Done - Goto “Process Response Code”

Else

Return with the failed error

Done - Goto “Process Response Code”

EndIf

Endif

Endif

Process Response Code

If “response code” Is Found

Goto appropriate step indicated in the goto field

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 27 | 10 22 00 00 | 34 02 7F 1A FD 1F FF 1F 00 00 |

## 28 Op-Code: Disable Normal Communications - Mode 28

Supported By:

Class 2 Interpreter 1

Description:

The 28 Op-Code is used to disable communications on the data link. The mode 28 message is sent to the controller at normal speed.  Normal communications will resume if the link is inactive for more than five seconds. Communications will remain inhibited while tester present messages are sent out on the link at least once every five seconds.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Disable Level |
|  | 00 - Disable normal Class 2 communications |
|  | 01 - Disable normal UART communications |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  28  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Build Disable Communications message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 28 | Disable communication message mode |
| 5 | AC1 | Disable Level |
| 6 | CS | 1 byte message checksum (handled by the system) |

Send Mode 28 message and get the response(s)

Goto Class 2 Response Processing

If No response was received

Look for ‘FD’ (no comm.) in goto fields

If ‘FD’ is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Else

Look for “response byte 4” in goto fields

If Response Byte 4 is found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 28 | 10 00 00 00 | 68 02 7F 04 FD 1B FF 1D 00 00 |

## 29 Op-Code: Resume Normal Communications - Mode 29 (Obsolete Use Op-Code 20)

Supported By:

Class 2 Interpreter 1 - For legacy ECUs only

Description:

The 29 Op-Code is used to resume normal communications on the data link. The mode 29 message is sent to the controller at normal speed. Normal communications will also resume if no activity exists on the link for more than five seconds.

This Op-Code will ignore the error of “no response” being received from the request. If this condition occurs, then Goto Step G1 will be executed.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  29  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Build Enable Communications message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global source address |
| 4 | 29 | Enable communications message mode |
| 5 | CS | 1 byte message checksum (handled by the system) |

Set the link to “Normal Speed” 10400 baud

Wait 750 milliseconds

Send Mode 29 message and get the response(s)

Goto Class 2 Response Processing

If No response was received

Goto step G1

Else

Look for “response byte 4” in goto fields

If Response Byte 4 is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | 29 | 10 00 00 00 | 69 09 7F 14 FF 20 00 00 00 00 |

## 31 Op-Code: Start a Diagnostic Routine by Test Number

Supported By:

Class 2 Interpreter 1

Description:

The 31 Op-Code is used to execute a diagnostic test. The diagnostic routine is executed in the control module by referencing the test number in AC1. Definition of the routine is defined by the control module specification. The module must be capable of exiting the test automatically to use this Op-Code.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Test Number |
| AC2: | 00 - Not used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  31  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Build Start Diagnostic Routine message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 31 | Start diagnostic routine message mode |
| 5 | AC1 | Test number |
| 6 | CS | 1 byte message checksum (handled by the system) |

Send Mode 31 message and get response

Goto Class 2 Response Processing

If No response was received

Look for “FD” (no comm.) in goto fields

If ‘FD’ is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Else

Look for “response byte 4” in goto fields

If Response Byte 4 is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | 31 | 40 22 00 00 | 71 09 73 09 FD 12 FF 99 00 00 |

## 34 Op-Code: Request the Control Module for Block Transfer - Mode 34

Supported By:

Class 2 Interpreter 1

Description:

The 34 Op-Code will build a standard mode 34 message to send to a device. A mode 34 command will prepare the device to receive executable code via block transfer from the serial data link. Mode 34 messages are used in conjunction with mode 36 (Op-Codes B0, B3, and B4) to download information to devices. The mode 34 may not be required prior to sending a mode 36 - it is device dependent.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Routine number to download |
| AC2: | Exceptions: 00 - Use address and length from the routine indicated by AC1 01 - Use address and length from the utility file header 02 - Send message without transfer type or length and address information. Ignores AC1 and AC3. |
| AC3: | Transfer type |

Pseudo Code:

XX  34  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

If AC2 is Not Equal to 02

Set Transfer Type to value is AC3

Else

Exclude Transfer Type from Message

Endif

If AC2=02

Build Download Request as indicated in the table below:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 34 | Request download message mode |
| 5 | CS | 1 byte message checksum (handled by the system) |

Send Mode 34 message and receive response

Goto Class 2 Response Processing

Goto Mode 34 Response Processing

Else

Find the routine indicated by value AC1

Build Download Request as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 34 | Request download message mode |
| 5 | AC3 | Transfer type |
| 6 | XX | High byte of number of data bytes |
| 7 | XX | Low byte of number of data bytes |
| 8 | XX | High byte of starting address |
| 9 | XX | Middle byte of starting address |
| 10 | XX | Low byte of starting address |
| 11 | CS | 1 byte message checksum (handled by the system) |

If AC2=00 (Use address and length from the routine indicated by AC1)						(continued)

Copy high byte of routine length from the routine into message (should not exceed length in header)

Copy low byte of routine length from the routine into message (should not exceed length in header)

Copy high byte of RAM address to download routine (from routine) into message

Copy middle byte of RAM address to download routine (from routine) into message

Copy low byte of RAM address to download routine (from routine) into message

Else If AC2=01 (Use address and length from the utility file header)

Copy high byte of data length from the Header into the message

Copy low byte of data length from the Header into the message

** Note:	  The data length value in the header can be changed via the “$F5 Override the Utility File

**		  Message Length Field” Op-Code.

Copy high byte of data address information from the Header into the message

Copy middle byte of data address information from the Header into the message

Copy low byte of data address information from the Header into the message

Endif AC2=00

Send Mode 34 message and receive response

Goto Class 2 Response Processing

Goto Mode 34 Response Processing

Endif

Mode 34 Response Processing

If a positive response was received

If AC2 = 2 (ignore transfer type, address and length information)

Set the “response code” to “response byte 5”

Else

Set the “response code” to “response byte 6”

Endif

Else

If a negative response was received

Set the “response code” to the last response byte in the negative response message

Else

Set the “response code” to ‘FD’ (no comm.)

Endif

Endif

If the “response code” is 00

Goto step G1

Else

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Endif

Typical Interpreter Line (ignore transfer type, address and length information):

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 34 | 10 00 02 00 | 44 11 12 0D FD 22 FF 99 00 00 |

Typical Interpreter Line (use address and length information from the routine) :

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 34 | 10 03 00 01 | 44 11 12 0D FD 22 FF 99 00 00 |

## 3B Op-Code: Write block of memory

Supported By:

Class 2 Interpreter 1

Description:

The 3B Op-Code is used to download data to EEPROM using mode 3B messages.  The data that is downloaded is determined by the information contained in the Action Fields.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Calibration ID or Block Number being downloaded |
|  | 01 - Send VIN bytes 1 – 5 |
|  | 02 - Send VIN bytes 6 – 11 |
|  | 03 - Send VIN bytes 12 – 17 |
|  | 08 - Send end model part number (Send in USN format unless AC3=03, then send in BCD format) |
|  | 22 - Send Repair Shop Code bytes 1 – 4 |
|  | 23 - Send Repair Shop Code bytes 5 – 10 |
|  | 24 - Send Programming Date |
|  | 5F - Send Saturn PCM Transmission Oil Life data (“TOL”) |
|  | 62 - Send Saturn PCM engine distance counter (“EDC”) |
|  | 6D - Send Saturn PCM Engine Oil Life data (“OIL”) |
|  | 6E - Reset PCM P0601 Diagnostic Trouble Code Latch Indicator (Not Supported) |
|  | 80 - Send Saturn PCM Adaptive data (“PA2”) |
|  | 81 - Send Saturn PCM Transmission Adaptive data (“TFL”) |
|  | 82 - Send Saturn PCM Transmission Adaptive data [2000-2002 Z-Car (“ITN”), otherwise (“PA2”)] |
|  | 83 - Send Saturn PCM Transmission Adaptive data [Delta (“B83”), otherwise (“ITP”)] |
|  | - - Send Saturn Transmission adaptive data for PCM or SDM keying data for BCM [PCM (“DSL”), otherwise (“SDK” or “SDM”)] |
|  | 85 - Send Saturn BCM option data (“OP3”) |
|  | 86 - Send Saturn BCM engine & ABS indicator [try Delta (“B86”), if no config. data use (“OP4”)] |
|  | 87 - Send Saturn BCM Fault Code Enable data (“FCE”) |
|  | 88 - Send Saturn BCM option data [Use a combination of (“OP1”) and (“T88”)] |
|  | 89 - Send Saturn BCM option data [Use a combination of (“T89”) and (“OP2”)] |
|  | 8E - Send Saturn PCM DVT data (“DVT”) |
|  | A0 - Reset manufacturers enable counter [Saturn Delta and Z-car 2000-2004] |
|  | A1 - Send Saturn BCM odometer setting (“ODO”) |
|  | A2 - Send Saturn BCM SVS TT Enable Information (“SV1”) |
|  | A3 - Send Saturn BCM option data & ODO write inhibit (“SV2”) |
|  | A4 - Send Saturn BCM VIN bytes 1-6 [Saturn Z-car 2000-2002] or BCM DTC Enable Information [All others (“DC1”)] |
|  | A5 - Send Saturn BCM VIN bytes 7-12 [Saturn Z-car 2000-2002] or BCM DTC Enable Information [All others (“DC2”)] |
|  | A6 - Send Saturn BCM VIN bytes 13-17 |
|  | A8 - Send Saturn BCM VTD, ODO./VIN write inhibit & Content Theft [try (“BA8”), if no config. data use (“VTD”)] |
|  | A9 - Send Saturn BCM ODO (“ODO”) |
|  | AA - Send Saturn BCM ODO (“ODO”) |
|  | AB - Send Saturn BCM ODO (“ODO”) |
|  | C8 - Send Saturn BCM Transmitter TIC1 (“TR1”) |
|  | C9 - Send Saturn BCM Transmitter TIC1 (“TR2”) |
|  | CA - Send Saturn BCM Transmitter TIC2 (“TR3”) |
|  | CB - Send Saturn BCM Transmitter TIC2 (“TR4”) |
|  | CC - Send Saturn BCM Transmitter TIC3 (“TR5”) |
|  | CD - Send Saturn BCM Transmitter TIC3 (“TR6”) |
|  | CE - Send Saturn BCM Transmitter TIC4 (“TR7”) |
|  | CF - Send Saturn BCM Transmitter TIC4 (“TR8”) |
| AC2: | ## - Routine number to build into message. (See exception 04) Or ## - ID for storage location for data to write (0x00 – 0x13) (See exception 08) |
| AC3: | Exception control action field 00 - Used to write Configuration / VIN / RSCOSN / … data to a block (AC1) 01 - Used to configure a Module using the 3B mode by using data from a calibration file (AC1). Negative responses will be ignored. 02 - Used to reset (clear) all learned values associated with the data block to initial values as defined by the controller. The Block Number to reset is indicated by AC1. 03 - Used to write end model part number in binary coded decimal format. Usable option only when AC1 is 08. 04 - Used to write a routine from the utility file to a memory block, where AC2 is the routine number and AC1 is the block number. 05 - Used to configure a Module using the 3B mode by using data from a calibration file (AC1) that includes header information within the file. The header information will be skipped – 12 bytes. Negative responses will be ignored. 06 - Used to configure a Module using the 3B mode by using data from a calibration file (AC1). A negative response will be processed and will halt the configuration data writing. 07 - Used to configure a Module using the 3B mode by using data from a calibration file (AC1) that includes header information within the file. The header information will be skipped – 12 bytes. A negative response will be processed and will halt the configuration data writing. 08 – Used to write data from a 256-byte internal storage buffer (AC2) to the block number identified in AC1. |

Pseudo Code:

XX     3B  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use Ignore Class 2 Response Processing

If (AC3 = 01 or AC3 = 06)

See example 2 of Pseudo Code

Else

If AC3 = 02

See example 3 of Pseudo Code

Else

If AC3 = 04

See example 4 of Pseudo Code

Else

If (AC3 = 05 or AC3 = 07)

See example 5 of Psuedo Code

Else

If (AC3 = 00 or AC3 = 03)

See example 1 of Pseudo Code

ElseIf (AC3 = 08)

See example 6 of Pseudo Code

Endif

Endif

Endif

Endif

Endif

Example 1:

If AC3 = 00 or AC3 = 03, build Write Block of Memory message as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global source address |
| 4 | 3B | Write block of memory mode |
| 5 | AC1 | Block number |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N –1 | XX | Data byte N (maximum of 6) |
| 6 + N | CS | 1 byte message checksum (handled by the system) |

(continued)

If the AC1 is set to $A4, $A5, $A6, $A8, $A9, $AA, or $AB and “CAL” ECU configuration data is available.

Set the “response code” to 0x7B

Else

Set block number to AC1.

Copy the appropriate data (see the “Action Field” table value descriptions above)

Send 3B message and get response.

If a positive or negative response was received

Set the “response code” to Response Byte 4

Else

Set the “response code” to ‘FF’

Endif

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:  (AC3 = 00)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 3B | 10 01 00 00 | 7B 22 7F 24 FF 99 00 00 00 00 |

Typical Interpreter Line:  (AC3 = 03)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 3B | 10 08 00 03 | 7B 22 7F 24 FF 99 00 00 00 00 |

Example 2:

If AC3 = 01 or AC3 = 06, build Write Block of Memory message as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global source address |
| 4 | 3B | Write block of memory mode |
| 5 | XX | Block number |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N –1 | XX | Data byte N (maximum of 6) |
| 6 + N | CS | 1 byte message checksum (handled by the system) |

Example of Calibration:

020784226552130000078501E200000000  (must be in binary format)

Block count		= 02

Block 1  Len.	= 07

Block 1			= 84 22 65 52 13 00 00

Block 2  Len.  	= 07

Block 2			= 85 01 E2 00 00 00 00

Read calibration AC1

Set Block count based on byte 1 of calibration AC1 (each calibration consists of multiple blocks)

Do While Block Count > 0

Copy Block # length bytes of calibration AC1 Block # into message

(maximum of 6 data bytes plus one block ID, zero fill any unused bytes in the calibration file)

Send Tester Present message to all devices (0xFE)

Send 3B message

(continued)

If (a negative response is received and AC3 = 06)

End the Do While loop (no more data processing)

Else

(ignore any negative response)

Endif

If (no response is received)

End the Do While loop

Endif

Subtract 1 from Block count

Setup to get the next Block information (Length and Data)

End While

If (no response is received)

Set the “response code” to ‘FF’

Else

If AC3 = 01 and all positive or negative responses were received

Set the “response code” to “7B”

Else

Set the “response code” to Response Byte 4

Endif

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 3B | 40 01 00 01 | 7B 22 FF 99 00 00 00 00 00 00 |

Example 3:

If AC3 = 02, build Write Block of Memory message as indicated in the table below.

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global source address |
| 4 | 3B | Write block of memory mode |
| 5 | AC1 | Block number |
| 6 | CS | 1 byte message checksum (handled by the system) |

Send Mode 3B message with NO data bytes and receive response.

If a positive or negative response was received

Set the “response code” to Response Byte 4

Else

Set the “response code” to ‘FF’

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

(continued)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 13 | 3B | 10 83 00 02 | 7B 14 7F 5B FF 99 00 00 00 00 |

Example 4:

If AC3 = 04, build Write Block of Memory message as indicated in the table below.

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global source address |
| 4 | 3B | Write block of memory mode |
| 5 | AC1 | Block number |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N –1 | XX | Data byte N (maximum of 6) |
| 6 + N | CS | 1 byte message checksum (handled by the system) |

Set block number to AC1.

Copy all data bytes of routine AC2 (up to 6 bytes) into message.

Send Tester Present message to all devices (0xFE)

Send 3B message and get response.

If a positive or negative response was received

Set the “response code” to Response Byte 4

Else

Set the “response code” to ‘FF’

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 3B | 40 B3 01 04 | 7B 22 7F 2E FF 99 00 00 00 00 |

Example 5:

If AC3 = 05 or AC3 = 07, build Write Block of Memory message as indicated in the table below (contents of the XX locations is detailed in pseudo code following the table)

This example is identical to Example 2 except that this usage expects a 12 byte header on the calibration data file.  In this case, the operation of the data will begin at byte 13.

(continued)

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global source address |
| 4 | 3B | Write block of memory mode |
| 5 | XX | Block number |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N –1 | XX | Data byte N (maximum of 6) |
| 6 + N | CS | 1 byte message checksum (handled by the system) |

Example of Calibration:

8888000100F4240041410000020784226552130000078501E200000000  (must be in binary format)

Module Checksum 	= 8888

Module ID		= 0001

Part Number		= 00 F4 24 00  (16000000)

Design Level Suffix	= 41 41  (AA)

Pad Bytes		= 00 00

Block count		= 02

Block 1 Length		= 07

Block 1		= 84 22 65 52 13 00 00

Block 2 Length		= 07

Block 2		= 85 01 E2 00 00 00 00

Read calibration AC1

Set Block count based on byte 13 of calibration AC1 (each calibration consists of multiple blocks)

(skip the 12 byte header information)

Do While Block Count > 0

Copy Block # length bytes of calibration AC1 Block # into message

(maximum of 6 data bytes plus one block ID, zero fill any unused bytes in the calibration file)

Send Tester Present message to all devices (0xFE)

Send 3B message and process the response

If (a negative response is received and AC3 = 07)

End the Do While loop (no more data processing)

Else

(ignore any negative response)

Endif

If (no response is received)

End the Do While loop (done)

Endif

Subtract 1 from Block count

Setup to get the next Block information (Length and Data)

End While

If (no response is received)

Set the “response code” to ‘FF’

Else

If AC3 = 05 and all positive or negative responses were received

Set the “response code” to “7B”

Else

Set the “response code” to Response Byte 4

Endif

Endif

Look for “response code” in the goto fields

(continued)

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 3B | 40 01 00 05 | 7B 22 FF 99 00 00 00 00 00 00 |

Example 6:

If AC3 = 08, build Write Block of Memory message as indicated in the table below.

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global source address |
| 4 | 3B | Write block of memory mode |
| 5 | AC1 | Block number |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N –1 | XX | Data byte N (maximum of 6) |
| 6 + N | CS | 1 byte message checksum (handled by the system) |

Set block number to AC1.

If the storage location number (AC2) is valid

Copy the data from the 256-byte storage location indicated by AC2 into the message.

Note1: 	For the Class 2 protocol, the storage location should first be populated with the $3C OpCode for this exception to work; however, if the storage location is pre-populated with OpCode $54 or some other method, then see Note2.

Note2: 	The amount of data (length) stored in the storage location is located in the 7th storage buffer byte (buffer[6]).  The reading/writing of data (for Class 2) can never be more than 6 data bytes.  An example of 3 data bytes (0x12, 0x23, 0x34) stored in an internal storage location would be:  StorageLocation:  0x12, 0x23, 0x34, 0x00, 0x00, 0x00, 0x03, ….

Else

End with an error.

Endif

Send Tester Present message to all devices (0xFE)

Send 3B message and get response.

If a positive or negative response was received

Set the “response code” to Response Byte 4

Else

Set the “response code” to ‘FF’

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 3B | 10 09 04 08 | 7B 22 7F 2E FF 99 00 00 00 00 |

## 3C Op-Code: Read and verify block of memory

Supported By:

Class 2 Interpreter 1

Description:

The 3C Op-Code is used to read data from EEPROM using mode 3C messages.  The data that is read is determined by the information contained in the Action Fields.  The requested data can then be compared against a stored value in VIT2, however at this time only the VIN blocks 01, 02 and 03 are supported for the comparison.  The success of the function is based on either the comparison (if done), or the success of the data read request (if the comparison is not done).

The response data is stored in one of the 256-byte internal buffers based on the value of AC3.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Block Number being read |
|  | 01 - Read VIN bytes 1 – 5 |
|  | 02 - Read VIN bytes 6 – 11 |
|  | 03 - Read VIN bytes 12 – 17 |
| AC2: | Exceptions: 01 - Display Immobilizer Pin from data bytes 1 - 4 |
| AC3: | ID for storage location of response bytes (0x00 – 0x13) |

Pseudo Code:

XX     3C  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build Read Block of Memory message as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target address |
| 3 | ## | Global source address |
| 4 | 3C | Read block of memory mode |
| 5 | AC1 | Block number |
| 6 | CS | 1 byte message checksum (handled by the system) |

Send Mode 3C message and receive the response

If Block Number is 01

Compare the Response Data bytes 1 through 6 against 0x00 plus VIT2 VIN bytes 1 through 5

Else

If Block Number is 02

Compare the Response Data bytes 1 through 6 against VIT2 VIN bytes 6 through 11

Else

If Block Number is 03

Compare the Response Data bytes 1 through 6 against VIT2 VIN bytes 12 through 17

Endif

Endif

If the Exception Is 01 (AC2=01)

Display the VTD Pin value from response data bytes 1 - 4 (BCD)

Endif

If a Positive response is received and the storage location number (AC3) is valid

Save the data of the response starting with the first Data Byte (after the Block Number in the response) to the 256-byte storage location indicated by AC3

FYI: 	The amount of data (length) stored in the storage location is located in the 7th storage buffer byte (buffer[6]).  The reading/writing of data (for Class 2) can never be more than 6 data bytes.  An example of 3 data bytes (0x12, 0x23, 0x34) stored in an internal storage location would be:  StorageLocation:  0x12, 0x23, 0x34, 0x00, 0x00, 0x00, 0x03, ….

Endif

(continued)

If a VIN block comparison was successful or a response was received from a non-VIN block (01, 02, 03)

Set the “response code” to Response Byte 4

Else

If no response was received

Set the “response code” to “no comm.” (‘FD’)

Else

Set the “response code” to “05”  (Comparison Failed)

Endif

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 3C | 10 01 00 04 | 7B 22 7F 24 FD 30 FF 99 00 00 |

## 3E Op-Code: Modify Block of Memory

Supported By:

Class 2 Interpreter 1

Description:

The 3E Op-Code is used to read a block of data from a controller using mode 3C, change the value of a bit in an identified byte and write the entire block back to the controller using mode 3B. The target, block number to read and write, the byte number to modify and the new bit value are determined by the values in the action fields.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Block Number being read and written |
| AC2: | Data byte number in the response block being (1, 2, 3,…) |
| AC3: | Bit number in the identified byte being modified Defined as: High Nibble = value to set the bit(s) (1 or 0) Low Nibble = bit number to set (0 through 7) Example: AC3 = $17 set bit 7 to value of 1 AC3 = $07 set bit 7 to value of 0 |

Pseudo Code:

XX  3E  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Build Read Data Block message

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and Type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global Source ID |
| 4 | 3C | Read Data Block |
| 5 | AC1 | Block Number to Read |
| 6 | CS | 1 byte message checksum (handled by the system) |

Send Mode 3C message

If there is a successful read response

Determine the number of data bytes received from requested block

Load a temporary buffer with the response data bytes

Extract the new bit value from AC3

Extract the bit to modify from AC3

Reset the byte specified in AC2 with the new bit value

Build Write Data Block message

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and Type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global Source ID |
| 4 | 3B | Write Data Block |
| 5 | AC1 | Block Number to Write |
| 6 | XX | Data byte 1 |
| . . |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N – 1 | XX | Data byte N (maximum of 6) |
| 6 + N | CS | 1 byte message checksum (handled by the system) |

Send Mode 3B message

If there is a response to the “write” request (positive or negative)								(continued)

Set the “response code” to Response Byte 4

Else

Set the “response code” to failed (‘FF’)

Endif

Else  (failed read request)

Set the “response code” to failed (‘FF’)

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 04 | 3E | 10 08 02 15 | 7B 05 7F 99 FF 99 00 00 00 00 |

## 40 Op-Code: Read/Write Data Buffer

Supported By:

Class 2 Interpreter 1

Description:

The 40 Op-Code is used to:

- Read data from a controller using mode 3C or 2A,

- Save a specified number of bytes from the read

- Send a write data block (3B) or request device control (AE) message containing the saved bytes to the same or another controller.

The target device, block number/Dpid to read/write (and data rate for mode 2A) are contained in the utility file routine section. The routine number, number of bytes to read, offset into the read response and the offset into the outgoing message are determined by the action fields. The routine containing the read message data must be directly followed by the routine containing the outgoing message data.

Action Fields:

| AC0: | Routine containing read message data – outgoing msg data must be located in routine ACO+1 Read Routine (AC0) Example using 2A message: Target Read Block/Dpid Data Number of Pad Address Length Device Mode Number Rate Bytes to Pad Value 00000000 0006 10 2A 02 01 05 FF Real Data Format 000000000006102A020105FF Explanation: Target device is 10 and use mode 2A to read Dpid number 02 Read Routine (AC0) Example using 3C message: Target Read Block/Dpid Data Number of Pad Address Length Device Mode Number Rate Bytes to Pad Value 00000000 0006 10 3C 02 00 00 00 Real Data Format 000000000006103C02010000 Explanation: Target device is 10 and use mode 3C to read block number 02 NOTE: Mode 3C does not use data rate Write Routine (AC0+1) Example using 3B message: Target Write Block/Dpid Number of Pad Address Length Device Mode Number Bytes to Pad Value 00000000 0003 40 3B 8E 05 00 Real Data Format 000000000003103B8E0500 Explanation: Target device is 40 and use mode 3B to write data to block 8E |
| --- | --- |
| AC1: | Number of bytes to save |
| AC2: | Offset into the read message (3C or 2A) – first saved byte = AC2. (AC2 = 1…n) |
| AC3: | Offset into the outgoing message (3B or AE) – saved data put into message starting at byte AC3. (AC3 = 1…n) |

Pseudo Code:

XX  40  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Read Routine Data based on Routine # (AC0)

(Target, Read Mode, Block/DPID, # to Pad, Pad Value)

Load the data storage buffer with value indicated in byte 6 of read routine (Pad Value) – initialize	(continued)

Build the read message based on “read” Routine Data (AC0), Byte 2 (Read Mode)

If the Read Mode is $3C

Build Read Data Block message  ($3C)

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and Type |
| 2 | Routine data byte 1 | Target Device ID |
| 3 | ## | Global Source ID |
| 4 | 3C | Read Data Block |
| 5 | Routine data byte 3 | Block Number to Read |
| 6 | CS | 1 byte message checksum (handled by the system) |

Send Mode 3C message

Else

Build Read Diagnostic Data Packet message ($2A)

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and Type |
| 2 | Routine data byte 1 | Target Device ID |
| 3 | ## | Global Source ID |
| 4 | 2A | Read Data Block |
| 5 | Routine data byte 4 | Data Rate |
| 6 | Routine data byte 3 | DPID to Read |
| 7 … 7+n-1 | Routine data byte 6 | DPID Padded values (0-5 = Number of Bytes to Pad (n) ) (Routine data byte 5) |
| 7+n | CS | 1 byte message checksum (handled by the system) |

Send Mode 2A message

Endif

If there is a successful (positive) read response

Read Routine Data based on Routine # (AC0+1)

(Target, Write Mode, Block/DPID, Rate, # to Pad, Pad Value)

Load the data storage buffer with value indicated in byte 5 of write routine (Pad Value) – initialization

Load the Read data values (from $3C / $2A) into the data storage buffer for writing based on the

Read and Write Offset values (AC2, AC3)

Build the write message based on “write” Routine Data (AC0+1), Byte 2 (Write Mode)

If the Write Mode is $3B

Build mode 3B write message using buffer of saved bytes and padding bytes

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and Type |
| 2 | Routine data byte 1 | Target Device ID |
| 3 | ## | Global Source ID |
| 4 | 3B | Write Data Block |
| 5 | Routine data byte 3 | Block Number to Write |
| 6 … 6+n-1 | <data> | Data = “read data” and any “padded data” n = Bytes to write indicated by AC1 + Byte 4 of write routine |
| 6+n | CS | 1 byte message checksum (handled by the system) |

Send Mode 3B “write” message

If there is a response to the “write” request (positive or negative)

Set the “response code” to Response Byte 4

Else

Set the “response code” to failed (‘FF’)

Endif

Else

Build mode AE request device control message using buffer of saved and padding bytes

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and Type |
| 2 | Routine data byte 1 | Target Device ID |
| 3 | ## | Global Source ID |
| 4 | AE | Write Data Block |
| 5 | Routine data byte 3 | CPID Value |
| 6 … 6+n-1 | <data> | Data = “read data” and any “padded data” n = Bytes to write indicated by AC1 + Byte 4 of write routine |
| 6+n | CS | 1 byte message checksum (handled by the system) |

Send Mode AE “Device Control” message

Set the “response code” to Response Byte 6

If there is a positive response to the “device control” request

If the “response code” is E1

Indicate success

Else

Indicate failure

Endif

Endif

Endif

Endif

If any of the Routine Data Reads failed

Set the “response” code to failed (‘FF’)

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

Using $3C and $3B message combination

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 04 | 40 | 01 01 03 02 | 7B 05 7F 06 FF 06 00 00 00 00 |

Using $3C and $AE message combination

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 04 | 40 | 01 01 03 02 | E1 05 E3 06 FF 06 00 00 00 00 |

## 52 Op-Code: Write Checksum to Block 82 / 8E

Supported By:

Class 2 Interpreter 1

Description:

The 52 Op-Code reads a set of data blocks 01, 02, 03, 08, and (82 or 8E) using mode 3C messages to calculate a total checksum. The checksum is then appended to data block (82 or 8E) (in byte position 5) and the data is written back to the controller using a mode 3B message.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | 8E – Substitute block number ($8E) to read / write Default Block number to read / write is $82. 00-8D, 8F-FF – Use default block number ($82). |
| AC2: | 00 – Not Used |
| AC3: | 00 – Not Used |

Pseudo Code:

XX  52  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Build Read Checksum message as indicated in the table below:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and Type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global Source ID |
| 4 | 3C | Read Data Block |
| 5 | - - - | Block number (01, 02, 03, 08, (82 or 8E)) |
| 6 | CS | 1 byte message checksum (handled by the system) |

Set Block Number to 01

Send a mode 3C Read message for VIN bytes 1-5 and receive response

If a valid response was not received, Goto “Response Processing” (end processing)

Calculate checksum on received data bytes (6 data bytes)

Set Block Number to 02

Send a mode 3C message for VIN bytes 6-11 and receive response

If a valid response was not received, Goto “Response Processing” (end processing)

Increase checksum by received data bytes (6 data bytes)

Set Block Number to 03

Send a mode 3C message for VIN bytes 12-17 and receive response

If a valid response was not received, Goto “Response Processing” (end processing)

Increase checksum by received data bytes (6 data bytes)

Set Block Number to 08

Send a mode 3C message for Part Number bytes 1-4 (USN) and receive response

If a valid response was not received, Goto “Response Processing” (end processing)

Increase checksum by received data bytes (4 data bytes)

If AC1 = 8E

Set Block Number to 8E

Else

Set Block Number to 82

Endif

Send a mode 3C message for DVT Option bytes 1-4 and receive response

If a valid response was not received, Goto “Response Processing” (end processing)

Increase checksum by received data bytes

Add one (1) to the checksum value 																(continued)

Build Write Checksum message as indicated in the table below:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and Type |
| 2 | AC0 | Target Device ID |
| 3 | ## | Global Source ID |
| 4 | 3B | Read Data Block |
| 5 | - - - | Block number (82 or 8E) |
| 6 | RespData1 | Response Data byte #1 from the Block Read (82 or 8E) |
| 7 | RespData2 | Response Data byte #2 from the Block Read (82 or 8E) |
| 8 | RespData3 | Response Data byte #3 from the Block Read (82 or 8E) |
| 9 | RespData4 | Response Data byte #4 from the Block Read (82 or 8E) |
| 10 | Calculated CS | Calculated checksum value (blocks reads 01, 02, 03, …) |
| 11 | CS | 1 byte message checksum (handled by the system) |

Set Block Number to 01

Send a mode 3B Write message

Response Processing

If a response was received (positive or negative)

Set the “response code” to response message byte 4

Else

Set the “response code” to no response (‘FD’) (No Comm.)

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | 52 | 10 00 00 00 | 7B 11 7F 42 FD 42 FF 99 00 00 |

## 72 Op-Code: Set Programming Voltage

Supported By:

Class 2 Interpreter 1

Description:

The 72 Op-Code sets the programming voltage to 17V on the DLC connector pin #??.

Action Fields:

| AC0: | ## - DLC Pin number |
| --- | --- |
| AC1: | 00 – Voltage ON (Enable) 01 – Voltage OFF (Clear) |
| AC2: | 00 – Not Used |
| AC3: | 00 – Not Used |

Pseudo Code:

XX  72  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

If AC1 is 00

Apply voltage to the pin based on AC0 (Pin Number)

* When using Tech 2 Saturn/Catera adapter, voltage is always applied to DLC PIN #6

Else

If AC1 is 01

Remove voltage from DLC pin based on AC0 (Pin Number)

* When using Tech 2 Saturn/Catera adapter, voltage is removed from DLC PIN #6

Else

Goto step specified in Goto field 1 (G1)  (Invalid Action type – AC1)

Endif

Endif

If the Enable or Clear executes successfully

Wait for 2 seconds

Goto step specified in Goto field 1 (G1).

Else

Goto step specified in Goto field 3 (G3).

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | 72 | 06 01 00 00 | 00 11 00 5B 00 00 00 00 00 00 |

## 84 Op-Code: Set Communications Parameters (Add Delay)

Supported By:

Class 2 Interpreter 1

Description:

The 84 Op-Code is used to add a time delay between a response from a controller and the subsequent request from the tester tool (based on utility file processing).  This Op-Code is specifically designed for controllers that don’t adhere precisely to the Class 2 protocol.  A good example is during a calibration download, a “non-compliant” controller will positively respond to a block download; however, the controller is truly not ready for the next block download.  So when the next block is quickly sent to the controller, the controller is not able to process it.  Adding a small time delay prior to the next download will allow the controller to get ready for the next block.

The "time delay" will ONLY occur for the Class 2 “Transfer Data – Mode $36” messages

(Op-Codes $B0, $B2, $B3 and $B4).

This Op-Code may be used multiple times to turn on and turn off the “time delay”.

NOTE: GM NAO assembly plants will currently not support this Op-Code for the Class 2 protocol. If this Op-Code is encountered in the assembly plant utility file, the G1 “Go To” will be processed.

Action Fields:

| AC0: | 01 – 0F: Time delay (x100 ms) between response and next request 00, 10-FF: No Time Delay – Reset any previously set “time delay”. |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX    84    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

If AC0 is $00 or in the range $10-$FF

Turn OFF any previously set “delay timer”

If AC0 is in the range $01-$0F

Turn ON the “delay timer” based on a delay time of AC0 * 100ms

(Time delay between an ECU response and any subsequent TESTER request)

Endif

Goto step specified by goto field one (G1)

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 84 | 03 00 00 00 | 00 02 00 00 00 00 00 00 00 00 |

## A0 Op-Code: Request High Speed Communications - Mode A0

Supported By:

Class 2 Interpreter 1

Description:

The A0 Op-Code builds a standard mode A0 message to request high speed communications. This Op-Code will not actually begin high speed communications it will only request the control module to be ready for high speed communications.  To begin high speed communications an A1 Op-Code should be used.

This Op-Code will ignore the error of “no response” being received from the request. If this condition occurs, then Goto Step G1 will be executed.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Look at response byte 5 for response value 01 - Look at response byte 4 for response value |

Pseudo Code:

XX  A0  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Build Request High Speed message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID. |
| 4 | A0 | Request download message mode |
| 5 | CS | 1 byte message checksum (handled by the system) |

Send Tester Present message to all devices (0xFE)

Send Mode A0 message and receive response

Goto Class 2 Response processing

If No response was received

Goto step G1

Else

If AC3 = 00

Set the “response code” to response message byte 5

If the “response code” (response message byte 5) is equal to 0xBB

Set the “response code” to 0xAA

Endif

Else

Set the “response code” to response message byte 4

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 02 | A0 | 10 00 00 00 | AA 03 FF 99 00 00 00 00 00 00 |

## A1 Op-Code: Begin High Speed Communications - Mode A1

Supported By:

Class 2 Interpreter 1

Description:

The A1 Op-Code builds a standard mode A1 message to begin high-speed communications. After sending the request for high-speed communications the tool will delay approximately one second to ensure high speed communications has started.

This Op-Code will not check for a response being received from the request (none expected).

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  A1  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Build Start High Speed message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target address |
| 3 | ## | Global source address |
| 4 | A1 | Request download message mode |
| 5 | CS | 1 byte message checksum (handled by the system) |

Send Tester Present message to all devices (0xFE)

Send Mode A1 message and do not attempt to process a response (none expected).

Delay for (0.6) seconds

Send Tester Present message to all devices (0xFE)

If the message (Begin High Speed Communications) was successfully transmitted

Goto step specified in goto field 1 (G1)

Else

Set the “response code” to no response (‘FD’) (No Comm.) (i.e. transmit failed)

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 1F | A1 | 10 00 00 00 | 00 20 FD 2C FF 99 00 00 00 00 |

## A8 Op-Code: GM Propriety Test Mode

Supported By:

Class 2 Interpreter 1

Description:

The A8 Op-Code builds a standard mode A8 message and is a GM Propriety Test Mode not to be disclosed outside the Company.

NOTE:  This Op-Code is NOT functional within the software application.  The response processing will be based on GOTO field G1.

## AE Op-Code: Request Device Control - Mode AE

Supported By:

Class 2 Interpreter 1

Description:

The AE Op-Code will build a standard mode AE message to send to a device. A mode AE command will allow a test device to control functions that the device has normal software control over.

*NOTE: See General Motors Class 2 Physical Message Diagnostic Strategy Specification document.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | CPID number |
| AC2: | Routine Number to build into message |
| AC3: | Not Used |

Pseudo Code:

XX  AE  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Find the routine indicated by value AC2 and read the data from the routine

Build Download Request as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6C | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | AE | Request download message mode |
| 5 | AC1 | CPID Number |
| 6 | XX | 1st byte of routine (AC2) |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 6 + N | XX | Data byte N (maximum of 6) |
| 6 + N + 1 | CS | 1 byte message checksum (handled by the system) |

Copy “length” bytes of routine AC2 into message (should not exceed length of 6 and some controllers may require 6 - zero fill if necessary within the routine)

Send Mode AE message and receive response

Set the “response code” to the sixth byte of the response data

If a positive response was received (EE) and the sixth byte is E1

Mode AE “PASSED”

Else

Mode AE “FAILED”

Endif

Send Tester Present message to all devices (0xFE)

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | AE | 10 03 02 00 | E1 11 E4 0D FF 99 00 00 00 00 |

## B0 Op-Code: Block Transfer to RAM and Execute - Mode 36

Supported BY:

Class 2 Interpreter 1

Description:

The B0 Op-Code is used to build a mode 36 message for downloading a routine to control module RAM.  Depending on the product being programmed this Op-Code may need to be preceded by a mode 34 message (see Op-Code 34 ). Using the exceptions described, the utility controls when the code downloaded to RAM is executed.

The B0 Op-Code uses the global source device ID for all messages. If a source device ID other than the current source device ID needs to be used, the Interpreter instructions should contain a 01 Op-Code prior to this Op-Code.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Routine number to download |
| AC2: | Not used |
| AC3: | Exceptions 00-03. The RAM download address is determined from the routine section. |
|  | 00 - Download routine and receive completion response but DO NOT execute |
|  | 01 - Download routine and receive completion response then send separate execute message - The “execute” message will contain NO data bytes and also NO address information. |
|  | 02 - Send execute message with zero length and with the address of the indicated routine |
|  | 03 - Send routine with transmit type of execute |
|  | Exceptions 80-83. The RAM download address is determined from the routine section. |
|  | 80 - Download routine and receive completion response but DO NOT execute |
|  | 81 - Download routine and receive completion response then send separate execute message - The “execute” message will contain NO data bytes and will have the address information. |
|  | 82 - Send execute message with zero length and with the address of the indicated routine |
|  | 83 - Send routine with transmit type of execute |

*Note: A prolog and routine are both contained within the routine section of utility file; however, they are different. The prolog is a value that precedes each packet of information. With a routine, the data is downloaded to the controller only once. A prolog would remain constant during the downloading of a routine. (The Class 2 protocol uses routines.  Use of prologs in Class 2 is not currently supported as it is not required.)

Pseudo Code:

XX  B0  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Find the routine indicated by value AC1

Build Download message as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6D | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 36 | Request download message mode |
| 5 | XX | Transfer type |
| 6 | XX | High byte of number of data bytes |
| 7 | XX | Low byte of number of data bytes |
| 8 | XX | High byte of starting address |
| 9 | XX | Middle byte of starting address |
| 10 | XX | Low byte of starting address |
| 11 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 11 + N |  | Data byte N |

(continued)

| 12 + N |  | High byte of checksum |
| --- | --- | --- |
| 13 + N |  | Low byte of checksum |

If this Message is Strictly Download Without Execution on Completion (AC3 = 0x00, 0x01, 0x80, 0x81)

Set transfer type to 00 for download only (Byte 5 of Class 2 message)

Else

Set transfer type to 80 for download and execute (Byte 5 of Class 2 message)

Endif

Set the address bytes (for the mode $36 message) based on the routine address information.

If the address information is 0xFFFFFFFF

No address information will be included in the mode $36 message (Bytes 8, 9, 10 will be removed from the message)

Endif

If this Message is Strictly Execute with No Data Being Sent (AC3 = 0x02, 0x82)

Set the length bytes (for the mode $36 message) to zero

Set the data bytes (for the mode $36 message) to NULL (no data)

Else

Set the length bytes (for the mode $36 message) based on the routine length information

Set the data bytes (for the mode $36 message) based on the routine data

Endif

Calculate the checksum for the message buffer

Copy the two byte checksum to the end of the message buffer

Send Tester Present message to all devices (0xFE)

Send the mode $36 message and expect a response

If the Response to the Above Message is a Success (“0x76 .. 0x65” or “0x76 .. 0x73” or “0x76 .. 0x86”)

If this is a Download Then Send a Separate Message (AC3 = 0x01, 0x81)

Build Download message as indicated in the table below, contents of the XX locations is detailed in	 pseudo code following the table  (Length of 0x00, NO data bytes, transfer type is execute (0x80))

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6D | Format and type |
| 2 | AC0 | Target device ID |
| 3 | XX | Global source ID |
| 4 | 36 | Request download message mode |
| 5 | 80 | Transfer type |
| 6 | 00 | High byte of number of data bytes |
| 7 | 00 | Low byte of number of data bytes |
| 8 | XX | High byte of starting address |
| 9 | XX | Middle byte of starting address |
| 10 | XX | Low byte of starting address |
| 11 | CS | High byte of checksum |
| 12 | CS | Low byte of checksum |

If the Exception is 0x81 (AC3 = 01)

Set the address bytes (for the mode $36 “execute” message) based on the routine address

Else

No address information will be included in the mode $36 message (Bytes 8, 9, 10 will be removed from the message)

Endif

Calculate checksum for message buffer

Copy high byte of checksum into the end of the message buffer

Copy low byte of checksum into the end of the message buffer

Send the execute only message and expect a response

Endif

Endif

If a “positive response” was received (0x76)														(continued)

Set the “response code” to the sixth byte of the response

Else

If a “negative” response (0x7F) was received

Set the “response code” to the fourth byte of the response

Else

If “no response” was received

Set the “response code” to 0xFD (No Comm. Response)

Else

Set the “response code” to the sixth byte of the response

Endif

Endif

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | B0 | 10 01 00 81 | 65 11 62 0D FD 20 7F 49 FF 49 |

## B2 Op-Code: Block Transfer to RAM and Save Two Byte Response - Mode 36

Supported By:

Class 2 Interpreter 1

Description:

The B2 Op-Code is used to build a mode 36 message for downloading a routine to control module RAM.  Depending on the product being programmed this Op-Code may need to be preceded by a mode 34 message (see Op-Code 34). Using the exceptions the utility controls when the code downloaded to RAM is executed. The B2 Op-Code is functionally equivalent to the B0 Op-Code except two bytes are saved from the response (checksum).

The B2 Op-Code uses the global source device ID for all messages. If a source device ID other than the current source device ID needs to be used, the Interpreter instructions should contain a 01 Op-Code prior to this Op-Code.

Note:  This OpCode uses the 2-byte storage buffers.  For buffer usage information, see the section: “Programming Buffers – Recommended Usage”

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Routine number to download |
| AC2: | ID for storage location of two response bytes (0x00 – 0x13) |
| AC3: | Exceptions 00-03. The RAM download address is determined from the routine section. |
|  | 00 - Download routine and receive completion response but DO NOT execute |
|  | 01 - Download routine and receive completion response then send separate execute message - The “execute” message will contain NO data bytes and also NO address information. |
|  | 02 - Send execute message with zero length and with the address of the indicated routine |
|  | 03 - Send routine with transmit type of execute |
|  | Exceptions 80-83. The RAM download address is determined from the routine section. |
|  | 80 - Download routine and receive completion response but DO NOT execute |
|  | 81 - Download routine and receive completion response then send separate execute message - The “execute” message will contain NO data bytes and will have the address information. |
|  | 82 - Send execute message with zero length and with the address of the indicated routine |
|  | 83 - Send routine with transmit type of execute |

Pseudo Code:

XX  B2  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Process the Op-Code $B0 pseudo code logic.

If the response to the $36 message(s) is a Success (“0x76 .. 0x65” or “0x76 .. 0x73” or “0x76 .. 0x86”)

Save two bytes from the response to internal local storage ID indicated by AC2

(bytes 7 & 8 of response are saved)

Endif

Look for “response code” in the goto fields (see Op-Code $B0 pseudo code)

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | B2 | 10 04 05 01 | 65 11 62 0D FD 20 7F 49 FF 49 |

## B3 Op-Code: Block Transfer to Download VIN as a Calibration - Mode 36

Supported By:

Class 2 Interpreter 1

Description:

The B3 Op-Code is used to build a mode 36 message for downloading the VIN to a control module. The VIN is packaged in a block transfer message as if it were another calibration file. Depending on the product being programmed this Op-Code may need to be preceded by a mode 34 message (see Op-Code 34).

The B3 Op-Code uses the global source address for all messages. If a source address other than the current source address needs to be used the Interpreter instructions should contain a 01 Op-Code prior to this Op-Code.

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Block ID to include in block transfer for VIN |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used (exceptions) |

Pseudo Code:

XX  B3  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Build Download message as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6D | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source ID |
| 4 | 36 | Request download message mode |
| 5 | 00 | Transfer type |
| 6 | 00 | High byte of number of data bytes |
| 7 | 20 | Low byte of number of data bytes |
| 8 | XX | High byte of starting address |
| 9 | XX | Middle byte of starting address |
| 10 | XX | Low byte of starting address |
| 11 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 11 + N | XX | Data byte N |
| 12 + N | CS | High byte of checksum |
| 13 + N | CS | Low byte of checksum |

The length (number of data bytes for the mode $36 message) is set to 32 decimal (0x0020).

Set the address bytes (for the mode $36 message) based on the utility file address information.

Calculate ones complement checksum for VIN bytes and calibration ID byte

Copy the high byte of ones complement checksum into data byte 1

Copy the low byte of ones complement checksum into data byte 2

Copy zero into data byte 3 as the high byte of the VIN calibration ID

Copy AC1 (Block ID) into data byte 4 as the low byte of the VIN calibration ID

Copy the VIN data into message buffer starting at data byte 5

Calculate the checksum for the entire (Mode $36) message buffer								(continued)

Copy the two byte checksum to the end of the message buffer

Send Tester Present message to all devices (0xFE)

Send the mode $36 message and expect a response

If the response to the $36 message(s) is a Success (“0x76 .. 0x65” or “0x76 .. 0x73” or “0x76 .. 0x86”)

Set the “response code” to the sixth byte of the response

Else

If a “negative” response (0x7F) was received

Set the “response code” to the fourth byte of the response

Else

If “no response” was received

Set the “response code” to 0xFD (No Comm. Response)

Else

Set the “response code” to the sixth byte of the response

Endif

Endif

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | B3 | 10 02 00 00 | 65 09 FD 12 7F 4A FF 4A 00 00 |

## B4 Op-Code: Block Transfer to Download Calibration File - Mode 36

Supported By:

Class 2 Interpreter 1

Description:

The B4 Op-Code is used to build a mode 36 message for downloading a calibration file (or software) to a control module. Depending on the product being programmed this Op-Code may need to be preceded by a mode 34 message (see Op-Code 34 ).

The exception value (AC3) will allow the B4 Op-Code to support slightly different processing without having to create unique Op-Codes. Currently exception 0 should be used to download files to a 95 VCM-XA and exception 1 to download a car platform control module. (See pseudo code for exception descriptions.)

The B4 Op-Code uses the global source address for all messages. If a source address other than the current source address needs to be used the Interpreter instructions should contain a 01 Op-Code prior to this Op-Code.

If the GlobalHeaderLength value (OpCode $F3) is not zero then the downloading of the module will be separated into two parts with the “GlobalHeaderLength” number of bytes will be downloaded first, followed by the remaining calibration data.  The “default” GlobalHeaderLength value is zero.

Caution:  The “message data length” value in the Utility File header can be “over-written” via the “$F5 Override the Utility File Message Length Field” Op-Code.  (See the $F5 Op-Code for specifics.)

Action Fields:

| AC0: | Target Device ID |
| --- | --- |
| AC1: | Calibration ID for Calibration to Download |
| AC2: | Value of the routine number (taken from routine section of utility file) |
| AC3: | Exceptions: 00 - This option downloads each packet with a transmit type of download only. The calibration download data packet size is obtained from the utility file header. The RAM download address is obtained from the utility file header. This option uses the same download address for each data packet downloaded. Uses 3-byte addressing. If the last piece of data to download is less than the packet size, then the missing data is padded with 0x00 data. For examples of how this exception works see reprogramming specifications for the 1995 VCM-XA or the 1996 PCM-32U. 01 - This option downloads each packet with a transmit type of download & execute. The Mode $36 message contains “routine data” from the routine (AC2) that precedes each calibration packet data from the calibration file (AC1). The calibration download data packet size is obtained from the utility file header. The RAM download address is determined from the routine section (prolog). The download packet address is obtained from the utility file header and is incremented by the packet size after each download. Uses 2-byte download packet addressing. If the last piece of data to download is less than the packet size, then the packet length will adjusted to the actual number of remaining bytes to send (no padding with 0x00 data). For examples of how this exception works see reprogramming specifications for the 1996 PCM-66U. 02 - Follows the same logic as exception 01 with the exception of: Uses 3-byte download packet addressing. 03 - Follows the same logic as exception 00. 04 - Follows the same logic as exception 00 or 03 with the exception of: The download address will be incremented by the packet size after each download. If the last piece of data to download is less than the packet size, then the packet length will adjusted to the actual number of remaining bytes to send (no padding with 0x00 data). |

*Note: A prolog and routine are both contained within the routine section of utility file; however, they are different. The prolog is a value that precedes each packet of information, which could be a routine that is downloaded. With a routine, the data is downloaded to the controller only once. A prolog would remain constant during the downloading of a routine.

Pseudo Code:																						(continued)

XX  B4  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Find the calibration with ID of AC1

Set the packet length to “number of data bytes in a message” value in the Utility File header

Note:  	** The “message data length” value in the Utility File header can be “over-written” via the “$F5

** Override the Utility File Message Length Field” Op-Code.

Build Download message as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 6D | Format and type |
| 2 | AC0 | Target device ID |
| 3 | ## | Global source device ID |
| 4 | 36 | Request download message mode |
| 5 | XX | Transfer type |
| 6 | XX | High byte of number of data bytes |
| 7 | XX | Low byte of number of data bytes |
| 8 | XX | High byte of starting address |
| 9 | XX | Middle byte of starting address |
| 10 | XX | Low byte of starting address |
| 11 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 11 + N | XX | Data byte N |
| 12 + N | CS | High byte of checksum |
| 13 + N | CS | Low byte of checksum |

If the Exception is Zero, Three or Four (AC3=00, AC3=03 or AC3=04)

Set the transfer type to 0x00 (Download Only)

Set the “download message number of data bytes (2 bytes)” based on the packet length

Set the “download message address bytes (3 byte addressing)” based on the utility file address

Note: The “data bytes” based on the calibration data (dynamic)

Else 	(AC3=01 or AC3=02)

Set the transfer type to 0x80 (Download and Execute)

Find the routine section indicated by value in AC2

Set the “download message number of data bytes (2 bytes)” based on the packet length + the routine length + length size* + address size**

* 	The length size for the calibration packet data (2 bytes)

** 	The address size for the calibration packet data is (2 bytes if AC3=02 or 3 bytes if AC3=01)

Set the “download message address bytes (2 or 3 bytes***)” based on the routine address

** 	The number of address bytes is 2 if AC3=02.  The number of address bytes is 3 if AC3=01.

Note: The “data bytes” are based on the routine data (static) and the calibration data (dynamic)

Endif

Do while more calibration data to download

If the GlobalHeaderLength is not 0 (download the calibration file into 2 parts – header first)

Set the Packet & Data length values based on the GlobalHeaderLength

Decrement the GlobalHeaderLength by the number of header bytes to download

Endif

If this is the last packet of a file to download 													(continued)

If (AC3=01, AC3=02 or AC3=04)

Change the packet size and download message length values based on the amount of data

remaining

Copy the remaining data into the message buffer

Else

Copy the remaining data into the message buffer and PAD with 0x00

Endif

Else

Copy the next block of data into the message buffer

Endif

Calculate the checksum for the entire (Mode $36) message buffer

Copy the two byte checksum to the end of the message buffer

Send Tester Present message to all devices (0xFE)

Send the mode $36 message and expect a response

If an error occurred

End the calibration download

Endif

If the Exception is One (AC3=01)

Calculate next calibration download address and copy into address position of message (2 bytes)

Else If the Exception is Two (AC3=02)

Calculate next calibration download address and copy into address position of message (3 bytes)

Else If the Exception is Four (AC3=04)

Calculate the next Mode $36 message address (increment the address based on the packet size)

Endif

If an error occurred

End the calibration download

Endif

END – Do-While processing

If the response to the last $36 message(s) is a Success (“0x76 .. 0x65” or “0x76 .. 0x73” or “0x76 .. 0x86”)

Set the “response code” to the sixth byte of the response

Else

If a “negative” response (0x7F) was received

Set the “response code” to the fourth byte of the response

Else

If “no response” was received

Set the “response code” to 0xFD (No Comm. Response)

Else

Set the “response code” to the sixth byte of the response

Endif

Endif

Endif

Look for “response code” in the goto fields

If the “response code” is found

Goto appropriate step

Else

Goto step indicated by ‘FF’ goto field

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 19 | B4 | 10 04 00 00 | 86 26 80 0D FD 20 7F 28 FF 28 |

# Interpreter 2 KWP2000 Communications Programming Op-Codes

## KWP2000 No Communications Processing

Supported By:

KWP2000 Interpreter 2

Description:

Normal processing of no communications conditions is to re-send the request up to three times then fail with a no comm. fault if a response isn't received.

When writing instructions for a Utility File the FD code can be used to determine the next step to execute after a no comm. fault occurs. The 'FD' processing can be used for any step in the utility file where a no comm. condition can occur.

Pseudo Code:

XX     OP  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

If no comm. occurred

Check goto fields for an FD return code (G0, G2, G4, G6, or G8)

If found

Flag appropriate step to execute next and return

Else

Check goto fields for an FF return code (G0, G2, G4, G6, or G8)

If found

Flag appropriate step to execute next and return

Else

End programming with fatal error

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 28 | OP | B4 03 10 00 | 65 2A FD 12 FF 4A 00 00 00 00 |

## KWP2000 Response Processing

Supported By:

KWP2000 Interpreter 2

Description:

If a ECU doesn't respond (timeout) then No Communication Processing applies.

If a negative response is received, then the tool searches for the response code (byte 3) of the received negative response in the goto fields and if found flags the appropriate step to execute next.

In all other cases, the tool searches for the positive response service ID in the goto fields.

When an Op-Code refers to the response buffer the bytes are identified as follows:

Positive Response:

| Response Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | XX | Positive Response Service ID |
| n | XX | Byte n (where n <= 255) |

Negative Response:

| Response Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 7F | Negative Response Service ID |
| 2 | XX | Request Service ID |
| 3 | XX | Response Code |

Pseudo Code:

XX     OP  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Get response for the request

If a timeout occurred

Goto No Communication Processing

Endif

If a negative response service ID is returned (negative response service ID $7F)

If the negative response code = expected positive response service ID plus $40)

Set 7f-special process flag **(see details below)

Endif

Return negative response

Else

Return positive response

Endif

** Special Case 7F processing:

If the service request has a negative response code that is the same as  an expected positive response message response service ID plus $40:

The 7f-special-process flag will be set. The Interpreter shall look for the second occurrence of the response code in the goto fields to determine the next Interpreter step.

As indicated in the example below the second occurrence of the response code can occur anywhere in the remaining goto fields (note: For clarity, it is recommended that the “duplicate response code” goto

fields be on the first line and not separated as on a continuation line using the F8 continuation Op-Code).

(continued)

If the response code is not found in the goto fields then the $FF goto will be used to determine the next Interpreter step.

If the service request has a negative response code that is NOT the same as an expected positive response message response service ID plus $40:

The Interpreter shall look for the FIRST occurrence of the response code in the goto fields to determine the next Interpreter step.  If the response code is not found in the goto fields then the $FF goto will be used to determine the next Interpreter step.

Developers Note:

This processing puts the requirement for the Interpreter into the hands of the developers who know what response codes are possible for a particular service request. When a developer is writing the Interpreter instructions the need for duplicate goto fields will be obvious and should not present any problems.

Negative response processing Utility File Example and Pseudo Code

Following is the message structure and utility line for pseudo code example below.

|  |  |  |  | PositiveResponse ServiceID |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | $77 |  |  |
|  | RequestMessage ServiceID |  |  |  |  |  |
|  | $37 |  |  |  |  |  |
|  |  |  |  | NegativeResponse ResponseID | RequestID | ResponseCode |
|  |  |  |  | $7F | $37 | $77 |

| Step | Op-Code | AC0 | AC1 | AC2 | AC3 | G0-G1 | G2-G3 | G4-G5 | G6-G7 | G8-G9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 37 | 00 | 00 | 00 | 00 | 77-1B | 41-17 | 72-17 | 77-18 | FF-15 |

If a PositiveResponse is received (ResponseServiceID is RequestMessageServiceID + $40).

Use the first occurrence of the PositiveResponseServiceID ($77) to determine next step.

(For this example the next step is Step 1B. )

Else a NegativeResponse was received (ResponseServiceID is $7F).

If the NegativeResponseResponseCode equals the RequestMessageServiceID + $40.

Use the second occurrence of ResponseCode ($77), or the $FF Goto field if only

one occurrence of ResponseCode ($77) is found, to determine next step.

(For this example the next step is Step 18.)

Else.

Use the ResponseCode to determine the next step.

(For Response Code $41 in this example, the next step is Step 17.)

Endif.

Endif.

## 01 Op-Code: Setup KWP2000 Programming (Target and Source Bytes)

Supported By:

KWP2000 Interpreter 2

Description:

The 01 Op-Code is used to set the global variables that are used during the programming process.  The Op-Code allows the user to alter the global settings any time during the programming event by issuing another 01 Op-Code.  AC0 is used to set a global application variable that is used in all message for the Target Byte.  The default Target Byte is $11.   AC1 is used to set a global application variable that is used in all message for the Source Byte.  The default Source Byte is $F1.

Action Fields:

| AC0: | Global Target Byte |
| --- | --- |
| AC1: | Global Source Byte |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not used |

Pseudo Code:

XX    01    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

If AC0 is not equal to zero

Set global target byte to value in AC0

Endif

If AC1 is not equal to zero

Set global source byte to value in AC1

Endif

Goto step specified by goto field one (G1)

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 01 | 12 F0 00 00 | 00 02 00 00 00 00 00 00 00 00 |

## 03 Op-Code: Set Number of Repetitions

Supported By:

KWP2000 Interpreter 2

Description:

The 03 Op-Code is used to set the number of message repetitions in case of no response. If e.g. a value of 2 is used there are 2 repetitions making a sum of 3 request going out before a No Communication situation is reported. If this Op-Code is not used the number of repetitions defaults to 2.

Action Fields:

| AC0: | Number of repetitions |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | $00 - Not used |

Pseudo Code:

XX    01    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Goto step specified by goto field one (G1)

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 03 | 01 00 00 00 | 00 07 00 00 00 00 00 00 00 00 |

## 10 Op-Code: SR 10 to Start Diagnostic Session

Supported By:

KWP2000 Interpreter 2

Description:

The 10 Op-Code is used to start a programming diagnostic session.

Action Fields:

| AC0: | Diagnostic mode |
| --- | --- |
| AC1: | Baud Rate Identifier $00 - Not used $01 – 9600 Baud $02 – 19200 Baud $03 – 38400 Baud $04 – 57600 Baud $05 – 115200 Baud |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not used |

Pseudo Code:

XX    10    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Start Diagnostic Session Request as indicated in the table below

If (AC1 == 0)

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 02 | Length of request from byte 5 up to checksum (not included) |
| 5 | 10 | Start routine by local ID request service ID |
| 6 | AC0 | Diagnostic mode |
| 7 | CS | 1 byte message checksum |

If (AC1 >= $01) & (AC1 <=$05)

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 02 | Length of request from byte 5 up to checksum (not included) |
| 5 | 10 | Start routine by local ID request service ID |
| 6 | AC0 | Diagnostic mode |
| 7 | AC1 | Baud rate Identifier |
| 8 | CS | 1 byte message checksum |

Calculate checksum and copy into service request (byte 7)

Send Start Diagnostic Session Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

(continued)

If Start Diagnostic Session positive response is received

Switch to baud rate specified by AC1

Look for Start Diagnostic Session positive response service ID ($50) in goto fields

Else

Look for Start Diagnostic Session negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 10 | 00 00 00 00 | 50 11 FF 2D 00 00 00 00 00 00 |

## 11 Op-Code: SR 11 ECU Reset

Supported By:

KWP2000 Interpreter 2

Description:

The 11 Op-Code is used to issue an ECU Reset Service Request.

Action Fields:

| AC0: | Reset mode |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not used |

Pseudo Code:

XX    11    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build ECU Reset Service Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 02 | Length of request from byte 5 up to checksum (not included) |
| 5 | 11 | ECU Reset service ID |
| 6 | AC0 | Reset mode |
| 7 | CS | 1 byte message checksum |

Calculate checksum and copy into service request (byte 7)

Send ECU Reset Service Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If ECU Reset Service positive response is received

Look for ECU Reset Service positive response service ID ($50) in goto fields

Else

Look for ECU Reset Service negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 11 | 01 00 00 00 | 51 11 FF 2D 00 00 00 00 00 00 |

## 14 Op-Code: SR 14 Clear Diagnostic Trouble Codes

Supported By:

KWP2000 Interpreter 2

Description:

The 14 Op-Code is used to issue a Clear DTCs Service Request. The group of diagnostic information selects which DTCs shall be cleared. The below values are defined in the document "SAE Recommended Practice J2012" section "Diagnostic Trouble Codes"

| Group of diagnostic information | Description |
| --- | --- |
| $0000 | All powertrain DTCs |
| $0001 - $3999 | Specific powertrain DTCs |
| $4000 | All chassis DTCs |
| $4001 - $7999 | Specific chassis DTCs |
| $8000 | All body DTCs |
| $8001 - $B999 | Specific Body DTCs |
| $C000 | All undefined DTCs |
| $C001 - $C999 | Specific undefined DTCs |
| $FF00 | All DTCs |

Action Fields:

| AC0: | Group of diagnostic information (high byte) |
| --- | --- |
| AC1: | Group of diagnostic information (low byte) |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 – Not used |

Pseudo Code:

XX    14    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Clear DTCs Service Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 03 | Length of request from byte 5 up to checksum (not included) |
| 5 | 14 | Clear DTCs service ID |
| 6 | AC0 | Group of diagnostic information (high byte) |
| 7 | AC1 | Group of diagnostic information (low byte) |
| 7 | CS | 1 byte message checksum |

Calculate checksum and copy into service request (byte 7)

Send Clear DTCs Service Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If ECU Reset Service positive response is received

Look for ECU Reset Service positive response service ID ($50) in goto fields

Else

Look for ECU Reset Service negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 2E | 14 | FF 00 00 00 | 54 2F FF 33 00 00 00 00 00 00 |

## 20 Op-Code: SR 20 to Stop Diagnostic Session

Supported By:

KWP2000 Interpreter 2

Description:

The 20 Op-Code is used to stop a programming diagnostic session.

Action Fields:

| AC0: | $00 - Not used |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not used |

Pseudo Code:

XX    20    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Stop Diagnostic Session Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 01 | Length of request from byte 5 up to checksum (not included) |
| 5 | 20 | Start routine by local ID request service ID |
| 6 | CS | 1 byte message checksum |

Calculate checksum and copy into service request (byte 6)

Send Stop Diagnostic Session Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Stop Diagnostic Session positive response is received

Switch to default baud rate (10400Bd)

Look for Stop Diagnostic Session positive response service ID ($60) in goto fields

Else

Look for Stop Diagnostic Session negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 20 | 00 00 00 00 | 60 11 FF 3D 00 00 00 00 00 00 |

## 23 Op-Code: SR 23 to Read Memory by Address and Save Bytes

Supported By:

KWP2000 Interpreter 2

Description:

The Op-Code 23 will build a standard service request 23 to send to an ECU. This Op-Code is used to read a specified number of data bytes from a specified memory address and save them. The address and size information for the service request either comes from a routine in the Utility File or from the global address and length information set by Op-Code F1 and F2, response. If the value in Action field 0 (AC0) is $00 the global address and length information is used, otherwise the routine information is used. The value in Action Field 2 (AC2) is the ID of the storage location for a 2-byte buffer or a 256-byte buffer. The valid IDs for both types are 0x00 thru 0x13. The value in Action Field 1 (AC1) specifies whether a 2-byte or a 256-byte buffer shall be used. For details on how to use these features see the Action Fields and the Pseudo Code.

Note:   This OpCode uses either the 2-byte or the 256-byte storage buffers.  For buffer usage information, see the section: “Programming Buffers – Recommended Usage”

Action Fields:

| AC0: | $00 – Get memory address from global address information set by Op-Code $F1 and memory size from global length information set by Op-Code $F2 $XX – Routine number from Utility File to get memory address and memory size from |
| --- | --- |
| AC1: | $00 – Use 2-byte buffer $01 – Use 256-byte buffer |
| AC2: | ID of storage location (either 2-byte or 256-byte buffer) (0x00 – 0x13) |
| AC3: | Exceptions: $00 – Not used |

Pseudo Code:

XX    23    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Start Routine by Address Request as indicated in the table below, contents of the XX locations is detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 5 | Length of request from byte 5 up to checksum (not included) |
| 5 | 23 | Read Memory by Address request service ID |
| 6 | XX | High byte of address |
| 7 | XX | Middle byte of address |
| 8 | XX | Low byte of address |
| 9 | XX | Memory size |
| 10 | CS | 1 byte request checksum |

If AC0 is equal to $00

Copy the global address into request starting at byte 6

Copy the global length (low byte) into request starting at byte 9

Else

Find the routine indicated by the value in AC0

Copy the routine address into request starting at byte 6

Copy the routine length (low byte) into request starting at byte 9

Endif

Calculate checksum and copy into service request byte 10

Send Read Memory By Address Request and receive response

(continued)

Goto KWP2000 Response Processing

If Read Memory By Address positive response is received

If AC1 is equal to $00

If no data bytes received

Set high byte of storage location identified by AC2 to 0

Set low byte of storage location identified by AC2 to 0

Else if 1 data byte received

Copy data byte to high byte of storage location identified by AC2

Set low byte of storage location identified by AC2 to 0

Else

Copy 1.data byte to high byte of storage location identified by AC2

Copy 2.data byte to low byte of storage location identified by AC2

Else

Clear 256-byte data buffer identified by AC2

Copy over as many data bytes as received into data buffer identified by AC2 (max. 256)

Look for Read Memory By Address Request positive response service ID ($63) in goto fields

Else

Look for Read Memory By Address Request negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 23 | 03 00 03 00 | 63 11 10 0D FF 99 00 00 00 00 |

## 27 Op-Code: SR 27 for Security Access Request

Supported By:

KWP2000 Interpreter 2

Description:

The 27 Op-Code is used to open an ECU's memory for reprogramming.  The value in Action Field zero (AC0),  identifies which security algorithm to use for unlocking the ECU for programming.  The security algorithm defines the functions that are performed on the seed to determine the ECU key.  A response code of 37 indicates that the security access time out has not expired.  When a $37 is received the Interpreter delays approximately 2 seconds then retries the security request.  The Interpreter repeats the looping process for 5 tries or approximately 10 seconds.

If security fails for "Invalid Key - $35"  the Interpreter will retry the entire security process (starting with request 1) using the ones complement security algorithm.  This provides a means to program development ECUs using a production Utility File.

Action Fields:

| AC0 | $00 - Default Security Algorithm (One's Complement) $01 to $FE are Production Security Algorithms |
| --- | --- |
| AC1 | $00 – Don't send key if seed is 0 (according to specification) $01 – Send key even if seed is 0 |
| AC2 | $00 – GM Security Algorithm, Security Key Calculation in Server Application (exchanged via VIT 2) $01 – Not implemented: GM Security Algorithm, Security Key Calculation in VCS $02 – Not implemented: Non GM Security Algorithm, Security Key Calculation in Server Application (exchanged via VIT 2) $03 – Non GM Security Algorithm, Security Key Calculation in VCS |
| AC3 | Exceptions: $00 - Security level 1 (level $01 and $02 – get seed/send key) $01 - Security level 3 ( DVT level $03 and $04 – DVT get seed/send key) $XX – Security level (2*XX + 1) Example : $7D = security level $FB ($FB and $FC – get seed/send key) |

Pseudo Code:

XX   27    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Security Access Request 1 as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 02 | Length of request from byte 5 up to checksum (not included) |
| 5 | 27 | Security access request service ID |
| 6 | ## | AC3 (1) Access mode - if AC3 = 0, this byte would be 1, if AC3 = 1, this byte would be 3, a. s. o.. |
| 7 | CS | 1 byte message checksum |

Calculate checksum and copy into service request (byte 7)

Send Security Access Request AC3 (1) for request seed and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

(continued)

If Security Access positive response AC3 (1) is received

If both bytes of the seed (Security Access positive response bytes 3 & 4) are zeros and AC1 isn't 1

Set Security Access positive response service ID to success ($34)

Indicate Security Access positive response received do not second Security Access Request AC3 (2)

Else

Use security algorithm corresponding to AC0 to determine ECU's key

Build Security Access request AC3(2)  as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 04 | Length of request from byte 5 up to checksum (not included) |
| 5 | 27 | Security access request service ID |
| 6 | ## | AC3 (2) Access mode - if AC3 = 0, this byte would be 2, if AC3 = 1 this byte would be 4, a. s. o. |
| 7 | XX | High byte of key |
| 8 | XX | Low byte of key |
| 9 | CS | 1 byte message checksum |

Calculate checksum and copy into service request (byte 9)

Send Security Access Request AC3 (2) for send key and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

Endif

If Security Access positive response AC3 (2) is received

Look for Security Access positive response (AC3) service ID ($34) in goto fields

Else

If Security Access negative response "Invalid Key - $35" is received

Repeat entire security process using ones complement security algorithm

If security still fails for "Invalid Key - $37" and ones complement was used

Look for Security Access negative response AC3 (2) response code ($35) in goto fields

Endif

Else

Look for Security Access negative response AC3 (2) response code in goto fields

Endif

Endif

Else

If Security Access negative response "Time Delay not Expired - $37" is received

If ten second timer has not expired

Delay 2 seconds

Retry entire security process

Else

Look for Security Access negative response AC3 (1) response code ($37) in goto fields

Endif

Else

Look for Security Access negative response AC3 (1) response code in goto fields

Endif

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all action and goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 02 | 27 | 01 00 00 00 | 34 03 37 13 FF 22 00 00 00 00 |

## 30 Op-Code: SR 30 to Input/Output Control by Local Identifier

Supported By:

KWP2000 Interpreter 2

Description:

The 30 Op-Code will build a standard service request $30 to send to an ECU.  A Service Request $30 is used for input/output control by local identifier.  This Op-Code also supports the option to pass in Control Options from a Data Routine.  For details on how to use this feature see the Action Fields and the Pseudo Code.  The routine that contains the Control Options should be formatted with a zero load address and a valid length (see description of Data Routines).  The data portion of the routine should only contain the Control Options to include in the request with no additional data.

Action Fields:

| AC0: | Input/Output Local Identifier |
| --- | --- |
| AC1: | Routine number from Utility File to use as Control Options in the Service Request 30 (Shall be $00 if not used) |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not Used |

Pseudo Code:

XX    30    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Input/Output Control by Local ID Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 2 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 30 | Start routine by local ID request service ID |
| 6 | AC0 | Routine local ID for routine to begin executing |
|     | Control Options to Pass | Data from routine indicated by value in AC1. Number of bytes (N) is 1 to 253. For no Control Options AC1 = 0, and N = 0. |
| 7 + N | CS | 1 byte message checksum |

If AC1 is not $00

Find routine indicated by value in AC1 to include in request as Control Options

Copy entire Data Routine into request starting at byte 7

Endif

Calculate checksum and copy into service request (byte 7 + N)

Send Input/Output Control by Local ID Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Input/Output Control by Local ID positive response is received

Look for Input/Output Control by Local ID positive response service ID ($70) in goto fields

Else

Look for Input/Output Control by Local ID negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 30 | 41 03 00 00 | 70 11 10 0D FF 99 00 00 00 00 |

## 31 Op-Code: SR 31 to Start Routine by Local Identifier

Supported By:

KWP2000 Interpreter 2

Description:

The 31 Op-Code will build a standard service request $31 to send to an ECU.  A Service Request $31 will start a routine identified by a local identifier.  This Op-Code also supports the option to pass in Routine Entry Options from a Data Routine.  For details on how to use this feature see the Action Fields and the Pseudo Code.  The routine that contains the Routine Entry Options should be formatted with a zero load address and a valid length (see description of Data Routines).  The data portion of the routine should only contain the Routine Entry Options to include in the request with no additional data.    If the routine execution results in the ECU responding with two bytes of additional data then Op-Code 71 should be used instead of Op-Code 31.

Action Fields:

| AC0: | Local Identifier for routine number to start executing |
| --- | --- |
| AC1: | Routine number from Utility File to use as Routine Entry Options in the Service Request 31 (Shall be $00 if not used) |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not Used |

Pseudo Code:

XX    31    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Start Routine by Local ID Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 2 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 31 | Start routine by local ID request service ID |
| 6 | AC0 | Routine local ID for routine to begin executing |
|     | Routine Entry Options to Pass | Data from routine indicated by value in AC1. Number of bytes (N) is 1 to 253. For no Routine Entry Options AC1 = 0, and N = 0. |
| 7 + N | CS | 1 byte message checksum |

If AC1 is not $00

Find routine indicated by value in AC1 to include in request as Routine Entry Options

Copy entire Data Routine into request starting at byte 7

Endif

Calculate checksum and copy into service request (byte 7 + N)

Send Start Routine by Local ID Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Start Routine by Local ID positive response is received

Look for Start Routine by Local ID positive response service ID ($71) in goto fields

Else

Look for Start Routine by Local ID negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 31 | 04 00 00 00 | 71 11 10 0D FF 99 00 00 00 00 |

## 32 Op-Code: SR 32 to Stop Routine by Local Identifier

Supported By:

KWP2000 Interpreter 2

Description:

The 32 Op-Code will build a standard service request $32 to send to an ECU.  This Op-Code is used to stop a Device Specific Control routine, from the Utility File, which was started by an Op-Code 31.

Action Fields:

| AC0: | Local Identifier for routine number to stop executing |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not Used |

Pseudo Code:

XX    32    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Stop Routine by Local ID Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 2 | Length of request from byte 5 up to checksum (not included) |
| 5 | 32 | Stop routine by local ID request service ID |
| 6 | AC0 | Routine local ID for routine to stop executing |
| 7 | CS | 1 byte message checksum |

Calculate checksum and copy into service request (byte 7)

Send Stop Routine by Local ID Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Stop Routine by Local ID positive response is received

Look for Stop Routine by Local ID positive response service ID ($72) in goto fields

Else

Look for Stop Routine by Local ID negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 11 | 32 | 03 00 00 00 | 72 12 10 0D FF 99 00 00 00 00 |

## 33 Op-Code: SR 33 to Request Routine Results by Local Identifier

Supported By:

KWP2000 Interpreter 2

Description:

The 33 Op-Code will build a standard service request $33 to send to an ECU.  A Service Request $33 will request the results (e.g. exit status information) generated by a routine which is identified by a local identifier.  A positive response code $73 indicates the routine has successfully completed execution.

Action Fields:

| AC0: | Local Identifier for routine number of which results are requested |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not Used |

Pseudo Code:

XX    33    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Request Routine Results by Local ID Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 2 | Length of request from byte 5 up to checksum (not included) |
| 5 | 33 | Request routine results by local ID request service ID |
| 6 | AC0 | Routine local ID for routine from which results are requested |
| 7 | CS | 1 byte message checksum |

Calculate checksum and copy into service request (byte 7)

Send Request Routine Results by Local ID Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If  Request Routine Results by Local ID positive response is received

Look for Request Routine Results by Local ID positive response service ID ($73) in goto fields

Else

Look for Request Routine Results by Local ID negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 12 | 33 | 06 00 00 00 | 73 13 23 0E 64 99 FF 97 00 00 |

## 34 Op-Code: SR 34 for Download Request

Supported By:

KWP2000 Interpreter 2

Description:

The 34 Op-Code will build a standard Service Request 34 to send to an ECU.  A request 34 will prepare the ECU to receive data from the serial data link.  Request 34s are used in conjunction with service request mode 36s (Op-Codes 90, 91, and 93) to download information to ECU's.

Action Fields:

| AC0: | $00 - Not used |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | Data Format Identifier, only used with AC3 equal to $01 |
| AC3: | Exceptions: $00 - Standard Service Request 34 message format $01 - Include memory address, Data Format ID, and memory size in service request |

Pseudo Code:

XX    34    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Request Download Request as indicated below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 01 | Length of request from byte 5 up to checksum (not included) |
| 5 | 34 | Request download request service ID |

If AC3 is $01

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 6 | XX | Memory address - high byte |
| 7 | XX | Memory address - middle byte |
| 8 | XX | Memory address - low byte |
| 9 | AC2 | Data Format Identifier |
| 10 | XX | Uncompressed memory size - high byte |
| 11 | XX | Uncompressed memory size - middle byte |
| 12 | XX | Uncompressed memory size - low byte |

Set the high byte of the memory address to the high byte of the global address set in a F1 Op-Code

Set the middle byte of the memory address to the middle byte of the global address set in a F1 Op-Code

Set the low byte of the memory address to the low byte of the global address set in a F1 Op-Code

Set the Data Format Identifier to the value in AC2

Set the high byte of the memory size to the high byte of the global size set in a F2 Op-Code

Set the middle byte of the memory size to the middle byte of the global size set in a F2 Op-Code

Set the low byte of the memory size to the low byte of the global size set in a F2 Op-Code

Endif

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 6 or 13 | CS | 1 byte request checksum |

(continued)

Calculate checksum and copy into the service request (byte 6 or 13)

Send Request Download Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Request Download positive response is received

Look for Request Download positive response Service ID ($74) in goto fields

Else

Look for Request Download negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 34 | 00 00 00 00 | 74 11 10 0D FF 99 00 00 00 00 |

## 37 Op-Code: SR 37 for Request Transfer Exit

Supported By:

KWP2000 Interpreter 2

Description:

The 37 Op-Code will build a standard Service Request 37 to send to an ECU.  A request 37 will Request Transfer Exit.  A Request 37 may be required at the end of a reprogramming session after all of the Transfer Data requests are completed.

Action Fields:

| AC0: | $00 - Not used |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not used |

Pseudo Code:

XX    37    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Request Transfer Exit Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 01 | Length of request from byte 5 up to checksum (not included) |
| 5 | 37 | Request Transfer Exit request service ID |
| 6 | CS | 1 byte request checksum |

Calculate checksum and copy into the service request (byte 6)

Send Request Transfer Exit and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Request Transfer Exit positive response is received

Look for Request Transfer Exit positive response service ID ($77) in goto fields

Else

Look for Request Transfer Exit negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 37 | 02 04 00 01 | 77 11 FF 3D 00 00 00 00 00 00 |

## 38 Op-Code: SR 38 to Start Utility File Routine by Address

Supported By:

KWP2000 Interpreter 2

Description:

The 38 Op-Code will build a standard service request 38 to send to an ECU.   This Op-Code is used to start a  Device Specific Control Routine, from the Utility File, that was downloaded to RAM by an Op-Code 90.  The address information for the service request is read from the routine in the Utility File.  This Op-Code also supports the option to pass in Routine Entry Options from a Data Routine.  For details on how to use this feature see the Action Fields and the Pseudo Code.  The routine that contains the Routine Entry Options should be formatted with a zero load address and a valid length (see description of Data Routines).  The data portion of the routine should only contain the Routine Entry Options to include in the request with no additional data.  If the routine execution results in the ECU responding with two bytes of additional data then Op-Code 78 should be used instead of Op-Code 38.

Action Fields:

| AC0: | Routine number from Utility File to get execution address from |
| --- | --- |
| AC1: | Routine number from Utility File to use as Routine Entry Options in the Service Request 38 (Shall be $00 if not used) |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Use address from routine number AC0 $01 - Use global address information from an Op-Code 'F1' |

Pseudo Code:

XX    38    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Start Routine by Address Request as indicated in the table below, contents of the XX locations is

detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 4 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 38 | Start Routine by Address request service ID |
| 6 | XX | High byte of address |
| 7 | XX | Middle byte of address |
| 8 | XX | Low byte of address |
|     | Routine Entry Options to Pass | Data from routine indicated by value in AC1. Number of bytes (N) is 1 to 251. For no routine entry options AC1 = 0, and N = 0. |
| 9 + N | CS | 1 byte request checksum |

If AC0 is not equal to $00

Find the routine indicated by the value in AC0

Endif

If AC3 is $00

Copy the routine address into request starting at byte 6 (or byte 7 for two byte addressing)

Else

Copy the global address into request starting at byte 6 (or byte 7 for two byte addressing)

Endif

(continued)

If AC1 is NOT $00

Find routine indicated by value in AC1 to include in request as Routine Entry Options

Copy entire Data Routine into request starting at byte 9

Endif

Calculate checksum  and copy  into service request byte ( 9  + N )

Send Start Routine by Address Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Start Routine by Address positive response is received

Look for Start Routine by Address positive response service ID ($78) in goto fields

Else

Look for Start Routine by Address negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 38 | 20 00 00 00 | 78 11 10 0D FF 99 00 00 00 00 |

## 39 Op-Code: SR 39 to Stop Utility File Routine by Address

Supported By:

KWP2000 Interpreter 2

Description:

The 39 Op-Code will build a standard service request 39 to send to an ECU.   This Op-Code is used to stop a  Device Specific Control Routine, from the Utility File, which was started by an Op-Code 38.  The address information for the service request is read from the routine in the Utility File.

Action Fields:

| AC0: | Routine number from Utility File to get execution address from |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Use address from routine number AC0 $01 - Use global address information from an Op-Code 'F1' |

Pseudo Code:

XX    39    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Stop Routine by Address Request as indicated in the table below, contents of the XX locations is

detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 4 | Length of request from byte 5 up to checksum (not included) |
| 5 | 39 | Start Routine by Address request service ID |
| 6 | XX | High byte of address |
| 7 | XX | Middle byte of address |
| 8 | XX | Low byte of address |
| 9 | CS | 1 byte request checksum |

If AC0 is not equal to $00

Find the routine indicated by the value in AC0

Endif

If AC3 is $00

Copy the routine address into request starting at byte 6 (or byte 7 for two byte addressing)

Else

Copy the global address into request starting at byte 6 (or byte 7 for two byte addressing)

Endif

Calculate checksum  and copy  into service request byte 9

Send Start Routine by Address Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Start Routine by Address positive response is received

Look for Start Routine by Address positive response service ID ($79) in goto fields

Else

Look for Start Routine by Address negative response, response code in goto fields

Endif

(continued)

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 39 | 02 00 00 00 | 79 11 10 0D FF 99 00 00 00 00 |

## 3A Op-Code: SR 3A to Request Routine Results by Address

Supported By:

KWP2000 Interpreter 2

Description:

The 3A Op-Code will build a standard service request 3A to send to an ECU.  A Service Request 3A will request the results (e.g. exit status information) generated by a routine which is identified by a memory address.  A positive response code 7A indicates the routine has successfully completed execution.

Action Fields:

| AC0: | Routine number from Utility File to get execution address from |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Use address from routine number AC0 $01 - Use global address information from an Op-Code 'F1' |

Pseudo Code:

XX    3A    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Request Routine Results by Address Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 4 | Length of request from byte 5 up to checksum (not included) |
| 5 | 3A | Request routine results by address request service ID |
| 6 | XX | High byte of address |
| 7 | XX | Middle byte of address |
| 8 | XX | Low byte of address |
| 9 | CS | 1 byte request checksum |

Calculate checksum and copy into service request (byte 9)

Send Request Routine Results by Local ID Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If  Request Routine Results by Local ID positive response is received

Look for Request Routine Results by Local ID positive response service ID ($7A) in goto fields

Else

Look for Request Routine Results by Local ID negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 12 | 3A | 06 00 00 00 | 7A 13 23 0E 64 99 FF 97 00 00 |

## 3B Op-Code: SR 3B to Write Data by Local Identifier

Supported By:

KWP2000 Interpreter 2

Description:

The 3B Op-Code will build a standard service request 3B to send to an ECU. This Op-Code is used to write data by Local Identifier. The data to download is determined by the information contained in the Action Fields. This Op-Code also supports the option to pass in Record Values from a Data Routine or Module and from the ECU Configuration Data. For details on how to use this feature see the Action Fields and the Pseudo Code. The routine that contains the Record Values should be formatted with a zero load address and a valid length (see description of Data Routines). The data portion of the routine should only contain the Record Values to include in the request with no additional data.

Action Fields:

| AC0: | Local identifier of data being downloaded Reserved Local Ids*: $E0 - $EF Special services action codes (length = 1 byte) $90 - Send VIN in request (length = 17 bytes) $98 - Serial number in request (length = 10 bytes) $99 - Send programming date in request (length = 4 bytes BCD or 6 bytes ASCII) * if AC1 is not $00 all values are allowed |
| --- | --- |
| AC1: | Routine number from Utility File or module number to use as Record Values in the Service Request 3B or ECU Configuration ID (Shall be $00 if not used) |
| AC2: | $00 – Use routine for record values $01 – Use ECU configuration data for record values $02 – Use Module for record values $06 – Convert programming date into ASCII ** ** if AC0 is $99, AC1 and AC3 are $00 |
| AC3: | Exceptions: $00 - Use value in AC0 as Local ID of request. $01 - Local ID is in routine or module does not include AC0 value in request. |

Pseudo Code:

XX   3B    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Write Data by Local ID Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target ID |
| 3 | ## | Global source byte |
| 4 | 2 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 3B | Write data by local ID request service ID |
| 6 | XX | Local identifier of data to download |
|     | Record Values 1 to N | The pseudo code below explains how the Record Value portion of this request is filled. The maximum number of bytes is 253. N = VIN or part number length + routine length. |
| 6 + N | CS | 1 byte request checksum |

If AC1 is not $00

If AC2 is $00

Find routine indicated by value in AC1 to include in request as Record Values

if AC3 is $00

Use AC0 as local ID

Copy entire Data Routine into request starting at first record value byte

ElseIf AC3 is $01

Use first byte of routine as local ID

(continued)

Copy remaining Routine Data into request starting at first record value byte

Else

ERR_FATAL

Elseif AC2 is $01

if AC3 is $00

Use AC0 as local ID

Copy ECU configuration Data into request starting at first record value byte

Else

ERR_FATAL

Endif

Elseif AC2 is $02

if AC3 is $00

Use AC0 as local ID

Copy entire Module Data into request starting at first record value byte

ElseIf AC3 is $01

Use first byte of Module as local ID

Copy remaining Module Data into request starting at first record value byte

Else

ERR_FATAL

Endif

Else

ERR_FATAL

Else

if AC3 is $00

Use AC0 as local ID

If AC0 is 90

Copy the VIN into the request starting at byte 7

Elseif AC0 is 98

Copy the serial number into the request starting at byte 7

Elseif AC0 is 99

If AC2 is 6

Convert programming date into a 6 character ASCII string with format YYMMDD

Copy the 6 character ASCII string into the request starting at byte 7

Else

Copy the programming date into the request starting at byte 7

Endif

Elseif AC0 < $E0 or AC0 > $EF

ERR_FATAL

Endif

Else

ERR_FATAL

Endif

Calculate checksum and copy into the service request (byte 6 + N)

Send Write Data by Local ID Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Write Data by Local ID positive response is received

Look for Write Data by Local ID positive response service ID ($7B) in goto fields

Else

Look for Write Data by Local ID negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 3B | 90 00 00 00 | 7B 22 FF 99 00 00 00 00 00 00 |

## 3D Op-Code: SR 3D to Write Memory by Address

Supported By:

KWP2000 Interpreter 2

Description:

The 3D Op-Code will build a standard service request 3D to send to an ECU. This Op-Code is used to Write Data by Address to an ECU. The data to download is determined by the information contained in the Action Fields. Either the global address information or the routine address is used for the address in the Service Request. The data may either be the VIN, the Manufacturer Hardware Number or reside in a Data Routine. An exception allows the downloading of data with multiple services. For details on how to use this feature see the Action Fields and the Pseudo Code. The data portion of the routine should only contain the Record Values to include in the request with no additional data.

Action Fields:

| AC0: | Reserved IDs to indicate data to write to ECU (shall be $00 to use only routine data) |
| --- | --- |
|  | 80 - Include the VIN in Service Request (17 data bytes) 84 - Include the Manufacturer Hardware Number in Service Request (20 data bytes) |
| AC1: | Routine number from Utility File to use as Record Values in the Service Request 3D (Shall be $00 if not used) |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 – Download routine in one service (needs to be small enough) $01 – Download routine using multiple services, use routine address as download address and global header length as packet size |

Pseudo Code:

XX   3B    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Write Data by Memory Address Service Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target ID |
| 3 | ## | Global source byte |
| 4 | 5 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 3D | Write Memory By Address request service ID |
| 6 | XX | Memory address - High byte |
| 7 | XX | Memory address - Middle byte |
| 8 | XX | Memory address - Low byte |
| 9 | N | Memory size |
|     | Record Values 1 to N | The pseudo code below explains how the Record Value portion of this request is filled. The maximum number of bytes is 250. N = VIN or part number length + routine length. |
| 9 + N | CS | 1 byte request checksum |

If AC0 is $80

Copy the VIN into the service request starting at byte 10

Else

If AC0 is $84

Copy the Manufacturer Hardware Number into the service starting at byte 10

Endif

Endif

(continued)

If AC3 is $01

If AC1 is not $00

Find routine indicated by value in AC1 to include in request as Record Values

Copy the routine address into request starting at byte 6 (or byte 7 for two byte addressing)

Endif

If AC0 is $00

Do while more routine bytes downloaded

If this is the last packet of the routine

Copy the remaining routine into the data area of buffer

Else

Copy as many routine data bytes into the Write Memory By Address Service Request (starting at byte 10) as the utility file message length states (max. 250)

Endif

Copy number of data bytes into byte 9

Calculate checksum and copy into the Write Memory By Address Service Request

Send Write Data by Memory Address Service Request and receive response

If an error occurred

End the routine download

Endif

End do while more routine bytes downloaded

Else

Copy number of data bytes into byte 9

Calculate checksum and copy into the Write Memory By Address Service Request

Send Write Data by Memory Address Service Request and receive response

Endif

Else

Copy the global address into request starting at byte 6 (or byte 7 for two byte addressing)

If AC1 is not $00

Find routine indicated by value in AC1 to include in request as Record Values

Copy entire Data Routine into request starting at next record value byte

Endif

Copy utility file message length into byte 9

Calculate checksum and copy into the service request (byte 9 + N)

Send Write Data by Memory Address Service Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Write Data by Memory Address positive response is received

Look for Write Data by Memory Address positive response service ID ($7D) in goto fields

Else

Look for Write Data by Memory Address negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 3D | 90 00 2B 00 | 7D 22 FF 99 00 00 00 00 00 00 |

## 71 Op-Code: SR 31 to Start Routine by Local Identifier and Save 2 Bytes

Supported By:

KWP2000 Interpreter 2

Description:

The 71 Op-Code will build a standard service request $31 to send to an ECU.  A request $31 will start a routine identified by a local identifier.  The 71 Op-Code is identical to the 31 Op-Code, but in addition the 71 Op-Code expects 2 bytes of data to be included in the positive response from the ECU.  The value in Action Field 2 (AC2) is the ID of the storage location for the two bytes.  The valid IDs are: 0x00 thru 0x13.  This Op-Code also supports the option to pass in Routine Entry Options from a Data Routine.  For details on how to use this feature see the Action Fields and the Pseudo Code.  The routine that contains the Routine Entry Options should be formatted with a zero load address and a valid length (see description of Data Routines).  The data portion of the routine should only contain the Routine Entry Options to include in the request with no additional data.

Note:   This OpCode uses the 2-byte storage buffers.  For buffer usage information, see the section: “Programming Buffers – Recommended Usage”

Action Fields:

| AC0: | Local Identifier for routine number to start executing |
| --- | --- |
| AC1: | Routine number from Utility File to use as Routine Entry Options in the Service Request 31 (Shall be $00 if not used) |
| AC2: | ID of storage location to store 2 byte response (0x00 – 0x13) |
| AC3: | Exceptions: $00 - Not used |

Pseudo Code:

XX    71    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Start Routine by Local ID Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 2 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 31 | Start routine by local ID request service ID |
| 6 | AC0 | Routine local ID for routine to begin executing |
|     | Routine Entry Options to Pass | Data from routine indicated by value in AC1. Number of bytes (N) is 1 to 253. For no Routine Entry Options AC1 = 0, and N = 0. |
| 7 + N | CS | 1 byte message checksum |

If AC1 is not $00

Find routine indicated by value in AC1 to include in request as Routine Entry Options

Copy entire Data Routine into request starting at byte 7

Endif

Calculate checksum and copy into service request (byte 7 + N)

Send Start Routine by Local ID Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

(continued)

If Start Routine by Local ID positive response is received

Copy first byte of positive response data to high byte of storage location identified by AC2

Copy second byte of positive response data to low byte of storage location identified by AC2

Look for Start Routine by Local ID positive response service ID ($71) in goto fields

Else

Look for Start Routine by Local ID negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 71 | 04 00 01 00 | 71 11 10 0D FF 99 00 00 00 00 |

## 78 Op-Code: SR 38 to Start Utility File Routine by Address and Save 2 Bytes

Supported By:

KWP2000 Interpreter 2

Description:

The 78 Op-Code will build a standard service request 38 to send to an ECU.  This Op-Code is used to start a  Device Specific Control Routine, from the Utility File, that was downloaded to RAM by an Op-Code 90.  The address information for the service request is read from the routine in the Utility File.  The 78 Op-Code is identical to the 38 Op-Code, but in addition the 78 Op-Code expects 2 bytes of data to be included in the  positive response from the ECU.  The value in Action Field 2 (AC2) is the ID of the storage location for the two bytes.  The valid IDs are: 0x00 thru 0x13.    This Op-Code also supports the option to pass in Routine Entry Options from a Data Routine.  For details on how to use this feature see the Action Fields and the Pseudo Code.  The routine that contains the Routine Entry Options should be formatted with a zero load address and a valid length (see description of Data Routines).  The data portion of the routine should only contain the Routine Entry Options to include in the request with no additional data.

Note:   This OpCode uses the 2-byte storage buffers.  For buffer usage information, see the section: “Programming Buffers – Recommended Usage”

Action Fields:

| AC0: | Routine number from Utility File to get execution address from |
| --- | --- |
| AC1: | Routine number from Utility File to use as Routine Entry Options in the Service Request 38 (Shall be $00 if not used) |
| AC2: | ID of storage location to 2 byte response (0x00 – 0x13) |
| AC3: | Exceptions: $00 - Use address from routine number AC0 $01 - Use global address information from an Op-Code 'F1' |

Pseudo Code:

XX    78    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Start Routine by Address Request as indicated in the table below, contents of the XX locations is

detailed in pseudo code following the table

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 4 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 38 | Start Routine by Address request service ID |
| 6 | XX | High byte of address |
| 7 | XX | Middle byte of address |
| 8 | XX | Low byte of address |
|     | Routine Entry Options to Pass | Data from routine indicated by value in AC1. Number of bytes (N) is 1 to 251. For no Routine Entry Options AC1 = 0, and N = 0. |
| 9 + N | CS | 1 byte request checksum |

If AC0 is not equal to $00

Find the routine indicated by the value in AC0

Endif

(continued)

If AC3 is $00

Copy the routine address into request starting at byte 6 (or byte 7 for two byte addressing)

Else

Copy the global address into request starting at byte 6 (or byte 7 for two byte addressing)

Endif

If AC1 is NOT $00

Find routine indicated by value in AC1 to include in request as Routine Entry Options

Copy entire Data Routine into request starting at byte 9

Endif

Calculate checksum  and copy  into service request byte ( 9  + N )

Send Start Routine by Address Request and receive response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Start Routine by Address positive response is received

Copy appropriate response byte to high byte of storage location identified by AC2

Copy appropriate response byte to low byte of storage location identified by AC2

Look for Start Routine by Address positive response service ID ($78) in goto fields

Else

Look for Start Routine by Address negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 78 | 20 00 03 00 | 78 11 10 0D FF 99 00 00 00 00 |

## 81 Op-Code: SR 81 to Start Communications

Supported By:

KWP2000 Interpreter 2

Description:

The 81 Op-Code is used to start communications with an ECU.  Communications will remain established while  activity exists on the link.  If the link is inactive for five seconds the ECU will revert back to a waiting for communications state.  There are no negative responses allowed for the Start Communication request.

Action Fields:

| AC0: | $00 - Not used |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not used |

Pseudo Code:

XX    81    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Wait for W5min idle time

Send wake up pattern to the ECU

Build Start Communication Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 81 | Format byte for physical addressing and length |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 81 | Start communication request service ID |
| 5 | CS | 1 byte service request checksum |

Calculate checksum and copy into the service request (byte 5)

Send Start Communication Request and get the response (up to 10 Bytes)

Goto KWP2000 Response Processing

Set addressing mode and use of length information based on two returned key bytes

Look for Start Communication positive response service ID ($C1) in goto fields

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 81 | 00 00 00 00 | C1 02 FF 99 00 00 00 00 00 00 |

## 82 Op-Code: SR 82 to Stop Communications

Supported By:

KWP2000 Interpreter 2

Description:

The 82 Op-Code is used to stop communications with an ECU.  There is no negative response allowed for the Stop Communication request.

Action Fields:

| AC0: | $00 - Not used |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not used |

Pseudo Code:

XX    82    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Stop Communication Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 81 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 82 | Stop communication request service ID |
| 5 | CS | 1 byte service request checksum |

Calculate checksum and copy into the service request (byte 5)

Send Stop Communication Request and get the response (up to 10 Bytes)

Goto KWP2000 Response Processing

Look for Stop Communication positive response service ID ($C2) in goto fields

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 82 | 00 00 00 00 | C2 02 FF 09 00 00 00 00 00 00 |

## 83 Op-Code: SR 83 to Access Communications Parameters

Supported By:

KWP2000 Interpreter 2

Description:

The 83 Op-Code is used to access the communications parameters.  This Op-Code is issued after a security request (Op-Code 27) has passed.  This Op-Code is performed in two steps first, the optimum timing parameters for use during a reprogramming event are read from the ECU.  Second,  the optimum timing parameters are put into a service request 83 and sent to the ECU.

Action Fields:

| AC0: | $00 - Not used |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Not used |

Pseudo Code:

XX    83    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Access Communications Parameters Request 1 as indicated in the table below.  Provided this request is done after security has passed the ECU will return the reprogramming timing parameters.

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 82 | Format byte for physical addressing plus length |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 83 | Access communication parameters request service ID |
| 5 | 00 | Timing parameter identifier |
| 6 | CS | 1 byte service request checksum |

Calculate checksum and copy into the service request (byte 6)

Send Access Communications Parameters Request 1 and get the response (up to 10 Bytes)

Goto KWP2000 Response Processing

If Access Communications Parameter Request 1 positive response is received

Build Access Communications Parameters Request 2 as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 87 | Format byte for physical addressing plus length |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 83 | Access communication parameters request service ID |
| 5 | 03 | Timing parameter identifier |
| 6 | XX | P2 minimum |
| 7 | XX | P2 maximum |
| 8 | XX | P3 minimum |
| 9 | XX | P3 maximum |
| 10 | XX | P4 minimum |
| 11 | CS | 1 byte service request checksum |

(continued)

Copy P2 minimum from Access Communications Parameters Request 1 to Request 2 position 6

Copy P2 maximum from Access Communications Parameters Request 1 to Request 2 position 7

Copy P3 minimum from Access Communications Parameters Request 1 to Request 2 position 8

Copy P3 maximum from Access Communications Parameters Request 1 to Request 2 position 9

Copy P4 minimum from Access Communications Parameters Request 1 to Request 2 position 10

Calculate checksum and copy into the service request (byte 11)

Send Access Communications Parameter Request 2 and get the response (up to 10 Bytes)

Goto KWP2000 Response Processing

Else

Set accessing parameter list error

Endif

If Access Communication  positive response 2 is received

Set tester timing values as indicated in response 1

Look for Access Communication positive response 2 service ID ($C3) in goto fields

Else

Set error accessing parameter list

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 83 | 00 00 00 00 | C3 02 FF 39 00 00 00 00 00 00 |

## 84 Op-Code: SR 83 to Set Communications Parameters

Supported By:

KWP2000 Interpreter 2

Description:

The 84 Op-Code is used to set the communications parameters. This Op-Code is issued after a security request (Op-Code 27) has passed. This Op-Code is performed in one single step. The optimum timing parameters for use during a reprogramming event defined by action fields 0..3 are put into a service request 83 and sent to the ECU. If action fields 0..3 are all 0, use a defined set of timing parameters (see pseudo code).

Action Fields:

| AC0: | P2 maximum | - AC0 <= $F0: resolution 25 ms - AC0 > $F0: resolution 6.4 sec * (AC0 - $F0). |
| --- | --- | --- |
| AC1: | P3 minimum | - resolution 0.5 ms |
| AC2: | P3 maximum | - resolution 250 ms |
| AC3: | P4 minimum | - resolution 0.5 ms |

Pseudo Code:

XX    84    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Access Communications Parameters Request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 87 | Format byte for physical addressing plus length |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 83 | Access communication parameters request service ID |
| 5 | 03 | Timing parameter identifier |
| 6 | 00 | P2 minimum |
| 7 | AC0 | P2 maximum |
| 8 | AC1 | P3 minimum |
| 9 | AC2 | P3 maximum |
| 10 | AC3 | P4 minimum |
| 11 | CS | 1 byte service request checksum |

If AC0, AC1, AC2 and AC3 are all $00

Set P2 maximum to 1 (25 ms)

Set P3 minimum to 40 (20 ms)

Set P3 maximum to 1 (250 ms)

Set P4 minimum to 0 (0 ms)

Endif

Calculate checksum and copy into the service request (byte 11)

Send Access Communications Parameter Request and get the response (up to 10 Bytes)

Goto KWP2000 Response Processing

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 84 | 01 0A 78 00 | C3 02 FF 39 00 00 00 00 00 00 |

## 90 Op-Code: SR 36 to Transfer a Routine to ECU

Supported By:

KWP2000 Interpreter 2

Description:

The 90 Op-Code is used to build a service request 36 for downloading a reprogramming control routine to an ECU.  The number of data bytes to include in each request is retrieved from the Utility File header.  Depending on the ECU being programmed this Op-Code may need to be preceded by a service request 34 (see Op-Code 34 ).

The 90 Op-Code uses the global target and source byte for all requests.  If a target and source byte other than the global target and source byte needs to be used, the Interpreter instructions should contain a 01 Op-Code prior to this Op-Code.

Action Fields:

| AC0: | Routine number to download |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Use address from the routine in the Transfer Data Request (4 bytes) $01 - Do not include any address information in the Transfer Data Request $40 - Send a RequestDownload (use global memory address) before and a RequestTransferExit after each Transfer Data Request. Use address from the routine in the Transfer Data Service Request (4 bytes) $41 - Send a RequestDownload (use global memory address) before and a RequestTransferExit after each Transfer Data Request. Do not include any address information in the Transfer Data Service Request |

4 byte naming convention = High, MidHigh, MidLow, Low

Pseudo Code:

XX    90    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Find the routine indicated by value AC0

Use KWP2000 Normal Response processing

Build Transfer Data Request as indicated below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 4 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 36 | Transfer Data Request Service ID |

If AC3 is $00 or $40

For 2 bytes for address information:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 6 | XX | Memory address - High byte |
| 7 | XX | Memory address - Low byte |
| 8 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 7 + N | XX | Data byte N (N <= 252) |
| 8 + N | CS | 1 byte request checksum |

Set High byte of memory address to MidLow byte of routine address

Set Low byte of memory address to Low byte of routine address

(continued)

For 3 bytes for address information:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 6 | XX | Memory address - High byte |
| 7 | XX | Memory address - Middle byte |
| 8 | XX | Memory address - Low byte |
| 9 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 8 + N | XX | Data byte N (N <= 251) |
| 9 + N | CS | 1 byte request checksum |

Set High byte of memory address to MidHigh byte of routine address

Set Middle byte of memory address to MidLow byte of routine address

Set Low byte of memory address to Low byte of routine address

Else

Do not include any address information in the Service Request

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 5 + N | XX | Data byte N (N <= 254) |
| 6 + N | CS | 1 byte request checksum |

Endif

Do while more routine to download

If this is the last packet of the routine

Change request length (byte 4) to size of last packet

Copy the remaining routine into the data area of buffer

Else

Copy the next packet of routine into the data area of buffer starting at byte 6 or 10

Endif

Calculate checksum and copy into the Transfer Data Service Request

If AC3 is $40 or $41

Build Transfer Data Request as indicated below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 12 | Length of request from byte 5 up to checksum (not included) |
| 5 | 34 | Request Download Request Service ID |
| 6 | XX | Global address - High byte |
| 7 | XX | Global address - Middle byte |
| 8 | XX | Global address - Low byte |
| 9 | 00 | Data Format Identifier |
| 10 | XX | Uncompressed memory size - high byte |
| 11 | XX | Uncompressed memory size - middle byte |
| 12 | XX | Uncompressed memory size - low byte |

Note:	Increment Global address
If remaining data is less, use number of remaining bytes for Uncompressed memory size

Calculate checksum and copy into the Request Download service request

Send the Request Download request and expect a response

If an error occurred

End the routine download

Endif

Endif																					(continued)

Send the Transfer Data Request and expect a response (up to 10 bytes)

If AC3 is $00 or $40

Calculate the next download address

Copy the next download address in the Service Request

Endif

If an error occurred

End the routine download

Endif

If AC3 is $40 or $41

Build Request Transfer Exit Request as indicated below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 05 | Length of request from byte 5 up to checksum (not included) |
| 5 | 37 | Request Transfer Exit Request Service ID |

Calculate checksum and copy into the Request Transfer Exit service request

Send the Request Transfer Exit request and expect a response

If an error occurred

End the routine download

Else

Increment address used in Request Download service by the number of bytes transferred

If remaining data is less than uncompressed memory size

Use number of remaining bytes in Request Download service

Endif

Endif

Endif

End do while more routine to download

If Transfer Data positive response is received

Look for Transfer Data positive response service ID ($76) in goto fields

Else

Look for Transfer Data negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 90 | 01 00 03 00 | 76 11 FD 0D FF 20 00 00 00 00 |

## 93 Op-Code: SR 36 to Transfer a Calibration File to ECU

Supported By:

KWP2000 Interpreter 2

Description:

The 93 Op-Code is used to build a service request 36 for downloading a calibration file (or software) to an ECU.  Depending on the product being programmed this Op-Code may need to be preceded by a service request 34 (see Op-Code 34 ).  The 93 Op-Code supports the use of either the header address or the global address for the Service Requests.  To set the global address an 'F1' Op-Code should be included in the Interpreter Instructions prior to the '93' Op-Code.  The 93 Op-Code supports either two or three byte addressing.  The number of data bytes to include in each request is retrieved from the Utility File header.

The 93 Op-Code uses the global target and source byte for all requests.  If a target and source byte other than the global target and source byte needs to be used the Interpreter instructions should contain a 01 Op-Code prior to this Op-Code.

Action Fields:

| AC0: | Module ID for file to download |
| --- | --- |
| AC1: | $00 - Not used |
| AC2: | $00 - Not used |
| AC3: | Exceptions: $00 - Use utility file header address and keep address constant for entire calibration transfer (4 bytes) $01 - Use global address and keep address constant for entire calibration transfer (4 bytes) $02 - Do not include any address information in the Service Request $03 - Do not include any address information in the Service Request. Include data packet checksum byte and data packet length byte in front of the data packet bytes. Data packet checksum is a simple 8 bit sum series of all data packet bytes. The data packet length byte represents the number of data packet bytes. $80 - Use utility file header address and increment packet address by number of bytes transferred (4 bytes) $81 - Use global address and increment packet address by number of bytes transferred (4 bytes) $40 - Send a RequestDownload request before and a RequestTransferExit request after each TransferData request, use utility file header address in TransferData and keep address constant for entire calibration transfer (4 bytes) $41 - Send a RequestDownload request before and a RequestTransferExit request after each TransferData request, use global address in TransferData and keep address constant for entire calibration transfer (4 bytes) $42 – Send a RequestDownload request before and a RequestTransferExit request after each TransferData request, do not include any address information in the TransferData Service Request $43 - Send a RequestDownload request before and a RequestTransferExit request after each TransferData request do not include any address information in the TransferData Service Request. Include data packet checksum byte and data packet length byte in front of the data packet bytes. Data packet checksum is a simple 8 bit sum series of all data packet bytes. The data packet length byte represents the number of data packet bytes. $C0 - Send a RequestDownload request before and a RequestTransferExit request after each TransferData request, use utility file header address in TransferData and increment packet address by number of bytes transferred (4 bytes) $C1 - Send a RequestDownload request before and a RequestTransferExit request after each TransferData request, use global address in TransferData and increment packet address by number of bytes transferred (4 bytes) |

4 byte naming convention = High, MidHigh, MidLow, Low

(continued)

Pseudo Code:

XX    93    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Use KWP2000 Normal Response processing

Build Transfer Data Request as indicated below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |

If AC3 is $02 or $42

Do not include any address information in the Service Request

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 4 | 1 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 36 | Transfer Data request service ID |
| 6 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 5 + N | XX | Data byte N (N <= 254) |
| 6 + N | CS | 1 byte request checksum |

Else

If the low nibble of exception value AC3 is $0 (AC3 is $00, $40, $80 or $C0)

Set data address to address from the utility file Header

Else

If AC3 is $03 or $43

Set pointer of High byte of data address information as data packet checksum byte

Set pointer Low byte of data address information as data packet message length

Else

Set data address to Global Address

Endif

Endif

For 2 bytes for address information or for exception $03 or $43:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 4 | 1 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 36 | Transfer Data request service ID |
| 6 | XX | Data address information - High byte / data packet checksum byte |
| 7 | XX | Data address information - Low byte / data packet message length |
| 8 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 7 + N | XX | Data byte N (N <= 252) |
| 8 + N | CS | 1 byte request checksum |

Set High byte of memory address to MidLow byte of address information

Set Low byte of memory address to Low byte of address information

(continued)

For 3 bytes for address information::

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 4 | 1 + N | Length of request from byte 5 up to checksum (not included) |
| 5 | 36 | Transfer Data request service ID |
| 6 | XX | Data address information - High byte |
| 7 | XX | Data address information - Middle Byte |
| 8 | XX | Data address information - Low byte |
| 9 | XX | Data byte 1 |
|   |  | The pseudo code below explains how the data portion of this request is filled |
| 8 + N | XX | Data byte N (N <= 251) |
| 9 + N | CS | 1 byte request checksum |

Set High byte of memory address to MidHigh byte of address information

Set Middle byte of memory address to MidLow byte address information

Set Low byte of memory address to Low byte of address information

Endif

Find the calibration file with ID of AC0

Do while more calibration file to transfer

If this is the last packet of a file to transfer

Change data packet length to size of last packet

Copy the remaining data into the data area of buffer

If AC3 is $03 or $43

Calculate checksum and move as data packet checksum byte

Move packet length as data packet message length

Endif

Else

Copy the next packet of data into the data area of buffer

Endif

If AC3 is $03 or $43

Calculate checksum and move as data packet checksum byte

Move packet length as data packet message length

Endif

If exception value AC3 is greater than or equal to $80

Increment packet address by number of bytes transferred

Endif

Calculate checksum and copy into the Transfer Data Request

If AC3 is $40, $41, $42, $43, $C0 or $C1

Build Transfer Data Request as indicated below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 12 | Length of request from byte 5 up to checksum (not included) |
| 5 | 34 | Request Download Request Service ID |
| 6 | XX | Global address - High byte |
| 7 | XX | Global address - Middle byte |
| 8 | XX | Global address - Low byte |
| 9 | 00 | Data Format Identifier |
| 10 | XX | Uncompressed memory size - high byte |
| 11 | XX | Uncompressed memory size - middle byte |
| 12 | XX | Uncompressed memory size - low byte |

Note:	Increment Global address												(continued)
If remaining data is less, use number of remaining bytes for Uncompressed memory size

Calculate checksum and copy into the Request Download service request

Send the Request Download request and expect a response

If an error occurred

End the routine download

Endif

Endif

Send the Transfer Data Request and expect a response (up to 10 bytes)

If AC3 is $40, $41, $42, $43, $C0 or $C1

Build Request Transfer Exit Request as indicated below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 80 | Format byte for physical addressing |
| 2 | ## | Global target byte |
| 3 | ## | Global source byte |
| 4 | 05 | Length of request from byte 5 up to checksum (not included) |
| 5 | 37 | Request Transfer Exit Request Service ID |

Calculate checksum and copy into the Request Transfer Exit service request

Send the Request Transfer Exit request and expect a response

If an error occurred

End the routine download

Else

Increment address used in Request Download service by the number of bytes transferred

If remaining data is less than uncompressed memory size

Use number of remaining bytes in Request Download service

Endif

Endif

Endif

If an error occurred

End the calibration download

Endif

End do while more calibration to transfer

If Transfer Data positive response is received

Look for Transfer Data positive response service ID ($76) in goto fields

Else

Look for Transfer Data negative response, response code in goto fields

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers. * An FD  should be a goto field for a 93 Op-Code.)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 19 | 93 | 02 04 00 00 | 76 26 10 0D FD 20 FF 28 00 00 |

# Interpreter 3 - GMLAN Communications Programming Op-Codes

## GMLAN No Communications Processing

Supported By:

GMLAN Interpreter 3

Description:

For physical communication (one ECU) a no communication fault occurs when a request was sent to a single ECU and this ECU didn't respond (time out).

When writing instructions for a Utility File the FD code can be used to determine the next step to execute after a no comm. fault occurred.

The 'FD' processing can be used for any step in the utility file where a no comm. condition can occur.

Pseudo Code:

XX     OP  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

If no comm. occurred

Set return code to FD

Goto response code handling

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 28 | B0 | 10 01 00 00 | 76 29 FD 12 FF 4A 00 00 00 00 |

## GMLAN Response Processing

Supported By:

GMLAN Interpreter 3

Description:

If a ECU doesn't respond (timeout) then No Communication Processing applies.

If a negative response is received, then the tool searches for the response code (byte 3) of the received negative response in the goto fields.

In all other cases, the tool searches for the positive response service ID in the goto fields.

When an Op-Code refers to the response buffer the bytes are identified as follows:

Positive Response:

| Response Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | XX | Positive Response Service ID |
| n | XX | Byte n (where n <= 4095) |

Negative Response:

| Response Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 7F | Negative Response Service ID |
| 2 | XX | Request Service ID |
| 3 | XX | Response Code |
| 4 | XX | Ignored by interpreter |
| 5 | XX | Ignored by interpreter |

Pseudo Code:

XX     OP  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Get response for the request

If a timeout occurred

Goto No Communication Processing

Else

If a negative response service ID is returned (negative response service ID $7F)

If the negative response code = expected positive response service ID plus $40)

Set 7f-special process flag **(see details below)

Endif

Set return code to negative response code

Else

Set return code to positive response service ID

Endif

Goto response code handling

Endif

** Special Case 7F processing:

If the service request has a negative response code that is the same as  an expected positive response message response service ID plus $40:

The 7f-special-process flag will be set. The Interpreter shall look for the second occurrence of the response code in the goto fields to determine the next Interpreter step.

(continued)

As indicated in the example below the second occurrence of the response code can occur anywhere in the remaining goto fields (note: For clarity, it is recommended that the “duplicate response code” goto fields be on the first line and not separated as on a continuation line using the F8 continuation Op-Code).

If the response code is not found in the goto fields then the $FF goto will be used to determine the next Interpreter step.

If the service request has a negative response code that is NOT the same as an expected positive response message response service ID plus $40:

The Interpreter shall look for the FIRST occurrence of the response code in the goto fields to determine the next Interpreter step.  If the response code is not found in the goto fields then the $FF goto will be used to determine the next Interpreter step.

Developers Note:

This processing puts the requirement for the Interpreter into the hands of the developers who know what response codes are possible for a particular service request. When a developer is writing the Interpreter instructions the need for duplicate goto fields will be obvious and should not present any problems.

Negative response processing Utility File Example and Pseudo Code

Following is the message structure and utility line for pseudo code example below.

|  |  |  |  | PositiveResponse ServiceID |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | $77 |  |  |
|  | RequestMessage ServiceID |  |  |  |  |  |
|  | $37 |  |  |  |  |  |
|  |  |  |  | NegativeResponse ResponseID | RequestID | ResponseCode |
|  |  |  |  | $7F | $37 | $77 |

| Step | Op-Code | AC0 | AC1 | AC2 | AC3 | G0-G1 | G2-G3 | G4-G5 | G6-G7 | G8-G9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 37 | 00 | 00 | 00 | 00 | 77-1B | 41-17 | 72-17 | 77-18 | FF-15 |

If a PositiveResponse is received (ResponseServiceID is RequestMessageServiceID + $40).

Use the first occurrence of the PositiveResponseServiceID ($77) to determine next step.

(For this example the next step is Step 1B. )

Else a NegativeResponse was received (ResponseServiceID is $7F).

If the NegativeResponseResponseCode equals the RequestMessageServiceID + $40.

Use the second occurrence of ResponseCode ($77), or the $FF Goto field if only

one occurrence of ResponseCode ($77) is found, to determine next step.

(For this example the next step is Step 18.)

Else.

Use the ResponseCode to determine the next step.

(For Response Code $41 in this example, the next step is Step 17.)

Endif.

Endif.

## 01 Op-Code: Setup Global Variables

Supported By:

GMLAN Interpreter 3

Description:

The 01 Op-Code is used to set the global variables that are used during the programming process. The Op-Code allows the user to alter the global settings any time during the programming event by issuing another 01 Op-Code.

AC0 is used to set the Target Address. AC1 is used to set the Source Address (Tester ID).

Setting the Target Address to ‘FE’ will cause a fatal error.

If a Target Address cannot be translated into a physical CAN ID, any transmit request will return a TIMEOUT.

Action Fields:

| AC0: | Target Address |
| --- | --- |
| AC1: | Source Address |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX     01   AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

If AC0 is true

If AC0 is FE

End programming with a fatal error

Else

Set Target Address to value in AC0

Endif

Endif

If AC1 is true

Set Source Address to value in AC1

Endif

Goto step specified by goto field one (G1)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 01 | 11 F1 00 00 | 00 02 00 00 00 00 00 00 00 00 |

## 10 Op-Code: Mode 10 Initiate Diagnostic Operation

Supported By:

GMLAN Interpreter 3

Description:

The 10 Op-Code builds a standard mode 10 message. This service allows e.g. the tester to disable the setting of Diagnostic Trouble Codes (DTCs). This service is not required before requesting other services. For the activation of each level of operation one request message is required. A mode 20 or a Tester Present timeout deactivates a previous initiated diagnostic operation level.

Periodic 3E (Tester Present) service is required to keep activated levels active.

Action Fields:

| AC0: | Diagnostic Operation Level 02 Disable all DTCs 03 enableDTCsDuringDeviceControl |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX     10   AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 10 | Initiate Diagnostic Operation Request Service ID |
| 2 | AC0 | Diagnostic Operation Level |

Send message

Goto GMLAN Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 10 | 02 00 00 00 | 50 02 FF 22 00 00 00 00 00 00 |

## 14 Op-Code: Mode 04 Clear DTC's

Supported By:

GMLAN Interpreter 3

Description:

The 14 Op-Code builds a standard mode 04 message. It is used to clear stored DTC's.

This service might be used after programming to clear stored DTCs.

Action Fields:

| AC0: | 00 - Not Used |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX     14  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 04 | Clear Diagnostic Information |

Send message

Goto GMLAN Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 14 | 00 00 00 00 | 44 02 FF 22 00 00 00 00 00 00 |

## 1A Op-Code: Mode 1A Read Data by Identifier

Supported By:

GMLAN Interpreter 3

Description:

The 1A Op-Code builds a standard mode 1A message to read the content of pre-defined ECU data referenced by a Data Identifier (DID), which contains static information such as ECU identification data. The response data is stored in one of the 2-byte or 256-byte internal buffers based on the value of AC3.

Note:   This OpCode uses either the 2-byte or the 256-byte storage buffers.  For buffer usage information, see the section: “Programming Buffers – Recommended Usage”

Action Fields:

| AC0: | Data Identifier |
| --- | --- |
| AC1: | ID for storage location of response bytes (0x00 – 0x13) |
| AC2: | 00 - Not Used |
| AC3: | Exceptions: 00 – Use the 256-byte Data Buffer 01 – Use the 2-byte Data Buffer |

Pseudo Code:

XX     1A  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 1A | Read Data by Identifier Request Service ID |
| 2 | AC0 | Data Identifier |

Send message

If positive response then

If AC3 is false

Use Data Buffer (256-byte buffer)

Else

Use Byte Buffer (2-byte buffer)

Endif

Save the data of the response starting with the first Data Byte (after the echo of the Data Identifier) to storage location indicated by AC1 (2 or 256 bytes max, fill with zero)

Endif

Goto GMLAN Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 1A | C1 01 00 00 | 5A 07 FF 22 00 00 00 00 00 00 |

## 20 Op-Code: Mode 20 Return To Normal Mode

Supported By:

GMLAN Interpreter 3

Description:

The 20 Op-Code builds a standard mode 20 message to force the ECU to return to normal communication, i.e. to end the current diagnostic session. Once returned to normal communication, the ECU sends normal mode messages to and expects normal mode messages from other ECUs.

*** This Op-Code is for development only and the use of this Op-Code should be restricted. ***

Caution:	Op-Code 20 is ONLY allowed (and will be processed) in “Part 2” of a GMLAN utility file.

If an attempt is made to execute the 20 Op-Code in the “Part 1” of a GMLAN utility file, then utility file processing will immediately abort with an error (and the Mode 20 request will not be sent).

Hint:	More information about part 2 of the utility file can be found in section ‘Step $11 “Part 2” – Write the VIN’ of ‘Appendix D – GMLAN Utility File Guidelines’.

Action Fields:

| AC0: | 00 - Not Used |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX     20  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 20 | Read Data by Identifier Request Service ID |

If we are currently processing “Part 2” of the utility file

Send message

Else

Abort the utility file processing (fatal error)

Endif

Goto GMLAN Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 20 | 00 00 00 00 | 60 07 FD 12 FF 22 00 00 00 00 |

## 22 Op-Code: Mode 22 Read Data by Parameter Identifier (PID)

Supported By:

GMLAN Interpreter 3

Description:

The 22 Op-Code builds a standard mode 22 message to read the contents of pre-defined ECU data that is referenced by a Parameter Identifier (PID).  Only a single PID can be read via this Op-Code.  The response data is stored in one of the 2-byte or 256-byte internal buffers based on the value of AC3.

Note:   This OpCode uses the 2-byte or the 256-byte storage buffers.  For buffer usage information, see the section: “Programming Buffers – Recommended Usage”

Action Fields:

| AC0: | PID high byte |
| --- | --- |
| AC1: | PID low byte |
| AC2: | ID for storage location of response bytes (0x00 – 0x13) |
| AC3: | Exceptions: 00 – Use the 256-byte Data Buffer 01 – Use the 2-byte Data Buffer |

Pseudo Code:

XX     22  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 22 | Read Data by Parameter Identifier Request Service ID |
| 2 | AC0 | Parameter Identifier high byte |
| 3 | AC1 | Parameter Identifier low byte |

Send message

If positive response then

If AC3 is equal to 0x01

Use Byte Buffer (2-byte buffer)

Else

Use Data Buffer (256-byte buffer)

Endif

Save the data of the response starting with the first Data Byte (after the echo of the PID) to storage location indicated by AC2 (2 or 256 bytes max, fill with zero)

Endif

Goto GMLAN Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 22 | C1 01 05 00 | 62 07 FF 22 00 00 00 00 00 00 |

## 25 Op-Code: Mode AE Security Code

Supported By:

GMLAN Interpreter 3

Description:

The 25 Op-Code will build a special Mode $AE message. This service allows a test device to enter a security code, reset a security code or program a security code as designed for GM Global A vehicles. For the Service Programming System (SPS), the value of the security code(s) will be obtained from the service database based on the VIN.  For the Development Programming System (DPS), the value of the security code(s) will be manually entered by the user.

Periodic 3E (Tester Present) service is required to keep the level active.

Action Fields:

| AC0: | CPID number |
| --- | --- |
| AC1: | Device Control: 80 – Enter (e.g. unlock ECU) 40 – Program security code (e.g. write security code) 20 – Reset security code (e.g. write security code default value) |
| AC2: | 01 – Security Code 1 (Security Code 1 refers to the security code assigned to the vehicle) 02 – Security Code 2 (Security Code 2 is used to unlock an ECU that has a different security code than the vehicle) **Vehicle Assembly Plant ignores this action code** |
| AC3: | 01 – Security Code Format – ASCII (default) **Vehicle Assembly Plant ignores this action code** |

Pseudo Code:

XX     AE  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AE | Request Device Control |
| 2 | AC0 | CPID Number |
| 3 | AC1 | Security Access Device Control Trigger |
| 4 | SC | Security Code (MSB) SPS = VIT2 / DPS = manual entry |
| 5 | SC | Security Code SPS = VIT2 / DPS = manual entry |
| 6 | SC | Security Code SPS = VIT2 / DPS = manual entry |
| 7 | SC | Security Code (LSB) SPS = VIT2 / DPS = manual entry |

If AC1 equals 20, 40 or 80

If AC2 equals 01

Set the SecurityCode to the “Security Code 1”

ElseIf AC2 equals 02

Set the SecurityCode to the “Security Code 2”

Endif

Copy SecurityCode (ASCII) into message bytes 4 – 7

Else

End with error

Endif

Send message

Goto GMLAN Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 25 | 01 80 01 01 | EE 11 E3 0D FD 20 FF 2B 00 00 |

## 27 Op-Code: Mode 27 Security Access

Supported By:

GMLAN Interpreter 3

Description:

The 27 Op-Code is used to open an ECU's memory for reprogramming. For 2 byte Unlocking, the value in AC0 identifies which security algorithm to use for unlocking the ECU for programming. The security algorithm defines the functions that are performed on the seed to determine the ECU key.  For 5 byte Unlocking, the value in AC0 identifies the particular Key Table that was assigned to a specific ECU (eg. Delphi E38 ECM).

If security fails for "Invalid Key - 35" the Interpreter will retry the entire security process (starting with request 1) using the ones complement security algorithm. This provides a means to program development ECU's using a production Utility File.

Periodic 3E (Tester Present) service is required to keep the level active.

Note: (For 2 byte Unlocking) To allow for more than 256 unique GM Security Alogorithms, a second “Security Table” (GMLAN-1) has been created (see the high nibble of AC2).  All previously existing GM Security Algorithms will be referenced to the first “Security Table” (GMLAN-0).  So now for new GM Security Algorithms, the user needs to know the algorithm number as well as the corresponding GMLAN “Security Table” number (either 0 or 1).

(For 5 byte Unlocking) A value of F0 in AC2 denotes a 5 byte unlocking design.

Action Fields:

| AC0: | Security Algorithm 2 Byte Unlocking: 00 - Ones Complement (Default Security Algorithm) 01 - to FE Production Algorithm (Production Security Algorithms) (assigned by GM) 5 byte Unlocking: 00 - Ones Complement response to be used. 01 - to FE ECU Key Table reference (assigned by GM) |
| --- | --- |
| AC1: | 00 - Security level 1 (level $01 and $02 – get seed/send key) 01 - Security level 3 ( DVT level $03 and $04 – DVT get seed/send key) XX – Security level (2*XX + 1) Example : $7D = security level $FB ($FB and $FC – get seed/send key) |
| AC2: | 00 – GM Security Algorithm, Security Key Calculation References the original GMLAN Algorithms – Security Table GMLAN-0 (2 byte) 01 – Not implemented: GM Security Algorithm, Security Key Calculation in other source 02 – Not implemented: Non GM Security Algorithm, Security Key Calculation in other source 03 – Non GM Security Algorithm, Security Key Calculation 10 – GM Security Algorithm, Security Key Calculation References the new GMLAN Algorithms – Security Table GMLAN-1 (2 byte) F0 – GM Secure Algorithm, Security Key Calculation Denotes 5 byte Unlocking ECU (* See caution below.) |
| AC3: | Exceptions: 00 – Don't send key if seed is 0 (according to specification) 01 – Send key even if seed is 0 |

* Caution: If AC2 is F0, then the positive ‘Request Seed’ response MUST contain a 5 byte seed; for all GM Security Algorithm values, a 2 byte seed is required.  If the length of the seed received is not correct then utility file processing will immediately abort with an error.

Pseudo Code:

XX   27    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

(continued)

Build Security Access Request 1 as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 27 | Security Access Request Service ID |
| 2 | XX | AC1 (1) Access mode – if AC1 = 0 (service), this byte would be 1, if AC1 = 1 (DVT), this byte would be 3, etc. |

Send Security Access Request 1 for request seed and receive response

If Security Access positive response 1 is received

If AC2 is F0 and only 2 seed response data bytes --  OR  -- AC2 is not F0 and 5 seed response data bytes

Abort with an error

If all bytes of the seed (Positive response – seed data bytes) are zeros and the exception isn't 01

Indicate Security Access positive response received (do not send Security Access Request 2)

Else

2 byte Unlocking:

Use security algorithm corresponding to AC0 to determine ECU's key

(For GMLAN Security Algorithms, the High Nibble of AC2 will also be evaluated to determine

which GMLAN Security Table to reference --- Table 0 (GMLAN-0) or Table 1 (GMLAN-1).)

Build Security Access request 2 as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 27 | Security Access Request Service ID |
| 2 | XX | AC1 (2) Access mode – if AC1 = 0, this byte would be 2, if AC1 = 1 this byte would be 4, etc. |
| 3 | XX | High byte of key |
| 4 | XX | Low byte of key |

Send Security Access Request 2 for send key and receive response

5 byte Unlocking:

Use ECU Key Table corresponding to AC0 to determine ECU's key

Build Security Access request 2 as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 27 | Security Access Request Service ID |
| 2 | XX | AC1 (2) Access mode – if AC1 = 0 (service), this byte would be 2, if AC1 = 1 (DVT), this byte would be 4, etc. |
| 3 | XX | Key Byte 1 - MSB |
| 4 | XX | Key Byte 2 |
| 5 | XX | Key Byte 3 |
| 6 | XX | Key Byte 4 |
| 7 | XX | Key Byte 5 - LSB |

Send Security Access Request 2 for send key and receive response

Endif

If Security Access positive response 2 is received

Look for Security Access positive response 2 service ID (67) in goto fields

Else

If Security Access negative response "Invalid Key - 35" is received & ones complement was not used

Repeat entire security process using ones complement security algorithm

Else

Look for Security Access negative response 2 response code in goto fields

Endif

Endif

(continued)

Else

If Security Access negative response "Time Delay not Expired - $37" is received

If 5 retry attempts have not been performed (Time Delay not Expired)

Delay 2 seconds

Retry entire security process

Else

Look for Security Access negative response AC3 (1) response code ($37) in goto fields

Endif

Else

Look for Security Access negative response AC3 (1) response code in goto fields

Endif

Endif

If found

Goto appropriate step

Else

Goto step indicated by FF goto field

Endif

Typical Interpreter Line: (all action and goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 02 | 27 | 01 00 00 00 | 67 03 37 13 FF 22 00 00 00 00 |

## 34 Op-Code: Mode 34 Request Download

Supported By:

GMLAN Interpreter 3

Description:

The 34 Op-Code will build a standard mode 34 message. This mode is used in order to prepare a node to receive a download of a block of data.

The node must be unlocked, mode 28 (Disable Normal Communication) must be active and mode A5 (Programming Mode) must be passed prior to sending a mode 34 request.

Periodic 3E (Tester Present) service is required to keep the level active.

Action Fields:

| AC0: | Data Format Identifier 00 - Unencrypted, uncompressed 01 - to FF High nibble Compression Method, low nibble Encrypting Method |
| --- | --- |
| AC1: | Routine / Calibration Number |
| AC2: | 00 - Not Used |
| AC3: | Exceptions: (Information is nibble coded) 0X - Use length from the routine indicated by AC1 (2 bytes) 1X - Use length from the utility file header (2 bytes) 2X - Use Global Length (see F2 Op-Code) (4 bytes) 3X - Use length based on the size of the calibration file indicated by AC1 (4 bytes) X0 - Use TypeOfAddressing bytes for length information X2 - Use 2 bytes for length information X3 - Use 3 bytes for length information X4 - Use 4 bytes for length information |

Pseudo Code:

XX     34  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

For 2 bytes for length information:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 34 | Request Download Request Service ID |
| 2 | AC0 | Data Format Identifier |
| 3 | XX | Uncompressed Memory Size (High byte) |
| 4 | XX | Uncompressed Memory Size (Low byte) |

For 3 bytes for length information:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 34 | Request Download Request Service ID |
| 2 | AC0 | Data Format Identifier |
| 3 | XX | Uncompressed Memory Size (High byte) |
| 4 | XX | Uncompressed Memory Size (Middle byte) |
| 5 | XX | Uncompressed Memory Size (Low byte) |

For 4 bytes for length information:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 34 | Request Download Request Service ID |
| 2 | AC0 | Data Format Identifier |
| 3 | XX | Uncompressed Memory Size (High byte) |
| 4 | XX | Uncompressed Memory Size (MidHigh byte) |
| 5 | XX | Uncompressed Memory Size (MidLow byte) |
| 6 | XX | Uncompressed Memory Size (Low byte) |

(continued)

If the high nibble of AC3 is 0 (‘0X’)

Find the routine indicated by value AC1

Set MemorySize to routine length

ElseIf the high nibble of AC3 is 1 (‘1X’)

Set MemorySize to length from the utility file Header

ElseIf the high nibble of AC3 is 2 (‘2X)

Set MemorySize to Global Length (default ‘2X’)

ElseIf the high nibble of AC3 is 3 (‘3X’)

Find the calibration file indicated by value AC1

Set MemorySize to the size of the calibration file

Else

End with error (invalid high nibble of AC3)

Endif

If AC3 & 0F is true

Set LengthSize to AC3 & 0F

Else

Set LengthSize to TypeOfAddressing

Endif

if LengthSize is 2

Copy High byte of MemorySize into message byte 3

Copy Low byte of MemorySize into message byte 4

Elseif LengthSize is 3

Copy High byte of MemorySize into message byte 3

Copy Middle byte of MemorySize into message byte 4

Copy Low byte of MemorySize into message byte 5

Elseif LengthSize is 4

Copy High byte of MemorySize into message byte 3

Copy MidHigh byte of MemorySize into message byte 4

Copy MidLow byte of MemorySize into message byte 5

Copy Low byte of MemorySize into message byte 6

Endif

Send message

Goto GMLAN Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 34 | 10 00 00 20 | 74 11 FF 22 00 00 00 00 00 00 |

## 3B Op-Code: Mode 3B Write Data by Identifier

Supported By:

GMLAN Interpreter 3

Description:

The 3B Op-Code will build a standard mode 3B message. This service is used to provide the ability to change (write/program) the content of pre-defined ECU data referenced by a Data Identifier (DID), which contains static data like ECU identification data or other information, which does not require “real-time” updates. If multiple DIDs are written via a calibration file the GlobalHeaderLength is used to skip header information.

Action Fields:

| AC0: | Data Identifier (DID number) 90 – VehicleIdentificationNumber 98 – RepairShopCodeOrTesterSerial 99 – ProgrammingDate CB – End Model Part Number XX - ECU Configuration Data |
| --- | --- |
| AC1: | Routine Number / ID for saved bytes (0x00 – 0x13) / Internal Data Specifier (if AC3 is 0x03) (See Appendix J – Plant Manufacturing VIT Data) |
| AC2: | - Calibration ID (if AC3 is 22 or if AC3 is 20) - Number of bytes (if AC3 is 30) - Data Specifier Indicator Value (if AC3 = 03) (See Appendix J – Plant Manufacturing VIT Data) |
| AC3: | Exceptions: 00 – Use internal data from the VIT based on the AC0 Data Identifier (see above) Use AC0 as the Data Identifier (DID number) to write to. 03 – Use internal data from the VIT based on the AC1 and AC2 (‘Internal Data Specifier’ and ‘Data Specifier Indicator Value’ respectively) Use AC0 as the Data Identifier (DID number) to write to. This exception is ONLY supported by the Assembly Plants. If used in the “service” environment, a fatal error will occur. 10 – Use data from the routine indicated by AC1 Use AC0 as the Data Identifier (DID number) to write to. 11 – Use data from the routine indicated by AC1 Use the first byte of the routine data as the Data Identifier (DID number) to write to. 20 – Use data from the calibration file indicated by AC2 (single DID write). Use AC0 as the Data Identifier (DID number) to write to. The data in the calibration file contains the raw data that will get written to the DID location (no extra bytes or header). Maximum number of data bytes is 4093. 22 – Use data from the calibration file indicated by AC2 (multiple DIDs are possible) The Data Identifier(s) is defined within the calibration file. (see DID calibration file format) 3x – Use stored information where AC1 is ID for saved bytes, only 0x00 - 0x13 IDs are supported. Buffer size 256 bytes (‘x’ – don’t care what the low nibble value is) Use AC0 as the Data Identifier (DID number) to write to. 4x – Use VIN digits 10-17 retrieved from internal data. (‘x’ – don’t care what the low nibble value is) Use AC0 as the Data Identifier (DID number) to write to. |

Example of DID Calibration file:	(only if AC3 = 22)

88880002220000F424004141020784226552130000078501E200000000  (must be in binary format)

Module Checksum 	= 8888

Module ID		= 0002

Header Format ID		= 2200

SWMI		= 00 F4 24 00  (16000000)

Alpha Code		= 41 41  (AA)

Block count		= 02

Block 1 Length		= 07

Block 1		= 84 22 65 52 13 00 00

Block 2 Length		= 07

Block 2		= 85 01 E2 00 00 00 00

Pseudo Code:

XX     3B  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 3B | Write Data by Identifier |
| 2 | XX | Data Identifier |
| 3 | XX | Data Byte 1 |
|  | XX | Data Byte ... |

If AC2 is not zero and AC3 is 22

Open calibration AC2

Skip GlobalHeaderLength bytes

Read BlockCount

While BlockCount > 0

Read BlockLength

Copy BlockLength bytes of calibration AC2 into message (first byte is Data Identifier)

Send message

If not a positive response is received

Break while

Endif

Subtract 1 from BlockCount

Loop

Endif

If AC2 is not zero and AC3 is 20

Open calibration AC2

Determine DID data length (the file size must not exceed 4093 bytes)

Build $3B message (DID = AC0, data bytes from calibration file)

Send message

Endif

If high nibble of AC3 is 0

Set Data Identifier to AC0

If the low nibble of AC3 is 0

Copy internal VIT data based on AC0 into the message

Send message

Else (the low nibble of AC3 is 3

(ONLY supported for the Plant Manufacturing environment)

Copy internal VIT data based on AC1 (Internal Data Specifier) into the message

Send message

Endif

Endif

(continued)

If high nibble of AC3 is 1

Find the routine indicated by value AC1

If the low nibble of AC3 is 1

Set Data Identifier to first routine data byte

Copy remaining routine data bytes into the message

Send message

Else (the low nibble of AC3 is 0)

Set Data Identifier to AC0

Copy routine data into message

Send message

Endif

Endif

If high nibble of AC3 is 3

Set Data Identifier to AC0

Copy AC2 number of bytes from stored information where AC1 is the ID for saved bytes into the message

(max 256 bytes)

Send message

Endif

If high nibble of AC3 is 4

Set Data Identifier to AC0

Place the last 8 digits of the VIN (internal data) into the message

(Note:  If the last 8 digits are 0x00 data, then simulate a positive response without sending the message)

Endif

If a valid AC3 was processed

GoTo GMLAN Response Processing

Else

End with fatal error

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 3B | 90 00 00 00 | 7B 22 FF 32 00 00 00 00 00 00 |

## 84 Op-Code: Set Communications Parameters

Supported By:

GMLAN Interpreter 3

Description:

The 84 Op-Code is used to set the communications parameters. Currently, the only parameter, which can be set, is the STmin. This is the minimum interframe time for the tester normally dictated by a controller within its flow control frame. Op-code 84 allows to override the value in the controller’s flow control.

NOTE: GM NAO assembly plants will currently not support this Op-Code for the GMLAN protocol. If this Op-Code is encountered in the assembly plant utility file, the G1 “Go To” will be processed.

Action Fields:

| AC0: | 00 - use STmin value in flow control 01 - use STmin value in AC2 |
| --- | --- |
| AC1: | Affected subnet: 01 - SWCAN, 02 - MSCAN, 04 – HSCAN, 08 – ESCAN (can be logically OR-ed) |
| AC2: | STmin value, resolution 1 ms |
| AC3: | 00 - Not Used |

Pseudo Code:

XX    84    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

If AC0 == 0

Use STmin from controller’s flow control frame (default behavior) for the subnets specified in AC1

Else If AC0 == 1

Use STmin specified by AC2 for the subnets specified in AC1

Else

Stop immediately

Endif

If setting STmin succeeded

Goto step specified in goto field 1 (G1).

Else

Goto step specified in goto field 3 (G3).

Endif

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 84 | 01 07 02 00 | 00 02 FF 22 00 00 00 00 00 00 |

## A2 Op-Code: Mode A2 Report Programmed State and Save Response

Supported By:

GMLAN Interpreter 3

Description:

The A2 Op-Code will build a standard mode A2 message. This service is used to determine the current programmed state of the ECU.

Mode 28 (Disable Normal Communication) must be active prior to sending a mode A2 request.

Periodic 3E (Tester Present) service is required to keep the level active.

Note:   This OpCode uses the 2-byte storage buffers.  For buffer usage information, see the section: “Programming Buffers – Recommended Usage”

Action Fields:

| AC0: | ID for storage location of response bytes (0x00 – 0x13) Buffer size is 2-bytes |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX     A2  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | A2 | ReportProgrammedState |

Send message

If positive response then

Save byte2 from the response to storage location indicated by AC0 (use zero as high byte)

Endif

Goto GMLAN Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | A2 | 01 00 00 00 | E2 07 FF 22 00 00 00 00 00 00 |

## AA Op-Code: Mode AA Read Data by Packet Identifier

Supported By:

GMLAN Interpreter 3

Description:

The AA Op-Code builds a standard mode AA message to read the content of pre-defined ECU data referenced by a Dynamic Parameter Identifier (DPID). Only 1 DPID can be requested using sub-function $01 (single shot). The response is a UUDT frame not containing length information. The received data is stored in an internal buffer (one of 256- byte internal buffers) or in the VIT2.

NOTEs:

- Some diagnostic interfaces may not support this function as mixed mode support is a SAE J2534-2 optional feature.

- This OpCode should not be used in part 1 of a GMLAN utility file on an ECU which may not be able to support the $AA service until fully programmed.  Examples of this condition may be programming out of boot mode and attempting to program certain SPS Type B and Type C ECUs.

- This OpCode uses the 256-byte storage buffers.  For buffer usage information, see the section: “Programming Buffers – Recommended Usage”

Action Fields:

| AC0: | DPID (Dynamic Parameter Identifier) |
| --- | --- |
| AC1: | ID for storage location of response bytes 0x00..0x13 – 256-byte Data Buffers (0..19 decimal) 90 – VIN in VIT2 91 – Vehicle manufacturer hardware number (VMECUHN) in VIT2 98 – Repair shop code or serial number (RSCOSN) in VIT2 99 – Programming date (PD) in VIT2 CB – End model number (EMN) in VIT2 CC – System supplier hardware number (SSECUHN) in VIT2 |
| AC2: | Number of bytes to store |
| AC3: | Exceptions: Bit 7: 0 – Copy directly 1 – Convert into decimal number ASCII string Bit 0..6 – Start location of bytes to copy (0 refers to packet identifier) |

Pseudo Code:

XX     AA  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AA | Read Data by Identifier Request Service ID |
| 2 | 01 | Mode (single shot) |
| 2 | AC0 | Dynamic Packet Identifier |

Send message

(continued)

If response then

If AC2 specifies more bytes to copy than available from start position (AC3 $ $7F) to end

Set number of bytes to copy to number of available bytes

Else

Set number of bytes to copy to AC2

Endif

If AC3 & $80

If AC2 <= 4

Treat data as a 1..4 byte number, convert into a string as decimal to temporary buffer

Set number of bytes to copy to string length of string in temporary buffer

Else

Copy specified number of bytes into temporary buffer

Endif

If AC1 is in range $00..$13

Copy temporary buffer to one of the 256-byte Data Buffers (AC1 = index)

Else if AC1 is $90

Copy temporary buffer to VIN in VIT2

Else if AC1 is $91

Copy temporary buffer to Vehicle Manufacturer ECU Hardware Number in VIT2

Else if AC1 is $98

Copy temporary buffer to Repair Shop Code Or Serial Number in VIT2

Else if AC1 is $99

Copy temporary buffer to Programming Date in VIT2

Else if AC1 is $CB

Copy temporary buffer to End Model Number in VIT2

Else if AC1 is $CC

Copy temporary buffer to System Supplier ECU Hardware Number in VIT2

EndIf

Endif

Goto GMLAN Response Processing

Since the response is a UUDT frame, the SID position is occupied by the DPID. Hence, the fields G0, G2, etc. shall contain the DPID (see AC1) and NOT the expected response SID $EA to catch the positive case.

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | AA | 09 99 03 03 | 09 07 FF 22 00 00 00 00 00 00 |

## AE Op-Code: Mode AE Request Device Control

Supported By:

GMLAN Interpreter 3

Description:

The AE Op-Code will build a standard mode AE message. This service e.g. allows a test device to override normal output control functions in order to verify proper operation of a component or system, or to reset/clear variables used within normal control algorithms.

Periodic 3E (Tester Present) service is required to keep the level active.

Action Fields:

| AC0: | CPID Number |
| --- | --- |
| AC1: | Routine Number |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX     AE  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | AE | Request Device Control |
| 2 | AC0 | CPID Number |
|   |  | Control Bytes |
| 2 + N | XX | Control Byte N (N from 0 to 5) |

Find the routine indicated by value AC1

Copy routine data into message (number of control bytes from 0 to 5 bytes max)

Send message

Goto GMLAN Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | AE | FE 02 00 00 | EE 11 E3 0D FF 20 00 00 00 00 |

## B0 Op-Code: Mode 36 Block Transfer to RAM

Supported By:

GMLAN Interpreter 3

Description:

The B0 Op-Code will build a standard mode 36 message. This Op-Code is used to transfer data to a control module.

A single data transfer message is limited to a maximum of 4095 data bytes (including the SID, Identifier Option and 2, 3 or 4 Address Bytes -> 4091, 4090 or 4089 data bytes).

The node must be unlocked, mode 28 (Disable Normal Communication) must be active, mode A5 (Programming Mode) must be passed and mode 34 must be positive answered prior to sending a mode 36 request.

Periodic 3E (Tester Present) service is required to keep the level active.

The maximal number of bytes to send with each message is retrieved from the Utility File header.

If the GlobalHeaderLength is not zero then the download of the module will be separated in two parts. The GlobalHeaderLength number of bytes will be downloaded first.

Note:  Utility files for Global A GMLAN ECUs that employ the Global A Secure Bootloader design (digitally signed OpSoftware and Calibration files) require special negative response handling - - - Reading of the ProgrammingError Code DID.  Refer to the “Additional Utility File Information – Best Practices and recommendations” section (Item #17).

Action Fields:

| AC0: | Calibration ID 00 - Not Used 01 - FF Calibration ID |
| --- | --- |
| AC1: | Routine Number |
| AC2: | Exceptions: (Information is nibble coded) 0X – Download 1X - Download and Execute 2X - Execute Only (do not include data into message) X0 - Increment address X1 - Keep address constant |
| AC3: | Exceptions: (Information is nibble coded) 0X - Use address from the routine indicated by AC1 (4 bytes) 1X - Use address from the utility file header (4 bytes) 2X - Use Global Address (see F1 Op-Code) (4 bytes) X0 - Use TypeOfAddressing bytes for address information X2 - Use 2 bytes for address information X3 - Use 3 bytes for address information X4 - Use 4 bytes for address information |

4 byte naming convention = High, MidHigh, MidLow, Low

Pseudo Code:

XX     B0  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

(continued)

For 2 bytes for address information:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 36 | Transfer Data |
| 2 | XX | Level of Operation 00 – Download 80 - Download and Execute |
| 3 | XX | Starting Address (High byte) |
| 4 | XX | Starting Address (Low byte) |
| 5 | XX | Data Byte 1 |
|  |  | Data Byte ... |

For 3 bytes for address information:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 36 | Transfer Data |
| 2 | XX | Level of Operation 00 – Download 80 - Download and Execute |
| 3 | XX | Starting Address (High byte) |
| 4 | XX | Starting Address (Mid byte) |
| 5 | XX | Starting Address (Low byte) |
| 6 | XX | Data Byte 1 |
|  |  | Data Byte ... |

For 4 bytes for address information:

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 36 | Transfer Data |
| 2 | XX | Level of Operation 00 – Download 80 - Download and Execute |
| 3 | XX | Starting Address (High byte) |
| 4 | XX | Starting Address (MidHigh byte) |
| 5 | XX | Starting Address (MidLow byte) |
| 6 | XX | Starting Address (Low byte) |
| 7 | XX | Data Byte 1 |
|  |  | Data Byte ... |

If AC2 & 30 is true

Set Level of Operation to 80 (Level of Operation = Download and Execute)

Else

Set Level of Operation to 00 (Level of Operation = Download)

Endif

If AC3 & 30 is false

Find the routine indicated by value AC1

Set StartingAddress to routine address

Else

If AC3 & 10 is true

Set StartingAddress to address from the utility file Header

Else

Set StartingAddress to Global Address

Endif

Endif

If AC3 & 0F is true

Set LengthSize to AC3 & 0F

Else

Set LengthSize to TypeOfAddressing

Endif

If AC2 & 20 is false (Not Execute Only)															(continued)

if AC0 (Calibration ID) is false

Use data from the routine indicated by value AC1

Else

Use data from the calibration file indicated by value AC0

Endif

Do while more data to download

if LengthSize is 2

Copy High byte of StartingAddress into message byte 3

Copy Low byte of StartingAddress into message byte 4

Elseif LengthSize is 3

Copy High byte of StartingAddress into message byte 3

Copy Middle byte of StartingAddress into message byte 4

Copy Low byte of StartingAddress into message byte 5

Elseif LengthSize is 4

Copy High byte of StartingAddress into message byte 3

Copy MidHigh byte of StartingAddress into message byte 4

Copy MidLow byte of StartingAddress into message byte 5

Copy Low byte of StartingAddress into message byte 6

Endif

If this is the last packet to download or the GlobalHeaderLength is not 0 and splits the module

Copy the remaining data into the message

Else

Copy the next block of data into the message

Endif

If AC2 & 0F is false

Calculate next download address

end if

Send message

If negative response received

Break do while more data to download

Endif

End do while more data to download

Else (Execute Only)

if LengthSize is 2

Copy High byte of StartingAddress into message byte 3

Copy Low byte of StartingAddress into message byte 4

Elseif LengthSize is 3

Copy High byte of StartingAddress into message byte 3

Copy Middle byte of StartingAddress into message byte 4

Copy Low byte of StartingAddress into message byte 5

Elseif LengthSize is 4

Copy High byte of StartingAddress into message byte 3

Copy MidHigh byte of StartingAddress into message byte 4

Copy MidLow byte of StartingAddress into message byte 5

Copy Low byte of StartingAddress into message byte 6

Endif

Send message

Endif

Goto GMLAN Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | B0 | 10 00 01 10 | 76 11 85 0D FD 20 FF 49 00 00 |

# Interpreter 4 – UDS Communications Programming Op-Codes

## UDS No Communications Processing

Supported By:

UDS Interpreter 4

Description:

For physical communication (one ECU), a no communication fault occurs when a request was sent to a single ECU and this ECU did not respond (timed out).  In addition, a no communication fault can also occur when a request is sent out with the ‘suppress positive response message indicator’ is set (true) – refer to specific Op-Code usages.

When writing instructions for a Utility File the FD code can be used to determine the next step to execute after a no comm. fault occurred.

The 'FD' processing can be used for any step in the utility file where a no comm. condition can occur.

Pseudo Code:

If no comm. fault occurred

Set return code to FD

Goto response code handling

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 22 | 36 | 02 00 00 00 | 76 29 FD 12 FF 4A 00 00 00 00 |

## UDS Response Processing

Supported By:

UDS Interpreter 4

Description:

If a ECU doesn't respond (timeout) then No Communication Processing applies.

If a negative response is received, then the tool searches for the response code of the received negative response in the goto fields.

In all other cases, the tool searches for the positive response service ID in the goto fields.

When an Op-Code refers to the response buffer the bytes are identified as follows:

Positive Response:

| Response Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | XX | Positive Response Service ID |
| n | XX | Byte n (where n <= 4095) |

Negative Response:

| Response Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 7F | Negative Response Service ID |
| 2 | XX | Request Service ID |
| 3 | XX | Response Code |
| 4 | XX | Ignored by interpreter |
| 5 | XX | Ignored by interpreter |

Pseudo Code:

Get response for the request

If a timeout occurred

Goto No Communication Processing

Else

If a negative response service ID is returned (negative response service ID $7F)

Set return code to negative response code

Else

Set return code to positive response service ID

Endif

Goto response code handling

Endif

| Step | Op-Code | AC0 | AC1 | AC2 | AC3 | G0-G1 | G2-G3 | G4-G5 | G6-G7 | G8-G9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 22 | F1 | 90 | 0A | 00 | 62 13 | 22 1F | 33 35 | FD 35 | FF 44 |

## 01 Op-Code: Setup Global Variables

Supported By:

UDS Interpreter 4

Description:

The 01 Op-Code is used to set the global variables that are used during the programming process. The Op-Code allows the user to alter the global settings any time during the programming event by issuing another 01 Op-Code.

AC0 is used to set the Target Address. AC1 is used to set the Source Address (Tester ID).

Note:  If a Target Address cannot be translated into a physical CAN ID, any transmit request will return a TIMEOUT.

Action Fields:

| AC0: | Target Address |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Note:   Default Source ID defined within the application.

Pseudo Code:

XX     01   AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

If AC0 is true

Set Target Address to value in AC0

Endif

Set Source Address based on application (internal)

Goto step specified by goto field one (G1)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 01 | 11 F1 00 00 | 00 02 00 00 00 00 00 00 00 00 |

## 10 Op-Code: Mode 10 Diagnostic Session Control

Supported By:

UDS Interpreter 4

Description:

The 10 Op-Code builds a standard mode 10 message. This service allows the tester to request the specific diagnostic session.  For the activation of each level of operation, one request message is required.  A Periodic $3E (Tester Present) service is required to keep the current session control active (ex. Programming Session).  A Tester Present timeout will deactivate a previously initiated session control level.  A Mode $11 EcuReset will deactivate a Programming Session Control.

Action Fields:

| AC0: | Diagnostic Operation Level 02 - Programming Session |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX     10   AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 10 | Initiate Diagnostic Operation Request Service ID |
| 2 | AC0 | Diagnostic Operation Level |

If AC0 is valid (02)

Send message

Else

Abort the utility file processing (fatal error)

Endif

Goto UDS Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 02 | 10 | 02 00 00 00 | 50 03 22 2D FF 2F 00 00 00 00 |

## 11 Op-Code: Mode 11 ECU Reset

Supported By:

UDS Interpreter 4

Description:

The 11 Op-Code builds a standard Mode 11 message.  An ECU Reset is used by the Tester to request a specific ECU reset.  The type of ‘reset’ performed by the ECU is depend upon the ‘reset type’ (parameter). After an ECU reset, the ECU is expected to return to an active ‘default session’ state.

*** The use of this Op-Code should be restricted. ***

Caution:	Op-Code 11 is ONLY allowed (and will be processed) in “Part 2” of a GM UDS utility file.

If an attempt is made to execute the 11 Op-Code in the “Part 1” of a UDS utility file, then utility file processing will immediately abort with an error (and the Mode 11 request will not be sent).

Action Fields:

| AC0: | ECU Reset Type 01 – hardReset |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX     11  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 11 | ECU Reset Service ID |
| 2 | AC0 | ECU Reset Type |

If we are currently processing “Part 2” of the utility file  and  AC0 is valid (01)

Send message

Else

Abort the utility file processing (fatal error)

Endif

Goto UDS Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 11 | 01 00 00 00 | 41 07 FD 12 FF 22 00 00 00 00 |

## 22 Op-Code: Mode 22 Read Data by Identifier

Supported By:

UDS Interpreter 4

Description:

The 22 Op-Code builds a standard mode 22 message to read the content of pre-defined ECU data referenced by a 2-byte Data Identifier value (DID).  Only a single DID can be read via this Op-Code.  The response data can be stored into a 256 byte internal storage buffer based on the value of AC2.

Action Fields:

| AC0: | Data Identifier (MSB) |
| --- | --- |
| AC1: | Data Identifier (LSB) |
| AC2: | ID for storage location of response bytes (0x00 – 0x13) [ 256-byte storage buffer ] |
| AC3: | 00 – Not used |

Pseudo Code:

XX     22  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 22 | Read Data by Identifier Request Service ID |
| 2 | AC0 | Data Identifier (MSB) |
| 3 | AC1 | Data Identifier (LSB) |

Send message		(Only Single DID Read)

If positive response

If ‘valid buffer storage location’ (AC2)

Save the data of the response starting with the first Data Byte (after the echo of the DID) to storage location indicated by AC2 (fill with zero)

Else

Abort the utility file processing (fatal error)

Endif

Endif

Goto UDS Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 22 | F1 99 05 00 | 62 07 31 0A FF 22 00 00 00 00 |

## 27 Op-Code: Mode 27 Security Access

Supported By:

UDS Interpreter 4

Description:

The 27 Op-Code is used to open (unlock) an ECU's memory for reprogramming.

Periodic 3E (Tester Present) service is required to keep the security level active.

Action Fields:

| AC0: | 00 – Not Used |
| --- | --- |
| AC1: | 00 – Not Used |
| AC2: | Security Algorithm Type: 03..07 – Non-GM Security Algorithms, not to be used! 20 – Non-GM Security Algorithm, Security Key Calculation (PATAC) B0 – GM UDS Cyber Security (Secure Server) (See *Caution below) |
| AC3: | Exceptions: 00 – Not Used |

* Caution: If AC2 is B0, then the positive ‘Request Seed’ response MUST contain 31 data bytes.

Pseudo Code:

XX   27    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build Security Access ‘requestSeed’ as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 27 | Security Access Request Service ID |
| 2 | XX | Security Access Type (Access Level) Derived from calling application. “Service” – (01); “Manufacturing (DVT)” – (03); etc… |

Send Security Access ‘requestSeed’ message and receive response

If a positive response is received

If AC2 is B0 and a 31 byte seed data response is not received

Abort with an error

If all bytes of the seed are 0x00

Unlocked Indicator:  (do not need to send the Security Access sendKey request)

Else

If all bytes of the seed are 0xFF

Locked Indicator / Valid Key Not Required:  Set KEY value (any 12 byte value)

Else

Calculate Key (Secure Server) based on Seed (assume success)

Endif

(continued)

Build Security Access sendKey request as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 27 | Security Access Request Service ID |
| 2 | XX | Security Access Type (Access Level) Derived from calling application. “Service” – (02); “Manufacturing (DVT)” – (04); etc… |
| 3 | XX | Key Byte 1 - MSB |
| 4 | XX | Key Byte 2 |
| … | … | … |
| 13 | XX | Key Byte 11 |
| 14 | XX | Key Byte 12 - LSB |

Send Security Access Request sendKey message and receive response

Endif

Goto UDS Response Processing

Else (non-positive requestSeed response)

If Security Access negative response "Time Delay not Expired - $37" is received

If 5 retry attempts have not been performed (Time Delay not Expired)

Delay 2 seconds

Retry entire security process

Else

Goto UDS Response Processing

Endif

Else

Goto UDS Response Processing

Endif

Endif

Typical Interpreter Line: (all action and goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 02 | 27 | 00 00 B0 00 | 67 03 37 13 FF 22 00 00 00 00 |

## 2E Op-Code: Mode 2E Write Data by Identifier

Supported By:

UDS Interpreter 4

Description:

The 2E Op-Code will build a standard mode 2E message. This service is used to provide the ability to change (write/program) the content of pre-defined ECU data referenced by a Data Identifier (DID), which contains static data like ECU identification data or other information which does not require “real-time” updates.

Action Fields:

| AC0: | Data Identifier (MSB) |
| --- | --- |
| AC1: | Data Identifier (LSB) |
|  | DIDs (AC0 & AC1): 0xF190 – VehicleIdentification Number 0xF198 – RepairShopCodeOrSerialNumber 0xF199 – Programming Date 0x???? – ECU Configuration Data |
| AC2: | Source Specifier - Calibration File ID (if AC3 is 0x20) - Utility File Routine Number (if AC3 is 0x10 or 0x11) - Internal VIT data specifier (if AC3 is 0x00 or 0x03) |
| AC3: | Exceptions: 00 – Use internal data from the VIT based on the AC0 & AC1 Data Identifier (DID) Use AC0 & AC1 as the Data Identifier (DID number) to write to. 03 – Use internal data from the VIT based on the AC0 & AC1 Internal Data Specifier Use AC0 & AC1 as the Data Identifier (DID number) to write to. This exception is ONLY supported by the Assembly Plants. If used in the “service” environment, a fatal error will occur. (See Appendix J – Plant Manufacturing VIT Data) 10 – Use data from the routine indicated by AC2 Use AC0 & AC1 as the Data Identifier (DID) to write to. 11 – Use data from the routine indicated by AC2 Use the first TWO bytes of the routine data as the Data Identifier (DID) to write to. 20 – Use data from the calibration file indicated by AC2 (single DID write). Use AC0 & AC1 as the Data Identifier (DID) to write to. The data in the calibration file contains the raw data that will get written to the DID location (no extra bytes or header). Maximum number of data bytes is 4093. Note: Bit 7 of AC3 shall remain unused (possible usage in the future for message authentication) |

Pseudo Code:

XX     2E  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 2E | Write Data by Identifier |
| 2 | XX | Data Identifier (MSB) |
| 3 | XX | Data Identifier (LSB) |
| 4 | XX | Data Byte 1 |
| … | XX | Data Byte ... |

(continued)

If AC2 is not zero and AC3 is 20

Open calibration AC2

Determine DID data length (the file size must not exceed 4093 bytes)

Build $2F message (DID = AC0 & AC1, data bytes from calibration file)

Send message

Endif

If high nibble of AC3 is 0

Set Data Identifier to AC0 & AC1

If the low nibble of AC3 is 0

Copy internal VIT data based on AC0 & AC1 into the message

Send message

Else (the low nibble of AC3 is 3)

(ONLY supported for the Plant Manufacturing environment)

Copy internal VIT data based on AC2 (Internal Data Specifier) into the message

(See Appendix J – Plant Manufacturing VIT Data;   High nibble is the ‘Data Specifier Indicator Value)

(Assume – “Message Authentication” will not be used for these DIDs)

Send message

Endif

Endif

If high nibble of AC3 is 1

Find the routine indicated by value AC2

If the low nibble of AC3 is 1

Set Data Identifier to the first 2 routine data bytes

Copy remaining routine data bytes into the message

Send message

Else (the low nibble of AC3 is 0)

Set Data Identifier to AC0 & AC1

Copy routine data into message

Send message

Endif

Endif

If a valid AC3 was processed

GoTo UDS Response Processing

Else

End with fatal error

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 21 | 2E | F1 90 00 00 | 6E 22 FF 32 00 00 00 00 00 00 |

## 3B Op-Code: Mode 2E Write Data by Identifier from Internal Storage Buffer

Supported By:

UDS Interpreter 4

Description:

The 3B Op-Code will build a standard mode 2E message. It is used to extend the 2E Op-Code.  It is used specifically to write data from an Internal Storage buffer into a Data Identifier (DID).

John Stinks

Action Fields:

| AC0: | Data Identifier (MSB) |
| --- | --- |
| AC1: | Data Identifier (LSB) |
| AC2: | Internal Storage Buffer ID Only 0x00-0x13. Buffer ID values supported. Buffer size is 256 bytes. |
| AC3: | Number of Bytes to Write. |

Pseudo Code:

XX     2E  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 2E | Write Data by Identifier |
| 2 | XX | Data Identifier (MSB) |
| 3 | XX | Data Identifier (LSB) |
| 4 | XX | Data Byte 1 |
| … | XX | Data Byte ... |

If AC2 is a valid buffer ID (0x00 – 0x13)

Set Data Identifier to AC0 & AC1

Copy AC3 data bytes into the message from Buffer #(AC2)

Send message

GoTo UDS Response Processing

Else

End with fatal error

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 5B | 3B | F1 DF 07 00 | 6E 5C FF 7A 00 00 00 00 00 00 |

## 2F Op-Code: Mode 2F Input/Output Control By Identifier (Device Control)

Supported By:

UDS Interpreter 4

Description:

The 2F Op-Code will build a standard mode 2F message. This service can allow a test device to override normal output control functions in order to verify proper operation of a component or system, or to reset/clear variables used within normal control algorithms.

Periodic 3E (Tester Present) service is required to keep the level active.

Action Fields:

| AC0: | Data Identifier (MSB) |
| --- | --- |
| AC1: | Data Identifier (LSB) |
| AC2: | Routine Number |
| AC3: | 00 - Not Used (Must be set to 0x00) |

Pseudo Code:

XX     2F   AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 2F | Input/Output Control By Identifier |
| 2 | AC0 | Data Identifier (MSB) |
| 3 | AC1 | Data Identifier (LSB) |
| … | XX | Control Bytes |

Find the routine indicated by value AC2

Copy routine data into message

Send message

Goto UDS Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 10 | 2F | E0 22 01 00 | 6F 11 22 0D 33 2D FF 3D 00 00 |

## 31 Op-Code: Mode 31 Routine Control

Supported By:

UDS Interpreter 4

Description:

The 31 Op-Code will build a standard mode 31 message. The typical application of routine control is:  flash erase routine, checksum verification routine, …

Note:  This OpCode uses the 256-byte internal storage buffers.

Periodic 3E (Tester Present) service is required to keep the level active.

Action Fields:

| AC0: | Routine Control Service 01 – Start Routine 02 – Stop Routine 03 – Request Routine reset |
| --- | --- |
| AC1: | Routine Number (Utility File Routine Number) |
| AC2: | Internal Storage Buffer ID Only 0x00-0x13. Buffers ID values supported. Buffer size is 256 bytes. |
| AC3: | 00 - Not Used |

Pseudo Code:

XX     31   AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 31 | Input/Output Control By Identifier |
| 2 | AC0 | Routine Control Service |
| 3 | XX | Utility File Routine data (based on routine #AC1) ** |
| … | XX | … |

** Note:   The Utility File routine data will contain the RID (MSB), RID (LSB), control parameters.

Find the routine indicated by value AC1

Copy routine data into message

Send message

If positive response

If ‘valid buffer storage location’ (AC2)

Save the data of the response starting with the first Data Byte (after the echo of the RID) to storage location indicated by AC2 (fill with zero)

Else

Abort the utility file processing (fatal error)

Endif

Endif

Goto UDS Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 0A | 31 | 01 0A 02 00 | 71 0B 22 2C FF 2D 00 00 00 00 |

## 34 Op-Code: Mode 34 Request Download

Supported By:

UDS Interpreter 4

Description:

The 34 Op-Code will build a standard mode 34 message. This mode is used in order to prepare a node to receive a download of a block of data.

The node must be unlocked and work in bootloader mode.

Periodic 3E (Tester Present) service is required to keep the level active.

Note:  	The Block Length for the $36 service message is derived from the positive $34 response.

The maximum is 4095 and this length value can be only be overridden by the $F5 OpCode

(executed after OpCode $34).

Action Fields:

| AC0: | Data Format Identifier 00 - Unencrypted, uncompressed 01 - to FF High nibble Compression Method, low nibble Encrypting Method |
| --- | --- |
| AC1: | Routine (utility file) / Calibration Number |
| AC2: | Address and Length format identifier (information is nibble encoded) 44 – Use 4 bytes for length information & Use 4 bytes for address information |
| AC3: | Exceptions: (Information is nibble coded) 0X - Use length data from the utility file routine indicated by AC1 (2 byte field converted to 4 byte value) 1X - Use length data from the utility file header (2 byte field converted to 4 byte value) 2X - Use Global Length data (see F2 Op-Code) (4 bytes) 3X - Use length data based on the size of the calibration file indicated by AC1 (4 bytes) X0 - Use address from the utility file routine indicated by AC1 (4 bytes) X1 - Use address from the utility file header (4 bytes) X2 - Use Globall Address (see F1 Op-Code) (4 bytes) |

Note:  4 Byte Naming convension = High, MidHigh, MidLow, Low

Pseudo Code:

XX     34  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the example table below

(This example is for a 4-byte Address and a 4-byte Length)

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 34 | Request Download Service ID |
| 2 | AC0 | Data Format Identifier |
| 3 | XX | Address and Length Format Identifier (derived via AC2 value) |
| 4 | A1 | Memory Address (High) |
| 5 | A2 | Memory Address (MidHigh) |
| 6 | A3 | Memory Address (MidLow) |
| 7 | A4 | Memory Address (Low) |
| 8 | S1 | Memory Size (High) |
| 9 | S2 | Memory Size (MidHigh) |
| 10 | S3 | Memory Size (MidLow) |
| 11 | S4 | Memory Size (Low) |

(continued)

If the high nibble of AC3 is 0 (‘0X’)

Find the utility file routine indicated by value AC1

Set MemorySize to the routine length

ElseIf the high nibble of AC3 is 1 (‘1X’)

Set MemorySize to length from the utility file Header

ElseIf the high nibble of AC3 is 2 (‘2X)

Set MemorySize to Global Length

ElseIf the high nibble of AC3 is 3 (‘3X’)

Find the calibration file indicated by value AC1

Set MemorySize to the size of the calibration file

Else

End with error (invalid high nibble of AC3)

Endif

If the low nibble of AC3 is 0 (‘X0’)

Find the utility file routine indicated by value AC1

Set MemoryAddress to the routine address

ElseIf the low nibble of AC3 is 1 (‘X1’)

Set MemoryAddress to the address from the utility file Header

ElseIf the high nibble of AC3 is 2 (‘2X)

Set MemorySize to the Global Address

Else

End with error (invalid high nibble of AC3)

Endif

Send message

If Positive Response

Set the payload data length (max packet length) from the response

(used for subsequent TransferData Services)

If this Length is > 4095

Set the payload data length equal to 4095 (maximum)

Endif

Endif

Goto UDS Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 0A | 34 | 00 02 44 32 | 74 0B FF 3F 00 00 00 00 00 00 |

## 36 Op-Code: Mode 36 Transfer Data

Supported By:

UDS Interpreter 4

Description:

The 36 Op-Code will build a standard mode 36 message. This Op-Code is used to transfer data to a control module.

The number of bytes (length) to send with each message is derived from OpCode 34 (Service 34 Request Download).  The actual number of calibration/routine bytes will be ‘length – 2’ to account for the SID and the Block Sequence Counter bytes.  A single data transfer message is limited to a maximum of 4095 data bytes (including the SID and block Sequence Counter).

Periodic 3E (Tester Present) service is required to keep the level active.

If the GlobalHeaderLength (OpCode F3) is not zero then the download of the module will be separated in two parts. The GlobalHeaderLength number of bytes will be downloaded first.

The Block Sequence Counter assignment shall be handled internally by the application code.  This counter of the ECU shall be initialized to 0x01, meaning that the first Transfer Data ($36) request message will have a Block Sequence Counter of 0x01.  The Tester and ECU should increase the counter in a synchronized fashion.  This value is incremented by one for each subsequent Mode 36 request.  If the value of $FF is reached, the counter shall go back to the value $00 for the next request.

Note:  Utility files for GM UDS ECUs that employ the Secure Bootloader design (digitally signed OpSoftware and Calibration files) require special negative response handling - - - Reading of the ProgrammingError Code DID.

Note:  	The Block Length for the $36 service message is derived from the positive $34 response.

The maximum is 4095 and this length value can be only be overridden by the $F5 OpCode

(executed after OpCode $34).

Action Fields:

| AC0: | Calibration ID 00 - Not Used 01 - FF Calibration File ID |
| --- | --- |
| AC1: | 00 – Not Used |
| AC2: | 00 – Not Used |
| AC3: | 00 – Not Used |

Pseudo Code:

XX     36    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 36 | Transfer Data |
| 2 | XX | Block Sequence Counter (01, 02, .., FF, 00, 01, ….) |
| 3 | XX | Data Byte 1 |
| … | XX | Data Byte … |

Set LengthSize based on OpCode 34 (max is 4095) (can override value using OpCode $F5)

If AC0 (Calibration ID) is not zero

Use data from the calibration file indicated by value AC0

Endif

(continued)

Do while more data to download

If this is the last packet to download     or    the GlobalHeaderLength is not 0 ‘split the download’

Copy the remaining data into the message

Else

Copy the next block of data into the message

Endif

Send message  (increment BlockSequenceCounter for each Mode 36 Request)

If negative response received

Break do while more data to download

Endif

End do while more data to download

Goto UDS Response Processing

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 12 | 36 | 01 00 00 00 | 76 13 72 2E FD 2F FF 30 00 00 |

## 37 Op-Code: Mode 37 Request Transfer Exit

Supported By:

UDS Interpreter 4

Description:

The Op-Code 37 service is used to terminate a data transfer between the tester and ECU (server).

Action Fields:

| AC0: | 00 – Not Used |
| --- | --- |
| AC1: | 00 – Not Used |
| AC2: | 00 – Not Used |
| AC3: | 00 – Not Used |

Pseudo Code:

XX    37    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Build message as indicated in the table below

| Request Byte | Value | Byte Description |
| --- | --- | --- |
| 1 | 37 | Request Transfer Exit |

Send message

Goto UDS Response Processing

Typical Interpreter Line: (all Action and Goto fields are hex numbers)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 0C | 37 | 00 00 00 00 | 77 0D 72 22 FF 23 00 00 00 00 |

# Communications Independent Programming Op-Codes

## 50 Op-Code: Compare Bytes

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The 50 Op-Code compares 2 bytes of stored information to 2 bytes of information in the interpretive line. The stored information is not generated by the 50 Op-Code. Action Field 0 will provide the location ID of the 2 stored bytes.

Note:	The first (G0) and the third (G2) bytes of the Goto Fields are always set to 0x00 because they are not used (don't cares).

Note:   This OpCode uses only the 2-byte storage buffers.  For buffer usage information, see the section: “Programming Buffers – Recommended Usage”

Action Fields:

| AC0: | ID for saved bytes, only 0x00 – 0x13 IDs are supported. (2-byte storage buffer) |
| --- | --- |
| AC1: | First byte of compare (high byte) |
| AC2: | Second byte of compare (low byte) |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  50  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

If the 2 Bytes Stored in storage location ID (AC0) matches the 2 bytes in AC1 and AC2

Goto step specified in G1

Else

Goto step specified in G3

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 50 | 03 89 B9 00 | 00 07 00 29 00 00 00 00 00 00 |

## 51 Op-Code: Compare Checksum

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The 51 Op-Code compares a stored checksum to a checksum calculated from a calibration module. The “module” checksum is calculated by the Op-Code. The stored checksums may reside in 1 of 20 possible storage locations specified by AC0. Three different checksum algorithms are supported: 16 bit sum of all bytes, CRC-32 or CRC-32 complement. CRC-32 requires a 32-bit checksum. Hence, a 16-bit sum comparison will be done against 2-byte storage location while the CRC-32 checks will be done against the first 4 bytes of a 256-byte storage location. Care must be taken to use the proper Op-Code to store the controller’s checksum.

Note: 	The first (G0) and the third (G2) bytes of the Goto Fields are always set to 0x00 because they are not used (don't cares).

Note:   This OpCode uses either the 2-byte or the 256-byte storage buffers.  For buffer usage information, see the section: “Programming Buffers – Recommended Usage”

Action Fields:

| AC0: | ID for saved bytes, only 0x00 – 0x13 IDs are supported. |
| --- | --- |
| AC1: | Calibration module ID of calculated checksum |
| AC2: | Checksum Type (00, 01, 02 – see below) |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  51  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

If AC2 is 0 (16-bit checksum)

Sum up all data bytes of the module specified by AC1 as a 16-bit value

Else

Calculate the CRC-32 of the module specified by AC1 as a 32-bit value

Endif

If AC2 is 00

Compare calculated checksum with 2-byte storage location specified by AC0

Else If AC2 is 01

Complement checksum and compare with first 4 bytes of the 256-byte storage location specified by AC0

Else If AC2 is 02

Compare checksum with first 4 bytes of the 256-byte storage location specified by AC0

Else

Set the “compare” to failed

Endif

If the “compare” succeeded

Goto step specified in G1

Else

Goto step specified in G3

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 51 | 03 01 00 00 | 00 07 00 29 00 00 00 00 00 00 |

## 53 Op-Code: Compare Data

Supported By:

Class 2 Interpreter 1

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The 53 Op-Code compares a maximum of 256-bytes of stored information (based on AC0) against other information that can be internal data (VIT2), routine data or other stored data.

Note:  The first (G0) and the third (G2) bytes of the Goto Fields are always set to 0x00 because they are not used (don't cares).

DPS Note:  When Internal VIT2 Data is used for the comparison AND the conversion type of 01 is chosen (ASCII to 4 byte USN), the software/calibration file names (part numbers) within the DPS archive must be numeric in order to perform a valid data comparison.

Note:  This OpCode uses only the 256-byte storage buffers.  For buffer usage information, see the section:

“Programming Buffers – Recommended Usage”

| AC0: | ID for saved bytes, only 0x00 – 0x13 IDs are supported. Buffer size: 256 bytes |
| --- | --- |
| AC1: | Second ID (See Exceptions) |
| AC2: | Conversion 00 - Not Used 01 - ASCII to 4 byte USN |
| AC3: | Exceptions: 00 - Use internal data (VIT2) where AC1 identifies the information 01 - Use data from the routine indicated by AC1 02 - Use stored information where AC1 is ID for saved bytes, only 0x00 – 0x13 IDs are supported. Buffer size: 256 bytes 03 – Use Design Level Suffix (DLS) of the part number referenced in AC1. DLS must be 2 ASCII bytes. (This is for GM Assembly plants only. 'Service' will always goto the step listed in G1). For the Assembly plants: If there is no DLS, the G1 path will be taken. The DLS will be compared to the DLS in the file name, not the GMLAN header. |

Action Fields:

Formats: ASCII, BCD = Binary Coded Decimal, USN = Unsigned Numeric, BIN = Binary or bit encoded.

Internal Data Identifiers

| 0x01 | Part Number Module ID 1 |
| --- | --- |
| 0x02 | Part Number Module ID 2 |
| 0x03 | Part Number Module ID 3 |
| ... | ... |
| 0x13 | Part Number Module ID 19 |
| 0x14 | Part Number Module ID 20 |
|  | - OR - |
| 0x81 | Part Number Module ID 1 |
| 0x82 | Part Number Module ID 2 |
| 0x83 | Part Number Module ID 3 |
| ... | ... |
| 0xE2 | Part Number Module ID 98 |
| 0xE3 | Part Number Module ID 99 |
| 0x41 | VIN |
| 0x42 | Manufacturer HW Number (End Model ECU Software Number) |
| 0x43 | Supplier HW Number (Base Model ECU Hardware Number) |

| 0x44 | Tire Type (e.g. P275/55R20) *** |
| --- | --- |
| 0x45 | TPM tire placard data *** |
| 0x46 | Diesel Injector Mass Adjustment (IMA) or Injector Quantity Adjustment (IQA) data. *** |
| 0x47 | Transfer Case Control Module “B-Cal” (spring rate) data. *** |
| 0x48 | Key Fob data. This internal data specifier is used to program key fob data into a controller on the vehicle. *** |
| 0x49 | Engine Serial Number. *** |
| 0x4A | Rear suspension Trim set. *** |
| 0x4B | ELSD Clutch Specification. *** |
| 0x4C | TCM Characterization. *** This data also is limited to a maximuim of 256 bytes for comparison. |
| 0x4D | Current Date/Time will NOT be supported in this OpCode due to the complications of doing a date/time comparison. This value shall not be allowed in this field. |

*** Note: 	This internal data is only supported in GM Vehicle Assembly Plants.   Comparison of this data in

a ‘Service’ environment is treated as an invalid exception and results in termination of utility file

execution with error.  (See “Appendix J - Plant Mfg VIT Data” for more information)

Pseudo Code:

XX     53  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Set “Data2” to stored information indicated by AC0

If AC3 is 00

Set “Data1” to internal data (VIT2) where AC1 identifies the information

Else If AC3 is 01

Set “Data1” to routine data indicated by AC1

Else If AC3 is 02

Set “Data1” to stored information indicated by AC1

Else If AC3 is 03

Set “Data1” to the Design Level Suffix associated with the part number data indicated by AC1

Endif

If AC2 is not 00

Convert “Data1” using the “conversion” format indicated by AC2

Endif

If all bytes in “Data1” (max. 256 bytes) matches the bytes in “Data2” 	or    If AC3 is 03 and the environment is “service” (not a Vehicle Assembly Plant)

Goto step specified in G1

Else

Goto step specified in G3

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 53 | 00 02 00 00 | 00 07 00 29 00 00 00 00 00 00 |

## 54 Op-Code: Change Data

Supported By:

Class 2 Interpreter 1

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The 54 Op-Code manipulates one or more bytes within one of the 256-byte storage buffers (AC0) or can be used to load data from a routine into one of the 256-byte buffers for subsequent manipulation.  It can also be used to copy the data from one internal storage buffer to another.

Note:  The first (G0) byte of the Goto Field is always set to 0x00 because it is not used (don't care).

See appendix G for further information on this Op-Code.

Action Fields:

| AC0: | ID for saved bytes, only 0x00 – 0x13 IDs are supported. Buffer size: 256 bytes |
| --- | --- |
| AC1: | Second ID - Byte position if AC2 < 04 ($00-$FF – zero based indexing) - Number of bytes to “shift” if AC2 is 04 or 05 ($00-$FF) - Routine # if AC2 is 06 ($01-$FF) - Internal Data Specifier if AC2 is 07 ($41, ”more: See Appendix J – Plant Mfg VIT Data” ) |  |  |  |
| AC2: | Operation 00 - EQUAL 01 - AND 02 - OR 03 - XOR 04 - SHL shift left, fill with zero (byte shift left) 05 - SHR shift right, fill with zero (byte shift right) 06 - Load data from routine specified by AC1 into buffer specified by AC0 (maximum of 256 bytes will be loaded from the routine) 07 – Load internal data as specified in AC1 into buffer specified by AC0 (maximum of 256 bytes). If less than 256 bytes the remaining bytes in 256 byte buffer shall be set to $00). Only data entries $41 (VIN) and “Plant Mfg VIT Data” (See Appendix J) are currently considered valid entries for this op-code. Specifying other data is treated as an invalid exception and results in termination with error of the utility file. Currently, this Operation is ONLY supported by the vehicle assembly plant programming. Use of this exception in service is invalid and results in termination with error of the utility file. 08 – Copy data contents from the buffer speified in AC1 to the buffer specified in AC0 |  |
| AC3: | Exceptions: XX - Bit Mask used if AC2 < 04 XX – Cylinder number if AC2 = 07 and AC1 is $46. ** - Key Fob number if AC2 = 07 and AC1 is $48. ** - TCM Characterization if AC2 = 07 and AC1 is $4C. ** (up to 256 bytes) - Current Date/Time data if AC2 = 07 and AC1 is $4D. ** ** Data Indicator values are defined in Appendix J – Plant Manufacturing VIT Data. |  |  |

Pseudo Code:

XX     54  AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Identify Storage Buffer specified by AC0

If AC2 is $08 (Copy the data from one buffer to another)

Copy the complete set of data from storage buffer ID (AC1) into storage buffer (AC0)

Else If AC2 is $06 (Load routine data into Data Buffer specified by AC0)

Read routine data based on Routine # (AC1)

Store the “routine data” into storage buffer    (max. 256 bytes)

Else If AC2 is $07 	(vehicle assembly plant only)      (See Appendix J – Plant Manufacturing VIT Data)

(Load appropriate internal data into Data Buffer specified by AC0)

Else

Change “storage buffer data” using operation indicated by AC2 (use AC1 and AC3 information appropriately)

Endif

Goto step specified in G1

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 54 | 00 01 00 01 | 00 07 00 00 00 00 00 00 00 00 |

## 55 Op-Code: Evaluate RPO (for use in vehicle assembly plants only)

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The 55 Op-Code compares an ASCII RPO string stored in the routine section of the utility file against the option content of the vehicle.  If the comparison evaluates to “true” (option content matches the option string) then the next step executed will be the one in G1.  If the comparison fails then the next step executed is the one in G3.  Since Service does not have access to RPO data in the field, dealership tools will always jump to the instruction in G5.

Note: 	The first (G0), third (G2), and 5th (G4) bytes of the Goto Fields are always set to 0x00 because they are not used (don't cares).

Action Fields:

| AC0: | Use RPO data from the routine indicated in this byte |
| --- | --- |
| AC1: | Format Byte: 00 = RPO strings formatted using the General Assembly Test System (GATS) option processing format (see appendix H for examples). 01-FF reserved for future use. |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  55  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

In Vehicle Assembly plants the following logic is executed:

Obtain RPO string from routine indicated by AC0

Compare RPO string against vehicle build data

If compare succeeded

Goto step specified in G1

Else

Goto step specified in G3

In Service the following logic is executed:

Goto step specified in G5

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 55 | 01 00 00 00 | 00 07 00 29 00 35 00 00 00 00 |

## 56 Op-Code: Interpreter Identifier

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The 56 Op-Code is used to branch to a specific action which may be required only in Service (GM CCA GADE), Vehicle Assembly Plant (AVG), or the FCP (Connected Vehicle).

Note:	The first (G0), the third (G2) and the fifth (G4) bytes of the Goto Fields are always set to 0x00 because they are not used (don't cares).

Action Fields:

| AC0: | 00 - Not Used |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  56  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

If the “Service” Interpreter is being run (DPS and SPS)

Goto step specified in G1

Else if the “Manufacturing” Interpreter is being run (Vehicle Assembly Plant Tool)

Goto step specified in G3

Else if the “FCP” Interpreter is being run (Connected Vehicle FCP)

Goto step specified in G5

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | 56 | 00 00 00 00 | 00 07 00 09 00 11 00 00 00 00 |

## E0 Op-Code: End of ‘flash erase’

Supported By:

UDS Interpreter 4

Description:

The E0 Op-Code is used to signal to the tool programming software that the ‘flash erase’ has completed.    This Op-Code should be called after the Op-Code (UDS: routine control) to erase the flash has been successfully performed.

This Op-Code is essential for manufacturing (plant) programming (which programs multiple ECUs in parallel).

See Pseudo Code below.

Action Fields:

| AC0: | 00 - Not Used |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

Manufacturing Plant Application:

Wait for other ‘parallel’ programming ECUs have reached this stage (erase complete)

Goto step specified in G1

Non-Manufacturing Application (DPS, Tis2Web SPS, OTA, …):

Goto step specified in G1 (immediately)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 98 | EE | 00 00 00 00 | 00 A5 00 00 00 00 00 00 00 00 |

## EE Op-Code: End with ERROR

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The EE Op-Code is used to signal the tool programming software that an error has occurred. The Interpreter will end processing and return to the main software.

The main software will then process any “error codes” that were set from previously executed and failed Op-Codes.

Action Fields:

| AC0: | 00 - Not Used |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

End reprogramming and return to the main software for “error” processing.

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 98 | EE | 00 00 00 00 | 00 00 00 00 00 00 00 00 00 00 |

## F1 Op-Code: Set Global Memory Address

Supported By:

UART Interpreter 0

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The F1 Op-Code is used to set a global memory address for use in applicable Service Requests.  The global memory address can be used when Op-Codes require address information and the normal address is not correct.  To use the global address information in an Op-Code, an F1 must be included in the Interpreter instructions prior to the Op-Code that is going to use the global address information.  The global address information can be changed as many times as necessary during a programming event.

Note:  The first (G0) byte of the Goto Field is always set to 0x00 because it is not used (don't care).

Action Fields:

| AC0: | MidHigh byte of address |
| --- | --- |
| AC1: | MidLow byte of address |
| AC2: | Low byte of address |
| AC3: | High byte of address (allows for a 4 byte address value) |

Pseudo Code:

XX    F1    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

MidHigh byte of global address equals AC0

MidLow byte of global address equals AC1

Low byte of global address equals AC2

High byte of global address equals AC3

Goto step specified in G1

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | F1 | 2F FF F0 00 | 00 09 00 00 00 00 00 00 00 00 |

## F2 Op-Code: Set Global Memory Length

Supported By:

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The F2 Op-Code is used to set a global memory length for use in applicable Service Requests.  The global memory length can be used when Op-Codes require length information and the normal length is not correct.  To use the global memory length information in an Op-Code, an F2 must be included in the Interpreter instructions prior to the Op-Code that is going to use the global memory length information.  The global memory length information can be changed as many times as necessary during a programming event.

Note:  The first (G0) byte of the Goto Field is always set to 0x00 because it is not used (don't care).

Action Fields:

| AC0: | MidHigh byte of length |
| --- | --- |
| AC1: | MidLow byte of length |
| AC2: | Low byte of length |
| AC3: | High byte of length (allows for a 4 byte memory length value) |

Pseudo Code:

XX    F2    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

MidHigh byte of global length equals AC0

MidLow byte of global length equals AC1

Low byte of global length equals AC2

High byte of global length equals AC3

Goto step specified in G1

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | F2 | 01 A0 00 00 | 00 09 00 00 00 00 00 00 00 00 |

## F3 Op-Code: Set Global Header Length

Supported By:

Class 2 Interpreter 1

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The F3 Op-Code is used to set a global header length for use in applicable Service Requests.  The global header length can be used when Op-Codes require header length information and the normal header length is not correct (defaults to zero).  To use the global header length information in an Op-Code an F3 must be included in the Interpreter Instructions prior to the Op-Code that is going to use the global header length information.  The global header length information can be changed as many times as necessary during a programming event.

Note:  The first (G0) byte of the Goto Field is always set to 0x00 because it is not used (don't care).

Action Fields:

| AC0: | MidHigh byte of header length |
| --- | --- |
| AC1: | MidLow byte of header length |
| AC2: | Low byte of header length |
| AC3: | High byte of header length (allows for a 4 byte header length value) |

Pseudo Code:

XX    F2    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

MidHigh byte of global header length equals AC0

MidLow byte of global header length equals AC1

Low byte of global header length equals AC2

High byte of global header length equals AC3

Goto step specified in G1

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | F3 | 00 00 14 00 | 00 09 00 00 00 00 00 00 00 00 |

## F4 Op-Code: Ignore Responses for Milliseconds

Supported By:

KWP2000 Interpreter 2

Description:

The F4 Op-Code is used to ignore responses being received within a specified time frame. This feature is required to avoid request/response inconsistencies caused by controllers responding with more than one positive or negative response (other than negative $78).

Note:  The first (G0) byte of the Goto Field is always set to 0x00 because it is not used (don't care).

Action Fields:

| AC0: | High byte of time frame (milliseconds) |
| --- | --- |
| AC1: | Low byte of time frame (milliseconds) |
| AC2: | $00 - Not used |
| AC3: | $00 – Not used |

Pseudo Code:

XX    F4    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

Calculate time frame by (AC0 * 256) + AC1

Do until time frame expired

Receive response and disregard

End “Do processing”

Goto step specified in G1

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 19 | F4 | 01 2C 00 00 | 00 20 00 00 00 00 00 00 00 00 |

## F5 Op-Code: Override the Message Length Value

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

Non UDS:    Override Utility File Message Length Value

The F5 Op-Code is used to override the utility file header value that contains the “number of data bytes in a message” (header positions $16 - $17).  This will allow utility files to be constructed so that varying calibrations can be downloaded with different data message lengths.  In addition, this Op-Code can be used to revert back to the original “number of data bytes in a message”, defined within the original utility file header.

The new message length value is permanent, for the single execution of the utility file, until a subsequent F5 Op-Code is executed.  (I.e. The new message length value will be used for all subsequent “download” Op-Codes where that Op-Code specifies the use of the utility file message length.)

Caution: 	For utility files that contain retry logic where processing is returned back to a previous calibration

download step, then the corresponding message length value may also need to be reset back to

its’ correct value.

UDS (Interpreter 4):    Override Message Length Value

The F5 Op-Code is used to override the message length value used in the TransferData $36 Service.  The defaulting length value is derived by the length reported back in the positive response of a Request Download $34 Service (with a current maximum value of 4095).

The new message length value is applicable ONLY until the next Request Download $34 Service is executed.

Note:  The first (G0) byte of the Goto Field is always set to 0x00 because it is not used (don't care).

Action Fields:

| AC0: | High byte of data message length |
| --- | --- |
| AC1: | Low byte of data message length |
| AC2: | 00 – Not Used |
| AC3: | 00 – Not Used |

Pseudo Code:

XX    F5    AC0   AC1   AC2   AC3     G0-G1   G2-G3   G4-G5   G6-G7   G8-G9

If the Length is Zero (AC0=00 and AC1=00)

(non UDS) 	Set the Utility File Data Message Length to the original “utility file” value

(UDS) 		Do nothing

Else

Set the Data Message Length to the value defined by AC0 and AC1 ((AC0 * 256) + AC1)

Endif

Goto step specified in G1

Typical Interpreter Lines:

Switch to new Data Message Length (0x0480)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | F5 | 04 80 00 00 | 00 09 00 00 00 00 00 00 00 00 |

Revert back to the default Data Message Length (defined within the utility file)  (non UDS)

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 0A | F5 | 00 00 00 00 | 00 0B 00 00 00 00 00 00 00 00 |

## F7 Op-Code: No Operation Op-Code

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The F7 Op-Code is for NO operation. These lines in the Utility File are used as filler. In the case that the line is the destination of a Goto Field it will continue with the next step.

Note: 	All Action Fields and Goto Fields are 0x00 (don't care).

Action Fields:

| AC0: | 00 - Not Used |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  F7  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Goto next step number

(Example:  If the current utility file step is 09, then the next step executed will be 0A.)

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 09 | F7 | 00 00 00 00 | 00 00 00 00 00 00 00 00 00 00 |

## F8 Op-Code: Goto Field continuation

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The F8 Op-Code is used for continuation of the Goto Fields from the previous step. If an 'F8' Op-Code is the destination of a goto field an Interpreter error will occur.

Action Fields:

| AC0: | 00 - Not Used |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  F8  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

If this Op-Code is used as a continuation of the goto fields the Interpreter will check to see if the F8 Op-Code is present before checking the goto fields.  If the F8 is not present an Interpreter error will occur.

If a previous utility file step attempts to GOTO a step with Op-Code $F8, then an Interpreter error will occur.

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | C0 | 01 39 F4 02 | 06 11 05 0D 04 0E FD 20 15 12 |
| 09 | F8 | 00 00 00 00 | AA 12 01 0F FF 99 00 00 00 00 |

## FB Op-Code: Set and Decrement Counter

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The FB Op-Code is used to set and decrement loop counters. Action field 0 (AC0) is used to identify the counter and action field 1 (AC1) is for the loop limit. There is a maximum of 20 loop counters that can be active at one time.

Note:  The first (G0) and the third (G2) bytes of the Goto Fields are always set to 0x00 because they are not used (don't cares).

Action Fields:

| AC0: | Counter ID: (Valid counter ID's are: 0x00 – 0x13) |
| --- | --- |
| AC1: | Loop limit |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  FB  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

If the Counter Value is $FF (counter reset)

Load the counter ID and set the loop limit

Endif

Decrement the Counter Value

If the Loop Counter Is Greater Than Zero

Goto step specified in G1

Else

Goto step specified in G3, counter has expired

Endif

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 98 | FB | 01 03 00 00 | 00 10 00 0D 00 00 00 00 00 00 |

## FC Op-Code: Delay for ?? Seconds/Minutes

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The FC Op-Code is used for timing purposes. The Op-Code uses Action 0 as the number of seconds to pause before processing the step specified in goto field 1 (G1).

Note:  The first (G0) byte of the Goto Field is always set to 0x00 because it is not used (don't care).

Action Fields:

| AC0: | Number of Seconds/Minutes to Pause (Hex value) |
| --- | --- |
| AC1: | Protocol: Class 2 00 - No Tester Present messages will be sent during the delay 01 - Send Tester Present messages during the delay Protocol: All Other 00 - Not Used (Default: Tester Present messages will be sent) |
| AC2: | 00 - Not Used |
| AC3: | Exceptions: 00 - AC0 will be in seconds 01 - AC0 will be in minutes |

Pseudo Code:

XX  FC  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

Pause for AC0 Seconds/Minutes (based on AC3)

If the communication protocol is Class 2

If AC1 is 01

Send Tester Present messages during the delay time

Endif

Else

Send Tester Present messages during the delay time

Endif

Goto step specified in G1

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | FC | 01 00 00 00 | 00 0A 00 00 00 00 00 00 00 00 |

## FD Op-Code: Reset counter

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The FD Op-Code is used to reset counters. The counter with ID specified in action field 0 will be set to negative one ($FF) and the step in the first goto field will be executed next. If AC0 is set to $FF then all counters are reset to negative one ($FF).

Note:  The first (G0) byte of the Goto Field is always set to 0x00 because it is not used (don't care).  All counters are reset by default before a programming session.  Op-Code $FD is needed when a counter is reused in the same programming session.  Counters are NOT reset by default between part 1 and part 2 of a utility file (GMLAN, UDS).

Action Fields:

| AC0: | ID of Counter to reset (Valid counter ID's are: 0x00 – 0x13 for individual counters or 0xFF to reset all counters.) |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

XX  FD  AC0  AC1  AC2  AC3   G0-G1  G2-G3  G4-G5  G6-G7  G8-G9

If AC0 is $FF

Reset all loop counters (set the value of each loop counter to 0xFF)

Else

Reset loop counter with ID of AC0 (set the value to 0xFF)

Endif

Goto step specified in G1

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | FD | 06 00 00 00 | 00 0B 00 00 00 00 00 00 00 00 |

## FF Op-Code: End with SUCCESS

Supported By:

UART Interpreter 0

Class 2 Interpreter 1

KWP2000 Interpreter 2

GMLAN Interpreter 3

UDS Interpreter 4

Description:

The FF Op-Code is used to signal the reprogramming software that the device has been reprogrammed successfully. The Interpreter will end processing and return a success to the main program.

Note:  To eliminate any confusion, the programming status “progress” bar will be incrementally updated

up to 100 percent upon the execution of this OpCode.  For a 2-part utility file, the update will take

place in part 2 of the utility file.

Action Fields:

| AC0: | 00 - Not Used |
| --- | --- |
| AC1: | 00 - Not Used |
| AC2: | 00 - Not Used |
| AC3: | 00 - Not Used |

Pseudo Code:

End reprogramming and return success

Typical Interpreter Line:

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 98 | FF | 00 00 00 00 | 00 00 00 00 00 00 00 00 00 00 |

# System Errors and Op-Code Failures

Interpreter system errors will be assigned a global error number when they occur. This error number will be displayed along with a brief description. Below is a list of the current Interpreter system errors and descriptions (the descriptions may not match the screen text):

100 - The user requested to quit from the No Communications screen.

101 - Communications failed (maximum retry is three).

102 - The Utility file header contained an Interpreter type that is not supported.

103 - The Goto field of an interpreter step leads to a No Operation Op-Code.

104 - Not used.

105 - Could not find the prolog in the routine section of the Utility file.

106 - Could not find the routine in the routine section of the Utility file.

107 - Invalid storage ID specified in the Interpretive Information.

120 - Encountered a step 00 in the Interpretive Information.

121 - Offset to device routines points beyond the end of the Utility file.

122 - Invalid Section ID for file to download.

123 - Loop step sent to an EE Op-Code on first pass.

160 - Fatal Programming Error - Attempt to program while engine is running or vehicle speed exceeds limit.

161 - Fatal Programming Error - Invalid module ID sent to module.

166 - Fatal Programming Error - Failure to program to zero.

169 - Fatal Programming Error - Failure to Erase.

170 - Fatal Programming Error - Device /Manufacturer code not supported.

171 - Fatal Programming Error - Devices not compatible.

172 - Fatal Programming Error - Security problem.

173 - Fatal Programming Error - Flash reprogram error.

174 - Fatal Programming Error - Checksum fault.

175 - Fatal Programming Error - Processor reset.

213 - Reprogramming Attempt has failed.

281 - Error trying to open a calibration file.

297 - Reprogramming Attempt has failed.

298 - Reprogramming Attempt has failed.

If an Op-Code fails during the programming process the Step and Op-Code that failed will be displayed with a brief description of what occurred. All error codes are prefaced by the letters SPS before they are displayed to the user.

For example: SPS05D0 - Invalid response received from the device.

The above error code indicates that a failure occurred during step 05, which was attempting a D0 - Mode 6 Download Calibration/Software File.

The complete list of reprogramming error codes and descriptions will be included in a later version of this document.

Disclaimer:   Not all vehicle protocols are setup to report these “global error code” conditions.  In addition, not all SPS applications utilize these error codes for displaying corresponding error messages to the user.

# Procedure for Adding New Op-Codes or Exceptions to Existing Op-Codes

When new devices are released for production, new Op-Codes and add exceptions to existing Op-Codes may be required. The Op-Codes and their functionality need to be included in the SPS software before being included in the interpreter instructions of a Utility File. To ensure the timing of changes, the Development Engineers from the controller’s supplier will have to work closely with the Systems Engineers at GM Customer Care & Aftersales. A formal procedure has to be created, to ensure that the development process is structured and kept under control. The supplier must provide the GM Customer Care & Aftersales SE with a complete description of the Op-Codes exact functionality. The description must contain a byte-by byte breakdown of how to build the message as well as a byte-by byte description of all the possible responses (for an example see CD-1024). The detail of the request should be comparable to the Pseudo Code sections of the Op-Code descriptions. The GM Customer Care & Aftersales SE will review the request and decide whether a new Op-Code or an exception to an existing Op-Code is required.

After the Op-Codes/exceptions have been created, a new software package will be available for testing. The Op-Code updates will be documented and tested by the supplier and GM Customer Care & Aftersales personnel. Provided the new software works as specified the new interpreter will be released for production. Any changes or additions to the Op-Codes will result in updates to this document. In some cases a request for change may be generated by individuals outside of the ECU development area, for these cases the requirements are the same.  The described interactions will be between the requesting group and GM Customer Care & Aftersales.

Prior to release of any interpreter/documentation updates any affected groups that use an interpreter-based software must be notified.

The key to the process is the detail of the request from the engineering group. If the request is lacking in any way, GM Customer Care & Aftersales personnel will not be able to accurately include the Op-Code in the Interpreter.

Note: Once an Interpreter is released, GM Customer Care & Aftersales cannot modify an existing Op-Code/exception combination unless the original functionality is unaffected. It is GM Customer Care & Aftersales responsibility to verify that the existing support provided by the Op-Code still works properly. If this is not possible, for any reason, then the Op-Code/exception combination cannot be changed. GM Customer Care & Aftersales should always plan to create a new Op-Code or a new exception to an existing Op-Code when new Interpreter functions are needed. This rule can only be violated when the change has NO impact on existing Interpreter Op-Code function.

# Appendixes

A - UART Interpreter Examples

B - Class 2 Interpreter Examples

C - Keyword Protocol 2000 Interpreter Examples

D – GMLAN Utility File Guidelines

E – UDS Utility File Guidelines

F - Revision Log

G - Related Documents

H - $54 Op-Code Additional Information

I - $55 Plant Manufacturing VIT Data

## Appendix A - UART Interpreter Examples

The following lines of interpretive code are examples of how lines in the interpretive section of a Utility File are used by the tools. The examples are not in a particular order and are strictly for explanatory purposes.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 05 | F4 01 00 00 | AA 02 05 04 09 05 FF 06 00 00 |

The first step is a 05 Op-Code (Ram Download Request). The tool will send an ALDL mode 5 message. The 01 exception tells the interpreter to wait for an F0 poll before sending the mode 5 message. For this example assume an AA response was received from the device, after the mode 5 message was sent, the interpreter would jump to the step specified after the AA in the goto fields (step 02). If a 05 response were received, the interpreter would jump to step 04. A 09 response would jump to step 05 and any other responses would jump to step 06.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 02 | C0 | 02 03 F4 00 | AA 03 05 04 FF 06 00 00 00 00 |

This step is a C0 Op-Code that is used to Download Routines to RAM and then execute the routines. The line instructs the interpreter to send routine 02 to device ID F4 using routine 03 as the prolog. Prologs are only used when a routine is larger then the maximum routine packet size. Prologs generate responses for the device as the pieces of a routine are being downloaded to RAM. All of the routines are contained in the routine section of the Utility Files. After all of the routine is downloaded the device will execute the routine and a response will be returned. Assume that an AA is returned from the device, so step 03 (Goto field 1) will be the next step interpreted.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 03 | D0 | 01 39 F4 01 | 06 07 15 04 FF 05 00 00 00 00 |

The next Op-Code to execute is D0 (Download Calibration / Software file). The tool will send to device F4 the file with ID 01 and since the exception 01 is specified, 39 will be the prolog. After the file is downloaded to the device a response will be returned to the tool. Assume a 15 is returned, the Goto field 2 will be the next step executed (04). This return is an error and will cause the process to loop. See the next Op-Code. If a 06 was returned then the programming event would be a success and the process would end (see step 07). If any other return value were returned then Goto step 3 (05) would be executed. This will signal that the programming event has failed and the module should to be replaced (see step 05).

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 04 | FB | 01 03 00 00 | 00 02 00 05 00 00 00 00 00 00 |

If this step is executed the counter 01 is set to 03 and the interpreter will jump to the step specified in Goto step 1 (02). Each subsequent time this step is executed, counter 01 will be decremented by one. If the counter is above zero the first Goto step (02) is executed again. If the counter is zero, meaning that the counter has expired, the step specified in Goto step 2 (05) is executed.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 05 | EE | 00 00 00 00 | 00 00 00 00 00 00 00 00 00 00 |

This step is executed if an error has occurred during the programming process and the replace module message should be displayed.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | EE | 01 00 00 00 | 00 00 00 00 00 00 00 00 00 00 |

This step is executed if an error has occurred during the programming process. The 01 in AC0 indicates that a message instructing the user to replace the control module should be displayed on the error screen.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 07 | FF | 00 00 00 00 | 00 00 00 00 00 00 00 00 00 00 |

This step indicates a successful completion of the programming process.

## Appendix B - Class 2 Interpreter Examples

The following lines of interpretive code are examples of how lines in the interpretive section of a Utility File are used by the tools. The examples are not in a particular order and are strictly for explanatory purposes.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 01 | 01 | F1 00 00 00 | 00 02 00 00 00 00 00 00 00 00 |

The first step is a 01 Op-Code (Setup Global Variables). The tool sets the global tester ID to the value in AC0 ('F1'). The zero in AC1 instructs the tool to turn the periodic transmission of 3F messages off. The step then goes to the step indicated by the value in G1 ('02').

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 03 | 28 | 10 00 03 00 | 68 04 FD 14 FF 16 00 00 00 00 |

Step 3 is an example of an Op-Code 28, which is used to disable normal communications. The 10 in AC0 is used by the tool as the target address of the mode 28 message. The 00 in AC1 instructs the tool to disable normal class 2 communications. The 03 in AC2 is used as a time-out (3 * 50ms) value used while waiting for multiple devices to respond to the mode 28 message. The variable time-out should be used whenever a message is sent to all devices ("FE") and the default time-out of 250ms is not acceptable. The tool will keep looping while it receives successful mode 28 responses or until the time-out value is exceeded without another response being returned.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 09 | A0 | 10 FE 05 00 | AA 0A BB 12 FF 0D 00 00 00 00 |

Step 9 is an Op-Code A0 that is used to request high-speed communications. The 10 is used by the tool as the target address of the mode A0 message. The 05 in AC2 is used by the tool to determine the number of 50ms increments to wait (5 * 50ms) for all devices to respond to the mode A0 message. In this case, the tool will process all responses until an error is received or 250ms have elapsed without another response. The 00 in AC3 instructs the tool to look at response byte 5 for the next goto step to execute.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 1A | B0 | 10 02 00 83 | 73 26 74 28 FF 29 00 00 00 00 |

Step 1A is an example of an Op-Code B0 which is used to download routines to RAM using mode 36 messages. The value in AC0 is used as the target address in the mode 36 message. The value in AC1 indicates the routine number, from the routine section of the Utility File, to download to the control module. The tool parses through the Utility File to find the routine corresponding to the value in AC1. The value in AC3 is used to determine the maximum packet length and transmit type to use in the message. Since the value is 83, which is greater than 80, the tool uses 4000 bytes as the maximum packet length.  The 83 also indicates to the tool to send the mode 36 messages with a transmit type of download and execute.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 06 | B4 | 10 01 03 01 | 73 07 FF 09 00 00 00 00 00 00 |

Step 06 is an example of an Op-Code B4 that is used to download calibrations to the control module using mode 36 messages. The value in AC0 is used as the target address in the mode 36 message. The value in AC1 corresponds to the calibration ID for the calibration to be downloaded. The value in AC2 identifies the routine number to use as a data prolog while downloading the calibration to the control module. The value in AC3 is used to determine the type of download strategy to use during programming. In this example, 01 indicates the tool should use car platform strategies with two byte addressing.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 07 | EE | 01 00 00 00 | 00 00 00 00 00 00 00 00 00 00 |

This step is executed if an error has occurred during the programming process. The 01 in AC0 indicates that a message instructing the user to replace the control module should be displayed on the error screen.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| 08 | FF | 00 00 00 00 | 00 00 00 00 00 00 00 00 00 00 |

This step is indicates a successful completion of the programming process.

## Appendix C - KWP2000 Interpreter Example

Example Utility File (with Control Routines)

0A94000000F7349900000000000200B80002000020000080010110F1000000020000000000000000028110000000C103FF09000000000000038310000000C304FF09000000000000042702000000670537083308FF0900000531030000007106FD08FF09000000000634000000007407FF09000000000000079301000000730AFD083608FF09000008FB010300000004000900000000000009EE01000000000000000000000000000AFF000000000000000000000000000000004000001A8932A5B399CDD44E75345E86E5CB72363536DEA4324EDAC43645000020000005908AB35634

Following are the explanations of how a reprogramming tool would flow through the lines of the Interpreter Instructions from the example Utility File above.  The first highlighted section above represents the Header Information.  The middle section (not highlighted) contains the Interpreter Instructions.  The second highlighted section is the Control Routine section.  In this example the Control Routine section contains two routines.  The example data is not for a particular ECU it is strictly for explanatory purposes.  The data in the Interpreter Instructions is hexadecimal values.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | AC0 AC1 AC2 AC3 | G0-G1 G2-G3 G4-G5 G6-G7 G8-G9 |
| 01 | 01 | 10 F1 00 00 | 00 02 00 00 00 00 00 00 00 00 |

Step $01 is a 01 Op-Code (Setup Global Variables).  The tester sets the global target byte to the value in AC0, or $10, and the global source byte to the value in AC1, or $F1.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | AC0 AC1 AC2 AC3 | G0-G1 G2-G3 G4-G5 G6-G7 G8-G9 |
| 02 | 81 | 10 00 00 00 | C1 03 FF 09 00 00 00 00 00 00 |

Step $02 is an example of a Start Communications Op-Code (81).  If the positive response service ID of $C1 is returned this step goes to step $03.  For any other response values the tester uses the $FF goto value and jumps to step $09.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | AC0 AC1 AC2 AC3 | G0-G1 G2-G3 G4-G5 G6-G7 G8-G9 |
| 03 | 83 | 10 00 00 00 | C3 04 FF 09 00 00 00 00 00 00 |

Step $03 is an example of an Access Communication Parameter Op-Code (83) which is used to setup the optimum reprogramming times.  If the positive response service ID of $C3 is returned this step goes to step $04.  For any other response values the tester uses the $FF goto value and jumps to step $09.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | AC0 AC1 AC2 AC3 | G0-G1 G2-G3 G4-G5 G6-G7 G8-G9 |
| 04 | 27 | 02 00 00 00 | 67 05 37 08 33 08 FF 09 00 00 |

Step $04 is an example of a Security Access Request Op-Code (27) which is used to unlock an ECU for reprogramming.  The value in AC0 represents the algorithm that is used on the seed to determine the key.  The tester uses the response bytes to determine the next step to execute.  If the positive response byte service ID of $67 is returned this step goes to step $05.  If the security request fails for either a "required time

(continued)

delay not expired" or "security access denied" error this step jumps to step $08.  For any other response values the tester uses the $FF goto value and jumps to step $09.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | AC0 AC1 AC2 AC3 | G0-G1 G2-G3 G4-G5 G6-G7 G8-G9 |
| 05 | 31 | 03 00 00 00 | 71 06 FD 08 FF 09 00 00 00 00 |

Step $05 is an example of a Start Routine by Local Identifier Op-Code (31).  This step is used to start executing a routine that is embedded in the ECU's memory.  The local Identifier of the routine to execute is contained in AC0; this example starts routine $03.  If the positive response service ID of $71 is returned this step goes to step $06.  If a "No Communications" error occurred then this step goes to $08.  For any other response values the tester uses the $FF goto value and jumps to step $09.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | AC0 AC1 AC2 AC3 | G0-G1 G2-G3 G4-G5 G6-G7 G8-G9 |
| 06 | 34 | 00 00 00 00 | 74 07 FF 09 00 00 00 00 00 00 |

Step $06 is an example of a Download Request Op-Code (34).  If a positive response service ID of $74 is returned this step goes to step $07.  For any other response values the tester uses the $FF goto value and jumps to step $09.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | AC0 AC1 AC2 AC3 | G0-G1 G2-G3 G4-G5 G6-G7 G8-G9 |
| 07 | 93 | 01 00 00 00 | 73 0A FD 08 36 08 FF 09 00 00 |

Step $07 is an example of a Transfer Calibration Op-Code (93) which is used to transfer calibration files to an ECU.   This step will transfer the calibration with a calibration ID equal to the value in AC0 ($01).  If the positive response byte $73 is returned indicating the ECU is ready for the next block this Op-Code continues with the transfer until the entire calibration is transferred to the ECU.  When the calibration is completely transferred if a positive response code $73 is returned this step goes to step $0A.  If either a "No Communications" or a "Block Transfer Data Checksum" error occurs then this step goes to $08.  For any other response values the tester uses the $FF goto value and jumps to step $09.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | AC0 AC1 AC2 AC3 | G0-G1 G2-G3 G4-G5 G6-G7 G8-G9 |
| 08 | FB | 01 03 00 00 | 00 04 00 09 00 00 00 00 00 00 |

Step $08 is an example of a Set and Decrement Counter Op-Code (FB) which is used to perform loop processing in a Utility File.  This Op-Code sets or decrements (if counter was previously set) counter $01 (AC0).  This step will goto step $04 (G1) if counter $01 is greater than zero otherwise this step will goto step $09 (G3) if counter $01 is zero.  To reset a counter, so it can be reused for another loop, a $FD Op-Code should be issued for the counter number to reset.

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | AC0 AC1 AC2 AC3 | G0-G1 G2-G3 G4-G5 G6-G7 G8-G9 |
| 09 | EE | 01 00 00 00 | 00 00 00 00 00 00 00 00 00 00 |

Step 09 is an End with Error Op-Code (EE) that is used to indicate a reprogramming event ended with an error.  The $01 in AC0 is used to toggle the display of a message instructing the user to replace the ECU on the error screen ($01 - display message, $00 - do not display message).

| Step | Op-Code | Action Fields | Goto Fields |
| --- | --- | --- | --- |
| XX | OP | AC0 AC1 AC2 AC3 | G0-G1 G2-G3 G4-G5 G6-G7 G8-G9 |
| 0A | FF | 00 00 00 00 | 00 00 00 00 00 00 00 00 00 00 |

This step is an example of a Programming Successful Op-Code (FF) that is used to indicate a successful completion of a reprogramming event.

## Appendix D – GMLAN Utility File Guidelines

GMLAN

Utility File Guidelines

Version 1.3

Rick Kacel

#### Revision History (Appendix D:  GMLAN Utility File Guidelines)

| Version | Date | Author | Comments |
| --- | --- | --- | --- |
| 1.0 | 20Aug2004 | Rick Kacel | Initial Release |
| 1.1 | 19Aug2004 | Andrea Merrick | Added assumption |
| 1.2 | 24Aug2004 | John Cline | Fixed Flowcharts and miscellaneous changes |
| 1.3 | 24Sept2004 | Erwin Frank | Several Modifications |
|  | 06Mar2006 | Setterlun | 256 Byte “internal data buffers” – update. |
|  | 24Jan2012 | Setterlun | Name reference ‘SPO’ – to – ‘CCA’. |
|  | 13Mar2013 | Setterlun | Add Note to assumptions page. |

Related Documents

GMW3110 GMLAN Enhanced Diagnostic Test Mode Specification

Version 1.4, 1.5, 1.6

Service Programming System (SPS) Interpreter Programmers Reference Manual

Version Date: November 13, 2003

#### Assumptions

This file represents the programming of a module with one Operating Software module and 2 Calibration Software modules.  Please use this as an example for setting up a utility program.

Note:  		This example does not take into account reading the ProgrammingErrorCode DID ($F1)

upon a failed programming event - - - Global A GMLAN ECUs that employ the Global A

Secure Bootloader design (digitally signed OpSoftware and Calibration files).

The ECUs security algorithm number ($00 - $FF) has already been assigned by GM CCA for levels 1&2.

The example ECU is a simple, straightforward implementation.

The ECU has only one type of Flash memory.  A Flash routine, included in the Utility file, is downloaded to the ECU and handles the programming of the Operating software and Calibration modules.

The Utility file is being executed to program the ECU either for the first time, or as an update to the Operating software or Calibration modules.

If the Operating software for the ECU is downloaded, all Calibration modules for the ECU will be downloaded.  This ensures consistency.

If one Calibration module needs to be updated, all Calibration modules for the ECU will be downloaded.

The VIN number, the Tester Serial number, and the Programming date need to be programmed into ECU memory after a successful programming event.

The ECU supports the GMLAN CAN protocol.

The ECU sends a $7F $31 response if the software part number is requested and the software has not been programmed in the box.

The Module IDs of the calibration files should be assigned to align with the order the files will be downloaded.  Module ID $01 is reserved for the primary Operational Software as stated in GMW3110

#### Programming Flow

#### Step $01 – Set Target and Source Address

The Interpreter uses these values for the programming process.  It needs to know the target address (the Diagnostic address) of the ECU being programmed and the source address (the Diagnostic address) of the test tool.  This step will place the values into global memory locations for access as GMLAN messages are output and received by the Interpreter.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $01 | $01 | $11 | $F1 | $00 | $00 | 00 | $02 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode - $01   Setup Global Variables

This step sets up global variables that are used during the programming process.  This is an internal Interpreter command that does not output any messages on the GMLAN bus.

Action Fields

AC0 – Target Address

This is the target address (the Diagnostic address) of the ECU module the tester communicates with.  This value MUST be either the value returned in the response to the service $1A DID $B0 request, or the least significant byte of the SPS_TYPE_C CAN ID as described in GMW 3110, Chapter 11.

AC1 – Source Address

This is the source address (the Diagnostic address) of the tester that is programming the ECU.  This value is not required for GMLAN implementations, but is placed for consistency with other protocols.

AC2 – Not Used

Set to $00.

AC3

Set to $00. Goto Fields

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

Always a $00 on return.  Go to the next step.

G2-G3, G4-G5, G6-G7, G8-G9 – Not used

#### Step $02 “Start”– Perform Seed and Key

This step unlocks an ECU by performing the Diagnostic security services on the ECU.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $02 | $27 | $68 | $00 | $00 | $00 | $67 | $03 | $FD | Retry1 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode - $27   Security Access

This Interpreter command sends out two Diagnostic Commands to the ECU.  The first command is Diagnostic service $27 SecurityAccess with subfunction 1 – SPSrequestSeed.  The ECU returns its seed value to the tester.  Next, Diagnostic service $27 SecurityAccess with subfunction 2 – SPSsendKey is sent to the ECU that contains the key to unlock the ECU based on the Security Algorithm number that was previously assigned (see assumptions).

Action Fields

AC0

Security Algorithm number previously assigned or known in the range of $00 - $FF.

AC1

Set to $00.

AC2 – Not used

Set to $00.

AC3

A $00 tells the Interpreter to disable sending the key if the seed returned by the ECU is 0, indicating that the ECU is not currently protected.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response $67 is received, then go to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to “Retry1” step.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to the “Error” step

G6-G7, G8-G9 – Not used

#### Step $03 “SetUp” – Prepare to Download Flash Routine #1 from Utility File

The Flash routine resides within the Utility file archive, and is given a numeric value (1, 2, etc).  The routine is downloaded from the Utility file to the ECU, and erases and writes data to the Flash memory device.  It is downloaded and placed into RAM by the bootloader software.  The flash routine usually has two components: – one that erases memory and one that writes memory.  The Utility file has information about the length of the routine.  The Bootloader has information about each of the components.

In some cases the Flash routine is embedded into the Bootloader software and permanently resides in the ECU.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $03 | $34 | $00 | $01 | $00 | $00 | $74 | $04 | $FD | Retry3 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode - $34   Request Download

This command outputs the Diagnostic service $34 RequestDownload message to the ECU.  When it is received by the ECU, the ECU jumps to the Bootloader software (if not already there) and indicates that a download of data is about to occur.  This command must be executed prior to the Block Transfer to RAM command.

Action Fields

AC0

AC0 is used in the $34 service for the dataFormatIdentifier field.  It tells the ECU the format of the data to be downloaded.  In this example, the data is unencrypted and uncompressed ($00).

AC1

AC1 is the Routine number identifier so that the Interpreter retrieves the correct Flash routine from the Utility file.  In this example the routine number is 1.

AC2

Set to $00

AC3

The upper nibble of AC3 tells the Interpreter where to get the length.  In this case it is retrieved from the Routine number specified in AC1 ($0X).

The lower nibble of AC3 determines how many bytes represent the address.  In this example, the address length is specified by the Utility file header in the ‘Type of Addressing’ field ($X0).

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response $74 is received continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to “Retry3” step.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $04 – Download Flash Routine #1 from Utility File

Flash Programming Routine 1 is downloaded to the ECU and stored in RAM memory.  This Interpreter command outputs a series of Diagnostic service TransferData $36 messages to download the data.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $04 | $B0 | $00 | $01 | $00 | $00 | $76 | $05 | $FD | Retry3 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode - $BO   Block Transfer to RAM

This command downloads the specified software module to the ECU.  The downloading of data to the module is made up of many Diagnostic service TransferData $36 messages.  The number of messages is dependent on the length of the data to download.

Action Fields

AC0

Not used when Flash routines are downloaded into the ECU from the Utility File.  Set to $00.

AC1

Specifies the Flash routine number to be downloaded.  In this example, the Flash routine is embedded in Routine #1.

AC2

The upper nibble specifies the subfunction value for the TransferData $36 service.  It determines what action to take after the download is complete.  In this example the Flash routine is being downloaded, not executed, so it is set to download only ($0X).

The lower nibble determines if the address is incremented or held constant.  Since the Flash routines are being downloaded to RAM, the value ($X0) is set to increment the address for storage after every $36 message.  The address is incremented by the number of bytes in each download.

AC3

The upper nibble of AC3 tells the Interpreter where to get the start address.  In this case it is retrieved from the Routine number specified in AC1 ($0X).

The lower nibble of AC3 determines how many bytes represent the address.  In this example, the address length is specified by the Utility file header in the ‘Type of Addressing’ field ($X0).

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response ($76) is received, continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to “Retry3” step.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $05 – Request the Operating Software Part Number

Step 5 retrieves the Operating Software Part Number and stores the value in Data Storage #1.  This value is used for comparison in the next step to determine if the Operating software needs updating.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $05 | $1A | $C1 | $01 | $00 | $00 | $5A | $06 | $31 | ProgOp | $FD | Retry3 | $FF | Error | 00 | 00 |

OpCode - $1A Read Data By Identifier

This Interpreter command requests data from DIDs in the ECU by invoking Diagnostic service ReadDataByIdentifier $1A.  The software part number of the specified ECU is returned.

Action Fields

AC0

Set to $C1, which is the corporate DID to retrieve the Operating software part number from the ECU.  $C1 specifies software module 1, which is always the Operating software.

AC1

Specifies the buffer in the Interpreter program to store the software part number information.  In this example, it is Data Buffer #1.

AC2 – Not Used

Set to $00.

AC3

AC3 determines if a 256-byte or a 2-byte data buffer is to be used.  In this example, a 256-byte buffer is being used since the part number is more than two bytes.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response $5A is received, then a part number was retrieved.  Continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If a RequestOutOfRange $31 negative response is returned, jump to the programming of the Operating software and skip comparison.  The Operating software was never programmed into this ECU.

G4-G5 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to “Retry3” step.

G6-G7 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G8-G9 – Not used

#### Step $06 – Evaluate the Application Software Part Number Value

Compares the value of the Operating software part number retrieved from the ECU with the Module part number stored in the VIT2 table of the programming application.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $06 | $53 | $01 | $01 | $01 | $00 | $00 | ProgCal | $00 | $07 | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $53 Compare Data

This Interpreter command compares the part number retrieved from the ECU and stored in Data Buffer #1 with the part number for Module 1 stored in the VIT2 table.  The part number in the VIT2 table is in ASCII format.  Please see the Interpreter.doc file for more information on the VIT2 table.

Action Fields

AC0

Set this to the Data Buffer to be used in the comparison.  In this example, Data Buffer #1 holds the part number retrieved from the ECU.

AC1

Specifies the Module ID to use for the comparison.  In this example, the Operating software part number is being compared, so the module ID is always 1.

AC2

Set AC2 to $01 if the part number is reported by the ECU as a 4 byte hex value.  Set to $00 if the part number is reported out in ASCII format.  In this example, it is reported by the ECU in hexadecimal and must be converted for comparison.

AC3

The value in AC3 tells the Interpreter where to get the part number for comparison.  Set to $00 for this example to tell the interpreter to get it from the VIT2 table.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If the values match, then the Interpreter jumps to the step indicated in G1.  In this example, the Operating software programming is skipped and the Calibrations modules are programmed.

G2-G3 (Return Code/Jump To Step Number)

If the values don’t match, then the Interpreter jumps to the step indicated in G3.  In this example, the next step is executed, which will begin the programming of the Operating software and Calibration modules.

G4-G5, G6-G7, G8-G9 – Not used

#### Step $07 “ProgOp” – Set the Global Header Length

Set the length of the Operating software header that will be sent in the first packet of data to the ECU.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $07 | $F3 | $00 | $00 | $32 | $00 | 00 | $08 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $F3 Set Global Header Length

In most cases, this command is not required.  This command sets the length of the header for the Operating software.  Setting the global header length allows the tool to send a file to the ECU in two parts.  The first part is the header whose length is defined by the $F3 op-code, and the second part is the rest of the Operating software.  If you don't specify the header length (or set it to 0), then the file is downloaded without a break between the header and the program data.

It is sometimes necessary to begin downloading the data with a new mode $36 message at the point in the file where the actual program would begin getting written to flash.  One instance where this is needed is if the microprocessor cannot put the header at the beginning of the code space.  Certain micros have their interrupt table at the beginning of flash memory.  They download the header first, flash program it to memory (or have a duplicate copy embedded in the Operating software) to support reprogramming, and then flash program the Operating software at the beginning on the flash memory.

Action Fields

AC0 - AC3

Specifies the length of the header.

AC0 MidHigh byte of header length

AC1 MidLow byte of header length

AC2 Low byte of header length

AC3 High byte of header length (allows 4 byte length)

In this example the header is specified as $32 bytes long.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

Go to the step specified in G1, in this example the next step.

G2-G3, G4-G5, G6-G7, G8-G9 – Not used

#### Step $08 – Prepare to Download the Operating Software

This step prepares the ECU to receive the data associated with the Operating software.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $08 | $34 | $00 | $00 | $00 | $10 | $74 | $09 | $FD | Retry3 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode $34 Request Download

This command outputs the Diagnostic service RequestDownload $34 message to the ECU.  When it is received by the ECU, the ECU jumps to the Bootloader software (if not already there) and indicates that a download of data is about to occur.  This command must be executed prior to the Block Transfer to RAM command.

Action Fields

AC0

AC0 is used in the $34 service for the dataFormatIdentifier field.  It tells the ECU the format of the data to be downloaded.  In this example, the data is unencrypted and uncompressed.

AC1

AC1 is set to $00 in this example based on the value in AC3.

AC2

Set to $00.

AC3

The upper nibble of AC3 tells the Interpreter where to get the length.  In this case it’s from the header of the Utility file ($1X).

The lower nibble of AC3 determines how many bytes represent the length.  In this example, the address length is specified by the Utility file header in the ‘Type of Addressing’ field ($X0).

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response $74 is received continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to “Retry3” step.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $09 – Download the Operating Software

The Operating software is downloaded to the ECU and programmed into memory.  This Interpreter command outputs a series of Diagnostic service TransferData $36 messages to download the data.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $09 | $B0 | $01 | $00 | $01 | $10 | $76 | $0A | $FD | Retry3 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode - $BO   Block Transfer to RAM

This command downloads the specified Operating software to the ECU.  The downloading of data to the ECU is made up of many Diagnostic service TransferData $36 messages.  The number of messages is dependent on the length of the data to download.

Action Fields

AC0

Set to the Module ID number of the data to be downloaded.  The Operating software is always set to $01.

AC1

Set to $00.  This is not a Routine being downloaded.

AC2

The upper nibble specifies the subfunction value for the TransferData $36 service.  It determines what action to take after the download of data is complete.  In this example the Operating software is being downloaded, so it is set to download only ($0X).

The lower nibble determines if the address is incremented or held constant.  In this example, it is held constant ($X1).  The ECU is placing the data where it belongs and wants the $36 message to put each block of data into the same location in RAM for processing.

AC3

The upper nibble of AC3 tells the Interpreter where to get the start address.  In this case it’s from the header of the Utility File ($1X).

The lower nibble of AC3 determines how many bytes represent the address.  In this example, the address length is specified by the Utility file header in the ‘Type of Addressing’ field ($X0).

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response ($76) is received, continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to “Retry3” step.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $0A “ProgCal” – Set the Global Header Length

Since step $07 set this to the size of the Operating software header, this will cause the Calibration file header to be downloaded contiguously with the rest of the calibration file.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $0A | $F3 | $00 | $00 | $00 | $00 | $00 | $0B | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $F3 Set Global Header Length

In most cases, this command is not required.  In this example, step $07 sets the header length so that the Operating software downloads the header separate from the program data.  Setting all the AC fields to a $00 will default the system to a one part download.

Action Fields

AC0, AC1, AC2, AC3

Specifies the length of the header.  Set to $00 $00 $00 $00 in this example to reset the value.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

Go to the step specified in G1, in this example the next step.

G2-G3, G4-G5, G6-G7, G8-G9 – Not used

#### Step $0B – Prepare to Download the First Calibration Module

This step prepares the ECU to receive the data associated with the first Calibration file.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $0B | $34 | $00 | $00 | $00 | $10 | $74 | $0C | $FD | Retry3 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode $34 Request Download

This command outputs the Diagnostic service RequestDownload $34 message to the ECU.  This command must be executed prior to the Block Transfer to RAM command.

Action Fields

AC0

AC0 is used in the $34 service for the dataFormatIdentifier field.  It tells the ECU the format of the data to be downloaded.  In this example, the data is unencrypted and uncompressed.

AC1

AC1 is set to $00 in this example based on the value in AC3.

AC2

Set to $00.

AC3

The upper nibble of AC3 tells the Interpreter where to get the length.  In this case it’s from the header of the Utility file ($1X).

The lower nibble of AC3 determines how many bytes represent the length.  In this example, the address length is specified by the Utility file header in the ‘Type of Addressing’ field ($X0).

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response $74 is received continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to “Retry3” step.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $0C – Download the First Calibration Module

The first Calibration module is downloaded to the ECU and programmed into memory.  This Interpreter command outputs a series of Diagnostic service TransferData $36 messages to download the data.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $0C | $B0 | $02 | $00 | $01 | $10 | $76 | $0D | $FD | Retry3 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode - $BO   Block Transfer to RAM

This command downloads the specified Calibration module to the ECU.  The downloading of data to the module is made up of many Diagnostic service TransferData $36 messages.  The number of messages is dependent on the length of the data to download.

Action Fields

AC0

Set to the Module ID number of the data to be downloaded.  The Module ID follows in order after the Operating software Module ID 1.  In this case, the value is 2

AC1

AC0 set to $00.  This is not a Routine being downloaded.

AC2

The upper nibble specifies the subfunction value for the service TransferData $36.  It determines what action to take after the download of data is complete.  In this example the first Calibration module is being downloaded, so it is set to download only ($0X).

The lower nibble determines if the address is incremented or held constant.  In this example, it is held constant ($1).  The ECU is placing the data where it belongs and wants the $36 message to put each block of data into the same location in RAM for processing.

AC3

The upper nibble of AC3 tells the Interpreter where to get the start address.  In this case it’s from the header of the Utility file ($1X).

The lower nibble of AC3 determines how many bytes represent the address.  In this example, the address length is specified by the Utility file header in the ‘Type of Addressing’ field ($X0).

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response ($76) is received, continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to “Retry3” step.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $0D – Prepare to Download the Second Calibration Module

This step prepares the ECU to receive the data associated with the second Calibration file.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $0D | $34 | $00 | $00 | $00 | $10 | $74 | $0E | $FD | Retry3 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode $34 Request Download

This command outputs the Diagnostic service RequestDownload $34 message to the ECU.  This command must be executed prior to the Block Transfer to RAM command.

Action Fields

AC0

AC0 is used in the $34 service for the dataFormatIdentifier field.  It tells the ECU the format of the data to be downloaded.  In this example, the data is unencrypted and uncompressed.

AC1

AC1 is set to $00 in this example based on the value in AC3.

AC2

Set to $00.

AC3

The upper nibble of AC3 tells the Interpreter where to get the length.  In this case it’s from the header of the Utility file ($1X).

The lower nibble of AC3 determines how many bytes represent the length.  In this example, the address length is specified by the Utility file header in the ‘Type of Addressing’ field ($X0).

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response $74 is received continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to “Retry3” step.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $0E – Download the Second Calibration Module

The second Calibration module is downloaded to the ECU and programmed into memory.  This Interpreter command outputs a series of Diagnostic service TransferData $36 messages to download the data.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $0E | $B0 | $03 | $00 | $01 | $10 | $76 | $0F | $FD | Retry3 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode - $BO   Block Transfer to RAM

This command downloads the specified Calibration module to the ECU.  The downloading of data to the module is made up of many Diagnostic service TransferData $36 messages.  The number of messages is dependent on the length of the data to download.

Action Fields

AC0

Set to the Module ID number of the data to be downloaded.  In this example, the Module ID is 3.

AC1

AC0 set to $00.  This is not a Routine being downloaded.

AC2

The upper nibble specifies the subfunction value for the service TransferData $36.  It determines what action to take after the download of data is complete.  In this example the second Calibration module is being downloaded, so it is set to download only ($0X).

The lower nibble determines if the address is incremented or held constant.  In this example, it is held constant ($1).  The ECU is placing the data where it belongs and wants the $36 message to put each block of data into the same location in RAM for processing.

AC3

The upper nibble of AC3 tells the Interpreter where to get the start address.  In this case it’s from the header of the Utility file ($1X).

The lower nibble of AC3 determines how many bytes represent the address.  In this example, the address length is specified by the Utility file header in the ‘Type of Addressing’ field ($X0).

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response ($76) is received, continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to “Retry3” step.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $0F “ProgState” – Request the Programmed State

Step $0F requests the programmed status of the ECU and stores it into Data Storage #2.  This value is used for comparison in the next step to determine programming was successful.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $0F | $A2 | $02 | $00 | $00 | $00 | $E2 | $10 | $FD | Retry2 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode - $A2 Report Programmed State and Save Response

This command outputs the Diagnostic service ReportProgrammedState $A2 message to the ECU.  The programmed state of the specified ECU is returned and stored.

Action Fields

AC0

Specifies the 2-byte buffer in the Interpreter program to store the programming state information.  In this example, it is Data Buffer #2.

AC1, AC2, AC3 – Not Used

Set to $00

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response $E2 is received, then the programmed state was returned.  Continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to step “Retry2”.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $10 – Evaluate the Programmed State Value

Compares the 2-byte value of the programmed state in Data Storage 2 to the “fully programmed’ value ($00) as defined in GMW 3110.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $10 | $50 | $02 | $00 | $00 | $00 | 00 | Success | 00 | Error | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $50 Compare Data

This Interpreter command compares the programmed state retrieved from the ECU and stored in Data Buffer #2 with the ‘fully programmed’ value.

Action Fields

AC0

Set this to the Data Buffer to be used in the comparison.  In this example, Data Buffer #2 holds the programming state retrieved from the ECU.

AC1, AC2

AC1 is the most significant byte (MSB), and AC2 is the least significant byte (LSB) of the vale used for comparison.  In this example, the ‘Fully Programmed” value of $0000 is used.

AC3 – Not used

Set to $00

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If the values match, then the Interpreter jumps to the step indicated in G1.  In this example, the ‘Success’ step.

G2-G3 (Return Code/Jump To Step Number)

If the values don’t match, then the Interpreter jumps to the step indicated in G3.  In this example, the ‘Error’ step.

G4-G5, G6-G7, G8-G9 – Not used

#### Step $11 “Part 2” – Write the VIN

This is the second part of the programming of an ECU.  Part one of a utility file ends when either an ‘end with success’ or ‘end with error’ op-code is encountered.  If part one ended with success, a reset and request for CAN IDs will be sent, and part 2 of the Utility file will be executed.  The step number for part two is stored in the Utility file header in the ‘Type of Header/Utility’ field.  If this is set to 0 then there is no part two.  Part 2 is concluded with either an end with success or end with error op-code.

Please Note:

The programming application will automatically send service messages ReturnToNormalMode $20, and ReadDataByIdentifier $1A, DID $B0 (ECUDiagnosticAddress) before Part 2 of the utility file is processed.  If an ECU needs additional time after a reset and prior to the VIN write, a delay should be includes in the Utility file as the first step of Part 2.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $11 | $3B | $90 | $00 | $00 | $00 | $7B | $12 | $FD | Retry4 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode - $3B Write Data By Identifier

This Interpreter command writes data to DIDs in the ECU by invoking Diagnostic service WriteDataByIdentifier $3B.

Action Fields

AC0

Set to $90, which is the corporate DID VehicleIdentificationNumber.

AC1

AC1 specifies where to get the VIN data.  When set to $00, the Interpreter uses internal data that is stored by the test tool or the manufacturing device.  The VIN number was previously retrieved from the system (see assumptions).

AC2

Set to $00.

AC3

Set to $00, which tells the Interpreter to use internal data.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response $7B is received, then the VIN was written successfully.  Continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to step “Retry4”.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $12 – Write the Tester Serial Number

This step writes the Tester Serial Number using DID 0x98 into the ECU.  This method can be used to write different types of information to the ECU.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $12 | $3B | $98 | $00 | $00 | $00 | $7B | $13 | $FD | Retry4 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode - $3B Write Data By Identifier

This Interpreter command writes data to DIDs in the ECU by invoking Diagnostic service WriteDataByIdentifier $3B.

Action Fields

AC0

Set to $98, which is the corporate DID RepairShopCodeorTesterSerialNumber.

AC1

AC1 specifies where to get the data.  When set to $00, the Interpreter uses internal data that is stored by the test tool or the manufacturing device.  The data was previously retrieved from the system (see assumptions).

AC2

Set to $00.

AC3

Set to $00 that tells the Interpreter to use internal data.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response $7B is received, then it was written successfully.  Continue to the next step.

G2-G3 (Return Code/Jump To Step Number)

If response $FD is returned, then a timeout has occurred.  Go to step “Retry4”.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $13 – Write the Programming Date

This step is writes the Programming Date using DID 0x99 into the ECU.  This method can be used to write different types of information to the ECU.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $13 | $3B | $99 | $00 | $00 | $00 | $7B | Success | $FD | Retry4 | $FF | Error | 00 | 00 | 00 | 00 |

OpCode - $3B Write Data By Identifier

This Interpreter command writes data to DIDs in the ECU by invoking Diagnostic service WriteDataByIdentifier $3B.

Action Fields

AC0

Set to $99, which is the corporate DID ProgrammingDate.

AC1

AC1 specifies where to get the data.  When set to $00, the Interpreter uses internal data that is stored by the test tool or the manufacturing device.  The data was previously retrieved from the system (see assumptions).

AC2

Set to $00.

AC3

Set to $00 that tells the Interpreter to use internal data.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If a positive response $7B is received, then it was written successfully.

G2-G3

If response $FD is returned, then a timeout has occurred.  Go to step “Retry4”.

G4-G5 (Return Code/Jump To Step Number)

Wildcard $FF.  If any other response is returned, terminate the sequence immediately by jumping to “Error” step.

G6-G7, G8-G9 – Not used

#### Step $14 “Retry1” – Initialize Retry counter and decrement

This is a retry counter that is called if a timeout occurs at step $02 during the seed and key operation.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $14 | $FB | $01 | $03 | $00 | $00 | 00 | $15 | 00 | Error | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $FB Set and Decrement Counter

This is an internal Interpreter command that does not output any messages on the GMLAN bus.  It is used with one of the 20 Interpreter loop counters.  Each loop counter is initially set to $FF (or reset), and the first time this OpCode is called the counter is loaded with the counter value in AC1.  The counter also gets decremented after initialization so that must be accounted for in the total count.  Subsequent calls simply decrement the counter until it reaches zero.

In this example, the retry counter is set to 3 that result in 2 retries.

Action Fields

AC0

This is the loop counter to be used ($00-$13).

AC1

This is the initial value of the counter.

AC2, AC3 – Not used

Set to $00.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If the loop counter is greater than zero, jump to the step indicated in G1.  In this example, the next step.

G2-G3 (Return Code/Jump To Step Number)

If the loop counter equals zero, jump to the step indicated in G3.  In this example, jump to the “Error” step.

G4-G5, G6-G7, G8-G9 – Not used

#### Step $15 – Retry1 Delay

On a retry it is normally desirable to delay for a certain period of time.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $15 | $FC | $01 | $00 | $00 | $00 | $00 | Start | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $FC Delay for nn Seconds

This is an internal Interpreter command that outputs the Tester Present message on the GMLAN bus during the delay.  It waits for the specified amount of time before proceeding.

Action Fields

AC0

AC0 is set to the number of seconds of delay.  In this example, it is 1 second.

AC1, AC2, AC3 – Not used

Set to $00.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

When the time has expired, jump to the step indicated in G1.  In this example it goes to the “Start” step that performs the Seed and Key operations.

G2-G3, G4-G5, G6-G7, G8-G9 – Not used

#### Step $16 “Retry2” – Initialize Retry counter and decrement

This is a retry counter that is called if a timeout occurs at step $0F when the programmed state is requested.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $16 | $FB | $02 | $03 | $00 | $00 | 00 | $17 | 00 | Error | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $FB Set and Decrement Counter

This is an internal Interpreter command that does not output any messages on the GMLAN bus.  It is used with one of the 20 Interpreter loop counters.  Each loop counter is initially set to $FF (or reset), and the first time this OpCode is called the counter is loaded with the counter value in AC1.  The counter also gets decremented after initialization so that must be accounted for in the total count.  Subsequent calls simply decrement the counter until it reaches zero.

In this example, the retry counter is set to 3 that result in 2 retries.

Action Fields

AC0

This is the loop counter to be used ($00-$13).

AC1

This is the initial value of the counter.

AC2, AC3 – Not used

Set to $00.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If the loop counter is greater than zero, jump to the step indicated in G1.  In this example, the next step.

G2-G3 (Return Code/Jump To Step Number)

If the loop counter equals zero, jump to the step indicated in G3.  In this example, jump to the “Error” step.

G4-G5, G6-G7, G8-G9 – Not used

#### Step $17 – Retry2 Delay

On a retry it is normally desirable to delay for a certain period of time.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $17 | $FC | $01 | $00 | $00 | $00 | $00 | ProgState | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $FC Delay for nn Seconds

This is an internal Interpreter command that outputs the Tester Present message on the GMLAN bus.  It waits for the specified amount of time before proceeding.

Action Fields

AC0

AC0 is set to the number of seconds of delay.  In this example, it is 1 second.

AC1, AC2, AC3 – Not used

Set to $00.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

When the time has expired, jump to the step indicated in G1.  In this example it goes to the “ProgSat” step.

G2-G3, G4-G5, G6-G7, G8-G9 – Not used

#### Step $18 “Retry3” – Initialize Retry counter and decrement

This is a retry counter that is called if a timeout occurs for most of the interpreter steps.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $18 | $FB | $03 | $03 | $00 | $00 | 00 | $19 | 00 | Error | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $FB Set and Decrement Counter

This is an internal Interpreter command that does not output any messages on the GMLAN bus.  It is used with one of the 20 Interpreter loop counters.  Each loop counter is initially set to $FF (or reset), and the first time this OpCode is called the counter is loaded with the counter value in AC1.  The counter also gets decremented after initialization so that must be accounted for in the total count.  Subsequent calls simply decrement the counter until it reaches zero.

In this example, the retry counter is set to 3 that result in 2 retries.

Action Fields

AC0

This is the loop counter to be used ($00-$13).

AC1

This is the initial value of the counter.

AC2, AC3 – Not used

Set to $00.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If the loop counter is greater than zero, jump to the step indicated in G1.  In this example, the next step.

G2-G3 (Return Code/Jump To Step Number)

If the loop counter equals zero, jump to the step indicated in G3.  In this example, jump to the “Error” step.

G4-G5, G6-G7, G8-G9 – Not used

#### Step $19 – Retry3 Delay

On a retry it is normally desirable to delay for a certain period of time.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $19 | $FC | $01 | $00 | $00 | $00 | $00 | SetUp | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $FC Delay for nn Seconds

This is an internal Interpreter command that outputs the Tester Present message on the GMLAN bus.  It waits for the specified amount of time before proceeding.

Action Fields

AC0

AC0 is set to the number of seconds of delay.  In this example, it is 1 second.

AC1, AC2, AC3 – Not used

Set to $00.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

When the time has expired, jump to the step indicated in G1.  In this example it goes to the “SetUp” step.

G2-G3, G4-G5, G6-G7, G8-G9 – Not used

#### Step $1A “Retry4” – Initialize Retry counter and decrement

This is a retry counter only used by Part 2 steps if a timeout occurs.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $1A | $FB | $04 | $03 | $00 | $00 | 00 | $1B | 00 | Error | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $FB Set and Decrement Counter

This is an internal Interpreter command that does not output any messages on the GMLAN bus.  It is used with one of the 20 Interpreter loop counters.  Each loop counter is initially set to $FF (or reset), and the first time this OpCode is called the counter is loaded with the counter value in AC1.  The counter also gets decremented after initialization so that must be accounted for in the total count.  Subsequent calls simply decrement the counter until it reaches zero.

In this example, the retry counter is set to 3 that result in 2 retries.

Action Fields

AC0

This is the loop counter to be used ($00-$13).

AC1

This is the initial value of the counter.

AC2, AC3 – Not used

Set to $00.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

If the loop counter is greater than zero, jump to the step indicated in G1.  In this example, the next step.

G2-G3 (Return Code/Jump To Step Number)

If the loop counter equals zero, jump to the step indicated in G3.  In this example, jump to the “Error” step.

G4-G5, G6-G7, G8-G9 – Not used

#### Step $1B – Retry4 Delay

On a retry it is normally desirable to delay for a certain period of time.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $1B | $FC | $01 | $00 | $00 | $00 | $00 | Part2 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $FC Delay for nn Seconds

This is an internal Interpreter command that outputs the Tester Present message on the GMLAN bus.  It waits for the specified amount of time before proceeding.

Action Fields

AC0

AC0 is set to the number of seconds of delay.  In this example, it is 1 second.

AC1, AC2, AC3 – Not used

Set to $00.

Goto Fields

G0-G1 (Return Code/Jump To Step Number)

When the time has expired, jump to the step indicated in G1.  In this example it goes to the “Part2” step.

G2-G3, G4-G5, G6-G7, G8-G9 – Not used

#### Step $1C “Success” – Exit Interpreter with Success

Exit the Interpreter program indicating that the programming was successful.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $1C | $FF | $00 | $00 | $00 | $00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $FF End with SUCCESS

This is an internal Interpreter command that does not output any messages on the GMLAN bus.  The $FF OpCode is used to signal the successful completion of the Utility file part (Part 1 or Part 2).

Action Fields

AC0, AC1, AC2, AC3 – Not used

Set to $00.

Goto Fields

G0-G1, G2-G3, G4-G5, G6-G7, G8-G9 – Not used

#### Step $1D “Error” – Exit Interpreter with Error

Exit the Interpreter program with an error indicating that the programming failed.

| Step | Op-Code | Action Fields |  |  |  | Goto Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC0 | AC1 | AC2 | AC3 | G0-G1 |  | G2-G3 |  | G4-G5 |  | G6-G7 |  | G8-G9 |  |
| $1D | $EE | $00 | $00 | $00 | $00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |

OpCode $EE End with ERROR

This is an internal Interpreter command that does not output any messages on the GMLAN bus.  The $EE Op-Code is used to signal the tool programming software that an error has occurred.  The Interpreter ends processing and returns an error code to the main software.

Action Fields

AC0

AC0 is used by the tool to indicate if the product has to be replaced.  This example, it is set it to $00 signifying an error occurred and the product may not need to be replaced.

AC1, AC2, AC3 – Not used

Set to $00.

Goto Fields

G0-G1, G2-G3, G4-G5, G6-G7, G8-G9 – Not used

#### SPS Editor Output of Interpreter Instructions

Checksum:                0x0         0

ModuleId:                0x0         0

PartNumber:              0x0         0

DesignLevelSuffix:       0x0         0

TypeOfHeaderUtility:     0x0B        11

TypeOfInterpreter:       0x3         3

OffsetToRoutineSection:  0x1E8       488

TypeOfAddressing:        0x4         4

DataAddressInformation:  0xFFFF4000  -49152

DataPacketLength:        0x806       2054

| Label | Step# | Action Fields | Goto Fields |
| --- | --- | --- | --- |
|  | 01 | 01_SetupGlobalVariables (11; F1) | (00:Next) |
| Start | 02 | 27_SecurityAccess (68) | (67:Next; FD:Retry1; FF:Error) |
| SetUp | 03 | 34_RequestDownload (00; 01; 00) | (74:Next; FD:Retry3; FF:Error) |
|  | 04 | B0_BlockTransferToRam (00; 01; 00; 00) | (76:Next; FD:Retry3; FF:Error) |
|  | 05 | 1A_ReadDataByIdentifier (C1; 01; 00) | (5A:Next; 31:ProgOp; FD:Retry3; FF:Error) |
|  | 06 | 53_CompareData (01; 01; 01; 00) | (00:ProgCal; 00:Next) |
| ProgOp | 07 | F3_SetGlobalHeaderLength (00; 00; 32; 00) | (00:Next) |
|  | 08 | 34_RequestDownload (00; 00; 10) | (74:Next; FD:Retry3; FF:Error) |
|  | 09 | B0_BlockTransferToRam (01; 00; 01; 10) | (76:Next; FD:Retry3; FF:Error) |
| ProgCal | 0A | F3_SetGlobalHeaderLength (00; 00; 00; 00) | (00:Next) |
|  | 0B | 34_RequestDownload (00; 00; 10) | (74:Next; FD:Retry3; FF:Error) |
|  | 0C | B0_BlockTransferToRam (02; 00; 01; 10) | (76:Next; FD:Retry3; FF:Error) |
|  | 0D | 34_RequestDownload (00; 00; 10) | (74:Next; FD:Retry3; FF:Error) |
|  | 0E | B0_BlockTransferToRam (03; 00; 01; 10) | (76:Next; FD:Retry3; FF:Error) |
| ProgState | 0F | A2_ReportProgrammedStateSave2 (02) | (E2:Next; FD:Retry2; FF:Error) |
|  | 10 | 50_CompareBytes (02; 00; 00) | (00:Success; 00:Error) |
| Part2 | 11 | 3B_WriteDataByIdentifier (90; 00; 00; 00) | (7B:Next; FD:Retry4; FF:Error) |
|  | 12 | 3B_WriteDataByIdentifier (98; 00; 01; 00) | (7B:Next; FD:Retry4; FF:Error) |
|  | 13 | 3B_WriteDataByIdentifier (99; 00; 00; 00) | (7B:Success; FD:Retry4; FF:Error) |
| Retry1 | 14 | FB_SetAndDecCounter (01; 03) | (00:Next; 00:Error) |
|  | 15 | FC_DelayForSeconds (01; 00; 00) | (00:Start) |
| Retry2 | 16 | FB_SetAndDecCounter (02; 03) | (00:Next; 00:Error) |
|  | 17 | FC_DelayForSeconds (01; 00; 00) | (00:ProgState) |
| Retry3 | 18 | FB_SetAndDecCounter (03; 03) | (00:Next; 00:Error) |
|  | 19 | FC_DelayForSeconds (01; 00; 00) | (00:SetUp) |
| Retry4 | 1A | FB_SetAndDecCounter (04; 03) | (00:Next; 00:Error) |
|  | 1B | FC_DelayForSeconds (01; 00; 00) | (00:Part2) |
| Success | 1C | FF_EndWithSuccess () | () |
| Error | 1D | EE_EndWithError (00) | () |

#### GMLAN Message Flow Diagrams

## Appendix E – UDS Utility File Guidelines

TBD

## Appendix F - Revision Log   (Interpreters Document)

| Name | Date | Description of Revision |
| --- | --- | --- |
| Jay Wells | 03/01/93 | Initial release of document. |
| Jay Wells | 04/15/93 | Removed signature page. |
| Jay Wells | 06/09/93 | Updated B4 Op-Code to reflect changes added for the PCM-66U and the PCM-32U. |
| Jay Wells | 12/08/93 | Completed error descriptions. Added class 2-interpreter example section. Added FE loop response processing for class 2 messages, which will generate multiple responses. This is in Class 2 response processing. Added capability to turn on and off the transmission of a periodic 3F to all devices to keep modules not being communicated with disabled. This is in 01 Op-Code of Interpreter 1. |
| Jay Wells | 12/20/93 | Added more detail to B4 Op-Code concerning packet size. |
| Jay Wells | 03/15/94 | Attempted to clean up the format of the document. Changed the planned implementation of the 3B Op-Code. |
| Jay Wells | 03/24/94 | Initial release of document for review only. |
| Jay Wells | 04/30/94 | Incorporated changes and added functionality based on input from STG-E. Added all Op-Codes to support programming based on final version of KWP2000 - Part 3: Implementation. |
| Jay Wells | 05/13/94 | Sent to STG-E for distribution and final review. |
| Jay Wells | 11/18/94 | Added Op-Code F2 to set global memory size. Modified Op-Codes 34, 90, and 93 per request from Bosch. |
| Jerry Anderson | 2/3/95 | Added Op-Codes 32, 33, 39 and 3A. |
| Jerry Anderson | 04/20/95 | Compared pseudo code to source code and updated where necessary. |
| Jerry Anderson | 06/02/95 | Added 7F response processing to pseudo code for Class 2 response processing. |
| Jay Wells | 7/18/95 | Added Op-Code 02 to allow users to define Op-Codes. Added Op-Code 11 for ECU Reset. Added Op-Code 30 for IO Control by Local ID. Modified Op-Code 10 Programming mode byte to be Action Field 0. |
| Jay Wells | 8/10/95 | Updated the Security Op-Code 27 to reflect actual implementation. |
| Dennis Pesonen | 9/28/95 | Update FB Op-Code examples, Update 3B & 3D sections |
| Dennis Pesonen | 04/22/96 | Modified 3B, AE and FC Op-Codes |
| Dennis Pesonen | 8/02/96 | Update negative response processing for duplicate response codes. |
| Philip Barone | 01/23/97 | Added new Op-Codes 52 and 72. Modified Op-Code 3B to reset a block of memory. |
| Philip Barone | 03/06/97 | Modified Op-Code 3B to write end model number in BCD format. |
| Philip Barone | 07/31/97 | Added Op-Code 31 - Start Diagnostic Routine by Test Number. Modified Op-Code 3B to send a routine from the utility file. |
| Rob | 10/07/97 | Added Op-Code 20 – return to normal |
| Dennis Pesonen | 10/12/97 | Fixed several errors in the B4 Op-Code, the normal and block response processing |
| Dennis Pesonen | 10/24/97 | Revised text for clarification in the following sections: 10, A0, B0, B2, F0, FC Op-Codes and the 7F negative response processing section. Added header byte description (pg. 6-7). Added Programming Algorithm Overview (page 8). |
| Phil Barone | 01/27/98 | Modified Op-Code 34 to support sending simplified message with no extra data. |
| Greg Powell | 09/21/98 | Enhanced and revised the Overview, Architecture and Technical Specifications, Utility File Structure, Reprogramming Algorithm Overview and Op-Code Structure sections. Added utility file examples with supporting step-by-step descriptions. Added UART Op-Codes 00, 02, 03, A3, 0A, C3, D4, D5. Modified UART Op-Codes C0 & D0. Added Class 2 Op-Code 3C. Compared source code to pseudo code for all Op-Codes and made revisions as necessary. Updated the System Errors and Op-Code failures section. Revised the Procedure for adding new Op-Code and Op-Code Testing sections. |
| Greg Powell | 10/6/98 | Updated the Class 2 3B, 3C, 72 and 52 Op-Codes to include Saturn programming information. Made minor formatting changes to page numbers. Revised description of B4 exceptions. |
| Greg Powell | 10/30/98 | Updated the Uart 08, 0A and D4 Op-Codes. Update the Class 2 34 Op-Code. Made minor formatting changes to page numbers. Fixed minor spelling errors. |
| Greg Powell | 11/02/98 | Added the Uart 20 Op-Code. |

| Greg Powell | 12/14/98 | Modified the Class 2 B4 Op-Code by adding exception 04. |
| --- | --- | --- |
| Greg Powell | 1/6/99 | Removed all references to STG and replaced with GM Service and Parts Operations. |
| Greg Powell | 1/12/99 | Added exception 05 to Class2 3B Op-Code. Added the Class 2 14 Op-Code. |
| Dennis Pesonen | 2/18/99 | Added AC3 option in Op-Code 27 for security level 2. Added AC3 option x03 to Op-Code 93. These changes were for SAAB. |
| Dirk Heinemann | 3/23/00 | GMLAN B0 Op-Code Mode 36 Block Transfer to RAM changed. Exceptions have changed from the beta version of the document. |
| John Cline | 4/13/00 | Accepted changes from 3/23/00. Added 04 Byte address to utility file header. Removed Class 2 indication from Op-Code F1. Updated footer for 1993-2000. |
| John Cline | 5/01/00 | Revised Table of Contents to show Interpreter 2 |
| John Cline | 6/07/00 | Add Op-Code 3E and 40 to Class 2 interpreter 1 |
| John Cline | 7/11/00 | Move UART Op-Code A3 to proper location |
| John Cline | 7/11/00 | Remove initials DH from GMLAN Op-Code 34 |
| John Cline | 7/11/00 | Correct description for GMLAN Op-Code F2 |
| John Cline | 7/11/00 | Add KWP2000 Op-Code 02 |
| John Cline | 7/11/00 | Add Op-Code F3 to Communications Independent Op-Codes |
| John Cline | 7/11/00 | Update GMLAN Op-Code B0 description |
| John Cline | 7/11/00 | Update KWP2000 Op-Code 27 for valid security algorithm range |
| John Cline | 7/11/00 | Update UART Op-Code 0C description |
| John Cline | 7/24/00 | Added GMLAN Interpreter to Op-Codes 50, 51, EE, F7, F8, FB, FC, FD and FF |
| John Cline | 7/24/00 | Added Blocks 22, 23 and 24 to Class 2 Op-Code 3B |
| Dirk Heinemann | 8/15/00 | Removed Target ID from GMLAN Op-Codes |
| Dirk Heinemann | 8/15/00 | Added Target ID to GMLAN Setup Global Variables |
| Dirk Heinemann | 8/15/00 | Changed GMLAN Op-Codes 34, B0 exception handling from bit to nibble coded |
| Thomas Walter | 8/22/00 | Added GMLAN example |
| Dirk Heinemann | 8/29/00 | Removed GMLAN Op-Code 27 5 tries 2 second wait logic |
| Dirk Heinemann | 8/29/00 | Added note for FC Op-Code AC1 and AC3 |
| Dieter Rapp | 8/31/00 | Updated Class 2 Op-Code 27 (Perform Security) pseudo-code |
| Dirk Heinemann | 11/13/00 | Added Exception to Mode 1A Read Data by Identifier to identify storage location 2 bytes byte buffer or 10 bytes data buffer. |
| Dirk Heinemann | 4/18/01 | Changed Compare Data Op-Code 52 to 53 (had same value as Class2 Write Checksum to Block 82) |
| Dirk Heinemann | 4/18/01 | Added more detailed description for Op-Code 90 Transfer a Routine to ECU (2 – 3 byte address information) |
| Dirk Heinemann | 4/18/01 | Added more detailed description for Op-Code 93 Transfer a Calibration File to ECU (2 – 3 byte address information) |
| Dirk Heinemann | 6/29/01 | Modified GMLAN Op-Code AE Request Device Control to allow 0 to 5 control bytes |
| Jerry Rainko | 8/3/01 | Documented the block descriptions (AC1) for the Class2 3B Op-Code. |
| Dirk Heinemann | 11/13/01 | Updated KWP2000 3B Op-Code description |
| Dirk Heinemann | 11/13/01 | Fixed GMLAN example step 4 G1 jump to 05 |
| Dirk Heinemann | 11/13/01 | Updated Format of Header definition (Utility File module ID = 0x0000) |
| Dirk Heinemann | 11/14/01 | Added GMLAN Op-Code 3B write multiple DIDs via calibration file support |
| Dirk Heinemann | 12/5/01 | Added more detailed description to Common Op-Code 51 Compare Checksum |
| Jerry Rainko | 1/9/02 | Updated Op-Code 3C logic to reflect what is actually being done. |
| David Setterlun | 01/10/02 | Added Class 2 support for the F3 Op-Code “Set Global Header Length”. Added Global Header Length descriptions & logic to the Class 2 B4 Op-Code. |
| David Setterlun | 02/28/02 | Updated Op-Code $53 to change VIT2 compare reference values used within the “AC1” field to reference the Part #, VIN, information. Previous documentation was incorrect and also updated to allow for future P/N expandability. |
| Dieter Rapp | 04/29/02 | Added KWP2000 Op-Code $84 to Set Communications Parameters. |
| Dieter Rapp | 04/29/02 | Updated KWP2000 Op-Code $3B Write Data by Local ID – added conversion of programming date into a 6 character ASCII string formatted as YYMMDD if AC0 is $00, AC1 is $99, AC2 is $06 and AC3 is $00. |
| Dirk Heinemann | 07/08/02 | Changed GMLAN 3B Op-Code Mode 3B Write Data by Identifier to support access to 20 bytes buffers |
| Dirk Heinemann | 07/08/02 | Added Common 54 Op-Code Change Data |
| Dirk Heinemann | 09/20/02 | Changed F7 Op-Code description to match existing code behaviour (from should never the destination to goto step G1) |
| Jun Bradburn | 09/20/02 | Modified FC Op-Code to indicate support for Delay For Minutes for KWP2000 protocol. |
| David Setterlun | 10/30/02 | Modified Class 2 Op-Code $27 to document retry logic for a TIME DELAY NOT EXPIRED response condition. |
| Dirk Heinemann | 11/21/02 | Added KWP2000, new 23 Op-Code SR 23 to Read Memory by Address and Save Bytes |
| Dirk Heinemann | 11/21/02 | Added KWP2000, 3B Op-Code, new AC2 $02 – Use module for record values |
| Dirk Heinemann | 11/21/02 | Added KWP2000, 10 Op-Code, new AC1 – Baud rate Identifier |

| Dirk Heinemann | 11/21/02 | Added KWP2000, 27 Op-Code, new AC2 Security IDs |
| --- | --- | --- |
| Dirk Heinemann | 12/04/02 | Changed FB Op-Code Set and Decrement Counter description to match existing code behaviour |
| Dirk Heinemann | 12/04/02 | Updated KWP2000 90 Op-Code: SR 36 to Transfer a Routine to ECU – added AC3 $40, $41 |
| Dirk Heinemann | 12/04/02 | Updated KWP2000 93 Op-Code: SR 36 to Transfer a Calibration File to ECU – added AC3 $40, $41, $42, $43, $C0, $C1 |
| Dieter Rapp | 05/19/03 | Updated KWP2000 10 Op-Code: Added baud rate switch |
| Dieter Rapp | 05/19/03 | Updated KWP2000 20 Op-Code: Added baud rate switch to 10400 Bd |
| Dieter Rapp | 05/19/03 | Updated KWP2000 3D Op-Code: new AC3 – Multiple services to download |
| Dieter Rapp | 05/19/03 | Added KWP2000, new F4 Op-Code to Ignore Responses for a Specified Number of Milliseconds |
| Dieter Rapp | 05/19/03 | Updated Common 51 Op-Code: new AC2 – Checksum Type |
| Dieter Rapp | 05/20/03 | Updated Common F7 Op-Code: Changed description to match existing code behaviour (always go to next step) |
| David Setterlun | 22 May 2003 | - Updated common Op-Code $53 (Compare Data) to support 20 (decimal) Part Numbers (from 16). - Updated GMLAN Op-Code $3B (Write Data by Identifier) to support the data identifier 0xCB (End-Model Part Number). - Resolved a few minor spelling/grammar errors – Seemed to have “lost” some of these fixes from Cline (13 Dec. 2002). |
| Dieter Rapp | 06/30/03 | Added UART $C4 Op-Code for Holden Telematics (Download Routine to RAM using Mode 20) |
| Dieter Rapp | 06/30/03 | Added UART $C5 Op-Code for Holden Telematics (Execute Routine using Mode 21) |
| Dieter Rapp | 06/30/03 | Added UART $D6 Op-Code for Holden Telematics (Erase Flash Block using Mode 22) |
| Dieter Rapp | 06/30/03 | Added UART $D7 Op-Code for Holden Telematics (Download Software Module using Mode 23) |
| Dieter Rapp | 06/30/03 | Added UART $D8 Op-Code for Holden Telematics (Execute Software Module using Mode 24) |
| Dieter Rapp | 07/01/03 | Added GMLAN $02 Op-Code for development use only (Send Single Frame) |
| Dieter Rapp | 07/01/03 | Added GMLAN $03 Op-Code for development use only (Re-Initialize Network for Programming) |
| Dieter Rapp | 07/01/03 | Updated GMLAN Op-Code $27 (Security Access): Added exception $01 – send key even if seed is 0. |
| Dieter Rapp | 09/18/03 | Moved Op-Code $F4 from the KWP2000 section to the communication independent section. |
| Setterlun | 10/08/03 | Added Op-Code $F5 (Common Op-Code) – Override Utility File Message Length Field (Changes made based on Dieter’s “18-Sept-2003” interpreters document. Also, updated the copyright date within the footer. |
| John Cline/ Lisa Cleaveland | 11/13/03 | Reformat document - Remove 30 pages without losing content. Remove development only information from document. |
| John Cline | 11/13/03 | Update Copyright Information per GM management direction |
| Dieter Rapp | 03/03/04 | Added GMLAN Op-Code $84 (SetCommunicationParameters) to override STmin |
| Dieter Rapp | 03/03/04 | Updated KWP2000 Op-Code $27 (Security Access): Added special handling if AC1 is $01 – send key even if seed is 0. |
| Dieter Rapp | 04/22/04 | Updated GMLAN Op-Code $27 (Security Access): Added handling of AC1 (security level) and AC2 (algorithm type - GM vs. non-GM). |
| Dieter Rapp | 04/22/04 | Added GMLAN Op-Code $AA Read Data By Packet Identifier. |
| David Setterlun | 08/12/04 | Update Op-Code $53 to support part number module IDs 1 through 99 by adding more VIT2 reference numbers 0x81-0xE3 (Mod IDs 01-99)). Update the copyright date. Update Op-Code $54 to not list $53 as the Op-Code (oops – bug fix). Put in disclaimers stating that the following is not supported for SAE J2534 GMLAN interface types (code is not in VCS 4.0 yet): - GMLAN Op-Code $84 - GMLAN Op-Code $AA - GMLAN Op-Code $27 (Security Level and Security Access support – AC1/AC2) - Common Op-Code $F5 (not supported for GMLAN J2534) |
| David Setterlun | 08/19/04 | Add documentation to Class 2 Op-Code $27 to state that a mode $28 will be sent out to all devices prior to the $27 $01 (update the interpreter documentation to be the same as the code). And also add support for AC3, so that if AC3 is set to 0x01, then the mode $28 will not be sent out. |

| John Cline/ Lisa Cleaveland | 12/01/04 | Replace Appendix D with updated GMLAN utility file guidelines document. Correct UART Op Code D7 AC1: description to reflect calibration number |
| --- | --- | --- |
| Dirk Heinemann | 03/08/05 | Corrected FC Op-Code description for Tester Present and Delay For Minutes. |
| David Setterlun | 04/21/05 | - Update the copyright date (2005). - Update the complete “Interpreter 1 – Class 2” section to be more consistent with the newer interpreter documentation and to better document the logic within the VCS code. - Update the complete “Independent opcode” (common) section to be more consistent with the newer interpreter documentation and to ensure that it “matches” the VCS code. - Removed the disclaimers stating that certain functionality is “not supported for SAE J2534”. It is now supported with VCS 4.1: GMLAN Op-Code $84, GMLAN Op-Code $AA, GMLAN Op-Code $27 (Security Level and Security Access support – AC1/AC2), Common Op-Code $F5 - Repage the TOC. |
| Setterlun | 04/28/05 | - Reformat the TOC (issues with Appendix D). *** Be carfull when updating Appendix D (GMLAN Utility File Guidelines) because of a FORMATTING conflict (Header 1, Header 2, … usage) - - - DON’T DO IT ! |
| Dieter Rapp | 10/19/05 | Added GMLAN Op-Code $20 Return to Mormal Mode. |
| Dieter Rapp | 10/19/05 | Fixed typo in GMLAN Op-Code $34 Request Download, changed AC3 description: Replaced “0X - Use length from the routine indicated by AC2 (2 bytes)” by “0X - Use length from the routine indicated by AC1 (2 bytes)”. The routine is indicated by AC1, not by AC2. |
| Setterlun | 10/20/05 | Four items (all very minor): - Misc. formatting pagebreak items, “(continued)” belongs on top of the next page. - Add the “bubble comments” by Dirk and Dieter into document text. - Put a disclaimer in the “System Errors and Op-Code Failures” section. - Updated comments for OpCodes that reference an “index” to state if that index is zero based or not (list acceptable values). OpCodes: UART OpCode $C2; Class 2 OpCode $B2; KWP2000 OpCodes $23, $71, $78; GMLAN OpCodes $3B; Common OpCode $54. |
| Cline | 1/13/2006 | - Update Copyright for 2006 - Add 52 Op-Code - Add Appendix G – 54 Op-Code Additional Information - Modify Caution message in 20 Op-Code - GMLAN 3B Op-Code Use VIN digits 10-17 (exception 4X) |
| Cline | 1/19/2006 | Add appendix H – GATS Option string processing |
| Cline | 1/20/2006 | New RPO Op Code changed from 52 to 55 due to conflict with Class 2 Op Code 52 |
| Wist | 2/14/06 | Modified document to expand 20 byte internal buffers to 256 byte buffers. Affected op-codes include KWP2000 op-code $23, GMLAN op-codes $1A, $3B, and $AA. Communications independent op-codes affected include $51, $53, and $54. Also made changes to Appendix G and Appendix H where the 20 byte buffers were referenced. |
| Wist | 2/14/06 | Modified GMLAN op-code $34 pseudo code to correct refererence to AC1 (was AC2) when obtaining routine data. |
| Wist | 2/14/06 | Modified op-code $54 to add new exception that allows data to be read in from a routine and stored into one of the 256 byte buffers. |
| Setterlun | 03/06/06 | - GMLAN Mode 20 Update (ONLY allowed in Part 2 of a utility file). CR1627 - Minor cosmetic updates to the 256 byte buffer changes (from Al). CR1635 - Date/CoverPage/Revision Log/TOC update. - Pseudo code Update OpCode $54 – loading “buffer” data via a routine. - Other very minor updates (cosmetic only) throughout the document. - Appendix G updates (Op-Code $54 changes) – simplify. |
| Setterlun/Cline | 5/09/2006 | - Op Code $FF – bar graph increments to 100% - Op Code $34 GMLAN – length based on calibration option - Op Code $3B Class 2 – add AC3 exception 6 and 7 - Op Code $72 Class 2 – Update for SAE J2534-1 |

| Setterlun/Cline | 6/20/2006 | - Page 235 – correct offset to routine in utility example - Page 235 – correct TypeOfHeaderUtility value to 11 - Added GMLAN op-code $01 fatal error on Target Address FE and TIMEOUT description. - Changed GMLAN op-code $14 description to indicate physical addressed usage. - Changed GMLAN op-code $20 description to indicate development only usage. |
| --- | --- | --- |
| Setterlun/Cline | 8/16/2006 | - Class 2 OpCode $3C and OpCode $3B Use of internal storage buffers to read/write data from these buffers. |
| Kacel/Cline | 9/20/2006 | Appendix D GMLAN message flow steps $12 and $13 correct messages from $34 to $3B |
| Cline/Setterlun | 9/20/2006 | Add GMLAN Op Code $25 for Global A Security Code |
| Cline/Setterlun | 9/20/2006 | Modify all storage buffer values from 00-09 to 0x00 - 0x13 |
| Cline/Setterlun | 10/04/2006 | Add comments to GMLAN mode $27 for “Timer Not Expired – 37” functionality |
| Cline | 4/01/2007 | Remove GMLAN Development Op Codes $02 and $03 |
| Cline | 4/01/2007 | Add GMLAN Op Code $22 – Read data by PID |
| Cline/Koski | 4/01/2007 | Per Devin Koski – modify Op Code $25 action fields AC1, AC2 and AC3 |
| Cline | 4/01/2007 | Add Op Code $84 for Class 2 – Set Communications Parameters – Add delay |
| Cline/Setterlun | 4/01/2007 | - Update KWP2000 and GMLAN Op Code $27 security access information - Add clarification to the statement “from the header” which becomes “from the utility file header” where appropriate |
| J. Cline | 4/04/2007 | Update Op Code $51 text for 20 buffers. Add Class 2 to Op Codes $53 and $54 |
| J. Cline | 4/05/2007 | Spelling and grammar check. |
| J. Cline | 7/12/2007 | GMLAN Op Code $AA – add note concerning optional interface support GMLAN Op Code $84 – add note concerning assembly plant support GMLAN Op Code $3B – Correct AC1 ID for saved bytes (0x00 – 0x13) Page 243 – correct spelling of Cleaveland Class 2 Op Code $3B – Correct AC3 and Example 6 – storage buffer is AC2 |
| Setterlun | 8/15/2007 | Class 2 OpCodes 3B & 3C: Updated these OpCodes to better explain how data is read from and stored to one of the “internal storage locations” - - - specifically regarding the amount of data. |
| Cline/Koski | 8/26/2007 | GMLAN Op Code $25 – Update Security Code Text description for AC2 to Security Code 1 or Security Code 2 |
| Cline/Wist | 12/06/2007 | Add Internal Data Specificers $44 (Tire Type) and $45 (TPIM placard data) to Op Code $53 and Op Code $54 |
| Koski/Wist | 2/29/2008 | Op Code $25 – Update text for improved security code description |
| Cline/Setterlun | 2/29/2008 | GMLAN Op Code $27 – Add Security Table GMLAN-1 information to AC2 and description |
| A. Wist | 2/29/2008 | Op Code $53/$54 – Add Internal Data Specificer $46 (Diesel Injector Mass Adjustment) |
| Setterlun/Wist | 2/29/2008 | Op Code $54 – Add AC2 exception $08 Copy the data from one storage buffer to another |
| Cline/Setterlun | 2/29/2008 | Op Codes $FB/$FD – Increase loop counters from 10 to 20 ( 0x00 – 0x13) |
| Cline | 2/29/2008 | Update copyright for 2008 |
| Cline Setterlun | 4/12/08 | Add Op Code $56 – Plant/Service/FCP Add note to Op Code $53 – numeric part number names in DPS GMLAN Op Code $84 – Add ESCAN link info. Minor Cosmetic Updates based on comments from Dirk (Setterlun) -- OpCode FF, GMLAN OpCode 20, Class 2 OpCode A8 My Update OpCode $53 comment (DPS). |
| Cline/Setterlun | 8/01/2008 | Add Internal Data Specifier support for GMLAN mode $3B per GM AVG request. Add documentation to clarify $3B usage and Data integration into $3B message. |
| Cline | 8/01/2008 | Replace all reference to GM Service Operations with GM Service and Parts Operations |
| Cline/Setterlun | 8/12/2008 | Add option to FD Op-Code to clear all counters per GM AVG. Update documentation to include default operation of counters. |
| Cline/Setterlun | 12/15/2008 | Add “Programming Storage Buffers – Recommended Usage” section (beginning of this document) and appropriate ‘Note’ updates to several OpCodes that use storage buffers: Class2: $B2 - Kwp2000: $23, $71, $78 – GMLAN: $1A, $22, $A2, $AA Common: $50, $51, $53, $54. |
| Cline | 12/15/2008 | Add Note concerning Type “C” ECUs and OpCode $AA usage in part one of utility file |
| Cline/Wist | 9/01/2009 | Add Comments to Op Code $54 and GMLAN Op Code $3B for assembly plant data ID $46. Update GM Corporation to GM Company. |
| Wist | 03/29/2011 | Plant Manufacturing Specific Updates to: - GMLAN OpCode $3B (AC2, AC3==03 Update) – Special Plant Data IDs. - OpCode $53 – Added Internal Data Identifiers $47-$49 (plant only). - OpCode $54 – AC1, AC2, AC3: Plant data IDs $47-$49 update. |
| Setterlun | 04/01/2011 | Minor Updates: (for April 1, 2011 document) - Copyright update, “SPO” changed to CCA, version/date updates, … - KWP OpCode 84 – P2Max resolution is 25ms not 250ms. - OpCode $54 – “shift” actions are exceptions 04 or 05 (not 03 or 04). |
| Setterlun | 10/01/2011 | Updates from Tim Proctor – Plant Manufacturing Only - OpCode $3B, OpCode $53, OpCode $54 - $4A (Rear Suspension Trim Set) added - Version/date updates |
| Setterlun | 1/24/2012 | Updates from Tim Proctor – Plant Manufacturing Only - OpCode $3B, OpCode $53, OpCode $54: - $4A / $4B references added (Clutch / TCM characterization data) Version/date updates |
| Setterlun | 8/16/2012 | Updates to OpCode $53 for support in Manufacturing the ability to check DLS data for Part Number comparison – (Tim Proctor). Version/date updates. |
| Setterlun | 1.Feb.2013 | Update to Cover Page/Footer (new version/date info). Update to OpCode $53, $54 and GMLAN $3B for Assembly Plant only (Current Date/Time). (Per Tim Proctor) |
| Setterlun | 8.May.2013 22-May-13 final updates | Updated to include some Enhanced Global A security changes. - GMLAN OpCode $3B (add exception 20) - GMLAN OpCode $27 (5 byte seed/key support) - Added GMLAN Req #17 to the ‘Additional Utility File Information - Best Practices and recommendations’ section - Appendix – GMLAN Utility File Example (note) - Appendix F – added “3110 addendum” reference - GMLAN OpCode $B0 – PEC DID reading ‘note’. - Updated “Appendixes” (contents) page |
| Setterlun | Mar.2014 (17th) thru May 20, 2014 | Updates for GM UDS (still considered a draft copy). “Interpreter 4’. Misc updates to other various sections to add UDS support (Utility File Header, Common OpCodes, …). More changes will be coming. |
| Setterlun | May 16,2014 | Broke out the Special Plant Manufacturing elements for Writing Data By Identifier (GMLAN/UDS) – common single implementation. Created the Appendix J. And modified the GMLAN OpCode $3B and Common OpCodes $53, $54 (where appropriate). |
| Setterlun | May 20, 2014 | Added the “Common” E0 OpCode which is purely an indicator to state that the “Erase” (erase routine control) is completed. This request is from Plant Manufacturing to accomidate sequential programming and optimization of the programming event time in the plant. |
| Setterlun | May 20, 2014 | Update to OpCode $F5 – required a variation to the implementation based on UDS. |
| Setterlun (Interpreters_June_10_2014) | June10, 2014 | Very minor cosmetic changes (no realy functional changes). - Limit/Remove use of term “Global B” - Minor/Misc updates to few UDS OpCodes – pseudo code. - UDS $2E Exception 03 (not x3). |

## Appendix G - Related Documents

CD-1039

CD-1036

UART Programmers Quick Reference Guide – XDE 5024

Keyword Protocol 2000 Part 1: Physical Layer

Keyword Protocol 2000 Part 2: Communication, and Error Handling

Keyword Protocol 2000 Part 3: Implementation

GME12000 Diagnostic Protocol KWP2000 Functional Description

XDE-1039 Generic UART Flash Programming Communications Protocol and Software Requirements

XDE-1045 Generic Class 2 Flash Programming Communications Protocol and Software Requirements

XDE-3001B General Motors Class 2 Message Strategy and Application

SAE J-2190 Enhanced E/E Diagnostic Test Modes

Corporate ALDL Data Communications Specification

GMW 3110 GMLAN Enhanced Diagnostic Test Mode Specificationn

GMW3110 (GMLAN) v1p6 Addendum Specification

ISO_FDIS_14229-1_(E).pdf

GB6000_Unified_Diagnostic_Services_Specification.pdf

## Appendix H – $54 Op-Code Additional Information

Helpful hints on using Op-Code $54:

(The best way to explain it is by example – see below.)

Op-Code $54: Description / Action Fields / Pseudo Code

See Op-Code $54 information above (“Communications Independent Programming Op-Codes”).

General Information (typical scenario):

- The application has 20 “internal” storage buffers to temporarily hold data (mostly vehicle data).  These storage buffers can be thought of as storage buffer #1 through #20; however, most Op-Codes (and internal code) refer to these buffers as #0x00 through #0x13.

In addition, the 256 bytes of data within a single storage buffer is referenced by starting at position #00 and going up to position #255 ($FF hexadecimal).  This referencing is done by “most” Op-Codes (including this Op-Code) and also within internal code.

Also, bit positions are referenced starting at position 0 through position 7 (and NOT 1-8).

- Data can be placed into these storage buffers via a “Read Data” type of OpCode ($1A for GMLAN).

- OpCode $54 can then be used to change the data within one of these “internal” storage buffers.  More than one OpCode $54 line may need to be in the utility file if multiple data bytes/bits need to change.

- And finally, this newly “manipulated” data can be written back into the controller, typically with a “write data” type of OpCode ($3B for GMLAN).

GMLAN Example:

Step	Op		AC					Goto

02		1A		03  09  00  00  		5A 03     FD 12     FF 12     00 00     00 00

- Read DID $03 and place in storage buffer #09 – upon a positive response, goto step 03

03		54		09  01  00  AA		00 04      00  00     00  00     00 00     00 00

- Set the second byte (01) of buffer #9 EQUAL (00) to a value of AA – go to step #4

04		54		09  00  01  F0 		00 05      00  00     00  00     00 00     00 00

- Set bits 0-3 of the first byte (00) of buffer #9 equal to zero (AND (01) with 0xF0) – go to step #5

05		54		09  03  02  E0 		00 06      00  00     00  00     00 00     00 00

- Set bits 5-7 of the fourth byte (03) of buffer #9 equal to one (OR (02) with 0xE0) – go to step #6

06		3B		03  09  05  30  		7B 07     FD 12     FF 12     00 00     00 00

- Write the first 05 bytes of data from storage location 09 to DID #03 – upon a positive response, goto

step 07

…

Byte / Bit Manipulation:

| Value Before | Operand | “Mask” Value | Value After | Comments |
| --- | --- | --- | --- | --- |
| 0x1A | EQUAL (00) | 0xD1 | 0xD1 | Full byte replacement. |
| 0x3F | AND (01) | 0xE7 | 0x27 | Set bits 3 and 4 to zero: 0 0 1 1 1 1 1 1 (0x3F) 1 1 1 0 0 1 1 1 (0xE7) ------------------- - Like multiplication. 0 0 1 0 0 1 1 1 (0x27) |
| 0x85 | OR (02) | 0xF0 | 0xF5 | Set bits 4, 5, 6, 7 to a one: 1 0 0 0 0 1 0 1 (0x85) 1 1 1 1 0 0 0 0 (0xF0) ------------------- - As long as one of the bits is a 1. 1 1 1 1 0 1 0 1 (0xF5) |
| 0xD3 | XOR (03) (exclusive OR) | 0xFF | 0x2C | Reverse the bits (compliment) of the byte: 1 1 0 1 0 0 1 1 (0xD3) 1 1 1 1 1 1 1 1 (0xFF) ------------------- - Set to 1 if NOT identical bit values. 0 0 1 0 1 1 0 0 (0x2C) |
| See Comments | SHL (04) (shift left – fill w/ 0x00) | N/A | See Comments | Byte Shift Left Shift the bytes in the buffer LEFT (3 2, 2 1, …) by the number of positions defined in AC1 (fill vacated positions with 0x00). |
| See Comments | SHR (05) (shift right – fill w/ 0x00) | N/A | See Comments | Byte Shift Right Shift the bytes in the buffer RIGHT (1 2, 2 3, …)by the number of positions defined in AC1 (fill vacated positions with 0x00). |

## Appendix I – $55 Op-Code Additional Information - GATS Option String Processing

The General Assembly Test System (GATS) options processing evaluates an ASCII string (mask) consisting of RPO data, operands (e.g. logical and, or, not, wildcard characters), and option specifiers.  Option specifiers consist of the vehicle VIN, and/or model designator information.   Any set of characters in the string that is not white space or an option specifier, and is not preceded by a specified letter and colon is considered an RPO.  If the RPO matches one of the RPOs in the build data, that RPO is considered a match.  The entire string must match for the evaluation to result as true.

Option specifiers are identified by a single letter followed by a colon followed by the data to match against.  There cannot be a space on either side of the colon.  The following

are the valid options identifiers that can be used when building a GATS options string:

M:	Represents the Model Designator.  The string following the colon is compared to the Model Designator in the build data.  Wildcard characters may be used as part of this specifier to match on multiple designators.  See examples for further clarification

V:	Represents the VIN.  The string following the colon is compared to the VIN extracted from the build data.  Wildcard characters may also be used as part of this option specifier so that individual characters within the VIN can be masked on (e.g. restraint code or vehicle model year).  See examples for further clarification.

The option string can contain several special characters to interpret portions of the build string.  These characters allow greater flexibility when setting up the masks. Following is a description of how these wild card characters work.

?	A question mark is used to indicate that any one character can occupy that position in the string.  Matches occur for any single character in the same location of the string.

*	An asterisk is used to indicate that any character can occupy that position and all the trailing positions in the string.  Any number of characters are valid. All characters that follow an asterisk are ignored.

^N	These characters (where N represents any character) are used to indicate any single character in that position in the string that is not N.  Matches occur for any single character in the same location of the string that is NOT that value.

[M-N]   These characters (where M and N represent any characters) are used to indicate a range of M to N inclusive.  Matches occur for any character in the same location of the string that is in the range of M to N.

^[M-N]  These characters (where N and M represent any characters) are used to indicate a "NOT" range of M to N inclusive.  Matches occur for any character in the same location of the string that is NOT in the range of M to N.

[M,N,O,...]   These characters (where N, M and O represent any characters) are used to indicate a list of valid characters.  Matches occur for any character in the same location of the string that matches M, N, O.  More than three characters may be specified.

^[M,N,O,...]   These characters (where N, M and O represent any characters) are used to indicate a list of invalid characters.  Matches occur for any character in the same location of the string that is NOT M, N, O.  More than 3 numbers may be specified.

The option configuration string uses wild card and Boolean characters to interpret each part of the build data.  These characters allow greater flexibility when setting up the masks.  The following is a description of how these Boolean characters work.  The previous section describes the wild card characters.  This section describes the Boolean characters.

&	This character represents the Boolean "AND" operation.  When used, if both operands are true, then the entire statement is true.

|	This character represents the Boolean "OR" operation.  When used, if either operand is true, then the entire statement is true.

!	This character represents the Boolean "NOT" operation.  When used, if the operand is true, then the entire statement is false.  If the operand is false, the entire statement is true.

(  )	These characters represent that Boolean association rules apply.  They are used to specify a grouping of conditions.

| Option String Examples |  |
| --- | --- |
| Option String | Comments |
| L26 & OP1 | True if the RPOs L26 and OP1 exist in the build data. |
| L35 ¦ OP4 | True if the RPOs L35 or OP4 exist in the build data. |
| !LY7 | True if the RPO LY7 does not exist in the build data. |
| JM4 & (OP4 ¦ OP5) | True if RPOs JM4 and either OP4 or OP5 exist in the build data. Notice the parentheses change the logic of this statement. If OP4 and OP5 RPOs exist in the build data and JM4 does not, then this statement evaluates to false. |
| JM4 & OP4 ¦ OP5 | True if RPOs JM4 and OP4 are part of the build data or if OP5 is part of the build data. |
| V:1G2W51567N* | True if the vehicle VIN exactly matches the first 10 characters. Characters 11 through 17 don’t matter. |
| V:1G2W^51567N* | True if the first 4 character of the vehicle VIN are 1G2W and the 5th character is not 5, and digits 6 through 10 are 1567N. Digits 11 through 17 don’t matter. |
| V:1G2W[4-6]* | True if the first 4 character of the vehicle VIN are 1G2W and the 5th character is either a 4, 5, or a 6. The value of the characters after the 5th are ignored. |
| V:1G2W^[4-6]* | True if the first 4 character of the vehicle VIN are 1G2W and the 5th character is not 4, 5, or 6. The value of the characters after the 5th are ignored. |
| V:1G2[A,C,W]5* | True if the first 4 3 characters of the vehicle VIN are 1G2, the 4th character is either A, C, or W, and the 5th character is 5. Characters 6 through 17 are ignored. |
| V:1G2W[1-12]567N* | Not a valid string. The number of characters inside the brackets must be the same for all possible values within the range. A valid string would be V:1G2W[01-12]567N* |
| V:[1,3]G2W^[23-44,55]?6* | True if the first character of the vehicle VIN is a 1 or a 3, characters two through four are G2W, characters five and six are not 23 through 44 and also not 55, character 7 can be anything, character eight is a 6, and the rest of the characters are ignored. |
| M:1W[B,C,D]19 | True if the vehicle model designator is 1WB19, 1WC19, or 1WD19 |
| M:1WD19 ¦ AU0 ¦ AP3 | True if the vehicle model designator is 1WB19 or if the RPO AU0 is part of the build data, or if the RPO AP3 is part of the build data. |
| V:?????????6* | True if the 10th digit of the VIN is a 6. The leading ? are single character wildcards and the * indicates that all characters after the 6 in the 10 digit are don’t cares. |

## Appendix J – Plant Manufacturing VIT Data

This appendix defines the Plant Manufacturing VIT Data elements that may be referenced during a manufacturing programming event:

  - Internal Data Specifier:		 			(See Data Specifier table below)

  - Data Specifier Indicator Value:  		(See Indicaor Value table below)

Supported for the following OpCodes:

GMLAN/CAN:				Internal Data Specifier				= AC1

OpCode $3B				Data Specifier Indicator Value		= AC2

Exception (0x03)					= AC3

UDS:						Internal Data Specifier				= AC2

OpCode $2E				Data Specifier Indicator Value		= AC3 (High Nibble)

Exception (0x?3)					= AC3 (Low Nibble  = ‘3’)

Common:					Internal Data Specifier				= AC1

OpCode $54				Operation (0x07)					= AC2

Data Specifier Indicator Value	 	= AC3

Internal Data Specifier  		Description

| 0x44 | Tire Type (e.g. P275/55R20) |
| --- | --- |
| 0x45 | TPM tire placard data |
| 0x46 | Diesel Injector Mass Adjustment (IMA) or Injector Quantity Adjustment (IQA) data. |
| 0x47 | Transfer Case Control Module “B-Cal” (spring rate) data. |
| 0x48 | Key Fob data. |
| 0x49 | Engine Serial Number. |
| 0x4A | Rear suspension Trim set. |
| 0x4B | ELSD Clutch Specification. |
| 0x4C | TCM Characterization. |
| 0x4D | Current Date/Time |

Data Specifier 		Used for:

Indicator Value	s	Internal Data Specifier				   Description

| 0 | 0x46 (IMA) | Data for All Injectors |
| --- | --- | --- |
| 1..F | 0x46 | Data for Injector # 1…15 (decimal) |
| 0 | 0x48 (Key Fob Data) | Data for All Key Fobs |
| 1..F | 0x48 | Data for Key Fob # 1…15 (decimal) |
| 0 | 0x4C (TCM Characterization) | Characterization Data for All solenoids |
| 1..F | 0x4C | Data for solenoid # 1…15 (decimal) |
| 0 | 0x4D Current Date/Time | Current Date/Time: ‘YYMMDDhhmmss’ |
| 1 | 0x4D | Data: YY (00 is 2000 & FF is 2255) |
| 2 | 0x4D | Data: MM |
| 3 | 0x4D | Data: DD |
| 4 | 0x4D | Data: hh |
| 5 | 0x4D | Data: mm |
| 6 | 0x4D | Data: ss |