| GM Global Powertrain Engineering Controls Sector Software and Calibration Parts Application Programming Interface Release Date:; February 28, 2014. Filename:; Software_and_Calibration_Parts_API.docx. Formerly Known As:; TL.07.0054.Rxx GM World Wide Software and Calibration Parts Application Program Interface. Note: If you have a printed copy of this document, you must be sure that it is the latest version before reading it. Refer to the GMPT Controls Engineering web site to verify the latest version. |
| --- |
| Contributors: - Gregory Ambrose - Yiing Maa - Monica Mitchell - Patricia Wombwell |  |  |  |  |
| Document Owner: Yiing Maa Phone: 248-930-7331 E-Mail: Yiing.Maa@gm.com |  |  | GM Powertrain General Motors Corporation Mail Code : 483-331-201 3300 GM Road Milford, Michigan 48380 |  |

Table Of Contents

1	Introduction	3

1.1	Applicability	3

1.2	Granting Exceptions To the Standard	3

1.3	Scope	3

1.4	Rationale for a Common API	4

1.5	Using the Common API	5

1.6	End Model (Controller Assembly) Concept	5

1.7	Communications Security of the Common API	5

2	Application Programming Interface (API) Definition	5

2.1	API - Full GM/InfraNet Access	6

2.1.1	System requirements	6

2.2	Standard Data Formats	16

2.2.1	Data File Formats	16

2.2.2	Drawing File Formats	22

2.2.3	End Model Bill of Materials (EBOM) Format	32

3	CRC Codes Defined	34

Appendix A - Revision History	37

Appendix B – Glossary	38

Appendix C - Related Documents	40

Appendix D - GM Concepts of API Functionality	40

Appendix E - End Model (Controller Assembly) Configurations	41

Appendix F - GM File Naming Conventions	43

Appendix G - GM Scenarios for Inter-Organization Communication	43

Appendix H - Layout of API Binary Files examples	45

Appendix I - Example of API Binary Files and Drawing Files	52

Appendix J - Example of GDFRS upload function	54

Table of Tables

No table of figures entries found.

Table of Figures

No table of figures entries found.

# Introduction

This document defines a common Application Programming Interface that shall include GM and non-GM providers of controller data files.  These controller data files will be used at GM assembly plants, service departments, and non-GM assembly plants such as controller manufacturing plants.  Also defined here are recommendations and requirements for the TCP/IP computer-to-computer communication protocols.

## Applicability

The audience for this document includes all personnel who create, release, or use data files for GM controllers.

## Granting Exceptions To the Standard

Under some circumstances, it may be necessary to deviate from the standard.  Legitimate requests for deviation may be granted, provided that the situation has been properly recorded and approved.

## Scope

Once communication software is programmed into an electronic control module, software parts may be programmed into the module during the vehicle assembly process.  This programming scheme allows a generic controller to be configured for a specific vehicle application in order to reduce both the number and variety of physical parts stocked at the GM assembly plant.  This same reasoning applies to the service environment.  Programming electronic controllers in this manner also increases flexibility in managing changes to the software and calibration parts.  In order to meet the needs of the assembly plants and the service operations in a timely manner, a common electronic communication arena must be defined that comprehends both software and hardware suppliers and users.  The arena chosen is an Application Programming Interface (API).

This document defines the requirements for the Application Programming Interface (API) between computer systems of controller data file suppliers and the computer systems of controller data file consumers.  All electronic communication between computer systems of controller data file suppliers and the computer systems of controller data file consumers shall conform to the requirements of this document.

It is the responsibility of each organization to build and validate their own computer systems interface to this Common API.

## Rationale for a Common API

Normally, embedded microcomputer code for controllers is written, compiled or assembled, linked to libraries, located into a known set of absolute address locations, and converted into a microcomputer supplier standard HEX-ASCII format. For reprogrammable controllers, this HEX-ASCII format gets converted to binary data bytes and transmitted (downloaded) into the embedded microcomputer’s EEPROM, FLASH, or other reprogrammable memory storage devices. Application software code and calibration data are given part numbers and released for use.  The released form of the code is stored in, a standard binary format defined in this document.  Meta-data for “drawings” are provided in separate ASCII-based files.  Specifics of the ASCII-based drawing file format are provided later in this document.

An Application Programming Interface consists of the following parts:

- a computer-to-computer communication link and protocols,

- message and file format definitions,

- message, file, and protocol “handlers” for sending and receiving information.

## Using the Common API

Each organization is responsible for managing their communications such that they are in compliance with the API standard. This may involve joint activities between organizations in order to test and verify the communications between systems. Examples of some possible methods are included in the Appendix Section.  The examples are not part of the requirements and, in fact, detail some activities that are outside the API, but are necessary for the communication of API functionality.

## End Model (Controller Assembly) Concept

An End Model (controller assembly) is the item sold by a controller supplier to a vehicle assembly plant or to a service organization.  The end model is an assembly consisting of hardware and software parts with each having its own part number. In addition to transmitting software part (binary) data files and their companion drawing (ASCII) data files, the API shall be the mechanism for the transmission of End Model information via the END MODEL BILL OF MATERIALS (EBOM) drawing data file.

## Communications Security of the Common API

GM Position Statement:

Whereas the contents of the binary data is easily obtainable from GM Service Departments, and

Whereas the part numbers, descriptions, and end model bill of materials drawing (ASCII) data files contain information are published in GM Service Manuals, and

Whereas the computer system name, the username, and the user password are an acceptable level of security access on the GM/InfraNet communication networks,

all binary and ASCII data transmitted via this Common API shall be GM Non-Classified Data!

If in the future, should GM wish to classify data transferred via this Common API, this API shall support the transmittal of encrypted data. (NOTE:  This does NOT imply that the API shall perform the data encryption or encryption key management!)

# Application Programming Interface (API) Definition

The following functionality shall be provided via the API:

- Manual and automated computer-to-computer connections

- Computer-to-computer connections security

- Ability to “upload” an electronic end model bill of materials to another computer system

- Ability to “upload” single ‘soft’ part data file or a companion electronic drawing to another computer system

- Ability to “inquire” about parts  by individual part number

- Ability to “retrieve” an end model bill of materials from another computer system

- Ability to “retrieve” single ‘soft’ part data file or a companion electronic drawing from another computer system

- Ability to “force” a part retrieval (retforce) even if that part status indicates corrupt

## API - Full GM/InfraNet Access

The provider and the receiver of the controller software and calibration data shall establish a manually initiated and/or automated set of procedures for uploading or retrieving the drawings and the binary files to the controller supplier (or GM Assembly Plants, or GM/SO).  In addition the API shall:

- Comply with the file format requirements defined in this document.

- Comply with the status message structure defined in this document.

The World Wide Web (http) communication protocol is the standard protocol for the API. Each controller or software supplier must provide a World Wide Web (WWW) server, which supports the Hypertext Transport Protocol (http), the http protocol supported includes:

- http 1.0

- http 1.1

- https 1.0

- https 1.1

### System requirements

#### Establishing a connection

The connection between the non-GM controller data file suppliers/consumers and the GM Global data file repository shall be done via a secure network connection. This connection method shall be a GM approved solution, i.e. GMSE, SSLVPN, etc.

##### Communication protocol

Data transfers between the provider and receiver computer systems shall be accomplished as follows:

- Use the URL to specify the server according to the standard WWW protocol.

- Use the WWW basic access, which requires an encoded username and password to establish the connection.

- Use the WWW (http/https) protocol, and CGI (Common Gateway Interface) scripts (recommended but not required), to do all file(s) transfers using the API functions defined in this document.

##### Network security

The WWW server software at each server site can provide network security at various levels.

#### API Functions

Each data supplier will have a WWW server that has access to the parts data.  This access is most likely by means of a CGI script. The CGI script can be written in any language and can accept request parameters in the form of a string.

Each client must be capable of encoding the request parameters and basic authentication, and transferring that information to the WWW server according to one of the http/https supported formats.

Each API request shall be sent by the client to GM global data file repository (i.e. CPFA or GDFRS) as a string with the following parameters:

FUNC=function&PRTN=partNumber&TYPE=type&FRMT=format

For the GDFRS function upload only, there are extra parameters based on the EVO Readiness of the part number and the file type being uploaded or part status being updated:

For non-EVO Ready (Z-Iteration) uploaded PTI data files, the parameters are:

FUNC=upload&PRTN=partNumber&DLS=dlsPls&TYPE=data&FRMT=binary&STAT=status&CPFA=repository

For non-EVO Ready (Z-Iteration) uploaded DBI data files, the parameters are:

FUNC=upload&PRTN=partNumber&DLS=dlsPls&TYPE=drawing&FRMT=ascii&STAT=status&CPFA=repository &PLTUSE=plantUse

For EVO Ready (DLS-PLS release) uploaded PTI data files, the parameters are:

FUNC=upload&PRTN=partNumber&DLS=dlsPls&TYPE=data&FRMT=binary&STAT=status&CPFA=repository

For EVO Ready (DLS-PLS release) uploaded DBI data files, the parameters are:

FUNC=upload&PRTN=partNumber&DLS=dlsPls&TYPE=drawing&FRMT=ascii&STAT=status&PLTUSE=plantUse&CPFA=repository&USE= MY1.PC1. eventRPO1a.bucketRPO1a.plantRPO1a,eventRPO1b.bucketRPO1b.plantRPO1b; MY2.PC2.eventRPO2a.bucketRPO2a.plantRPO2a; MYn.PCn. eventRPOna.bucketRPOna.plantRPOna, …, eventRPOnx.bucketRPOnx.plantRPOnx

For EVO Ready (DLS-PLS release) status update without uploading data files, the parameters are:

FUNC=update&PRTN=partNumber&DLS=dlsPls&STAT=status&PLTUSE=plantUse&CPFA=repository&USE= MY1.PC1. eventRPO1a.bucketRPO1a.plantRPO1a,eventRPO1b.bucketRPO1b.plantRPO1b; MY2.PC2.eventRPO2a.bucketRPO2a.plantRPO2a; MYn.PCn. eventRPOna.bucketRPOna.plantRPOna, …, eventRPOnx.bucketRPOnx.plantRPOnx

Where:

- Suffix “1” through “n” represents the required one or more parameters for EVO Ready releases to GDFRS headed by a MY and PC.  Each MY and PC phrase is ended with a semi-colon.

- Suffix “a” through “x” represents the required one or more parameters for EVO Ready releases to GDFRS for each MY and PC phrase.  Each event, bucket, plant phrase is ended with a comma if more than one phrase exists.

Note#1: The string cannot contain any spaces or newlines and is case insensitive (i.e. upper case characters and lower case characters must be interpreted as the same characters – ‘A’ is identical to ‘a’).  The GDFRS string shown above has newlines included for readability which are NOT to be included in the communication to GDFRS

Note#2: For any part number, part data PTI files and to be sent prior to part drawing DBI files

Note #3: Keyword USE and its arguments are mandatory for EVO Ready DBI data file transactions for status PPV ONLY

The different values for the parameters are:

function

inquire		Inquire about a part status.

retrieve		Retrieve the part data or drawing.

retforce		Retrieve the corrupted part data or drawing.

upload 		Upload part data or drawing to the server.

remove		Remove part data or drawing from the archive.

update			Part data and drawing files are not uploaded.  Update status and

processing only. Parameters “type” and “format” are ignored

partNumber

The part number is the number assigned to the data for identification within a product description system (e.g. GPDS).  The part number can be an 8-digit part number (production release) or an 8 digit part number plus z#-suffix (where # is 0-9) or plus 2-alpha (DLS/PLS).

dlsPls

For EVO Ready (DLS-PLS) parts, the dlsPls should be a two character alpha value (DLS/PLS).

For non-EVO Ready (Z-Iteration) parts, the dlsPls should be the DLS associated to the Z iteration (actual DLS of the Non-EVO part).

type

data		Part data file

drawing	Part drawing file

format

binary		Part data in the API standard binary data format.

ascii		Part drawing information in the API standard drawing format.

status 	(required for GDFRS upload function only – mandatory argument – EVO and non-EVO Ready)

dev		Part release status is development

preprod	Part release status is pre-production

ppv		Part release status is PPV

prod		Part release status is production

plantUse 	(required for GDFRS upload function only – mandatory argument when status equals ppv or prod only – EVO and non-EVO Ready)

A single character where Y indicates that the part will be used in assembly plants and N indicates that the part will not be used in assembly plants.

repository	(required for GDFRS EVO Ready upload and update function only)

Values are Platform or Powertrain

MYn  		(required for GDFRS upload function only – one MY argument minimum required for EVO Ready where status equals ppv)

Model Year of the part being released which is a 4 digit model year.

PCn 		(required for GDFRS upload function only – one PC argument minimum required for EVO Ready where status equals ppv)

Product Code of the part being released which is a one or two alphanumeric product code defined in GPDS

eventRPOnm (required for GDFRS upload function only – one eventRPO argument minimum required for EVO Ready where status equals ppv)

Valid Plant Build Event RPO as defined by the EVO Program

bucketRPOnm (required for GDFRS upload function only – one bucketRPO argument minimum required for EVO Ready where status equals ppv)

Valid Plant Build Bucket RPO as defined by the EVO Program

plantRPOnm (required for GDFRS upload function only – one plantRPO argument minimum required for EVO Ready where status equals ppv)

Valid Plant RPO as defined by the VDS

Note 1:  If during an upload, a part file or drawing file by the same name already exists, then an error code shall be returned. The remove function must be used prior to replacing a part file or drawing file of the same name.

Note 2:  To upload API binary files, two methods shall be supported:

- The “Content-Encoding” header of the POST can be set to “application/octet-stream” and the API binary file must immediately follow the HTTP headers.

- The “Content-Encoding” header of the POST can be “multi-part/form-data” and one or more binary files can be sent in the MIME encoded portion of the POST following the HTTP headers. (The MIME encoding type must be set so that API binary files is sent unchanged in the POST between the MIME boundaries.)

The GM global data file repository systems implementation of the API specification shall allow a MIME message with any number of files to be sent to the web server using a web browser at the same time. The web server can be set to decode this MIME encoded message and extract the multiple files.

Note 3: Each supplier may choose to not support all of the above functions or formats, however, the "inquire" and "retrieve" functions and the "binary" and "ascii" formats must be supported.

The server can accept parameters by several methods.  The most common methods are GET and POST.  The server should be able to accept either of these methods.  The POST method will be required for the data upload function.

All data returned (ASCII) shall have MIME message format “text/plain.”  The requested data will be returned as a web page containing the data.  The servers will either return the requested information or a one-line string.  In the case of a part inquiry, only a string will be returned.  The strings shall consist of a two-digit number followed by a space and then the name in the table below.  Examples are:

00   Part Available

01   Not Released for Production

52   Invalid Parameters

The format for return strings shall be:

<Return code><space><Name><eol>

Note that the server can also return a MIME format object describing the status code to the http/https request.  This object may be returned instead of expected data when an error occurs.  The object may be in “text/html” or “text/plain” or one of the formats specified as acceptable in the request.  The body section may contain a document describing the error in human readable form.

The client must be able to distinguish what it expects from the types of error conditions above.  For example, if the client requested a drawing for a particular part and put the returned web page into a file, the client must then examine the file to see if the file contains the drawing information expected.  The file may contain a one line string starting with a two digit number, or the file may contain HTML text describing a client or server error instead of the expected drawing information.

In most cases, clients will request data or a drawing for a particular part.  The returned web page may be put into a file and the first six bytes examined.

- In the case of a drawing, the first two bytes will be upper case ASCII alphabetic, the next two will be upper case alphanumeric, and the next two will be an equal sign and a single quote.

- In the case of data, the first four bytes would most likely not be normal ASCII characters.  If a one-line string were returned, then the first two bytes would be ASCII numeric.  Any other combination can be construed as a client or server error.

Clarification:

To be compliant with HTTP 1.0, the error strings must be returned in the HTTP object not in the HTTP header. For example a response containing part data might look like:

HTTP/1.0 200 OK<CRLF>

Content-Encoding: application/octet-stream<CRLF>

Content-Length: 2123<CRLF>

<CRLF>

<2123 bytes of data in API binary format>

A response containing an error might look like:

HTTP/1.0 200 OK<CRLF>

Content-Encoding: text/plain<CRLF>

<CRLF>

01 Not Released for Production<CRLF>

The different strings are:

a.	Normal Return Strings - The procedure shall return one of the following strings if no error occurred in processing the user's request:

| Return strings |  |  |
| --- | --- | --- |
| Return Code | Name | Description |
| 00 | Operation Successful | Indicates that the operation requested has completed successfully. If the request was a part inquiry, it indicates that the part's data is available and released for production. |
| 01 | Not Released for Production | Indicates the part number is valid for this host and that the part's data is available. If the request was a part inquiry, it indicates that the part's data is available but not released for production. |
| 02 | Not Available | Indicates the part number is valid for this host but the part's data is not available ( i.e. part has been removed). |
| 03 | Part Data Corrupt | Indicates the part number is valid for this host but the part's data is corrupted. |
| 62 | Warning:<reason> | GDFRS upload function only. Command completed successfully and part data files were released but part specific Build Event Designator information was overwritten with more general information |

b.	Error Return Strings - The procedure shall return one of the following strings if an error occurred in processing the user's request:

| Return strings |  |  |
| --- | --- | --- |
| Return Code | Name | Description |
| 51 | Part Not Found | Indicates the host system does not have the part number. |
| 52 | Invalid Parameters | Indicates that parameters for the request are invalid or missing. The name of the return string can also be “Type/Format Mismatch” when the type and format of the file uploaded do not match the specifications. |
| 53 | Not Authorized | Indicates the user is not authorized to perform this function. |
| 54 | Host System Down | Indicates the database host system is unable to process the user request. |
| 55 | Unexpected Error | An unexpected error occurred and the host is unable to process the request. |
| 56 | Not Supported | Indicates the requested function is not supported. |
| 57 | Already Exists | Indicates the data for the part in the supplied type/format already exists in the host database. |
| 58 | CRC Error | Indicates that the CRC check failed for the uploaded file. The CRC is a mandatory item in the API binary file. However, it’s an optional attribute (ZCRC) in the API drawing file and will be checked when present. |
| 61 | Invalid data file | GDFRS upload function only. Multiple cases where DLS-PLS is submitted and all Product Code are not EVO-Ready, or Z-iteration is submitted and all Product Code are EVO-Ready |
| 65 | Part has not been released to GPDS for the <a model year> and <a product code> | GDFRS upload function only. Indicates the part is not released to GPDS for the selected model year and selected product code. |
| 66 | Items released earlier using FTP2GDFRS cannot be released using TL.07 | GDFRS upload function only. Indicates items already released using GDFRS via FTP2GDFRS cannot be released using TL.07. |

##### Part inquire request

The part inquiry request can return one of the above strings.

##### Part retrieve request

The API shall NOT provide a procedure to retrieve multiple parts from a part-list by type and format. This means that the ‘calling’ interface shall set up its own parts lists and write their own looping mechanism to ‘call’ the common API’s Part Retrieval function.

The part retrieval request can return

- the requested data

- Any other string in place of the data except “00 Operation Successful”

##### Part retforce request

The API shall provide a function for forcing the retrieval of a part.  This function (retforce) may be used to override return codes 01 and 03 of the retrieve function above.

The part retrieval forced (retforce) request can return

- the requested data

- Any other string in place of the data except “00 Operation Successful”

While this capability or function is required, it is NOT a requirement that an API data supplier provide proprietary or non-production type data. If data is not supplied, then an appropriate return string shall be the response.

##### Part upload request

The upload requests can return

- “00 Operation Successful”

- Any strings “52” through “58”, “61”, “62”, “65”, or “66”

##### Part remove request

The API shall provide a function for removing the part data.   If error code 57 for the Upload function is encountered, the Remove function can be used to remove the part data, then the user can attempt to use the Upload function again

The remove requests can return

- “00 Operation Successful”

- Any strings “51” through “56”

While this capability or function is required, it is NOT a requirement that an API data supplier provide a capability for other organizations to delete data from their database.  If parts data removal is not permitted, then an appropriate error return string shall be the response.

##### Part update request

The API shall provide a function for updating the part status.

The update requests can return

- “00 Operation Successful”

- Any strings “51” through “56”

#### Part Types

Each part number may be associated with multiple types.  Each type is a different representation of the part in the host database.  For example, a part number that represents a programmable calibration part may be associated with a data file type and a drawing type.  The host computer system portion of the API shall verify that any requested action contains compatible types and formats for the part number supplied.  The Appendix Section contains paragraphs identifying some possible file extensions and their usage.

a.	Data File Type - Data File types shall contain the memory image to be programmed into the electronic controller.  The format of this type is described in section 2.2 Standard Data Formats.

- Drawing File Type  - Drawing types shall contain the electronic representation of the part drawing.  The format of this type is described in section 2.2 Standard Data Formats.

## Standard Data Formats

Each part number may be associated with multiple file types. For example, a Data File type which is supplied as an API binary file and associated drawing file type is supplied as ASCII.  The sections below define the requirements for the binary data and ASCII drawing formats.

### Data File Formats

The following section defines the requirements for the binary data format.  The GM host system shall support all the Binary file formats.

NOTE1:  The header and address sections must be transmitted as a string of bytes in file offset order.  Therefore, a common representation of multi-byte numeric quantities is needed.  Four byte numbers in the above format are represented as unsigned integers starting with the most significant byte, followed by less significant bytes, and ending with the least significant byte.  For example, the most significant byte of the four-byte format type will be the first byte transmitted, and therefore the first byte in the file.  Similarly, two byte numbers are represented as unsigned integers starting with the most significant byte first followed by the least significant byte and addresses are represented with the most significant byte first and ending with the least significant byte.

#### Data File Binary Format

The Binary file shall be constructed of one or multiple information blocks.  The binary file format type defines the parameters available for that format type.

The following sections describe the parameters defined for Binary File Format Type 2 and 3. If the Binary file contains multiple information blocks, the format type shall be the same in each information block.

##### Data File Format Type 2

##### For Format Type 2, each information block shall contain the following four sections:

a.	The Header Section - The Header Section shall conform to the following table:

| Binary format - Header Section |  |  |
| --- | --- | --- |
| Offset into File / Size of Field (Bytes) | Description | Comments |
| 0 / 4 | Format Type | Binary File Format version. The format shall be unsigned integer (binary integer) - 0X2 Format Type contains the parameters defined in this section. The format type is used by all API tools to provide upward compatibility and to ensure that the API can detect which formats are supported by the API tool revision level. |
| 4 / 36 | Part Number | Represented as an ASCII string of characters. Although 36 positions are allocated, if the part number is less than 36 characters, then the extra fields shall be null filled (0x00). (If a part number is considered to be left justified in this field, and read from left to right, the 1st character of the part number begins at byte offset 4.) |
| 40 / 4 | Block Number | The file may contain multiple information blocks. The number in this block may not be greater than the Number of Blocks parameter at offset 44. Format shall be a binary unsigned integer. |
| 44 / 4 | Number of Blocks | The total number of information blocks in this file. Must be at least 1. Format shall be unsigned binary integer. |
| 48 / 14 | Data Creation Date | The format shall be ASCII characters - YYYYMMDDHHMMSS |
| 62 / 1 | Data Type | Identifies the data section type. The format shall be unsigned binary integer. 0x0 = Normal Data Section, to be programmed into product memory 0x1 = GMLAN Header Section, not to be programmed into product memory NOTE1: Tools which see a Data Type greater than 0x1 should exit with an error if Format Type is 2 NOTE2: Only one GMLAN Header Section per data part is supported. |
| 63 / 1 | Data Section Compression algorithm type | Identifies the compression algorithm type used to compress the data section. The format shall be unsigned binary integer. May be zero, in which case data is not compressed. 0x0 = no data compression, 0x1 = compression algorithm type 1, 0x2 = compression algorithm type 2, etc. NOTE1: The compression algorithm types are defined by the GM Global Infrastructure software group. NOTE2: The compression/decompression tool “giz” developed by the GM Global Infrastructure software group is available to the suppliers. |
| 64 / 20 | Spare | Until spares defined, the format shall be unsigned binary integer - null filled (0x00). |
| 84 / 4 | Number of Address Bytes (A) | Contains the number of bytes in the Address Section of this block. May be zero, in which case there is no Address Section. Format shall be an unsigned binary integer. 0X0 = No Address Section 0X2 = 16-bit Address 0X4 = 32-bit Address |
| 88 / 4 | Number of Data Bytes (N) | Contains the number of bytes in the Data Section of this information block. Maximum of 4,294,967,295 bytes. Format shall be a binary unsigned integer. |
| 92 / 4 | CRC Type | The type of CRC algorithm implemented to obtain the data in the CRC Section of this binary file format. Format shall be a binary unsigned integer. Valid types are as follows: 0X0 = No CRC (Number of CRC Bytes must be 0!) 0X1 = CRC32 with Width : 32 Poly : 0x04C11DB7 Init : 0xFFFFFFFF RefIn : True RefOut : True XorOut : 0xFFFFFFFF NOTE1: Production data shall never use type 0X0 |
| 96 / 4 | Number of CRC Bytes (C) | Contains the number of bytes in the CRC Section of this block. For example, a 32-bit CRC would contain 4. Format shall be a binary unsigned integer. |

b.	The Address Section - The Address Section shall immediately follow the Header Section and shall be A bytes in size as defined by Number of Address Bytes parameter in the Header Section.  If in the header section offset 84 / 4 (Number of Address Bytes), zero (0) bytes are defined, then this address section will not exist.

| Binary format - Address Section |  |  |
| --- | --- | --- |
| Offset into File / Size of Field (Bytes) | Description | Comments |
| 100 / A | Address Section | The starting address in product memory where the data in the Data Section should be placed. Format shall be a binary unsigned integer. |

c.	The Data Section - The Data Section shall immediately follow the Address Section if it exists, or the Header Section if it does not, and shall be N bytes in size as defined by the Number of Data Bytes parameter in the Header Section.

| Binary format - Data Section |  |  |
| --- | --- | --- |
| Offset into File / Size of Field (Bytes) | Description | Comments |
| (100 + A) / N | Data Section | The actual data to be programmed into product memory, if the Data Type is a Normal Data Section; otherwise, a GMLAN Header. Data values are binary bytes stored in address ascending order. (Note: There are exceptions to this statement; i.e., the utility file data is not programmed into the controller, it is used with a programming tool to reprogram the device.). |

d.	The CRC Section - The CRC Section shall contain the CRC generated by the algorithm defined in the CRC Type parameter of the Header Section and shall be C bytes in size as defined by the Number of CRC Bytes parameter of the Header Section.

| Binary format - CRC Section |  |  |
| --- | --- | --- |
| Offset into File / Size of Field (Bytes) | Description | Comments |
| (100 + A + N) / C | CRC Section | The CRC generated by the CRC algorithm defined. The CRC is applied beginning with the first byte of the header and ending with the last byte of data. (Please see Appendix - CRC Codes Defined.) |

It is the responsibility of the provider of the controller software and calibration data to convert the HEX-ASCII (currently Intel Hex-ASCII or Motorola S-Record ‘PTP’) into the common binary format detailed in this document. The binary format is the default.

Note: A general purpose tool "bintool" developed by the GM Controls Engineering Tools Group is available to the suppliers ("as is" without any kind of warranty) to convert the PTI format files to Motorola S-Record ‘PTP’.

##### Data File Format Type 3

##### For Format Type 3, each information block shall contain the following three sections (changes made from previous format type are highlighted):

a.	The Header Section - The Header Section shall conform to the following table:

| Binary format - Header Section |  |  |
| --- | --- | --- |
| Offset into File / Size of Field (Bytes) | Description | Comments |
| 0 / 4 | Format Type | Binary File Format version. The format shall be unsigned integer (binary integer) - 0X3 Format Type is the latest format release level. This version shall be incremented by 1 each time a format change is made. The format type is used by all API tools to provide upward compatibility and to ensure that the API can detect which formats are supported by the API tool revision level. |
| 4 / 36 | Part Number | Represented as an ASCII string of characters. Although 36 positions are allocated, if the part number is less than 36 characters, then the extra fields shall be null filled (0x00). (If a part number is considered to be left justified in this field, and read from left to right, the 1st character of the part number begins at byte offset 4.) |
| 40 / 4 | Block Number | The file may contain multiple information blocks. The number in this block may not be greater than the Number of Blocks parameter at offset 44. Format shall be a binary unsigned integer. |
| 44 / 4 | Number of Blocks | The total number of information blocks in this file. Must be at least 1. Format shall be unsigned binary integer. |
| 48 / 14 | Data Creation Date | The format shall be ASCII characters – YYYYMMDDHHMMSS |
| 62 / 1 | Data Type | Identifies the data section type. The format shall be unsigned binary integer. 0x0 = Normal Data Block, to be programmed into product memory (except for the utility file data is not programmed into the controller, it is used with a programming tool to reprogram the device) 0x1 = GMLAN Header Block, not to be programmed into product memory 0x2 = Signed Data Block Signed Header, not to be programmed into product memory GMLAN Header (if there is one), not to be programmed into product memory Data regions, to be programmed into product memory 0x3 = Compressed Data Block GMLAN Header (if there is one), not to be programmed into product memory Data regions, to be programmed into product memory 0x4 = Signed and Compressed Data Block Compression Header, not to be programmed into product memory The following sections are compressed - Signed Header, not to be programmed into product memory GMLAN Header (if there is one), not to be programmed into product memory Data regions, to be programmed into product memory NOTE1: Tools which see a data section Type greater than 0x4 should exit with an error if Format Type is 3 NOTE2: Only one Data Block type (0x0, 0x2, 0x3, or 0x4) per data file is supported. NOTE3: Compression Header only appears in Signed and Compressed Data Block |
| 63 / 1 | Data Section Compression algorithm type | Identifies the compression algorithm type used to compress the 0x3 Compressed Data Block, or 0x4 Signed and Compressed Data Block. The format shall be unsigned binary integer. May be zero, in which case data is not compressed. 0x0 = no data compression, 0x1 = compression algorithm type 1, 0x2 = compression algorithm type 2, etc. NOTE1: The compression algorithm types are defined by the GM Global Infrastructure software group. NOTE2: The compression/decompression tool “giz” developed by the GM Global Infrastructure software group is available to the suppliers. |
| 64/1 | Data Section Compression header parameter | Identifies the compression header parameter used for the specific Compression algorithm type. The format shall be unsigned binary integer. If data is not compressed then this parameter is null filled (0x00). NOTE1: The compressed data may contain compression info not accounted in the Compression Header depending on the Compression header parameter. |
| 65 / 11 | Spare | Until spares defined, the format shall be unsigned binary integer – null filled (0x00). |
| 76 / 4 | Length of Data Section (N) | Contains the number of bytes in the Data Section of this information block. Maximum of 4,294,967,295 bytes. Format shall be a binary unsigned integer. |
| 80 / 2 | Length of Compression Header | Contains the number of bytes that define the compression header, not programmed into product memory. May be zero, in which case there is no compression header. |
| 82 / 2 | Length of Signed Header | Contains the number of bytes that define the signed header, not to be programmed into product memory. May be zero, in which case there is no signed header. |
| 84 / 2 | Length of GMLAN header | Contains the number of bytes that define the GMLAN header, not to be programmed into product memory. May be zero, in which case there is no GMLAN header. |
| 86 / 2 | Number of Data Regions(n) | Identifies the number of data regions the data is partition into, to be programmed into product memory. |
|  |  | The Data Region Info is a structure that contains information about the Data Region location in memory. The length of this field depends on the number of Data regions. Where n indicates the number of distinct Data regions. For example if n=2 then the length of this field shall be 16. Each data region is 8 bytes in length – Start address of the memory region, 4 bytes. Length of the region, 4 bytes. |
| 88 / 8*n | Data Region 1 | Specifies the first data region location in Memory |
|  | Data Region 2 | Specifies the second data region location in Memory |
|  | … | The Data Region Info repeat for the total number of Data regions as indicated by “Number of Data Regions(n)”. |
|  | Data Region n | Specifies the last data region location in Memory |
| (88+8*n) / 4 | CRC Type | The type of CRC algorithm implemented to obtain the data in the CRC Section of this binary file format. Format shall be a binary unsigned integer. Valid types are as follows: 0X0 = No CRC (Number of CRC Bytes must be 0!) 0X1 = CRC32 with Width : 32 Poly : 0x04C11DB7 Init : 0xFFFFFFFF RefIn : True RefOut : True XorOut : 0xFFFFFFFF NOTE1: Production data shall never use type 0X0 |
| (88+(8*n)+4) / 4 | Number of CRC Bytes (C) | Contains the number of bytes in the CRC Section of this block. For example, a 32-bit CRC would contain 4. Format shall be a binary unsigned integer. |

b.	The Data Section - The Data Section shall immediately follow the Header Section, and shall be N bytes in size as defined by the Length of Data Section parameter in the Header Section.

| Binary format - Data Section |  |  |
| --- | --- | --- |
| Offset into File / Size of Field (Bytes) | Description | Comments |
| (88+(8*n)+8) / N | Data Section | The actual data to be programmed into product memory except for the utility file and except for the following headers if contained in the Data Section. - GMLAN Header - Signed Header - Compression Header Data values are binary bytes stored in address ascending order. |

- The CRC Section - The CRC Section shall contain the CRC generated by the algorithm defined in the CRC Type parameter of the Header Section and shall be C bytes in size as defined by the Number of CRC Bytes parameter of the Header Section.

| Binary format - CRC Section |  |  |
| --- | --- | --- |
| Offset into File / Size of Field (Bytes) | Description | Comments |
| (88+8*n+8+N) / C | CRC Section | The CRC generated by the CRC algorithm defined. The CRC is applied beginning with the first byte of the header and ending with the last byte of data. (Please see Appendix - CRC Codes Defined.) |

It is the responsibility of the provider of the controller software and calibration data to convert the HEX-ASCII (currently Intel Hex-ASCII or Motorola S-Record ‘PTP’) into the common binary format detailed in this document. The binary format is the default.

Note: A general purpose tool "bintool" developed by the GM Controls Engineering Tools Group is available to the suppliers ("as is" without any kind of warranty) to convert the PTI format files to Motorola S-Record ‘PTP’.

### Drawing File Formats

These electronic drawing file formats have been designed to store the drawing meta-data information. Each provider of software and calibration data files and drawings shall be responsible for ensuring the correct attribute values are used in their delivered drawing files.

At a minimum, Drawing files shall be created for Data files, End Model Bill of Material files (EBOMs) which may include Manufacturing Programming Instructions.  All drawing files shall contain a KIND field to identify the specific drawing type.

A minimum required set of contents for Data file drawings format type 3, 4, and 5 are detailed in the following ASCII Format sections.

#### Drawing File ASCII Format

General rules –

- The ASCII Format type attribute FMTY that specifies the file format revision level shall be used by all API tools to provide upward compatibility and to ensure that the API can detect which formats are supported by the API tool revision level.

- An attribute is defined by its name followed by an equal sign and then the attribute value enclosed in single quotes.  Multiple values for a single attribute are separated by spaces or commas.  Order is important for these. (All attribute names are case sensitive, e.g., they shall be upper case ASCII characters , A-Z.)

For example, ASMA='7FC000,7FF400,7FFF10'

ASMS='13312,2832,240'

- All Hexadecimal digits shall be (0-9, A-F) ASCII characters.

- There shall be only one instance of an attribute per drawing.

- Each attribute shall be defined on one line.  The maximum line length is 250 characters (including terminator).

The <part number> drawing file consists of multiple lines, each of which defines a meta-data attribute.  The attributes have four character names, which are described below.  The attributes can be defined in any order (current software uses alphabetical order). Therefore, the scripts which read the drawing file should parse the data based on the attribute names instead of the position of information within the file. Attributes not defined can be omitted or defined as a null with two single quotes.  (NOTE: Other attributes may be defined and implemented between senders and receivers; however, these optional attributes shall not conflict with those defined within this document. Additional formal attributes may be added on request.)

##### Drawing File Format Type 3

##### The following section describes the attributes defined for drawing file format type 3:

KEY: 	R=Required attributes. 	O=Optional attributes.

| Drawing File format - Data Section |  |  |
| --- | --- | --- |
| Type | Attribute | Description |
| R | FMTY | ASCII File Format type version - 3 (ASCII), contains the attributes defined in this section. |
| O | ALPN | Alternate customer part number or supplier part number, not the PRTN. |
| O | ASMA | Assembler (EPROM, ROM, FLASH, etc.) starting memory addresses of Data Section. Format shall be Hexadecimal-ASCII (0-9, A-F). If there are multiple memory blocks, all starting addresses will be specified in a single ASMA attribute and separated by a comma. If no RLOC is defined, these individual starting addresses must be listed in increasing address order and cannot overlap. |
| R | ASMS | Assembler (EPROM, ROM, FLASH, etc.) memory sizes of Data Section. Format shall be Decimal. If there are multiple memory blocks, all memory sizes will be specified in a single ASMS attribute and separated by a comma. These individual memory sizes will be listed in an order that corresponds to the starting memory addresses shown in the ASMA attribute. |
| O | CHAN | Change Authorization number |
| R for emission related MY 2005 and higher O for non-emission related controllers or emission related before MY 2005 | CCVN | SAE J1979 CARB mode $09 compliant Calibration Verification Number. This number shall be reported out as 4 byte hex values, with the most significant byte reported first. If the calculation technique does not use all 4 bytes, the CVN shall be right justified and padded with $00. Format shall be Hexadecimal-ASCII (0-9, A-F). |
| O | DATE | Release date and time. The format shall be ASCII characters – ‘YYYYMMDDHHMMSS’. The Binary Header and ASCII Format DATE’s shall be identical. |
| R | DNAM | Drawing name. Examples are: ‘6511B DATA FILE-ECM OPERG SYS’ |
| O | EWON | Engineering work order number. |
| R | FILL | Fill string(s). This is a set of hexadecimal characters that specify which hexadecimal character string will fill each block to make it contiguous. Format shall be Hexadecimal-ASCII (0-9, A-F) There must be at least one fill string. The number of fill strings cannot exceed the number of sizes (ASMS). If the number of fill strings is less than the number of sizes (ASMS), the last fill string applies to the rest of the blocks. |
| O | GMLA | GMLAN Header. The structure shall be 4 fields separated by commas. The field order and definitions are as follows: Start_Addr – Location in the hex file where a copy of the GMLAN Header data can be found. Format shall be Hexadecimal-ASCII (0-9, A-F). Size – Size of the GMLAN header. Format shall be Decimal. Header_Size –Controller flash packet size. Format shall be Decimal. The GMLAN Header is padded to the “Header_Size” with the fill byte “GMLA_Fill”. GMLA_Fill - This is a set of hexadecimal characters that specify which hexadecimal character string will pad the GMLAN header. Format shall be Hexadecimal-ASCII (0-9, A-F) |
| R | KIND | Kind or type of drawing. It has a character value followed by an optional numeric value. ‘D’ - for a data file ‘B’ - for an EBOM file The first character alone is sufficient to detect KIND. Any other appended suffixes are optional. “00” – data file with fixed addresses “50” – data files with relocatable addresses |
| O | MAIR | This is the e-mail address of the controller release engineer. Multiple e-mail addresses are allowed and separated by commas. |
| O | MAIS | This is the e-mail address of the controller supplier. Multiple e-mail addresses are allowed and separated by commas. |
| O | MCOM | Miscellaneous Comments - i.e. MCOM =‘This is my miscellaneous comments.’ |
| O | MDID | Module ID. (Decimal value 00-99.) Nomenclature for multiple memory modules in FLASH programming Utility files. Module ID is a way of identifying the programming file to the utility program for reprogramming purposes. The ID starts at 00 (for the utility file itself) and each additional file is assigned numeric IDs starting with 01. Each product file and the utility file are individually considered a "module." The reprogramming tool software shall be able to handle the scenario of multiple modules in a single memory device. In addition, it shall be capable of handling several calibration files per product set. Refer to the Document GMW3110 for additional information Note: Powertrain uses 95 as the XML part Module ID, 98 as the 2ndMPU BOOT part Module ID, and 99 as the main BOOT part Module ID. |
| O | MODE | Release Mode. ‘prod’ = production data, ‘test’ = test data If omitted, "prod" is assumed. |
| R | PROG | Program name. This is the software name to be released. For a particular project, the program name designates a specific set of revisions of a program. |
| R | PROJ | Project name. This is a unique project name that specifies which set of programs are to be released. The project name, program name, and level together specify a particular revision of a particular program to be released. i.e. PROJ=’E86_10’ |
| R | PRTN | Data file and drawing file part number. (If a drawing file, this will be the same as in Binary Format - Header, Part Number (4 / 36).) (SAE J1979 CARB mode $09 compliant if 16 or less characters are used.) |
| R | RLBY | Released by. This is the computer system login ID or name of the engineer doing the release. |
| O | RLOC | Relocation Address. Format shall be Hexadecimal-ASCII (0-9, A-F). The blocks defined by ASMA and by ASMS can optionally be relocated to different memory addresses in the device. These are the device relocation addresses. If the device addresses are the same as the assembler addresses, then RLOC can be omitted. RLOC must have the same number of values defined as ASMA and ASMS. These are hexadecimal values. If specified, the relocation addresses must be in ascending order and cannot overlap. |
| R | RLVL | Release level. This is the release level that specifies the revision of the program that is being released. |
| R | RTYP | Release type. ‘pro’ =boot, operating system, or utility release, ‘cal’ = calibration release. |
| R | SITE | Release site (location). This is the name of the site where the source code for the program resides. |
| O | ZCRC | The CRC32 shall be computed using the same polynomial as in the Binary File Format. Attributes, with the exception of the ZCRC attribute, as well as all delimiters, except end-of-line delimiter(s), shall be included in the CRC32 computation. Format shall be Hexadecimal-ASCII (0-9, A-F). A 32-bit CRC of the ASCII format file (exclusive of end-of-line control character(s) and the ZCRC attribute and CRC itself). This value is intended for ASCII data integrity and verification. |

##### Drawing File Format Type 4

##### The following section describes the attributes defined for drawing file format type 4 (changes made from previous format type are highlighted):

KEY: 	R=Required attributes. 	O=Optional attributes.

| Drawing File format - Data Section |  |  |
| --- | --- | --- |
| Type | Attribute | Description |
| R | FMTY | ASCII File Format type version - 4 (ASCII), contains the attributes defined in this section. |
| O | ALPN | Alternate customer part number or supplier part number, not the PRTN. |
| O | ASMA | Assembler (EPROM, ROM, FLASH, etc.) starting memory addresses of Data Section. Format shall be Hexadecimal-ASCII (0-9, A-F). If there are multiple memory blocks, all starting addresses will be specified in a single ASMA attribute and separated by a comma. If no RLOC is defined, these individual starting addresses must be listed in increasing address order and cannot overlap. |
| O | ASMO | For compressed data blocks, - The memory sizes in ASMS indicate the memory sizes of the compressed data blocks. - The memory sizes in ASMO indicate the memory sizes of the original uncompressed data blocks. Format shall be Decimal. If there are multiple memory blocks, all memory sizes will be specified in a single ASMO attribute and separated by a comma. These individual memory sizes will be listed in an order that corresponds to the starting memory addresses shown in the ASMA attribute. |
| R | ASMS | Assembler (EPROM, ROM, FLASH, etc.) memory sizes of Data Section. Format shall be Decimal. If there are multiple memory blocks, all memory sizes will be specified in a single ASMS attribute and separated by a comma. These individual memory sizes will be listed in an order that corresponds to the starting memory addresses shown in the ASMA attribute. For compressed data blocks, - The memory sizes in ASMS indicate the memory sizes of the compressed data blocks. - The memory sizes in ASMO indicate the memory sizes of the original uncompressed data blocks. |
| O | CHAN | Change Authorization number |
| R for emission related MY 2005 and higher O for non-emission related controllers or emission related before MY 2005 | CCVN | SAE J1979 CARB mode $09 compliant Calibration Verification Number. This number shall be reported out as 4 byte hex values, with the most significant byte reported first. If the calculation technique does not use all 4 bytes, the CVN shall be right justified and padded with $00. Format shall be Hexadecimal-ASCII (0-9, A-F). |
| O | COMP | The compression algorithm type used to compress the data blocks. Format shall be Decimal. 0 = no data compression, 1 = compression algorithm type 1, 2 = compression algorithm type 2, etc. The compression algorithm type is defined by the GM Global Infrastructure software group. |
| O | DATE | Release date and time. The format shall be ASCII characters – ‘YYYYMMDDHHMMSS’. The Binary Header and ASCII Format DATE’s shall be identical. |
| R | DNAM | Drawing name. Examples are: ‘6511B DATA FILE-ECM OPERG SYS’ |
| O | EWON | Engineering work order number. |
| R | FILL | Fill string(s). This is a set of hexadecimal characters that specify which hexadecimal character string will fill each block to make it contiguous. Format shall be Hexadecimal-ASCII (0-9, A-F) There must be at least one fill string. The number of fill strings cannot exceed the number of sizes (ASMS). If the number of fill strings is less than the number of sizes (ASMS), the last fill string applies to the rest of the blocks. |
| O | GMLA | GMLAN Header. The structure shall be 4 fields separated by commas. The field order and definitions are as follows: Start_Addr – Location in the hex file where a copy of the GMLAN Header data can be found. Format shall be Hexadecimal-ASCII (0-9, A-F). Size – Size of the GMLAN header. Format shall be Decimal. Header_Size –Controller flash packet size. Format shall be Decimal. The GMLAN Header is padded to the “Header_Size” with the fill byte “GMLA_Fill”. GMLA_Fill - This is a set of hexadecimal characters that specify which hexadecimal character string will pad the GMLAN header. Format shall be Hexadecimal-ASCII (0-9, A-F) |
| R | KIND | Kind or type of drawing. It has a character value followed by an optional numeric value. ‘D’ - for a data file ‘B’ - for an EBOM file The first character alone is sufficient to detect KIND. Any other appended suffixes are optional. “00” – data file with fixed addresses “50” – data files with relocatable addresses |
| O | MAIR | This is the e-mail address of the controller release engineer. Multiple e-mail addresses are allowed and separated by commas. |
| O | MAIS | This is the e-mail address of the controller supplier. Multiple e-mail addresses are allowed and separated by commas. |
| O | MCOM | Miscellaneous Comments - i.e. MCOM =‘This is my miscellaneous comments.’ |
| O | MDID | Module ID. (Decimal value 00-99.) Nomenclature for multiple memory modules in FLASH programming Utility files. Module ID is a way of identifying the programming file to the utility program for reprogramming purposes. The ID starts at 00 (for the utility file itself) and each additional file is assigned numeric IDs starting with 01. Each product file and the utility file are individually considered a "module." The reprogramming tool software shall be able to handle the scenario of multiple modules in a single memory device. In addition, it shall be capable of handling several calibration files per product set. Refer to the Document GMW3110 for additional information Note: Powertrain uses 95 as the XML part Module ID, 98 as the 2ndMPU BOOT part Module ID, and 99 as the main BOOT part Module ID. |
| O | MODE | Release Mode. ‘prod’ = production data, ‘test’ = test data If omitted, "prod" is assumed. |
| R | PROG | Program name. This is the software name to be released. For a particular project, the program name designates a specific set of revisions of a program. |
| R | PROJ | Project name. This is a unique project name that specifies which set of programs are to be released. The project name, program name, and level together specify a particular revision of a particular program to be released. i.e. PROJ=’E86_10’ |
| R | PRTN | Data file and drawing file part number. (If a drawing file, this will be the same as in Binary Format - Header, Part Number (4 / 36).) (SAE J1979 CARB mode $09 compliant if 16 or less characters are used.) |
| R | RLBY | Released by. This is the computer system login ID or name of the engineer doing the release. |
| O | RLOC | Relocation Address. Format shall be Hexadecimal-ASCII (0-9, A-F). The blocks defined by ASMA and by ASMS can optionally be relocated to different memory addresses in the device. These are the device relocation addresses. If the device addresses are the same as the assembler addresses, then RLOC can be omitted. RLOC must have the same number of values defined as ASMA and ASMS. These are hexadecimal values. If specified, the relocation addresses must be in ascending order and cannot overlap. |
| R | RLVL | Release level. This is the release level that specifies the revision of the program that is being released. |
| R | RTYP | Release type. ‘pro’ =boot, operating system, or utility release, ‘cal’ = calibration release. |
| R | SITE | Release site (location). This is the name of the site where the source code for the program resides. |
| O | ZCRC | The CRC32 shall be computed using the same polynomial as in the Binary File Format. Attributes, with the exception of the ZCRC attribute, as well as all delimiters, except end-of-line delimiter(s), shall be included in the CRC32 computation. Format shall be Hexadecimal-ASCII (0-9, A-F). A 32-bit CRC of the ASCII format file (exclusive of end-of-line control character(s) and the ZCRC attribute and CRC itself). This value is intended for ASCII data integrity and verification. |

##### Drawing File Format Type 5

##### The following section describes the attributes defined for drawing file format type 5 (changes made from previous format type are highlighted):

KEY: 	R=Required attributes. 	O=Optional attributes.

| Drawing File format - Data Section |  |  |
| --- | --- | --- |
| Type | Attribute | Description |
| R | FMTY | ASCII File Format type version – latest revision = 5 (ASCII), contains the attributes defined in this section. |
| O | ALPN | Alternate customer part number or supplier part number, not the PRTN. |
| O | ASMA | Assembler (EPROM, ROM, FLASH, etc.) starting memory addresses of Data Section. Format shall be Hexadecimal-ASCII (0-9, A-F). If there are multiple memory blocks, all starting addresses will be specified in a single ASMA attribute and separated by a comma. If no RLOC is defined, these individual starting addresses must be listed in increasing address order and cannot overlap. |
| O | ASMO | Assembler (EPROM, ROM, FLASH, etc.) memory sizes of Data Regions. Format shall be Decimal. If there are multiple memory blocks, all memory sizes will be specified in a single ASMO attribute and separated by a comma. These individual memory sizes will be listed in an order that corresponds to the starting memory addresses shown in the ASMA attribute - For unsigned and compressed data type, the memory sizes in ASMO indicate the memory sizes of the original uncompressed data regions. - For signed and compressed data type, the memory sizes in ASMO indicate the memory sizes of the original uncompressed data regions with the signed header removed (signed header size defined in SIGN attribute). |
| R | ASMS | Assembler (EPROM, ROM, FLASH, etc.) memory sizes of Data Section. Format shall be Decimal. If there are multiple memory blocks, all memory sizes will be specified in a single ASMO attribute and separated by a comma. These individual memory sizes will be listed in an order that corresponds to the starting memory addresses shown in the ASMA attribute. - For unsigned and compressed data type, the size (single value) in ASMS indicates the size of the compressed data. - For signed and compressed data type, the size (single value) in ASMS indicates the size of the compression header plus the compressed data. For other data types, the ASMS contains same contents as ASMO. |
| O | CCID | Cal Compatibility ID. Operational software and calibrations may contain these values and the bootloader performs pre-check to verify whether the calibration data is compatible with the programmed application software during flash. Format shall be Hexadecimal-ASCII (0-9, A-F). Application Software - multiple CCID values sorted by Module ID and separated by a comma Calibration - one CCID value |
| O | DCID | Data Compatibility ID. The bootloader and the operational software contain this value and the bootloader pre-checks to verify if the application software is compatible with the bootloader software during flash Format shall be Hexadecimal-ASCII (0-9, A-F). Boot Software - one DCID value Application software - one DCID value |
| O | CHAN | Change Authorization number |
| R for emission related MY 2005 and higher O for non-emission related controllers or emission related before MY 2005 | CCVN | SAE J1979 CARB mode $09 compliant Calibration Verification Number. This number shall be reported out as 4 byte hex values, with the most significant byte reported first. If the calculation technique does not use all 4 bytes, the CVN shall be right justified and padded with $00. Format shall be Hexadecimal-ASCII (0-9, A-F). |
| O | COMP | The compression algorithm type used to compress the data blocks. Format shall be Decimal. 0 = no data compression, 1 = compression algorithm type 1, 2 = compression algorithm type 2, etc. The compression algorithm type is defined by the GM Global Infrastructure software group. |
| O | DATE | Release date and time. The format shall be ASCII characters – ‘YYYYMMDDHHMMSS’. The Binary Header and ASCII Format DATE’s shall be identical. |
| R | DNAM | Drawing name. Examples are: ‘6511B DATA FILE-ECM OPERG SYS’ |
| O | EWON | Engineering work order number. |
| R | FILL | Fill string(s). This is a set of hexadecimal characters that specify which hexadecimal character string will fill each block to make it contiguous. Format shall be Hexadecimal-ASCII (0-9, A-F) There must be at least one fill string. The number of fill strings cannot exceed the number of sizes (ASMS). If the number of fill strings is less than the number of sizes (ASMS), the last fill string applies to the rest of the blocks. |
| O | GMLA | GMLAN Header. The structure shall be 4 fields separated by commas. The field order and definitions are as follows: Start_Addr – Location in the hex file where a copy of the GMLAN Header data can be found. Format shall be Hexadecimal-ASCII (0-9, A-F). Size – Size of the GMLAN header. Format shall be Decimal. Header_Size –Controller flash packet size. Format shall be Decimal. The GMLAN Header is padded to the “Header_Size” with the fill byte “GMLA_Fill”. GMLA_Fill - This is a set of hexadecimal characters that specify which hexadecimal character string will pad the GMLAN header. Format shall be Hexadecimal-ASCII (0-9, A-F) |
| R | KIND | Kind or type of drawing. It has a character value followed by an optional numeric value. ‘D’ - for a data file ‘B’ - for an EBOM file The first character alone is sufficient to detect KIND. Any other appended suffixes are optional. “00” – data file with fixed addresses “50” – data files with relocatable addresses |
| O | MAIR | This is the e-mail address of the controller release engineer. Multiple e-mail addresses are allowed and separated by commas. |
| O | MAIS | This is the e-mail address of the controller supplier. Multiple e-mail addresses are allowed and separated by commas. |
| O | MCOM | Miscellaneous Comments - i.e. MCOM =‘This is my miscellaneous comments.’ |
| O | MDID | Module ID. (Decimal value 00-99.) Nomenclature for multiple memory modules in FLASH programming Utility files. Module ID is a way of identifying the programming file to the utility program for reprogramming purposes. The ID starts at 00 (for the utility file itself) and each additional file is assigned numeric IDs starting with 01. Each product file and the utility file are individually considered a "module." The reprogramming tool software shall be able to handle the scenario of multiple modules in a single memory device. In addition, it shall be capable of handling several calibration files per product set. Refer to the Document GMW3110 for additional information Note: Powertrain uses 95 as the XML part Module ID, 98 as the 2ndMPU BOOT part Module ID, and 99 as the main BOOT part Module ID. |
| O | MODE | Release Mode. ‘prod’ = production data, ‘test’ = test data If omitted, "prod" is assumed. |
| R | PROG | Program name. This is the software name to be released. For a particular project, the program name designates a specific set of revisions of a program. |
| R | PROJ | Project name. This is a unique project name that specifies which set of programs are to be released. The project name, program name, and level together specify a particular revision of a particular program to be released. i.e. PROJ=’E86_10’ |
| R | PRTN | Data file and drawing file part number. (If a drawing file, this will be the same as in Binary Format - Header, Part Number (4 / 36).) (SAE J1979 CARB mode $09 compliant if 16 or less characters are used.) |
| R | RLBY | Released by. This is the computer system login ID or name of the engineer doing the release. |
| O | RLOC | Relocation Address. Format shall be Hexadecimal-ASCII (0-9, A-F). The blocks defined by ASMA and by ASMS can optionally be relocated to different memory addresses in the device. These are the device relocation addresses. If the device addresses are the same as the assembler addresses, then RLOC can be omitted. RLOC must have the same number of values defined as ASMA and ASMS. These are hexadecimal values. If specified, the relocation addresses must be in ascending order and cannot overlap. |
| R | RLVL | Release level. This is the release level that specifies the revision of the program that is being released. |
| R | RTYP | Release type. ‘pro’ =boot, operating system, or utility release, ‘cal’ = calibration release. |
| O | SIGN | Signed Header Size (for signed data type and signed/compressed data type) Format shall be Decimal. |
| R | SITE | Release site (location). This is the name of the site where the source code for the program resides. |
| O | ZCRC | The CRC32 shall be computed using the same polynomial as in the Binary File Format. Attributes, with the exception of the ZCRC attribute, as well as all delimiters, except end-of-line delimiter(s), shall be included in the CRC32 computation. Format shall be Hexadecimal-ASCII (0-9, A-F). A 32-bit CRC of the ASCII format file (exclusive of end-of-line control character(s) and the ZCRC attribute and CRC itself). This value is intended for ASCII data integrity and verification. |

### End Model Bill of Materials (EBOM) Format

Each provider of software and calibrations data files and drawings shall be responsible for the following EBOM contents.

- Hardware part number of the controller, a software configuration of data files to be programmed by the controller supplier, and optionally manufacturing programming instructions are combined into a bill of materials, consisting of an itemized list of part numbers used to build an End Model. This list shall also contain identification of the Utility software used by the Vehicle Assembly plant and Service programming tools.  The controller supplier may require additional information to complete the EBOM data for manufacturing requirements.

- The itemized list provided by the software and calibration supplier shall be constructed in attribute sequence as follows using the attributes detailed in the following ASCII Format Table:

- nn-th Part Number of the nn-th Part Type

- nn-th Part Type of the nn-th Part Number

- nn-th Device Start Address (Absolute Address in HEX) of the nn-th Part Number [optional]

- nn-th Device Size (Byte Count in Decimal) of the nn-th Part Number [optional]

- nn-th Part Description of the nn-th Part Number [optional]

#### EBOM File ASCII Format

This table contains the

- Required attributes FMTY, PRTN, and KIND common to the Drawing File ASCII Format.

- Attributes specific to an EBOM file.

Where appropriate, other optional attributes as defined in the Drawing File ASCII Format (such as DATE, etc.) shall be used consistently within this EBOM file ASCII format.

| BILL OF MATERIALS DRAWING format - Data Section |  |  |
| --- | --- | --- |
| Type | Attribute | Description |
| R | FMTY | EBOM ASCII File Format version - current revision = 2 (ASCII) |
| R | PRTN | The end model part number (EBOM controller assembly part number). |
| R | KIND | Kind or type of drawing. KIND =‘B’ -- bill of materials or EBOM |
| The following are EBOM specific attributes |  |  |
| R | PNnn | nn-th Part number contained in the end model (controller assembly) enclosed by single quotes, where nn = 01, 02,...99. |
| R | PTnn | nn-th Part type contained in the end model (controller assembly) enclosed by single quotes, where nn = 01, 02,...99: ‘H’ = Hardware; ‘U’ = Utility file used in the programming or re-programming of a controller; ‘D’ = Data File; ‘A’ = Another or alternate |
| O | DAnn | nn-th Device addresses (in HEX) enclosed by single quotes and corresponding to the starting address for each part number contained in the end model (controller assembly), where nn = 01, 02,...99. . This can only be used for PTnn = ‘D’ type objects. |
| O | DSnn | nn-th Device size (decimal) enclosed by single quotes corresponding to each part number (PNnn) contained in the end model (controller assembly), where nn = 01, 02,...99. Both DAnn and DSnn are required when the individual data file header section does not have address information. This can only be used for PTnn = ‘D’ type objects. |
| O | PDnn | nn-th Part description enclosed by single quotes corresponding to each part number (PNnn) contained in the end model (controller assembly), where nn = 01, 02,...99. |
| O | MCOM | Miscellaneous Comments - (enclosed by single quotes), i.e. MCOM=‘This is my miscellaneous comments.’ |
| O | MFmmm | mmm-th Manufacturing Programming Instructions ‘line’- (enclosed by single quotes, where mmm = 001, 002,...999; or 01, 02, …999) |
| O | NUMP | Total decimal number of data parts identified by unique part identifiers which are to be programmed into a controller by the supplier (01…99). |

#### Manufacturing Programming Instructions attributes requirements

The manufacturing requirements can be placed within the EBOM drawing MFmmm attributes. The detailed requirements are in the “Controller Manufacturing Programming Instructions Standard”.

Controller products that use re-programmable FLASH memory are required to contain either EEPROM or EEPROM Emulation as well. Ownership of the information necessary to program EEPROM resides with both the software supplier and the controller product development team. Therefore, the provider of software and calibrations data files and drawings, and the product development team for the controller shall work together to create Manufacturing Programming Instructions, but the release of the EBOM drawing shall occur through the software supplier’s organization.  In addition to EEPROM contents, the Manufacturing Programming Instructions conveys the security algorithm type assigned to a particular controller product. For product security reasons, the contents of EEPROM are different during the development of the product than at production.  They are also different for controllers manufactured specifically for the GM Service Operations. A unique EBOM drawing file will need to be released for each type of configuration (GM vehicle assembly, GM Service Operations, etc.).

# CRC Codes Defined

CRC Codes Implementation Concepts

Mathematically, a CRC is the remainder of a polynomial division of the data by a certain polynomial chosen for its error detection properties.  The least significant bit of the first byte of the data is the most significant bit of the (possibly huge) dividend.  The division is actually binary division: modulo two.  The process can be performed in hardware by a shift register.  That shift register is simulated in software.

Because of the superior error detection of the 32 bit polynomial, it is recommended.

For 32 bits, the CRC-32 polynomial is:

x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1

For all CRC polynomials, the most significant bit is set.  Therefore it can be dropped to get a HEX representation of the polynomial.  For the 32 bit polynomial, this is 0X04C11DB7.

This dropped bit is taken into account in the construction of the shift register.

Because serial data is sent least significant bit first, the polynomial is reversed.  The reversed 32 bit polynomial is 0XEDB88320. The starting CRC value is always set to ~0 (0XFFFFFFFF.)

This reversed polynomial is used in all of the calculations. To append a CRC value to data, the CRC bytes should be placed in the data from the least to the most significant byte of the CRC value generated.  The CRC bytes appended are the ones complement of the CRC value. This will make the CRC of the data plus appended value a constant (0XDEBB20E3.)

CRC Calculation Examples

These three examples of actual calculations have been included to “prove” the implementation. You really can’t prove it (which would require an infinite number of test patterns) but you can reduce the initial implementation problems.

The first two tests are based on a simple “dbi” file which has the following text:

TEST='This is a test string'
ZCRC='FA378374'

In Unix binary format this file would look like:

00000000: 54 45 53 54 3D 27 54 68 69 73 20 69 73 20 61 20  |TEST='This is a |
00000010: 74 65 73 74 20 73 74 72 69 6E 67 27 0A 5A 43 52  |test string'.ZCR|
00000020: 43 3D 27 46 41 33 37 38 33 37 34 27 0A           |C='FA378374'.   |

In Windows binary format this file would look like:

00000000: 54 45 53 54 3D 27 54 68 69 73 20 69 73 20 61 20  |TEST='This is a |
00000010: 74 65 73 74 20 73 74 72 69 6E 67 27 0D 0A 5A 43  |test string'..ZC|
00000020: 52 43 3D 27 46 41 33 37 38 33 37 34 27 0D 0A     |RC='FA378374'.. |

Both of these files should pass the CRC test before the algorithm can be assumed to be correct. In addition functions written to append the CRC onto dbi files should correctly calculate and append the line:

ZCRC='FA378374'

to a file containing just the line:

TEST='This is a test string'

The third test is based on a regular binary (i.e. non-ASCII) file:

00000000: 54 68 69 73 20 69 73 20 61 20 74 65 73 74 20 73  |This is a test s|
00000010: 74 72 69 6E 67                                   |tring           |

This file should produce a CRC of 0x94B10C92 which when bit-wise inverted (i.e. 0x94B10C92 XOR 0xFFFFFFFF) is 0x6B4EF36D and which appended (LSB first) to the file makes it look like:

00000000: 54 68 69 73 20 69 73 20 61 20 74 65 73 74 20 73  |This is a test s|
00000010: 74 72 69 6E 67 6D F3 4E 6B                       |tringm.Nk       |

If the CRC is calculated on this new file it must be 0xDEBB20E3 which is the required constant which indicates that the file is not corrupted.

Note:  When 0xDEBB20E3 is bit-wise inverted (i.e. 0xDEBB20E3 XOR 0xFFFFFFFF), the result is 0x2144DF1C.

APPENDIX SECTION

Appendix A - Revision History

| Revision Date | Author | Description | Change Request Number(s) |
| --- | --- | --- | --- |
| February 28, 2014 | Yiing Maa Monica Mitchell | Added new sections for - Data File Format Type 3 - Drawing File Format Type 4 and 5 - Layout of API binary file examples Also added compression/decompression “giz” info and general purpose tool “bintool” info. Added GDFRS changes to include dlspls in API functions. Added GDFRS changes to include new return strings for API functions | EACR E55699 (secure programming) EACR E56763 (software compression) GDFRS story #175 |
| November 26, 2012 | Yiing Maa | Updated/added the following sections per GDFRS EVO project based on the GDFRS project team’s input (Lou Perlotto and Mark Cosens) - 2.1.1.2 (API Functions) - Appendix B ( Glossary) - Appendix F (GM File Naming Conventions) - Appendix J (Example of GDFRS upload function ) | GDFRS EVO project |
| June 7, 2012 | Yiing Maa | Updated cal compression info in the PTI file (Data Section Compression algorithm type, Spare) and DBI file (ASMS, ASMO, COMP, FMTY). | E51350 |
| July 15, 2010 | Yiing Maa | Updated many sections with up-to-date information. Removed obsolete sections (i.e. Delphi data file related requirements). Added GDFRS specific requirements. |  |
| July 10, 2009 | Yiing Maa | Updated to the new Document Format Standard. Also changed the document title and owner (no content change). | E43637 |
| March 27, 2003 | Gregory T. Ambrose | Modifications to handle prepended GMLAN Header information. These changes included updating the Binary Format Type to 2, adding the new binary Data Type attribute, modifying the spare area. In order to support APImake functionality (S-record to API binary conversion), a new drawing file attribute was added called GMLA along with updating the Drawing Format Type to 3. |  |
| February 3, 1999 | Jerry D. Laney | Latest corrections to released document based on new API formats agreements between EDS, Delphi Delco, and GMPT implementation experiences |  |
| March 14, 1997 | Jerry D. Laney | Latest corrections to released document based on new API formats agreements between EDS, Delphi Delco, and GMPT implementation experiences |  |
| January 9, 1997 | Jerry D. Laney | Latest corrections to released document based on oversight committee status review meeting plus latest CARB/SAE-J1979 requirements |  |
| March 29, 1996 | Jerry D. Laney | Latest corrections prior to formal acceptance |  |
| March 25, 1996 | Doris A. Fisher | Delphi Delco Electronics re-organization of document based on March 22, 1996, conference call |  |
| March 22, 1996 | Doris A. Fisher | Delphi Delco Electronics clarifications from review on March 20, 1996 |  |
| March 20, 1996 | Craig A. Brown | Clarifications from review on March 20, addressing Eugene’s comments |  |
| March 14, 1996 | Jerry D. Laney | Updates from technical team meetings resulting in the communications protocol decision |  |
| January 24, 1996 | Jerry D. Laney | Updates from multiple technical team, oversight committee meetings and individual critiques |  |
| November 30, 1995 | Jerry D. Laney | Updates from 11/29/95 GMTG, DE, GMPT, GM/NAO-MIS, EDS, GMSO review meeting |  |
| November 27, 1995 | Jerry D. Laney | Clean up for 11/28/95 Meeting |  |
| November 21, 1995 | Jerry D. Laney Randy Sencaj | Updates from 11/10/95 documentation reviews - Delphi Delco CRC codes and sample modules |  |
| November 10, 1995 | Jerry D. Laney Randy Sencaj | Updates from 11/2/95 Meeting feedback - Delphi Delco drawings data definitions updated |  |
| October 30, 1995 | Jerry D. Laney Scott Kirchner Randy Sencaj | Updates from 10/11/95 & 10/25/95 Meetings - GMTG - Delphi Delco Electronics |  |
| September 27, 1995 | Jerry D. Laney | Dave Poirier review and recommendations |  |
| September 25, 1995 | Jerry D. Laney | Kathy Rose review and suggestions |  |
| September 22, 1995 | Jerry D. Laney | Delphi Delco / GMTG Meeting feedback |  |
| September 21, 1995 | Jerry D. Laney | Original Draft |  |

Appendix B – Glossary

The following acronyms are used throughout this document:

| ASM | Assembly of components in a build of hardware or software |
| --- | --- |
| BASE MODEL | Identifies the part number of the controller hardware. |
| CALN | Calibration data file |
| CRC | Cyclic Redundancy Check (similar to but more sophisticated than a checksum). |
| CPFA | Controller Production File Access, GM global data file repository |
| DIAGN | Diagnostics calibration data file. |
| EBOM | End model Bill Of Materials - a list of part numbers used in an End Model (Controller Assembly) build of hardware, software and calibrations files being programmed into a controller hardware item. |
| END MODEL | Sometimes called CONTROLLER ASSEMBLY. An End Model normally contains the following: - Hardware part number - Main boot software part number - Main operating software part number - Any 2nd/3rd/4th boot/operating software part numbers (if applicable) - Any 2nd/3rd/4th operating software part numbers (if applicable) In the case of Non-Programming in the Assembly Plants, the calibration part numbers also become a part of this assembly. |
| ENG | Engine (automobile - diesel or gasoline) |
| EVO | Evolutionary (GDFRS project) |
| GDFRS | Global Data File Release System, future GM global data file repository |
| GMLAN | General Motors vehicle local area network (LAN) header for the GM standard CAN protocol. |
| GMSE | GM supplier extranet |
| HTTP/HTTPS | World Wide Web's Hypertext Transfer Protocol that uses TCP/IP to get to the Internet |
| MIME | Is the official proposed standard format for extended InterNet electronic mail. The MIME format permits e-mail to include enhanced text, graphics, audio, and more, in a standardized and interoperable manner. If you send and receive main with a MIME-compliant mail system, you will be able to send and receive far more than ASCII text. |
| OPERATING SYS | Operating system for controllers containing all applications software, real-time operating system, and GM/vendor developed I/O drivers. |
| RPO | Regular Production Option |
| TCP/IP | Transmission Control Protocol/Internet Protocol of the internet |
| TRANS | Transmission (automobile, automatic or manual) |
| UTILITY | Utility file (module ID 00) used in controller programming. This file is used by the assembly plant programming tools (i.e. PPTH -Portable Programming and Test Head /NGFTT - Next Generation Flash Test Tool), the DPS Tools (Development), and the SPS Tools (Service Reprogramming). A utility file has 3 sections: Header, programming instructions, and reprogramming algorithms. A utility file is required for every end model containing reprogrammable memory (not just FLASH), but one utility file may serve all production end models for one product. |
| VEH SPD SEN CALN | Vehicle (platform) speed sensor calibrations. |
| VEH SYS CALN | Vehicle (platform) systems calibrations including HVAC, passlock, etc. |
| VDS | Vehicle Description System |

Appendix C - Related Documents

The following documents are either referenced by this document or contain related information.  These documents may provide useful information to the reader and should be reviewed for compatibility in the event that this document (or any of the related documents) undergoes revision.

|  | Document Title | Document Filename |
| --- | --- | --- |
| [1] | GMLAN ENHANCED DIAGNOSTIC TEST MODE SPECIFICATION GMW 3110 | GMW 3110 |
| [2] | Controller Programming Manufacturing Instructions Standard | Controller_Programming_Mfg_Instructions.docx |

The following pages contain descriptive text and suggestions for items designed to clarify the use of the API.

NOTE: These scenarios have been added to this document for clarification purposes only and do NOT imply that any API member organization is required to provide access to any proprietary hardware or software part information, nor do these scenarios imply any hidden requirements.

Appendix D - GM Concepts of API Functionality

The provider of the controller software and calibration data has completed early development and testing and wants to have controllers built with certain data files. Such a scenario would be applicable to both development testing as well as production. The data file provider is now preparing to transmit his ‘soft’ parts to the controller supplier.

The following steps would occur:

- The software/calibration provider will use the standard format (defined earlier in this document) to build an EBOM (End Model Bill of Materials), the related drawing files, and the related binary files. These files will be uploaded onto the GM global data file repository.

- The software/calibration provider will send an email to the controller supplier to inform them of the release.

- The controller provider (supplier) will connect to GM/Infranet network using the secured connection method to download the EBOM (End Model Bill of Materials), the related drawing files, and the related binary files.

- Once these downloaded files are received, the controller supplier can process the EBOM, the part drawing files, and convert the software and calibration API binary files into a format usable by their controller manufacturing flash tool.

- The software/calibration provider and the controller provider (supplier) shall verify the controller contents to ensure the correct binary files and manufacturing requirements are flashed into the controller correctly.

An alternative scenario allowing retrieval by the user of the software and calibration data is as follows:

- The software/calibration provider updates the part number system (could be a GM part number or a customer part number) and archives the formal, documented version of the data in an official repository.  These actions shall be initiated upon receipt of authorization from a previously agreed to source which may be internal or external, e.g. a customer.

- The software/calibration provider will send an email to the customer that data is available for use.  Examples of customers are:

- a non-GM assembly plant and associated service organization

i.e. GM sells transmission with controller to another car manufacturer.

- a non-GM controller supplier, the GM assembly plant using the controller, and GM-SO

i.e. GM buys transmission with controller from an outside transmission manufacturer.

- Once the data is available, the customer may request that it be sent to the agreed upon electronic address.

Appendix E - End Model (Controller Assembly) Configurations

The end model (controller assembly) is used by vehicle assembly plants and service organizations.  It is therefore important to include a short description of possible end models to be communicated across the API.

The following are examples of some end model configurations; due to technology changes other configurations are possible:

- A controller with non-reprogrammable memory (ROM) fully programmed when delivered by the supplier.

- A controller with a mixture of non-reprogrammable and reprogrammable memory that is fully programmed when delivered by the supplier and contains a reference to the software used by service for reprogramming.

- A controller with more than one processor and associated memory devices. This controller is fully programmed when delivered by the supplier and contains a reference to the software used for reprogramming.

- A controller with non-reprogrammable memory (ROM) that is pre-programmed and reprogrammable memory that is shipped in an erased state and a reference to the software for reprogramming.

- A controller with reprogrammable memory that is partially programmed (with just communication routines) and a reference to the software for reprogramming.

- A controller with reprogrammable memory that is partially programmed (contains the application software but the calibration areas are left erased for assembly plant programming) and a reference to the software used by service for reprogramming.

For all controllers containing reprogrammable memory certain information is required to be stored in EEPROM or EEPROM emulation in the Flash. This is also part of the end model.

In order to ensure this information is communicated consistently, GM has created the “Controller Manufacturing Programming Instructions Standard” to define the format of the manufacturing programming instructions in the controller end model bill of materials (EBOM) drawing.  Manufacturing programming instructions are used to communicate the information that the controller supplier must program other than software and calibration data. This includes the following:

- Information for the Controller label.

- Information which is required by GM to support testing, manufacturing, and service.

- Data that must be written by the controller supplier for the following reasons:

- data is not the same for each controller

- data must be initialized to a state that is different from a reprogramming update

- data is not known at release time

Appendix F - GM File Naming Conventions

It is required that the filename and extension be standardized for GM as follows:

- For Production released parts, the filename of the data file shall be an 8 digit part number followed by the file extension, i.e. 12345678.pti.

- For Development or Pr-Prod released parts, the filename of the data file shall be an 8 digit part number plus 2-alpha (DLS/PLS) followed by the file extension, i.e. 12345678AA.pti.

- For PPV released parts, the filename of the data file shall be an 8 digit part number plus z#-suffix (where # is 0-9) for non-EVO Ready releases followed by the file extension (eg: 12345678Z0.pti) or 2-alpha (DLS/PLS) for EVO Ready releases followed by the file extension (12345678AA.pti)

- For Production released parts, the filename of the data file shall be an 8 digit part number followed by the file extension (eg: 12345678.pti)

- Drawing files (for data files and EBOMs) shall follow the same naming convention as the data files except with the file extension .dbi.

Appendix G - GM Scenarios for Inter-Organization Communication

GM proposes to construct its interface to the Common API to provide ‘soft’ parts to its controller suppliers, GM/SO, and GM Assembly Plants.

The controller hardware, software, and calibrations ‘soft’ parts are to be assembled into an Engineering End Model Bill of Materials (EBOM) based on the Functional Name Address (FNA) codes, the Functional Names, and the UPC’s . Please note the Items to Supplier column in the following tables. Each item will be either:

- a module assembly item with an accompanying bill of materials (EBOM) information which can be displayed or printed as a drawing using a drawing template.

- a base hardware item with an accompanying standard engineering drawing (or electronic drawing file);

- or a data file with an accompanying set of information which can be displayed or printed as a drawing designating the filename and location of this ‘soft’ part data file.

EBOM information required to supply partially programmed controllers, i.e.

- programming in the assembly plant

For example, the part numbers for calibration files are not included in an EBOM. Therefore, the programming of calibration files into the controller is the responsibility of the assembly plant.

| Indent | Part Number | FNA | FNA Description | Files to Supplier (experimental release) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ‘Soft’ Part data file | Info. Drawing file |
| 0 | 11126750 | 0500M | Module ASM-ENG CONT (W/ 2nd MPU) & (W/O CALN) | N/A | 11126750AA.dbi |
| 1 | 16126744 | 0500D | Module ASM-ENG CONT (W/O CALN & OPERG SYS) | N/A | 16126744AH.dbi |
| 1 | 11126660 | 6512B | DATA FILE-ECM UTILITY | 11126660AA.pti | 11126660AA.dbi |
| 1 | 11126663 | 6513B | DATA FILE-ECM BOOT PRGM | 11126663AA.pti | 11126663AA.dbi |
| 1 | 11126664 | 6511B | DATA FILE-ECM OPERG SYS | 11126663AA.pti | 11126664AA.dbi |
| 1 | 11126665 | 6511E | DATA FILE-ECM (W/ 2nd MPU) OPERG SYS | 11126665AA.pti | 11126665AA.dbi |
| 1 | 11126666 | 6513E | DATA FILE-ECM (W/ 2nd MPU) BOOT PRGM | 11126666AA.pti | 11126666AA.dbi |

Note:  The EBOM retrieved shall contain the part numbers and descriptions of the controller assembly and associated base model, data files, and manufacturing programming instructions.

EBOM information required to supply fully programmed controllers, i.e.

- non-programming in assembly plant (supplier flashed)

- programming in assembly plant (manufacturing cals flashed by supplier)

- service controller which contains no-start cal(s)

| Indent | Part Number | FNA | FNA Description | Files to Supplier (production release) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ‘Soft’ Part data file | Info. Drawing file |
| 0 | 11126750 | 0500H | Module ASM-ENG CONT (W/ 2nd MPU) | N/A | 11126750.dbi |
| 1 | 16126744 | 0500D | Module ASM-ENG CONT (W/O CALN & OPERG SYS) | N/A | N/A |
| 1 | 11126660 | 6512B | DATA FILE-ECM UTILITY | 11126660.pti | 11126660.dbi |
| 1 | 11126663 | 6513B | DATA FILE-ECM BOOT PRGM | 11126663.pti | 11126663.dbi |
| 1 | 11126664 | 6511B | DATA FILE-ECM OPERG SYS | 11126664.pti | 11126664.dbi |
| 1 | 11126671 | 6501B | DATA FILE-ECM ENG CALN | 11126671.pti | 11126671.dbi |
| 1 | 11126672 | 6501S | DATA FILE-ECM ENG DIAGN | 11126672.pti | 11126672.dbi |
| 1 | 11126673 | 6502X | DATA FILE-ECM FUEL SYS CALN | 11126673.pti | 11126673.dbi |
| 1 | 11126674 | 6506B | DATA FILE-ECM VEH SPD SEN CALN | 11126674.pti | 11126674.dbi |
| 1 | 11126675 | 6504P | DATA FILE-ECM VEH SYS CALN | 11126675.pti | 11126675.dbi |
| 1 | 11126665 | 6513E | DATA FILE-ECM (W/ 2nd MPU) BOOT PRGM | 11126665.pti | 11126665.dbi |
| 1 | 11126666 | 6511E | DATA FILE-ECM (W/ 2nd MPU) OPERG SYS' | 11126666.pti | 11126666.dbi |
| 1 | 11126676 | 6501E | DATA FILE-ECM (W/ 2nd MPU) ENG CALN | 11126676.pti | 11126676.dbi |

Appendix H - Layout of API Binary Files examples

Data File Format Type 2 -

A Cal (1 data region) -

Memory Content

| 0x4000 0x4FFF | Cal region |
| --- | --- |

Associated API Binary File

|  | Cal Block - API Header (with Address section) |
| --- | --- |
| Cal region | Cal content |
|  | Cal Block - API CRC section |

| Cal binary API (Cal region) |  |  |
| --- | --- | --- |
| Offset | Content | Comments |
| 0 | 0x00000002 | Format Type = 2 |
| 4 | 0x313233343536373900000000…0x00 | 12345679 in ASCII, Null filled = 0x00 |
| 40 | 0x00000001 | Block Nr.1 (unsigned binary integer) |
| 44 | 0x00000001 | No. of Blocks (unsigned binary integer) |
| 48 | 0x3230303930333331323233303333 | Date (20090331223033): 03/31/2009 22:30:33 in ASCII |
| 62 | 0x00 | Normal data block |
| 63 | 0x00 | Compression algorithm (not compressed) |
| 64 | 0x00 … 0x00 | Spare (20 bytes = Null filled = 0x00) |
| 84 | 0x00000004 | No. Address Bytes -- 4 = 32 bit address |
| 88 | 0x00001000 | length Cal part region = 0x1000 |
| 92 | 0x00000001 | CRC32 method |
| 96 | 0x00000004 | CRC32 length = 4 bytes |
| 100 | 0x00004000 | Cal part start address = 0x4000 |
| 104 | cal memory content | data section (memory content of Cal region) |
| 104 + 0x1000 | 0x7AFF0080 | CRC32 value |

An Op SW (Operational Software) with 2 data regions (and a GMLAN header is required) -

Memory Content

| 0x10000 0x1BFFF | OP SW region 1 |
| --- | --- |
|  | not used |
| 0x20000 0x1BFFFF | OS SW region 2 |

Associated API Binary File

|  | GMLAN - API Header section (without address section) |
| --- | --- |
| GMLAN prepended | GMLAN prepended (size is 0x0800) |
|  | GMLAN - API CRC section |
|  | OP SW region 1 - API Header (with Address section) |
| OP SW region 1 | Op SW region 1 content |
|  | OP SW region 1 - API CRC section |
|  | OP SW region 2 - API Header (with Address section) |
| OP SW region 2 | OP SW region 2 content |
|  | OP SW region 2 - API CRC section |

| GMLAN API (GMLAN header) |  |  |
| --- | --- | --- |
| Offset | Content | Comments |
| 0 | 0x00000002 | Start API Block Nr.1 - Format version #2 |
| 4 | 0x313233343536373800000000…0x00 | 12345678 in ASCII, Null filled = 0x0 |
| 40 | 0x00000001 | Block Nr.1 (unsigned binary integer) |
| 44 | 0x00000003 | No. of Blocks (unsigned binary integer) |
| 48 | 0x3230303930333331323233303333 | Date (20090331223033): 03/31/2009 22:30:33 in ASCII |
| 62 | 0x01 | GMLAN data block |
| 63 | 0x00 | Compression algorithm (not compressed) |
| 64 | 0x00 … 0x00 | Spare (20 bytes = Null filled = 0x0) |
| 84 | 0x00000000 | No. Address Bytes -- 0 = no address |
| 88 | 0x00000800 | length GMLAN = 0x0800 |
| 92 | 0x00000001 | CRC32 method |
| 96 | 0x00000004 | CRC32 length = 4 byte |
| 100 | prepended GMLAN content | data section (GMLAN content) |
| 100 + 0x800 | 0x09DE7860 | CRC32 value |
| OP SW binary API (OP SW region 1) |  |  |
| Offset | Content | Comments |
| 0 | 0x00000002 | Start API Block Nr.2 - Format version #2 |
| 4 | 0x313233343536373800000000…0x00 | 12345678 are ASCII, Null filled = 0x0 |
| 40 | 0x00000002 | Block Nr.2 (unsigned binary integer) |
| 44 | 0x00000003 | No. of Blocks (unsigned binary integer) |
| 48 | 0x3230303930333331323233303333 | Date (20090331223033): 03/31/2009 22:30:33 in ASCII |
| 62 | 0x00 | Normal data block |
| 63 | 0x00 | Compression algorithm (not compressed) |
| 64 | 0x00 … 0x00 | Spare (20 bytes = Null filled = 0x0) |
| 84 | 0x00000004 | No. Address Bytes -- 4 = 32 bit address |
| 88 | 0x0000C000 | length OS SW part region 1 = 0xC000 |
| 92 | 0x00000001 | CRC32 method |
| 96 | 0x00000004 | CRC32 length = 4 byte |
| 100 | 0x00010000 | OS SW part region 1 start address = 0x10000 |
| 104 | OpSW region 1 content | data section (memory content of OS SW region 1) |
| 104 + 0xC000 | 0x6EF8970C | CRC32 value |
| OP SW binary API (OP SW region 2) |  |  |
| 0 | 0x00000002 | Start API Block Nr.3 - Format version #2 |
| 4 | 0x313233343536373800000000…0x00 | 12345678 are ASCII, Null filled = 0x0 |
| 40 | 0x00000003 | Block No.3 (unsigned binary integer) |
| 44 | 0x00000003 | No. of Blocks (unsigned binary integer) |
| 48 | 0x3230303930333331323233303333 | Date (20090331223033): 03/31/2009 22:30:33 in ASCII |
| 62 | 0x00 | Normal data block |
| 63 | 0x00 | Compression algorithm (not compressed) |
| 64 | 0x00 … 0x00 | Spare (20 bytes = Null filled = 0x0) |
| 84 | 0x00000004 | No. Address Bytes -- 4 = 32 bit address |
| 88 | 0x001A0000 | length OS SW part region 2 = 0x001A0000 |
| 92 | 0x00000001 | CRC32 method |
| 96 | 0x00000004 | CRC32 length = 4 byte |
| 100 | 0x00020000 | OS SW part region 2 start address = 0x00020000 |
| 104 | Op SW region 2 content | data section (memory content of OS SW region 2) |
| 104 + 0x1A0000 | 0xCD302010 | CRC32 value |

Data File Format Type 3 -

A Cal (1 data region) – signed; compressed

Memory Content

| 0x4000 0x4FFF | Cal region |
| --- | --- |

Associated API Binary File

|  | Cal - API Header (signed/compressed block) |
| --- | --- |
|  | Compression Header (i.e. 8 bytes) |
| Cal region | signed and compressed Cal region content (i.e. 0x0500 bytes) |
|  | API CRC section |

| Cal binary API (Cal region) |  |  |
| --- | --- | --- |
| Offset | Content | Comments |
| 0 | 0x00000003 | Format Type = 3 |
| 4 | 0x313233343536373900000000…0x00 | 12345679 in ASCII, Null filled = 0x00 |
| 40 | 0x00000001 | Block Nr.1 (unsigned binary integer) |
| 44 | 0x00000001 | No. of Blocks (unsigned binary integer) |
| 48 | 0x3230303930333331323233303333 | Date (20090331223033): 03/31/2009 22:30:33 in ASCII |
| 62 | 0x04 | Signed and Compressed block |
| 63 | 0x01 | Compression Algorithm |
| 64 | 0x03 | Compression Header Parameter |
| 65 | 0x00 … 0x00 | Spare (11 bytes = Null filled = 0x00) |
| 76 | 0x00000508 | Length of Block |
| 80 | 0x0008 | Length of Compression Header |
| 82 | 0x0360 | Length of Signed Header |
| 84 | 0x0000 | Length of GMLAN Header |
| 86 | 0x0001 | Number of Data Regions |
| 88 | 0x00004000 | Address of Data Region 1 |
| 92 | 0x00001000 | Length of Data Region 1 |
| 96 | 0x00000001 | CRC32 method |
| 100 | 0x00000004 | CRC32 length = 4 byte |
| 104 | Signed and compressed cal content | data section (0x508 bytes) |
| 104 + 0x0508 | 0x7AFF0080 | CRC32 value |

An Op SW (Operational Software) with 2 data regions – signed; not compressed; no GMLAN header

Memory Content

| 0x20000 0x3FFFF | OP SW region 1 |
| --- | --- |

| 0x180000 0x3FFFFF | OS SW region 2 |
| --- | --- |

Associated API Binary File

|  | OP SW - API Header (signed block) |
| --- | --- |
|  | Signed header for 2 data blocks (i.e. 0x370 bytes) |
| OP SW region 1 | 0x20000 0x3FFFF |
| OS SW region 2 | 0x180000 0x3FFFFF |
|  | OP SW - API CRC section |

| GMLAN API (GMLAN header) |  |  |
| --- | --- | --- |
| Offset | Content | Comments |
| 0 | 0x00000003 | Start API Block Nr.1 - Format version #3 |
| 4 | 0x313233343536373800000000…0x00 | 12345678 in ASCII, Null filled = 0x0 |
| 40 | 0x00000001 | Block Nr.1 (unsigned binary integer) |
| 44 | 0x00000001 | No. of Blocks (unsigned binary integer) |
| 48 | 0x3230303930333331323233303333 | Date (20090331223033): 03/31/2009 22:30:33 in ASCII |
| 62 | 0x02 | Signed block |
| 63 | 0x00 | Compression Algorithm |
| 64 | 0x00 | Compression Header Parameter |
| 65 | 0x00 … 0x00 | Spare (11 bytes = Null filled = 0x0) |
| 76 | 0x002A0370 | Length of Block |
| 80 | 0x0000 | Length of Compression Header |
| 82 | 0x0370 | Length of Signed Header |
| 84 | 0x0000 | Length of GMLAN Header |
| 86 | 0x0002 | Number of Data Regions |
| 88 | 0x00020000 | Address of Data Region 1 |
| 92 | 0x00020000 | Length of Data Region 1 |
| 96 | 0x00180000 | Address of Data Region 2 |
| 100 | 0x00280000 | Length of Data Region 2 |
| 104 | 0x00000001 | CRC32 method |
| 108 | 0x00000004 | CRC32 length = 4 byte |
| 112 | Signed OpSW content | data section (0x002A0370 bytes) |
| 112 + 0x2A0370 | 0x09DE7860 | CRC32 value |

Appendix I - Example of API Binary Files and Drawing Files

EBOM (drawing file) –

DBI FMTY = 3 & PTI format type = 2 -

Utility (drawing file & binary file) –

Boot (drawing file & binary file) –

Operating Software (drawing file & binary file) –

Trans Cal (drawing file & binary file) –

Trans Diag Cal (drawing file & binary file) –

Veh Sys Cal (drawing file & binary file) –

Boot (drawing file & binary file) with two data regions –

Appendix J - Example of GDFRS upload function

GDFRS upload function only – NOT EVO Ready – Part data file – PPV status

FUNC=upload&PRTN=12345678Z0&DLS=Z0&TYPE=data&FRMT=binary&STAT=ppv&CPFA=Platform

GDFRS upload function only – NOT EVO Ready – Part drawing File – PPV status

FUNC=upload&PRTN=12345678Z0&DLS=Z0&TYPE=drawing&FRMT=ascii&STAT=ppv&PLTUSE=Y&CPFA=Platform

GDFRS upload function only – NOT EVO Ready – Part data file – Production status

FUNC=upload&PRTN=12345678&DLS=Z5&TYPE=data&FRMT=binary&STAT=prod&CPFA=Powertrain

GDFRS upload function only – NOT EVO Ready – Part drawing File – Production status

FUNC=upload&PRTN=12345678&DLS=Z5&TYPE=drawing&FRMT=ascii&STAT=prod&PLTUSE=Y&CPFA=Powertrain

GDFRS upload function only – EVO Ready – Part data file – PPV status

FUNC=upload&PRTN=12345678AA&DLS=AA&TYPE=data&FRMT=binary&STAT=ppv&CPFA=Platform

GDFRS upload function only – EVO Ready – Part drawing File - Single Model Year / Product Code and single Build Event Designator – PPV status

FUNC=upload&PRTN=12345678AA&DLS=AA&TYPE=drawing&FRMT=ascii&STAT=ppv&PLTUSE=Y&&CPFA=PowertrainUSE=2012.0G.OAN.0GA.|ALL|

GDFRS update function only – EVO Ready –PPV Status – Single Model Year / Product Code and single Build Event Designator – PPV status

FUNC=update&PRTN=12345678AA&DLS=AA&STAT=ppv&PLTUSE=Y&CPFA=Powertrain&USE=2012.0G.OAN.0GA.|ALL|

GDFRS upload function only – EVO Ready – Part drawing File - Single Model Year / Product Code and multiple Build Event Designators – PPV status

FUNC=upload&PRTN=12345678AA&DLS=AA&TYPE=drawing&FRMT=ascii&STAT=ppv&PLTUSE=Y&CPFA=Powertrain&USE=2012.0G.OAN.0GA.|ALL|,OAN.0GB.FAI

GDFRS upload function only – EVO Ready – Part drawing File - Multiple Model Year / Product Code with single Build Event Designator – PPV status

FUNC=upload&PRTN=12345678AA&DLS=AA&TYPE=drawing&FRMT=ascii&STAT=ppv&PLTUSE=Y&CPFA=Powertrain &USE=2012.0G.OAN.0GA.|ALL|;2012.Y.OAN.0GA.|ALL|

GDFRS upload function only – EVO Ready – Part drawing File - Multiple Model Year / Product Code with multiple Build Event Designators – PPV status

FUNC=upload&PRTN=12345678AA&DLS=AA&TYPE=drawing&FRMT=ascii&STAT=ppv&PLTUSE=Y&CPFA=Powertrain&USE=2012.0G.OAN.0GA.|ALL|,OAQ.0GA.FAI;2012.Y.OAN.0GA.|ALL|,OAQ.0GA.BMD,OAQ.0GA.FWI

GDFRS update function only – EVO Ready – PPV Status - Multiple Model Year / Product Code with multiple Build Event Designators – PPV status

FUNC=update&PRTN=12345678AA&DLS=AA &STAT=ppv&PLTUSE=Y&CPFA=Platform&USE=2012.0G.OAN.0GA.|ALL|,OAQ.0GA.FAI;2012.Y.OAN.0GA.|ALL|,OAQ.0GA.BMD,OAQ.0GA.FWI

GDFRS upload function only – EVO Ready – Part data file – Production status

FUNC=upload&PRTN=12345678&DLS=AD&TYPE=data&FRMT=binary&STAT=prod&CPFA=Powertrain

GDFRS upload function only – EVO Ready – Part drawing File - Single Model Year / Product Code and single Build Event Designator – Production status

FUNC=upload&PRTN=12345678&DLS=AD &TYPE=drawing&FRMT=ascii&STAT=prod&PLTUSE=Y&CPFA=Powertrain