|  | Component Specification Document Total pages: 38 pages |  |  |  |
| --- | --- | --- | --- | --- |
| Test / production New / Replacement | Title New Mimamori ECU examination demand specification document |  |  | Specification document number 1SKTS60500 |
| Distribution list 17 Total # | A. Applicable models, model years, device components B. Regulation summary and table of contents C. Summary of changes recorded in order. A. Applicable models Vehicles with new mimamori ECU installed -VC30, VF47-6H, VN57, VN60 -VB13-Diesel, VB-13-CNG-LV |  | Related document numbers Configuration. A0; A2; B4. A1A; A3; B5. A1; A4; 38; Total; 38. Control No.. |
|  | B. Regulation summary This specification document describes the new mimamori EOL data flash (input) process using the factory investigation equipment and summarizes the settings and requirements (including QA table) necessary for inspection. C. Summary of changes |  |  |  |
|  | [Stamp: Received Aug 18, 2009 IEC Drawings Dept] | (5). (4). (3). (2). (1). 0; Aug 18, 2009 6Z-0994. Revision (issue) history Isuzu Motors Co. Ltd.; Approved; Reviewed; Created. Owner; Electrical Equipment/Control Development Dept Control Design Group; Hirahara; [illegible]; Takeda. Related. |

Names with ○ receive cover only

Revision History

| Date | Ver | Change details | Notes |
| --- | --- | --- | --- |
| July 14, 2009 | 0 | Newly issued |  |

Table of Contents

1. Purpose											5

2. QA Table											5

2.1 QA Table										5

2.2 Wiring											6

2.3 Pin Layout										6

2.4 EOL Flash Process and Examination Items						7

3. Communication Protocols									8

3.1 EOL Communication Protocols								8

3.2 Specialized Vehicle Equipment/Data Download Communication Protocol			8

4. Related Requirement Specifications Documents						8

5. EOL Input Process/Examination Flow								8

6. EOL Data Flashing Process									9

6.1 Data for Flashing									9

6.2 Flash Process										9

6.2.1 Start Diagnostic Communication							10

6.2.2 Diagnostic Session Start (Programming mode)					10

6.2.3 Security Access (Check security conditions)						11

6.2.4 Flash ECU ID (Flash VIN)								11

6.2.5 Flash EOL data 1 (Flash mimamori data)						12

6.2.6 Flash EOL data 2 (Flash data for preventative maintenance)				14

6.2.7 Diagnostic communication completion						15

7. EOL Data Examination Process								16

7.1 EOL Examination Items									16

7.2 EOL Examination Procedure								16

7.2.1 Start Communication								17

7.2.2 ECU ID Download (VIN download)							17

7.2.3 EOL Data Download 1 (download data for mimamori)				17

7.2.4 EOL Data Download 2 (download data for preventative maintenance)		19

7.2.5 Security Access (ECU lock)								20

7.2.6 Security Access (Check security access)						20

7.2.7 Complete Diagnostic Communication						20

8. Operation Confirmation Examination Processing						21

8.1 Operation Confirmation Preparation							21

8.2 Confirmation Items									21

8.3 Confirmation Procedure									21

8.4 Shared Information Protocol Communication Start Process				22

8.5 DTC Clear/Check (When executed via shared information protocol)			25

8.5.1 DTC Clear										25

8.5.2 DTC Check										26

8.6 DTC Clear/Check (When executed via KWP2000)						27

8.6.1 Diagnostic Communication Start							27

8.6.2 DTC Clear										27

8.6.3 DTC Check										27

8.6.4 Diagnostic Communication Completion						27

8.7 Clear All Mimamori Data (shared information protocol)					28

8.7.1 Clear All Mimamori Data								28

8.7.2 Frequency Data Check								29

8.7.3 DRM/EDR Data Check								30

8.8 Clear All Mimamori Data & Check Process (KWP2000 protocol)				32

8.8.1 Diagnostic Communication Start							32

8.8.2 Clear All Mimamori Data								32

8.8.3 Diagnostic Communication Completion						32

9. EOL Data											33

9.1 EOL Data Definitions for Mimamori Use							33

9.1.1. Installed ECU Data (LID: $20~$3F)							33

9.1.2. Engine Model Data (LID: $40)							34

9.1.3. T/M Model Data (LID:$41)								34

9.1.4. Tire Radius Data (LID:$42)								34

9.1.5. Final Ratio Data (LID:$43)								34

9.1.6. Vehicle Model Data (LID:$44)							34

9.1.7. Electrical Equipment Data (LID:$45)						35

9.1.8. Gear Ratio Data (LID:$47)								35

9.1.9. Vehicle Speed Correction Coefficient (LID:$48)					35

9.1.10 New Mimamori Calibration Data (LID: $50)						35

9.2 Preventative Maintenance EOL Data Definitions						36

9.2.1. Minimum Guaranteed Drive Distance for Engine Oil Life (LID:$52)			36

9.2.2 Drive Distance for Engine Oil Life Warning (LID: $53)				36

9.2.3 Engine Oil Life Prior Warning Drive Distance (LID:$54)				36

9.2.4. Engine Oil Life Warning Total Engine Revolutions (LID: $55)				36

9.2.5. Engine Oil Life Prior Warning Total Revolutions (LID: $56)				36

9.2.6. Engine Oil Life Warning - Insoluble Material (LID: $57)				36

9.2.7. Engine Oil Life Prior Warning - Insoluble Material (LID: $58)				36

9.2.8. Starter Life Warning Number of Starts Data (LID: $59)				36

9.2.9 Exhaust Brake Valve OH Warning Number of Actuations (LID: $5A)			37

9.2.10 ACG OH Warning Drive Distance (LID:$5B)						37

9.2.11 ACG OH Warning Number of Operations (LID:$5C)					37

9.2.12 T/M OH Warning Number of Gear Engagements (LID:$5D)				37

10 DTC Codes											38

1. Purpose

This specification document specifies the EOL flash (input) process and investigation items required for the new mimamori ECU. For the new mimamori ECU, flashing of the EOL data in accordance with the model/spec and VIN number (SPS support) is necessary.

In addition, confirmation that the handling of data between mimamori <--> multi-meter is displayed correctly, the possibility of mistakes remaining in the processes, and deletion of diagnostic and recording data are necessary.

After flashing the EOL data, operate the multi-meter and confirm that the fuel consumption gauge is displaying correctly.

| Electrical Equipment/Control Development Dept Control Design Group 3 |  |  |
| --- | --- | --- |
| Approved | Reviewed | Created |
|  |  | Takeda |

In addition, confirm that normal communication for the special vehicle device/data download is possible, and clear diagnostic data and recording data before shipment from factory.

2. QA Table

2.1 QA Table

The QA table for the new mimamori ECU is shown below

|  |  | Control Unit Mimamori |  | Change date: Creation date: 6 July 2009 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No |  | Device name Function name | Function | Severity *1 | Product management point (operations check points) | Related terminals (Refer to appendix) | Effect on upper level systems if design quality is not met | Self diagnostic | Communicating Dept | Detected Y/N | Feedback from communicating dept (exam results) |
| 1 | ECU | EOL | Select appropriate operation based on data flashed by EOL | A | EOL data flashed in accordance with vehicle. | - | Mimamori data may not be recorded properly. | Partial | MFR QA |  |  |
|  |  | Self-diagnostic | Self-diagnostic | A | Self-diagnostic DTC is not raised. | - | Mimamori data may not be recorded properly. | Yes | MFR QA |  |  |
|  |  | Frequency data recording | Records frequency data in order to understand vehicle usage, etc. | B | Frequency data is cleared before leaving factory. All ECU do not fail after clearing. | - | Vehicle may be shipped out with data accidentally recorded when it was not in a shippable condition, data leaked to market. | No | MFR QA |  |  |
|  |  | DRM recording | Records control data, DTC, etc. for before & after subject ECU failure. (DRM function) | B | Frequency data is cleared before leaving factory. All ECU do not fail after clearing. | - | Shipped with diagnostic records still on board, data leaked to market. | No | MFR QA |  |  |
|  |  | EDR recording | Records before & after control data when subject ECU operates and/or when recording conditions are satisfied. (EDR function) | B | Frequency data is cleared before leaving factory. All ECU do not fail after clearing. | - | Shipped with data accidentally recorded in the factory, data leaked to market. | No | MFR QA |  |  |
|  |  | Communication with multi-meter | Operates fuel consumption gauge. Displays fuel consumption data. | A | Connectors are fully connected. No damage to harness during work process handling. Capable of communicating properly with factory equipment. After flashing EOL, fuel consumption gauge is displayed. | - | Fuel consumption data etc., mimamori <--> multi-meter data handling for multi-display is not displayed. | Partial | MFR QA |  |  |
| 2 | Harness | Power supply | Supplies power (B shift, KEY power, ACC power) and GND to ECU/ | A | Connectors are fully connected. No damage to harness during work process handling. CAN communication related DTC is not raised. | A1, A5, A16, A17 | Mimamori system does not operate, data is not recorded. Fuel consumption cannot be displayed. | No | MFR QA |  |  |
|  |  | CAN communication | Provides CAN communication to other ECU. | A | Connectors are fully connected. No damage to harness during work process handling. Capable of communicating properly with factory equipment. | A6, A23, A4, A21 | Mimamori data may not be properly recorded. Fuel consumption cannot be displayed. | Yes | MFR QA |  |  |
|  |  | KWP2000 communication | Performs KWP2000 communication with tester/factory equipment, sets the EOL data, and checks snapshot data. | A | Connectors are fully connected. No damage to harness during work process handling. Capable of communicating properly with factory equipment. | A3 | EOL cannot be flashed. Service tool (IDSS/TechII) cannot be used. | No | MFR QA |  |  |
|  |  | Data download RS232C communication (1) Shared information protocol | Provides RS232C communication (shared information protocol) for computer/vehicle devices, downloads accumulated data. | A | No damage to harness during work process handling. Capable of communicating properly with factory equipment. | A2, A18, A19, A26 | Cannot download mimamori data. | No | MFR QA |  |  |
|  |  | Communication with online mimamori RS232C communication (1) Shared information protocol | Provides RS232C communication (shared information protocol) for online mimamori, transmits data. | A | Connectors are fully connected. No damage to harness during work process handling. Capable of communicating properly with online mimamori (For confirmation items, follow online mimamori side instructions) *2 | B2, B6, B9 | Cannot communicate with online mimamori | No | MFR QA |  |  |
|  |  | Door SW | Transmits Door SW condition over CAN | B | Connectors are fully connected. No damage to harness during work process handling. | A27 | Cannot transmit Door SW condition properly over CAN. | No | MFR QA |  |  |

*1: Severity

A: Stops functionality

B: Degrades functionality

C: Reduces marketability

*2: Carry out only when online mimamori is set/installed

2.2 Wiring

For the harness indicated with the dotted line, these are improved additional components after vehicle completion, or spare circuits. These do not need to be implemented in the vehicle harness.

Mimamori ECU

To other ECU

K-line

RS232C ch1 Mimamori DL connector

Online mimamori

For TTi testing

(Install single pole connector/diagnostic connector pole)

M/T vehicle only

Diagnostic connector

Vehicle CAN BUS (ISO CAN)

Vehicle CAN BUS (SAEJ1939)

Diagnostic communication function

CAN communication function

Power supply GND

Mimamori (Data collection) function

Idling Stop & Start function

To starter

2.3 Pin Layout

Connector B

| B6 | B6 | B4 | B3 | B2 | B1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | B12 | B11 | B10 | B9 | B8 | B7 |

ECU side connector (JAE MX34012NF1)

Seen from connector fitting face.

| A16 | A15 | A14 | A13 | A12 | A11 | A10 | A9 | A8 | A7 | A6 | A5 | A4 | A3 | A2 | A1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A32 | A31 | A30 | A29 | A28 | A27 | A26 | A25 | A24 | A23 | A22 | A21 | A20 | A19 | A18 | A17 |

Connector A

ECU side connector (JAE MX34032NF2)

NOTE: Connections do not need to be implemented in the vehicle harness

2.4 EOL Flash Process and Examination Items

Details of the EOL flash processing and examination items are shown below.

| Item | Communication standard | Implementation conditions | Purpose/confirmation details | Effect when executed correctly | Reference |
| --- | --- | --- | --- | --- | --- |
| KWP2000 communication start (initialize) | KWP2000 | KEY-ON/Engine start | Check if KWP2000 communication is possible | May not be able to communicate with service tools | 6.2.1 6.2.2 |
| ECU lock condition check | KWP2000 | KEY-ON/Engine start Before VIN flashing | Check that ECU is not locked to flash VIN | Cannot flash VIN if ECU is locked | 6.2.3 |
| Flash VIN | KWP2000 | KEY-ON Before VIN download | Flash VIN, which is SPS key data. | SPS may not operate properly | 6.2.4 |
| Flash EOL data | KWP2000 | KEY-ON Before EOL download | Flash EOL data in vehicle in order to get vehicle data records, for self-diagnostics and maintenance warnings | Cannot record data for vehicle. Possibility of mis-diagnosis. Maintenance warnings for model not carried out | 6.2.5 6.2.6 |
| Download VIN | KWP2000 | KEY-ON/Engine start After VIN flashed | Check if VIN was properly flashed | SPS may not operate properly | 7.2.2 |
| Download EOL data | KWP2000 | KEY-ON/Engine start After EOL flashed | Check if EOL data was properly flashed | Cannot record data for vehicle. Possibility of mis-diagnosis. Maintenance warnings for model not carried out | 7.2.3 7.2.4 |
| ECU lock/check | KWP2000 | KEY-ON/Engine start After VIN flashed | Lock ECU so SPS related items cannot be overwritten. | SPS may not operate properly. Also, self-diagnostic may be triggered. | 7.2.5 7.2.6 |
| Multi-display check | - | KEY-ON/Engine start After EOL flashed | Operate multi-meter, confirm fuel consumption is displayed correctly. | Fuel consumption meter does not operate | - |
| Shared info protocol communication start process (initialize) | RS232 | KEY-ON/Engine start | Confirm that RS232C (shared information protocol) used for downloading mimamori data works properly. | Mimamori data may now be properly downloaded in the market. | 8.4 |
| DTC clear/ download | KWP2000 or RS232 | KEY-ON/Engine start After VIN & EOL data flashed. KEY_OFF, wait 15s or more | Clear DTC that occurred due to manufacturing methods before/after EOL data flashing, confirm no DTC raised. | When DTC is detected in post-shipping service, cannot determine if DTC occurred in-market or at factory. | 8.5 8.6 |
| Clear all mimamori data | KWP2000 or RS232 | KEY-ON/Engine start After VIN & EOL data flashed (before shipping from factory (*)) | Clear data that may have been accidentally recorded at the factory. | Data accidentally recorded during factory inspection may remain and after shipping, cannot determine if data was recorded in-market or at factory. | 8.7 8.8 |

(*) Mimamori all clear process is carried out after this process is executed, and at a time/process step when DTC will not be triggered for the factory inspection of other systems.

3. Communication Protocols

3.1. EOL Communication Protocols

The new mimamori ECU EOL data flashing/inspection uses the KEYWORD Protocol 2000 (hereinafter shortened to “KWP2000”).

For details about KWP2000, please refer to the follow ISO regulations:

- ISO/WD 14230-1 － Road Vehicles - Diagnostic Systems - Keyword Protocol 2000 - Physical Layer MAY 1994

- ISO/WD 14230-2 - Road Vehicles - Diagnostic Systems - Keyword Protocol 2000 - Part 2: Data Link Layer FEB 1997

- ISO/WD 14230-3 - Road Vehicles - Diagnostic Systems - Keyword Protocol 2000 - Part3: Application Layer FEB 1997

- Keyword Protocol 2000 Data Link Layer Recommended Practice - Ver. 1.5 OCT 1 1997

- Keyword Protocol 2000 Implementation of Diagnostic Services Recommended Practice - Ver. 1.5 OCT 1 1997

3.2 Specialized Vehicle Equipment/Data Download Communication Protocol

Investigation of whether communication for specialized vehicle equipment/data download can be performed correctly uses the shared information protocol (Isuzu & TTI’s own protocol).

For details, refer to “Shared information vehicle equipment interface specifications document”.

4. Related Requirement Specifications Documents

- New mimamori EOL input data specifications document

- Shared information vehicle equipment interface specifications document

5. EOL Input Process/Examination Flow

The EOL input process/examination flow is shown below:

Operation check examination process

- DTC clear/download

- Clear all mimamori data

Shared information protocol communication initialize

KEY-ON

KEY-OFF

EOL data examination process

- VIN data download

- EOL data download

- ECU lock check

EOL data flash process

- ECU lock check

- VIN data flash

- EOL data flash

Wait 15s or more, then KEY-ON

KWP2000 communication

initialize

6. EOL Data Flash Process

6.1 Data for flashing

The mimamori ECU flashes the following data:

Data for mimamori (For details refer to section 8.1)

- Installed ECU data

- Engine model data

- T/M type data

- Tire radius data

- Final gear ration data

- Vehicle model data

- Electrical equipment data

- Gear ratio

- Vehicle speed correction coefficient

- New mimamori calibration data

Maintenance data (For details, refer to section 8.2)

- Engine oil deterioration

- Starter life

- Exhaust brake valve OH

- ACG OH

- T/H OH

SPS support

- VIN

6.2. Flash Process

The flash process is shown below.

The tester (factory equipment) sends each of the request messages in the order shown in this chapter, and checks if the correct acknowledgement message is returned from the ECU.

In the following process, all of the requests use a 3-byte header, but the mimamori ECU can also support 4 byte headers. In addition, during communication, a Mode $3E (Tester Alive) message is transmitted at regular intervals. However, if communication is not interrupted for 5 seconds or more, transmission of the Mode $3E (Tester Alive) message is not necessary.

Furthermore, if a response outside of the acknowledgement response (including negative acknowledgement) is returned, ignoring the following exceptions, the instructions from the tester were not correctly sent to the ECU, and therefore the request is resent.

If communication cannot be achieved even with a repeat of the request, some kind of error has occurred, and a separate response should be carried out.

Exception 1: Security access

For the SEED request from the tester, if an acknowledgement response with a SEED value that is not $0000 is returned, that ECU is already locked for some reason, and the WIN cannot be flashed.

In this case, in order to continue with the EOL process, exchange the ECU or release the security using the Tech-II SPS PC/key.

Exception 2: Tester standby request

If the requested process cannot be completed within a specified time, in order to communicate that the request was correctly received and is being processed, the ECU will send a tester standby request response (response code: $78 negative acknowledgement). If this tester standby request response is received, the unit will wait in accordance with IS14230-2 section 4.4.1.

6.2.1 Start Diagnostic Communication

As a continuation of the wake up pattern defined in ISO 14230-2 section 5.1.4.2.3 Fast Initialization, the following request is sent, and communication is started. If the following acknowledgement response is returned, this step is OK.

| Diagnostic communication start | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 81 | 80 | Fx | 81 | CS |  |  |  |  |  |  |
| Response (ECUtester) | 83 | Fx | 80 | C1 | 6B | 8F | CS |  |  |  |  |

Fmt: Format Byte			SID: service Identifier

Tgt: Target Address			LID: Service Identifier

Src: Source Address			Data: Data Bytes

LEN: Length Byte			CS: Check Sum

Fx: Tester Address ($F0~$FD)					                                        All abbreviations same below

6.2.2 Diagnostic Session Start (Programming mode)

The following request is sent, and diagnostic session starts in programming mode.

If the following acknowledgement response is returned, this step is OK.

| Diagnostic communication start | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 10 | 85 | CS |  |  |  |  | $85: programming mode |
| Response (ECUtester) | 82 | Fx | 80 | 50 | 85 | CS |  |  |  |  |  |

6.2.3 Security Access (Check security conditions)

The following request is sent, and the cancellation of the ECU security (SEED=$0000) is checked.

If the following acknowledgement response is returned, this step is OK.

If the security is operational (SEED$0000), use Tech-II SPS PC/key to cancel security or replace ECU.

| Security check | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 27 | 01 | CS |  |  |  |  | Check SEED=$0000 |
| Response (ECUtester) | 84 | Fx | 80 | 67 | 01 | 00 | 00 | CS |  |  |  |

6.2.4 Flash ECU ID (flash VIN)

The following request is sent, VIN (ECU ID) data is flashed as ASCII code.

If the following acknowledgement response is returned, this step is OK.

| VIN flashing | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 93 | 80 | Fx | 3B | 90 | xx | … | xx | CS |  | xx…xx: WIN (ASCII) data |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 90 | CS |  |  |  |  |  |

6.2.5 Flash EOL data 1 (flash mimamori data)

The following request is sent, each mimamori EOL data is flashed.

(For details of the flashed EOL data values, refer to section 8 of this document and the new mimamori EOL input data specification document.)

If the following acknowledgement response is returned, this step is OK.

| Flash mimamori EOL data 1 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 83 | 80 | Fx | 3B | ** | xx | CS |  |  |  | **: Installed ECU data ($20~$3F) xx: not installed/installed (0x00/0x01) |
| Response (ECUtester) | 82 | Fx | 80 | 7B | ** | CS |  |  |  |  |  |

(NOTE) For Installed ECU, the installed ECU data ($20~3F) value is repeatedly changed.

| Flash mimamori EOL data 2 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 8A | 80 | Fx | 3B | 40 | xx | … | xx | CS |  | xx…xx: ENG model data |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 40 | CS |  |  |  |  |  |

| Flash mimamori EOL data 3 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 8A | 80 | Fx | 3B | 41 | xx | … | xx | CS |  | xx…xx: T/M model data |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 41 | CS |  |  |  |  |  |

| Flash mimamori EOL data 4 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 84 | 80 | Fx | 3B | 42 | xx | … | xx | CS |  | Tire radius data |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 42 | CS |  |  |  |  |  |

| Flash mimamori EOL data 5 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 84 | 80 | Fx | 3B | 43 | xx | … | xx | CS |  | xx…xx: Final gear ratio |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 43 | CS |  |  |  |  |  |

| Flash mimamori EOL data 6 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 83 | 80 | Fx | 3B | 44 | xx | CS |  |  |  | xx: Vehicle model data |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 44 | CS |  |  |  |  |  |

| Flash mimamori EOL data 7 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 83 | 80 | Fx | 3B | 45 | xx | CS |  |  |  | xx: Electrical equipment data |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 45 | CS |  |  |  |  |  |

| Flash mimamori EOL data 8 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | A8 | 80 | Fx | 3B | 47 | xx | … | xx | CS |  | xx…xx: Gear ratio data |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 47 | CS |  |  |  |  |  |

| Flash mimamori EOL data 9 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 83 | 80 | Fx | 3B | 48 | xx | CS |  |  |  | xx: Vehicle speed correction coefficient data |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 48 | CS |  |  |  |  |  |

| Flash mimamori EOL data 10 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 8A | 80 | Fx | 3B | 50 | xx | … | xx | CS |  | xx…xx: New mimamori calibration |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 50 | CS |  |  |  |  |  |

6.2.6 Flash EOL data 2 (flashing data for preventative maintenance)

The following request is sent, and each of the preventative maintenance EOL data items is flashed.

(For details of the flashed EOL data values, refer to section 8 of this document and the new mimamori EOL input data specification document.)

If the following acknowledgement response is returned, this step is OK.

| Flash mimamori EOL data 1 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 52 | xx | … | xx | CS |  | xx…xx: Minimum guaranteed drive distance for engine oil life |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 52 | CS |  |  |  |  |  |

| Flash mimamori EOL data 2 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 53 | xx | … | xx | CS |  | xx…xx: Drive distance for engine oil life warning |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 53 | CS |  |  |  |  |  |

| Flash mimamori EOL data 3 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 54 | xx | … | xx | CS |  | xx…xx: Engine oil life prior warning drive distance |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 54 | CS |  |  |  |  |  |

| Flash mimamori EOL data 4 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 55 | xx | … | xx | CS |  | xx…xx: Engine oil life warning total engine rotations |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 55 | CS |  |  |  |  |  |

| Flash mimamori EOL data 5 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 56 | xx | … | xx | CS |  | xx…xx: Engine oil life prior warning total rotations |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 56 | CS |  |  |  |  |  |

| Flash mimamori EOL data 6 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 57 | xx | … | xx | CS |  | xx…xx: Engine oil life warning insoluble material |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 57 | CS |  |  |  |  |  |

| Flash mimamori EOL data 7 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 58 | xx | … | xx | CS |  | xx…xx: Engine oil life prior warning insoluble material |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 58 | CS |  |  |  |  |  |

| Flash mimamori EOL data 8 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 59 | xx | … | xx | CS |  | xx…xx: Starter life warning number of starts data |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 59 | CS |  |  |  |  |  |

| Flash mimamori EOL data 9 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 5A | xx | … | xx | CS |  | xx…xx: Exhaust brake valve OH warning number of actuations |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 5A | CS |  |  |  |  |  |

| Flash mimamori EOL data 10 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 5B | xx | … | xx | CS |  | xx…xx: ACG OH warning drive distance |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 5B | CS |  |  |  |  |  |

| Flash mimamori EOL data 11 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 5C | xx | … | xx | CS |  | xx…xx: ACG OH warning number of operations |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 5C | CS |  |  |  |  |  |

| Flash mimamori EOL data 12 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 86 | 80 | Fx | 3B | 5D | xx | … | xx | CS |  | xx…xx: T/M OH warning number of gear engagements |
| Response (ECUtester) | 82 | Fx | 80 | 7B | 5D | CS |  |  |  |  |  |

6.2.7 Complete diagnostic communication

The following request is sent, and communication is completed.

If the following acknowledgement response is returned, this step is OK.

| Complete communication | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 81 | 80 | Fx | 82 |  |  |  |  |  |  |  |
| Response (ECUtester) | 81 | Fx | 80 | C2 |  |  |  |  |  |  |  |

7. EOL Data Examination Process

7.1. EOL Examination Items

In the examination, the following items are checked:

- The correct ECU ID/EOL data was flashed

- The ECU is locked

7.2 EOL Examination Procedure

The examination procedure is shown below.

The tester (factory equipment) sends each of the request messages in the order shown in this chapter, and checks if the data is correct in the acknowledgement response message returned from the ECU.

In the following process, all of the requests use a 3-byte header, but the mimamori ECU can also support 4 byte headers. In addition, during communication, a Mode $3E (Tester Alive) message is transmitted at regular interval. However, if communication is not interrupted for 5 seconds or more, transmission of the Mode $3E (Tester Alive) message is not necessary.

Furthermore, if a response outside of the acknowledgement response (including negative acknowledgement) is returned, ignoring the following exception, the instructions from the tester were not correctly sent to the ECU, and therefore the request is resent.

If communication cannot be achieved even with a repeat of the request, some kind of error has occurred, and a separate response should be carried out.

Exception: Tester standby request

Similar to the previous section, the tester standby request response (response code: $78 negative acknowledgement) indicates that the request was received correctly by the ECU, but processing could not be completed within the specified time.

If this tester standby request response is received, the unit will wait in accordance with IS14230-2 section 4.4.1.

7.2.1 Start Communication

As a continuation of the wake up pattern defined in ISO 14230-2 section 5.1.4.2.3 Fast Initialization, the following request is sent, and communication is started. If the following acknowledgement response is returned, this step is OK.

| Diagnostic communication start | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 81 | 80 | Fx | 81 | CS |  |  |  |  |  |  |
| Response (ECUtester) | 83 | Fx | 80 | C1 | 6B | 8F | CS |  |  |  |  |

7.2.2 ECU ID Download (VIN download)

The following request is sent, and the VIN (ECU ID) is downloaded.

If the downloaded VIN and the flashed VIN have the same value, this step is OK.

| VIN download | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 1A | 90 | CS |  |  |  |  | Check VIN |
| Response (ECUtester) | 93 | Fx | 80 | 5A | 90 | xx | … | xx | CS |  |  |

7.2.3 EOL Data Download 1 (download data for mimamori)

The following requests are sent, and each of the mimamori EOL data is downloaded.

If the downloaded data is the same as the flashed data, this step is OK.

| Mimamori EOL data download 1 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | ** | CS |  |  |  |  | **: Installed ECU data ($20~$3F) |
| Response (ECUtester) | 83 | Fx | 80 | 61 | ** | xx | CS |  |  |  | **: not installed/installed (0x00/0x01) |

| Mimamori EOL data download 2 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 40 | CS |  |  |  |  | Installed engine model |
| Response (ECUtester) | 8A | Fx | 80 | 61 | 40 | xx | .. | xx | CS |  |  |

| Mimamori EOL data download 3 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 41 | CS |  |  |  |  | Installed T/M |
| Response (ECUtester) | 8A | Fx | 80 | 61 | 41 | xx | ... | xx | CS |  |  |

| Mimamori EOL data download 4 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 42 | CS |  |  |  |  | Tire radius |
| Response (ECUtester) | 8A | Fx | 80 | 61 | 42 | xx | ... | xx | CS |  |  |

| Mimamori EOL data download 5 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 43 | CS |  |  |  |  | Final ratio |
| Response (ECUtester) | 8A | Fx | 80 | 61 | 43 | xx | ... | xx | CS |  |  |

| Mimamori EOL data download 6 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 44 | CS |  |  |  |  | Vehicle model data |
| Response (ECUtester) | 83 | Fx | 80 | 61 | 44 | xx | CS |  |  |  |  |

| Mimamori EOL data download 7 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 45 | CS |  |  |  |  | Electrical equipment data |
| Response (ECUtester) | 83 | Fx | 80 | 61 | 45 | xx | CS |  |  |  |  |

| Mimamori EOL data download 8 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 47 | CS |  |  |  |  | Gear ratio |
| Response (ECUtester) | A8 | Fx | 80 | 61 | 47 | xx | … | xx | CS |  |  |

| Mimamori EOL data download 9 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 48 | CS |  |  |  |  | Vehicle speed correction coefficient |
| Response (ECUtester) | 83 | Fx | 80 | 61 | 48 | xx | CS |  |  |  |  |

| Mimamori EOL data download 10 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 50 | CS |  |  |  |  | New mimamori calibration |
| Response (ECUtester) | 8A | Fx | 80 | 61 | 50 | xx | … | xx | CS |  |  |

7.2.4. EOL Data Download 2 (download data for preventative maintenance)

The following request is sent, and each of the EOL data items is downloaded.

If the downloaded data is the same as the flashed data, this step is OK.

| Download EOL preventative data 1 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 52 | CS |  |  |  |  | Minimum guaranteed drive distance for engine oil life |
| Response (ECUtester) | 86 | Fx | 80 | 61 | 52 | xx | … | xx | CS |  |  |

| Download EOL preventative data 2 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 53 | CS |  |  |  |  | Drive distance for engine oil life warning |
| Response (ECUtester) | 86 | Fx | 80 | 61 | 53 | xx | … | xx | CS |  |  |

| Download EOL preventative data 3 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 54 | CS |  |  |  |  | Engine oil life prior warning drive distance |
| Response (ECUtester) | 86 | Fx | 80 | 61 | 54 | xx | … | xx | CS |  |  |

| Download EOL preventative data 4 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 21 | 55 | CS |  |  |  |  | Engine oil life warning total engine revolutions |
| Response (ECUtester) | 86 | Fx | 80 | 61 | 55 | xx | … | xx | CS |  |  |

| Download EOL preventative data 5 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 3B | 56 | CS |  |  |  |  | Engine oil life prior warning total rotations |
| Response (ECUtester) | 86 | Fx | 80 | 7B | 56 | xx | … | xx | CS |  |  |

| Download EOL preventative data 6 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 3B | 57 | CS |  |  |  |  | Engine oil life warning insoluble material |
| Response (ECUtester) | 86 | Fx | 80 | 7B | 57 | xx | … | xx | CS |  |  |

| Download EOL preventative data 7 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 3B | 58 | CS |  |  |  |  | Engine oil life prior warning insoluble material |
| Response (ECUtester) | 86 | Fx | 80 | 7B | 58 | xx | … | xx | CS |  |  |

| Download EOL preventative data 8 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 3B | 59 | CS |  |  |  |  | Starter life warning number of starts |
| Response (ECUtester) | 86 | Fx | 80 | 7B | 59 | xx | … | xx | CS |  |  |

| Download EOL preventative data 9 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 3B | 5A | CS |  |  |  |  | Exhaust brake valve OH warning number of actuations |
| Response (ECUtester) | 86 | Fx | 80 | 7B | 5A | xx | … | xx | CS |  |  |

| Download EOL preventative data 10 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 3B | 5B | CS |  |  |  |  | ACG OH warning drive distance |
| Response (ECUtester) | 86 | Fx | 80 | 7B | 5B | xx | … | xx | CS |  |  |

| Download EOL preventative data 11 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 3B | 5C | CS |  |  |  |  | ACG OH warning number of operations |
| Response (ECUtester) | 86 | Fx | 80 | 7B | 5C | xx | … | xx | CS |  |  |

| Download EOL preventative data 12 | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 3B | 5D | CS |  |  |  |  | T/M OH warning number of gear engagements |
| Response (ECUtester) | 86 | Fx | 80 | 7B | 5D | xx | … | xx | CS |  |  |

7.2.5 Security Access (ECU lock)

The following request is sent, ECU security (flash control) is activated.

If the following acknowledgement response is returned, this step is OK.

This process is carried out after the correct flashing of the VIN data has been confirmed.

| ECU lock | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 31 | 08 | CS |  |  |  |  | LID $08: Activate security instruction |
| Response (ECUtester) | 82 | Fx | 80 | 71 | 08 | CS |  |  |  |  |  |

7.2.6 Security Access (Check security access)

The following request is sent, and whether the ECU is locked is checked.

If the following acknowledgement response is returned, and SEED in the data is $0000, this step is OK.

| Security check | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 27 | 01 | CS |  |  |  |  | Confirm that SEED(xxxx)0000 |
| Response (ECUtester) | 82 | Fx | 80 | 67 | 01 | xx | xx | CS |  |  |  |

7.2.7 Complete Diagnostic Communication

The following request is sent, and communication is completed.

If the following acknowledgement response is returned, this step is OK.

| Complete communication | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 81 | 80 | Fx | 82 | CS |  |  |  |  |  |  |
| Response (ECUtester) | 81 | Fx | 80 | C2 | CS |  |  |  |  |  | 4-byte header |

8. Operation Confirmation Examination Processing

8.1 Operation Confirmation Preparation

The operation confirmation process is not performed immediately after the EOL data flashing/examination is completed.

After the EOL examination has been normally completed, temporarily turn the KEY OFF, and after waiting for 15 seconds or more, turn the KEY ON again, and then carry out the operation confirmation process.

In this examination, the shared information protocol is initialized, the ability to communicate with the new mimamori ECU using the shared information protocol is confirmed, then the DTC clear/check, and the clear all mimamori data processes are carried out.

Note that the DTC clear/check, and the clear all mimamori data processes can be carried out either through RS232C (shared information protocol) or KWP2000 communication.

8.2. Confirmation Items

(1) Start shared information protocol communication

Carry out the start shared information protocol communication (initialize) process, and confirm that communication is possible between the new mimamori ECU using the shared information protocol.

(2) DTC clear/check

When executed via RS232C (shared information protocol)

After executing the DTC clear process, confirm that the DTC is not set.

Confirm that all downloaded statuses are 0x80: no failure.

When executed via KWP2000 communication

Execute the DTC clear process, confirm that positive response is received from ECU.

After executing the DTC clear process, confirm that DTC is not set.

(3) Clear all mimamori data

When executed via RS232C (shared information protocol)

After executing the clear all mimamori data process, wait 4 or more seconds, then confirm that the mimamori frequency data recording region and DRM/EDR recording region have been cleared (downloaded data is all 0).

(Because clearing all of the mimamori data regions requires significant time, wait 4 or more seconds to carry out the confirmation)

When executed via KWP2000 communication

Execute the clear all mimamori data process, confirm that positive response is received from ECU.

8.3 Confirmation Procedure

The operation confirmation examination procedure is shown below.

The tester (factory equipment) sends each of the request messages in the order shown in this chapter, and checks if the correct data values are in the response message returned from the ECU.

8.4 Shared Information Protocol Communication Start Process

The shared information protocol communication procedure is shown below:

New mimamori ECU			Examination device

| Transmit at approx. 200ms frequency until response received | Connected device information notification |  |
| --- | --- | --- |
|  | . . . |  |
|  | Connected device information notification | Response within 200ms |
|  | Connected device information response |  |
| Communication start process (initialize) | Current time notification |  |
|  | Current time response |  |
|  | Vehicle attribute notification |  |
|  | Vehicle attribute response |  |
|  | Vehicle number notification |  |
|  | Vehicle number response |  |
| After communication start process, transmit trigger confirmation notifications at 1000ms frequency. NB: if trigger response requires time, instead of 1000ms send trigger notification after receiving response from examination device | Trigger confirmation notification |  |
|  | Trigger confirmation response |  |
|  | Trigger response |  |
|  | Response for trigger response |  |
|  | Shared information data |  |
|  | Trigger confirmation notification |  |
|  | Trigger confirmation response |  |

(NOTE) For notifications/responses from the new mimamori ECU, because communication will stop if there is no response for more than 200ms, the examination device will return a response to a new mimamori ECU notification/response within 200ms.

(After communication has been completed once, new communication is carried out from another execution of the communication start process)

In addition, after the communication start process has succeeded, the shared information data shown below is sent regularly from the mimamori ECU, but because the data (notifications) are unrelated to the examination, they are ignored. (No response is OK.)

Shared information data

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  |  | Serial number |  | Message length |  |  | No. of data frame bytes |  |  | Shared information B (18byte) |  |  |  | Shared information B (18byte) |  |  |  | ETX |  | CRC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 00 | 32 | 00 | 00 | 00 | 00 |  | 31 | 04 |  | xx |  | 00 | 26 |  | 00 | 24 |  | xx | … | xx |  | xx | … | xx |  | 03 |  | xx | xx |  |

Connected device information notification (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  |  | Device number |  |  |  | Message ID |  |  | Serial number |  | Message length |  | Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ETX |  | CRC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Related data Y/N |  | Connected device info |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Mfr type ID |  |  |  | Software ver |  |  |  |  | Confirm |  | Initial |  |  |  |  |  |  |
| 02 | 00 | 17 | 80 | 16 | 00 |  | 00 | 00 | 00 | 31 | 00 | xx |  | 00 |  | 0A |  | FE |  | F0 | 01 | xx |  | xx | xx | xx | xx |  | xx |  | xx |  | 03 |  | xx | xx |  |

xx: arbitrary data (same below)

Software version/confirmation/initial are arbitrary data. They are not related to the examination, and can be ignored.

Connected device information response

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Data |  |  |  |  |  |  |  | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Status | Mfr type identifier | Year/month/day/hour/minute/second |  |  |  |  |  |  |  |  |
| 02 | 00 | 71 | 00 | 14 | 00 | 00 | 00 | 00 | 31 | 00 | xx | 00 | 08 | 00 | 08 | 00 | 00 | 00 | 00 | 00 | 00 | 03 | xx | xx |

Current time notification (mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Year/month/day/hour/minute/second |  |  |  |  |  | ETX |  | CRC |  |  |
| 02 | 00 | 17 | 80 | 13 | 00 | 00 | 00 | 00 | 31 | 0F | xx | 00 | 07 | xx | xx | xx | xx | xx | xx | 03 |  | xx | xx |  |

The data (year/month/day/hour/minute/second) is arbitrary data. Not related to the examination, and can be ignored.

Current time response (Examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 0D | 00 | 00 | 00 | 00 | 31 | 0F | xx | 00 | 01 | 00 | 03 | xx | xx |

Vehicle attribute notification (mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Engine model (8byte) |  |  | T/M model (8byte) |  |  | Tire radius |  | Final gear ratio |  | Dealer area (80byte) |  |  | VIN (20byte) |  |  |  |  |  |
| 02 | 00 | 17 | 80 | 84 | 00 | 00 | 00 | 00 | 31 | 0C | xx | 00 | 78 | xx | … | xx | xx | … | xx | xx | xx | xx | xx | xx | … | xx | xx | … | xx | 03 | xx | xx |

The data region is sending the EOL data inside the mimamori ECU, so it is arbitrary.

Not related to examination, therefore can be ignored.

Vehicle attribute response (examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Received byte count |  | ETX | CRC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 0F | 00 | 00 | 00 | 00 | 31 | 0C | xx | 00 | 03 | 00 | 00 | 78 | 03 |  | xx | xx |  |

Vehicle number notification (mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  |  | Serial number |  | Message length |  |  | No. of data frame bytes |  |  | Shared information A (18byte) |  |  |  | Shared information A (18byte) |  |  |  | ETX |  | CRC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | 32 | 00 | 00 | 00 | 00 |  | 31 | 04 |  | xx |  | 00 | 26 |  | 00 | 24 |  | xx | … | xx |  | xx | … | xx |  | 03 |  | xx | xx |  |

Shared information A is arbitrary data. Not related to examination, therefore can be ignored.

Vehicle number response (examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Received byte count |  | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 0F | 00 | 00 | 00 | 00 | 31 | 04 | xx | 00 | 03 | 00 | 00 | 24 | 03 | xx | xx |  |

8.5. DTC Clear/Check (When executed via shared information protocol)

8.5.1 DTC Clear

In this examination, DTC which were triggered in the factory when EOL was not yet set, etc. are cleared.

The process is shown below

Trigger confirmation notification (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Device identifier | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | 0D | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 01 | 08 | 03 | xx | xx |  |

Trigger confirmation response (DTC clear request) (Examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Message count | Trigger code | Message # | Command | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 11 | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 05 | 00 | 01 | 10 | 01 | 0E | 03 | xx | xx |

8.5.2 DTC Check

In this examination, whether DTC is raised/triggered is checked.

The process is shown below:

Trigger confirmation notification (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Device identifier | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | 0D | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 01 | 08 | 03 | xx | xx |  |

Trigger confirmation response (download address specified data) (examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Message count | Trigger code | Message # | Address |  |  |  | Data count | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 15 | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 09 | 00 | 01 | 03 | 01 | xx | xx | xx | xx | xx | 03 | xx | xx |

For downloaded DTC address and data count, refer to new mimamori EOL input data specification document.

Trigger response (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Trigger code | Message # | Data |  |  | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | xx | 00 | 00 | 00 | 00 | 31 | 0A | xx | 00 | xx | 03 | 01 | xx | … | xx | 03 | xx | xx |

Check is OK if all of the received data (DTC status) is 0x80: no failure.

If even 1 data unit is 0xE0: current failure, a separate failure process is carried out.

For the format of the sent data, refer to the new mimamori EOL input data specification document.

Response for trigger response (examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Received byte count | ETX | CRC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 0E | 00 | 00 | 00 | 00 | 31 | 0A | xx | 00 | 02 | 00 | xx | 03 |  | xx | xx |  |

8.6. DTC Clear/Check (When executed via KWP2000)

8.6.1 Diagnostic Communication Start

As a continuation of the wake up pattern defined in ISO 14230-2 section 5.1.4.2.3 Fast Initialization, the following request is sent, and communication is started. If the following acknowledgement response is returned, this step is OK.

| Diagnostic communication start | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 81 | 80 | Fx | 81 | CS |  |  |  |  |  |  |
| Response (ECUtester) | 83 | Fx | 80 | C1 | 6B | 8F | CS |  |  |  |  |

8.6.2 DTC Clear

In this examination, DTC which were triggered in the factory when EOL was not yet set, etc. are cleared.

The following request is sent, any mimamori frequency data, DRM/EDR data recorded during work processed are erased.

If the following acknowledgement response is returned, this step is OK.

| Clear all data | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 31 | F0 | CS |  |  |  |  | LID $F0: clear all data instruction |
| Response (ECUtester) | 82 | Fx | 80 | 71 | F0 | CS |  |  |  |  |  |

8.6.3 DTC Check

In this examination, whether DTC is raised/triggered is checked. The following request is sent, and no DTC settings in the ECU are checked. If the following acknowledgement response is returned, this step is OK.

| DTC download | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 84 | 80 | Fx | 18 | 00 | FF | 00 | CS |  |  | Confirm number of DTC is 0 |
| Response (ECUtester) | 82 | Fx | 80 | 58 | 00 | CS |  |  |  |  |  |

Note that, if a DTC is set, the following response will be returned, and a separate error/re-examination process is carried out. The number of set DTC is sent in the ND~DS data.

| Failure code download | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Response (ECUtester) | 80 | Fx | 90 | LEN | 58 | ND | DH | DL | DS | … | CS | There are set DTC |  |

LEN: Length Byte		                             ND: Number of DTC

DH: DTC High byte	                                             DL: DTC Lo byte

DS: DTC Status (DS=80: no failure, A0: past failure, E0: current failure)

8.6.4 Diagnostic Communication Completion

The following request is sent, and communication is completed.

If the following acknowledgement response is returned, this step is OK.

| Communication completion | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 81 | 80 | Fx | 82 | CS |  |  |  |  |  |  |
| Response (ECUtester) | 81 | Fx | 80 | C2 | CS |  |  |  |  |  | 4 byte header |

8.7 Clear All Mimamori Data (when executed via shared information protocol)

8.7.1 Clear All Mimamori Data

The following request is sent, any mimamori frequency data, DRM/EDR data recorded during work processed are erased.

Trigger confirmation notification (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Device identifier | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | 0D | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 01 | 08 | 03 | xx | xx |  |

Trigger confirmation response (Clear all mimamori data request) (Examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Message count | Trigger code | Message # | Command | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 11 | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 05 | 00 | 01 | 10 | 01 | 0F | 03 | xx | xx |

In the shared information protocol, for the clear all mimamori request from the examination device side, because a positive response is not returned to indicate that it had been correctly executed, use the previous processes shown in 7.7.2 & 7.7.3 to execute the clear.

8.7.2. Frequency Data Check

The process to check whether the frequency data has been cleared or not is shown below.

Because clearing all the regions requires 4 seconds or more, this check is carried out after more than 4 seconds has elapsed after the clear process was executed.

Trigger confirmation notification (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Device identifier | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | 0D | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 01 | 08 | 03 | xx | xx |  |

Trigger confirmation response (download address specified data) (examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Message count | Trigger code | Message # | Address |  |  |  | Data count |  | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 15 | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 09 | 00 | 01 | 03 | 01 | xx | xx | xx | xx |  | xx | 03 | xx | xx |  |

For the address and number of data for the downloaded frequency data, refer to the new mimamori EOL input data specification document.

Trigger response (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Trigger code | Message # | Data |  |  | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | xx | 00 | 00 | 00 | 00 | 31 | 0A | xx | 00 | xx | 03 | 01 | xx | … | xx | 03 | xx | xx |

If all of the regions for the data received from mimamori ECU are 0, this check is OK.

If it is not 0, a separate failure process is carried out.

Response for trigger response (examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Received byte count | ETX | CRC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 0E | 00 | 00 | 00 | 00 | 31 | 0A | xx | 00 | 02 | 00 | xx | 03 |  | xx | xx |  |

8.7.3 DRM/EDR Data Check

The process for checking whether or not DRM/EDR data is cleared is shown below.

Because clearing all the regions requires 4 seconds or more, this check is carried out after more than 4 seconds has elapsed after the clear process was executed.

[When 1 address is specified in a single message and downloaded]

If 1 address is specified in a single message and downloaded, the following process is repeated for only the number of target systems in each project

Trigger confirmation notification (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Device identifier | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | 0D | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 01 | 08 | 03 | xx | xx |  |

Trigger confirmation response (download address specified data) (examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Message count | Trigger code | Message # | Address |  |  |  | Data count |  | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 15 | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 09 | 00 | 01 | 03 | 01 | xx | xx | xx | xx |  | xx | 03 | xx | xx |  |

For the address and number of data units for the downloaded DRM/EDR data, refer to the new mimamori EOL input data specification document. Set single time download data to 128 bytes or less. If data exceeds 128 bytes, divide into multiple parts and multiple requests.

Trigger response (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Trigger code | Message # | Data |  |  | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | xx | 00 | 00 | 00 | 00 | 31 | 0A | xx | 00 | xx | 03 | 01 | xx | … | xx | 03 | xx | xx |

If all of the regions for the data received from mimamori ECU are 0, this check is OK.

If it is not 0, a separate failure process is carried out.

Response for trigger response (examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Received byte count | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 0E | 00 | 00 | 00 | 00 | 31 | 0A | xx | 00 | 02 | 00 | xx | 03 | xx | xx |

[When multiple addresses are downloaded in a single message]

Trigger confirmation notification (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Device identifier | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | 0D | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 01 | 08 | 03 | xx | xx |  |

Trigger confirmation response (download address specified data) (examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Message count | Trigger code | Message # | Address |  |  |  | Data count |  | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 15 | 00 | 00 | 00 | 00 | 31 | 0B | xx | 00 | 09 | 00 | 01 | 03 | 01 | xx | xx | xx | xx |  | xx | 03 | xx | xx |  |

For the address and number of data units for the downloaded DRM/EDR data, refer to the new mimamori EOL input data specification document.

Set single time download data to 128 bytes or less. If data exceeds 128 bytes, divide into multiple parts and multiple requests.

Trigger response (Mimamori  examination device)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Trigger code | Message # | Data |  |  | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 17 | 80 | xx | 00 | 00 | 00 | 00 | 31 | 0A | xx | 00 | xx | 03 | 01 | xx | … | xx | 03 | xx | xx |

If all of the regions for the data received from mimamori ECU are 0, this check is OK.

If it is not 0, a separate failure process is carried out.

Response for trigger response (examination device  mimamori)

| STX | Compression identifier | Request origin/target | Total data length |  | Device number |  |  |  | Message ID |  | Serial number | Message length |  | Status | Received byte count | ETX | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 00 | 71 | 00 | 0E | 00 | 00 | 00 | 00 | 31 | 0A | xx | 00 | 02 | 00 | xx | 03 | xx | xx |

8.8 Clear All Mimamori Data & Check Process (KWP2000 protocol)

8.8.1 Diagnostic Communication Start

As a continuation of the wake up pattern defined in ISO 14230-2 section 5.1.4.2.3 Fast Initialization, the following request is sent, and communication is started. If the following acknowledgement response is returned, this step is OK.

| Diagnostic communication start | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 81 | 80 | Fx | 81 | CS |  |  |  |  |  |  |
| Response (ECUtester) | 83 | Fx | 80 | C1 | 6B | 8F | CS |  |  |  |  |

8.8.2 Clear All Mimamori Data

The following request is sent, any mimamori frequency data, DRM/EDR data recorded during work processed are erased.

If the following acknowledgement response is returned, this step is OK.

(If the clearing process is executed via KWP2000, and because a positive response is returned to indicate the process was executed correctly from the new mimamori ECU side, and there is no need to check for recording data download and clearing, similar to the shared information protocol)

| Clear all data | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 82 | 80 | Fx | 31 | F0 | CS |  |  |  |  | LID $F0: clear all data instruction |
| Response (ECUtester) | 82 | Fx | 80 | 71 | F0 | CS |  |  |  |  |  |

8.8.3 Diagnostic Communication Completion

The following request is sent, and communication is completed.

If the following acknowledgement response is returned, this step is OK.

| Communication completion | Fmt | Tgt | Src | LEN/SID/LID/Data/CS |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request (tester ECU) | 81 | 80 | Fx | 82 | CS |  |  |  |  |  |  |
| Response (ECUtester) | 81 | Fx | 80 | C2 | CS |  |  |  |  |  | 4-byte header |

9. EOL Data

EOL data definitions for use in mimamori ECU are shown below.

9.1 EOL Data Definitions for Mimamori Use

9.1.1. Installed ECU Data (LID: $20~$3F)

If the system is installed, value is set to [0x01: Active], if it is not installed, set to [0x00: Inactive]

| ID | System name |  | ID | System name |
| --- | --- | --- | --- | --- |
| $20 | ECM (Diesel) |  | $30 | Hybrid System |
| $21 | ECM (CNG) |  | $31 | Fluid retarder |
| $22 | SCR |  | $32 | TPMS |
| $23 | Turbo Controller |  | $33 | ACT |
| $24 | Smoother G |  | $34 | ECON ECU |
| $25 | NEES1 |  | $35 | Reserve |
| $26 | NEES2 |  | $36 | Reserve |
| $27 | AT |  | $37 | Reserve |
| $28 | ESC |  | $38 | Reserve |
| $29 | ABS/ASR/EBS |  | $39 | Reserve |
| $2A | Inter-vehicle distance warning ACC/CMS |  | $3A | Reserve |
| $2B | Airsus |  | $3B | Reserve |
| $2C | Idling Stop & Start |  | $3C | Reserve |
| $2D | Driving concentration warning |  | $3D | Reserve |
| $2E | Multi-meter |  | $3E | Reserve |
| $2F | Lane departure warning |  | $3F | Reserve |

9.1.2. Engine Model Data (LID: $40)

The installed engine is specified using 8 byte ASCII data.

| ID | Byte | Contents |
| --- | --- | --- |
| 40 | 1 | Engine model name 1st character (ASCII code / blank filled with 0x20) |
|  | 2 | Engine model name 2nd character (ASCII code / blank filled with 0x20) |
|  | 3 | Engine model name 3rd character (ASCII code / blank filled with 0x20) |
|  | 4 | Engine model name 4th character (ASCII code / blank filled with 0x20) |
|  | 5 | Engine model name 5th character (ASCII code / blank filled with 0x20) |
|  | 6 | Engine model name 6th character (ASCII code / blank filled with 0x20) |
|  | 7 | Engine model name 7th character (ASCII code / blank filled with 0x20) |
|  | 8 | Engine model name 8th character (ASCII code / blank filled with 0x20) |

9.1.3. T/M Model Data (LID:$41)

The installed engine is specified using 8 byte ASCII data.

| ID | Byte | Contents |
| --- | --- | --- |
| 41 | 1 | T/M model name 1st character (ASCII code / blank filled with 0x20) |
|  | 2 | T/M model name 2nd character (ASCII code / blank filled with 0x20) |
|  | 3 | T/M model name 3rd character (ASCII code / blank filled with 0x20) |
|  | 4 | T/M model name 4th character (ASCII code / blank filled with 0x20) |
|  | 5 | T/M model name 5th character (ASCII code / blank filled with 0x20) |
|  | 6 | T/M model name 6th character (ASCII code / blank filled with 0x20) |
|  | 7 | T/M model name 7th character (ASCII code / blank filled with 0x20) |
|  | 8 | T/M model name 8th character (ASCII code / blank filled with 0x20) |

9.1.4. Tire Radius Data (LID: $42)

The dynamic load radius of the rear wheel is specified as 2 byte UNM (unsigned numeric) data (1bit=1mm).

Example: 380[mm]  0x017C

9.1.5. Final Ratio Data (LID: $43)

The final drive ratio is specified as 2 byte UNM (unsigned numeric) data (1bit=0.001)

Example: 4.300  4300=0x10CC

9.1.6. Vehicle Model Data (LID: $44)

The vehicle model identifier is specified as 1-byte bitmap data.

| ID | Bit | Contents | Meaning of 1 | Meaning of 0 |
| --- | --- | --- | --- | --- |
| 44 | 8 | Large model (C&E) | Applicable | Not applicable |
|  | 7 | Mid-size (F) | Applicable | Not applicable |
|  | 6 | Small model (N) | Applicable | Not applicable |
|  | 5 | Bus | Applicable | Not applicable |
|  | 4 | SKW | Applicable | Not applicable |
|  | 3 | US | Applicable | Not applicable |
|  | 2 | - | - | - |
|  | 1 | EXP | Applicable | Not applicable |

9.1.7. Electrical Equipment Data (LID: $45)

The electrical equipment identifier is specified as 1-byte bitmap data.

| ID | Bit | Contents | Meaning of 1 | Meaning of 0 |
| --- | --- | --- | --- | --- |
| 45 | 8 | Air conditioner control | Enable | Disable |
|  | 7 | - | - | - |
|  | 6 | - | - | - |
|  | 5 | - | - | - |
|  | 4 | - | - | - |
|  | 3 | - | - | - |
|  | 2 | CAN communication (J1939) | O1(B): 250k 10(B): 500k 00(B), 11(B): CAN communication/diagnostic stop |  |
|  | 1 |  |  |  |

9.1.8. Gear Ratio Data (LID: $47)

The number of main gear changer steps is specified as 1 byte UNM (unsigned numeric value) data (1bit = 1 step), and the transmission gear ratio is specified as 2 byte UNM (unsigned numeric value) (1bit=0.001), for a total 38 bytes.

| ID | Byte | Details |
| --- | --- | --- |
| 47 | 1 | Input number of main gear changer steps (1bit/step) |
|  | 2 | Reserve (0x00 fixed) |
|  | 3,4 | Gear 1: Multiply by 1000 and input as hexadecimal (For unused gear regions, enter 0x00) |
|  | 5,6 | Gear 2: Multiply by 1000 and input as hexadecimal (For unused gear regions, enter 0x00) |
|  | . . . | . . . |
|  | 35, 36 | Gear 17: Multiply by 1000 and input as hexadecimal (For unused gear regions, enter 0x00) |
|  | 37,38 | Gear 18: Multiply by 1000 and input as hexadecimal (For unused gear regions, enter 0x00) |

9.1.9. Vehicle Speed Correction Coefficient (LID: $48)

Identify the vehicle speed correction coefficient as 1 byte UNM (unsigned numeric value) data (1bit=1%)

Example: 100[%]  0x64

9.1.10 New Mimamori Calibration Data (LID: $50)

Specify the new mimamori calibration as 8-byte bitmap data.

| ID | byte | bit | Details | Meaning of 1 | Meaning of 0 |
| --- | --- | --- | --- | --- | --- |
| 50 | 1 | 8 | EOL transmission | Enable | Disable |
|  |  | 7 | - | - | - |
|  |  | 6 | - | - | - |
|  |  | 5 | - | - | - |
|  |  | 4 | - | - | - |
|  |  | 3 | - | - | - |
|  |  | 2 | - | - | - |
|  |  | 1 | - | - | - |
|  | 2-8 | - | Reserve | - | - |

9.2 Preventative Maintenance EOL Data Definitions

9.2.1. Minimum Guaranteed Drive Distance for Engine Oil Life (LID: $52)

Specify the minimum guaranteed drive distance for engine oil life as 4 byte UNM (unsigned numeric value) data (1bit=1m).

Example: 5,000[km]  5,000,000[m]  0x004C4B40

9.2.2 Drive Distance for Engine Oil Life Warning (LID: $53)

Specify the drive distance for engine oil life as 4 byte UNM (unsigned numeric value) data (1bit=1m).

Example: 5,000[km]  5,000,000[m]  0x004C4B40

9.2.3 Engine Oil Life Prior Warning Drive Distance (LID: $54)

Specify the engine oil life prior warning drive distance as 4 byte UNM (unsigned numeric value) data (1bit=1m).

Example: 5,000[km]  5,000,000[m]  0x004C4B40

9.2.4. Engine Oil Life Warning Total Engine Revolutions (LID: $55)

Specify the engine oil life warning total engine revolutions as 4 byte UNM (unsigned numeric value) data (1bit=1 revolution).

Example: 100,0000,000 [revolution]  0x05F5E100

9.2.5. Engine Oil Life Prior Warning Total Revolutions (LID: $56)

Specify the engine oil life warning total engine revolutions as 4 byte UNM (unsigned numeric value) data (1bit=1 revolution).

Example: 100,0000,000 [revolution]  0x05F5E100

9.2.6. Engine Oil Life Warning - Insoluble Material (LID: $57)

Specify the engine oil life warning for insoluble material as 4 byte UNM (unsigned numeric value) data (1bit=1wt%/100h)).

Example: 100,0000,000 [wt%/100h]  0x05F5E100

9.2.7. Engine Oil Life Prior Warning - Insoluble Material (LID: $58)

Specify the engine oil life prior warning insoluble material as 4 byte UNM (unsigned numeric value) data (1bit=1wt%/100h)).

Example: 100,0000,000 [wt%/100h]  0x05F5E100

9.2.8. Starter Life Warning Number of Starts Data (LID: $59)

Specify the starter life warning number of starts data as 4 byte UNM (unsigned numeric value) data (1bit=1 activation).

Example: 100,000 [activations] 0x000186A0

9.2.9 Exhaust Brake Valve OH Warning Number of Actuations (LID: $5A)

Specify the exhaust brake valve OH warning number of actuations as 4 byte UNM (unsigned numeric value) data

(1bit=1 actuation)

Example: 100,000 [activations] 0x000186A0

9.2.10 ACG OH Warning Drive Distance (LID:$5B)

Specify the ACG OH warning drive distance as 4 byte UNM (unsigned numeric value) data (1bit=1m)

Example: 5,000[km]  5,000,000[m]  0x004C4B40

9.2.11 ACG OH Warning Number of Operations (LID:$5C)

Specify the ACG OH warning number of operations as 4 byte UNM (unsigned numeric value) data (1bit=1 actuation)

Example: 100,000 [activations] 0x000186A0

9.2.12 T/M OH Warning Number of Gear Engagements (LID:$5D)

Specify the T/M OH warning number of gear engagements 4 byte UNM (unsigned numeric value) data

(1bit=1 gear engagement)

Example: 100,000 [gear engagement] 0x000186A0

10. DTC Codes

The self-diagnostics executed by the mimamori ECU are shown below

| DTC | Details | Examination request |
| --- | --- | --- |
| P0601 | Program flash fault | - |
| P0602 | Security unlock status (ECU lock process not executed) | ○ |
| B1000 | ECU fault (RAM flash error) | ○ |
| B1001 | EOL data fault (EOL data not set) | ○ |
| B1008 | T/M EOL data fault | ○ |
| B1011 | CAN EOL data fault | ○ |
| U2101 | Installed ECU EOL data fault | ○ |
| U0073 | CAN (ISO-CAN) communication failure | ○ |
| U2100 | CAN (SAE-J1939-CAN) communication failure | ○ |
| U2105 | Communication failure with ECM | ○ |
| U2106 | Communication failure with smoother/TCM/ACT | ○ |
| U2108 | Communication failure to ABS/ASR/EBS/IESC | ○ |
| U2112 | Communication failure to air suspension ECU | ○ |
| U2116 | Communication failure to multi-meter | ○ |
| U2117 | Communication failure to driving concentration warning (DSA) | ○ |
| U2144 | Communication failure to driving distance warning/ACC/CMS | ○ |
| U2202 | Communication failure to SCR(DCU) | ○ |
| U2203 | Communication failure to Turbo Controller | ○ |
| U2204 | Communication failure to Idling Stop & Start | ○ |
| U2205 | Communication failure to ECON ECU | ○ |