MQS与检测线设备交互约定

The Communication Protocol Of MQS and EOL

| 版 本 Version | 日 期 Data | 作 者 Author | 说 明 Instruction |
| --- | --- | --- | --- |
| 1.0.0 | 2014-7-21 | 严军 Yan Jun | 文档创建，增加“交互总图”，修改交互场景说明及xml格式定义 Create document，add “communication diagrams”，modify the communication explanation and XML format |
| 1.0.1 | 2014-07-21 | 严军 Yan Jun | 更改MES为MQS Change MES to MQS |
| 1.0.2 | 2014-07-22 | 谌田明 Shen Tianming | 删除检测次数字段、新增引车员说明 Delete the "checktime" field , add the "checker" field |
| 1.0.3 | 2014-08-20 | 严军 Yan Jun | MQS返回结果新增“检测类型”字段， MQS feedback message add “test type” field |

一交互总图

Communication diagram：

MQS

EOL Equipment

Test Result Data

Feedback Information  ininformationinformation

Instructions：

- 检测线设备作为Socket Client端主动向MQS发起连接，返回检测结果数据；

The EOL Equipment as the Socket Client  connect the MQS initiatively, and send

the test result to MQS.

- MQS作为Socket Server端，提供服务器IP地址以及端口号（可配置）供设备访问。

The MQS as the Socket Server has its own IP address and port which can be configured.

- Socket通信协议是TCP/IP；

The Socket communication protocol is TCP/IP.

- 如果向检测线设备发送消息失败，MQS会尝试多次发送数据。如果多次尝试失败后（设备宕机），MQS放弃发送此条数据，然后MQS以一定的频率去尝试连接设备。

If MQS send message to EOL equipment failed, it will retry some times. If failed (EOL equipment has broken),MQS will give up. And MQS will reconnect the EOL equipment every once in a fix time.

- 在检测线设备网络或MQS通讯异常时，设备端可以将未上传的数据保存在本地，在网络或系统恢复后具备再次上传功能。

If  the network  or MQS are broken,  EOL equipment can store the test  result  in the  local.After  the network or MQS recover, the EOL PC can upload the data automatically.

二、交互场景：检测结果返回（checkUpload）

communication work: send test result to MQS (chenckUpload)

MQS

EOL Equipment

Test Result data

Information of Feedback

2.1	检测线将检测结果发送给MQS。

EOL equipment send the test result  to MQS.

检测线设备发送给MQS的消息模板，以侧滑结果检测为例：

There is a sample of the message that the equipments send to MQS.

It's the message that side slip tester send to MQS.

<xmldata>

<messagetype>checkUpload </messagetype>

<tranid></tranid><!--the identifier of the message ,it's a primary key , created by the EOL equipment-->

<vin>100000001</vin>

<standarversion></standarversion><!—The version of the buyoff stander-->

<result></result>  // The test result,  the format is : P or F

<checker></checker>// The driver ID

<lineid></lineid>

<stationname></stationname><!-- station  name -->

<trantype></trantype><!--testtype-->

<checkendtime></checkendtime><!-- the time when finished-->

<checktime></checktime><!--test times-->

<sidslip></sidslip><!- - test items -->

<sidbok></sidbok>

</xmldata>

XML说明：

- 此消息分为两个部分：一部分为检测大项数据：tranid, vin, standarversion, result, checker, lineid, stationname, trantype, checkendtime, checktime。设备发送过来的检测消息，必须包含以上所有字段。如果某个字段没有值，则为空值。

This message contain two parts：part one is test items data: tranid, vin, standarversion, result, checker, lineid, stationname, trantype, checkendtime, checktime. The message that  equipment send, must contain these fields. If any field no value, set it a null value.

- 第二部分为具体检测项数据：例如：<sidbok></sidbok>字段名字都小写，以检测线接口表为准，具体检查项内容由供应商提供。

Part two is the specific test data: example :<sidbok></sidbok>field name must be  lowercase , refer to the EOL data interface. The items provided by supplier.

- “messagetype”中“checkUpload”为固定值，用来标识此消息为“检测线设备接口”的发送消息；

The " measagetype " Value is fixed ,its value is " checkUpload ",used to recognize the message that EOL equipment send.

2.2 	MQS收到检测线数据后，返回接收结果给检测线设备。

When MQS receive the test result ,then it send feedback information to EOL equipment.

MQS返回的消息格式如下：

It's the format of the feedback  information .

A Sample.

<xmldata>

<mesagetype>checkUploadResult</mesagetype>

<status>OK/ERROR</status>

<msg>获取成功</msg>

<tranid></tranid>

<vin>100000001</vin>

<trantype>NCA</trantype>//It's the same value as the EOL equipment send "trantype" .

</xmldata>

XML instruction：

- “mesagetype”中“checkUploadResult”为固定值，用来标识此消息为“检测线设备接口”的返回消息；

The " measagetype " Value is fixed ,its value is " checkUploadResult ",used to recognize the message that MQS send.

- “status”、“msg”为接口返回状态、接口返回信息。status取值ERROR:失败，OK:成功。如果是ERROR状态时，检测线设备应该记录并显示出错信息（msg）

“status”、“msg” are the MQS status. The format of  "status" value  is ERROR :fail,  and OK: succeed. If any ERROR status ,EOL equipment should record it and show the fault message in screen.