Basic outline of code

- Three main layers of code core, Customer, and plant.

  - Core: basic functionality that is consistent from rolls machine to rolls machine.

  - Customer: Customer specific functionality that is consistent between a specific customers’ plants.

  - Plant: Code and config changes that are only for that plant.

  - Within main layer there is 2 sub layers Services layer and Application Layer

    - Services layer: Machine control, Plant interfaces, MotorController etc

    - Application layer: Testing components

      - Test Component Layer: actually tests the components  compares values, sends prompts to the front panel etc

      - Module Layer: handles messages and module specific things

      - Protocol Filter: how the messages are formed and interpreted

How the function system works

Objectives are called out during the drive curve sequence. These objectives are located in the components respective (listed out in the beginning of the drive curve) config file.

Each objective has a list of TestSteps that get called out in that order.  Under the objectives in the config file are the test steps themselves. These list out information used by the test step(description, timeout .

These test steps names are what gets processed by the qxe. Each qxe contains a function called CommandTeststep. This takes the TestStep string from the test sequencer and calls the respective function associated with it.

How module messages work

There are multiple sections and a lot of variability in the way module messages can be setup/laid out in the config file. There are 5 main sections.

Vehicle messages

Each xml node under this section defines how a message is constructed and how the reply is interpreted by linking it to the other four sections.

TransmitMessages

These can either be a solid message or a variable transmit message(contain 0x?? to be filled in with transmit interpretations).

TransmitInterpretations

If the vehicle message is using a variable TransmitMessage a TransmitInterpretation is needed.  It fills in the 0x?? in the TransmitMessage

ReplyMessage

This is what determines whether the response is valid

ReplyInterpretation

This is where the module response gets turned into the a usable data type(float , int, string). Not needed if the response is not being read aka only commanding the module.

An example of the layout is included in exampleModuleMessage.xml

Good example of function layout

See exampleFuntion.cpp

List of useful functions

Functions that get parameters from TC config files

Bool GetParameterBool(string parameter)

If no parameter is present returns false

String GetParameter(string parameter)

If no parameter is present returns the string value

Float GetParameterFloat(string parameter)

If no parameter is present returns 0

Int GetPatameterInt(string parameter)

If no parameter is present returns 0

Functions read and write data from the system

String SystemRead(string systemVariable)

Void SystemWrite( string systemVariable, value)

Functions that communicate with module

BEP_STATUS_TYPE ReadModuleData(string messageName, type valueRecieveFromModule)

Type is the type that is expected from the module determined by the reply interpretation

BEP_STATUS_TYPE CommandModule(string messageName)

GetInfo() - This is a legacy function do not use unless you have to