- Compiling Code

  - Used virtual machines of qnx

    - 2 versions 6.2 and 6.32

    - The compliers are different but the code can be the same

- Vmware workstation – runs the virtual machines

  - Map a network drive with address \\IP\home\user

- Exporting the code to the virtual machine

  - Use tortoise SVN and export the code to the mapped QNX drive

    - Or diff the code in

  - Might encounter errors such as file name is to long

    - Use visual slick edit’s file compare tool

      - Multi file and recursive

    - Then copy the tree that has different files

- Compiling code

  - Navigate to the bin folder of the exported code in the QNX virtual machine

    - Use the command: cbuild –DCUSTOMER=”customer” ../Source

    - Use command make install

    - Go to newly created folder deploy

      - Use command createBuild .”name” this will create a tar ball ready for deployment/testing

      - Naming convention “customer.install.date”

- SVN – revision software

  - Check out all of the files necessary for the project

  - Update after compiling and preferable after tested

  - After modifying code update with a useful comment on what was changed and why

  - This software flags modified files may use full to find changed

- Setting up a machine for a client

  - Setting the customer on the QNX machine login using CCRT also need to use ctrl+alt+9 to get to the promt screen

    - Must do this every time there is anew customer

    - Command SetCustomer “customer number” “Customer” “machine serial”

    - This will set the base name link so the xml files are pulled correctly for the customer/machine

    - Set ipaddress using command IpAddress “192.168.lanenumber.device type”

    - Set host name using command makeHost ”customername_machineserial”

    - Restart the qnx machine so the changes take place with the command exit

    - Use command linkRuntime /home/Runtime/

    - And exit to restart again

  - Then go to the windows machine

    - Set a hard ip 192.168.1.2: 255.255.255.0: 192.168.1.1

    - Change the power settings so the pc never goes to sleep

    - Change the name of the pc to Customer_machineSerial

    - Restart

    - Map network drive “g” \\IP\home\ccrt\

      - Have it reconnect at login

    - Drop the newly complied code into the Rewrite folder

  - go back to the qnx machine

    - Then type exit this should restart the machine and automatically install this new software

    - Restart again to bring up the new task manager

  - Back to the widows machine

    - Put the ccrt client into the start up folder

    - Then launch it

    - Will get an error if the ip is not set up

    - Once launched go to tools-> configuration

      - User admin

      - Pass admin

    - Network: IP 192.168.1.1

    - Increase res a bit

  - Back to qnx

    - Exit to restart

- XML

  - ? = don’t care

- Random tidbits and troubleshooting

  - In 632 the cbuild does not create the right files for cmake have to copy them from another profile

  - If you get Cmake error about: CMAKE_SIZEOF_VOID_P  just run it again and it should be fine

  - Job orders

    - Located in P:\projects\00Ban\SOprojects

      - Find the project based on the project number

    - Shows what the job is and what is required to complete it

  - Ethernet association

    - Qnx use Ethernet port in the middle of case

    - Windows – use Ethernet port left side of case

  - TestStep randoms

    - If there is not a parameter for the teststep that is “necessary” This will end up forcing that parameter to false so its best practice to Add them and force them false

    - When looking for a speed test there are 2 tests low and high

      - Low is more for sensing a signal from the sensor

      - High is for an actual speed

  - There is a log file for every thing

  - Always be a super user (su)

  - Dynamic commands will fail bench testing

  - A negative 1 bytes response means that the module didn’t respond

  - Drive curves  objectives  teststeps  module commands

  - PlcTerm can be used to look at what is being communicated to the plc

    - There must be about 50 ish words that get sent to the plc sometimes in the code there are not enough of them so you must add the motor control stuff to it

CODE

- Test sequence -XML

- Xml layers to code

  - Test phase-XML

    - Objectives-XML

      - Test step-XML

        - Code – c++

- Code refers to date on the vehicle that is stored in xml files in the parts under setup and parameters

  - Things such as getparm; getdata

- Useful commands for testing code

  - Runs a process with the second argument being the xml config file

  - ./ApplicationLayer/Bin/NissanAdvicsTC.qxe –f ./Configuration/Components/Brake/NissanAdvicsTC.xml

  - Registers the process with CCRT

  - SendCmd –ncomponent/AdvicsCan –tRegister –xRegister,0

  - Sends individual objective commands to the process

  - SendCmd –ncomponent/AdvicsCan –tCommand –xObjectiveName

Process – how things run

- scan vin number on vehicle

- work cell server gets a large string of information about vehicle needed to run the tests

- this is given to the system monitor to check what state the machine is in

- vehicles data broker to load vehicle build

- vehicle test server compares vehicle info to a xml selection table to decide the test sequence to be used

  - each test is compared to the vehicle build and the more matches = more weight

  - test sequence with the highest weight gets selected

- then goes through the test sequence each test is a qxe

  - each test has phases

  - each phase has a foreground and or background  objectives

  - foreground must be completed to move to the next phase

  - background can finish when ever

- Navigating to code through the xml files

  - Look at a test under application layer\vehicletest.xml

  - Expand test ->testphase->objects ->objectives

  - Under that is a red slash that is the command being used

  - This command is in the component xml file

  - Under the command is test steps these are the code that is run to actually test the machine

- DriverQuestion

  - Is the catch all this is were extra testing goes

  - Can be multiple or single stuff

- System monitor

- telnet 192.168.1.1 will remote access the qnx machine

Start up

- Get loss comp to work

  - Make sure communication to the plc is correct

  - Get rolls spinning

- Set up tests to all run

  - Drive curves

  - Selection tables

  - Basebrake if possible

- Set up front panel to reflect what tests were doing

  - Link the right variables

  - Reports

- Scanners / keypads

- After all is set up

  - Compare configurations with what gets built make sure that testing changes are not in the final build

  - Add all the changed files the ccrt client side software to svn so that its consistent