Setting up a new QNX 6.3.2 PC for a customer

- Change the hostname and IP address

  - (as root) cd  /etc

  - Use “vim  HOSTNAME” to edit hostname

  - Use “vim  IPADDRESS” to edit IP address

- Make sure PC is set to use dual port NIC if equipped

  - Using ifconfig, check to make sure en0 and en1 are listed as network interfaces. If en1 does not exist, you will need to switch which network card is initialized in the netstart file.

  - In the netstart file, there will be a line that says “Use next line for Intel NIC.” Uncomment the following line using the # key.

  - There will be another line that says “Use next 2 lines for on-board Realtek NIC.” Comment the following line using the # key.

- Reboot

- Create customer directory

  - (as CCRT) mkdir  –p  /home/BVAP/Rewrite

- Link Runtime to customer directory

  - (as CCRT) linkRuntime  /home/BVAP

- Load tgz in customer Rewrite directory (from USB drive)

  - (as root) /usr/local/bin/usbstart

  - mount  –tdos  /dev/hd1t11  /usb

  - (as CCRT) cp  /usb/Ccrt.tgz  /home/BVAP/Rewrite

- Set customer data

  - (as root) SetCustomer  DCX  BVAP  1

- Reboot twice (make sure to wait for CCRT to load before rebooting the second time)

- Make sure there are no errors during the launching process (All the core tasks should show up green)

- Press Ctrl-Alt-9, and check to make sure there is no message indicating an invalid license. If the license is invalid, install a valid license file

  - Type ifconfig and write down the MAC address of en0.

  - cd  /config/license

  - Use “ls” to list the contents of the directory, and check to see if a .lic file exists for the MAC address you wrote down.

  - If it does not exist, request a customer PC license from Jeff Salzman, and give him the MAC address you wrote down.

  - A new license file will be emailed to you. Follow the instructions that accompany that license file to install it on the machine, then reboot.

  - If the software still marks the license as invalid, something is wrong with the software build being installed. Seek assistance from someone in the Roll Brake Software team.